| 3GPP TR 28.836 V0.3.0 (2022-11) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Telecommunication management; Study on intent-driven management for network slicing; (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	8

4	Concepts and Overview	8

4.1 	General	8

4.2 	Classification of use cases in TS 28.531	8

4.3 	Background information of Intent-driven management and intent driven management MnS	9

5	Use cases and potential requirements	10

5.1	Intent driven management for Network slice life cycle management	10

5.1.1 	Use case of Intent driven management to deliver a network slice instance	10

5.1.1.1	Introduction	10

5.1.1.2 	Pre-condition	10

5.1.1.3	Description	10

5.1.1.4	Post-condition	10

5.1.1.5	Requirements	10

5.1.2 	Use case of Intent driven management to deliver a network slice instance subnet	11

5.1.2.1	Introduction	11

5.1.2.2	Pre-condition	11

5.1.2.3	Description	11

5.1.2.4	Post-condition	11

5.1.2.5	Requirements	11

5.2	Intent driven management for network slice service assurance	11

5.2.1 	Use case of Intent driven management to express expectation of network slice service assurance	11

5.2.1.1	Introduction	11

5.2.1.2 	Pre-condition	11

5.2.1.3	Description	12

5.2.1.4	Post-condition	12

5.2.1.5	Requirements	12

5.2.2 	Use case of Intent driven management to express expectation of network slice subnet service assurance	12

5.2.2.1	Introduction	12

5.2.2.2 	Pre-condition	12

5.2.2.3	Description	12

5.2.2.4	Post-condition	13

5.2.2.5	Requirements	13

5.A	Use case <<A>>	13

5.A.1	Description	13

5.A.2	Details	13

5.A.3	Potential requirements	13

6	Potential solutions	13

6.1 	Solution for intent-driven management to deliver a network slice instance	13

6.1.1	Description	13

6.1.2	Details	13

6.x.3	Evaluation	14

6.B	 Solution <<B>>	15

6.B.1	Description	15

6.B.2	Details	15

6.B.3	Evaluation	15

7 	Conclusion and Recommendation	15

Annex A	15

A.1 	Key issues of intent driven management for network slice life cycle management	15

A.2 	Key issues of network slice service assurance	15

Annex <X> (informative): Change history	16

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

With the recent introduction of intent driven management a service provider or network operator can manage 5G network and services through the expression of intents (expectations). Network slice requirements are specified in service profile(s) which can be viewed as expectations on a network slice. The present document studies the use of intent driven management for management of network slices.

# 1	Scope

The present document specifies potential use cases, requirements, and solutions for intent driven  management for network slicing. The document provides conclusions and recommendations on the next steps in the standardization.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements"

[3] 	3GPP TS 28.531: "Management and orchestration; Provisioning"

[4]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3"

[5]	3GPP TR 28.541: "Management and orchestration; Intent driven management services for mobile networks"

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Concepts and Overview

### 4.1 	General

Network slicing is one of the key 5G features. The lifecycle of a Network Slice instance can be described by the following: commissioning, operation and decommissioning. Before a NetworkSlice instance can be created there is a preparation phase. The different phases are described in more detail in TS 28.530 [2].

The attributes in ServiceProfile represent the network slice related requirements, and the attributes in SliceProfile represent network slice subnet related requirements. The uses of ServiceProfile and SliceProfile are described in TS 28.531 [3] and TS 28.541 [4].

Figure 4.1-1: Intent-driven MnS

In Intent-driven management, the MnS consumer provides its intent to the MnS producer. The producer of an Intent-driven MnS shall allow the consumer to manage the service and / or network resources through the use of intents.

### 4.2 	Classification of use cases in TS 28.531

The use cases for provisioning of a network slice instance described in TS 28.530 [2] and TS 28.531 [3] are as follows:

-	Create a network slice instance;

-	Activate a network slice instance;

-	De-active a network slice instance;

-	Modify a network slice instance;

-	Terminate a network slice instance.

The technical specification TS 28.531 [3] describes specification level use cases related to network slice management. Table 4.2-1 provides the classification of those use cases related to network slice instance and network slice subnet management capabilities. The detail information about use cases and management capability supporting the use cases are described in TS 28.531 [3].

Table 4.2-1: The classification of use cases in TS 28.531

| Clause | Use case | Management capabilities required |
| --- | --- | --- |
| 5.1.1 | Network slice instance allocation | Provisioning of a network slice |
| 5.1.2 | Network slice instance subnet allocation | Provisioning of a network slice subnet |
| 5.1.3 | Network slice instance deallocation | Provisioning of a network slice |
| 5.1.4 | Network slice subnet instance deallocation | Provisioning of a network slice subnet |
| 5.1.5 | Obtaining network slice subnet instance information | Query information |
| 5.1.6 | Network slice feasibility check | Provisioning of a network slice (subnet) |
| 5.1.7 | Network slice instance activation | Provisioning of a network slice |
| 5.1.8 | Network slice instance deactivation | Provisioning of a network slice |
| 5.1.9 | Network slice instance modification | Modification of a network slice |
| 5.1.10 | Network slice subnet instance activation | Provisioning of a network slice subnet |
| 5.1.11 | Network slice subnet instance deactivation | Provisioning of a network slice subnet |
| 5.1.12 | Network slice subnet instance modification | Modification of a network slice subnet |
| 5.1.13 | Network slice subnet configuration | Modification of a network slice subnet |
| 5.1.14 | Exposure of network slice management data | Query information |
| 5.1.15 | Exposure of network slice management capability | Management capability exposure |
| 5.1.16 | Network slice subnet instance management capability exposure | Management capability exposure |

Editor’s Note: the Table 4.2-1 may be updated according to the outcome of release 17 work item eNETSLICE_PRO.

### 4.3 	Background information of Intent-driven management and intent driven management MnS

The intent driven management is specified in TS 28.312 from release 17. As described in TS 28.312, the intent specifies the expectations including requirements, goals and constraints for a specific service or network management workflow.

As described in TS 28.312 clause 4.1.2, based on network slice management, the intent can be categorized based on user types as follows:

- Intent-CSC MnS producer provides intent driven MnS for communication services.

- Intent-CSP MnS producer provides intent driven MnS for network services.

- Intent-NOP MnS producer provides intent driven MnS for network equipment.

Similar with the intent types for CSC, CSP and NOP, the TS 28.530 [2] describes roles related to 5G network and network slicing, some examples of roles are:

-	Communication Service Customer (CSC): Uses communication services.

-	Communication Service Provider (CSP): communication service provided by CSP can be built with or without network slice

-	Network Operator (NOP): Designs, builds and operates networks and provides related services, including network services and network slices.

# 5	Use cases and potential requirements

## 5.1	Intent driven management for Network slice life cycle management

### 5.1.1 	Use case of Intent driven management to deliver a network slice instance

#### 5.1.1.1	Introduction

To satisfy the requirement of network operator for allocation of a network slice instance with certain characteristics, the 3GPP management system requests creation of new or using existing network slice instance.

#### 5.1.1.2 	Pre-condition

CSP provides the intent driven management service to CSC to create a communication service.

Network operator is capable to provide the life cycle management of network slice instance in management system to invoke the proper intent driven MnS or existing MnSs.

#### 5.1.1.3	Description

In order to enable a group of end users to connect to communication service provided by a 3GPP network, a MnS Consumer expresses its intent to order a communication service to network operator.

The operator provides the expectation of intent of creating a network slice instance with a set of network characteristics. The intent driven management service producer is capable to translate the intent of creating a network slice instance to the network operator’s intent to relevant requirements of creating a network slice instance. Regarding the intent MnS request received, the intent driven-management service provider may decide management capabilities to meet the requirements of creating a network slice instance, for example allocating a network slice instance. The intent driven management service producer is also capable to provide the management capabilities to support the provisioning of a network slice instance addressing the capability of network slice instance, QoS requirement and other requirement derived from the expression of the intent.

#### 5.1.1.4	Post-condition

The communication service creation operation expressed by intent for a specified group of end users is fulfilled.

#### 5.1.1.5	Requirements

REQ-Intent_Deploy_Slice-CON-1: The intent driven MnS shall have capability enabling MnS consumer to express intent containing an expectation for delivering a communication service.

REQ-Intent_Deploy_Slice-CON-2: The intent driven MnS shall have capability enabling MnS consumer to express intent containing an expectation for delivering a network slice instance.

### 5.1.2 	Use case of Intent driven management to deliver a network slice instance subnet

#### 5.1.2.1	Introduction

To satisfy the requirement of network operator for allocation of a network slice instance subnet with certain characteristics, the 3GPP management system requests creation of new or using existing network slice subnet instance. The examples of network slice subnet instance are CN slice subnet instance or RAN slice subnet instance.

#### 5.1.2.2	Pre-condition

Network operator provides the intent driven management service to create a network slice subnet instance.

Network operator is capable to provide the life cycle management of network slice subnet instance in management system to invoke the proper intent driven MnS or existing MnSs.

#### 5.1.2.3	Description

In order to create the network slice subnet provided by a 3GPP network, a MnS Consumer expresses its intent to create network slice subnet instance.

The operator provides the expectation of intent to deliver a network slice subnet instance with a set of network characteristics. The intent driven management service producer is capable to translate the intent of creating a network slice subnet instance with relevant requirements of creating a network slice subnet instance. Regarding the intent MnS request received, the intent driven-management service provider may decide management capabilities to meet the requirements of creating a network slice subnet instance, for example allocating a network slice subnet instance. The intent driven management service producer should be capable to provide the management capability to support the provisioning of a network slice subnet instance addressing the capability of network slice subnet instance, QoS requirement and other requirement derived from the expression of the intent.

#### 5.1.2.4	Post-condition

The network slice subnet instance creation operation expressed by intent for a specified group of end users is fulfilled.

#### 5.1.2.5	Requirements

REQ-Intent_Deploy_Slice-CON-3: The intent driven MnS shall have capability enabling MnS consumer to express intent containing an expectation for delivering a network slice subnet instance.

## 5.2	Intent driven management for network slice service assurance

### 5.2.1 	Use case of Intent driven management to express expectation of network slice service assurance

#### 5.2.1.1	Introduction

To satisfy the requirement of network operator for network slice instance service assurance with certain characteristics, the intent driven MnS producer receive the intent expectation of network slice service assurance and provide the management capabilities to support network slice service assurance.

#### 5.2.1.2 	Pre-condition

CSP provides the intent driven management service to CSC to create a communication service.

Network operator is capable to provide network slice instance service assurance in management system to invoke the proper intent driven MnS.

#### 5.2.1.3	Description

In order to enable a group of end users to connect to communication service provided by a 3GPP network, a MnS Consumer expresses its intent for a communication service assurance with proper network characteristics to network operator.

The operator provides the expectation of intent of network slice instance service assurance with a set of network characteristics. The intent driven management service producer is capable to translate the intent of a network slice instance service assurance to relevant requirements e.g., keep QoS performance in specific time duration. Regarding the intent MnS request received, the intent driven-management service provider may decide management capabilities to meet the requirements of assurance a network slice instance service quality, for example monitoring particular performance, use closed control loop for assurance target(s). The intent driven management service producer is also capable to provide the report of intent fulfilment of a network slice instance service assurance addressing the capability QoS requirement of the intent.

#### 5.2.1.4	Post-condition

The communication service assurance expectation expressed by intent for a specified group of end users is fulfilled.

#### 5.2.1.5	Requirements

REQ-Intent_Assure_Slice-CON-1: The intent driven MnS shall have capability enabling MnS consumer to express intent containing an expectation for communication service assurance.

REQ-Intent_Assure_Slice-CON-2: The intent driven MnS shall have capability enabling MnS consumer to express intent containing an expectation for network slice service assurance.

## 5.2.2 	Use case of Intent driven management to express expectation of network slice subnet service assurance

#### 5.2.2.1	Introduction

To satisfy the requirement of network operator for network slice subnet instance service assurance with certain characteristics, the intent driven MnS producer receive the intent expectation of slice subnet instance service assurance and provide the management capabilities to support network slice subnet service assurance.

#### 5.2.2.2 	Pre-condition

NOP provides the intent driven management service to create a network slice subnet instance.

Network operator is capable to provide network slice subnet instance service assurance in management system to invoke the proper intent driven MnS.

#### 5.2.2.3	Description

In order to enable a group of end users to connect to communication service provided by a 3GPP network, a MnS Consumer expresses its intent for a communication service assurance with proper network characteristics to network operator.

The network operator provides the expectation of intent of network slice subnet instance service assurance with a set of network characteristics. The intent driven management service producer is capable to translate the intent of a network slice subnet instance service assurance to relevant requirements e.g., keep QoS performance in network slice subnet. Regarding the intent MnS request received, the intent driven-management service provider may decide management capabilities to meet the requirements of assurance a network subnet slice instance service quality, for example monitoring particular performance measurement, use closed control loop for assurance target(s) of the network slice subnet instance. The intent driven management service producer is also capable to report of intent fulfilment of a network slice subnet instance service assurance addressing the capability QoS requirement of the intent.

#### 5.2.2.4	Post-condition

The communication service assurance expectation expressed by intent for a specified group of end users is fulfilled.

#### 5.2.2.5	Requirements

REQ-Intent_Assure_Slice-CON-3: The intent driven MnS shall have capability enabling MnS consumer to express intent containing an expectation for network slice subnet service assurance.

## 5.A	Use case <<A>>

### 5.A.1	Description

### 5.A.2	Details

### 5.A.3	Potential requirements

# 6	Potential solutions

## 6.1 	Solution for intent-driven management to deliver a network slice instance

### 6.1.1	Description

This potential solution describes the possible actions of an intent MnS to deliver one network slice instances for the phase of the intent deployment. The intent-driven MnS allows a consumer to declare the desired intent following information model defined in 3GPP TS 28.312 [5] and implements a number of intent lifecycle management functionalities to reach a successful deployment of the intent, given by the availability of one or more network slice instances capable of meeting the intent expectations.

### 6.1.2	Details

Figure 6.1.2.1 represents the system components for the intent-driven management procedure to deliver a network slice instance. In the proposed solution:

- The Intent Owner acts as intent MnS consumer.

- The Intent Handler acts as intent MnS producer.

- Both Intent Owner and Intent Handler belong to the CSP administrative domain.

- The intent issued by the Intent Owner and fulfilled by the Intent Handler belongs to the Intent-CSP category. For further details on the different intent categories, see 3GPP TS 28.312 [5], clause 4.1.2.

Figure 6.1.2.1: Potential solution for intent-driven management to deliver a network slice

The Intent Owner issues an intent to the Intent Handler. The intent is initially validated by the Intent Validation module, which verifies (i) the compliance of the intent format with the information model defined in 3GPP TS 28.312 [5]; and (ii) the consistency among the intent expectations that constitute the whole intent. The intent asks for the accommodation of the requirements of one or more communication services.

Once validated, the intent is sent over to the Intent Translation module, which is in charge of translating the intent semantics into a number of executable actions. In this case, the executable actions consist of provisioning and activation of one or more network slice instances, re-using one or more existing network slice instances, re-configuring one or more existing network slice instances or a combination of them. To trigger the resulting executable actions, the Intent Translation module consumes management services produced by the network slice provisioning service [3].

NOTE: In case of new or updated network slices, the ServiceProfile [4] is also elaborated by the Intent Translation module on the basis of the intent expectations.

Finally, after the enforcement of executable actions, there is a need to verify whether the intent is fulfilled or not. In this vein, the Intent Fulfilment Evaluation module is defined. This module makes use of information retrieved from network slice provisioning service, and computes a validation result. This result is included in the intent report which is sent back to the intent owner.

### 6.x.3	Evaluation

The proposed solution splits the capabilities of the intent-driven MnS producer (see 3GPP TS 28.312 [5], clause 4.2.2) in dedicated modules that interacts together to implement the whole action of the intent deployment applied to the delivery of a network slice instance.

The translation module may use a rule-based mechanism to translate the intent expectations into one or more ServiceProfile [4] constructions. More elaborated solutions may involve AI/ML based mechanisms to improve the quality of the translation decisions.

The intent fulfilment evaluation module may use a rule-based mechanism to evaluate whether the expectations of the intent are satisfied or not. In case of failed validation, closed loop mechanisms potentially assisted by AI/ML based mechanisms may be adopted to suggest countermeasures that the intent deployment module may apply in the context of closed loop network automation.

## 6.B		Solution <<B>>

### 6.B.1	Description

### 6.B.2	Details

### 6.B.3	Evaluation

# 7 	Conclusion and Recommendation

Annex A

# A.1 	Key issues of intent driven management for network slice life cycle management

Regarding the use case of intent driven management for network slice life cycle management as described in clause 5.x in this document, the key issues derived by those use cases to investigate the technical solutions in this study are:

-	What and how to express the requirements of network slice (subnet) life cycle management.

-	What and how to report the fulfilment of management requirement of network slice (subnet) life cycle management expressed by intent.

-	What and how to use the attributes in service profile and slice profile (including the network slice subnet profile) which are used to form intent driven management of network slice (subnet) life cycle management.

# A.2 	Key issues of network slice service assurance

Regarding the use case of service assurance in scenario of network slice instance and network slice subnet instance assurance as described in clause 5 in this document, the intent driven management should take follow key issues into account:

-	What and how intent driven management expresses the requirements of assurance of network slice instance and network slice subnet instance, based on the input attributes of service profile and slice profile.

-	What and how intent driven management reports the fulfilment of assurance requirements of network slice (subnet) instance.

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-04 | SA5#142-e | S5-222098 |  |  |  | skeleton | 0.0.0 |
| 2022-04 | SA5#142-e | S5-222039 |  |  |  | Structure | 0.1.0 |
| 2022-04 | SA5#142-e | S5-222040 |  |  |  | Add scope | 0.1.0 |
| 2022-04 | SA5#142-e | S5-222041 |  |  |  | Add introduction | 0.1.0 |
| 2022-04 | SA5#142-e | S5-222636 |  |  |  | Add background of mapping in TS 28.531 | 0.1.0 |
| 2022-04 | SA5#142-e | S5-222637 |  |  |  | Add background information related to 28.312 | 0.1.0 |
| 2022-05 | SA5-143-e | S5-223572 |  |  |  | pCR 28.836 Add concept of service profile and slice profile | 0.2.0 |
| 2022-05 | SA5-143-e | S5-223573 |  |  |  | pCR 28.836 Add use case of network slicing allocation | 0.2.0 |
| 2022-05 | SA5-143-e | S5-223574 |  |  |  | pCR 28.836 Add key issue of network slice life cycle management | 0.2.0 |
| 2022-05 | SA5-143-e | S5-223575 |  |  |  | pCR 28.836 Add key issue of network slice service assurance | 0.2.0 |