C:\Users\shinji_w\AppData\Roaming\Microsoft\Templates\STD\ISO-IEC_23001-10_(E).docCOMMITTEE DRAFT© ISO/IEC 2014 – All rights reservedISO/IEC CD 23001-10 63Part 10: Carriage of Timed Metadata Metrics of Media in ISO Base Media File FormatInformation technology — MPEG systems technologiesÉlément introductif — Élément central — Partie 10: Titre de la partieInformation technology — MPEG systems technologies — Part 10: Carriage of Timed Metadata Metrics of Media in ISO Base Media File FormatE2014-01-17(30) CommitteeISO/IECISO/IEC J   International Standard2014 ISO/IEC 23001-ISO/IEC 23001-10ISO/IEC CD 23001-10  Coding of audio, picture, multimedia and hypermedia informationInformation technology11291 2見出し 2見出し 1    02 STD Version 2.1c230   4            ISO/IEC JTC 1/SC 29 N

Date:   2015-02-20

ISO/IEC FDIS 23001-10

ISO/IEC JTC 1/SC 29/WG 11

Secretariat:

Information technology — MPEG systems technologies — Part 10: Carriage of Timed Metadata Metrics of Media in ISO Base Media File Format

Élément introductif — Élément central — Partie 10: Titre de la partie

Warning

This document is not an ISO International Standard. It is distributed for review and comment. It is subject to change without notice and may not be referred to as an International Standard.

Recipients of this draft are invited to submit, with their comments, notification of any relevant patent rights of which they are aware and to provide supporting documentation.

Copyright notice

This ISO document is a working draft or committee draft and is copyright-protected by ISO. While the reproduction of working drafts or committee drafts in any form for use by participants in the ISO standards development process is permitted without prior permission from ISO, neither this document nor any extract from it may be reproduced, stored or transmitted in any form for any other purpose without prior written permission from ISO.

Requests for permission to reproduce this document for the purpose of selling it should be addressed as shown below or to ISO's member body in the country of the requester:

[Indicate the full address, telephone number, fax number, telex number, and electronic mail address, as appropriate, of the Copyright Manger of the ISO member body responsible for the secretariat of the TC or SC within the framework of which the working document has been prepared.]

Reproduction for sales purposes may be subject to royalty payments or a licensing agreement.

Violators may be prosecuted.

Contents	Page

1	Scope	1

2	Normative references	1

3	Terms, definitions and abbreviated terms	1

3.1	Terms and definitions	1

3.2	Abbreviated terms	2

4	Carriage of Quality Metadata	2

4.1	Introduction	2

4.2	Quality Metadata	2

4.2.1	Definition	2

4.2.2	Syntax	2

4.2.3	Semantics	3

4.3	Quality Metrics	3

4.3.1	Peak Signal to Noise Ratio (PSNR)	3

4.3.2	SSIM	4

4.3.3	MS-SSIM	5

4.3.4	VQM	7

4.3.5	PEVQ	7

4.3.6	MOS	8

4.3.7	Frame significance (FSIG)	8

5	Carriage of Green Metadata	10

5.1	Introduction	10

5.2	Decoder Power Indication Metadata	10

5.2.1	Definition	10

5.2.2	Syntax	10

5.2.3	Semantics	10

5.3	Display Power Reduction Metadata	10

5.3.1	Display Power Indication Metadata	11

5.3.1.1	Definition	11

5.3.1.2	Syntax	11

5.3.1.3	Semantics	11

5.3.2	Display Fine Control Metadata	11

5.3.2.1	Definition	11

5.3.2.2	Syntax	12

5.3.2.3	Semantics	12

Foreword

ISO (the International Organization for Standardization) and IEC (the International Electrotechnical Commission) form the specialized system for worldwide standardization. National bodies that are members of ISO or IEC participate in the development of International Standards through technical committees established by the respective organization to deal with particular fields of technical activity. ISO and IEC technical committees collaborate in fields of mutual interest. Other international organizations, governmental and non-governmental, in liaison with ISO and IEC, also take part in the work. In the field of information technology, ISO and IEC have established a joint technical committee, ISO/IEC JTC 1.

International Standards are drafted in accordance with the rules given in the ISO/IEC Directives, Part 2.

The main task of the joint technical committee is to prepare International Standards. Draft International Standards adopted by the joint technical committee are circulated to national bodies for voting. Publication as an International Standard requires approval by at least 75 % of the national bodies casting a vote.

Attention is drawn to the possibility that some of the elements of this document may be the subject of patent rights. ISO and IEC shall not be held responsible for identifying any or all such patent rights.

ISO/IEC 23001-10 was prepared by Joint Technical Committee ISO/IEC JTC 1, Information technology, Subcommittee SC 29, Coding of audio, picture, multimedia and hypermedia information.

ISO/IEC 23001 consists of the following parts, under the general title Information technology — MPEG systems technologies:

# Part 1: Binary MPEG format for XML

# Part 2: Fragment Request Units

# Part 3: XML IPMP messages

# Part 4: Codec configuration representation

# Part 5: Bitstream Syntax Description Language (BSDL)

# Part 7: Common encryption in ISO base media file format files

# Part 8: Coding-independent code-points

# Part 9: Common Encryption for MPEG-2 Transport Streams

# Part 10: Carriage of Timed Metadata Metrics of Media in ISO Base Media File Format

# Part 11: Energy-Efficient Media Consumption (Green Metadata)

Introduction

This part of ISO/IEC 23001 specifies the carriage of timed metadata related to two fields, in files belonging to the family based on ISO/IEC 14496-12 The ISO base media file format. The two families of metadata are ‘green’ metadata (related to energy conservation) and quality measurements of the associated media data (related to video quality metrics).

Information technology — MPEG systems technologies — Part 10: Carriage of Timed Metadata Metrics of Media in ISO Base Media File Format

# Scope

This part of ISO/IEC 28001 defines a storage format for timed metadata metrics. The timed metadata metrics can be associated with other tracks in the ISO Base Media File Format. Typical timed metadata – quality and power consumption information and their metrics – are defined in the specification for carriage in files based on the ISO Base Media File Format (ISO/IEC 14496-12 and ISO/IEC 15444-12). The timed metadata can be used for multiple purposes including supporting dynamic adaptive streaming.

# Normative references

The following documents, in whole or in part, are normatively referenced in this document and are indispensable for its application. For dated references, only the edition cited applies. For undated references, the latest edition of the referenced document (including any amendments) applies.

ISO/IEC 14496-10, Information technology — Coding of audio-visual objects — Part 10: Advanced Video Coding

ISO/IEC 14496-12, Information technology — Coding of audio-visual objects — Part 12: ISO base media file format)

ISO/IEC 23001-11, Information technology — MPEG Systems Technologies — Part 11: Energy-Efficient Media Consumption (Green Metadata)

ISO/IEC 23008-2, Information technology — High efficiency coding and media delivery in heterogeneous environments — Part 2: High efficiency video coding

ITU-T Recommendation J.144, Objective perceptual video quality measurement techniques for digital cable television in the presence of a full reference

ITU-T Recommendation J.247, Objective perceptual multimedia video quality measurement in the presence of a full reference

ITU-R BT.500-12 Recommendation: Methodology for the subjective assessment of the quality of television pictures

IETF RFC 6381, The 'Codecs' and 'Profiles' Parameters for "Bucket" Media Types, August 2011

# Terms, definitions and abbreviated terms

## Terms and definitions

For the purposes of this document, the terms and definitions given in ISO/IEC 14496-10, ISO/IEC 23008-2, and the following apply.

## Abbreviated terms

FSIG	Frame SIGnificance

MOS	Mean Opinion Score

MSE	Mean Signal Error

MS-SSIM	Multi-Scale Structural SIMilarity index

PEVQ	Perceptual Evaluation of Video Quality

PSNR 	Peak Signal to Noise Ratio

SSIM 	Structural SIMilarity Index

VQM	Video Quality Metric

# Carriage of Quality Metadata

## Introduction

If quality metrics are carried in an ISO Base Media File Format, they shall be carried in the metadata tracks within the ISO Base Media File Format. Different metric types and corresponding storage formats are identified by their unique code names. This section defines those quality metrics.

The metadata track is linked to the track it describes by means of a ‘cdsc’ (content describes) track reference.

Codes not defined in this specification are reserved and files must use only codes defined here.

## Quality Metadata

### Definition

Sample Entry Type:	‘vqme’ 
Container: 	Sample Description Box (‘stsd’)
Mandatory:	No
Quantity:	0 or 1

The sample entry for video quality metrics is  defined by the QualityMetricsSampleEntry.

The quality metrics sample entry shall contain a QualityMetricsConfigurationBox, describing metrics that are present in each sample, and the constant field size that is used for the values. The quality metrics are defined in section 4.3.

Each sample is an array of quality values, corresponding one for one to the declared metrics. Each value is padded by preceding zero bytes, as needed, to the number of bytes indicated by field_size_bytes.

The codecs parameter value for this track as defined in RFC 6381 shall be set to ‘vqme’.The sub-parameter for the ‘vqme’ codec is a list of the metrics present in the track as indicated by the metrics code names, joined by “+”, e.g. ‘vqme.psnr+mssm’.

### Syntax

aligned(8) class QualityMetricsSampleEntry() 
	extends MetadataSampleEntry (‘vqme’) {
	QualityMetricsConfigurationBox();
}

aligned(8) class QualityMetricsConfigurationBox
	extends FullBox(‘vqmC’, version=0, 0){
	unsigned int(8) field_size_bytes;
	unsigned int(8) metric_count;
	for (i = 1 ; i <= metric_count ; i++){
		unsigned int(32) metric_code;
		
			
	}
}

### Semantics

field_size_bytes indicates the constant size in byte of the value for a metric in each sample

metric_count the number of metrics for quality values in each sample

metric_code is the code name of the metrics in the sample

## Quality Metrics

### Peak Signal to Noise Ratio (PSNR)

#### Definition

PSNR for encoded video sequence is defined based on per-picture mean square error (MSE) differences:

where I denotes luma plane of the reference m×n picture, K denotes luma plane of the reconstructed picture, and i,j denote indices enumerating all pixel locations.

The picture-level PSNR is defined as:

where MAXI  = 2B−1 where B is the number of bits per sample in pictures.

PSNR for a given video sequence is computed as an average of all picture-level PSNR values obtained for all pictures in the sequence, i.e., for a sequence with N pictures we have

Only luma component of the video signal is used for PSNR computation.

Note 1:	This is the traditional metric referred to as PSNR in the academic literature and in the context of video compression research.

Note 2:	In cases when the spatial resolution of the reference pictures and the reconstructed ones do not match, reconstructed pictures must be up-sampled to match the spatial resolution of the reference

Note 3:	In cases when the pictures of reconstructed video represent only a subset of pictures in the reference video sequence, reconstructed pictures must be replicated to produce time-aligned reconstructed pictures for all pictures in the reference sequence.

#### Metric code name

PSNR quality metric values shall be provided as ones under the ‘psnr’ metric code name.

#### Sample storage format

Each PSNR metric value shall be stored as an unsigned 16-bit integer value.

#### Decoding operation

Given stored 16-bit integer value x, the corresponding PSNR value (in dB) is derived as follows (expressed in floating point):

PSNR = (real) x / 100; with the exception of PSNR = infinity for x=0

### SSIM

#### Definition

SSIM for encoded video sequence is defined based on SSIM index map obtained for each picture. Per-picture SSIM index map is computed as follows:

where

x   	denotes the 8x8 window in the reference picture

y 	denotes the 8x8 window in the reconstructed picture

denotes the average sample value for pixels in x

denotes the average sample value for pixels in y

denotes the variance computed for pixel values in x

denotes the variance computed for pixel values in y;

denotes the covariance computed for pixel values in x and y;

and where

,

are constants computed using

, , and  L = 2B−1

where B is the number of bits per sample in reference video.

This formula is applied using an 8x8 sliding window, and producing a map of SSIM index values for all pixel positions within a picture. The overall SSIM index is then computed as the average of index values in the SSIM map.

This formula is applied only on luma components in each picture.

SSIM for video sequence is computed as an average of all picture-level SSIM values obtained for all pictures in the sequence, i.e., for a sequence with N pictures we have

Note 1:	This is the traditional metric referred to as SSIM in the academic literature and in the context of video compression research [1].

Note 2:	The nominal range of SSIM index values is [-1..1].

Note 3:	In cases when the resolution of the reference pictures and the reconstructed ones do not match, reconstructed pictures must be up-sampled to match the resolution of the reference

Note 4:	In cases when the pictures of reconstructed video represent only a subset of pictures in the reference video sequence, reconstructed pictures must be replicated to produce time-aligned reconstructed pictures for all pictures in the reference sequence.

#### Metric code name

SSIM quality metric values shall be provided under the ‘ssim’ metric code name.

#### Sample storage format

Each SSIM metric value shall be stored as an unsigned 8-bit integer value.

#### Decoding operation

Given stored 8-bit integer value x, the corresponding SSIM value  is derived as follows (expressed in floating point):

SSIM = (real) (x-127) / 128.

### MS-SSIM

#### Definition

MS-SSIM calculation procedure is described in Figure 1. Taking the reference and distorted image signals as the input, the system iteratively applies a low-pass filter and downsamples the filtered image by a factor of 2. The original scale is indexed by j=1 and the highest scale is indexed by j=M, for M-1 levels of iteration. Further details can be found in [2].

Figure 1 – MS-SSIM calculation procedure

Based on such M scales of processing, MS-SSIM for encoded video sequence is defined as follows:

,

where:

- is the contrast comparison at scale j (j=1,…M) given by

- is the structure comparison at scale j (j=1,…,M) given by

- is the luma comparison (only computed at scale M) given by

where

x   	denotes the 8x8 window in the reference picture

y 	denotes the 8x8 window in the reconstructed picture

denotes the average sample value for pixels in x

denotes the average sample value for pixels in y

denotes the variance computed for pixel values in x

denotes the variance computed for pixel values in y;

denotes the covariance computed for pixel values in x and y;

and where

, , ,  and

are constants computed using

, , and  L = 2B−1

where B is the number of bits per sample in reference video.

This formula is applied only on luma components in each picture.

MS-SSIM for video sequence is computed as an average of all picture-level MS-SSIM values obtained for all pictures in the sequence, i.e., for a sequence with N pictures we have

#### Metric code name

MS-SSIM quality metric values shall be provided under the ‘msim’ metric code name.

#### Sample storage format

Each MS-SSIM metric value shall be stored as an unsigned 8-bit integer value.

#### Decoding operation

Given stored 8-bit integer value x, the corresponding MS-SSIM value shall be derived as follows (expressed in floating point):

MS-SSIM = (real) (x-127) / 128;

### VQM

#### Definition

VQM for encoded video sequence is defined as described in ITU-T Recommendation J.144.

#### Metric code name

VQM quality metric values shall be provided under the ‘j144’ metric code name.

#### Sample storage format

Each VQM metric value shall be stored as an unsigned 8-bit integer value.

#### Decoding operation

Given stored 8-bit integer value x, the corresponding VQM score is derived as follows (expressed in floating point):

VQM = (real)  x  / 50;

### PEVQ

#### Definition

PEVQ for encoded video sequence is defined as described in ITU-T Recommendation J.247.

#### Metric code name

PEVQ quality metric values shall be provided as ones carrying ‘j247’ metric code name.

#### Sample storage format

Each PEVQ metric value shall be stored as an unsigned 8-bit integer value.

#### Decoding operation

Given stored 8-bit integer value x, the corresponding PEVQ score is derived as follows (expressed in floating point):

PEVQ = (real)  x  / 50;

### MOS

#### Definition

MOS for encoded video sequence is defined as the arithmetic average of result of a set of standard, subjective tests [1] where a number of viewers rate the video sequence.

The MOS provides a numerical indication of the perceived quality from the users' perspective of received media after compression. The MOS is expressed as a single number in the range 1 to 5, where 1 is the lowest perceived quality, and 5 is the highest perceived quality. It can be obtained with reference to ITU recommendation ITU-R BT.500-12.

#### Metric code name

MOS quality metric values shall be provided as ones under the ‘mops’ metric code name.

#### Sample storage format

Each MOS metric value shall be stored as an unsigned 8-bit integer value.

#### Decoding operation

Given stored 8-bit integer value x ranging from 0 to 250 (251~255 are reserved) , the corresponding MOS value is derived as follows (expressed in floating point):

MOS = ceil((real) x / 50);

ceil(x) is an function which gives the smallest integer not less than x.

### Frame significance (FSIG)

#### Definition

FSIG, or frame significance, charaterizes the relative importance of frames in a video sequence, and the sequence level visual impact from various combinations of frame losses, e.g, from dropping a temporal layer, can be estimated from this frame significance representation.

For a sequence with frames {f1, f2, ..., fn}, The Frame Significance (FSIG) for frame fk is defined as,

vk=d(fk, fk-1)

where d() is the frame difference function of two successive frames in the sequence. It is a differential function that captures the rate of change in the sequence [5], and  is computed from the Eigen appearance metric of the scaled thumbnails [4][5] of the frames,

dfj, fk=S*fj-S*fkTATA(S*fj-S*fk)

where S is the  bi-cubicle smoothing and down-scaling function that brings the frames to the size of h x w pixels, and the matrix A is a metric of size d x (h x w), where d is the desired dimension of the metric. The metric A is computed from Eigen appearance modeling of thumbnail frames at size h=12, w=16, and d=12, its values are provided in the Appendix.

To characterize the QoE impact of different temporal layers in a video sequence, the visual impact of frame losses are computed from the FSIG in the following fashion. Let the frame loss index be, L={l1, l2, .., ln}, where lk =1 if there is a frame loss at time stamp k, and lk=0, if no frame loss, then the frame losses induced distortion is computed as,

DL=1nk=1nlkj=kpk+1e-a(k-j)vj

where p(k) is the last frame played in the sequence before the loss at frame time k. An exponentially decaying weight function with kernel size a=1 is introduced to model the temporal masking effects for consecutive frame losses.

#### Metric code name

FSIG quality metric values shall be provided as ones carrying ‘fsig‘ metric code name.

#### Sample storage format

Each FSIG metric value is limited to the max value of 255 and shall be stored as an unsigned 8-bit integer value.

#### Decoding operation

Given stored 8-bit unsigned integer value x, the corresponding FSIG value is directly decoded.

# Carriage of Green Metadata

## Introduction

If Green Metadata is carried in an ISO Base Media File Format, it shall be carried in the metadata tracks within the ISO Base Media File Format. Different Green Metadata types and corresponding storage formats are identified by their unique sample entry codes.

A metadata track carrying Green metadata is linked to the track it describes by means of a ‘cdsc’ (content describes) track reference.

## Decoder Power Indication Metadata

### Definition

Sample Entry Type:	‘depi’ 
Container: 	Sample Description Box (‘stsd’)
Mandatory:	No
Quantity:	0 or 1

The Decoder-Power Indication  Metadata is defined in ISO/IEC 23001-11 -  Energy-Efficient Media Consumption (Green Metadata). It provides decoder complexity reduction ratios for the media track to which the metadata track refers by means of ‘cdsc’ reference.

### Syntax

The decoder power indication metadata sample entry shall be as follows.

class DecoderPowerIndicationMetaDataSampleEntry() 
	extends MetaDataSampleEntry (‘depi‘) {

}

The Decoder-Power Indication sample shall conform to the following syntax :

aligned(8) class DecoderPowerIndicationMetaDataSample(){ 
	unsigned int(8) Dec_ops_reduction_ratio_from_max;
	signed int(16)  Dec_ops_reduction_ratio_from_prev;
}

### Semantics

Semantics are defined in ISO/IEC 23001-11.

## Display Power Reduction Metadata

The Display-Power Reduction Metadata is defined in ISO/IEC 23001-11 -  Energy-Efficient Media Consumption (Green Metadata). The Display Power Reduction Metadata provides frame statistics and quality indicators for the media track that the metadata track refers to by means of ‘cdsc’ reference. This Metadata allows the client to attain a specified quality level by scaling frame-buffer pixels  and to reduce power correspondingly by decreasing  the  display backlight or OLED voltage.

Display-Power Reduction Metadata is of two types:

- metadata that indicates power saving at different quality levels over the sample duration. This metadata shall use the 'dipi’ (display power indication) sample entry type.

- metadata that allows fine control of the display to achieve power reduction at a specified quality level.  This metadata shall use the ’dfce’ (display fine control) sample entry type.

Static metadata for the display fine control is stored in the sample entry. Dynamic metadata is stored in the samples.

### Display Power Indication Metadata

#### Definition

Sample Entry Type:	‘dipi’ 
Container:	Sample Description Box (‘stsd’)
Mandatory:	No
Quantity:	0 or 1

This metadata indicates potential power saving at different quality levels over the sample duration.

#### Syntax

Display Power Indication Metadata shall use the following sample entry:

aligned(8) class DisplayPowerIndicationMetaDataSampleEntry() extends MetaDataSampleEntry (‘dipi‘) {
}

The Display Power Indication sample shall use the following syntax :

class QualityLevels (num_quality_levels) {
	unsigned int(8) rgb_component_for_infinite_psnr;
	for (i = 1; i <= num_quality_levels; i++) {
		unsigned int(8) max_rgb_component; 
		unsigned int(8) scaled_psnr_rgb;
	} 
}

aligned class DisplayPowerIndicationMetaDataSample () {
	unsigned int(4) num_quality_levels;
	unsigned int(4) reserved=0;}
	QualityLevels(num_quality_levels)
}

Please note that the PSNR variables appearing in the syntax presented above are as defined in ISO/IEC 23001-11 and should not be confused with the PSNR metric defined in clause 4.2 of this specification.

#### Semantics

Semantics are defined in ISO/IEC 23001-11.

### Display Fine Control Metadata

#### Definition

Sample Entry Type:	‘dfce’
Container:	Sample Description Box (‘stsd’)
Mandatory:	No
Quantity:	0 or 1

The Display Fine Control Dynamic Metadata is stored in the samples and is associated with one or more video frames.

The Decoding Time to Sample box provides the decoding time for the sample so that the metadata contained therein is made available to the display with sufficient lead time relative to the video composition time. Note that the video composition time and metadata composition time are identical. The lead time is required because display settings must be adjusted in advance of presentation time for correct operation. If num_constant_backlight_voltage_time_intervals > 1, then the lead time should be larger than the largest constant_backlight_voltage_time_interval.

#### Syntax

The Display Fine Control Metadata sample entry shall store static metadata as follows.

class DisplayFineControlMetaDataSampleEntry() 
	extends MetaDataSampleEntry (‘dfce‘) {
 DisplayFineControlConfigurationBox();
}

aligned(8) class DisplayFineControlConfigurationBox 
	extends FullBox(‘dfcC’, version = 0, flags = 0) {
	unsigned int(2) num_constant_backlight_voltage_time_intervals;
	unsigned int(6) reserved = 0;   
	unsigned int(16)constant_backlight_voltage_time_interval[ 
							num_constant_backlight_voltage_time_intervals ];
	unsigned int(2) num_max_variations;
	unsigned int(6) reserved = 0;
	unsigned int(16) max_variation[ num_max_variations ];
}

The Display Fine Control Metadata sample shall use the following syntax:.

class QualityLevels (num_quality_levels) {
	unsigned int(8) rgb_component_for_infinite_psnr;
	for (i = 1; i <= num_quality_levels; i++) {
		unsigned int(8) max_rgb_component; 
		unsigned int(8) scaled_psnr_rgb;
	} 
}

class MetadataSet (num_quality_levels) {
	unsigned int(8) lower_bound; 
	if (lower_bound > 0)
		unsigned int(8) upper_bound;
	QualityLevels(num_quality_levels);
}

class DisplayPowerReductionMetaDataSample {
	unsigned int(4) num_quality_levels;
	unsigned int(4) reserved = 0;

	for (k=0; k<num_constant_backlight_voltage_time_intervals; k++) 
		for (j = 0; j < num_max_variations; j++) 
			MetadataSet(num_quality_levels);
}

### 5.3.2.3	   Semantics

Semantics are defined in ISO/IEC 23001-11.

Bibliography

- Z. Wang, A. C. Bovik, H. R. Sheikh and E. P. Simoncelli, "Image quality assessment: From error visibility to structural similarity," IEEE Transactions on Image Processing, vol. 13, no. 4, pp. 600-612, Apr. 2004.

- Z. Wang, E. P. Simoncelli, and A. C. Bovik. "Multiscale structural similarity for image quality assessment." Signals, Systems and Computers, 2003. Conference Record of the Thirty-Seventh Asilomar Conference on. Vol. 2. IEEE, 2003.

- K. Seshadrinathana, R. Soundararajanb, A. C. Bovik and L. K. Cormack, “A Subjective Study to Evaluate Video Quality Assessment Algorithms”, IS&T/SPIE Electronic Imaging. International Society for Optics and Photonics, 2010.

- Z. Li, G. Schuster, A. K. Katsaggelos, and B. Gandhi, “Rate-Distortion Optimal Video Summary Generation”. IEEE Transactions on Image Processing 14(10): 1550-1560 (2005)

- Z. Li, and I. Bouazizi, “Light Weight Content Fingerprinting for Video Playback Verification in MPEG DASH", Proceedings of IEEE PacketVideo, San Jose, 2013.”

Appendix

Eigen Appearance Metric Matrix Specificiation

The Eigen appearance of the frame thumbnails are represented by d=12 coefficients projected by A=[a1T, a2T,..., a12T]  which are given below,

a1=[0.5637, 0.7948, 0.7851, 0.8250, 0.8072, 0.7905, 0.8612, 0.8782, 0.8421, 0.8776, 0.9065, 0.8902, 0.8844, 0.9003, 0.9013, 0.7772, 0.6060, 0.9059, 0.8250, 0.8210, 0.7832, 0.7921, 0.8573, 0.8400, 0.8286, 0.8665, 0.8988, 0.9216, 0.9188, 0.9442, 0.9504, 0.8561, 0.5635, 0.8915, 0.8870, 0.8120, 0.7668, 0.7445, 0.7463, 0.7154, 0.7147, 0.7776, 0.8056, 0.8347, 0.8719, 0.8924, 0.9227, 0.8306, 0.5855, 0.8835, 0.8648, 0.7803, 0.7410, 0.7072, 0.6743, 0.6260, 0.6196, 0.6963, 0.7460, 0.7772, 0.8069, 0.8672, 0.9117, 0.8450, 0.5869, 0.9038, 0.8820, 0.7710, 0.7229, 0.6707, 0.6302, 0.5741, 0.5419, 0.6277, 0.7011, 0.7171, 0.7424, 0.8312, 0.8934, 0.8359, 0.5652, 0.8669, 0.8668, 0.7933, 0.6967, 0.6532, 0.6198, 0.5512, 0.5139, 0.5881, 0.6448, 0.6966, 0.7656, 0.8375, 0.8937, 0.8190, 0.5560, 0.8586, 0.8606, 0.8189, 0.7268, 0.7117, 0.6671, 0.5707, 0.5779, 0.6299, 0.6975, 0.7464, 0.7726, 0.8260, 0.8890, 0.8162, 0.5704, 0.8732, 0.8580, 0.8378, 0.7418, 0.7039, 0.6949, 0.6145, 0.6218, 0.6881, 0.7322, 0.7619, 0.8090, 0.8624, 0.8842, 0.8085, 0.5777, 0.8767, 0.8470, 0.7863, 0.6685, 0.6828, 0.7224, 0.6064, 0.6112, 0.6958, 0.7424, 0.7355, 0.7861, 0.8603, 0.8939, 0.8116, 0.5465, 0.8291, 0.7953, 0.7397, 0.6258, 0.6443, 0.6623, 0.6213, 0.6356, 0.6695, 0.7067, 0.7229, 0.7111, 0.7437, 0.7721, 0.7057, 0.3266, 0.4746, 0.4481, 0.4492, 0.4418, 0.4172, 0.4059, 0.4002, 0.4049, 0.4134, 0.4216, 0.4405, 0.4335, 0.4101, 0.4274, 0.4258, 0.2554, 0.4131, 0.4401, 0.4505, 0.4508, 0.4424, 0.4269, 0.4086, 0.3949, 0.4015, 0.4035, 0.4151, 0.4314, 0.4408, 0.4476, 0.4346, ]

a2=[-0.5756, -1.0646, -0.9233, -0.8619, -0.9356, -1.0360, -0.9027, -0.9101, -0.9893, -0.9654, -0.9877, -1.0265, -1.0232, -1.0157, -1.0630, -1.0707, -0.6478, -1.0293, -0.7629, -0.7996, -0.8736, -0.7677, -0.6384, -0.6009, -0.5471, -0.6105, -0.8498, -0.8735, -0.8259, -1.0498, -1.1239, -0.9199, -0.6718, -0.9221, -0.9231, -0.8848, -0.8378, -0.8236, -0.5308, -0.3908, -0.3809, -0.4991, -0.6460, -0.7472, -0.8497, -0.9599, -1.0608, -1.0007, -0.3817, -0.5613, -0.6587, -0.6254, -0.5516, -0.4667, -0.2410, 0.0523, 0.1388, -0.1232, -0.3148, -0.3745, -0.5009, -0.6176, -0.7143, -0.7899, -0.2491, -0.3857, -0.2864, -0.2468, -0.2694, -0.1260, 0.1639, 0.4451, 0.3777, 0.2464, 0.0579, -0.0288, -0.1865, -0.4067, -0.4514, -0.4410, -0.1546, -0.1912, -0.0185, 0.0243, -0.0005, 0.2102, 0.5496, 0.7468, 0.7138, 0.5656, 0.3856, 0.2989, 0.1429, -0.1112, -0.1879, -0.2278, -0.0253, 0.0340, 0.0832, 0.1855, 0.2470, 0.4501, 0.7395, 0.9126, 0.9984, 0.8508, 0.6905, 0.6608, 0.5784, 0.2831, 0.0342, -0.0803, 0.0451, 0.1190, 0.2331, 0.4031, 0.5569, 0.8529, 0.9957, 1.0738, 1.1815, 1.0830, 1.1711, 1.0447, 0.7653, 0.4478, 0.1839, -0.0015, 0.1547, 0.3178, 0.4590, 0.7580, 0.8777, 1.1366, 1.1179, 1.3218, 1.4127, 1.2906, 1.4067, 1.3865, 1.1298, 0.6237, 0.3385, 0.1060, 0.2214, 0.4425, 0.5939, 0.8598, 0.8869, 1.1357, 1.1749, 1.2390, 1.2628, 1.3530, 1.3616, 1.3106, 1.0130, 0.5601, 0.3505, 0.0774, 0.3577, 0.5456, 0.5571, 0.5963, 0.6186, 0.6724, 0.7344, 0.7861, 0.8133, 0.8001, 0.8130, 0.7817, 0.6309, 0.4718, 0.4378, 0.2889, 0.2815, 0.5002, 0.5685, 0.5778, 0.5900, 0.6558, 0.6966, 0.6907, 0.7061, 0.7245, 0.7147, 0.6840, 0.6840, 0.6203, 0.5461, 0.4408, ]

a3=[0.5648, 0.7600, 0.8563, 0.9031, 0.8191, 0.7165, 0.6719, 0.6450, 0.5955, 0.6126, 0.6071, 0.6309, 0.6775, 0.7781, 0.8141, 0.6759, 0.3599, 0.4982, 0.4552, 0.4951, 0.3171, 0.2674, 0.1063, 0.0186, 0.0923, 0.1300, 0.1035, 0.1323, 0.1899, 0.2728, 0.3836, 0.3980, 0.1301, 0.1975, 0.1097, -0.0181, -0.1734, -0.3060, -0.4360, -0.4738, -0.3966, -0.3954, -0.4249, -0.4023, -0.2870, -0.1666, 0.0133, 0.1076, 0.1266, 0.0807, -0.1447, -0.3595, -0.5090, -0.6381, -0.7470, -0.7926, -0.7205, -0.7828, -0.7628, -0.6909, -0.5583, -0.3542, -0.1663, -0.0373, 0.0896, -0.0474, -0.2496, -0.5138, -0.6984, -0.8690, -0.9373, -0.9443, -0.9432, -0.9447, -0.8760, -0.7818, -0.6323, -0.4924, -0.2839, -0.0675, 0.0293, -0.1111, -0.3108, -0.5481, -0.7948, -0.8955, -0.9183, -0.8945, -0.9086, -0.8833, -0.9025, -0.7312, -0.4903, -0.4341, -0.2425, -0.0969, 0.0284, -0.1241, -0.3046, -0.5414, -0.7573, -0.8292, -0.8327, -0.8282, -0.8154, -0.7873, -0.7427, -0.6170, -0.4281, -0.3577, -0.2039, -0.0944, 0.0467, -0.1194, -0.2596, -0.4854, -0.6902, -0.7419, -0.7536, -0.6805, -0.6901, -0.6112, -0.5578, -0.4921, -0.3053, -0.2552, -0.1521, -0.0758, 0.0993, 0.0416, -0.0962, -0.1834, -0.4520, -0.4455, -0.4053, -0.4207, -0.3980, -0.2696, -0.1907, -0.1820, -0.0736, -0.0213, 0.1255, 0.1128, 0.2596, 0.3366, 0.2719, 0.2262, -0.0219, -0.0073, 0.0071, -0.0083, 0.0513, 0.1561, 0.1810, 0.2073, 0.0495, 0.0610, 0.2283, -0.0693, 0.9505, 1.4210, 1.2049, 1.3180, 1.1651, 1.1159, 1.2055, 1.3815, 1.4471, 1.3611, 1.3823, 1.3148, 0.8823, 0.8701, 1.1247, 0.8313, 0.7880, 1.4176, 1.5653, 1.6111, 1.5736, 1.5504, 1.4922, 1.4488, 1.4256, 1.4452, 1.4246, 1.4850, 1.6211, 1.6520, 1.6749, 1.5319, ]

a4=[-0.3219, -0.5448, -0.3802, -0.2359, -0.1222, -0.0722, 0.1741, 0.4518, 0.6024, 0.7251, 0.7909, 0.8523, 0.8549, 0.8576, 0.7711, 0.5337, -0.5045, -0.6312, -0.5025, -0.4701, -0.3729, -0.1494, 0.2011, 0.6480, 0.9276, 1.0588, 1.0479, 1.0712, 1.0461, 0.9407, 0.7784, 0.5670, -0.7294, -0.9243, -0.8555, -0.7489, -0.5789, -0.4866, -0.0283, 0.5280, 0.8639, 0.9821, 0.9724, 1.0023, 0.8628, 0.8779, 0.7244, 0.4405, -0.7689, -1.1521, -1.2061, -1.2029, -1.0516, -0.7101, -0.1663, 0.4497, 0.8648, 0.9160, 0.7465, 0.7429, 0.6858, 0.6818, 0.6124, 0.3791, -0.8287, -1.3496, -1.3605, -1.4097, -1.2410, -0.9038, -0.2503, 0.4094, 0.7123, 0.7625, 0.6072, 0.6288, 0.5672, 0.5135, 0.5252, 0.2882, -0.8915, -1.4469, -1.5475, -1.5558, -1.2460, -0.8969, -0.2977, 0.4134, 0.6368, 0.6457, 0.4748, 0.4710, 0.4361, 0.4520, 0.4621, 0.2800, -0.8688, -1.4809, -1.5601, -1.4872, -1.3074, -0.8680, -0.3227, 0.3598, 0.6564, 0.6422, 0.5579, 0.4727, 0.4700, 0.4317, 0.3347, 0.2160, -0.8912, -1.4591, -1.3872, -1.3350, -1.1825, -0.6633, -0.2627, 0.4415, 0.7647, 0.8125, 0.7572, 0.6460, 0.5018, 0.4458, 0.3506, 0.1541, -0.8171, -1.2882, -1.1400, -1.0879, -1.0665, -0.5948, -0.1773, 0.5371, 0.7468, 0.8068, 0.7297, 0.6680, 0.6499, 0.5184, 0.4084, 0.2206, -0.6806, -1.0200, -0.9581, -0.8573, -0.9363, -0.5283, -0.1372, 0.3475, 0.5521, 0.6599, 0.6213, 0.5790, 0.4338, 0.3365, 0.2859, 0.1450, -0.2720, -0.4064, -0.4426, -0.3869, -0.3123, -0.1789, -0.0084, 0.1818, 0.2625, 0.1972, 0.2284, 0.1935, 0.1440, 0.1620, 0.1808, 0.1678, -0.1584, -0.2608, -0.2844, -0.2621, -0.2342, -0.1067, -0.0146, 0.1041, 0.1628, 0.1596, 0.1810, 0.1820, 0.1499, 0.1561, 0.1718, 0.1857, ]

a5=[0.1859, 0.4072, 0.5816, 0.6679, 0.7591, 0.8703, 0.8424, 0.9086, 0.8627, 0.6349, 0.4223, 0.2784, 0.0025, -0.2496, -0.3594, -0.3394, 0.0711, 0.2673, 0.4983, 0.6951, 0.7998, 0.9441, 1.1553, 1.3430, 1.2532, 0.8444, 0.4754, 0.1777, -0.1411, -0.4716, -0.6784, -0.6633, -0.0185, 0.0939, 0.2437, 0.5376, 0.6944, 0.8918, 1.3037, 1.5344, 1.3682, 0.9443, 0.5313, 0.1532, -0.3665, -0.6894, -0.8844, -0.8464, -0.0703, -0.0512, 0.1407, 0.4230, 0.5549, 0.8284, 1.2095, 1.4465, 1.3319, 0.8748, 0.3293, -0.1322, -0.5401, -0.8751, -1.0947, -1.0007, -0.1869, -0.1897, -0.0172, 0.1888, 0.3825, 0.6500, 0.9997, 1.2788, 1.1394, 0.7368, 0.1695, -0.2893, -0.7222, -1.0735, -1.2096, -1.1032, -0.2160, -0.2772, -0.1169, 0.1319, 0.3788, 0.6188, 0.9739, 1.1782, 1.0774, 0.6549, 0.0652, -0.3578, -0.9527, -1.2762, -1.3800, -1.1918, -0.3011, -0.4699, -0.3895, -0.1623, 0.0840, 0.4106, 0.7433, 1.0387, 0.8735, 0.5063, -0.0442, -0.5176, -1.1094, -1.4097, -1.5377, -1.3556, -0.2936, -0.4857, -0.5187, -0.3985, -0.0819, 0.3568, 0.5850, 0.9013, 0.7937, 0.4551, -0.0683, -0.5474, -1.2100, -1.5726, -1.5984, -1.3971, -0.3688, -0.5594, -0.6053, -0.3851, -0.0473, 0.2039, 0.4159, 0.9078, 0.7950, 0.4801, -0.0421, -0.5705, -1.0916, -1.4884, -1.5673, -1.3253, -0.3600, -0.5320, -0.4767, -0.2918, -0.0796, 0.0898, 0.2978, 0.5969, 0.5725, 0.2930, -0.0882, -0.5128, -0.8306, -1.1398, -1.2124, -1.1170, 0.1292, 0.2388, 0.2656, 0.3266, 0.3276, 0.4021, 0.4668, 0.5768, 0.6298, 0.4766, 0.3377, 0.1444, -0.1450, -0.2400, -0.2435, -0.2886, 0.1287, 0.2589, 0.2862, 0.3128, 0.3583, 0.4022, 0.4016, 0.4155, 0.4844, 0.4164, 0.3044, 0.1856, 0.0947, -0.0079, -0.0536, -0.0908, ]

a6=[-0.7194, -0.4715, -0.7272, -0.9629, -0.7461, -0.1521, -0.5896, -0.5612, -0.2903, -0.4072, -0.4319, -0.3181, -0.2711, -0.6179, -0.7144, -0.3828, -0.6709, -0.9930, -1.0732, -0.7216, 0.1131, -0.0716, -0.4820, -0.4404, -0.5155, -0.3290, 0.0229, 0.1404, 0.0375, -0.0238, -0.2903, -0.8116, -0.2613, -0.8096, -0.7682, 0.0195, 0.5406, 0.9624, 0.5375, 0.0701, -0.1474, 0.1679, 0.6951, 0.9845, 1.0762, 0.7853, 0.2122, -0.2492, -0.7878, -0.9770, -0.3517, 0.5775, 1.2240, 1.2668, 0.8158, -0.1239, -0.6690, 0.1445, 1.0406, 1.3906, 1.5137, 1.0426, 0.4627, -0.0943, -0.7428, -0.8229, -0.4695, 0.7103, 1.4991, 1.4876, 0.7914, -0.3027, -0.3401, 0.1066, 0.9910, 1.3406, 1.6238, 1.4596, 0.6709, -0.1845, -0.5544, -0.7040, -0.5584, 0.5858, 1.4535, 1.4742, 0.6370, -0.6146, -0.5114, -0.0141, 0.8636, 1.3228, 1.4585, 1.2542, 0.3781, -0.3323, -0.6216, -0.8237, -0.4074, 0.4452, 1.2165, 1.1301, 0.4547, -0.5982, -0.9568, -0.3663, 0.4409, 0.7972, 0.6134, 0.3964, -0.0163, -0.4626, -0.6772, -0.8880, -0.5902, 0.0829, 0.8858, 0.4859, 0.0236, -0.7630, -1.0772, -0.8204, -0.4066, -0.0506, 0.1601, -0.0707, -0.3966, -0.5729, -0.7167, -0.9187, -0.6586, -0.2368, 0.6296, 0.0199, -0.0674, -1.0698, -1.2723, -1.0321, -0.8801, -0.7881, -0.4967, -0.3480, -0.5715, -0.6995, -0.5697, -0.7192, -0.5182, -0.3777, 0.5446, -0.2603, -0.4018, -1.0470, -1.0286, -1.0956, -0.9298, -0.7775, -0.5165, -0.3245, -0.3072, -0.4697, 0.2497, 0.4851, 0.5269, 0.6662, 0.6576, 0.7075, 0.7374, 0.8152, 0.7692, 0.7291, 0.7373, 0.7418, 0.5015, 0.3977, 0.4522, 0.2764, 0.2498, 0.5029, 0.6231, 0.6855, 0.7073, 0.7604, 0.7586, 0.7461, 0.7066, 0.7359, 0.7260, 0.7325, 0.7880, 0.7463, 0.6812, 0.5767, ]

a7=[0.0752, 0.4395, 0.6916, 0.6249, 0.5661, 0.8660, 0.6026, 0.2795, 0.5185, 0.5845, 0.4984, 0.7976, 0.9451, 0.3942, 0.0350, 0.2336, 0.2281, -0.0007, 0.7414, 0.6512, 0.7838, 1.0602, 0.6058, 0.0321, -0.1176, 0.3859, 0.4533, 0.9117, 1.2927, 0.5370, 0.0583, 0.2235, 0.2819, -0.1763, -0.4460, 0.0081, 0.2558, 0.3407, -0.2283, -0.7561, -0.8562, -0.5790, 0.0612, 0.4058, 0.5941, 0.2971, -0.0962, -0.0889, -0.0710, -0.4148, -0.6847, -0.4082, 0.1997, 0.1048, -0.5829, -1.3067, -1.1031, -0.8331, -0.1831, 0.2613, 0.3140, 0.2719, -0.0028, -0.2534, -0.4655, -0.8629, -0.8156, -0.1053, -0.0321, -0.0415, -0.5673, -1.2229, -1.0824, -0.9553, -0.3920, -0.1249, 0.0390, 0.0390, -0.2569, -0.4480, -0.6230, -1.2019, -1.0275, -0.3026, -0.0262, 0.1640, -0.0070, -0.9961, -0.9758, -1.0100, -0.4666, -0.5187, -0.1478, -0.2895, -0.6673, -0.7710, -0.6415, -1.2084, -1.0530, -0.4991, 0.2703, 0.3613, 0.0279, -0.7055, -0.9337, -1.0447, -0.6224, -0.2604, 0.2892, -0.1608, -0.9686, -0.9988, -0.5326, -0.8518, -0.6159, 0.1595, 1.6471, 1.7910, 0.8935, -0.2677, -0.4780, -0.7239, 0.5015, 0.4346, 0.0673, -0.3820, -0.8202, -0.8957, -0.3870, -0.4863, -0.0963, 1.0039, 2.9513, 2.6383, 1.2535, 0.1905, 0.0316, -0.2575, 0.8779, 1.2477, 0.7968, -0.1626, -0.5577, -0.6075, -0.2436, -0.2510, 0.1098, 1.0255, 2.5866, 2.1426, 1.5156, 0.4908, 0.1695, 0.5984, 1.0025, 1.2373, 1.0506, 0.1903, -0.2215, -0.1424, -0.4280, -0.5580, -0.2851, -0.1862, 0.0634, -0.0358, -0.1825, -0.5369, -0.6477, -0.4850, -0.4058, -0.3161, -0.0009, -0.1310, -0.3981, -0.2741, -0.3061, -0.5374, -0.4938, -0.3342, -0.2447, -0.2900, -0.3837, -0.5468, -0.6007, -0.5493, -0.4856, -0.4705, -0.5212, -0.5551, -0.6378, -0.6368, ]

a8=[-0.4905, -0.8020, -0.9043, -0.9593, -0.9432, -0.9492, -0.6606, -0.2933, -0.0413, 0.3316, 0.5777, 0.6586, 0.8416, 0.9082, 0.8424, 0.6541, -0.4804, -0.7824, -1.0250, -1.2539, -1.2678, -1.2763, -0.8131, -0.3034, 0.1754, 0.6316, 0.8838, 1.0441, 1.2110, 1.1805, 1.0538, 0.8109, -0.4412, -0.7308, -1.0063, -1.2577, -1.3689, -1.3456, -0.7615, -0.2148, 0.3419, 0.7851, 0.9577, 1.0681, 1.2745, 1.1424, 0.9179, 0.6563, -0.2456, -0.3915, -0.6850, -1.0140, -1.2010, -1.0808, -0.6251, -0.2143, 0.3003, 0.7813, 0.7560, 0.7468, 0.8863, 0.6767, 0.4433, 0.3319, 0.0519, 0.1036, -0.1551, -0.4765, -0.7067, -0.7035, -0.4180, -0.2497, 0.1659, 0.5185, 0.4926, 0.3084, 0.2715, 0.0522, -0.0881, -0.0524, 0.4102, 0.6774, 0.4973, 0.0818, -0.2781, -0.3543, -0.2487, -0.1479, 0.0760, 0.2836, 0.1474, -0.0865, -0.2853, -0.4394, -0.5042, -0.2701, 0.6156, 1.0868, 0.9520, 0.5320, 0.0630, -0.1162, -0.0391, -0.0810, 0.0478, 0.2209, -0.0818, -0.4666, -0.5971, -0.8162, -0.8209, -0.4986, 0.6954, 1.2892, 1.4224, 1.0332, 0.3007, 0.0294, -0.0279, 0.0736, 0.0351, 0.0965, -0.4534, -0.8633, -1.0510, -1.2244, -1.1819, -0.9043, 0.7878, 1.5444, 1.6431, 1.2457, 0.5679, 0.3712, 0.2574, 0.3276, 0.3335, 0.1279, -0.4893, -0.9503, -1.2925, -1.3252, -1.2609, -1.0925, 0.8283, 1.4969, 1.5116, 1.1517, 0.7043, 0.4008, 0.2657, 0.2859, 0.3032, -0.0321, -0.3825, -0.7662, -1.1380, -1.1313, -1.0792, -1.0062, 0.4437, 0.7177, 0.6173, 0.6208, 0.5315, 0.3154, 0.1841, 0.1648, 0.0564, -0.1115, -0.2755, -0.4837, -0.7261, -0.6473, -0.5969, -0.5582, 0.2771, 0.5343, 0.5388, 0.5176, 0.4478, 0.3042, 0.1804, 0.0419, 0.0043, -0.0878, -0.1830, -0.2964, -0.3604, -0.4253, -0.4268, -0.3544, ]

a9=[0.1054, 0.2374, 0.1790, 0.1926, 0.4899, 0.7322, 0.5694, 0.6305, 0.6133, 0.3206, 0.3561, 0.4832, 0.6657, 1.0667, 1.4294, 1.4766, -0.2834, -0.5986, -0.8676, -0.7773, -0.3598, -0.0782, -0.1100, -0.1604, -0.3374, -0.8331, -0.8571, -0.7951, -0.5456, 0.2471, 0.9221, 1.0321, -0.3449, -0.8452, -1.3220, -1.0818, -0.6670, -0.2072, 0.0582, -0.0047, -0.3417, -0.9908, -1.2119, -1.2474, -0.8944, -0.0989, 0.6383, 0.9634, -0.4943, -1.0104, -1.2422, -0.8619, -0.1768, 0.3968, 0.6818, 0.5258, 0.1047, -0.5912, -1.0178, -1.1926, -1.0489, -0.2214, 0.6116, 1.0439, -0.4942, -0.9483, -1.0813, -0.4276, 0.3704, 0.8392, 1.1862, 0.9553, 0.4607, -0.3476, -0.8534, -0.9826, -0.8506, 0.0600, 0.8893, 1.1454, -0.5806, -0.9316, -0.7619, 0.0307, 0.8568, 1.2561, 1.4637, 1.2271, 0.6272, -0.2574, -0.7393, -0.8778, -0.5760, 0.2177, 1.0676, 1.3041, -0.4933, -0.7530, -0.5132, 0.1534, 1.0091, 1.4761, 1.4934, 1.3794, 0.6309, -0.3561, -0.8695, -0.9487, -0.6081, 0.0823, 0.9779, 1.1938, -0.3303, -0.4857, -0.3242, 0.1518, 0.8898, 1.0462, 1.0798, 1.2442, 0.5158, -0.5930, -1.3012, -1.1816, -0.7880, 0.0214, 0.8559, 1.1877, -0.1251, -0.1572, 0.0197, -0.0026, 0.5044, 0.5327, 0.6817, 0.8623, 0.2480, -0.5766, -1.3607, -1.3400, -0.9277, -0.0355, 0.7284, 1.1576, 0.0153, 0.0104, 0.0419, 0.0454, 0.3720, 0.3439, 0.3872, 0.5271, 0.0197, -0.6788, -1.3305, -1.1802, -0.7962, -0.1295, 0.4983, 1.0139, 0.0878, 0.2494, 0.2640, 0.1991, 0.0769, -0.0089, -0.0374, -0.0051, -0.1305, -0.3466, -0.4483, -0.3659, -0.2421, 0.1046, 0.3323, 0.5832, 0.1272, 0.3060, 0.2907, 0.1963, 0.0826, -0.0241, -0.0501, -0.0402, -0.1309, -0.2505, -0.3161, -0.2992, -0.1792, 0.0317, 0.2103, 0.3766, ]

a10=[-0.8675, -1.4813, -1.2994, -1.2090, -1.1703, -1.2475, -1.1761, -1.0076, -1.1339, -1.2177, -1.2378, -1.3347, -1.3455, -0.9896, -0.7908, -0.7911, -0.4340, -0.5323, -0.1031, -0.0748, 0.0292, 0.4195, 0.7020, 0.7485, 0.6630, 0.3729, 0.0156, -0.1388, -0.1940, 0.0874, 0.2300, 0.1441, -0.0976, 0.2448, 0.5280, 0.7448, 1.0396, 1.1813, 1.4302, 1.5555, 1.4774, 1.0868, 0.5713, 0.5619, 0.4295, 0.8047, 1.0933, 0.7581, 0.0664, 0.5178, 0.5718, 0.6292, 0.7844, 0.9957, 1.1763, 1.2985, 1.2107, 0.7044, 0.2039, 0.3586, 0.3368, 0.9792, 1.3847, 0.9344, -0.0764, 0.0533, 0.2442, 0.1684, 0.0427, 0.2256, 0.3489, 0.4732, 0.3294, -0.1926, -0.7641, -0.7318, -0.4801, 0.2691, 0.9158, 0.6405, -0.3482, -0.4321, -0.2703, -0.4482, -0.6881, -0.6282, -0.3873, -0.1567, -0.4424, -1.0440, -1.6078, -1.7523, -1.3270, -0.3075, 0.5481, 0.4325, -0.3146, -0.4191, -0.4574, -0.7840, -1.0580, -1.0422, -0.8617, -0.6006, -0.6582, -1.3769, -1.8606, -1.9145, -1.4068, -0.4306, 0.3158, 0.3899, -0.1966, -0.2246, -0.1105, -0.3844, -0.4850, -0.5215, -0.6410, -0.4054, -0.4242, -0.9252, -1.1062, -1.1171, -0.9891, -0.2738, 0.3251, 0.4296, -0.0018, 0.1065, 0.3110, 0.3014, 0.3741, 0.4820, 0.2570, 0.3666, 0.2951, -0.0994, -0.1277, -0.1605, -0.1220, 0.2109, 0.6353, 0.5707, 0.1922, 0.3760, 0.6756, 0.9431, 0.9953, 1.3191, 1.1495, 0.9425, 0.7351, 0.6317, 0.6335, 0.6818, 0.7490, 0.7054, 0.7305, 0.7905, -0.1064, -0.1030, 0.2062, 0.2007, 0.3836, 0.3861, 0.2100, -0.0177, -0.0779, 0.0451, 0.1235, 0.1616, 0.3834, 0.4060, 0.2628, 0.2740, -0.0958, -0.1814, -0.0881, -0.0230, 0.0145, 0.0543, 0.0466, -0.0091, -0.0113, 0.0471, 0.0724, 0.0832, 0.0593, 0.0643, 0.0281, -0.0529, ]

a11=[0.5306, 0.7791, 0.8079, 0.4511, -0.1270, -0.6237, -0.9982, -1.4228, -1.3378, -1.1278, -1.0397, -0.6396, -0.1049, 0.1887, 0.3456, 0.4963, 0.8005, 1.0401, 1.2378, 0.5984, -0.2526, -0.4652, -1.2576, -1.7654, -1.6622, -1.3145, -1.1615, -0.3987, 0.4407, 0.6145, 0.8862, 1.1057, 0.9597, 1.2825, 1.0189, 0.3527, -0.3649, -0.7007, -1.2405, -1.4252, -1.1944, -1.0313, -0.8050, -0.1234, 0.6102, 1.0319, 1.3266, 1.2134, 0.9845, 1.3666, 1.2172, 0.5448, -0.0141, -0.3037, -0.6423, -0.5825, -0.0763, -0.2302, -0.1266, 0.2754, 0.7432, 1.1854, 1.4288, 1.1748, 0.7986, 1.0370, 1.1112, 0.8002, 0.0223, -0.0136, -0.0274, 0.3520, 0.6948, 0.5437, 0.1412, 0.2390, 0.4740, 0.8815, 1.1632, 0.9130, 0.4504, 0.3389, 0.3424, 0.1668, -0.3005, -0.0188, 0.4210, 0.7093, 0.9086, 0.7093, 0.1245, -0.2633, -0.1359, 0.0540, 0.3357, 0.4938, 0.0631, -0.3506, -0.5891, -0.7719, -0.6882, -0.2917, 0.1835, 0.5724, 0.8964, 0.5074, -0.1193, -0.6573, -0.4132, -0.3349, -0.3792, -0.0262, -0.3290, -0.9253, -1.1023, -0.9759, -0.2735, 0.4896, 0.6352, 0.7030, 0.8748, 0.4024, 0.4322, -0.3361, -0.8662, -0.9601, -0.8794, -0.5782, -0.7094, -1.4198, -1.4947, -0.8409, 0.2206, 0.8006, 0.5186, 0.8210, 0.9033, 0.1386, 0.3243, 0.1339, -0.4981, -1.1717, -1.1585, -0.8661, -0.6319, -1.2368, -1.4548, -0.7907, 0.0496, 0.4785, 0.4896, 0.4171, 0.3630, 0.3564, 0.3078, 0.0471, -0.3561, -1.0486, -1.0327, -0.8120, -0.0241, -0.1420, -0.2087, -0.0504, 0.1819, 0.3071, 0.4231, 0.4939, 0.4894, 0.4295, 0.3394, 0.1386, -0.0190, -0.1613, -0.2284, -0.2882, -0.0873, -0.2116, -0.1980, 0.0518, 0.2979, 0.3726, 0.4088, 0.4233, 0.4360, 0.4042, 0.2927, 0.1791, 0.1173, -0.0096, -0.1121, -0.1084, ]

a12=[-1.1662, -2.1102, -1.5183, -0.8441, -0.6002, -0.5878, 0.0778, 0.1599, -0.3738, -0.3116, -0.1078, -0.0023, 0.1544, 0.7800, 0.7986, 0.1853, -1.0356, -1.1459, -0.5195, 0.0331, 0.2935, 0.8637, 1.2530, 0.9219, 0.7025, 0.3962, 0.2174, 0.6634, 1.0727, 1.5577, 1.5478, 1.2398, -1.2948, -0.9833, 0.0597, 0.4044, 0.7628, 0.8530, 0.8137, 0.2379, -0.2537, -0.5937, -0.8086, -0.3194, 0.3764, 0.8628, 1.0108, 0.7125, -0.6842, -0.6155, 0.1122, 0.4775, 0.3199, 0.0952, -0.0499, -0.3394, -0.8471, -1.3949, -1.7744, -1.2045, -0.3142, 0.4722, 0.4695, 0.3609, -0.2882, -0.0921, 0.6514, 0.7452, 0.5479, 0.1307, -0.2636, -0.3517, -1.1480, -1.6665, -2.0144, -1.5340, -0.8093, -0.2144, 0.0729, 0.0842, -0.0679, 0.4774, 1.2320, 1.1714, 0.7244, 0.2584, -0.1505, 0.0290, -0.5124, -1.0709, -1.2214, -0.8636, -0.5174, -0.3327, -0.1487, -0.4102, 0.1020, 0.7681, 1.3851, 1.5912, 1.1300, 0.7245, 0.5439, 0.3200, 0.3971, 0.2017, 0.0947, 0.2007, 0.0908, -0.1760, -0.0334, -0.3017, -0.0710, 0.1257, 0.6374, 0.6529, 0.1167, 0.3148, 0.5375, 0.2650, 0.4847, 0.7191, 0.9222, 1.0331, 0.7745, 0.2176, -0.2279, -0.6252, -0.6244, -0.6981, -0.4543, -0.3109, -1.1935, -0.5494, -0.0077, -0.1774, 0.1164, 0.5742, 0.9511, 1.0279, 0.7421, 0.1932, -0.3411, -0.9109, -0.8500, -1.1403, -0.9698, -0.8739, -1.7214, -0.8436, -0.4766, -0.2900, 0.1332, 0.4804, 0.7221, 0.5334, -0.0993, -0.5449, -0.8084, -1.5456, -0.1162, -0.2135, -0.4921, -0.2128, -0.2578, -0.2106, -0.0104, 0.4360, 0.6740, 0.6708, 0.6353, 0.3133, -0.3828, -0.6155, -0.5904, -0.9195, -0.1018, -0.1806, -0.1773, -0.1717, -0.1541, -0.1106, 0.0172, 0.1264, 0.2511, 0.3586, 0.2998, 0.1507, 0.1821, 0.1217, 0.0120, -0.0223, ]

)	ISO/IEC 14496-12 is technically identical to ISO/IEC 15444-12.