| Radiocommunication Study Groups |  |
| --- | --- |
| 31st Meeting of Working Party 5D Fukuoka, Japan, 9 - 16 October 2018 |  |
| Attachment 7.4 to Document 5D/1110 (Source: Document 5D/TEMP/636(Rev.2)) |  |
|  | 15 October 2018 |
|  | English only |
| Working Party 5D |  |
| Liaison Statement to 3GPP |  |
| Characteristics of IMT-Advanced, IMT-2020 and Advanced Antenna Systems for ITU-r sharing and compatibility studies in the frequency band 3 300-3 400 MHz |  |

Further to the WRC-15 conclusion, Working Party 5D (WP 5D), as the responsible group for IMT in ITU-R, initiated the development of the working document towards preliminary draft new Report ITU-R [RADAR&IMT SHARING] on “In band and adjacent band coexistence and compatibility studies between IMT systems in 3 300-3 400 MHz and Radiolocation systems in 3 100-3 400 MHz”.

WP 5D has agreed to include IMT-2020 systems and Advanced Antenna Systems in the above mentioned report.

WP 5D had previously developed sharing parameters for IMT-Advanced technologies for bands between 3 and 6 GHz, which are contained in Report ITU-R M.2292 and summarized in Annex 1. In addition, the beamforming antenna characteristics of IMT-2020, which are summarized in Annex 2, were developed in WP 5D for sharing and compatibility studies under WRC-19 agenda item 1.13.

Due to the situation that, in addition to parameters of IMT-2020, the beamforming antenna characteristics of IMT-Advanced and IMT-2020 in the frequency band 3.3-3.4 GHz are not currently contained in ITU-R deliverables, WP 5D would ask for appropriate input from 3GPP (considering also Annexes 1 and 2) for consideration by WP 5D on:

- the parameters and antenna characteristics for IMT-2020 in the frequency band 3.3-3.4 GHz, as well as;

- the applicability of the beamforming antenna characteristics of IMT-2020 to IMT-Advanced in the frequency band 3.3-3.4 GHz.

WP 5D would appreciate to receive the above mentioned information preferably in time for its next meeting which is planned for 11th to 15th February 2019. The deadline for submission of input contributions for that meeting is 4th February 2019 (1600 hours UTC).

| Status: For action |  |
| --- | --- |
| Contact: Sergio Buonomo Counsellor ITU-R SG 5 | E-mail: sergio.buonomo@itu.int |

Annexes:	2

1)	IMT-Advanced technology-related and deployment-related parameters for bands between 3 and 6 GHz (Source: Report ITU-R M.2292)

2)	Antenna characteristics for IMT-2020 (Source: Document 5-1/36, Attachment 2, Table 10)

ANNEX 1

IMT-Advanced technology-related and deployment-related parameters 
for bands between 3 and 6 GHz

TABLE A1-1

IMT-Advanced specification related parameters

|  |  | IMT-Advanced |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Duplex mode | FDD |  | TDD |  |
| No. | Parameter | Base station | Mobile station | Base station | Mobile station |
| 1 | Access technique | OFDM | SC-FDMA | OFDM | SC-FDMA |
| 2 | Modulation parameters | QPSK 16-QAM 64-QAM |  |  |  |
| 3 | Channel spacing | Nominal channel spacing = (BWChannel(1) + BWChannel((2))/2 |  |  |  |
| 4 | Channel bandwidth (MHz) | (1) | (1), (19) | (1) | (1), (19) |
| 5 | Signal bandwidth (MHZ) | (1) | (1), (19) | (1) | (1), (19) |
| 6 | Transmitter characteristics |  |  |  |  |
| 6.1 | Power dynamic range (dB) | (2) | 63(18) | (2) | 63(18) |
| 6.2 | Spectral mask | (3), (17) | (4), (17) | (3), (17) | (4), (17) |
| 6.3 | ACLR | (5), (17) | (6), (17) | (5), (17) | (6), (17) |
| 6.4 | Maximum output power | (7) | (8) | (7) | (8) |
| 6.5 | Spurious emissions | (15), (17) | (16), (17) | (15), (17) | (16), (17) |

TABLE A1-1 (end)

|  |  | IMT-Advanced |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Duplex mode | FDD |  | TDD |  |
| No. | Parameter | Base station | Mobile station | Base station | Mobile station |
| 7 | Receiver characteristics |  |  |  |  |
| 7.1 | Noise figure | 5 dB (macro) 10 dB (micro) 13 dB (pico/femto) | 9 dB | 5 dB (macro) 10 dB (micro) 13 dB (pico/femto) | 9 dB |
| 7.2 | Sensitivity | (9) | (10) | (9) | (10) |
| 7.3 | Blocking response | (11) | (12) | (11) | (12) |
| 7.4 | ACS | (13) | (14) | (13) | (14) |
| Notes to the Table: (1) See 3GPP Document TS 36.101 v.11.2.0, § 5.6. Signal bandwidth in MHz corresponds to “Transmission bandwidth configuration*0.180”. (2) See 3GPP Document TS 36.104 v.11.2.0, § 6.3.2.1. (3) See 3GPP Document TS 36 104 v.11.2.0, § 6.6.3. (4) See 3GPP Document TS 36 101 v.11.2.0, §§ 6.6.2.1, 6.6.2.1A, 6.6.2.2 and 6.6.2.2A describe UE spectrum emissions masks for different channel bandwidths. In case multiple UEs are transmitting simultaneously on the same channel they will share the available radio resource blocks. As the actual transmission bandwidth is thus decreased the unwanted emissions performance might be improved. This may be taken into account during sharing analysis when measurements or detailed models are available. (5) See 3GPP Document TS 36.104 v.11.2.0, § 6.6.2. (6) See 3GPP Document TS 36.101 v.11.2.0, § 6.6.2.3. (7) See 3GPP Document TS 36.104 v.11.2.0, § 6.2. (8) See 3GPP Document TS 36.101 v.11.2.0, § 6.2. (9) See 3GPP Document TS 36.104 v.11.2.0, § 7.2. (10) See 3GPP Document TS 36.101 v.11.2.0, § 7.3. (11) See 3GPP Document TS 36.104 v.11.2.0, § 7.6. (12) See 3GPP Document TS 36.101 v.11.2.0, § 7.6 and § 7.7. (13) See 3GPP Document TS 36.104 v.11.2.0, § 7.5. (14) See 3GPP Document TS 36.101 v.11.2.0, § 7.5. (15) See 3GPP Document TS 36.104 v.11.2.0, § 6.6.4. (16) See 3GPP Document TS 36.101 v.11.2.0, § 6.6.3. (17) These unwanted emission limits are the upper limits from SDO specifications for laboratory testing with maximum transmitting power. It is assumed that when the in-band transmitting power is reduced by x dB through power control, the unwanted emission levels would be reduced by x dB in consequence in the coexistence simulations. (18) See 3GPP Document TS 36.101 v.11.2.0, § 6.3. (19) In case multiple UEs are transmitting simultaneously on the same channel they will share the available radio resource blocks. As the actual transmission bandwidth is thus decreased the unwanted emissions performance might be improved. This may be taken into account during sharing analysis when measurements or detailed models are available. |  |  |  |  |  |

TABLE A1-2

IMT-Advanced Deployment-related parameters for bands between 3 and 6 GHz

|  | Macro suburban | Macro urban | Small cell outdoor/ Micro urban | Small cell indoor/ Indoor urban |
| --- | --- | --- | --- | --- |
| Base station characteristics/Cell structure |  |  |  |  |
| Cell radius/Deployment density | 0.3-2 km (typical figure to be used in sharing studies 0.6 km) | 0.15-0.6 km (typical figure to be used in sharing studies 0.3 km) | 1-3 per urban macro cell <1 per suburban macro site | Depending on indoor coverage/capacity demand |
| Antenna height | 25 m | 20 m | 6 m | 3 m |
| Sectorization | 3 sectors | 3 sectors | Single sector | Single sector |
| Downtilt | 6 degrees | 10 degrees | n.a. | n.a. |
| Frequency reuse | 1 | 1 | 1 | 1 |
| Antenna pattern | Recommendation ITU-R F.1336 (recommends 3.1) ka = 0.7 kp = 0.7 kh = 0.7 kv = 0.3 Horizontal 3 dB beamwidth: 65 degrees Vertical 3 dB beamwidth: determined from the horizontal beamwidth by equations in Recommendation ITU-R F.1336. Vertical beamwidths of actual antennas may also be used when available. |  | Recommendation ITU-R F.1336 omni |  |

TABLE A1-2 (continued)

|  | Macro suburban | Macro urban |  | Small cell outdoor/ Micro urban | Small cell indoor/ Indoor urban |  |
| --- | --- | --- | --- | --- | --- | --- |
| Base station characteristics/Cell structure |  |  |  |  |  |  |
| Antenna polarization | Linear/±45 degrees |  | Linear/±45 degrees | Linear | Linear |  |
| Indoor base station deployment | n.a. |  | n.a. | n.a. | 100% |  |
| Indoor base station penetration loss | n.a. |  | n.a. | n.a. | 20 dB (3-5 GHz) 25 dB (5-6 GHz) (horizontal direction) Rec. ITU-R P.1238, Table 3 (vertical direction) |  |
| Below rooftop base station antenna deployment | 0% |  | 50% | 100% | n.a. |  |
| Feeder loss | 3 dB |  | 3 dB | n.a | n.a |  |
| Maximum base station output power (5/10/20 MHz) | 43/46/46 dBm |  | 43/46/46 dBm | 24 dBm | 24 dBm |  |
| Maximum base station antenna gain | 18 dBi |  | 18 dBi | 5 dBi | 0 dBi |  |
| Maximum base station output power/sector (EIRP) | 58/61/61 dBm |  | 58/61/61 dBm | 29 dBm | 24 dBm |  |
| Average base station activity | 50% |  | 50% | 50% | 50% |  |
| Average base station power/sector taking into account activity factor | 55/58/58 dBm |  | 55/58/58 dBm | 26 dBm | 21 dBm |  |
| User terminal characteristics |  |  |  |  |  |  |
| Indoor user terminal usage | 70% |  | 7% | 70% |  | 100% |
| Indoor user terminal penetration loss | 20 dB |  | 20 dB | 20 dB |  | 20 dB (3-5 GHz) 25 dB (5-6 GHz) (horizontal direction) Rec. ITU-R P.1238, Table 3 (vertical direction) |
| User terminal density in active mode to be used in sharing studies | 2.16/5 MHz/km2 |  | 3/5 MHz/km2 | 3/5 MHz/km2 |  | Depending on indoor coverage/capacity demand |

TABLE A1-2 (end)

|  | Macro suburban | Macro urban |  | Small cell outdoor/Micro urban | Small cell indoor/Indoor urban |  |
| --- | --- | --- | --- | --- | --- | --- |
| User terminal characteristics |  |  |  |  |  |  |
| Maximum user terminal output power | 23 dBm |  | 23 dBm | 23 dBm |  | 23 dBm |
| Average user terminal output power | –9 dBm |  | –9 dBm | –9 dBm |  | –9 dBm |
| Typical antenna gain for user terminals | –4 dBi |  | –4 dBi | –4 dBi |  | –4 dBi |
| Body loss | 4 dB |  | 4 dB | 4 dB |  | 4 dB |

ANNEX 2

Antenna characteristics for IMT-2020

TABLE A2-1

Antenna Characteristics for IMT-2020

|  |  | Outdoor Suburban hotspot |  | Outdoor Urban hotspot | Indoor |
| --- | --- | --- | --- | --- | --- |
| 1 | Base station Antenna Characteristics |  |  |  |  |
| 1.1 | Antenna pattern | Refer to Recommendation ITU-R M.2101 |  |  |  |
| 1.2 | Element gain (dBi) | 5 | 5 |  | 5 |
| 1.3 | Horizontal/vertical 3 dB beamwidth of single element (degree) | 65º for both H/V | 65º for both H/V |  | 90º for both H/V |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V | 30 for both H/V |  | 25 for both H/V |
| 1.5 | Antenna polarization | Linear ±45º | Linear ±45º |  | Linear ±45º |
| 1.6 | Antenna array configuration (Row × Column) NOTE 2 | 8×8 elements | 8×8 elements |  | 8×8 elements |
| 1.7 | Horizontal/Vertical radiating element spacing | 0.5 of wavelength for both H/V | 0.5 of wavelength for both H/V |  | 0.5 of wavelength for both H/V |
| 1.8 | Array Ohmic loss (dB) | 3 | ## 3 |  | 3 |
| 1.9 | Conducted power (before Ohmic loss) per antenna element (dBm/200 MHz) | 10 | 10 |  | 5 |
| 1.10 | Base station maximum coverage angle in the horizontal plane (degrees) | 120 | 120 |  | 120 |
| 2 | User terminal Antenna Characteristics |  |  |  |  |
| 2.1 | Antenna pattern | Refer to Recommendation ITU-R M.2101 |  |  |  |
| 2.2 | Element gain (dBi) | 5 | 5 |  | 5 |
| 2.3 | Horizontal/vertical 3 dB beamwidth of single element (degree) | 90º for both H/V | 90º for both H/V |  | 90º for both H/V |
| 2.4 | Horizontal/vertical front-to-back ratio (dB) | 25 for both H/V | 25 for both H/V |  | 25 for both H/V |
| 2.5 | Antenna polarization | Linear ±45º | Linear ±45º |  | Linear ±45º |
| 2.6 | Antenna array configuration (Row × Column) NOTE 2 | 4×4 elements | 4×4 elements |  | 4×4 elements |
| 2.7 | Horizontal/Vertical radiating element spacing | 0.5 of wavelength for both H/V | 0.5 of wavelength for both H/V |  | 0.5 of wavelength for both H/V |
| 2.8 | Array Ohmic loss (dB) | 3 | 3 |  | 3 |
| 2.9 | Conducted power (before Ohmic loss) per antenna element (dBm / 200 MHz) | 10 | 10 |  | 10 |

________________

____________________