| To: Mr Dino FLORE, Qualcomm Chairman 3GPP RAN oflore@qti.qualcomm.com Mr Xutao ZHOU, Samsung Electronics Chairman 3GPP RAN4 xutao.zhou@samsung.com |  |  |
| --- | --- | --- |
|  |  | Date: 20th January 2017 |
| Source: ECC PT1 |  |  |
| Subject: Channeling arrangement and Least Restrictive Technical Conditions applicable to 1427-1518 MHz in CEPT |  |  |

Dear Dino, dear Xutao,

ECC Decision (13)03 defines the harmonized use of the frequency band 1452-1492 MHz for Mobile/Fixed Communications Networks Supplemental Downlink (MFCN SDL) in CEPT.

The bands 1427-1452 MHz and 1492-1518 MHz were identified for IMT at WRC-15 and CEPT launched the work in order to harmonise the bands for mobile and fixed communications networks. CEPT supports that the frequency band 1427-1518 MHz for IMT is for a one direction downlink service, used in connection with another IMT band that provides the uplink capabilities.

ECC is currently developing channelling arrangement and harmonised technical conditions for Mobile/Fixed Communications Networks Supplemental Downlink (MFCN SDL) in the frequency bands 1427-1452 MHz and 1492-1518 MHz. The studies are conducted by ECC PT1.

ECC PT1 would like to inform 3GPP that the harmonised frequency arrangement currently considered by CEPT is based on a block size of 5 MHz, resulting in the following 18 frequency blocks in 1427-1517 MHz.

Harmonised frequency arrangement

| 1427 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1517 MHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1427-1432 MHz | 1432-1437 MHz | 1437-1442 MHz | 1442-1447 MHz | 1447-1452 MHz | 1452-1457 MHz | 1457-1462 MHz | 1462-1467 MHz | 1467-1472 MHz | 1472-1477 MHz | 1477-1482 MHz | 1482-1487 MHz | 1487-1492 MHz | 1492-1497 MHz | 1497-1502 MHz | 1502-1507 MHz | 1507-1512 MHz | 1512-1517 MHz |  |
|  | Downlink (base station transmit) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 90 MHz (18 blocks of 5 MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Furthermore, ECC PT1 would like to inform 3GPP that:

- base stations operating in 1492-1517 MHz should be restricted to an Out Of Band e.i.r.p. limits from 1520 to 1559 MHz of -30 dBm/MHz.

- the ITU Resolution 750 indicates that unwanted emission power level of IMT base stations operating in 1427-1452 MHz should be limited to −72dBW/27MHz in the 1400-1427 MHz band, where unwanted emission power level is to be understood here as the level measured at the antenna port.

ECC PT1 is planning to update 3GPP on the BEM applicable to the 1427-1517 MHz band following its 55th meeting, 24-28th April 2017.

Best regards,

Steve Green

Chairman ECC PT1

E-mail: steve.green@ofcom.org.uk