3GPP TSG-RAN WG4 Meeting #AH1807                     	  					R4-1809547

Montreal, Canada, July 2nd –6th, 2018

Title:	LS on missing SSB information

Response to:

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN WG4

To:	RAN WG2

CC:

Contact Person:

Name:	Tsang-wei Yu

E-mail Address:	ato.yu@mediatek.com

Attachments:

1. Overall Description:

RAN4 would like to inform RAN2 that some of the SSB-related assistance information are missing in current TS 36.331 spec and TS 38.331 spec.

- In TS 38.331,

  - For NR PSCell/SCell addition and handover procedure,

    - The assistance information about SSB timing offset for the target cell is missing in ServingCellConfigCommon.

    - It is suggested to provide additional optional information for timing offset of SSB for the target cell based on the timing reference of current serving cell in ServingCellConfigCommon to avoid UE blindly search for the SSB of target cell the whole SSB period.

  - For RRC Release with redirection to an inter-frequency NR cell,

    - The SCS of SSB and SMTC are missing in RedirectedCarrierInfo.

    - It is suggested to provide additional optional information for SCS of SSB and SMTC periodicity/offset/duration in RedirectedCarrierInfo to provide the needed information for UE to search for the SSB of target cell.

- In TS 36.331

  - For RRC Release with redirection to an inter-RAT NR cell,

    - The SCS of SSB and SMTC are missing in RedirectedCarrierInfo.

    - It is suggested to provide additional optional information of SCS of SSB and SMTC periodicity/offset/duration in RedirectedCarrierInfo to provide the needed information for UE to search for the SSB of target cell.

2. Actions:

To RAN WG2

ACTION: 	RAN4 respectfully asks RAN2 to consider above suggestions in modifying RRC signalling specs.

3. Date of Next TSG-RAN WG4 Meetings:

TSG RAN WG4 Meeting #88		Aug 20 – 24, 2018			             Gothenburg, Sweden

TSG RAN WG4 Meeting #88b		October 8 – 12, 2018				        Chengdu, China