3GPP TSG-RAN WG4 Meeting#96-e	R4- 2012770

# Online, 17th Aug – 28th Aug, 2020

Title:	LS on Test Methodology for UE URLLC Ultra Low BLER Tests

Release:	Rel-16

Work Item:	NR_L1enh_URLLC

Source:	RAN WG4

To:	RAN WG5

Cc:

Contact Person:

#### Name:	Gaurav Nigam

E-mail Address:	gnigam@qti.qualcomm.com

Attachments:

1. Overall Description:

Under the release 16 work item on Physical layer enhancements for NR URLLC (NR_L1enh_URLLC), RAN4 discussed the feasibility of testing 1e-5 BLER test cases. RAN4 agreed to define 1e-5 BLER requirements. When developing the requirements, it was agreed that testing shall be performed with a 99.999% confidence level. RAN4 concluded that using the early pass/fail test methodology unacceptably long test times could be avoided. Furthermore, RAN4 agreed that for the test requirement, in addition to the test tolerance a further 0.5dB should be added to the test requirement. The additional 0.5dB ensures that for a compliant device population, early pass will occur within a reasonable test duration. Based on these assumptions, RAN4 concluded that test times for fixed MCS test with 1e-5 BLER requirements are feasible. RAN4 suggests to add a supplementary X = 0.5dB to the RAN4 1e-5 BLER test requirements (in addition to TT) and capture this parameter X in RAN5 specification. Furthermore, RAN4 suggests RAN5 to use an adjusted early termination test method that ensures 99.999% CL. For information, the per step decision risk that was calculated as achieving this CL by different companies is provided in the Annex for reference purposes.

2. To RAN WG5 group.

ACTION: RAN4 respectfully asks RAN5 to take above into consideration when defining the test methodology for URLLC 1e-5 BLER test cases.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN WG4 Meeting #97-e	26 Oct - 13 Nov 2020	 Online

TSG RAN WG4 Meeting #98                   1 - 5 Mar 2021            	 Athens, GR

## Appendix: Early Pass/Fail Test Methodology

Company A (R4-1913495):

Using the test methodology in Appendix F.6.1 in TS 34.121-1, D = 0.000075% is used for early pass/fail methodology to achieve 99.999% confidence level.

Company B (R4-1913482):

Using the test methodology in Appendix F.6.1 in TS 34.121-1, D = 0.00008% is used for early pass/fail methodology to achieve 99.998% confidence level.

Company C (R4-1913406):

Using the test methodology in Appendix G.7.10 in TS 36.521-1, dearly fail=8e-7 and clearly pass=1-1e-7=0.9999999 is used for early pass/fail methodology to achieve 99.999% confidence level.