# 3GPP TSG SA WG3 (Security) eMeeting	S3c0011

On LTE Call redirection to GERAN, 9 Novemebr 2017

Title:	Reply to Reply LS on LTE call redirection to GERAN

Response to:	S3-171011/C1-173752, S3-171017/ R2-169124 LS on GERAN redirection

Release:	Rel-15

Work Item:	TEI15

Source:	SA3

To:	CT1, RAN2, RAN3

Contact person:	Suresh Nair

Suresh.p.nair@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S3c0006 (Modified AS Token.zip)

# 1	Overall description

SA3 thanks CT1 and RAN2 for their respective LSs in S3-171011/C1-173752 and S3-171017/R2-169124 on LTE call redirection to GERAN.

SA3 agrees with the CT1 on default behaviour to distrust a new network until an attach or tracking area update procedure is successfully completed and the secured redirection policy of the network is known to the UE.

To integrity protect the redirection command to the UE, SA3 discussed and arrived at possible two solutions.

- Solution 1: Calculate a NAS key from the NAS context and further calculate an integrity key at the eNB using this key to protect the redirection parameters. The solution is described in the attachment S3-17c0006.

- Solution 2: Establish regular AS context between the UE and the eNB and then issue the secured redirect command to the UE.

Both solutions require the UE to learn the network policy on secured redirection.

Note: even for network that do not support GERAN, the network policy of rejecting unsecured redirections to GERAN is needed to protect the UE.

The protection of re-direction commands proposed in Solution 2 is already supported in specifications, and in use by some networks (eNBs). SA3 has understood that Solution 2 has the additional cost of AS security context establishment and roundtrip messages but leaves this to RAN2 to evaluate.

Solution 1 require updates in UE, eNB and MME to protect the LTE call redirection to GERAN.  But Solution 1 could be generically used in any scenarios where a redirection to another RAT/network is required without transitioning the UE to active mode or incurring additional signalling and delay associated with it.

Both solutions have their own advantages. MME knows from configuration which solution is supported by the eNB and send the trigger accordingly. The UE and eNB could use Solution 1 or 2 based on the trigger.

SA3 thinks both solutions can provide secured redirection, the differences are in implementation overhead and performance in different usage scenarios. SA3 leave the final decision on which of the above solution(s) to include to RAN2 and CT1.

# 2	Actions

To RAN2, CT1

ACTION:  	SA3 kindly asks RAN2 and CT1 to take the above feedback into account and choose the appropriate security solution(s) to specify.

To RAN3

ACTION: If RAN2 and CT1 choose to specify Solution 1, then SA3 kindly asks RAN3 to choose an appropriate S1-AP message to send the request message to eNB for solution #1.

# 3	Dates of next TSG SA WG3 meetings

TSG SA WG3 Meeting 89	27 November - 1 December 2017	Reno, USA

TSG SA WG3 Meeting 90	22 -26 January 2018	Gothenburg, Sweden