| Title*: | Mapping functions for alignment of model A and B (TS 103 281) |  |  |
| --- | --- | --- | --- |
| from Source*: | HEAD acoustics GmbH |  |  |
| Contact: | Jan Reimes |  |  |
| input for Committee*: | STQ |  |  |
| Contribution For*: | Decision |  |  |
|  | Discussion | X |  |
|  | Information |  |  |
| Submission date*: | 2018-02-21 |  |  |
| Meeting & Allocation: | STQ#57 - |  |  |
| Relevant WI(s), or deliverable(s): | RTS/STQ-208-1 ; RTS/STQ-208-2 |  |  |

ABSTRACT: The document provides additional information on model A and B of TS 103 281 in order to define comparable performance requirements for terminals.

# Introduction

The prediction models for noisy speech quality according to TS 103 281 were trained on a large number of auditory databases according to ITU-T P.835. These databases included a wide range of varying parameters like e.g., speech/noise quality ranges or typical noise reduction artefacts. In consequence, each database has its own context, which had also impact on the training of the models: training for model A tried to compensate for each individual context, while model B did not apply any pre-processing of auditory data. In consequence, each model provides high correlation with auditory data, but with different offsets and/or shifts.

This complicates of defining common performance requirements, for e.g., terminal measurement specifications. Based on the validation data included in TS 103 281, this contribution proposes a mapping function per category in order to align prediction results of both models.

# Derivation of mapping functions

## Comparison to auditory databases

For the proposed mapping functions, the validation databases DES-25 and -26 are investigated in the following. These databases consist of recordings made with real terminals and thus can be regarded as realistic use cases of both models. In order to analyze if one of the models systematically over- or underestimates a certain attribute, Figure 1 to Figure 3 shows the merged data of the two databases (2x48 conditions = 96 data points) within one scatter plot per attribute.

The following observations can be made:

- S-MOS vs SIG: Model A slightly underestimates scores, results of model B requires less correction in the upper MOS range (>3.5)

- N-MOS vs BAK: Model A overestimates scores over the full range while model B does not need any mapping function in this case.

- G-MOS vs OVRL: Results for model A and B provide very similar results. If at all, both require only slight mapping.

| Figure 1: Prediction results for databases DES-25/-26 for S-MOS |  |
| --- | --- |

| Figure 2: Prediction results for databases DES-25/-26 for N-MOS |  |
| --- | --- |

| Figure 3: Prediction results for databases DES-25/-26 for G-MOS |  |
| --- | --- |

## Mapping between the models

To avoid mapping function for both models and the three categories (as shown in Figure 1 to Figure 3), it is proposed to map only model A to the scores of model B, since model B provides instrumental results which are closer to the auditory ones. By mapping results of model A, it will also be possible to define common performance requirements which are valid for both models.

For this purpose, Figure 4 to Figure 6 illustrate results of model B vs A for each attribute. In addition, each plot provides a linear mapping function, which can be used to transform results from model A into the range of model B.

Figure 4: Prediction results of model B vs A for databases DES-25/-26 (S-MOS)

Figure 5: Prediction results of model B vs A for databases DES-25/-26 (N-MOS)

Figure 6: Prediction results of model B vs A for databases DES-25/-26 (G-MOS)

## Proposed mapping functions

In order to transform results of model A for the evaluation of performance requirements, the following functions are proposed:

S-MOSLQOf'=1.418⋅S-MOSLQOf-1.145
N-MOSLQOf'=1.346⋅N-MOSLQOf-1.584
G-MOSLQOf'=1.279⋅G-MOSLQOf-0.7364

## Performance requirements

The contribution STQ(18)057_015 presented initial results of the TS 103 281 algorithm for eight several state-of-the-art mobile devices in EVS-SWB mode (handset and handheld hands-free). The intention of the data is to provide indication for possible performance requirements. Table 1 presents updated analysis results with the shown mapping functions. It can be observed that statistics of both model are now aligned more accurately.

Table 1: Updated statistics for data presented in STQ(18)057_015

| Mode | Model | Attribute | Min | Q25 | Avg | Median | Q75 | Max |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HS | A’ | S-MOS | 4.20 | 4.28 | 4.34 | 4.29 | 4.35 | 4.66 |
|  |  | N-MOS | 2.87 | 3.18 | 3.57 | 3.78 | 3.87 | 4.13 |
|  |  | G-MOS | 3.29 | 3.51 | 3.57 | 3.60 | 3.67 | 3.86 |
|  | B | S-MOS | 4.19 | 4.32 | 4.36 | 4.34 | 4.41 | 4.53 |
|  |  | N-MOS | 2.98 | 3.35 | 3.56 | 3.64 | 3.76 | 4.22 |
|  |  | G-MOS | 3.27 | 3.57 | 3.70 | 3.72 | 3.81 | 4.14 |
| HF | A’ | S-MOS | 3.39 | 3.46 | 3.61 | 3.57 | 3.78 | 3.91 |
|  |  | N-MOS | 2.02 | 2.56 | 2.73 | 2.80 | 2.99 | 3.10 |
|  |  | G-MOS | 2.58 | 2.60 | 2.73 | 2.68 | 2.80 | 3.00 |
|  | B | S-MOS | 3.29 | 3.32 | 3.64 | 3.66 | 3.88 | 4.15 |
|  |  | N-MOS | 2.36 | 2.58 | 2.71 | 2.74 | 2.85 | 2.98 |
|  |  | G-MOS | 2.60 | 2.67 | 2.79 | 2.68 | 2.87 | 3.12 |