SD-357

3GPP - BBF Common Functions

Revision: 03

Revision Date: October 2016

Notice

The Broadband Forum is a non-profit corporation organized to create guidelines for broadband network system development and deployment.  This Broadband Forum Study Document is a draft, and has not been approved by members of the Forum.  Even if approved, this Broadband Forum Study Document is not binding on the Broadband Forum, any of its members, or any developer or service provider.  This Broadband Forum Study Document is subject to change.  This Broadband Forum Study Document is copyrighted by the Broadband Forum, and portions of this Broadband Forum Study Document may be copyrighted by Broadband Forum members. This Study Document is for use by Broadband Forum members only.  Advance written permission by the Broadband Forum is required for distribution of this Broadband Forum Study Document in its entirety or in portions outside the Broadband Forum.

Recipients of this document are requested to submit, with their comments, notification of any relevant patent claims or other intellectual property rights of which they may be aware that might be infringed by any implementation of the Specification set forth in this document, and to provide supporting documentation.

THIS SPECIFICATION IS BEING OFFERED WITHOUT ANY WARRANTY WHATSOEVER, AND IN PARTICULAR, ANY WARRANTY OF NONINFRINGEMENT IS EXPRESSLY DISCLAIMED. ANY USE OF THIS SPECIFICATION SHALL BE MADE ENTIRELY AT THE IMPLEMENTER'S OWN RISK, AND NEITHER the Forum, NOR ANY OF ITS MEMBERS OR

SUBMITTERS, SHALL HAVE ANY LIABILITY WHATSOEVER TO ANY IMPLEMENTER OR THIRD PARTY FOR ANY DAMAGES OF ANY NATURE WHATSOEVER, DIRECTLY OR INDIRECTLY, ARISING FROM THE USE OF THIS SPECIFICATION.

The text of this notice must be included in all copies of this Broadband Forum Study Document.

Revision History

| Revision Number | Revision Date | Revision Editor | Changes |
| --- | --- | --- | --- |
| 00 | February 2015 | Devasena Morrissette Verizon Frederick Klamm Orange | Initial Draft – ToC Creation |
| 01 | July 2015 | Devasena Morrissette Verizon Frederick Klamm Orange | Integration of bbf2015.589 as per discussions and Q2 minutes. |
| 02 | June 2016 | Frederic Klamm | -Reviewed as proposed by DT in bbf2016.658. -Included recommendations for future work in § 8 (bbf2016.658) - Included graphic view of common functions in §3.2 (bbf2016.651) |
| 03 | October 2016 | Frederic Klamm | -Cleaned by editor |

Comments or questions about this Broadband Forum Study Document should be directed to:

| Editors | Devasena Morrissette | Verizon | Devasena.morrissette@verizon.com |
| --- | --- | --- | --- |
| WWC area Area directors | Frederick Klamm David Allan Hongyu Li | Orange Ericsson Huawei | Frederic.klamm@orange.com david.i.allan@ericsson.com hongyu.li@huawei.com |

Table of Contents

Executive Summary	6

1	Purpose and Scope	7

1.1	Purpose	7

1.2	Scope	7

References and Terminology	9

1.3	Conventions	9

1.4	References	9

1.5	Definitions	10

1.6	Abbreviations	10

2	Study Document Impact	12

2.1	Energy Efficiency	12

2.2	IPv6	12

2.3	Security	12

2.4	Privacy	12

3	Identification of Common Functions	13

3.1	Reference Diagram	13

3.2	List of Common functions and sub-functions	15

3.2.1	Functional Decomposition	16

3.2.2	Functional separation of Control Plane and Forwarding Plane	17

4	Impact of Combined Functions	17

5	Network and Infrastructure Considerations	17

6	Requirements	18

7	Other Impact (Non-Functional)	18

7.1	Security	18

7.2	Fault Tolerance	18

8	Recommendations for future work	18

8.1	Opportunities	18

8.2	Recommended Scope of work	18

8.2.1	Recommended BBF scope of work	18

8.2.2	Recommended SDO work	18

Appendix Appendix Example	18

List of Figures

Figure 1:Legends	13

Figure 2: provisioning (mostly physical resources)	13

Figure 3: Access session establishment	14

Figure 4: Access session in steady state	14

Figure 5: Graphic representation of common functions	16

Figure 6: Permutations and combinations	19

List of Tables

Table 1: references	10

Table 2: Definitions	10

Table 3: abbreviations	11

Table 4: terminology	17

# Executive Summary

Convergent network architectures and solutions were developed and have evolved over the recent years, but have so far only been partially implemented in today’s fixed and mobile broadband networks. At the time of this writing, the networking industry is making major advances with SDN, Virtualization and the overall End-to-End Network and Service Orchestration, unlocking new potentials for Fixed Mobile Convergence (FMC) in general as well as in the 5G context.

In the BBF there are several projects underway in the area of SDN and Virtualization. Network Functions Virtualization provides a new level of flexibility where functional components can be distributed and/or regrouped. This regrouping allows for common functions and sub-functions belonging to different network elements to be instantiated as one.

Leveraging this potential for the purpose of this project, which focuses on the commonality of the network infrastructures and (virtualized) network functions in fixed and mobile networks, provides significant opportunities, including: product and service innovation, simplified operation, increased flexibility and cost reduction.  It makes perfect sense to combine common functions as there is no reason to recreate a fixed-wireless split when the same network function can be instantiated and used in a flexible way.

This document contains the results of the study on 3GPP-BBF Common Functions, that is performed in a stepwise approach, addressing the following business needs:

1 – Identification of common network functions between the 3GPP and BBF domains is a necessity and offers the following benefits:

- Possibly lower function cost (one product for multiple uses)

- Encourage product differentiation on a new and more innovative level

- Simplify operation (operator deploys similar objects for similar purposes)

- Increase dynamicity (when possible, common functions are colored for wireless and/or wireline use at instantiation time)

- Enable new services and business opportunities, combing fixed and /mobile access technologies

2 –Analysis of virtualization aspects of identified common functions, as an enabler for future study in the area of Virtualized Network Functions (VNFs) and Network Function Virtualization Infrastructures (NFVI), destined for mobile and fixed networks.

The results of this work in the BBF serve as a foundation for further collaboration opportunities with the 3GPP and with the ETSI ISG NFV.

# Purpose and Scope

## Purpose

The purpose of this study is to identify and compare network functions implemented in fixed and mobile network architectures with the goal to identify common functions. These functions need to be deployed only once in a convergent network architecture covering both fixed and mobile access.

The study includes an analysis of functional de-composition wherein certain functions and associated sub-functions may be 
a.) Completely different, 
b.) Have a common part, 
c.) One may be included in the other, 
d.) Or be different specializations of a more generic functions.

Further goal is to derive requirements and make recommendations for deployment of common functions on the network and infrastructure.

The outcome of this project is also expected to support optimizing common functions for deployment as Virtualized Network Functions (VNF). This would allow leveraging benefits inherent to NFV, including, but not limited to, dynamic instantiation of given common functions for 3GPP or BBF use, on demand and in a flexible way.

Furthermore, SD-357 “3GPP-BBF common functions” supports ongoing 5G related enabler work.

## Scope

This project focuses on combining or mutualizing common 3GPP and BBF functions and related sub-functions. In order to combine them, those common functions and sub-functions need to first be identified. As an example below:

Some functions that are common between the PGW and the BNG are:

- PCEF (hosted on BNG or PGW NE)

- PCRF

- OCS, OFCS

Examples of other common functions are:

- AAA

- PPP

- Ethernet

- IP assignment

- Network Address Translation

- Queuing/Scheduling/Marking/Filtering

- Tunneling technologies / transport layer technologies

Existing BBF documents TR-134, TR-146, TR-291, WT-300 and SD-340 are leveraged in this work. The functions and sub-functions already discussed in these documents can be prioritized whilst identifying new ones in parallel.

In addition the following items are addressed:

- Identify the relationship between the common functions and sub-functions

- Analyse the effect on the network of such common functions and sub-functions

- Aspects of common functions at the forwarding and control plane level

- Make recommendations and derive requirements based on above

The study is structured into two phases:

Phase 1: Identification of 3GPP-BBF common functions

Identify 3GPP and BBF functions that are common, and possibly what distinguishes them. Functions/components/sub-components already discussed in BBF TR-291 and TR-300 will be included in the consideration.

Phase 2: Analysis of structural and functional convergence, enabled by SDN/NFV

Analyze location dependency in connection with BBF work on SDN & NFV, analyze what may be function-dependent, especially the location of functions.

# References and Terminology

## Conventions

In this Study Document, several words are used to signify the requirements of the specification. These words are always capitalized. More information can be found be in RFC 2119 [5].

| MUST | This word, or the term “REQUIRED”, means that the definition is an absolute requirement of the specification. |
| --- | --- |
| MUST NOT | This phrase means that the definition is an absolute prohibition of the specification. |
| SHOULD | This word, or the term “RECOMMENDED”, means that there could exist valid reasons in particular circumstances to ignore this item, but the full implications need to be understood and carefully weighed before choosing a different course. |
| SHOULD NOT | This phrase, or the phrase "NOT RECOMMENDED" means that there could exist valid reasons in particular circumstances when the particular behavior is acceptable or even useful, but the full implications need to be understood and the case carefully weighed before implementing any behavior described with this label. |
| MAY | This word, or the term “OPTIONAL”, means that this item is one of an allowed set of alternatives. An implementation that does not include this option MUST be prepared to inter-operate with another implementation that does include the option. |

## References

The following references are of relevance to this Study Document. At the time of publication, the editions indicated were valid. All references are subject to revision; users of this Study Document are therefore encouraged to investigate the possibility of applying the most recent edition of the references listed below.

A list of currently valid Broadband Forum Technical Reports is published at 
www.broadband-forum.org.

| Document | Title | Source | Year |
| --- | --- | --- | --- |
| RFC 2119 | Key words for use in RFCs to Indicate Requirement Levels | IETF | 1997 |
| TR-134 Issue 1 | Broadband Policy Control Framework | BBF | 2012 |
| TR-146 Issue 1 | Subscriber Sessions | BBF | 2013 |
| TR-291 Issue 1 | Nodal Requirements for Interworking between Next Generation Fixed and 3GPP Wireless Access | BBF | 2014 |
| TR-300 Issue 1 | Policy Convergence for Next Generation Fixed and 3GPP Wireless Networks | BBF | 2014 |
| SD-340 | Stage 1 for Introduction of Network Function Virtualization in MSBN | BBF | 2015 |

Table 1: references

## Definitions

The following terminology is used throughout this Study Document.

| ACS | Auto-Configuration Server. This is a component in the broadband network responsible for auto-configuration of the CPE for advanced services. |
| --- | --- |
| API | Application Programming Interface; a set of tools, interfaces, protocols and logic that is necessary to execute software based services and applications |
| KPI | Key Performance Indicator |
| NBI | North Bound Interface; Between the Controller and Control Plane to the Orchestration/Application layer |
| SDN | Software Defined Networks |
| SBI | South Bound Interface; From Controller or Control plane to the Forwarding plane |

Table 2: Definitions

## Abbreviations

This Study Document uses the following abbreviations:

| AAA | Authentication Authorization Accounting |
| --- | --- |
| BNG | Broadband Network Gateway |
| CP | Control Plane |
| CPE | Customer Premise Equipment |
| DC | Data Center |
| DNS | Domain Name System |
| EPC | Evolved Packet Core |
| FMC | Fixed Mobile Convergence |
| FP | Forwarding Plane |
| LTE | Long Term Evolution |
| MME | Mobility Management Entity |
| MSBNG | Multi Service Broadband Network Gateway |
| NFV | Network Function Virtualization |
| NFVI | Network Functions Virtualization Infrastructure |
| OCS | Online charging system |
| OFCS | Offline Charging System |
| PCEF | Policy Control Enforcement Point |
| PCRF | Policy Control Resource Function |
| PDN | Packet Data Network |
| P-GW | Packet Gateway |
| QoS | Quality of Service |
| RAN | Radio Access Network |
| S-GW | Serving Gateway |
| SD | Study Document |
| TR | Technical Report |
| UE | User Endpoint |
| VLAN | Virtual LAN |
| VNF | Virtualized Network Function |
| WG | Working Group |
| WT | Working Text |

Table 3: abbreviations

# Study Document Impact

## Energy Efficiency

Since SD-357 focuses on combining common functions that could potentially get instantiated on a virtualized platform and brought down depending on the need of the network and associated services, it is highly likely that power saving could be achieved to a considerable extent.

SD-357 assists in power saving measures.

## IPv6

IPv6 and IPv4 are supported by both 3GPP and BBF domains for various protocols and functions. IPv6 may be an enhancer for virtualization of common functions compared to IPv4, i.e there may be examples where combining functions could be feasible with IPv6 where it may not be the case with IPv4

SD-357 also follows IPv6 specifications,

In order to achieve a sun setting of IPv4 and with focus on a future common network architecture IPv6 should be seen as the optimal supported IP technology here. To phase out IPv4 services using the version 4 will be tunneled over IPv6.

## Security

Authentication and data protection methods may depend on the service level and the risk level associated to the way the UE is attached to the core.

## Privacy

Privacy impact on SD-357 is to be further studied

# Identification of Common Functions

<Insert text here>

## Reference Diagram

The following legend should be helpful to read the figures of this section.

Figure 1:Legends

4.1.1 BBF TR-101 and 3GPP TS 23.401functional architectures

The following diagrams outline the functions involved in network access service for a given subscriber. 3 typical steps in which different functions takes part, are distinguished:

- access service provisioning,

- access session establishment

- session steady state

The first set of diagrams (figures 2, 3, 4) shows side by side BBF TR-101 and 3GPP TS 23.401 architectures.

Figure 2: provisioning (mostly physical resources)

Figure 3: Access session establishment

Figure 4: Access session in steady state

## List of Common functions and sub-functions

IP network Access

Every function hereafter is regarded in the context of network access service subscription.

Access control

Authentication

authentication and authorization are generally presented as 2 inseparable steps of a same process by which a subscriber get access to a service or not.

So authentication is the function realizing the first step of the authentication/authorization.

process sub-functions or components:

- supplicant = UE or RG

- in-band authentication protocol = (see 401) or DHCP

- authenticator = MME or BNG out-of-band protocol = (401 Diameter xxx) or RADIUS

- authentication server = HSS or RADIUS Server

Authorization

The authorization function is invoked by the authentication function once the requesting subscriber’s rights are known. authorization does the minimal resource allocation (gating) and invokes the resource allocation function The authorization step completes the authentication/authorization process.

Resource allocation

Resource allocation brings together all types of resource configuration for the device requesting access on behalf of the access service provider subscriber.

Resource allocation sub-functions

WAN Network interface configuration

UE or RG requesting IP network informations by sending a broadcast to the corresponding address assingment server and receiving appropriate IP prefix or IP address, domaine name server adresses and other useful information that may be specific to the service

Access connection control

or access session control

A subscriber access session is created as a result of positive authorization. This (pseudo-) connection symbolizes a subscriber device set of traffic going to and from to the target network. It is subject to monitoring and accounting and may experience changes of configuration achieved by the policy control function during its lifetime.

Policy control

Depending on the subscriber contract and the access network conditions, policies for QoS, accounting or routing may be applied on the the appropriate node along the access connection.

This node may hold default rules and these may be overwritten by new ones provided by the policy control function.

Accounting

The "accounting" function yields information on living sessions, mainly for charging purpose. Accounting…

Access transfer

L2 aggregation

xxx

L3 aggregation

xxx

traffic scheduling

xxx

Figure 5: Graphic representation of common functions

### Functional Decomposition

<Certain functions enlisted in section 4.2 can be decomposed into sub-functions; List those>

Components involved in authentication

- supplicant = UE or RG

- authenticator = MME or BNG

- in-band authentication protocol = [3GPP] NAS or RRC or DHCP

- out-of-band authentication protocol = [3GPP] NASoS1-AP or RADIUS

- authentication server = HSS or RADIUS Server

|  | 3GPP function/terminology | BBF function/terminology |
| --- | --- | --- |
| Network node | UE | RG |
| Authenticator | MME | BNG |
| in-band authentication protocol |  |  |
| out-of-band authentication protocol |  |  |

Table 4: terminology

### Functional separation of Control Plane and Forwarding Plane

<Certain functions enlisted in section 2 could have control and/or data plane/user components; eg PCEF-C that receives rules from a PCRF and instantiates them upon subscriber connect>

<Definitions needed first:

User Plane

Data Plane

Control Plane  >

# Impact of Combined Functions

<Study of analysis and impact; This section will go into details of positive impact as well as negative impact. The combining of functions that could negatively impact the network and infrastructure could be later considered in the requirements section along with the rest and could be a foundation to recommendations to the industry>

# Network and Infrastructure Considerations

<This section will detail several things to be considered when taking into account the combined functions such as

- Placement:	what is the location of the function

- Latency:	assume that a CP and FP function are never co-located

- Etc…

# Requirements

<Enlist requirements>

# Other Impact (Non-Functional)

## Security

<Security related considerations if any>

## Fault Tolerance

<Redundancy, High Availability>

# Recommendations for future work

## Opportunities

This document provides, as a foundation (status quo), an overview of relevant functions, including their location, as 3GPP-BBF common functions study phase 1.

The next step, in phase 2, should include functional convergence study, based on a detailed use case and requirements analysis, involving both mobile (3GPP-defined) and fixed (BBF-defined) access networks, and should be performed in interaction and cooperation with 3GPP SA2.

## Recommended Scope of work

### Recommended BBF scope of work

### Recommended SDO work

- Appendix 
Appendix Example

Figure 6: Permutations and combinations

Figure 1 depicts the relationship between functions using Venn Diagrams. Any two functions could be

- Completely different in functionality

- They may have a more or less important common functionality

- One may be included in the other

They may be different specializations of a more generic function

End of Broadband Forum Study Document SD-357