China Communications Standards Association

# Suzhou, 29th-30th October, 2019

TC9 WG1 #51 meeting

Ref: 2019-10-29

Tel: +86 10 62302645

Fax: +86 10 62301849

E-mail: zhaosz@ccsa.org.cn

# Kyoungseok Oh, Ron Borsato

# 3GPP RAN4, CTIA

E-mail: kyoungseok.oh@etsi.org

ron.borsato@pctest.com

Subject: Liaison Statement on CCSA progress on NR FR1 OTA

Source: CCSA TC9 WG1

To: 3GPP RAN4, CTIA

Contact Person: Ruixin Wang (CAICT), wangruixin@caict.ac.cn

- Overall Description:

CCSA would like to kindly inform 3GPP RAN4 and CTIA that the following NR FR1 OTA test configurations were approved in CCSA.

  - NR FR1 TRP measurements

For standalone NR FR1 TRP measurements, test shall be carried out for different frequencies and RB allocations across the band supported by the EUT, as defined in Table 1. The NR system simulator and EUT shall be configured per 3GPP TS 38.521-1, Section 6.2 and continuous uplink power control “up” commands shall be sent to the EUT to ensure the EUT’s transmitter is at maximum output power during TRP measurement.

Table 1 Standalone NR FR1 TRP Measurement Configurations

| Band | Channel Bandwidth (MHz) | SCS （kHz） | UL Modulation | Carrier Center Frequency（MHz） | UL RB Allocation | DL RB Allocation |
| --- | --- | --- | --- | --- | --- | --- |
| n41 | 100 | 30 | DFT-s-OFDM QPSK | 2565 | 24@12 | N/A |
|  |  |  |  | 2595 | 24@125 | N/A |
|  |  |  |  | 2625 | 24@237 | N/A |
| n78 |  |  |  | 3450 | 135@67 | N/A |
|  |  |  |  | 3549.99 |  | N/A |
| n79 |  |  |  | 4850.01 | 24@12 | N/A |
|  |  |  |  |  | 24@125 | N/A |
|  |  |  |  |  | 24@ 237 | N/A |

For non-standalone mode, LTE TRP and NR TRP shall be measured at the same time or in sequence while the status of EUT shall remain consistent and LTE and NR shall transmit simultaneously. The maximum output power shall be configured as PLTE = [20] dBm，PNR= None and PEMAX, EN-DC= None.

  - NR FR1 TRS Measurements

For standalone NR FR1 TRS measurements, test shall be carried out for different frequencies and RB allocations across the band supported by the EUT, as defined in Table 2. The NR system simulator and EUT shall be configured per 3GPP TS 38.521-1, Section 7.3 and continuous uplink power control “up” commands shall be sent to the EUT to ensure the EUT’s transmitter is at maximum output power during TRS measurement.

Table 2 Standalone NR FR1 TRS Measurement Configurations

| Band | UL/DL channel | Channel Bandwidth (MHz) | SCS （kHz） | Carrier Center Frequency （MHz） | UL RB Allocation | DL RB Allocation |
| --- | --- | --- | --- | --- | --- | --- |
| n41 | 513000 | 100 | 30 | 2565 | 270@0 | 273@0 |
|  | 525000 |  |  | 2625 |  |  |
| n78 | 630000 |  |  | 3450 |  |  |
|  | 636666 |  |  | 3549.99 |  |  |
| n79 | 723334 |  |  | 4850.01 |  |  |

For non-standalone mode, LTE TRS and NR TRS shall be measured in sequence while the status of EUT shall remain consistent and LTE and NR shall transmit simultaneously. The maximum output power shall be configured as PLTE = [20] dBm，PNR= None and PEMAX, EN-DC= None. When measuring NR TRS, the LTE TRS shall remain error-free stable connection. Same approach for LTE TRS testing is used under NSA mode with error-free NR connection.

CCSA TC9 WG1 respectfully asks RAN4 and CTIA to take these developments into account.

- Actions:

None

- Date of Next CCSA TC9 plenary Meetings:

CCSA TC9 plenary Meeting #29	November, 2019	TBD, China

Reference

[1] CCSA YD/T 1484.9-2019 draft, Measurement method for radiated RF power and receiver performance of wireless device, Part9: NR wireless device (Sub-6GHz)