# E/WBI APIs v0.9 (Current)

# Version 0.9

# [Publication Date]

This is a draft for a Non-binding Permanent Reference Document of the GSMA

Security Classification: Non-confidential

Access to and distribution of this document is restricted to the persons permitted by the security classification. This document is confidential to the Association and is subject to copyright protection. This document is to be used only for the purposes for which it has been supplied and information contained in it must not be disclosed or in any other way made available, in whole or in part, to persons other than those permitted under the security classification without the prior written approval of the Association.

Copyright Notice

Copyright © 2022 GSM Association

Disclaimer

The GSM Association (“Association”) makes no representation, warranty or undertaking (express or implied) with respect to and does not accept any responsibility for, and hereby disclaims liability for the accuracy or completeness or timeliness of the information contained in this document. The information contained in this document may be subject to change without prior notice.

Antitrust Notice

The information contain herein is in full compliance with the GSM Association’s antitrust compliance policy.

Table of Contents

1	Introduction	4

1.1	Overview	4

1.2	Scope	4

1.3	Definitions	4

1.4	Abbreviations	4

1.5	References	5

1.6	Conventions	5

2	Procedures over OP East/West Bound Interface	5

2.1	General	5

2.1.1	Federation	5

2.1.2	Directed Federation	5

2.1.3	Federation Identifier	6

2.1.4	Origination OP	6

2.1.5	Partner OP	6

2.1.6	Offered Zones	6

2.1.7	Accepted Zones	6

2.1.8	Mobile Country Codes	6

2.1.9	Mobile Network Codes	6

2.1.10	Zone Meta-info	7

2.1.11	Edge Discovery Service	7

2.1.12	Mobility Strategy	7

2.1.13	Latency Constraints	7

2.1.14	Application Identifier	7

2.1.15	Artifact Identifier	7

2.1.16	Edge Node	8

2.2	Generic E/WBI Procedures	8

2.2.1	Procedures for federation establishment between OP partners	8

2.2.2	Procedures for availability zone information synchronization	11

2.2.3	Procedures for registration and authorization of end users in a federated OP partner	13

2.3	Application Services Procedures	14

2.3.1	Edge Service Procedures	14

3	OP East/West Bound APIs	28

3.1	Generic East/West Bound Service APIs	28

3.1.1	East/West Bound Interface Management - API	28

3.1.2	Availability Zone Information Synchronization Service – API	38

4	Application Service APIs	47

4.1	Edge Service APIs	47

4.1.1	Application Artifacts Management - APIs	47

4.1.2	Application Provider Resource  Management - APIs	61

4.1.3	Application Onboarding Management - API	68

4.1.4	Application Instance Lifecycle Management - API	76

4.1.5	Edge Node Sharing - API	83

4.1.6	LBO Roaming Authentication – API	85

4.1.7	Network Events – API	87

5	Security	88

6	OpenAPI Specification Sample	89

Annex A	Document Management	114

A.1	Document History	114

# Introduction

## Overview

This document specifies RESTful APIs that allow an OP to share the edge cloud resources and capabilities securely to other partner OPs over E/WBI.

## Scope

The present specification describes the APIs, sequence flows and the representation of the API and parameters in REST for the East West Bound Interface between the two operator platforms (OPs). The E/WBI related stage 1 functional requirements are defined in OP PRD.

## Definitions

| Term | Description |
| --- | --- |
| API Initiator | API Initiator is the entity that originates the first message in the API sequences |
| Federation | Federation refers to relationship among member OPs who agrees to offer OP PRD defined services and capabilities to the application providers and end users of member OPs |
| Directed Federation | A Federation between two OP instances A and B, in which edge compute resources are shared by B to A, but not from A to B. |
| Discovery Service | OP service identified by a well-defined FQDN or IP:Port and protocol pair to assist UCs over UNI to discover adequate edge cloud in the current location of the end users |
| Home OP | The Operator Platform instance belonging to the subscriber's Operator; that is, whose PLMN identity (MCC and MNC) matches with the MCC and MNC of the subscriber's IMSI, as defined in 3GPP TS 23.122. [REF#OP PRD] |
| LCM Service | Lifecycle Management Service to enable UCs for requesting dynamic application instantiation or termination |
| Leading OP | The Operator Platform instance connected to the Application Provider and receiving the onboarding requests, sharing them to the selected federated platforms/operators. [REF#OP PRD] |
| Originating OP | The Operator Platform instance initiating the federation creation request to selected federated platforms/operators. Both leading OP and Home OP will be acting as Originating OP while creating the federation with partner OP. |
| OP Id | Operator id is a uniquely identifier assigned to each OP instance of the federation to identify the member OP |
| Mobility Strategy | It refers to defining an application mobility strategy that includes QoE, geographical store and privacy policies intent |
| Zone | Zone refers to an availability zone as defined in OP PRD |

## Abbreviations

| Term | Description |
| --- | --- |
| E/WBI | East/West Bound Interface |
| LBO | Local Break Out (also defined in PRD as Local BreakOut) |
| OP | Operator Platform |
| OPG | Operator Platform Group |
| PRD | Permanent Reference Document |

## References

| Ref | Doc Number | Title |
| --- | --- | --- |
|  | OPG.02 | Operator Platform Telco Edge Requirements", Version 2.0 14 April 2022 |
|  | RFC 2119 | “Key words for use in RFCs to Indicate Requirement Levels”, S. Bradner, March 1997. Available at http://www.ietf.org/rfc/rfc2119.txt |
|  | Telco Edge Cloud | Telco Edge Cloud: Edge Service Description & Commercial Principles Whitepaper, version 1.0, 27 October 2020 https://www.gsma.com/futurenetworks/resources/telco-edge-cloud-october-2020-download/ |

## Conventions

<If the document includes binding material this section shall contain the following statement, and RFC 2119 shall be included as a reference:

“The key words “must”, “must not”, “required”, “shall”, “shall not”, “should”, “should not”, “recommended”, “may”, and “optional” in this document are to be interpreted as described in RFC2119 [2].”

For non-binding documents, this section may be deleted.>

# Procedures over OP East/West Bound Interface

## General

This section describes some of the key concepts and terms which applies to E/WBI procedures.

### Federation

A federation between two OPs conceptually refers an agreement to share Edge Cloud and Network resources. The procedures which enable the establishment or creation of federation between the OPs are referred as E/WBI  procedures. These procedures can be initiated by an OP towards the partner OP using the set of APIs corresponding to E/WBI.

### Directed Federation

A federation relationship in context of operator platforms is a directional relationship wherein a federation creation request initiated by an OP to partner OP results in partner OP to share edge cloud resources to the requesting OP. Thus, if two OP both wants to share edge cloud resources with each other than both the OP would need to initiate a direction federation creation request towards each other.

### Federation Identifier

A federation identifier is a dynamically generated identifier created by the OP which receives the federation creation request from its partner OPs. Based on the prior information if the OP accepts the federation create request, then the federation identifier is generated and returned to the requesting OP to represent the successful creation of the federation.

This federation identifier shall be included in all the subsequent E/WBI APIs having operations associated to this federation.

### Origination OP

The creation of directed federation from an OP to partner OP may be initiated by an administrative action by the OP administrator. Procedures like E/WBI interconnect management as defined in OP PRD are independent of any application management procedures and any OP can independently initiate such requests towards the partner OP.

The OP when initiating the federation creation request towards the partner OP is defined as the Originating OP. OP PRD defines the term “leading OP” which can be interpreted as a role an OP instance is playing when it is serving applications providers on NBI.

As described, the OP when initiating federation creation request without any dependency to NBI, requires an additional identification which in this document is termed as “Origination OP”.

### Partner OP

The partner OP, also defined as Operator Platform which offers its Edge Cloud capabilities to the other Operator Platforms via E/WBI. In this document the E/WBI procedure considers that the partner OP on receiving federation create request from Originating OP may validate, authenticate, and authorize the request and initiating OP identity and accepts the federation request by generating and sharing the federation identifier with Origination OP.

### Offered Zones

Partner OP may offer to share one or more availability zone(s) and associated resources to the Origination OP based on the prior agreement and local configuration. These zone(s) are defined as offered zones wherein the applications from Origination OP (also leading OP here) can be orchestrated on requests from the application providers of the Originating OP.

### Accepted Zones

Based on the offered zone(s) from partner OP, Originating OP may accept one or more availability zone(s) from partner OP and confirm the accepted zone(s) over E/WBI to partner OP by initiating the availability zone registration procedures.

### Mobile Country Codes

Mobile Country Code (MCC) represents the serving country of the OP when it is shared in federation establishment procedures. For any of the E/WBI APIs, the MCC associated to an OP shall have a single value and it is a non-modifiable parameter.

### Mobile Network Codes

Mobile Network Code (MNC) represents the serving network code(s) of the OP when it is shared in federation establishment procedures. For any of the E/WBI APIs, there can one or more instances of MNC and its E/WBI procedures consider the MNC9s) to be modifiable parameter.

MNCs are having a significant role for determination of the roaming users in visited OP networks and in conjunction with MCC they can be used by home OP to determine the roaming in partner OP footprints.

### Zone Meta-info

Zone or availability zone meta-information refers to the attributes associated to group of edge cloud which an OP can define as zone with a unique zone identifier and other locality information e.g., city, lat/long, country, locality etc.

Zone related meta-information can be shared by an OP with partner OP for various purposes e.g., in availability zone offer during federation create procedure, in application onboarding request to indicate intended availability zone(s) for app deployment etc.

### Edge Discovery Service

Edge discovery service is defined as HTTP based API endpoint identified by a well-defined FQDN or IP: Port and protocol pair to assist UCs to discover adequate edge cloud in the current location of the end users. Every OP may host a publicly accessible discovery service which can be reached by the UCs over UNI to enquire about the nearby edge clouds or application instance(s).

The OP can also use the edge discovery service to redirect the edge discovery requests from roaming users from partner OP networks to be redirected to partner OP edge discovery service based on the network identification.

### Mobility Strategy

As per the OP PRD, it pertains to defining an application mobility strategy that includes QoE, geographical store and privacy policies intent.

An app provider may be able to define the mobility strategy over NBI for an application and it may additionally include the application sensitivity to UCs mobility events.

Mobility strategy may cause OP to take application relocation decisions based on end users’ mobility events and following the mobility strategy defined by the app provider.

### Latency Constraints

The latency constraints refer to the limits on end-to-end latency between the UCs and edge application exceeding which may result into degradation of user experience or quality of experience as defined by the application provider. OP may provide information about different latency profile for the availability zone(s) to application providers and such information can be used as latency constraints with an application on NBI.

### Application Identifier

The leading OP while communicating with a partner OP uses application identifier to refer uniquely to an applications from the leading OP in context to federation relationship with partner OP. The application identifier can be used to maintain uniqueness among the applications, application instances, application monitoring information etc.

### Artifact Identifier

The leading OP while communicating with a partner OP uses artifact identifier to refer uniquely to an artifact from the leading OP in context to federation relationship with partner OP. The artifact identifier can be used to distinguish artifacts for all the app providers of the leading OP on E/WBI and can be reused by different applications as a common reusable service.

### Edge Node

The term Edge Node used in context with the Edge Node Sharing refers to the compute resources offered by the partner OP to the leading OP. The leading OP can request the partner OP to allocate resources for the leading OP applications when requested by the leading OP over E/WBI. Leading OP may initiate E/WBI edge node sharing request to partner OP on application provider actions over NBI or on receiving request from UCs on UNI.

## Generic E/WBI Procedures

These procedures generically covers the federation interconnect and availability zone management functions to provide application deployment and lifecycle management capabilities across partner OPs.

### Procedures for federation establishment between OP partners

These procedures will have key functionalities to establish federation between two OP partners as described in the section 3.5.4.1 of the PRD OPG.02.

Basic functionalities supported must cover:

- Create federation between OP partners

- Update an already establish federation between OP partners

- Remove a federation establishment between OP partners.

#### Create Federation between OP partners

Create Federation Operation is initiated by the originating OP towards the partner OP to establish a directed federation relationship between the two partners. By invoking this operation, the API initiator say ‘OP-A’ requests partner OP-B permission to consume the OP-B resources on edge sites of 'OP-B'.

: Create Federation

The message flow for creating a one-way (directed) federation relationship is as follow:

- A Federation create request (HTTP POST) is sent by the originated OP to the partner OP.

- The Originating OP provides all required identification, authentication, and authorization information elements required to allow the Partner OP to decide if the request can be granted.

- The partner OP sends a HTTP POST response to the originating OP to inform about the result of the operation.

- On success, a 200 OK message is sent along with a message body containing partner OP edge discovery service FQDN, list of availability zone(s) meta-info (e.g., zone Id, geolocation details etc.) that the partner OP can offer to the originating OP.

- On failure, an appropriate error codes (e.g., 401, 404 etc.) along with application-level error message shall be returned. In this case the operator will remove federation data from the database.

- The server errors 500 (Internal Server Error), 503 (Service Unavailable) may also indicate that the request could not be processed by the partner OP and should be retried at a later point of time.

Note: Two OPs in federation relationship are partners to each other but in the context of current document the partner OP is referred to the OP responding to the Federation Create request from the originating OP.

Note: The edge discovery service FQDN shared by the partner OP is for supporting roaming users when they visit partner OP network. In those cases, home OP on receiving the edge discovery requests from UCs, detect the roaming condition, and based on the current network code of the UE determines the partner OP and corresponding edge discovery FQDN and redirects UCs to partner OP edge discovery service.

#### Update Federation between OP partners

To make an update over a federation partnership the request initiator sends an HTTP PATCH message to partner OP to update modifiable parameters which were earlier exchanged during the create federation request flow (e.g., MNC, MCC or Edge Discovery Service URL).

: Update Federation

#### Remove Federation configuration between OP partners

This procedure is intended to remove existing federation information within a partner OP. By Remove Federation Operation, the API initiator OP say ‘OP-A’ sends an HTTP DELETE request to the partner OP say ‘OP-B’ to terminate the existing federation configuration from OP-A to OP-B (earlier created by OP-A via create federation procedures).

: Remove Federation

#### Retrieve partner federated zone meta-information

Originating OP may use this procedure towards federated partners OP to retrieve availability zone(s) meta-information e.g., zone identifier(s), zone(s) location etc. which the partner OP may offer to originating OP. This operation can be invoked on existing federation between the two OPs.

: Retrieve partner federated zone meta-information

The message flow for retrieving partner availability zone(s) meta-information by origination to partner OP on an established federation relationship is as follow:

- A partner federated zone meta-information get request (HTTP GET) is sent by the originated OP to the partner OP.

- The Originating OP provides all required identification, authentication, and authorization information elements required to allow the Partner OP to decide if the request can be granted.

- The partner OP sends a HTTP GET response to the originating OP to inform about the result of the operation.

- On success, a 200 OK message is sent along with a message body containing a list of zones and their geolocation details that the partner OP has available to share with the operator.

- On failure, an appropriate error codes (e.g., 401, 404 etc.) along with application-level error message shall be returned. In this case the operator will remove federation data from the database.

- The server errors 500 (Internal Server Error), 503 (Service Unavailable) may also indicate that the request could not be processed by the partner OP and should be retried at a later point of time.

### Procedures for availability zone information synchronization

As described in the section 3.5.4.2 of the PRD OPG.02 these procedures will enable sharing of pre-provisioned zone information and updating the resource information, notifying partners if there are new zones available etc.

When the partner OP accepts the create federation request from originating OP, the partner OP also provides the availability zones meta information which it can offer to originating OP and their application providers.

This request can be sent only after successful creation of the federation relationship between the originating and partner OPs. The API initiator confirms one or more zones from the list of zones being offered by the partner OP to originating OP. The ISVs of the originating OP can deploy their applications on the zone(s) being confirmed by this API.

#### Zone registration

On receiving the availability zones meta information (e.g., zone id, serving location etc.), the originating OP may send an HTTP POST message that contain the accepted availability zone(s) registration request for one or more availability zones offered by partner OP.

The partner OP may reserve the resources for the originating OP on indicated zone(s) and provides the details of resources configuration and quantity in the availability zones to the originating OP in the response to zone registration request.

: Availability Zone Registration

#### View Zone information

Originating OP at moment of time can query the partner OP for the availability zone(s) status information (e.g., resource availability, serving location etc.). The originating OP may send an HTTP GET request that contain the accepted availability zone(s) identifier for one or more availability zones offered by partner OP.

: View Availability zone information

### Procedures for registration and authorization of end users in a federated OP partner

As defined in the PRD when moving to a visited network, the end user shall firstly contact the home network OP platform. In case the visited network is a federated partner and that local break out is available the end user is redirected to the visited OP platform.

#### Authorization of end users by federated OP

The visited OP platform needs to authenticate and authorize the service to the end users it can access edge nodes available in the visited network. This model is preferred because the edge cloud service is provided closer to the User Client. As described in the PRD, the Home OP is involved managing the subscriber’s authentication and authorization. The following figure is intended to describe the interactions between OP partners to validate and authenticate end users.

- The UE A registers to OP-A (Home Domain).

  - Authentication/Authorization procedures in the home network

  - OP-A retrieve UE location information

- The OP-A steers the user to OP-B based on the user location and considering that both operators have agreed that LBO can be used. Information regarding UE access to OP-B must be included in the redirection message (e.g., IP address, FQDN). One approach from 3gpp is to have provisioned the URSP tables with the appropriate rules to reach the partner OP based on the provided IP address

- After receiving OP-B access information the UE-A proceed to register in the visited OP

- These steps represent the federation connection for enabling the application availability on Operator B by sharing and validating user’s authorization information (HTTP POST).

- In case of failure, in the response message should be reflected the cause, so it can be notified to UE-A

- Finally, UE-A gets authorized in OP-B and can request access to edge services provided based on the user’s location.

- In case of failure the corresponding status message must be provided showing the cause.

: User Client registration on access from a visited OP

## Application Services Procedures

The OP services as defined in OP PRD can be provided to the application providers via NBI to manage, deploy and monitor applications with leading OP and with its federated partner OPs.

This section provides the coverage to the edge services which requires E/WBI APIs to enable the OP services with federated partner OP based on NBI operations invoked by the application providers.

### Edge Service Procedures

The following section describes the OP supported edge service procedures over E/WBI to provide application providers access to federated partners OP services.

#### Procedures for Application Artifacts Management Service

According to section 3.5.4.3 of the PRD OPG.02 an OP shall be capable of onboarding and manage application artifacts towards an OP partner, considering that federation establishment between partners have been set up previously.

Following procedures needs to be supported:

- Transfer application images (container or VMs per section 3.6 and 3.7 of the PRD)

##### Application Artifacts Upload

This is intended for an OP to upload application images e.g., docker container image file(s) and associated application component descriptors i.e., artifacts e.g., Helm charts, Terraform scripts etc. to a partner OP.

Same artifacts can be reused by multiple applications within an application provider account and for this purpose the artifacts are delivered to those partner availability zones which are linked to the applications.

For this operation message flows should be as follow:

- An artifact upload request i.e., HTTP POST message with the application artifacts provided by the Application provider over NBI from originating OP is sent to a partner OP.

- The partner OP authenticates the originating OP and validate the requested operation and the parameters e.g., country code, federation keys and the indicated zone(s) status

- Once the artifact push is finish

  - If the procedure is completed successfully, a response message HTTP POST response with “202: Artifact Accepted” shall be sent from the partner OP. (onboarded artifact information can be included as well).

  - Originating OP may send GET request at later point of time to enquire about the actual upload status with partner OP

: Artifact upload request

##### Application Artifacts Update

This is used by an OP to update the already submitted artifacts e.g., docker container image file(s) and scripts to a partner OP. Artifacts are onboarded by the leading OP and stored in local repositories. There artifacts are linked to the applications by the app provider and are delivered to partner OP based on the application zones indicated by the application provider.

After successfully completing the update procedure, the originating OP shall inform the Application provider of the of the availability of artifacts in various availability zones on NBI. The updated information e.g., application images, helm charts etc. shall be used by the partner OP when requested by the application provider for applications deployed in the partner OP footprint.

: Artifact update request

##### Application Artifacts Delete

An OP must support to delete the already submitted artifacts e.g., docker container image file(s), application components descriptor scripts e.g., Helm charts etc. to a partner OP.

OP may initiate the application artifact deletion process on receiving the request from application provider over NBI.

After successfully completing the delete procedure, the leading OP shall inform the Application provider of the outcome of the remove operation on NBI. The artifacts e.g., application images, helm charts etc. shall be removed by the partner OP.

Note: Aspects like audit of complete removal of artifacts any references of the same in any of the partner OP environment is beyond the scope of this document and is not covered here.

#### Application Provider Resource Management Service

According to section 5.2 of the Telco Edge Cloud whitepaper, the capacity reservation model is described as using a preselected combination of service units (computing, storage, networking) that is permanently allocated to the Customer. Usually chosen for longer time periods in which the Customer has a permanent demand to attend.

Following procedures needs to be supported:

- To reserve compute resources with partner OP based on the request from application provider on NBI

- To update or modify the already reserved resource pool e.g., to add or remove resources in existing reservation

- Delete already reserved resource pool created for an application provider

##### Resource Reservation

This is intended for an OP to reserve resources for an application provider e.g., compute resource flavors when the application provider initiate the reservation using NBI.

The application provider shall be able to request reservation of resources with partner OP on per availability zone basis. The partner OP shall be able to reserve resources for a given application provider from the allocated quota for the leading OP. The partner OP may have allocated the pool of resources for the leading OP during the availability zone registration process. Once the resource reservation request is approved by the partner OP, a resource pool identifier is provided to the leading OP to refer to specific resource pool for application provider. Application provider can use the identifier while instantiating the application to indicate which resource are to be used when deploying applications in partner OP zones.

For this operation message flows should be as follow:

- A resource reservation request i.e., HTTP POST message containing the resources to be reserved along with the availability zone as provided by the Application provider over NBI is sent to the partner OP by the originating OP.

- The partner OP authenticates the originating OP and validate the requested operation and the parameters e.g., federation keys, app provider identifier, resource identifiers and the indicated zone.

- Once the request is validated

  - If the procedure is completed successfully, a response message HTTP POST response with “200: Resource reservation request accepted ” shall be sent from the partner OP.

  - Originating OP may send GET request at later point of time to retrieve the reservation details with partner OP

: Resource Reservation request

##### View Resource Reservation

This is used by the leading OP to retrieve the status of the already created resource pool with a partner OP when requested by the application provider over NBI. Leading OP uses the HTTP GET method to fetch the details of the resource pool as indicated by the application provider in a given availability zone.

After receiving the result for the GET request from the partner OP, the leading OP shall provide the information received in GET response to the Application provider on NBI.

: View Resource Reservation request

##### Update Resource Reservation

This is used by an OP when requested by the application provider via the NBI to update the existing resource reservation to a partner OP. The leading OP provides the application provider Identifier, availability zone and operation to be performed e.g., add or remove the resources from a given resource pool etc.

After receiving the response from the partner OP of the outcome of the update request, the leading OP shall inform the Application provider of the of the outcome of the update request on NBI.

The leading OP uses the HTTP PATCH method to inform the partner OP about the application provider, zone and resources to be updated.

: Resource reservation update request

##### Delete Resource Reservation

Leading OP shall support the delete procedure to remove existing resource reservation with partner OP when requested by the application provider over NBI.

OP uses the HTTP DELETE method to inform the partner OP of the application provider identifier, availability zone and existing reservation identifier earlier generated by the partner OP during the creation of reservation.

After receiving the delete procedure response from the partner OP, the leading OP shall inform the Application provider of the outcome of the remove operation on NBI.

#### Procedures for Application Onboarding Management Service

According to section 3.5.4.3 of the PRD OPG.02 an OP shall be capable to onboard and manage applications towards an OP partner, considering that federation establishment between partners have been set up previously.

Following procedures needs to be supported:

- Transfer Application Provider Criteria towards a partner OP. The procedure may also request the launch of application instance(s) in partner OP edge clouds as a follow-up action after onboarding.

- Transfer of other application-specific files, e.g., application manifest, specifying the workload information like mobility strategy, QoS and privacy policies etc., and other optional characteristics indicating the application's needs (latency, prioritization, reservation)

- Removal of applications (application images and metadata.

##### Application Organization

An application is logical group of related components that can be managed as a single unit by the OP. An application is collection of runnable components and associated properties which together form an application. Application components descriptors e.g., Helm Charts, Container Specs, Terraform scripts etc. are provided by the application providers along with other application characteristics e.g., QoS profile, availability zone info with leading and federated OP, resource requirements etc. which may be considered for application orchestration decisions by the OP.

Application component descriptors as runnable units of an application require references to component images which can be from public repositories, private repositories or may also be provided by the application providers to the OP by using OP supported image management capabilities. Leading OP on behalf of application provider is responsible to transfer applications and corresponding component descriptors and images to the partner OP over E/WBI.

: Application Schema

##### Onboard Application

An application provider uses the OP NBI to manage edge applications via the application management capabilities provided by the leading OP. Using these capabilities an application provider can also request leading OP to share and deploy applications in the federated partner availability zones.

These events on NBI may also result into the leading OP to initiate application management procedures towards the partner OP(s) over E/WBI and share the application images, artifacts, and other meta information as provided by the app provider over NBI.

An OP can use app onboarding APIs to submit an application to a partner OP availability zone(s). Submitting applications may include application images, application type, Application Provider criteria, target availability zones etc. towards a Partner OP.

- An onboard application request is sent to a partner OP.

  - HTTP POST message contains application details e.g., app name, app identifier, app provider identifier, availability zone(s), QoS profile etc.

- The partner OP validates the OP identity and authorization info, federation keys and zone onboarding status

  - If application is already onboarded or is ongoing a correspondent failure response will be sent.

  - Otherwise, after OP validation the partner OP proceed to push application data (container images) to the edges and update/store all the info related to a database.

- Once the application push is finish

  - If the procedure is completed successfully, a response message HTTP POST response with “201: Application onboarded” shall be sent from the partner OP. (appId and requestId information can be included as well).

  - In other case a correspondent failure message will be generated from the partner OP.

Note: This version of the document covers the resource model for application where the applications get resources as they need them. Coverage for resource reservation model will be provided in the next releases of this document.

: Onboard Application

##### Update Application Information

An OP must have alternatives to update parameters of an application onboarded on a partner OP. Update application information towards a Partner OP (e.g., application versions, Application Provider criteria, target availability zones).

Application update request can be initiated by the leading OP due to the application provider initiated application update action over NBI.

- Leading OP shall send the HTTP PATCH request message to the partner OP(s) to start the application update procedure.

  - HTTP PATCH message may contain application details e.g., app name, app identifier, app provider identifier, availability zone(s), QoS profile etc.

- The partner OP validates the OP identity and authorization info, federation keys etc.

- On successful validation, if application indicated by app identifier is already onboarded and parameters to be updated are valid then

  - If the update procedure is completed successfully, a HTTP PATCH response with “201: Application updated successfully” shall be sent from the partner OP. (app name and app Identifier information can be included as well).

  - In other case a correspondent failure message will be generated by the partner OP.

: Update Onboarded Application

Note: Leading OP shall make the application update results available on the NBI interface as application provider may have started the update procedure over NBI. It is to be noted that the application information update does not results in updating the existing instances of the application or create new instances with updated information.

Note: After successful update of the application information, the application provider can request to instantiate the application instances with updated information on one or more availability zones used earlier during the onboarding procedure.

##### Remove Application

This will be use by an OP to remove an application from a partner OP zone. Removal of applications (application images and metadata) from a Partner OP. Leading OP shall make the application de-boarding result available on the NBI interface.

After successful de-boarding of the application, the application and any of the associated information e.g., images, metadata etc. shall no longer be available in the indicated availability zones.

Note: Verification and compliance of the removal of application information by an OP is beyond the scope of this document and such requirements and verification process shall be part of other specifications e.g., OP PRD.

: Delete Application

#### Application Deployment Management Service

As defined in OP PRD, the Application Deployment Management Service on E/WBI shall control the launch and termination of applications that have been onboarded on a partner OP.

##### Instantiate Application

This will be use by an OP to instantiate an application to edge clouds of leading OP and to a partner OP zone(s) as requested by application provider over NBI.

Leading OP shall make the application instantiation result available on the NBI interface. Partner OP shall also provide the application instance status over E/WBI to leading OP which leading OP may expose to application providers on NBI.

: Application Instantiation

##### Notify Application Instance Information

After successful instantiation of the application, the application provider should be able to view the application instance information on partner availability zone(s) e.g., app Identifier, instance identifier, health status, network interfaces communication endpoints etc.

The application instantiation request may take time for partner OP to create the application instance on the indicated availability zone(s). Based on the result of the instantiation the partner OP sends the notification request (HTTP POST) to the leading OP with the application instance information e.g., application instance identifier, application identifier, zone meta-info, application instance endpoints etc.

: Application Instance Status Notification

##### Terminate Application Instance Information

After successful instantiation of the application, the application provider should be able to request the termination of application instance on one or more availability zone of leading and/or partner OP.

Application providers request the application instance termination via NBI, and the leading OP shall initiate HTTP DELETE request towards the partner OP. The request may contain the information e.g., application identifier, instance identifier etc. to enable partner OP to identify the application instance uniquely on his edge clouds.

: Application Instance Termination

#### Procedures for network event support

As described in section 3.5.1.7 of the PRD, an application provider may require to be notified about network events or may want to request specific information about UE, network status or information.

Requesting network event information is an extrapolation of the access to NWDAF through NEF to retrieve analytics exposure information from the network and delivered to an AF, in this case the OP. Therefore, current API procedures for analytics information exposure and associated API definition from 3GPP can be re used (summarize in section 3.6).

All procedures described in 3GPP TS 29.522 section 4.4.14: “Procedures for analytics information exposure” must be supported. These procedures are used by the AF to subscribe/unsubscribe to retrieve analytics information via NEF and are used by the NEF to notify the AF about the requested analytics information as described in 3GPP TS 23.288.

The references mentioned above only cover the scenarios depicted in the figure x.

: Retrieving network status and analytics information

##### Requesting Network events information

Scenarios and procedures where an OP can request the same type of network analytics information from a partner OP with a federation relationship are not covered in the 3GPP specification (which would be equivalent to an AF-to-AF interaction), see figure below.

: Federated scenario for network event information

This target the capability of the OP to access network information such as performance available in the federated network when Local break out is applied. The information that is missing in the 3GPP references and need to be considered are steps 1 and 4.

Considering that these OP partners are already federated, in the step 1, the OP- A should be capable to send an HTTP request to OP-B containing the type of network analytics information of interest. The information in the request would be used to trigger NEF APIs in the OP-B Network and retrieve the appropriate information, so that OP-B could send an HTTP response to OP-A containing the requested analytics. In addition to the type of network analytics, in the HTTP request must be included federation related information generated previously in the partnership establishment between OPs.

#### Procedures for sharing edge resources between federated OP partners

As defined in the section 3.3.5 of the PRD OPG, Edge node sharing is the concept for two operators to share edge nodes(should be read as compute resources in partner OP availability zones) between their coverage area for example from a geographical point of view (south and north).

##### Edge node discovery procedure with partner OP

A subscriber of Operator A accesses its home network/operator platform and requests for the required Edge-Enhanced or Edge-Native Application instantiation. When Operator A's OP identifies that the most suitable edge resources are in Partner B, Operator A's OP requests by an HTTP POST message over the E/WBI to Partner B's OP (see Figure below, steps 3, 3a and 3b) to provide the suitable availability zone(s) where application can be hosted in partner OP edge clouds. Alternatively, partner OP can also provide the communication endpoints of existing application instances to home OP.

In this example, since the two OPs have a federation agreement, they may have pre-established commercial agreements, security relationships and policy decisions (for instance, QoS-related). Operator B’s OP sends the response for the HTTP POST request to OP-A (assuming enough edge resources are available at OP-B). The OP-B response contains the application endpoint (e.g., FQDN) on the Cloudlet of OP-B at which the subscriber can connect to the application.

: Sharing edge resources between federated OP partners

Based on the OP-B response to edge node(compute resources in partner OP availability zones) discovery request, the OP A sends an Edge discovery response to the UE, which includes information about the discovered application endpoint (e.g., FQDN) from OP B.

If the OP-A includes the Edge node discovery filters in Edge discovery request, the OP-B response may include additional information regarding matched capabilities, e.g., service permissions levels, KPIs, Edge application locations(s) that the Edge node can support.

The Edge discovery response from OP-B may contain a list of Edge Node endpoints. This list may be based on Edge discovery filters containing a Geographical or Topological Service Area, e.g., Lat/Long of the UCs, Application Identifier etc. In case of failure, OP B should send an appropriate failure response including the cause to leading OP.

# OP East/West Bound APIs

This section provides the information on various APIs and associated parameters and data models for the procedures mentioned in the previous sections.

## Generic East/West Bound Service APIs

The interface management APIs provides the capabilities to perform the handshake between the two operator platforms and share the availability zone(s) and resource information with partner OPs.

### East/West Bound Interface Management - API

The interface management APIs provides the capabilities to perform the handshake between the two operator platforms and share the availability zone(s) and resource information with partner OPs.

The following subclauses specify the resource methods supported by the resource as described in below section.

#### Introduction

Following table describe the HTTP Methods for the federation resource.

| Operation | HTTP Methods | Resource URI | Qualifier |
| --- | --- | --- | --- |
| Create Federation | POST | /operatorplatform/federation/v1/ partner | M |
| Update Federation | POST | {federationNotificationDest} | M |
| Remove Federation | DELETE | /operatorplatform/federation/v1/partner/{federationContextId} | M |
| Get Federation Meta Info | GET | /operatorplatform/federation/v1/partner/{federationContextId}/federationinfo | M |

: E/WBI Interface Management APIs

#### Create Federation : POST Method

The POST method creates a new federation relationship resource for a given OP.

This method shall support the request data structures, response data structures and response codes as specified in data model section.

The table below describes the data structures supported by the POST Request Body on this resource.

| Parameter Name | P | Cardinality | Description |
| --- | --- | --- | --- |
| origOPFederationId | M | 1 | Operators in federation shall be governing the namespace and operator identifier assigned to it. NEED INPUTS from operators? |
| origOPCountryCode | C | 1 | Mobile Country code of the originating OP (i.e., the OP sending the federation create request). |
| origOPMobileNetworkCodes | C | 1..N | List of mobile network codes where an operator may have one or more network codes assigned |
| origOPFixedNetworkCodes | C | 1..N | Need the identifiers to refer to fixed network operators |
| initialDate | M | 1 | Date and time, time zone info of the federation initiated by the originating OP |
| federationNotificationDest | M | 1 | Contains the API endpoint to receive the notifications from the partner OP.for any updates done by partner OP on this federation |

: Request Parameter for Create Federation Request

The tables below describe the data structures supported by the POST Response Body on this resource for 200 OK.

| Parameter Name | P | Cardinality | Description |
| --- | --- | --- | --- |
| partnerOPFederationId | M | 1 | Operators in federation shall be governing the namespace and operator identifier assigned to it. NEED INPUTS from operators? |
| partnerOPCountryCode | M | 1 | Mobile Country Code of operator sending the response. |
| partnerOPMobileNetworkCodes | C | 1..N | Mobile Network Codes of operator sending the response to federation create request. |
| origOPFixedNetworkCodes | C | 0..N | Fixed line network identifier [Yet to be finalized] |
| federationContextId | M | 1 | This identifier shall be provided by the partner OP on successful verification and validation of the federation create request. The identifier is the indicator of a successful federation establishment between the two OP. This identifier shall be used in subsequent requests by originating OP |
| edgeDiscoveyServiceEndPoint | M | 1 | IP and Port of Edge Discovery Service URL of partner OP. This can also be a FQDN |
| offeredAvailabilityZones | O | 0..N | List of zones partner OP is willing to share. Partner may configure such information using system management interface |
| platformCaps | M | 1 | List of extended capabilities HomeRouting, Anchoring as supported by partner OP. |
| NOTE: partnerOPAvailabilityZones is a data type which has the following attributes: zoneId, geolocation, city, state, locality, edgeCount. |  |  |  |

: Response Parameter for Create Federation Request

The tables below describe the header supported by the POST Response Body on this resource.

| Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | Contains the URI of the newly created resource i.e., /operatorplatform/federation/v1/partner/{federationContextId} |

: Header parameter for Create Federation Response

The tables below describe the data structures supported by the POST Response Body on this resource for non-200 response codes.

| Parameter Name | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| problemDetails | C | 1 | 400 | Bad Request. Parameters in the request has conflicting values. |
| problemDetails | C | 1 | 404 | Content Not Found |
| problemDetails | C | 1 | 409 | Conflict. Federation already exists or state mismatch |
| problemDetails | C | 1 | 422 | Unprocessable Entity. Mandatory parameters are not sent in the request. |
| problemDetails | C | 1 | 500 | Internal Server Error |
| problemDetails | C | 1 | 503 | Service Unavailable. |
| problemDetails | C | 1 | 520 | Web Server Returned an Unknown Error |

: Failure Responses for Create Federation Request

#### POST Method : Update Federation

POST HTTP method is used by the partner OP towards the originating OP to update the parameters associated to the existing federation. Partner OP sends update request on the URI defined by parameter ‘federationNotificationDest ‘. The table below describes the POST request body for updating existing federation.

| Parameter Name | P | Cardinality | Description |  |
| --- | --- | --- | --- | --- |
| federationContextIdentifier | M | 1 | This identifier shall be provided by the originating OP to partner OP to identify the existing federation relationship. |
| objectType | M | 1 | Refers to the resource being modified for e.g., Federation status, zone status, edge discovery URL, network codes etc. |
| operationType | M | 1 | Type of update for e.g., Change in status, add network code, update edge discovery URL etc. |
| modificationDate | M | 1 | Date and time of the federation modification by the partner OP |
| edgeDiscoverySvcEndPoint | O | 1 | Edge discovery service URL for UNI interface. |
| lcmSvcEndPoint | O | 1 | LCM service URL for UNI interface |
| addMobileNetworkIds | O | 1..N | List of MNCs to be added |
| removeMobileNetworkIds | O | 1..N | List of MNCs to be removed |
| addFixedNetworkIds | O | 1..N | List of fixed network codes to be added |
| removeFixedNetworkIds | O | 1..N | List of fixed network codes to be removed |
| addZones | O | 1..N | New zones to be added. List of ‘availabilityZone’. |
| removeZones | O | 1..N | List of zoneIds to be removed |
| zoneStatus | O | 1..N | Availability status of zones |

: Update federation request parameters

The tables below describe the data structures supported by the POST Response Body on this resource.

| Parameter Name | P | Response Codes | Description |
| --- | --- | --- | --- |
| N/A | C | 200 | Completion status of the PATCH request handling procedure at originating OP |
| problemDetails | C | 400 | Bad Request. Parameters in the request has conflicting values, content have semantic error. |
| problemDetails | C | 404 | Content Not Found |
| problemDetails | C | 409 | Conflict. Federation does not exist |
| problemDetails | C | 422 | Unprocessable Entity. Mandatory parameters are not sent in the request. |
| problemDetails | C | 500 | Internal Server Error |
| problemDetails | C | 503 | Service Unavailable. |
| problemDetails | C | 520 | Web Server Returned an Unknown Error |

: Update Federation response parameters

#### DELETE Method : Remove Federation Relationship

DELETE method supports the query parameters.

| Parameter Name | P | Cardinality | Description |
| --- | --- | --- | --- |
| federationContextIdentifier | M | 1 | This identifier shall be provided by the originating OP to partner OP to identify the existing federation relationship. |

: Remove Federation request parameters

The tables below describe the data structures supported by the DELETE Response Body on this resource.

| Parameter Name | P | Response Codes | Description |
| --- | --- | --- | --- |
| status | C | 200 | Federation removed successfully |
| problemDetails | C | 400 | Bad Request. |
| problemDetails | C | 404 | Content Not Found |
| problemDetails | C | 409 | Conflict. Federation already being terminated |
| problemDetails | C | 422 | Unprocessable Entity. Mandatory parameters are not sent in the request. |
| problemDetails | C | 500 | Internal Server Error |
| problemDetails | C | 503 | Service Unavailable. |
| problemDetails | C | 520 | 520 Web Server Returned an Unknown Error |

: Remove Federation response parameters

#### GET Method : Get Federation Meta Information

GET method supports the path parameters.

| Parameter Name | P | Cardinality | Description |
| --- | --- | --- | --- |
| federationContextIdentifier | M | 1 | This identifier shall be provided by the Originating OP to Partner OP to identify the existing federation relationship. |

: Zone meta info request parameters

The tables below describe the data structures supported by the GET Response Body on this resource for response code 200 OK.

| Parameter Name | P | Cardinality | Description |
| --- | --- | --- | --- |
| edgeDiscoveryServiceEndPoint | M | 1 | IP and Port of Edge Discovery Service URL of partner OP. This can also be a FQDN. E.g., “discovery.operator1.com” or IPv4Addr:Port (in dotted decimal notation). |
| offeredAvailabilityZones | O | 0..N | List of zones partner OP is willing to share. Partner may configure such information using system management interface |
| allowedMobileNetworkIds | O | 1..N | List of mobile network codes where an operator may have one or more network codes assigned |
| allowedFixedNetworkIds | O | 1..N | List of Fixed network codes |
| lcmServiceEndPoint | M | 1 | IP and Port of LCM Service URL of partner OP. This can also be a FQDN. |

: Federation meta info response parameters

The tables below describe the HTTP codes supported by the GET Response on this resource.

| Parameter Name | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| Status | C | 1 | 200 | Federation meta-info request accepted |
| N/A | C | 1 | 400 | Bad Request. Parameters in the request has conflicting values. |
| N/A | C | 1 | 404 | Content Not Found |
| N/A | C | 1 | 409 | Conflict. |
| N/A | C | 1 | 422 | Unprocessable Entity. Mandatory parameters are not sent in the request. |
| N/A | C | 1 | 500 | Internal Server Error |
| N/A | C | 1 | 503 | Service Unavailable. |
| Error Message | C | 1 | 520 | 520 Web Server Returned an Unknown Error |

: Response codes for zone meta-info Request

#### Data Model

##### General

This subclause specifies the application data model supported by the E/WBI interface management API.

| Data Type | Clause Defined | Description |
| --- | --- | --- |
| federationContextId | 3.1.2.3.1 | Federation relationship Identifier generated by the partner OP |
| partnerOPFederationId | 3.1.2.3.1 | Unique public identifier for the partner OP |
| partnerOPCountryCode | 3.1.2.3.1 | Mobile Country Code (MCC) of the partner OP |
| partnerOPNetworkCodes | 3.1.2.3.1 | Mobile Network Codes (MNCs) of the partner OP |
| partnerOPFixedNetworkCodes | 3.1.2.3.1 | Fixed Network Codes of the partner OP |
| origOPFederationId | 3.1.2.3.1 | Unique public identifier for the originating OP |
| origOPCountryCode | 3.1.2.3.1 | Mobile Country Code (MCC) of the originating OP |
| origOPNetworkCodes | 3.1.2.3.1 | Mobile Network Codes (MNCs) of the originating OP |
| origOPFixedNetworkCodes | 3.1.2.3.1 | Fixed Network Codes of the originating OP |
| offeredAvailabilityZones | 3.1.2.2.1 | List of zones partner OP offers to share with originating OP |
| edgeDiscoveyServiceEndPoint | 3.1.2.2.3 | IP and Port of Edge Discovery Service URL of partner OP |
| updateType | 3.1.2.3.1 | Indicates which Parameter being updated by partner OP for existing federation |
| mncChangeInfo | 3.1.2.3.3 | Structure for add or remove mobile network code(s) |

: 5.1	East/West Bound Interface Management Params

##### Structured Data Types

This clause defines the structured data types to be used in resource representations.

###### offeredAvailabilityZones

Following table describe the mapping of IS operations to SS equivalents.

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| offeredAvailabilityZones | Array(availabilityZone) | M | 1..N | List of availability zone Ids |

: Availability Zones meta information

###### availabilityZone

Following table describe the data elements of an availability zone.

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| zoneId | String | M | 1 | Unique Identity of a Zone |
| geolocationInfo | String | M | 1 | Lat/Long of Zone |
| geographyDetails | String | O | 1 | Details about cities or state covered by the edge. Details about the type of locality for e.g., rural, urban, industrial etc. This information is defined in human readable form. |

: Availability Zone location parameters

###### edgeDiscoveryServiceEndPoint

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| serviceURL | String | M | 1 | FQDN or Public IP Address of the Edge Discovery service |
| Port | Int | M | 1 | Port number of the Edge Discovery service where UCs can send requests to over UNI |

: Edge Discovery Service Endpoint

###### mncChangeInfo

Following table describe the network code update structure to notify change in supported network codes by partner OP.

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| operationType | Enum | M | 1 | Whether the network code being added or removed |
| networkCodes | Array(String) | M | 1..N | The list of network codes being added or removed |

: Availability Zones meta information

###### ProblemDetails

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| title | String | M | 1 | A short, human-readable summary of the problem type. It should not change from occurrence to occurrence of the problem. |
| detail | String | O | 0..1 | A human-readable explanation specific to this occurrence of the problem. |
| cause | String | O | 0..1 | A machine-readable application error cause specific to this occurrence of the problem This IE should be present and provide application-related error information, if available. |
| invalidParams | array(InvalidParam) | O | 1..N | Description of invalid parameters, for a request rejected due to invalid parameters. |

: Response body for error responses

###### InvalidParam

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| param | String | M | 1 | Parameter name |
| reason | String | O | 0..1 | A human-readable reason |

: InvalidParam

###### zoneStatus

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| zoneId | String | M | 1 | Zone Identifier |
| status | String | M | 1 | Availability Status for the zone. |

: InvalidParam

##### Simple data types and enumerations

This subclause defines simple data types and enumerations that can be referenced from data structures defined in the previous subclauses.

###### Simple data types

| Attribute Name | Data Type | Description |
| --- | --- | --- |
| federationContextIdentifier | String | Federation relationship Identifier generated by the partner OP |
| initialDate | String | date/time value as a string in ISO 8601 format., “ 2018-12-10T13:45:00.000Z” |
| partnerOPFederationId | String | Unique public identifier for the partner OP |
| partnerOPCountryCode | String | Mobile Country Code (MCC) of the partner OP |
| partnerOPMobileNetworkCodes | Array(String) | Mobile Network Codes (MNCs) of the partner OP |
| partnerOPFixedNetworkCodes | Array(String) | Fixed Network Codes of the partner OP |
| origOPFederationId | String | Unique public identifier for the originating OP |
| origOPCountryCode | String | Mobile Country Code (MCC) of the originating OP |
| origOPMobileNetworkCodes | Array(String) | Mobile Network Codes (MNCs) of the originating OP. MNCs are 2- or 3-digits codes with each digit is from the set {0,9} |
| origOPFixedNetworkCodes | Array(String) | Fixed Network Codes of the originating OP |
| zoneId | String | Identifier for a zone |

: E/WBI Interface Management Simple Datatype table

###### Enumeration: objectType

The enumeration updateType represents the attribute being updated by the partner OP on existing federation.

| Enumeration value | Description |
| --- | --- |
| FEDERATION | Change in status of federation relationship |
| ZONES | Change in the availability status of a zone |
| EDGE_DISCOVERY_SERVICE | Edge discovery service endpoints are modified |
| LCM_SERVICE | LCM service endpoints are modified |
| MOBILE_NETWORK_CODES | Addition or removal of mobile network code |
| FIXED_NETWORK_CODES | Addition or removal of fixed network codes |

: Federation Modification Parameter types

###### Enumeration: operationType

The enumeration operationType represents the if the network codes are being added or removed by the partner OP on existing federation.

| Enumeration value | Description |
| --- | --- |
| STATUS_CHANGE | Availability status of the resource has changed |
| UPDATE_SERVICE | Update endpoint of a service |
| ADD_CODES | Add network (mobile or fixed) network codes |
| REMOVE_CODES | Remove network (mobile or fixed) network codes |
| UPDATE_CODES | Update network (mobile or fixed) network codes. Some new codes are added, and some old codes are removed. |
| ADD_ZONES | Add new zones. Partner OP is willing to share new zones |
| REMOVE_ZONES | Remove old zones. Partner OP is no longer willing to share some previous zones |
| UPDATE_ZONES | Add some new zones and remove some old zones. |
| UPDATE_ZONE_STATUS | Change in the availability status of a zone |

: Operations type for network code change

###### Enumeration: Status

| Enumeration value | Description |
| --- | --- |
| FAILED | Resource is in failure state |
| TEMPORARY_FAILURE | Temporary failure for the resource |
| AVAILABLE | Resource is available |
| LOCKED | Resource is locked and is no longer accessible |

: Allowed status values

### Availability Zone Information Synchronization Service – API

The APIs for Availability Zone Information Synchronisation Service are used to share and update specific information on the Availability Zone corresponding to an OP's Edge Cloud resources provided to another.

#### Introduction

Following table describe the APIs for availability zone resources synchronization services.

| Operations | HTTP Method | Resource URI | Qualifier |
| --- | --- | --- | --- |
| Zone Registration | POST | /operatorplatform/federation/v1/zones/{federationContextId} | M |
| Zone Deregistration | DELETE | /operatorplatform/federation/v1/zones/{federationContextId}/{zoneId} | M |
| View Zone Information | GET | /operatorplatform/federation/v1/zones/{federationContextId}/{zoneId} | M |
| Notify Zone Information | POST | { availZoneNotifLink} | M |

: Availability Zone Synchronization APIs

#### Zone Registration : POST Method

The availability zone registration POST request contains the following parameters towards the partner OP.

| Parameter Name | P | Cardinality | Description |
| --- | --- | --- | --- |
| federationContextIdentifier | M | 1 | This identifier shall be provided by the originating OP to partner OP to identify the existing federation relationship. |
| acceptedAvailabilityZones | M | 1..N | Accepted list of one or more availability zones selected from the offered list of zones by partner OP which originating OP intends to use. |
| availZoneNotifLink | M | 1 | An availability zone info notification URL which shall be used by the partner OP to inform the about any changes to zone information e.g., resource quota updates, addition of new zones etc. asynchronously |

: Availability Zones registration request parameters

The tables below describe the data structures supported by the POST Response Body on this resource.

| Parameter Name | P | Cardinality | Description |
| --- | --- | --- | --- |
| acceptedZoneResourceInfo | M | 1 | Available Zone Resource information provided by partner OP for accepted zone IDs by originating OP. It includes zoneId, guaranteed Resources and upper Limit Quota (E.g., vCPU, Memory, Storage, GPU etc.) |

: Availability zones registration response parameters

The tables below describe the data structures supported by the POST Response Body on this resource.

| Parameter Name | P | Response Codes | Description |
| --- | --- | --- | --- |
| Status | C | 200 | Zone registration request accepted |
| problemDetails | C | 400 | Bad Request. |
| problemDetails | C | 404 | Content Not Found |
| problemDetails | C | 422 | Unprocessable Entity. Mandatory parameters are not sent in the request. |
| problemDetails | C | 500 | Internal Server Error |
| problemDetails | C | 503 | Service Unavailable. |
| problemDetails | C | 520 | Web Server Returned an Unknown Error |

: Availability Zones registration response parameters

#### Zone De-registration : DELETE Method

Table provides parameters which an originating OP sends to partner OP in zone de-registration request to relinquish availability zone(s) and associated resources for indicated availability zones which the originating OP may have been using in partner OP footprint.

| Parameter Name | P | Cardinality | Description |
| --- | --- | --- | --- |
| federationContextId | M | 1 | This identifier shall be provided by the originating OP to partner OP to identify the existing federation relationship. |
| zoneId | M | 1 | Zone identifier of partner operator. The partner OP shall deregister the indicated zone and may reclaim the resources. |

: Availability Zones de-registration request parameters

The tables below describe the data structures supported by the DELETE Response Body on this resource.

| Parameter Name | P | Response Codes | Description |
| --- | --- | --- | --- |
| Status | C | 200 | Zone de-registration request accepted |
| problemDetails | C | 400 | Bad Request. |
| problemDetails | C | 404 | Content Not Found |
| problemDetails | C | 422 | Unprocessable Entity. Mandatory parameters are not sent in the request. |
| problemDetails | C | 500 | Internal Server Error |
| problemDetails | C | 503 | Service Unavailable. |
| problemDetails | C | 520 | Web Server Returned an Unknown Error |

: Availability Zones de-registration response parameters

#### View Zone Information : GET Method

Table below provides parameters which an originating OP sends to partner OP in view zone information request for indicated availability zones which the originating OP may have been already using.

| Parameter Name | P | Cardinality | Description |
| --- | --- | --- | --- |
| federationContextIdentifier | M | 1 | This identifier shall be provided by the originating OP to partner OP to identify the existing federation relationship. |
| zoneId | M | 1 | Zone identifier of partner operator availability zone. The partner OP shall deregister the indicated zone. |

: Availability Zones information request parameters

The tables below describe the data structures supported by the GET Response Body on this resource for HTTP 200 response.

| Parameter Name | P | Response Codes | Description |
| --- | --- | --- | --- |
| acceptedZoneResourceInfo | C | 200 | Available Zone Resource information provided by partner OP for accepted zone IDs by originating OP. It includes zoneId, guaranteed Resources and upper Limit Quota (E.g., vCPU, Memory, Storage, GPU etc.) |
| N/A | C | 401 | Authorization information is missing or invalid |
| N/A | C | 404 | Availability Zone Not Found |
| N/A | C | 422 | Unprocessable Entity. Mandatory parameters are not sent in the request. |
| N/A | C | 500 | Internal Server Error |
| N/A | C | 503 | Service Unavailable. |

: Availability Zones information response parameters

#### Notify Zone Information : POST Method

The availability zone notifies POST request sent by partner OP contains the following parameters towards the originating OP sent over a different HTTP session on the notification URL to provide update to existing resources or zone information. This can further be periodically sent to update the availability of resources.

| Parameter Name | P | Cardinality | Description |
| --- | --- | --- | --- |
| federationContextIdentifier | M | 1 | This identifier shall be provided by the originating OP to partner OP to identify the existing federation relationship. |
| zoneId | M | 1 | Identifier of the availability zone |
| zoneResUpdInfo | M | 1 | Available Zone Resource information provided by partner OP to originating OP. It may include zoneId, guaranteed Resources and upper Limit Quota (E.g., vCPU, Memory, Storage, GPU etc.) |

: Availability Zones notify request parameters

The tables below describe the data structures supported by the POST Response Body on this resource.

| Parameter Name | P | Response Codes | Description |
| --- | --- | --- | --- |
| N/A | C | 200 | Zone info notification acknowledged |
| problemDetails | C | 401 | Authorization information is missing or invalid |
| N/A | C | 422 | Unprocessable Entity. Mandatory parameters are not sent in the request. |
| N/A | C | 500 | Internal Server Error |
| N/A | C | 503 | Service Unavailable. |

: Availability zones async response parameters

#### Data Model

##### General

This section provides the data types for the Availability Zone and resource management.

| Parameter Name | Clause Defined | Description |
| --- | --- | --- |
| acceptedZoneResourceInfo |  | Available Zone Resource information provided by partner OP for accepted zone IDs to originating OP. It includes zoneId, guaranteed Resources and upper Limit Quota (E.g., vCPU, Memory, Storage, GPU etc.) |
| partnerAvailabilityZones |  | List of zones partner OP is willing to share. Partner may configure such information using system management interface |

: Data structures used in availability zones management services

##### Structured Data Types

This clause defines the structured data types to be used in resource representations.

###### acceptedZonesResourceInfo

Following table describe the mapping of IS operations to SS equivalents.

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| acceptedZoneResourceInfo | Array (zoneResourceInfo) | M | 1..N | Partner edge resource information available for applications consumptions |

: List of Availability Zones with offered resources

###### zoneResourceInfo

The zone resource information represents the computing resources which an OP can offer to the application providers of partner OP from a zone.

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| zoneId | String | M | 1 | Zone identifier to refer to a zone |
| reservedResources | Array (computeResourceInfo) | M | 1..N | Resources exclusively reserved for a partner OP |
| resourceQuotaLimits | Array (computeResourceInfo) | M | 1..N | Max quota on Resources that an OP may allow to partner OP over reserved resources if available |
| flavorsSupported | Array(computeFlavor) | M | 1..N | Compute resources flavors are set of OP defined compute resources combination which a partner OP supports and offers to application providers to be link them to applications for runtime resource requirements |
| networkResources | Array(networkResourceInfo) | O | 1..N | Type of networks supported by the partner zone |

: Availability Zones information data structure

###### computeResourceInfo

Compute resources indicates the resource profile applicable for a particular CPU architecture.

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| cpuArchType | Enum | M | 1 | CPU instruction set architecture (ISA). E.g., Intel, ARM etc. |
| numCPU | Integer | M | 1 | Total number of vCPUs |
| memory | Long | M | 1 | Total physical memory (RAM) for given ISA type (in Mbytes) |
| diskStorage | Long | M | 1 | Total storage (RAM) for workloads for given ISA type (in GB) |
| gpuInfo | Array(gpuResourceInfo) | O | 0..1 | Total GPU for workloads for given ISA type |
| FPGA | Int | O | 0..1 | Total storage (RAM) for workloads for given ISA type |
| vpu | Int | O | 0..1 | Total VPUs (Visual Processing Units) for workloads for given ISA type |
| hugepages | Array(haugepageInfo) | O | 1..N | Huge pages for workload for a given ISA type |
| cpuExclusivity | Boolean | O | 0..1 | Support for exclusive CPUs |

: Availability zone Compute resource information

###### gpuResourceInfo

GPU resources indicates the resource profile applicable for a particular CPU architecture.

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| gpuVendorType | Enum | M | 1 | GPU vendor name e.g., NVIDIA, AMD etc. |
| gpuModeName | String | M | 1 | Model name corresponding to vendorType may include info e.g., for NVIDIA, model name could be “Tesla M60”, “Tesla V100”, |
| gpuMemory | Int | M | 1 | GPU memory in GB |
| numGPU | Int | M | 1 | Number of GPU of a given model |

: GPU resources data model

###### computeFlavor

Compute flavors indicate templates associated to the computing capabilities of the application runtime environment in an OP edge clouds.

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| flavorId | Int | M | 1 | An identifier to refer to the combination of compute resource configuration as indicated by the other attributes in this table |
| cpuArchType | Enum | M | 1 | CPU Instruction Set Architecture (ISA) E.g., Intel, Arm etc. |
| supportedOSTypes | Array(operatingSystemInfo) | M | 1..N | A list of operating systems which a flavor configuration can support e.g., RHEL Linux, Ubuntu 18.04 LTS, MS Windows 2012 R2, macOS |
| numCPU | Int | M | 1 | Number of CPU for a given flavor |
| memorySize | Int | M | 1 | RAM size for a given flavor |
| storageSize | Int | M | 1 | Amount of disk space (GB) to use for the root (/) partition. |
| gpuInfo | Array(gpuResourceInfo) | O | 0..1 | Total GPU for workloads for given ISA type |
| vpuInfo | Integer | O | 0..1 | Number of Intel VPUs available |
| hugepages | Array(hugePageInfo) | O | 0..1 | Hugepages supported on the zone |
| cpuExclusivity | Boolean | O | 1 | If the zone supports exclusive allocation of Intel CPUs. |

: Compute flavor for Virtual Machines

###### operatingSystemInfo

The table below provides the information about the operating systems which may be supported by OP.

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| osAddrSize | Enum | M | 1 | Provides machine architecture e.g., x86_64, x86 |
| distroType | Enum | M | 1 | e.g., RHEL, Debian, Ubuntu etc. |
| versionInfo | Enum | M | 1 | Provides OS version information e.g., RHEL 8, Debian 11, Ubuntu 22.04 LTS etc. |
| licenseType | Enum | M | 1 | License type may include “on-Demand”, “Free” etc. |

: Operating system information

###### networkResourceInfo

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| egressBandWidth | Integer | M | 1 | Max dl throughput that this edge can offer. It is defined in Mbps. |
| dedicatedNIC | Integer | M | 1 | Number of network interface cards which can be dedicatedly assigned to application pods on isolated networks. This includes virtual as well physical NICs |
| supportSriov | Boolean | M | 1 | If the zone supports SRIOV based networking or not. |
| supportDPDK | Boolean | M | 1 | IF the zone supports DPDK enabled userspace networking or not. |

: Operating system information

###### hugePageInfo

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| pageSize | Enum | M | 1 | Size of hugepage |
| number | Integer | M | 1 | Total number of hugepages |

: GPU resources data model

###### zoneResUpdInfo

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| availableCompResources | Array (computeResourceInfo) | M | 1..N | Resources exclusively reserved for a partner OP |
| availableNetResources | Array(networkResourceInfo) | O | 1..N | Type of networks supported by the partner zone |

: Definition of zoneResUpdInfo

##### Simple data types and enumerations

This subclause defines simple data types and enumerations that can be referenced from data structures defined in the previous subclauses.

###### Enumeration: computeAccel

The enumeration computeAccel represents the hardware acceleration supported.

| Enumeration value | Description |
| --- | --- |
| HW_ACCEL_ GPU | GPU as accelerator |
| HW_ACCEL_ FPGA | FPGA as accelerator |

: Instruction Set Architecture types

###### Enumeration: cpuArchType

The enumeration cpuArchType represents the Instruction Set Architecture (ISA) for CPU.

| Enumeration value | Description |
| --- | --- |
| ISA_X86_64 | Intel x86 ISA (CISC) |
| ISA_ARM_64 | ARMv8 ISA (RISC) |

: Instruction Set Architecture types

###### Enumeration: gpuVendorType

The enumeration gpuVendorType represents the GPU providers.

| Enumeration value | Description |
| --- | --- |
| GPU_PROVIDER_NVIDIA | Nvidia GPUs for applications |
| GPU_PROVIDER_AMD | AMD GPUs for applications |

: GPU Providers types

###### Enumeration: versionInfo

The enumeration versionInfo represents the Operating System which may be supported by OP.

| Enumeration value | Description |
| --- | --- |
| OS_VERSION_UBUNTU_2204_LTS | Refer to Ubuntu 22.04 LTS Linux operating system |
| OS_VERSION_RHEL_8 | Refer to Red Hat Enterprise Linux 8 operating system |
| OS_VERSION_RHEL_7 | Refer to Red Hat Enterprise Linux 7 operating system |
| OS_VERSION_ DEBIAN_11 | Refer to Debian Linux 11 operating system |
| OS_VERSION_COREOS_STABLE | Refer to Fedora CoreOS Linux Stable operating system |

: Operating system version info

###### Enumeration: licenseType

The enumeration licenseType represents the license model which may be supported by OP and can be exposed over NBI.

| Enumeration value | Description |
| --- | --- |
| OS_LICENSE_TYPE_FREE | Refer to free license and is the default option |
| OS_LICENSE_TYPE_ON_DEMAND | Refer to on-demand license which may be required with certain OS(s) which require mandatory license to deploy the operating system in virtual environment |

: Operating system version info

###### Enumeration: hugePageSize

| Enumeration value | Description |
| --- | --- |
| HUGE_PAGE_2MB | Refer to a hugepage of 2 Megabytes |
| HUGE_PAGE_4MB | Refer to a hugepage of 4 Megabytes |
| HUGE_PAGE_1GB | Refer to a hugepage of 1 Gigabyte |

: Operating system version info

# Application Service APIs

The interface management APIs provides the capabilities to perform the edge application management functions and other OP PRD specified services e.g., network slicing etc. with partner OPs.

## Edge Service APIs

This section provides the details of the edge centric services as part of the operator platform.

### Application Artifacts Management - APIs

Application artifact management APIs enables an OP to share application component descriptors information with partner OP. The application providers via NBI interface submits artifacts information and link artifacts with their edge applications. Leading OP based on application provider intent can share the artifacts with partner OP over E/WBI interface.

#### Introduction

Following table describe the supported operations and resource URIs for artifacts and file upload management.

| Operations | HTTP Method | Resource URI | Qualifier |
| --- | --- | --- | --- |
| Onboard Artifact | POST | /operatorplatform/federation/v1/artifact/{federationContextId} | M |
| Remove Artifact | DELETE | /operatorplatform/federation/v1 /artifact/{federationContextId}/{artifactId} | M |
| View Artifact | GET | /operatorplatform/federation/v1 /artifact/{federationContextId}/{artifactId} | M |
| Upload File | POST | /operatorplatform/federation/v1 /files/{federationContextId} | M |
| Remove File | DELETE | /operatorplatform/federation/v1/files/{federationContextId}/{fileId} | M |
| View File Info | GET | /operatorplatform/federation/v1/files/{federationContextId}/{fileId} | M |

: Artifacts Management APIs

#### Onboard Artifact : POST Method

The tables below describe the data structures supported by the POST Request Body on this resource. This method is used for submitting the artifacts as provided by the application providers over NBI and contains the application component descriptors which lays out the component images, connectivity to user clients, resource requirements etc. The application component descriptors also contain references to the image files submitted by the application providers over NBI to be used with the components.

| Parameter Name | P | Cardinality | Description |
| --- | --- | --- | --- |
| federationContextId | M | 1 | This identifier shall be provided by the originating OP to partner OP to identify the existing federation relationship. |
| artifactId | M | 1 | Identifier unique within an federation context to distinguish different artifacts |
| appProviderId | M | 1 | A unique app provider identifier managed at leading OP representing the association of a given artifact with an app provider on leading OP NBI |
| artifactName | M | 1 | Name of the artifact |
| artifactDescription | O | 1 | Brief description of the artifact by the application provider |
| artifactVersionInfo | M | 1 | Artifact version information |
| artifactVirtType | M | 1 | Indicate if the artifact refers to a containerized or VM type workload descriptor |
| artifactDescriptorType | M | 1 | Descriptor type associated with the artifactType refers to a descriptor e.g., Helm, Terraform, ComponentSpec etc. Helm Charts or Terraform scripts files can be uploaded to OP managed repo or can be pulled from external repo e.g., Github, Helm.sh etc. ContainerSpec schema is proposed as part of this document to deploy containerized workloads to OP managed edge resources. |
| artifactRepoLocation | C | 1 | Artifact image repository location URL and access credentials e.g., Github, local OP repo, bitnami etc. from which given artifacts like charts, Terraform scripts etc. can be retrieved. Artifacts can also be uploaded to OP managed local repo and can be associated to application components. Application providers may be able to upload artifacts over NBI which can be referenced from artifactDescriptorFile(s), and an OP shall also submit them to the partner OP over E/WBI if requested by app provider |
| artifactDescriptorFileFormat | C | 1 | Artifacts like Helm charts or Terraform scripts may need compressed format while ContainerSpec can be plane text file (YAML format) |
| componentSpec | O | 1..N | A component specification to define the image, metainfo and resource requirements |
| artifactFile | O | 1 | Actual file embedded in the request. |

: Onboard Artifact request parameters

The tables below describe the data structures supported by the POST Response Body on this resource.

| Parameter Name | P | Response Codes | Description |
| --- | --- | --- | --- |
| N/A | C | 200 | Artifacts uploaded successfully at partners OP |
| N/A | C | 401 | Authorization information is missing or invalid |
| N/A | C | 404 | Federation not found |
| N/A | C | 422 | Unprocessable Entity. Mandatory parameters are not sent in the request. |
| N/A | C | 500 | Internal Server Error |
| N/A | C | 503 | Service Unavailable. |

: Onboard Artifact response parameters

#### DELETE Method : Remove Artifact

The tables below describe the data structures supported by the DELETE Request Body on this resource.

| Parameter Name | P | Cardinality | Description |
| --- | --- | --- | --- |
| federationContextId | M | 1 | This identifier shall be provided by the originating OP to partner OP to identify the existing federation relationship. |
| artifactId | M | 1 | Identifier unique within an appProviderId to distinguish different artifacts |

: Remove Artifact request parameters

The tables below describe the data structures supported by the DELETE Response Body on this resource.

| Parameter Name | P | Response Code | Description |
| --- | --- | --- | --- |
| Status | C | 200 | Artifact deleted successful |
| N/A | C | 401 | Authorization information is missing or invalid |
| N/A | C | 404 | Federation not found |
| N/A | C | 422 | Unprocessable Entity. Mandatory parameters are not sent in the request. |
| N/A | C | 500 | Internal Server Error |
| N/A | C | 503 | Service Unavailable. |

: Remove Artifact response parameters

#### GET Method : View Artifact Information

The tables below describe the data structures supported by the GET Request Body on this resource.

| Parameter Name | P | Cardinality | Description |
| --- | --- | --- | --- |
| federationContextId | M | 1 | This identifier shall be provided by the originating OP to partner OP to identify the existing federation relationship. |
| artifactId | M | 1 | Identifier unique within an appProviderId to distinguish different artifacts |

: View Artifact request parameters

The tables below describe the data structures supported by the DELETE Response Body on this resource.

| Parameter Name | P | Cardinality | Description |
| --- | --- | --- | --- |
| artifactId | M | 1 | Identifier unique within an appProviderId to distinguish different artifacts |
| appProviderId | M | 1 | App provider identifier managed at leading OP representing the association of a given artifact with an app provider |
| artifactVersionInfo | M | 1 | Artifact version information |
| artifactName | M | 1 | Name of the artifact |
| artifactDescription | O | 1 | Brief description of the artifact by the application provider |
| artifactVersionInfo | M | 1 | Artifact version information |
| artifactVirtType | M | 1 | Indicate if the artifact refers to a containerized or VM type workload descriptor |
| artifactDescriptorType | M | 1 | Descriptor type associated with the artifactType refers to a descriptor e.g., Helm, Terraform, ContainerSpec etc. Helm Charts or Terraform scripts files can be uploaded to OP managed repo or can be pulled from external repo e.g., Github, Helm.sh etc. ContainerSpec schema is proposed as part of this document to deploy containerized workloads to OP managed edge resources. |
| artifactRepoLocation | C | 1 | Artifact image repository location URL and access credentials e.g., Github, local OP repo, bitnami etc. from which given artifacts like charts, Terraform scripts etc. can be retrieved. To refer to OP local repo, application provider can provide artifacts over NBI contained in file associated to artifactDescriptor to submit the a |
| artifactDescriptorFileFormat | C | 1 | Artifacts like Helm charts or Terraform scripts may need compressed format while ContainerSpec can be plane text file (YAML format) |

: View Artifact response parameters

The tables below describe the data structures supported by the GET Request Body on this resource for non-200 HTTP codes.

| Parameter Name | P | Response Codes | Description |
| --- | --- | --- | --- |
| N/A | C | 401 | Authorization information is missing or invalid |
| N/A | C | 404 | Federation not found |
| N/A | C | 422 | Unprocessable Entity. Mandatory parameters are not sent in the request. |
| N/A | C | 500 | Internal Server Error |
| N/A | C | 503 | Service Unavailable. |

: Non-200 Response Codes for View Artifact Response

#### POST Method : Upload File

The tables below describe the data structures supported by the POST Request Body on this resource.

| Parameter Name | P | Cardinality | Description |
| --- | --- | --- | --- |
| federationContextId | M | 1 | This identifier shall be provided by the originating OP to partner OP to identify the existing federation relationship. |
| fileId | M | 1 | Identifier unique within a federation context to distinguish different artifacts |
| appProviderId | M | 1 | A unique app provider identifier managed at leading OP representing the association of a given artifact with an app provider |
| fileName | M | 1 | Name of the file provided by the app provider on NBI. The NBI may provide capabilities to upload files from local filesystems from where NBI is accessed |
| fileDescription | O | 1 | Brief description of the file by the application provider |
| fileVersionInfo | M | 1 | File version information |
| fileType | M | 1 | Indicate if the file is Container image or VM image (QCOW2) |
| imgOSType | M | 1 | Base OS for the image. Currently only “Linux” is supported |
| imgInsSetArch | M | 1 | “x86_64”, “arm64” |
| file | C | 1 | Binary Images of application components (e.g., container images) which can be referenced from the files indicated by artifactDescriptor (E.g., Helm charts) |
| repoLocation | C | 1 | File Repository location information and same as artifactRepoLocation parameter as defined in artifact onboarding API |

: Upload File request Parameters

The tables below describe the data structures supported by the POST Response Body on this resource.

| Parameter Name | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A | C | 200 | File uploaded successfully |
| N/A | C | 401 | Authorization information is missing or invalid |
| N/A | C | 404 | Federation not found |
| N/A | C | 415 | Unsupported Media Type |
| N/A | C | 422 | Unprocessable Entity. Mandatory parameters are not sent in the request. |
| N/A | C | 500 | Internal Server Error |
| N/A | C | 503 | Service Unavailable. |

: Upload File response Parameters

#### DELETE Method : Remove Upload File

The tables below describe the data structures supported by the DELETE Request Body on this resource.

| Parameter Name | P | Cardinality | Description |
| --- | --- | --- | --- |
| federationContextId | M | 1 | This identifier shall be provided by the originating OP to partner OP to identify the existing federation relationship. |
| fileId | M | 1 | Identifier unique within an appProviderId to distinguish different fileId |

: Remove File request parameters

The tables below describe the data structures supported by the DELETE Response Body on this resource.

| Parameter Name | P | Response Codes | Description |
| --- | --- | --- | --- |
| N/A | C | 200 | File deleted successfully |
| N/A | C | 401 | Authorization information is missing or invalid |
| N/A | C | 404 | Federation not found |
| problemDetails | C | 422 | Unprocessable Entity. Mandatory parameters are not sent in the request. |
| N/A | C | 500 | Internal Server Error |
| N/A | C | 503 | Service Unavailable. |

: Remove File response parameters

#### GET Method : View File Information

The tables below describe the data structures supported by the GET Request Body on this resource.

| Parameter Name | P | Cardinality | Description |
| --- | --- | --- | --- |
| federationContextId | M | 1 | This identifier shall be provided by the originating OP to partner OP to identify the existing federation relationship. |
| fileId | M | 1 | Identifier unique within an appProviderId to distinguish different fileId |

: View File request parameters

The tables below describe the data structures supported by the GET Response Body on this resource.

: View File response

The tables below describe the data structures supported by the GET Response Body on this resource.

| Parameter Name | P | Response Codes | Description |
| --- | --- | --- | --- |
| fileDetails | C | 200 | File Details |
| problemDetails |  | 400 | Bad request |
| N/A problemDetails | C | 401 | Authorization information is missing or invalid |
| N/A problemDetails | C | 404 | Federation not found |
| problemDetails |  | 409 | Conflict |
| problemDetails | C | 422 | Unprocessable Entity. Mandatory parameters are not sent in the request. |
| N/A problemDetails | C | 500 | Internal Server Error |
| N/A problemDetails | C | 503 | Service Unavailable. |
| problemDetails |  | 520 | Server Returned an Unknown error |

: View file error response

#### Data Model

##### General

This subclause specifies the application data model supported by the Artifacts Management API.

##### Structured Data Types

This clause defines the structured data types to be used in resource representations.

###### artifactDescriptor

Following table describe the artifactDescriptor which defines schema of an application component. Application component may refer to an artifactDescriptor in form of e.g., Helm Chart, Terraform Script, ContainerSpec etc. The descriptors to be supported by the two OP can be extended based on supported capabilities.

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| helmChartRootDir | Compressed File | C | 1 | Zip file containing the Helm Chart directories and files |
| terraformScript | CompressedFile | C | 1 | Zip file containing terraform scripts |
| componentSpec | Object | C | 1 | A containerized component specification to define the image, metainfo and resource requirements |

: artifactDescriptor

###### componentSpec

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| componentName | String | M | 1 | App provider defined name of the container |
| OSType | Enum | M | 1 | Base OS for the container. Currently only “Linux” is supported |
| cpuInstSetArch | Enum | M | 1 | A list of OP supported ISAs e.g., “x86_64”, “arm64” etc. |
| imagePath | String | M | 1 | File identifier as used in upload file API |
| numOfInstances | Int | M | 1 | Number of container instances to be launched |
| restartPolicy | Enum | O | 1 | Container restart policy “Always” or “Never” defines the action to be taken on container failure |
| commandLineParams | Object | O | 1 | Any input parameters to passed to component instance during instantiation |
| exposedInterfaces | Array (ExposedInterface) | M | 1..N | List of interfaces having public visibility exposed by the application component. It could be combination of container port and IP protocol (TCP, UDP) and/or upstream HTTP root URL etc. |
| computeResourceProfile | Object | M |  | Refers to the compute resources required for the container e.g., CPU, RAM, GPU etc. |
| compEnvParams | Array (compEnvParameter) | O | 0..N | Environ variable are key value pairs to provide application provider input parameters to be passed to container process during container process creation |
| persistentVolumes | Array (persistentVolume) | O | 0..1 | The ephemeral volume a container process may need to temporary store internal data |

: componentSpec

###### commandLineParams

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| command | Array(String) | M | 1 | This overrides the command operation of the container file while running container inside a pod |
| commandArgs | Array(String) | O | 1 | These arguments will be added while running containers |

: Command line parameters for application component

###### exposedInterface

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| interfaceId | String | M | 1 | defines the unique identifier/name of the component's API endpoint. It is a logical API endpoint and will be used to provide session handle by SDK. |
| commPort | Integer | M | 1 | Defines the internal port value for the application component to exposed to UCs. OP may generate a dynamic port towards the UCs corresponding to this internal port and forward the client traffic from dynamic port to containerPort. |
| commProtocol | Enum | M | 1 | Defines the IP transport communication protocol i.e., TCP, UDP |
| visibilityType | Enum | M | 1 | defines whether the interface is exposed to outer world or not i.e., external, or internal. If this is set to "external", then it is exposed to external applications otherwise it is exposed internally to edge application components within edge cloud. When exposed to external world, an external dynamic port is assigned for UC traffic and mapped to the internal containerPort |
| network | String | O | 1 | Name of the network. In case the application must be assoisated with more than 1 network then app provider must define the name of the network on which this interface has to be exposed. This parameter is required only if the port must be exposed on a specific network other than default. |
| interfaceName | String | O | 1 | Interface Name. Required only if application must be attached to a network other than default. |

: Component interface exposure information

###### computeResourceProfile

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| cpuArchType | Enum | M | 1 | CPU instruction set architecture (ISA). e.g. Intel, ARM etc. |
| numCPU | Integer | M | 1 | Total number of vCPUs |
| memory | Long | M | 1 | Total physical memory (RAM) for given ISA type (in Mbytes) |
| diskStorage | Long | M | 1 | Total storage (RAM) for workloads for given ISA type (in GB) |
| gpuInfo | Array(gpuResourceInfo) | O | 0..1 | Total GPU for workloads for given ISA type |
| FPGA | Int | O | 0..1 | Total FPGA for workloads for given ISA type |
| vpu | Int | O | 0..1 | Total VPUs for workloads for given ISA type |
| hugepages | Array(haugepageInfo) | O | 1..N | Huge pages for workload for a given ISA type |
| cpuExclusivity | Boolean | O | 0..1 | Support for exclusive CPUs |

: Compute Resource model for application components

###### compEnvParams

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| envVarName | String | M | 1 | Environment variable name |
| envValueType | enum | M | 1 | Defines the content present in envVarValue. Possible value could be “network”, “constant”, “ewbi-dns”, “pri-dns”. Based on envValueType OP may either assign the constant value to the environment variable and pass it to the application component. Or, the value to the be assigned to “envVarValue” will be generated by the application runtime environment and passed on to the component instance during instantiation. If set to “network”, then the dynamic port assigned |
| envVarValue | String | M | 1 | Value assigned to the envVarName attribute and passed to the container instance during instantiation phase |
| envValSrc | String | C | 1 | Network interface Id defined by the application provider in ContainerSpec. Based on the given network interface Id, OP will assign the value of dynamic port it generates for the containerPort and assign to the envVarValue. |

: Component Environment Variables

###### persistentVolume

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| volumeName | String | M | 1 | Human readable name for the volume |
| volumeSize | Integer | M | 1 | size of the volume given by user (10GB, 20GB, 50 GB or 100GB) |
| volumeMountPath | string | M | 1 | defines the mount path of the volume where the volume will be available to containers |
| ephemeralType | Enum | M | 1 | It indicates the ephemeral storage on the node and contents are not preserved if containers restart |
| accessMode | String | M | 1 | Values are RW (read/write) and RO (read-only) |
| sharingPolicy | Enum | M | 1 | Exclusive or Shared. If shared, then in case of multiple containers same volume will be shared across the containers. |

: Persistent Volume

###### artifactRepoLocation

Following table describe the artifactRepoLocation which could be an external repository from where application component images e.g., artifacts can be pulled.

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| repoType | String | M | 1 | Github, Helm, localRepo. For ContainerSpec valid value is “localRepo” |
| repoURL | Link | M | 1 | defines the path/URL of the source artifact |
| userName | String | M | 1 | defines the container repo username incase external repository is used to provide component images |
| Password | String | M | 1 | defines the container repo password in case external repository is used to provide component images |
| Token | String | O | 1 | Authorization Token |

: artifactRepoLocation

###### fileDetails

| Parameter Name | P | Cardinality | Description |
| --- | --- | --- | --- |
| fileId | M | 1 | Identifier unique within a federation context to distinguish different artifacts |
| appProviderId | M | 1 | A unique app provider identifier managed at leading OP representing the association of a given artifact with an app provider |
| fileName | M | 1 | Name of the file provided by the app provider on NBI. The NBI may provide capabilities to upload files from local filesystems from where NBI is accessed |
| fileDescription | O | 1 | Brief description of the file by the application provider |
| fileVersionInfo | M | 1 | File version information |
| fileType | M | 1 | Indicate if the file is Container image or VM image (QCOW2) |
| imgOSType | M | 1 | Base OS for the image. Currently only “Linux” is supported |
| imgInsSetArch | M | 1 | “x86_64”, “arm64” |

: File Details

##### Simple data types and enumerations

This subclause defines simple data types and enumerations that can be referenced from data structures defined in the previous subclauses.

###### Simple data types

| Type Name | Type Definition | Description |
| --- | --- | --- |
| artfactId | String | Identifier unique within an appProviderId to distinguish different artifacts |
| appProviderId | String | A unique app provider identifier managed at leading OP representing the association of a given artifact with an app provider |
| artifactName | String | Name of the artifact |
| artifactDescription | String | Brief description of the artifact by the application provider |
| artifactVersionInfo | String | Artifact version information |
| artifactImageFileName | String | Artifact image file name |
| artifactDescriptorFileName | String | File Name of the artifact descriptor e.g. Helm File Name |

: Artifacts simple datatype table

###### Enumeration: artifactVirtType

The enumeration cpuArchType represents the Instruction Set Architecture (ISA) for CPU.

| Enumeration value | Description |
| --- | --- |
| VM_TYPE | Indicates virtual machines (VM) images |
| CONTAINER_TYPE | Indicate containers images |

: artifactVirtType table

###### Enumeration: artifactDescriptorType

The enumeration artifactDescriptorType represents the artifact descriptor which could be a helm chart for containers deployment, Terraform script for virtual machine deployment etc.

| Enumeration value | Description |
| --- | --- |
| CONTAINERSPEC_TYPE | Indicates Container-as-a-service deployment specification |
| HELM_TYPE | Indicate Helm charts |
| TERRAFORM_TYPE | Indicates Terraform script for VM deployment |

: artifactDescriptorType table

###### Enumeration: containerOSType

The enumeration containerOSType represents the operating system for which a container image is built for.

| Enumeration value | Description |
| --- | --- |
| CONTAINER_OS_LINUX | Indicates Linux OS based container |
| CONTAINER_OS_WINDOWS | Indicate Windows OS based container |

: artifactDescriptorType table

###### Enumeration: restartPolicy

The enumeration restartPolicy represents the action to be taken if a container instance fails.

| Enumeration value | Description |
| --- | --- |
| RESTART_POLICY_ALWAYS | Indicates always restart the failed container |
| RESTART_POLICY_NEVER | Indicate never restart the failed container |

: restartPolicy table

###### Enumeration: commProtocol

The enumeration commProtocol represents the IP network protocol i.e., TCP or UDP.

| Enumeration value | Description |
| --- | --- |
| IP_PROTO_TCP | Indicates TCP protocol |
| IP_PROTO_UDP | Indicate UDP protocol |

: commProtocol table

###### Enumeration: visibilityType

The enumeration visibilityType represents the if a given interface of application component to be exposed to external clients or to internal components only.

| Enumeration value | Description |
| --- | --- |
| VISIBILITY_EXTERNAL | Indicates container interface is exposed externally to clients |
| VISIBILITY_INTERNAL | Indicate container interface is only internally accessible to other components of the application |

: commProtocol table

### Application Provider Resource  Management - APIs

The REST APIs mentioned in this section provides the capabilities to reserve and manage compute resources of an application provider on a partner OP zone.

#### Introduction

Following table describe the applicable HTTP methods for managing resource reservation with partner OP. Resources can be reserved on per zone for an application provider and once reserved, the application provider can associate an application to consume the reserved resources.

| Operations | HTTP Method | Resource URI | Qualifier |
| --- | --- | --- | --- |
| Reserve Compute Resources | POST | /operatorplatform/federation/v1/ /isv/resource/{federationContextId}/{zoneId}/{appProviderId} | M |
| Update Compute Resource Reservation | PATCH | /operatorplatform/federation/v1 /isv/resource/{federationContextId}/{zoneId}/{appProviderId}/{poolId} | M |
| View Reserved Resources | GET | /operatorplatform/federation/v1 /isv/resource/{federationContextId}/{zoneId}/{appProviderId} | M |
| Remove Reserved Resources | DELETE | /operatorplatform/federation/v1 /isv/resource/{federationContextId}/{zoneId}/{appProviderId}/{poolId} | M |
| Resource Reservation Notification | POST | { resourceReservationCallbackLink } | M |

: Compute Resource Reservation Management Methods

#### POST Method : Reserve Compute Resources

The tables below describe the data structures supported by the POST Request Body on this resource.

| Parameter Name | S | Cardinality | Description |
| --- | --- | --- | --- |
| federationContextId | M | 1 | This identifier shall be provided by the originating OP to partner OP to identify the existing federation relationship |
| zoneId | M | 1 | Identifier of partner zone where resources are to be reserved. |
| appProviderId | M | 1 | A unique app provider Identifier referring an application provider account with leading OP |
| poolName | M | 1 | App provider defines a name to identify the resources reserved on the zone |
| resRequest | M | 1 | compute flavors to be reserved and their counts |
| resourceReservationCallbackLink | M | 1 | Callback URI for partner OP to provide status update to the resource reservation request initiated by leading OP |

: Reserve Compute Resources request parameters

The tables below describe the data structures supported by the POST Response Body on this resource.

| Parameter Name | P | Response Codes | Description |
| --- | --- | --- | --- |
| reservedPoolId | C | 200 | ISV Resource reservation request accepted |
| problemDetails | C | 400 | Bad Request |
| problemDetails | C | 401 | Authorization information is missing or invalid |
| problemDetails | C | 404 | Content not found |
| problemDetails | C | 409 | Conflict |
| problemDetails | C | 412 | Pre-condition failed. Application not onboarded or resources not available |
| problemDetails | C | 422 | Unprocessable Entity. Mandatory parameters are not sent in the request. |
| problemDetails | C | 500 | Internal Server Error |
| problemDetails | C | 503 | Service Unavailable. |
| problemDetails | C | 520 | Server Returned an Unknown Error |

: Reserve Compute Resource response parameters

#### PATCH Method : Update Compute Resource Reservation

The tables below describe the data structures supported by the PATCH Request Body on this resource to modify already reserved resources.

| Data Type | P | Cardinality | Description |
| --- | --- | --- | --- |
| federationContextId | M | 1 | This identifier shall be provided by the originating OP to partner OP to identify the existing federation relationship |
| zoneId | M | 1 | Zone where resources are reserved. |
| appProviderId | M | 1 | A unique app provider Identifier referring an application provider account with leading OP |
| poolId | M | 1 | Identifier of the resource pool |
| UpdResInfo | M | 1 | List of modification to be done |

: Update Compute Resource Reservation request parameters

The tables below describe the data structures supported by the PATCH Response Body on this resource.

| Parameter Name | P | Response Codes | Description |
| --- | --- | --- | --- |
| NA | C | 200 | Resource pool updated |
| problemDetails | C | 400 | Bad Request |
| problemDetails | C | 401 | Authorization information is missing or invalid |
| problemDetails | C | 404 | Content not found |
| problemDetails | C | 409 | Conflict |
| problemDetails | C | 412 | Pre-condition failed. Application not onboarded or resources not available |
| problemDetails | C | 422 | Unprocessable Entity. Mandatory parameters are not sent in the request. |
| problemDetails | C | 500 | Internal Server Error |
| problemDetails | C | 503 | Service Unavailable. |
| problemDetails | C | 520 | Server Returned an Unknown Error |

: Update Compute Resource Reservation response parameters

#### GET Method : View Reserved Resources

The tables below describe the data structures supported by the GET Request Body on this resource.

| Data Type | P | Cardinality | Description |
| --- | --- | --- | --- |
| federationContextId | M | 1 | This identifier shall be provided by the originating OP to partner OP to identify the existing federation relationship |
| zoneId | M | 1 | Zone where resources are reserved. |
| appProviderId | M | 1 | A unique app provider Identifier referring an application provider account with leading OP |

: View Reserved Resource request parameters

The tables below describe the data structures supported by the GET Response Body on this resource.

| Parameter Name | P | Response Codes | Description |
| --- | --- | --- | --- |
| reservedPools | C | 200 | Reserved Resources Details |
| problemDetails | C | 400 | Bad Request |
| problemDetails | C | 401 | Authorization information is missing or invalid |
| problemDetails | C | 404 | Content not found |
| problemDetails | C | 409 | Conflict |
| problemDetails | C | 412 | Pre-condition failed. Application not onboarded or resources not available |
| problemDetails | C | 422 | Unprocessable Entity. Mandatory parameters are not sent in the request. |
| problemDetails | C | 500 | Internal Server Error |
| problemDetails | C | 503 | Service Unavailable. |
| problemDetails | C | 520 | Server Returned an Unknown Error |

: Notify resource reservation status response parameters

#### DELETE Method : Remove Reserved Resources

The tables below describe the data structures supported by the DELETE Request Body on this resource.

| Data Type | P | Cardinality | Description |
| --- | --- | --- | --- |
| federationContextId | M | 1 | This identifier shall be provided by the originating OP to partner OP to identify the existing federation relationship |
| zoneId | M | 1 | Zone where resources are reserved. |
| appProviderId | M | 1 | A unique app provider Identifier referring an application provider account with leading OP |
| poolId | M | 1 | Identifier of the resource pool |

: Remove Reserved Resource request parameters

The tables below describe the data structures supported by the DELETE Response Body on this resource.

| Parameter Name | P | Response Codes | Description |
| --- | --- | --- | --- |
| NA | C | 200 | Resource pool deleted |
| problemDetails | C | 400 | Bad Request |
| problemDetails | C | 401 | Authorization information is missing or invalid |
| problemDetails | C | 404 | Content not found |
| problemDetails | C | 409 | Conflict |
| problemDetails | C | 412 | Pre-condition failed. Application not onboarded or resources not available |
| problemDetails | C | 422 | Unprocessable Entity. Mandatory parameters are not sent in the request. |
| problemDetails | C | 500 | Internal Server Error |
| problemDetails | C | 503 | Service Unavailable. |
| problemDetails | C | 520 | Server Returned an Unknown Error |

: Remove Reserved Resources response parameters

#### POST Method: Notify Resource Reservation Status

| Parameter Name | P | Cardinality | Description |
| --- | --- | --- | --- |
| federationContextId | M | 1 | Federation context identifier |
| appProviderId | M | 1 | A unique app provider Identifier referring an application provider account with leading OP |
| zoneId | M | 1 | Identifier of partner zone where resources are to be reserved. |
| poolId | M | 1 | Identifier of resource pool |
| grantedFlavors | M | 0..N | List of flavourResvInfo indicating the allocated resources against the requested resources by partner OP |

: ISV resource reservation status notification parameters

The tables below describe the data structures supported by the POST Response Body on this resource.

| Parameter Name | P | Response Codes | Description |
| --- | --- | --- | --- |
| N/A | C | 204 | Resource reservation status updated |
| problemDetails | C | 400 | Bad Request |
| problemDetails | C | 401 | Authorization information is missing or invalid |
| problemDetails | C | 404 | Content not found |
| problemDetails | C | 409 | Conflict |
| problemDetails | C | 412 | Pre-condition failed. Application not onboarded or resources not available |
| problemDetails | C | 422 | Unprocessable Entity. Mandatory parameters are not sent in the request. |
| problemDetails | C | 500 | Internal Server Error |
| problemDetails | C | 503 | Service Unavailable. |
| problemDetails | C | 520 | Server Returned an Unknown Error |

: Application Onboarding response parameters

#### Data Model

##### General

This subclause specifies the application data model supported by the app provider resource  Management APIs.

##### Structured Data Types

This clause defines the structured data types to be used in resource representations.

###### resRequest

List of all flavors and their respective counts

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| flavors | Array(flavorResvInfo) | M | 1 | An identifier to refer to the combination of compute resource configuration as indicated by the other attributes in this table |
| reserveDuration | Object | M | 1 | Time period for which resources are to be reserved starting from now |

: resRequest

###### flavorResvInto

The table provides flavors and the corresponding amount to be reserved.

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| flavorId | Int | M | 1 | Flavor Identifier |
| numFlavor | Int | M | 1 | Number of flavor to be reserved |
| minNumOfFlavors | Int | O | 1 | If specified, indicate the minimum numbers of flavors to be reserved up to maximum as given in “count” member. If partner OP cannot reserve the minimum number of flavors, then the request shall be failed. |

: flavorResvInfo

###### reservedPoolId

The table provides the information on the resource pool identifier and resource pool name which can be used to refer to an existing pool of resources reserved earlier on request from application providers.

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| poolName | String | M | 1 | Name of the pool |
| poolId | String | M | 1 | Identifier generated by the OP to identify these reserved resources |

: reservedPoolId

###### UpdResInfo

The data structure in the below table provides the information to modify existing resource pool created earlier on request from application providers towards partner OP.

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| updateType | String | M | 1 | Enumerations – Add, Remove, Duration |
| flavorId | String | M | 1 | Flavor identifier |
| count | Int | M |  | Final count of flavor that should be reserved. Value 0 mean remove all such flavor |
| reserveDuration | Object | C | 1 | New time period for which resources are to be reserved from initiat reservation time |

: updResInfo

###### reservedPools

The following table defines the relationship between the resource pool identifier and the resource flavors associated to it.

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| reservedPoolName | String | M | 1 | Name of the pool |
| reservedPoolId | Object | M | 1 | App provider defined name of the pool |
| reservedFlavors | Array(flavorId) | M | 1 | List of flavors and their count reserved for this poolId |
| reserveDuration | Object | O | 1 | Time period for which resources are to be reserved starting from now |
| reservationTime | Date-Time | O | 1 | Date and time of resource reservation by the app provider |

: Reserved Pool Info

###### reserveDuration

The following table defines the time duration for which resource reservation is being requested.

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| numOfDays | Int | C | 1 | Number of days to be reserved |
| numOfMonths | Int | C | 1 | Number of months to be reserved |
| numOfYears | Int | C | 1 | Number of years to be reserved |

: reservationDuration

###### grantedFlavors

The following table defines the structure of granted resources for a resource reservation request.

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| grantedFlavors | Array(flavorResvInfo) | M | 1..N | Number of flavors reserved |

: grantedFlavors

### Application Onboarding Management - API

Application onboarding management APIs are used to share the application information on request from application providers to partner OP.

#### Introduction

Following table describe the mapping of IS operations to SS equivalents.

| Operation | HTTP Method | Resource URI | Qualifier |
| --- | --- | --- | --- |
| Onboard Application | POST | /operatorplatform/federation/v1/{federationContextId}/ application/onboarding | M |
| Update Application | PATCH | /operatorplatform/federation/v1/{federationContextId}/application/onboarding/appprovid/{appprovid}/appid/{appid} | M |
| Remove Application | DELETE | /operatorplatform/federation/v1/{federationContextId}/ application/onboarding/{appprovid}/appid/{appid} | M |
| View Application | GET | /operatorplatform/federation/v1/{federationContextId}/application/onboarding/{appprovid}/appid/{appid} | M |
| Notify Application State Info | POST | { appStatusCallbackLink } | M |

: Application Onboarding Management APIs

#### Onboard Applications : POST Method

The tables below describe the data structures supported by the POST Request Body on this resource.

| Parameter Name | P | Cardinality | Description |
| --- | --- | --- | --- |
| federationContextId | M | 1 | This identifier shall be provided by the originating OP to partner OP to identify the existing federation relationship |
| applInformation | M | 1 | Application compute resource, component images, QoS, availability zone information |

: Application Onboarding request parameters

The tables below describe the data structures supported by the POST Response Body on this resource.

| Parameter Name | P | Response Codes | Description |
| --- | --- | --- | --- |
| N/A | C | 201 | Application onboarded successfully |
| N/A | C | 401 | Authorization information is missing or invalid |
| N/A | C | 404 | Federation not found |
| N/A | C | 422 | Unprocessable Entity. Mandatory parameters are not sent in the request. |
| N/A | C | 500 | Internal Server Error |
| N/A | C | 503 | Service Unavailable. |

: Application Onboarding response parameters

#### Update Application Information : PATCH Method

The tables below describe the data structures supported by the PATCH Request Body on this resource.

| Parameter Name | P | Cardinality | Description |
| --- | --- | --- | --- |
| federationContextId | M | 1 | This identifier shall be provided by the originating OP to partner OP to identify the existing federation relationship |
| appld | M | 1 | Application compute resource, components, associated artifactId, |
| appUpdQoSProfile | O | 1 | Application resource requirement or deployment attributes that needs to be updated |
| appComponentSpecs | O | 1 | Application components and their assosiated artifcats or DNS. |

: Modify application information request parameters

The tables below describe the data structures supported by the PATCH Response Body on this resource.

| Parameter Name | P | Response Codes | Description |
| --- | --- | --- | --- |
| N/A | C | 201 | Application Updated successfully |
| N/A | C | 401 | Authorization information is missing or invalid |
| N/A | C | 404 | Federation not found |
| N/A | C | 422 | Unprocessable Entity. Mandatory parameters are not sent in the request. |
| N/A | C | 500 | Internal Server Error |
| N/A | C | 503 | Service Unavailable. |

: Modify application information response parameters

#### DELETE Method : Remove Application Information

The tables below describe the data structures supported by the DELETE Request Body on this resource.

| Parameter Name | P | Cardinality | Description |
| --- | --- | --- | --- |
| federationContextId | M | 1 | This identifier shall be provided by the originating OP to partner OP to identify the existing federation relationship |
| appId | M | 1 | Application Identifier for a given appProviderId. |
| zoneId | M | 1 | zone identifiers from where application must be deboarded. |

: Remove application request parameters

The tables below describe the data structures supported by the DELETE Response Body on this resource.

| Parameter Name | P | Response Codes | Description |
| --- | --- | --- | --- |
| N/A | C | 202 | Application Updated successfully |
| N/A | C | 401 | Authorization information is missing or invalid |
| problemDetails | C | 404 | Content not found |
| problemDetails | C | 422 | Unprocessable Entity. Mandatory parameters are not sent in the request. |
| N/A | C | 500 | Internal Server Error |
| N/A | C | 503 | Service Unavailable. |

: Remove application response parameters

#### POST Method: Notify resource reservation Status Information

The table below describes the POST request which the partner OP initiate towards the leading OP to provide status update or completion of an earlier resource reservation request.

| Parameter Name | P | Cardinality | Description |
| --- | --- | --- | --- |
| federationContextId | M | 1 | This identifier shall be provided by the originating OP to partner OP to identify the existing federation relationship |
| appld | M | 1 | Application Identifier |
| statusInfo | M | 1 | Status of an application on zone. |

: Resource reservation notification parameters

The tables below describe the data structures supported by the POST Response Body on this resource.

| Parameter Name | P | Response Codes | Description |
| --- | --- | --- | --- |
| N/A | C | 204 | Resource reservation status updated |
| problemDetails | C | 400 | Bad Request |
| problemDetails | C | 401 | Authorization information is missing or invalid |
| problemDetails | C | 404 | Content not found |
| problemDetails | C | 409 | Conflict |
| problemDetails | C | 412 | Pre-condition failed. Application not onboarded or resources not available |
| problemDetails | C | 422 | Unprocessable Entity. Mandatory parameters are not sent in the request. |
| problemDetails | C | 500 | Internal Server Error |
| problemDetails | C | 503 | Service Unavailable. |
| problemDetails | C | 520 | Server Returned an Unknown Error |

: Resource reservation notification response parameters

#### Data Model

##### General

This subclause specifies the application data model supported by the Application Onboarding Management API.

##### Structured Data Types

This clause defines the structured data types to be used in resource representations.

###### appInformation

Following table describe the mapping of IS operations to SS equivalents.

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| appId | String | M | 1 | Identifier of the application |
| appProviderId | String | M | 1 | Unique Identifier to identify the application providers of the leading OP |
| appDeploymentZones | Array(regionInfo) | M | 1..N | Geographical location where application should be made available |
| appMetaData | Object | M | 1 | Application metadata details |
| appQoSProfile | Object | O | 1 | Parameters corresponding to the performance constraints, tenancy details etc. |
| appProvisioning | Bool | O | 1 | Define if application can be instantiated or not |
| appComponentSpecs | Array(appComponentSpec) | M | 1..N | Details about application components, associated component images and descriptors, compute resources etc. |
| appStatusCallbackLink | uri | M | 1 | An application callback URL which shall be used by the partner OP to inform home OP about change in application status or changes in status or an application instance. |

: appInformation

###### regionInfo

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| countryCode | Object | M | 1 | ISO 3166-1 Country Code where application is to be deployed |
| zoneInfo | Object | M | 1 | Availability zone identifiers for given targerOPId |

: regionInfo

###### appMetaData

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| version | String | M | 1 | Application version |
| appName | string | M | 1 | Name of the application |
| appDescription | String | O | 1 | Brief application description provided by application provider |
| accessToken | String | M | 1 | An application Access key to be used with UNI interface to authorize UCs Access to a given application |
| mobilitySupport | String | O | 1 | Indicates if an application is sensitive to user mobility and can be relocated. Default is “NO” |

: Application meta data

###### appQoSProfile

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| latencyConstraints | String | M | 1 | Latency requirements for the application. Allowed values (non-standardized) are none, low and very low. Very Low may corresponds to range 15 - 30 msec, Low correspond to range 30 - 50 msec. None means 51 and above |
| bandwidthRequired | String | O | 1 | Data transfer bandwidth requirement (minimum limit) for the application. It should in Mbits/sec |

: Application QoS profile

###### appComponentsSpec

An application may consist of one or more components where a component represents a runnable unit of the application. A component tie together one or more artifacts i.e., an artifact associated to an image type and/or an artifact which refers to a component descriptor e.g., Helm chart, Terraform file etc.

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| appComponentsSpec | Array(appComponentDetail) | M | 1 |  |

: Application Components

###### appComponentDetail

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| serviceNameNB | String | M | 1 | Must be a valid RFC 1035 label name not more than 64 characters. This defines the DNS name via which the component can be accessed over NBI. Access via serviceNameNB is restricted on specific ports. Platform shall expose component access externally via this DNS name |
| serviceNameEW | String | O | 1 | Must be a valid RFC 1035 label name not more than 64 characters. This defines the DNS name via which the component can be accessed via peer components. Access via serviceNameEW is open on all ports. Platform shall not expose serviceNameEW externally outside edge. |
| componentName | String | M | 1 | Must be a valid RFC 1035 label name. Component name must be unique with an application. It should be atleast 8 characters in length and not more than 64 characters |
| artifactId | String | M | 1 | Identifier of the already onboarded artifact to be used for instantiating the component of the associated application. It refers to artifactDescriptors e.g., Helm chart, Container Spec, Terraform script etc. |

: Application Component Details

###### countryCode

ISO 3166-1 country code to uniquely provide the country information where OP services have been deployed by an operator.

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| countryName | String | M | 1 | Name of the country |
| countryCode | String | M | 1 | Two digit ISO 3166-1-alpha-2 country code e.g., “ES” for Spain |

: Country Code

###### zoneInfo

Zone details where application shall be onboarded

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| zoneId | String | M | 1 | Zone identifier |
| flavorId | String | M | 1 | Flavor that should be used for the application on a zone |
| resPool | String | C | 1 | Id of resource pool that was reserved by the app provider and that shall be used to instantiate the application. |
| resourceConsumption | enum | C | 1 | Specifies if the application can be instantiated using pre-reserved resource or not. App provider can pre-reserve a pool of compute resource on each zone. |

: Zone and flavor info for application onboarding

###### appUpdQoSProfile

Update request shall contain atleast one of the optional parameter defined in below table.

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| latencyConstraints | String | O | 1 | Latency requirements for the application. Allowed values (non-standardized) are none, low and ultra-low. Ultra-Low may corresponds to range 15 - 30 msec, Low correspond to range 30 - 50 msec. None means 51 and above |
| bandwidthRequired | String | O | 1 | Data transfer bandwidth requirement (minimum limit) for the application. It should in Mbits/sec |
| mobilitySupport | String | O | 1 | Indicates if an application is sensitive to user mobility and can be relocated. Default is “NO” |
| multiUserClients | Enum | O | 1 | Define if app supports single user or multiple user clients (UCs) |
| noOfUsersPerAppInst | Integer | C | 1 | For multi user client’s app, how many UCs an app instance can support |
| appProvisioning | Bool | O | 1 | Define if application can be instantiated or not |

: Application QoS profile Update Parameters

###### statusInfo

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| zoneId | String | O | 1 | Zone Identifier |
| onboardStatusInfo | Enum | O | 1 | Application onboarding status |

: StatusInfo

##### Simple data types and enumerations

This subclause defines simple data types and enumerations that can be referenced from data structures defined in the previous subclauses.

###### Enumeration: multiUserClients

Define if an application supports single or multiple User Clients (UCs).

| Enumeration value | Description |
| --- | --- |
| APP_TYPE_SINGLE_USER | A single user client (UC) can connect to an instance of the application |
| APP_TYPE_MULTI_USER | Multi user client (UCs) can connect to an instance of the application |

: multiUserClients

###### Enumeration: onboardingStatusInfo

The table below defines the application onboarding status information.

| Enumeration value | Description |
| --- | --- |
| PENDING | Application onboarding in progress |
| ONBOARDED | Application onboarded successfully |
| DEBOARDING | Application deboarding in progress |
| FAILED | Application onboarding failed |

: Onboarding status info

###### Enumeration: resourceConsumption

Define if an application supports single or multiple User Clients (UCs).

| Enumeration value | Description |
| --- | --- |
| RESERVED_RES_ONLY | Instruct OP to use only the reserved resources |
| RESERVED_RES_PREFER | Instruct OP to first give preference to already reserved resource, If none available OP may use non reserved resources |
| RESERVED_RES_FORBID | instruct OP not to use pre-reserved resources |

: Resource reservation indication table

### Application Instance Lifecycle Management - API

The API mentioned in this section provides the capabilities for managing the edge applications instantiation and terminating the running instance, inquire the status of the application instance etc for applications with partner OPs.

#### Introduction

Following table describe the applicable HTTP methods for applications lifecycle management.

| Operations | HTTP Method | Resource URI | Qualifier |
| --- | --- | --- | --- |
| Instantiate Application | POST | /operatorplatform/federation/v1/application/lcm/{federationContextId} | M |
| Remove Application Instance | DELETE | /operatorplatform/federation/v1 /application/lcm/{federationContextId}/appId/{appInstanceId}/{zoneId} | M |
| View Application Instance | GET | /operatorplatform/federation/v1 /application/lcm/{federationContextId}/appId/{appInstanceId}/{zoneId} | M |
| List Application Instances | GET | /operatorplatform/federation/v1/application/lcm/{federationContextId}/{appId}/{appProviderId} | M |
| Notify Application Instance state information | POST | {appInstCallbackLink} | M |

: Application Instance Management Methods

#### POST Method : Instantiate Applications

The tables below describe the data structures supported by the POST Request Body on this resource.

| Parameter Name | S | Cardinality | Description |
| --- | --- | --- | --- |
| federationContextId | M | 1 | This identifier shall be provided by the originating OP to partner OP to identify the existing federation relationship |
| appId | M | 1 | Application Identifier for a given federation context |
| appProviderId | M | 1 | A unique app provider Identifier referring an application provider account with leading OP |
| appVersion | M | 1 | Application Version of the application provided by the leading OP application provider |
| zoneId | M | 1 | zoneId to which already onboarded application can be instantiated |
| appInstCallbackLink | M | 1 | An application instance callback URL which shall be used by the partner OP to inform the application instance information asynchronously |

: Application instantiation request parameters

The tables below describe the data structures supported by the POST Response Body on this resource.

| Parameter Name | P | Response Codes | Description |
| --- | --- | --- | --- |
| N/A | C | 202 | Application provisioning accepted |
| N/A | C | 401 | Authorization information is missing or invalid |
| problemDetails | C | 404 | Content not found |
| problemDetails | C | 422 | Unprocessable Entity. Mandatory parameters are not sent in the request. |
| N/A | C | 500 | Internal Server Error |
| N/A | C | 503 | Service Unavailable. |

: Application instantiation response parameters

#### DELETE Method : Terminate Application Instance

The tables below describe the data structures supported by the DELETE Request Body on this resource.

| Data Type | P | Cardinality | Description |
| --- | --- | --- | --- |
| federationContextId | M | 1 | This identifier shall be provided by the originating OP to partner OP to identify the existing federation relationship |
| appId | M | 1 | Application Identifier for a given appProviderId . |
| zoneId | M | 1 | Zone Identifier where app instance is running |
| appInstIdentifier | M | 1 | Application instance identifier to refer to a running instance of an application denoted by appId |

: Application instance termination request parameters

The tables below describe the data structures supported by the DELETE Response Body on this resource.

| Parameter Name | P | Response Codes | Description |
| --- | --- | --- | --- |
| appInstanceId | C | 202 | Application instance termination request Accepted |
| N/A | C | 401 | Authorization information is missing or invalid |
| problemDetails | C | 404 | Content not found |
| problemDetails | C | 422 | Unprocessable Entity. Mandatory parameters are not sent in the request. |
| N/A | C | 500 | Internal Server Error |
| N/A | C | 503 | Service Unavailable. |

: Application instance termination response parameters

#### Notify Application Instance Information : POST Method

The availability zone update POST request contains the following parameters towards the partner OP sent over a different HTTP session on the notification URL.

| Parameter Name | P | Cardinality | Description |
| --- | --- | --- | --- |
| federationContextId | M | 1 | This identifier shall be provided by the originating OP to partner OP to identify the existing federation relationship. |
| appId | M | 1 | Application identifier unique per application in an appProviderId |
| appInstIdentifier | M | 1 | Application instance identifier sent by the partner OP in response to application instantiation request |
| zoneId | M | 1 | Zone identifier of the app referred by appId |
| appInstanceInfo | M | 1 | Application instance information e.g., communication end points of various components of the app, zone where it is deployed denoted by appId. |

: Application instance async request parameters

The tables below describe the data structures supported by the POST Response Body on this resource.

| Parameter Name | P | Response Codes | Description |
| --- | --- | --- | --- |
| N/A | C | 202 | Application provisioning notification acknowledged |
| N/A | C | 401 | Authorization information is missing or invalid |
| problemDetails | C | 404 | Content not found |
| problemDetails | C | 422 | Unprocessable Entity. Mandatory parameters are not sent in the request. |
| N/A | C | 500 | Internal Server Error |
| N/A | C | 503 | Service Unavailable. |

: Application instance async response parameters

##### View Application Instance Details : Get Method

View application instance details GET request contains the following parameters towards the partner OP.

| Parameter Name | P | Cardinality | Description |
| --- | --- | --- | --- |
| federationContextId | M | 1 | This identifier shall be provided by the originating OP to partner OP to identify the existing federation relationship. |
| appInstanceId | M | 1 | Application instance identifier sent by the partner OP in response to application instantiation request |
| zoneId | M | 1 | Identifier of partner zone where application instance is created. |

: Application instance async request parameters

The tables below describe the data structures supported by the POST Response Body on this resource.

| Parameter Name | P | Response Codes | Description |
| --- | --- | --- | --- |
| appInstanceInfo | C | 200 | Application instance details |
| problemDetails | C | 400 | Bad Request |
| problemDetails | C | 401 | Authorization information is missing or invalid |
| problemDetails | C | 404 | Content not found |
| problemDetails | C | 409 | Conflict |
| problemDetails | C | 412 | Pre-condition failed. Application not onboarded or resources not available |
| problemDetails | C | 422 | Unprocessable Entity. Mandatory parameters are not sent in the request. |
| problemDetails | C | 500 | Internal Server Error |
| problemDetails | C | 503 | Service Unavailable. |
| problemDetails | C | 520 | Server Returned an Unknown Error |

: View application instance details response parameters

##### List Application Instances : Get Method

View application instance GET request contains the following parameters towards the partner OP.

| Parameter Name | P | Cardinality | Description |
| --- | --- | --- | --- |
| federationContextId | M | 1 | This identifier shall be provided by the originating OP to partner OP to identify the existing federation relationship. |
| appId | M | 1 | Application Identifier for a given appProviderId |
| zoneId | M | 1 | zone identifier where app referred by appId is deployed |

: List application instance parameters

The tables below describe the data structures supported by the POST Response Body on this resource.

| Parameter Name | P | Response Codes | Description |
| --- | --- | --- | --- |
| appInstanceList | C | 200 | Application instance list |
| problemDetails | C | 400 | Bad Request |
| problemDetails | C | 401 | Authorization information is missing or invalid |
| problemDetails | C | 404 | Content not found |
| problemDetails | C | 409 | Conflict |
| problemDetails | C | 412 | Pre-condition failed. Application not onboarded or resources not available |
| problemDetails | C | 422 | Unprocessable Entity. Mandatory parameters are not sent in the request. |
| problemDetails | C | 500 | Internal Server Error |
| problemDetails | C | 503 | Service Unavailable. |
| problemDetails | C | 520 | Server Returned an Unknown Error |

: List application instance response parameters

#### Data Model

##### General

This subclause specifies the application data model supported by the Application Onboarding Management API.

##### Structured Data Types

This clause defines the structured data types to be used in resource representations.

###### appInstanceInfo

Following table describe the mapping of IS operations to SS equivalents.

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| appInstanceState | enum | M | 1 | Pending, Running, Failed etc. |
| accessPointInfo | Array(Object) | M | 1 | Information on external connectivity parameters where clients can connect to the application instance over UNI |

: appInstanceInfo

###### accessPointInfo

Following table describe the mapping of IS operations to SS equivalents.

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| interfaceId | String | M | 1 | Developer/ISV defined logical name for TCP/UDP endpoint exposed by the application as part of the app component structure |
| accessPoints | Object | M | 1 | Details of IP address, port, FQDN etc. |

: accessPointInfo

###### accessPoints

Following table describe the mapping of IS operations to SS equivalents.

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| fqdn | String | C | 1 | fqdn of the app component instance on requested zone where UC can connect with app instance on edge |
| ipv4Addresses | IPv4 Address | C | 1 | IPv4 address of the app component instance on requested zone where UC can connect with app instance on edge |
| ipv6Addresses | IPv6 Address | C | 1 | IPv6 address of the app component instance on requested zone where UC can connect with app instance on edge |
| port | string | M | 1 | Port of the app component instance on requested zone where UC can connect with app instance on edge |

: accessPoints

###### appInstanceList

Following table describe the application instance list containing the details of the application running instances created for an application.

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| zoneId | String | M | 1 | fqdn of the app component instance on requested zone where UC can connect with app instance on edge |
| appInstanceInfo | Array | M | 1..N | List for app instance Identifier and instance state |

: application Instance list

###### InstanceIdentifiers

List of zoneIds and application instances created on that zone

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| zoneId | String | M | 1 | Partner zone identifier |
| appInstIdentifier | String | M | 1 | Application instance identifier. This identifier the instance created on the zone. |

: Application Instance Identifiers

### Edge Node Sharing - API

#### Introduction

Following table describe the operations, applicable HTTP methods and the resource URI for edge node sharing API.

| Operations | HTTP Method | Resource URI | Qualifier |
| --- | --- | --- | --- |
| Edge Node Discovery | POST | /operatorplatform/federation/v1/ edgenodesharing /edgeDiscovery/{federationContextId}/ | M |

: Edge Node Sharing Operations and Resource URI

#### POST Method: Edge Node Discovery

This operation is intended for OP A to fetch the edge nodes discovery information to be shared from a partner OP B.

| Parameter name | P | Cardinality | Description |
| --- | --- | --- | --- |
| federationContextId | M | 1 | This identifier shall be provided by the originating OP to partner OP to identify the existing federation relationship. |
| appProviderId | M | 1 | Application provider identifier |
| appId | M | 1 | Application identifier |
| edgeDiscoveryFilters | O | 1 | Edge node discovery filters to help partner OP to select adequate edge(s) |

: Edge Node Discovery Request Parameters

The following table contains the HTTP Response body parameters for 200 OK response.

| Parameter name | P | Response Codes | Description |
| --- | --- | --- | --- |
| easDiscoveryResp | M | 1 | Indicates the application access information in locations indicated in edge node share request |

: Edge node discovery response Parameters

The tables below describe the data structures supported by the POST Response Body on this resource for non-200 OK responses.

| Parameter Name | P | Response Codes | Description |
| --- | --- | --- | --- |
| N/A | C | 401 | Authorization information is missing or invalid |
| problemDetails | C | 404 | Content not found |
| problemDetails | C | 422 | Unprocessable Entity. Mandatory parameters are not sent in the request. |
| N/A | C | 500 | Internal Server Error |
| N/A | C | 503 | Service Unavailable. |

: Edge  node discovery failure responses

#### Data Model

##### General

| Parameter name | P | Cardinality | Description |
| --- | --- | --- | --- |
| easDiscoveryResp | C | 1 | Edge node share response parameter |
| edgeDiscoveryFilters | O | 1 | Edge node discovery filters to help partner OP to select adequate edge(s) |

: Data structures for edge node discovery API

##### Structured Data Types

This clause defines the structured data types to be used in resource representations.

###### easDiscoveryResp

The following table provides the information about the response parameters that may be returned by the partner OP for edge node discovery request.

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| discoveredEdgeNodes | Array (discoveredEdgeNodes) | M | 1..N | List of Edge discovery information (e.g. URI, FQDN, IP address) |

: easDiscoveryResp

###### discoveredEdgeNodes

The following table provides the information about the response parameter discoveredEdgeNodes that may be returned by the partner OP in response to edge node discovery request.

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| zoneId | String | M | 1 | Availability zone identifier of partner OP |
| latencyServiceEndPoints | Object | M | 1 | FQDN, IP and Port information about the probe responder service that can be further used by the user device to detrnmine traffic latency. |

: discoveredEdgeNodes

###### edgeDiscoveryFilters

The following table provides the information about the edge discovery filters which originating OP may include as additional qualifying information to partner OP for filtering the available edge node(s) using this information.

| Attribute Name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| locationInfo | String | O | 0..1 | Information obtained from the home OP regarding UE location.to help partner OP locate the adequate availability zones in UE location. It could be Lat/Long or zoneId of the UE |

: edgeDiscoveryFilters

##### Simple data types and enumerations

This subclause defines simple data types and enumerations that can be referenced from data structures defined in the previous subclauses.

### LBO Roaming Authentication – API

#### Introduction

An OP uses the HTTP POST method on the resource URI described in table below to authenticate roaming users of the partner OP. Following table describe the applicable HTTP methods and the resource URI for LBO roaming authentication API .

| Operations | HTTP Method | Resource URI | Qualifier |
| --- | --- | --- | --- |
| User Authentication | POST | /operatorplatform/federation/v1 roaminguserauth /{federationContextId}/ | M |

: Roaming user authentication Operations

#### POST Method : Authenticate roaming user

The tables below describe the data structures supported by the POST Request Body on this resource.

| Parameter Name | S | Cardinality | Description |
| --- | --- | --- | --- |
| federationContextId | M | 1 | This identifier shall be provided by the visited OP to home OP of the user to identify the existing federation relationship |
| deviceId | M | 1 | Roaming user device Id to identify and authenticate the roaming user by home mobile network |
| authToken | M | 1 | An authentication token assigned to the roaming user by home OP when UC tries to register from visited network. The token is provided to the visited OP by the user client when it is redirected to register with visited OP. It is used by visited OP to authenticate the roaming user by home OP |

: Roaming user authentication request parameters

The tables below describe the data structures supported by the POST Response Body on this resource.

| Parameter Name | P | Response Codes | Description |
| --- | --- | --- | --- |
| N/A | C | 200 | Device Auth Token validated |
| N/A | C | 401 | Authorization information is missing or invalid |
| problemDetails | C | 404 | Content not found |
| problemDetails | C | 422 | Unprocessable Entity. Mandatory parameters are not sent in the request. |
| N/A | C | 500 | Internal Server Error |
| N/A | C | 503 | Service Unavailable. |

: Roaming user authentication response parameters

#### Data Model

##### Simple data types and enumerations

This subclause defines simple data types and enumerations that can be referenced from data structures defined in the previous subclauses.

###### Simple data types

| Attribute Name | Data Type | Description |
| --- | --- | --- |
| deviceId | String | Device identifier as determined by the visited mobile network for roaming user |
| authToken | String | Temporary token to be used by the client application to authenticate itself to partner OP |

: Roaming user authentication simple datatype table

### Network Events – API

For the API definition, RESTful HTTP base solution and data model please refer to 3GPP TS 29.522 section 5.6: “AnalyticsExposure API”.

#### Introduction

Following table describe the operations, applicable HTTP methods and the resource URI for network events API.

| Operations | HTTP Method | Resource URI | Qualifier |
| --- | --- | --- | --- |
| Fetch Network Analytics Info | GET | /operatorplatform/federation/v1/{federationContextId}/networkanalytics | M |

: Fetch Network Analytics Info Operations and Resource URI

As described in section 2.5, procedures for interaction between OPs to retrieve network analytic information it is not covered. As a result, this is a missing capability in the API definition as well.

Since this API is intended to work in a federated scenario there must be the possibility to include in the HTTP requests and responses, parameters generated from the federation establishment, as defined in section 3.1. For instance, an HTTP GET message that contains the federationContextId, UE identifier and network analytics type to be obtained.

Note: Alternatively, it may be considered as an option for having a HTTP POST method that allows an OP to subscribe to notifications related to a specific type of analytics on a partner OP in future revision of this document.

| Section | 3GPP Spec | API Name | Description |
| --- | --- | --- | --- |
| 5.6 | 29.522 R17 | AnalyticsExposure | Allows AF to create a new subscription to retrieve analytics information |

: Network Status references against 3GPP

| Section | 3GPP Spec | API Name | Description |
| --- | --- | --- | --- |
| 5.6.3 | 29.522 R17 | AnalyticsExposure | AF to AF scenario. Data model should cover parameters related to federation establishment |

: Network Status missing capabilities in 3GPP API reference

#### Data Model

##### Structured Data Types

It is assumed that the OP shall be using the structured data types as defined in the data model section 5.6 in 3GPP TS 29.522 for both the GET request and corresponding response from the partner OP.

##### Simple data types and enumerations

It is assumed that the OP shall be using the simple data types as defined in the 3GPP TS 29.522 section 5.6 for both the GET request and corresponding response from the partner OP.

# Security

TLS shall be used to support the security communication between the OPs. The access to the E/WBI APIs shall be authorized by means of OAuth2 protocol (see IETF RFC 6749), based on local configuration, using the "Client Credentials" authorization grant. If OAuth2 is used, a client, prior to consuming services offered by an OP E/WBI APIs, shall obtain a "token" from the authorization server.

# OpenAPI Specification Sample

openapi: 3.0.3

info:

version: '1.0.0'

title: 'Federation Management Service'

description:

Federation Management Service.

© 2022 GSM Association.

All rights reserved.

externalDocs:

description: GSMA, E/WBI APIs v0.9

url: 'http://www.xxxx.com'

servers:

- url: '{apiRoot}/operatorplatform/federation/v1'

variables:

apiRoot:

default: https://operatorplatform.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- oAuth2ClientCredentials:

- fed-mgmt

components:

securitySchemes:

ApiKeyAuth:

type: apiKey

name: federationkey

in: header

description: A simple encryted string, provided by the partner, Ex a4db08b7-5729-4ba9-8c08-f2df493465a1.

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '/oauth2/token'

scopes:

fed-mgmt: Access to the federation APIs

schemas:

AppIdentifier:

type: string

pattern: ^[A-Za-z][A-Za-z0-9_]{7,63}$

description: Identfier used to refer to an application. This idenfier is globally unique so that application can be identified uniquely across different OPs.

AppProviderId:

type: string

pattern: ^[A-Za-z][A-Za-z0-9_]{7,63}$

description: UserId of the app provider.  Identifier is relevant only in context of this federation.

ArtifactId:

type: string

format: uuid

description: A globally unique identifier assosiated with the artifact. Originating OP generates this identifier when artifact is submitted over NBI.

CountryCode:

type: string

description: "ISO 3166-1 Alpha-2 code for the country of Partner operator"

pattern: ^[A-Z]{2}$

CPUArchType:

type: string

enum:

- ISA_X86_64

- ISA_ARM_64

description: CPU Instruction Set Architecture (ISA) E.g., Intel, Arm etc.

InstanceIdentifier:

type: string

pattern:  ^[A-Za-z0-9][A-Za-z0-9_]{6,62}[A-Za-z0-9]$

description: Unique identifier generated by the partner OP to identify an instance of the application on a specific zone.

InstanceState:

type: string

enum:

- PENDING

- READY

- FAILED

- TERMINATING

description: Running status of the application instance.

Ipv4Addr:

type: string

pattern: '^(([0-9]|[1-9][0-9]|1[0-9][0-9]|2[0-4][0-9]|25[0-5])\.){3}([0-9]|[1-9][0-9]|1[0-9][0-9]|2[0-4][0-9]|25[0-5])$'

example: '198.51.100.1'

Ipv6Addr:

type: string

allOf:

- pattern: '^((:|(0?|([1-9a-f][0-9a-f]{0,3}))):)((0?|([1-9a-f][0-9a-f]{0,3})):){0,6}(:|(0?|([1-9a-f][0-9a-f]{0,3})))$'

- pattern: '^((([^:]+:){7}([^:]+))|((([^:]+:)*[^:]+)?::(([^:]+:)*[^:]+)?))$'

example: '2001:db8:85a3::8a2e:370:7334'

PoolId:

type: string

pattern:  ^[A-Za-z0-9][A-Za-z0-9_]{6,30}[A-Za-z0-9]$

description: OP defined Identifier for the pool reserved for the ISV.  It should be unique with an OP.

Port:

type: integer

minimum: 0

Fqdn:

type: string

FixedNetworkIds:

type: array

items:

type: string

description: List of network identifier assosisted with the fixed line network of the operator platform.

minItems: 1

FederationContextId:

type: string

pattern: '^([0-9]{5,6}-)?[^-]+$'

readOnly: true

description: This key shall be provided by the partner OP on successful verification and validation of the federation create request and is used by partner op to identify this newly created fedartion context. Originating OP shall provide this key in any subsequent request towards the partner op.

FileId:

type: string

format: uuid

description: A globally unique identifier assosiated with the image file. Originating OP generates this identifier when file is uploaded over NBI.

Mcc:

type: string

pattern: '^\d{3}$'

Mnc:

type: string

pattern: '^\d{2,3}$'

MobileNetworkIds:

type: object

properties:

mcc:

$ref: '#/components/schemas/Mcc'

mncs:

type: array

items:

$ref: '#/components/schemas/Mnc'

minItems: 1

ServiceEndpoint:

type: object

required:

- port

anyOf:

- required:

-  fqdn

- required:

- ipv4Addresses

- required:

- ipv6Addresses

properties:

port:

$ref: '#/components/schemas/Port'

fqdn:

$ref: '#/components/schemas/Fqdn'

ipv4Addresses:

type: array

items:

$ref: '#/components/schemas/Ipv4Addr'

minItems: 1

ipv6Addresses:

type: array

items:

$ref: '#/components/schemas/Ipv6Addr'

minItems: 1

Status:

type: string

enum:

- FAILED

- TEMPORARY_FAILURE

- AVAILABLE

- LOCKED

FederationIdentifier:

type: string

pattern: '^[A-Fa-f0-9]*$'

description: Globally unique Identifier allocated to an operator platform. This is valid and used only in context of  MEC federation interface.

GeoLocation:

type: string

description: "Latitude, Longitude as decimal fraction upto 4 digit precision"

pattern: ^([-+]?)([\d]{1,2})(((\.)(\d+)(,)))(\s*)(([-+]?)([\d]{1,3})((\.)(\d+))?)$

SupportedFeatures:

type: string

pattern: '^[A-Fa-f0-9]*$'

InvalidParam:

type: object

properties:

param:

type: string

reason:

type: string

required:

- param

Uri:

type: string

Version:

type: string

pattern:  ^(\d{1,2}\.)?(\d{1,2}\.)?(\d{1,2})$

description: Versioning info in the format major.minor.patch

ZoneIdentifier:

type: string

pattern: '^[A-Fa-f0-9]*$'

description:  Human readable name of the zone.

ZoneDetails:

type: object

required:

- zoneId

- geolocation

- geographyDetails

properties:

zoneId:

$ref: '#/components/schemas/ZoneIdentifier'

geolocation:

$ref: '#/components/schemas/GeoLocation'

geographyDetails:

type: string

description:  Details about cities  or state covered by the edge. Details about the type of locality  for eg rural, urban, industrial etc.

This information is defined in human readable form.

#

# STRUCTURED DATA TYPES

#

ComputeResourceInfo:

type: object

required:

- cpuArchType

- numCPU

- memory

- diskStorage

properties:

cpuArchType:

type: string

enum:

- ISA_X86_64

- ISA_ARM_64

description: CPU Instruction Set Architecture (ISA) E.g., Intel, Arm etc.

numCPU:

type: integer

format: int32

description: Number of available vCPUs.

memory:

type: integer

format: int64

description: Amount of RAM  in mega bytes

diskStorage:

type: integer

format: int32

description: Amount of disk storage  in giga bytes for a given ISA type

gpu:

type: array

items:

$ref: '#/components/schemas/GpuInfo'

vpu:

type: integer

description: Number of Intel VPUs available for a given ISA type

fpga:

type: integer

description: Number of FPGAs available for a given ISA type

hugepages:

type: array

items:

$ref: '#/components/schemas/HugePage'

cpuExclusivity:

type: boolean

description: Support for exclusive CPUs

FederationRequestData:

type: object

required:

- origOPFederationId

- initialDate

- federationNotificationDest

properties:

origOPFederationId:

$ref: '#/components/schemas/FederationIdentifier'

origOPCountryCode:

$ref: '#/components/schemas/CountryCode'

origOPMobileNetworkCodes:

$ref: '#/components/schemas/MobileNetworkIds'

origOPFixedNetworkCodes:

$ref: '#/components/schemas/FixedNetworkIds'

initialDate:

type: string

format: date-time

description: Time zone info of the federation initiated by the originating OP

federationNotificationDest:

$ref: '#/components/schemas/Uri'

FederationResponseData:

type: object

required:

- partnerOPFederationId

- partnerOPCountryCode

- edgeDiscoveyServiceEndPoint

- lcmServiceEndPoint

- offeredAvailabilityZones

- capabilities

oneOf:

- required:

-  partnerOPMobileNetworkCodes

- required:

- partnerOPFixedNetworkCodes

properties:

partnerOPFederationId:

$ref: '#/components/schemas/FederationIdentifier'

partnerOPCountryCode:

$ref: '#/components/schemas/CountryCode'

federationContextId:

$ref: '#/components/schemas/FederationContextId'

edgeDiscoveyServiceEndPoint:

$ref: '#/components/schemas/ServiceEndpoint'

lcmServiceEndPoint:

$ref: '#/components/schemas/ServiceEndpoint'

partnerOPMobileNetworkCodes:

$ref: '#/components/schemas/MobileNetworkIds'

partnerOPFixedNetworkCodes:

$ref: '#/components/schemas/FixedNetworkIds'

offeredAvailabilityZones:

type: array

items:

$ref: '#/components/schemas/ZoneDetails'

minItems: 1

description: List of zones, which the operator platform wishes to make available to developers/ISVs of  requesting operator platform.

platformCaps:

type: array

items:

type: string

enum:

- homeRouting

- Anchoring

description:  Home routing - Operator platform is capable of routing edge application data traffic from its edges to user device in thier home location.

This is the case where user devices are  served in thier home region (requesting platform region, non roaming)  but the corresponding edge application  are in  operator platform edges.

Anchoring - Operator platform is capable of routing edge application traffic for roaming user devices to  edge application in user device home network.

flavor:

type: object

required:

- flavorId

- cpuArchType

- supportedOSTypes

- numCPU

- memorySize

- storageSize

properties:

flavorId:

type: integer

format: int32

description: An identifier to refer to this combination of compute resources.

cpuArchType:

$ref: '#/components/schemas/CPUArchType'

supportedOSTypes:

description: A list of operating systems which this flavor configuration can support e.g., RHEL Linux, Ubuntu 18.04 LTS, MS Windows 2012 R2.

type: array

items:

$ref: '#/components/schemas/OSType'

minItems: 1

numCPU:

type: integer

format: int32

description: Number of available vCPUs

memorySize:

type: integer

format: int32

description: Amount of RAM  in mega bytes

storageSize:

type: integer

format: int32

description: Amount of disk storage  in giga bytes

gpu:

type: array

items:

$ref: '#/components/schemas/GpuInfo'

vpu:

type: integer

description: Number of Intel VPUs available

hugepages:

type: array

items:

$ref: '#/components/schemas/HugePage'

cpuExclusivity:

type: boolean

description: Support for exclusive CPUs

GpuInfo:

type: object

required:

- gpuVendorType

- gpuModeName

- gpuMemory

- numGPU

properties:

gpuVendorType:

type: string

enum:

- GPU_PROVIDER_NVIDIA

- GPU_PROVIDER_AMD

description: GPU vendor name e.g. NVIDIA, AMD etc.

example: Nvidia

gpuModeName:

type: string

description: Model name corresponding to vendorType may include info e.g. for NVIDIA, model name could be “Tesla M60”, “Tesla V100” etc.

gpuMemory:

type: integer

description: GPU memory in mega bytes

numGPU:

type: integer

description: Number of GPUs

HugePage:

type: object

required:

- pageSize

- number

properties:

pageSize:

type: string

enum:

- 2MB

- 4MB

- 1GB

description: Size of hugepage

number:

type: integer

description: Total number of hugepages

OSType:

type: object

required:

- architecture

- distribution

- version

- license

properties:

architecture:

type: string

enum:

- x86_64

- x86

example: x86_64

distribution:

type: string

enum:

- RHEL

- UBUNTU

- COREOS

- FEDORA.

- WINDOWS

version:

type: string

enum:

- OS_VERSION_UBUNTU_2204_LTS

- OS_VERSION_RHEL_8

- OS_VERSION_RHEL_7

- OS_VERSION_ DEBIAN_11

- OS_VERSION_COREOS_STABLE

- OS_MS_WINDOWS_2012_R2

license:

type: string

enum:

- OS_LICENSE_TYPE_FREE

- OS_LICENSE_TYPE_ON_DEMAND

ProblemDetails:

type: object

properties:

title:

type: string

detail:

type: string

cause:

type: string

invalidParams:

type: array

items:

$ref: '#/components/schemas/InvalidParam'

minItems: 1

ZoneRegisterationRequestData:

type: object

required:

- acceptedAvailabilityZones

- availZoneNotifLink

properties:

acceptedAvailabilityZones:

type: array

items:

$ref: '#/components/schemas/ZoneIdentifier'

minItems: 1

availZoneNotifLink:

$ref: '#/components/schemas/Uri'

ZoneRegisterationResponseData:

type: object

required:

- acceptedZoneResourceInfo

properties:

acceptedZoneResourceInfo:

type: array

items:

type: object

required:

- zoneId

- reservedResources

- resourceQuotaLimits

- flavorsSupported

properties:

zoneId:

$ref: '#/components/schemas/ZoneIdentifier'

reservedComputeResources:

description: Resources exclusively reserved for the originator OP.

type: array

items:

$ref: '#/components/schemas/ComputeResourceInfo'

minItems: 1

computeResourceQuotaLimits:

description: Max quota on  resources  partner OP allows over reserved resources.

type: array

items:

$ref: '#/components/schemas/ComputeResourceInfo'

minItems: 1

flavorsSupported:

type: array

items:

$ref: '#/components/schemas/flavor'

minItems: 1

networkResources:

type: object

required:

- egressBandWidth

- dedicatedNIC

- supportSriov

- supportDPDK

properties:

egressBandWidth:

type: integer

format: int32

description: Max dl thoughput that this edge can offer. It is defined in Mbps.

dedicatedNIC:

type: integer

format: int32

description: Number of  network interface cards which can be dedicatedly assigned to application pods on isolated networks.

This includes veirtual as well physical NICs

supportSriov:

type: boolean

description: If this zone support SRIOV networks or not

supportDPDK:

type: boolean

description: If this zone supports DPDK based networking.

minItems: 1

#

# HTTP responses

#

responses:

'400':

description: Bad request

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'401':

description: Unauthorized

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'404':

description: Not Found

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'409':

description: Conflict

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'412':

description: Precondition Failed

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'422':

description: Unprocessable Entity

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'500':

description: Internal Server Error

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'501':

description: Not Implemented

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'503':

description: Service Unavailable

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'520':

description: Web Server Returned an Unknown Error

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

default:

description: Generic Error

paths:

/partner:

post:

summary: Creates one direction federation with partner operator platform.

operationId: CreateFederation

tags:

- FederationManagement

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/FederationRequestData'

responses:

'200':

description: Federation meta-info request accepted

content:

application/json:

schema:

$ref: '#/components/schemas/FederationResponseData'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the

structure: {apiRoot}/operatorplatform/federation/v1/partner/{federationContextId}'

required: true

schema:

type: string

Accept-Encoding:

description: Accept-Encoding, described in IETF RFC 7694

schema:

type: string

Content-Encoding:

description: Content-Encoding, described in IETF RFC 7231

schema:

type: string

'400':

$ref: '#/components/responses/400'

'401':

$ref: '#/components/responses/401'

'409':

$ref: '#/components/responses/409'

'422':

$ref: '#/components/responses/422'

'500':

$ref: '#/components/responses/500'

'503':

$ref: '#/components/responses/503'

'520':

$ref: '#/components/responses/520'

default:

$ref: '#/components/responses/default'

callbacks:

onPartnerStatusEvent:

'{$request.body#/federationNotificationDest}':

post:

parameters:

-   name: federationContextIdentifier

in: path

required: true

schema:

$ref:  '#/components/schemas/FederationContextId'

requestBody:

description: Notification payload.

content:

application/json:

schema:

type: object

required:

- objectType

- operationType

- modificationDate

properties:

objectType:

type: string

enum:

- FEDERATION

- ZONES

- EDGE_DISCOVERY_SERVICE

- LCM_SERVICE

- MOBILE_NETWORK_CODES

- FIXED_NETWORK_CODES

operationType:

type: string

enum:

- STATUS_CHANGE

- UPDATE_SERVICE

- ADD_CODES

- REMOVE_CODES

- UPDATE_CODES

- ADD_ZONES

- REMOVE_ZONES

- UPDATE_ZONES

- UPDATE_ZONE_STATUS

edgeDiscoverySvcEndPoint:

$ref: '#/components/schemas/ServiceEndpoint'

lcmSvcEndPoint:

$ref: '#/components/schemas/ServiceEndpoint'

addMobileNetworkIds:

$ref: '#/components/schemas/MobileNetworkIds'

removeMobileNetworkIds:

$ref: '#/components/schemas/MobileNetworkIds'

addFixedNetworkIds:

$ref: '#/components/schemas/FixedNetworkIds'

removeFixedNetworkIds:

$ref: '#/components/schemas/FixedNetworkIds'

addZones:

type: array

items:

$ref: '#/components/schemas/ZoneDetails'

description: List of zones, which the operator platform wishes to make available to developers/ISVs of  requesting operator platform.

minItems: 1

removeZones:

type: array

items:

$ref: '#/components/schemas/ZoneIdentifier'

description: List of zones, which the operator platform no longer wishes to share.

minItems: 1

zoneStatus:

type: array

items:

type: object

required:

- zoneId

- status

properties:

zoneId:

$ref: '#/components/schemas/ZoneIdentifier'

status:

$ref: '#/components/schemas/Status'

minItems: 1

modificationDate:

type: string

format: date-time

description: Date and time of the federation  modification by the originating partner OP

responses:

'204':

description: Expected response to a successful callback processing

'400':

$ref: '#/components/responses/400'

'401':

$ref: '#/components/responses/401'

'404':

$ref: '#/components/responses/404'

'409':

$ref: '#/components/responses/409'

'422':

$ref: '#/components/responses/422'

'500':

$ref: '#/components/responses/500'

'503':

$ref: '#/components/responses/503'

'520':

$ref: '#/components/responses/520'

default:

$ref: '#/components/responses/default'

/partner/{federationContextId}:

get:

summary: Retrieves details about the federation context with the partner OP. The response shall provide info about the zones offered by the partner, partner OP network codes, information about edge discovery and LCM service etc.

operationId: GetFederationDetails

tags:

- FederationManagement

parameters:

-   name: federationContextId

in: path

required: true

schema:

$ref:  '#/components/schemas/FederationContextId'

responses:

'200':

description: Federation meta-info request accepted

content:

application/json:

schema:

type: object

required:

- edgeDiscoveryServiceEndPoint

- lcmServiceEndPoint

properties:

edgeDiscoveryServiceEndPoint:

$ref: '#/components/schemas/ServiceEndpoint'

lcmServiceEndPoint:

$ref: '#/components/schemas/ServiceEndpoint'

allowedMobileNetworkIds:

$ref: '#/components/schemas/MobileNetworkIds'

allowedFixedNetworkIds:

$ref: '#/components/schemas/FixedNetworkIds'

offeredAvailabilityZones:

type: array

items:

$ref: '#/components/schemas/ZoneDetails'

minItems: 1

'400':

$ref: '#/components/responses/400'

'401':

$ref: '#/components/responses/401'

'404':

$ref: '#/components/responses/404'

'409':

$ref: '#/components/responses/409'

'422':

$ref: '#/components/responses/422'

'500':

$ref: '#/components/responses/500'

'503':

$ref: '#/components/responses/503'

'520':

$ref: '#/components/responses/520'

default:

$ref: '#/components/responses/default'

delete:

summary: Remove existing federation with the partner OP

operationId: DeleteFederationDetails

tags:

- FederationManagement

parameters:

-   name: federationContextId

in: path

required: true

schema:

$ref:  '#/components/schemas/FederationContextId'

responses:

'200':

description: Federation removed successfully

'400':

$ref: '#/components/responses/400'

'401':

$ref: '#/components/responses/401'

'404':

$ref: '#/components/responses/404'

'409':

$ref: '#/components/responses/409'

'422':

$ref: '#/components/responses/422'

'500':

$ref: '#/components/responses/500'

'503':

$ref: '#/components/responses/503'

'520':

$ref: '#/components/responses/520'

default:

$ref: '#/components/responses/default'

/zones/{federationContextId}:

post:

summary: Registers zones offered by the partner OP.Originating OP informs partner OP that it is willing to access the specified zones  and partner OP shall reserve compuational and network resources for these zones.

operationId: RegisterZones

tags:

- AvailabilityZoneInfoSynchronization

parameters:

- name: federationContextId

in: path

required: true

schema:

$ref:  '#/components/schemas/FederationContextId'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/ZoneRegisterationRequestData'

required: true

responses:

'200':

description: Zone registered successfully

content:

application/json:

schema:

$ref:  '#/components/schemas/ZoneRegisterationResponseData'

'400':

$ref: '#/components/responses/400'

'401':

$ref: '#/components/responses/401'

'404':

$ref: '#/components/responses/404'

'409':

$ref: '#/components/responses/409'

'422':

$ref: '#/components/responses/422'

'500':

$ref: '#/components/responses/500'

'503':

$ref: '#/components/responses/503'

'520':

$ref: '#/components/responses/520'

default:

$ref: '#/components/responses/default'

callbacks:

onZoneResourceUpdateEvent:

'{$request.body#/availZoneNotifLink}':

post:

parameters:

- name: federationContextId

in: path

required: true

schema:

$ref:  '#/components/schemas/FederationContextId'

- name: zoneId

in: path

required: true

schema:

$ref:  '#/components/schemas/ZoneIdentifier'

requestBody:

description: Notification about resource availability.

content:

application/json:

schema:

type: object

required:

- zoneResUpdInfo

properties:

zoneResUpdInfo:

type: array

items:

type: object

minProperties: 1

properties:

availableCompResources:

description: Resources exclusively reserved for the originator OP.

type: array

items:

$ref: '#/components/schemas/ComputeResourceInfo'

minItems: 1

availableNetResources:

type: object

properties:

egressBandWidth:

type: integer

format: int32

description: Max dl thoughput that this edge can offer. It is defined in Mbps.

dedicatedNIC:

type: integer

format: int32

supportSriov:

type: boolean

description: If this zone support SRIOV networks or not

supportDPDK:

type: boolean

description: If this zone supports DPDK based networking

minProperties: 1

responses:

'200':

description: Resource data updated

'400':

$ref: '#/components/responses/400'

'401':

$ref: '#/components/responses/401'

'404':

$ref: '#/components/responses/404'

'409':

$ref: '#/components/responses/409'

'422':

$ref: '#/components/responses/422'

'500':

$ref: '#/components/responses/500'

'503':

$ref: '#/components/responses/503'

'520':

$ref: '#/components/responses/520'

default:

$ref: '#/components/responses/default'

/zones/{federationContextId}/{zoneId}:

delete:

summary: Deregister a zone of partner OP. Originating OP informs partner OP that it will no longer access the specified zone.

operationId: DeregisterZones

tags:

- AvailabilityZoneInfoSynchronization

parameters:

- name: federationContextId

in: path

required: true

schema:

$ref:  '#/components/schemas/FederationContextId'

- name: zoneId

in: path

required: true

schema:

$ref:  '#/components/schemas/ZoneIdentifier'

responses:

'200':

description: Zone deregistered successfully

'400':

$ref: '#/components/responses/400'

'401':

$ref: '#/components/responses/401'

'404':

$ref: '#/components/responses/404'

'409':

$ref: '#/components/responses/409'

'422':

$ref: '#/components/responses/422'

'500':

$ref: '#/components/responses/500'

'503':

$ref: '#/components/responses/503'

'520':

$ref: '#/components/responses/520'

default:

$ref: '#/components/responses/default'

get:

summary: Retrieves details about the computation and network resources that partner OP has reserved for this zone.

operationId: GetZoneData

tags:

- AvailabilityZoneInfoSynchronization

parameters:

- name: federationContextId

in: path

required: true

schema:

$ref:  '#/components/schemas/FederationContextId'

- name: zoneId

in: path

required: true

schema:

$ref:  '#/components/schemas/ZoneIdentifier'

responses:

'200':

description: Zone metadata

content:

application/json:

schema:

type: object

required:

- zoneId

- reservedResources

- resourceQuotaLimits

- flavorsSupported

properties:

zoneId:

$ref: '#/components/schemas/ZoneIdentifier'

reservedComputeResources:

description: Resources exclusively reserved for the originator OP.

type: array

items:

$ref: '#/components/schemas/ComputeResourceInfo'

minItems: 1

computeResourceQuotaLimits:

description: Max quota on  resources  partner OP allows over reserved resources.

type: array

items:

$ref: '#/components/schemas/ComputeResourceInfo'

minItems: 1

flavorsSupported:

type: array

items:

$ref: '#/components/schemas/flavor'

minItems: 1

networkResources:

type: object

required:

- egressBandWidth

- dedicatedNIC

- supportSriov

- supportDPDK

properties:

egressBandWidth:

type: integer

format: int32

description: Max dl thoughput that this edge can offer. It is defined in Mbps.

dedicatedNIC:

type: integer

format: int32

supportSriov:

type: boolean

description: If this zone support SRIOV networks or not

supportDPDK:

type: boolean

description: If this zone supports DPDK based networking.

'400':

$ref: '#/components/responses/400'

'401':

$ref: '#/components/responses/401'

'404':

$ref: '#/components/responses/404'

'409':

$ref: '#/components/responses/409'

'422':

$ref: '#/components/responses/422'

'500':

$ref: '#/components/responses/500'

'503':

$ref: '#/components/responses/503'

'520':

$ref: '#/components/responses/520'

default:

$ref: '#/components/responses/default'

Document Management

Document History

| Version | Date | Brief Description of Change | Approval Authority | Editor / Company |
| --- | --- | --- | --- | --- |
| 0.9 | 23-06-2022 | New OPAG Block A API Specification. | GSMA OPAG |  |