# Operator Platform – Southbound Interface Network Resources APIs

# Version 0.3

# [Publication Date]

This is a draft Non-binding Permanent Reference Document of the GSMA

Security Classification: Non-confidential

Access to and distribution of this document is restricted to the persons permitted by the security classification. This document is confidential to the Association and is subject to copyright protection. This document is to be used only for the purposes for which it has been supplied and information contained in it must not be disclosed or in any other way made available, in whole or in part, to persons other than those permitted under the security classification without the prior written approval of the Association.

Copyright Notice

Copyright © 2022 GSM Association

Disclaimer

The GSM Association (“Association”) makes no representation, warranty or undertaking (express or implied) with respect to and does not accept any responsibility for, and hereby disclaims liability for the accuracy or completeness or timeliness of the information contained in this document. The information contained in this document may be subject to change without prior notice.

Antitrust Notice

The information contain herein is in full compliance with the GSM Association’s antitrust compliance policy.

Table of Contents

1	Introduction	4

1.1	Overview	4

1.2	Scope	4

1.3	Definitions	5

1.4	Abbreviations	5

1.5	References	6

1.6	Conventions	6

1.7	Summary SDO Reference Mapping Table	6

2	Network Integration Support APIs in SBI-NR Interface	7

2.1	Network Events API	7

2.1.1	Description	7

2.1.2	Requirements and Service Aspects	7

2.1.3	Procedures	7

2.1.4	API	7

2.2	QoS Management API	8

2.2.1	Description	8

2.2.2	Requirement and Service Aspects	8

2.2.3	Procedures	8

2.2.4	API	9

2.3	Traffic Influence API	9

2.3.1	Description	9

2.3.2	Requirement and Service Aspects	9

2.3.3	Procedures	9

2.3.4	API	9

2.4	Application Relocation API	10

2.4.1	Description	10

2.4.2	Requirement and Service Aspects	10

2.4.3	Procedures	10

2.4.4	API	11

2.5	Confirm User Location API	11

2.5.1	Description	11

2.5.2	Requirement and Service Aspect	11

2.5.3	Procedures	11

2.5.4	API	11

2.6	Mobility Triggers API	12

2.6.1	Description	12

2.6.2	Requirement and Service Aspects	12

2.6.3	Procedures	12

2.6.4	API	12

2.7	Mobility Control API	12

2.7.1	Description	12

2.7.2	Requirements and Service Aspects	13

2.7.3	Procedures	13

2.7.4	API	13

Annex A	Document Management	14

A.1	Document History	14

A.2	Other Information	14

# Introduction

## Overview

The Operator Platform's (OP) Network Integration Support Application Programming Interfaces (APIs) are mainly related to the Telco Network Interfaces (i.e. the Southbound Interface-Network Resources ,SBI-NR), but also aiming to provide the end-to-end fulfilment of the specific topic, hence including any other OP Interface as described in the OP Reference Architecture of the OP Requirements PRD [1].

The list of the Topics and the associated APIs in the SBI-NR interface are presented in the Table 1 below. Grey items are still under analysis and will be available in a later release.

| No | OPAG APIs in SBI-NR Interface (Phase 1) | OP Interface |
| --- | --- | --- |
| 1 | Network Events | SBI-NR |
| 2 | QoS Management | SBI-NR |
| 3 | Traffic Influence | SNI-NR |
| 4 | Application Relocation | SBI-NR |
| 5 | Confirm User Location | SBI-NR |
| 6 | Mobility Triggers | SBI-NR |
| 7 | Mobility Control | SBI-NR |
| 8 | Managing Service Availability in LADN | SBI-NR |
| 9 | User Location Privacy Indicator | SBI-NR |
| 10 | User Authentication and Authorisation (defined in Block D) | SBI-NR |

: List of APIs in SBI-NR Interface

The purpose of this document is to provide the API requirements and the Standards Developing Organisation (SDO) reference mapping for each API listed.

The structure to document each API consists of:

Descriptions: summary of the expected use of the API

Requirement and Service Aspects: References to requirements related to the API identified in OP Requirements PRD [1]

Procedures: References to procedures and flows from an SDO's specification (e.g. 3GPP) that match with the OP's view for the API.

API: References to API (parameters, HTTP solution and YAML) located in SDOs specifications.

The main reference source of the API Requirements is the “OP PRD Reference Architecture” document [1]. Other sources for the API Requirements are found in the relevant SDO's reference documents definitions.

## Scope

The present document aims to define OP APIs that are related to SBI-NR Interface in the OP architecture. This set of APIs define the Network Integration Support APIs.

## Definitions

| Term | Description |
| --- | --- |
| Network Events | The OP in the role of AF would need to manage network events and notifications over the SBI-NR interface (NEF APIs) and orchestrate edge Application Instances in target Cloudlets and synchronise the associated application states to provide application Session Continuity. (from [1]) |
| Application Relocation | The Operator Platform shall be able to consider the application-specific requirements for managing mobility over different edge nodes. (from [1]) |
| Mobility Triggers | Many different elements shall monitor and control the end-to-end service delivery for detecting any modification and trigger a change on the path. (from [1]) |
| Mobility Control | Because of this User Equipment (UE) mobility, or because of the OP’s measurements or knowledge, or hints from the application about performance degradations, the OP may decide that a different edge compute resource can better host the Edge Application. An OP may be able to negotiate the UE data plane mobility process based on the application instance relocation process. An OP may be able to specify the request for routing, influencing network mobility and routing. (from [1]) |
| Traffic Influence | Based on some of the events on the SBI-NR interface, e.g. location monitoring events, QoS status notification events etc., the OP may determine the level of QoS provided by the mobile network to application sessions against the Quality of Service (QoS) level requested by the application. In such cases, the OP may initiate the user plane relocation (e.g., by using Traffic Influence APIs) services on the SBI-NR interface. Possibly this may result in the triggering of session mobility procedures in the mobile network. (from [1]) |

## Abbreviations

| Term | Description |
| --- | --- |
| 5QI | 5G QoS Identifiers |
| AF | Application Function |
| API | Application Programming Interface |
| AS | Application Server |
| EAC | Edge Application Client |
| EAS | Edge Application Server |
| EEC | Edge Enabled Client |
| EES | Edge Enabled Server |
| NBI | Northbound Interface |
| NEF | Network Exposure Function |
| OP | Operator Platform |
| OPG | Operator Platform Group |
| PRD | Permanent Reference Document |
| QoE | Quality of Experience |
| QoS | Quality of Service |
| SBI-NR | Southbound Interface – Network Resources |
| SCEF | Service Capability Exposure Function |
| SDO | Standards Developing Organisation |
| UE | User Equipment |

## References

| Ref | Doc Number | Title |
| --- | --- | --- |
|  | OP PRD | Operator Platform Telco Edge Requirements”, Version 2.0, 14 April 2022 |
|  | RFC 2119 | “Key words for use in RFCs to Indicate Requirement Levels”, S. Bradner, March 1997. Available at http://www.ietf.org/rfc/rfc2119.txt |
|  | 3GPP TS 29.522 | 5GS, Network Exposure Function Northbound APIs, Stage 3, Release 17.6, June 2022 |
|  | 3GPP TS 23.558 | 5GS, Architecture for enabling Edge Applications, Release 17, June 2022 |
|  | 3GPP TS 29.558 | 5GS, Enabling Edge Applications API Specifications, Stage 3, Release 17, June 2022 |
|  | 3GPP TS 29.591 | 5GS, Network Exposure Function Southbound Services, Stage 3, Release 17, June 2022 |
|  | 3GPP TS 23.502 | Procedures for the 5G Systems (5GS), Stage 2, Release 17, June 2022 |

## Conventions

The key words “must”, “must not”, “required”, “shall”, “shall not”, “should”, “should not”, “recommended”, “may”, and “optional” in this document are to be interpreted as described in RFC2119 [2].

## Summary SDO Reference Mapping Table

The OP's SBI-NR Interface is linking with the 3GPP 5G Core Network Exposure Function (NEF). Table below summarises the SDO Reference mapping between the OP SBI-NR and 3GPP NEF in its Northbound Interface (NBI) and associated APIs.

| No | APIs in SBI-NR Interface | OP Interface | SDO Reference Mapping - 3GPP |
| --- | --- | --- | --- |
| 1 | Network Events | SBI-NR | 3GPP TS 29.522 |
| 2 | QoS Management | SBI-NR | 3GPP TS 29.522/29.122 |
| 3 | Traffic Influence | SNI-NR | 3GPP TS 29 522 |
| 4 | Application Relocation | SBI-NR | 3GPP TS 29 558 |
| 5 | Confirm User Location | SBI-NR | 3GPP TS 29.522/29.122 3GPP TS 29.558 |
| 6 | Mobility Triggers | SBI-NR | 3GPP TS 29.522/29.122 |
| 7 | Mobility Control | SBI-NR | 3GPP TS 29.522 |

# Network Integration Support APIs in SBI-NR Interface

## Network Events API

### Description

Goal of the OP's Network Events service is to provide interface for Operators and Application Providers to support enhanced application-aware exposure for 5G network events including federated topologies. Two interfaces between 5G System, Edge System and Application Client should be described:

Collecting Network Status Events SBI-NR which supports User Plane related notifications (Session Establish, UE IP address allocation, Access and Mobility with Registration, Connection, Reachability and Mobility Management) and other events related to application traffic. Those events are mostly exposed by the 4G Service Capability Exposure Function (SCEF)/5G NEF.

Network Events NBI exposed by an OP to an Application Provider allowing introducing context awareness regarding occurred network events and provide support for quality, performance and security to ensure the required Quality of Experience (QoE) of the OP's customers.

### Requirements and Service Aspects

The requirements for the API are specified in the OP Requirements PRD Document [1] in the following sections:

| Section | Document | Reference Section Title |
| --- | --- | --- |
| 3.5.1.7.3 | OPG.02 | Network Event Support (*UE location information and events; * UE network connection events; *Application to UE connection) |
| 3.5.2.2.1 | OPG.02 | Network (*Collecting radio network information, e.g. cell change notification, measurement reports etc. for mobility decisions) |

### Procedures

The following procedures defined in 3GPP shall apply to the OP Network Events capabilities:

| NF | Section | TS | Procedure Name |
| --- | --- | --- | --- |
| NEF/SCEF | 4.11.6.3 | 3GPP 23.502 | Nnef_EventExposure_Subscribe (*Configuration of monitoring events for common network exposure) |

### API

The following API as defined in 3GPP shall be endorsed for Operator Platform

| NF | Section | TS | API name |
| --- | --- | --- | --- |
| SCEF | 4.4.2 | 29.122 | Procedures over T8 reference point / Monitoring Procedures) (*Loss of connectivity; UE reachability; Location Reporting; Change of IMSI-IMEI(SV) Association; Roaming Status; Communication Failure; PDN connectivity status; Availability after DDN Failure; API support capability.) |
| SCEF | Annex A (pp. 323) | 29.122 | Monitoring Event API - YAML |

The following API parameters have been identified as missing in 3GPP

| NF | Section | TS | API name |
| --- | --- | --- | --- |
| NEF | 4.4.2 | 29.522 | NEF Northbound Interface / Procedures for Monitoring (*29.122 emphasizes SCEF-T8-Procedures can be applied in the NEF as well) |

## QoS Management API

### Description

The purpose of the QoS Management API is to enable an OP the ability to expose the network driven capabilities related to 5G QoS Identifiers (5QI). With this API, the OP provides to 3rd party consumer the capability to adapt the quality and capacity of the network based on its application requirements. This kind of customization can be applied to many Business-to-Business (V2x, drone control, etc.) and Business-to-Consumer (immersive communications, gaming, etc.) use cases where the quality can be adapted and configured in the network to guarantee the best user experience.

For this API definition it makes sense to reuse existing 3GPP NEF/SCEF API for QoS management. The corresponding API in the NBI interface will be defined by the Linux Foundation's CAMARA project considering mapping to the references included in the following sections.

### Requirement and Service Aspects

The requirements for the API are specified in the OP Requirements PRD [1] in the following section(s).

| Section | Document | Reference Section Title |
| --- | --- | --- |
| 5.1.4 | OPG.02 | Southbound Interface to Network Resources |

### Procedures

The following procedures defined in 3GPP shall apply to the OP's QoS Management capabilities:

| NF | Section | TS | Procedure Name |
| --- | --- | --- | --- |
| NEF | 4.15.6.6/6a | 23.502 | Setting up an Application Function (AF) session with required QoS procedure |
| SCEF | 4.4.13 | 29.122 | Procedures for setting up an Application Server (AS) session with required QoS |
| NEF | 4.4.9 | 29.522 | Procedures for setting up an AF session with required QoS |

NOTE: 	The AsSessionWithQoS API is define in the NEF 3GPP spec as a reused API (section 5.3 TS 29.522). It adapts the development and definition from the SCEF API (4G architecture) to the NEF (5G Architecture).

### API

The following API as defined in 3GPP shall be endorsed for Operator Platform.

| NF | Section | TS | API name |
| --- | --- | --- | --- |
| NEF | 5.3 | 29.522 | AsSessionWithQoS (Reused APIs Table) |
| SCEF | 5.14 | 29.122 | AsSessionWithQoS |

## Traffic Influence API

### Description

The purpose of the Traffic Influence API is to enable Operator Platform (OP) the ability to expose the network driven capabilities to influence on the traffic transport paths between the UE and the Application Server hosted at the Cloud Resources. The selected API is based on 3GPP definitions. Some modifications the API would be required in the EWBI interface to be usable in the federated environment. The corresponding API in the NBI interface will be defined by the Linux Foundation's CAMARA project's specifications for the Service APIs in the NBI interface.

### Requirement and Service Aspects

The requirements for the API are specified in the OP Requirements PRD [1] in the following section(s).

| Section | Document | Reference Section Title |
| --- | --- | --- |
| 5.1.4 | OPG.02 | Southbound Interface to Network Resources |

### Procedures

The following procedures defined in 3GPP shall apply to the OP Traffic Influence capabilities.

| NF | Section | TS | Procedure Name |
| --- | --- | --- | --- |
| NEF | 4.3.6 | 23.502 R 17 | Application Function influence on traffic routing |
| NEF | 5.2.6.7 | 23.502 R 17 | Nnef_TrafficInfluence Service |
| NEF | 5.4 | 23.502 R 17 | TrafficInfluence |
| NEF | 4.4.7 | 29.522 R 17 | Procedures for Traffic Influence |

### API

The following API as defined in 3GPP shall be endorsed for Operator Platform.

| NF | Section | TS | API name |
| --- | --- | --- | --- |
| NEF | 5.4 | 29.522 R 17 | TrafficInfluence |

## Application Relocation API

### Description

The purpose of the Application Relocation API is to enable the Operator Platform (OP) to manage and maintain inter- and intra-connectivity between applications as they move between different cloud and network resources. This concept is aligned with the architecture and definitions in TS 23.558 [4] - Architecture for enabling Edge Applications.

### Requirement and Service Aspects

The requirements for the API are specified in the OP Requirements PRD [1] in the following section(s).

| Section | Document | Reference Section Title |
| --- | --- | --- |
| 4.7 | OPG.02 | Applications deployment in the Federated Operator Domain (Figure 15 and text) |
| 5.1.6.2.6 | OPG.02 | Service Provisioning |
| 5.2.2.3.6 | OPG.02 | Mobility Enforcements |
| 5.2.2.6.3 | OPG.02 | Network and OP responsibilities for application session continuity |
| 5.2.2.6.5 | OPG.02 | Edge Applications Role in Session Continuity Process |
| Annex F | OPG.02 | 5G Core Network Application Session Continuity Enabler Services |

### Procedures

The following procedures defined in 3GPP shall apply to the Application Relocation capabilities:

| NF | Section | TS | Procedure Name |
| --- | --- | --- | --- |
|  | 8.8 | 23.558 | Service Continuity |
| EES | 5.8 | 29.558 | Eees_ACRManagementEvent Service |
| EES | 5.9 | 29.558 | Eees_AppContextRelocation Service |
| EES | 5.11 | 29.558 | Eees_EELManagedACR Service |
| EES | 5.12 | 29.558 | Eees_ACRStatusUpdate Service |
| EES | 5.8.2.3 | 29.558 | Eees_ACRManagementEvent_UpdateSubscription |
| EES | 5.8.2.4 | 29.558 | Eees_ACRManagementEvent_Unsubscribe |
| EES | 5.8.2.5 | 29.558 | Eees_ACRManagementEvent_Notify |
| EES | 5.9.2.3 | 29.558 | Eees_AppContextRelocation_ACRDetermination_Request |
| EES | 5.11.2.2 | 29.558 | Eees_EELManagedACR_Request |
| EES | 5.11.2.3 | 29.558 | Eees_EELManagedACR_Subscribe |
| EES | 5.11.2.4 | 29.558 | Eees_EELManagedACR_Notify |
| EES | 5.12.2.2 | 29.558 | Eees_ACRStatusUpdate_Request |

### API

The following API as defined in 3GPP shall be endorsed for Operator Platform.

| NF | Section | TS | API name |
| --- | --- | --- | --- |
| EES | 8.7 | 29.558 | Eees_ACRManagementEvent API |
| EES | 8.9 | 29.558 | Eees_EECContextRelocation API |
| EES | 8.10 | 29.558 | Eees_EELManagedACR API |
| EES | 8.11 | 29.558 | Eees_ACRStatusUpdate API |

## Confirm User Location API

### Description

The purpose of the Confirm User Location API is to enable an OP with the ability to provide user location information to consumers. This can be used in combination with other network driven capabilities to trigger configurations within the network to provide services based on the location of the final users (e.g., to deploy resources and applications closer to the user so that latency can be reduced to the minimum and improve the user experience).

### Requirement and Service Aspect

The requirements for the API are specified in the OP Requirements PRD [1] in the following section(s).

| Section | Document | Reference Section Title |
| --- | --- | --- |
| 3.5.2.2.1 | OPG.02 | UC Location Retrieval |
| 5.1.6.2.4 | OPG.02 | Cloudlet selection |

### Procedures

The following procedures defined in 3GPP shall apply to the OP's user location information capabilities.

| NF | Section | TS | Procedure Name |
| --- | --- | --- | --- |
| NEF | 4.4.2 | 29.522 | Procedures for Monitoring |
| SCEF | 4.4.2 | 29.122 | Monitoring Procedures |
| EES | 8.6.2 | 23.558 | UE Location |

### API

The following API as defined in 3GPP shall be endorsed for Operator Platform

| NF | Section | TS | API name |
| --- | --- | --- | --- |
| SCEF | 5.3 | 29.122 | MonitoringEvent |
| NEF | 5.3 | 29.522 | MonitoringEvent (Reused APIs Table) |
| EES | 8.2 | 29.558 | Eees_UELocation |

## Mobility Triggers API

### Description

This API provides the required support from the network so that the OP can be informed about the need to move an application session to a different anchor point or of the actual move. A possible use case is when the user moves to a new location. By means of this API, the network alerts the OP about the change, so that the OP can perform the required new configurations for the user to access the applications in the new location.

### Requirement and Service Aspects

The requirements for the API are specified in the OP Requirements PRD [1] in the following section(s).

| Section | Document | Reference Section Title |
| --- | --- | --- |
| 2.2.7 | OPG.02 | Mobility Requirements (*5G Connectivity Models for Edge; *Roaming Requirements; *Geographic Conditions on Mobility) |
| 5.2.2.3.2 | OPG.02 | Mobility Management |

### Procedures

The following procedures defined in 3GPP shall apply to the Mobility Management capabilities.

| NF | Section | TS | Procedure Name |
| --- | --- | --- | --- |
| NEF | 4.4.2 | 29.522 | Procedures for Monitoring |
| SCEF | 4.4.2 | 29.122 | Monitoring Procedures |

### API

The following API as defined in 3GPP shall be endorsed for Operator platform.

| NF | Section | TS | API name |
| --- | --- | --- | --- |
| NEF | 5.3 | 29.522 | MonitoringEvent API (Reused API Table) |
| SCEF | 5.3 | 29.122 | MonitoringEvent API |

These APIs, defined in 3GPP, provides support to retrieve information related to different events related to the OP's Mobility Triggers concept. For instance:

LOSS_OF_CONNECTIVITY

UE_REACHABILITY

LOCATION_REPORTING

AREA_OF_INTEREST

## Mobility Control API

### Description

The Mobility Control API is responsible for controlling when an application session is to be moved to a different anchor point. For instance, due to a failure an application relocation occurs. After relocating an application, a change of anchor point may be required, if this is the case, the OP can use this API to adjust in the network the most appropriate configuration to access the application new endpoint.

### Requirements and Service Aspects

The requirements for the API is specified in the OP Requirements PRD [1] in the following sections:

| Section | Document | Reference Section Title |
| --- | --- | --- |
| 2.2.7.3 | OPG.02 | Requirements for Application Session Continuity |

### Procedures

The following procedures defined in 3GPP shall apply to the OP's traffic influence capabilities:

| NF | Section | TS | Procedure Name |
| --- | --- | --- | --- |
| NEF | 4.3.6 | 23.502 R 17 | Application Function influence on traffic routing |
| NEF | 5.2.6.7 | 23.502 R 17 | Nnef_TrafficInfluence Service |
| NEF | 5.4 | 23.502 R 17 | TrafficInfluence |
| NEF | 4.4.7 | 29.522 R 17 | Procedures for Traffic Influence |

### API

The following API as defined in 3GPP shall be endorsed for Operator Platform

| NF | Section | TS | API name |
| --- | --- | --- | --- |
| NEF | 5.4 | 29.522 R 17 | TrafficInfluence |

Document Management

Document History

| Version | Date | Brief Description of Change | Approval Authority | Editor / Company |
| --- | --- | --- | --- | --- |
| 0.3 | 11/8/2022 | Baseline of New PRD for review and approval in OPAG/OPG |  |  |

Other Information

| Type | Description |
| --- | --- |
| Document Owner | Operator Platform Group (OPG) |
| Editor / Company | Milan Lalovic, British Telecom |

It is our intention to provide a quality product for your use. If you find any errors or omissions, please contact us with your comments. You may notify us at prd@gsma.com

Your comments or suggestions & questions are always welcome.