| Change Request Form |  |  |
| --- | --- | --- |
|  | TS.43 Service Entitlement Configuration, v11.0 TS.43 CR1098 – App Token |  |
| Document Summary |  |  |
| Change Request Security Classification | Non-confidential |  |
| This document is for | Discussion |  |
| Input Editor and Organisation | Fernando Pascual Blanco (Telefónica) |  |
| Additional Contributors | Nacho Blázquez (HPE), Kay Fritz (Vodafone), Michele Zarri (Huawei), Florian-Leon Schmitt (DT) |  |
| Approving Group/Project | TSG & ISAG |  |
| Change Request Creation Date | 24 May 2024 |  |
| Reason for the Change Request? | As per NG.144 E2E-S Slicing B2B User Story, there are identified some gaps when building an end-to-end slicing user story for B2B use cases. Concretely, App token concept is conceived to fill some of those gaps and this CR is focused on the app token generation and consumption. |  |
| Change Request Version History |  |  |
| Meeting Doc No Note: TS.60 states each new version of a CR needs a new meeting document number. | CR No & Version | Change |

© GSMA © 2024. The GSM Association (“Association”) makes no representation, warranty or undertaking (express or implied) with respect to and does not accept any responsibility for, and disclaims liability for the accuracy or completeness or timeliness of the information contained in this document. The information contained in this document may be subject to change without prior notice. This document has been classified according to the GSMA Document Confidentiality Policy. GSMA meetings are conducted in full compliance with the GSMA Antitrust Policy.

Copy and paste the original text here making sure that section numbers are correct. Turn on “Track Changes” and make your proposed change.

Start of the first change

14. Device App authentication with OperatorToken

This section describes different use cases where the operator provides a token to authenticate subscribers and/or applications installed in the subscriber´s device.

- Operator token use case

- App token use case

For both cases:

- A 3rd party application is installed on the device and is not capable of performing TS.43 functions.

- The TS.43 client is able to call TS.43 methods, including EAP_AKA authentication.

- The 3rd party App and the ECS previously exchanged a shared security information to enable authentication. This could be done directly between ECS and app-backend or with a dedicated partner management system e.g. as described in GSMA IDG specifications.

- The 3rd party App is authorized to request app_authentication from the TS.43 client via device-specific access control.

- Note: ECS may implement additional features to enable enhanced access control mechanisms (e.g. Auth Server)

For these use cases the appID ap2015 is used in the requests.

14.1 OperatorToken use case

The benefit of this procedure is that an app can gain SIM based authentication, without requiring access to the EAP-AKA token or the temporary_token, so that the security and integrity of the tokens is maintained.

  - 14.1.1 Device App authentication Request Parameters

For device App authentication, devices require additional parameters in the HTTP requests, outside of the ones described in 2.3 and 6.2. Table 105 presents the new parameters and their associated operations.

| New GET parameters app authentication | Type | Values | Description |
| --- | --- | --- | --- |
| operation | String | AcquireOperatorToken | Indicates the operation requested by the TS.43 client |
| access_token (optional) | String | Used by the AcquireOperatorToken and AcquireTemporaryToken operation to verify the requesting application. This parameter is also used when consuming the OperatorToken through any of the operations described in section 14.6 |  |
|  |  | Any string value | Token based on pre-shared security information |
| client_id (conditional) | String | Used by the AcquireOperatorToken operation to identify the requesting application. Used in combination with the TemporaryToken serving as secret for authentication. This parameter will be mandatory for validateOperationToken in case the user wants to validate client_id for a specific token. It could be used in combination (for validation) with scope parameter. |  |
|  |  | Any string value | Identifier of the requesting application |
| scope (conditional) | String | Used by the AcquireOperatorToken operation to indicate the access privileges being requested for OperatorToken. Used in combination with client_id. This parameter will be mandatory for validateOperationToken in case the user wants to validate scope for a specific token. It could be used in combination (for validation) with client_id parameter. |  |
|  |  | Any string value | Indicates which access privileges are being requested for OperatorToken |

Table 105. New parameters for device app authentication

  - 14.1.2 AcquireOperatorToken Operation Configuration Parameters

Parameter names and presence:

- OperatorToken: Conditional. Operators token to allow authentication for a 3rd party application on the device that may not have the means to acquire token or temporary_token.

- OperatorTokenExpiry: Conditional. Indicates the time the provided OperatorToken expires.

- OperatorTokenAuthURL: Conditional. The URL to representing the endpoint when validating OperatorToken

- ClientID: Conditional. ID identifying the requesting application.

The different values for the configuration parameters of the operation AcquireOperatorToken are provided in Table 106

| “AcquireOperatorToken” configuration parameters | Type | Values | Description |
| --- | --- | --- | --- |
| OperatorToken (Conditional) | String | Any string value | This Operator token can be provided by the ECS if the requesting 3rd party application can be authenticated based on ClientID and access_token. The operator token can be used by the 3rd party application to authenticate the device against the app backend. |
| OperatorTokenExpiry (Conditional) | Timestamp | ISO 8601 format, of the form YYYY-MM-DDThh:mm:ssTZD | This UTC value provides the expiration time for the Operator token. After the time expiration the Operator token cannot be used for authentication. |
| OperatorTokenAuthURL (Conditional) | String | URL to validate OperatorToken | URL representing the endpoint to validate the OperatorToken |
| ClientID (Conditional) | String | Any string value | Identifies the app requesting the OperatorToken |

Table 106. Configuration Parameters – AcquireOperatorToken ODSA Operation

  - 14.1.3 AcquireOperatorToken Request Example

Table 107 presents an example for the AcquireOperatorToken operation for an ODSA application.

GET ? terminal_id = 06170799658& token = es7w1erXjh%2FEC%2FP8BV44SBmVipg& terminal_vendor = TVENDOR& terminal_model = TMODEL& terminal_sw_version = TSWVERS& entitlement_version = ENTVERS& client_id = 08723459340765ß91& scope = openid%20profile& app = ap2015& access_token = ab2d52xaix%2FEC%2FoMNs12Sammctz& operation = AcquireOperatorToken& vers = 1 HTTP/1.1 Host: entitlement.telco.net:9014 User-Agent: PRD-TS43 TVENDOR/TMODEL Primary-ODSA/TSWVERS OS-Android/8.0 Accept: text/html,application/xhtml+xml,application/xml;q=0.9,*/*;q=0.8 Accept-Language: en-US,en;q=0.5 Accept-Encoding: gzip, deflate Connection: keep-alive

Table 107. Example of an AcquireOperatorToken ODSA Request

  - 14.1.4 AcquireOperatorToken Response Example

Table 108 presents an example for the AcquireOperatorToken response in XML format to a Primary ODSA application. This response provides the TS.43 client with the OperatorToken to be used for an app authentication.

<?xml version="1.0"?> <wap-provisioningdoc version="1.1"> <characteristic type="VERS" <parm name="version" value="1"/> <parm name="validity" value="172800"/> </characteristic> <characteristic type="TOKEN"> <parm name="token" value="ASH127AHHA88SF"/> </characteristic> <characteristic type="APPLICATION"> <parm name="AppID" value="ap2015"/> <parm name="OperatorToken" value="A8daAd8ads7fau34789947kjhsfad;kjfh"/> <parm name="OperatorTokenExpiry" value="2019-01-29T13:15:31-08:00"/> <parm name="OperatorTokenAuthURL" value="http://verifyurl.example.net"/> <parm name="OperationResult" value="1"/> <parm name="ClientID" value="68485498622168489104"/> </characteristic> </wap-provisioningdoc>

Table 108. Example of an AcquireOperatorToken Response in XML

  - 14.1.5 Device App authentication with OperatorToken call flow.

Necessary preconditions for this use case:

- ECS and App Backend exchanged information for OperatorToken encryption.

- e.g. ECS uses Public Key of App Backend to encrypt OperatorToken. App-Backend can later decrypt OperatorToken with own private Key

- ECS and App Backend exchanged information for access_token validation.

- e.g. App Backend uses Public Key of ECS, and forwards information to the app-client on the device. Client can then use this information in the access_token

The workflow then follows as described in Figure 60:

- The 3rd party App requests an Operator Token from the TS.43 client of the device

- The TS.43 client initiates the EAP-AKA authentication procedure with the ECS, using app_ID ap2015.

- Device and ECS perform EAP-AKA authentication as described in section 2.8.1.

- The TS.43 requests a TemporaryToken, using the EAP-AKA token and the access_token of the 3rd party app. The operation_target should be AcquireOperatorToken.

- The ECS validates the request including the identifiers and the AuthToken. The access_token is validated with the information shared between the ECS and the app_backend. Optionally the ECS can also verify the access_token with the 3rd party app backend. When successful, ECS creates the temporary_token.

- ECS sends the AcquireTemporaryToken response including TemporaryToken, TemporaryTokenExpiry and the OperationTargets = “AcquireOperatorToken”. The client stores the temporary_token in a secured space not accessible to 3rd party apps.

- Client uses the temporary_token to acquire the OperatorToken. In the AcquireOperatorToken Request the client also provides the client_id, which uniquely identifies the app.
If the client & ECS support encrypting all information in one token, steps 6, 7 and 8 are optional here.
If steps 6, 7 and 8 are skipped the client should send the access_token in this step.

- ECS validates the temporary_token together with the client_id. e.g. the ECS could use OAuth with client_id and temporary_token as secret. If successful, the ECS generates the OperatorToken

- The 3rd Party App can use OperatorToken to authenticate at its own Backend Service. By using OperatorToken, the device is authenticated by the MNO based on the inserted SIM-Card

Figure 60. App Authentication using OperatorToken.

  - 14.1.6 Operator Token Consumption

An External Entity (e.g. Application Server, etc.), may use the OperatorToken implementing new operations that will be specific for ap2015. These new operations are identified in Table 109.

It’s important to note that external entity could not be a terminal but a server, but, even so, in the request there will be some parameters referring to terminal_* present on the requests as part of the RCC.14 standard. For these mandatory parameters, it is recommended to use dummy values.

| “Operator Token” operations | Section | Description |
| --- | --- | --- |
| ValidateOperatorToken | 14.6.1 | Validates the operator token for a specific client_id and/or scope. This operation requires as part of the request, at least, one of the following parameters to be checked: client_id, scope. |
| GetSubscriberDeviceInfo | 14.6.2 | Provides information related to the subscriber device that acquired the operator token. |

Table 109. Operations available for Operator Token usage

Operations in Table 109 needs to be mapped to one or more scopes for validation. This scope definition is out of scope of TS.43 and should be the ECS (as it is the system generating the operator token) the one taking care of this mapping.

Using operations like the ones defined in Table 109 is similar, and the flow will follow the example as described in Figure 61, where:

- The External Entity makes a request using the operator_token and for a specific operation.

- ECS checks the validity of the operator_token. Validation could also require crosschecking with requestor_id.

- Optional. Depending on the operation, ECS could require interacting with backend systems.

- As a result, ECS will send the response containing the response parameters specific to the operation.

Figure 61. Example for Operator Token Usage Flow

Application requests using operator token mainly differs in the operation parameter. Table 110 shows a generic example which could be applicable for any request.

GET ? requestor_id = 06170799658& terminal_vendor = TVENDOR& terminal_model = TMODEL& terminal_sw_version = TSWVERS& entitlement_version = ENTVERS& app = ap2015& operator_token = <OPERATOR_TOKEN>& operation = <OPERATION>& scope= <SCOPE>& access_token = <ACCESS_TOKEN>& // Optional vers = 1 HTTP/1.1 Host: entitlement.telco.net:9014 User-Agent: PRD-TS43 TVENDOR/TMODEL Primary-ODSA/TSWVERS OS-Android/8.0 Accept: text/html,application/xhtml+xml,application/xml;q=0.9,*/*;q=0.8 Accept-Language: en-US,en;q=0.5 Accept-Encoding: gzip, deflate Connection: keep-alive

Table 110. Generic operation request for specific Operator Token Usage

The same approach is used for the responses where the main difference between each of the operations will defer in the response parameters. Table 111 shows a generic example where the response contains two parameters (<Param_response_X> and <Param_response_Y>).

<?xml version="1.0"?> <wap-provisioningdoc version="1.1"> <characteristic type="VERS" <parm name="version" value="1"/> <parm name="validity" value="172800"/> </characteristic> <characteristic type="APPLICATION"> <parm name="AppID" value="ap2015"/> <parm name="OperationResult" value="1"/> <parm name="<Param_response_X>" value="<Param_response_X_value>"/> <parm name="<Param_response_Y>" value="<Param_response_Y_value>"/> </characteristic> </wap-provisioningdoc>

Table 111. Generic operation request for specific Operator Token Usage

IMPORTANT.- Note that Operation Token Usage responses (Table 110) do not contain a token for Fast Authentication.

    - 14.1.6.1 Operator Token Validation

Parameters in the response are described in Table 112.

| ValidateOperatorToken response parameters | Type | Values | Description |
| --- | --- | --- | --- |
| OperatorTokenValidity | Integer | Indicates if the parameters (client_id and/or scope) are valid ones for the specific operator token in the request. |  |
|  |  | 0 – NOT VALID | The operator token provided is not a valid one |
|  |  | 1 – VALID | The operator token provided is valid one |
| OperatorTokenValidatedParams (Optional) | String | Comma-separated list with parameters validated. | List the parameters that have been validated for the operator_token. |

Table 112. Response parameters for ValidateOperatorToken operation

Table 113 shows an example of a request for ValidateOperatorToken, validating the OperatorToken for a specific scope and client_id.

GET ? requestor_id = 06170799658& terminal_vendor = TVENDOR& terminal_model = TMODEL& terminal_sw_version = TSWVERS& entitlement_version = ENTVERS& app = ap2015& operator_token = ab2d52xaix%2FEC%2FoMNs12Sammctz& operation = ValidateOperatorToken& scope= "scope1"& client_id= "25625441& access_token = 32487234987238974& // Optional vers = 1 HTTP/1.1 Host: entitlement.telco.net:9014 User-Agent: PRD-TS43 TVENDOR/TMODEL Primary-ODSA/TSWVERS OS-Android/8.0 Accept: text/html,application/xhtml+xml,application/xml;q=0.9,*/*;q=0.8 Accept-Language: en-US,en;q=0.5 Accept-Encoding: gzip, deflate Connection: keep-alive

Table 113. ValidateOperatorToken Request example

Table 114 shows a response example for the previous (ValidateOperatorToken) request (Table 113).

<?xml version="1.0"?> <wap-provisioningdoc version="1.1"> <characteristic type="VERS" <parm name="version" value="1"/> <parm name="validity" value="172800"/> </characteristic> <characteristic type="APPLICATION"> <parm name="AppID" value="ap2015"/> <parm name="OperationResult" value="1"/> <parm name="OperatorTokenValidity" value="1"/> <parm name="OperatorTokenValidatedParams" value="scope,client_id"/> </characteristic> </wap-provisioningdoc>

Table 114. ValidateOperatorToken Response example

    - 14.1.6.2 Subscriber Device Information

Parameters in the response are described in Table 115.

| GetSubscriberDeviceInfo response parameters | Type | Values | Description |
| --- | --- | --- | --- |
| MSISDN | String | Any string value | E.164 formatted phone number. It is possible to provide the base64 encoding of the value by preceding it with encodedValue= |
| IMSI (Optional) | String | A 15 digits (max) string | International Mobile Subscriber Identity as per ITU E.212 or 3GPP TS 23.003 standards. |

Table 115. Response parameters for GetSubscriberDeviceInfo operation

Table 116 shows an example of a request for GetSusbcriberDeviceInfo.

GET ? requestor_id = 06170799658& terminal_vendor = TVENDOR& terminal_model = TMODEL& terminal_sw_version = TSWVERS& entitlement_version = ENTVERS& app = ap2015& operator_token = ab2d52xaix%2FEC%2FoMNs12Sammctz& operation = GetSubscriberDeviceInfo& access_token = 32487234987238974& // Optional vers = 1 HTTP/1.1 Host: entitlement.telco.net:9014 User-Agent: PRD-TS43 TVENDOR/TMODEL Primary-ODSA/TSWVERS OS-Android/8.0 Accept: text/html,application/xhtml+xml,application/xml;q=0.9,*/*;q=0.8 Accept-Language: en-US,en;q=0.5 Accept-Encoding: gzip, deflate Connection: keep-alive

Table 116. GetSubscriberDeviceInfo Request example

Table 117 shows a response example for the previous (GetSubscriberDeviceInfo) request (Table 116).

<?xml version="1.0"?> <wap-provisioningdoc version="1.1"> <characteristic type="VERS" <parm name="version" value="1"/> <parm name="validity" value="172800"/> </characteristic> <characteristic type="APPLICATION"> <parm name="AppID" value="ap2015"/> <parm name="OperationResult" value="1"/> <parm name="MSISDN" value="34616210000"/> <parm name="IMSI" value="214990011223344"/> </characteristic> </wap-provisioningdoc>

Table 117. GetSubscriberDeviceInfo Response example

## 14.2 App token use case

For some use cases, like accessing operator special network capabilities, there might be situations where the device needs to be aware of application information managed by the network operator. For this case, devices require additional parameters in the HTTP requests, outside of the ones described in 2.3, 6.2 and 14.1.1, which are described in the following sections presenting the new parameters and their associated operations.

### App Token consumption

For the consumption of the App token, the new operation and parameters described in table ff are needed.

App token may be used only once, and once it is consumed by a device invoking the Get3ppAppInfo operation, it shall become useless (further operations shall return an error). Additionally, it should have associated a short expiration time (minutes range).

| New GET parameters app information | Type | Values | Description |
| --- | --- | --- | --- |
| operation | String | Get3PAppInfo | Indicates the operation requested by the TS.43 client |
| App_token | String | Used by the Get3PAppInfo operation to verify the application information managed by the operator. |  |
|  |  | Any string value | Token based on pre-shared security information. It can be generated as per section 2.8.3. Note that App_token is going to be used by the ECS to identify the 3rd party application backend (server) who requested this token. |

Table ff. Configuration Parameters – Get3PAppInfo ODSA Operation

In table gg there is also a description about the parameters and presence needed for the Get3PAppInfo response.

Parameter names and presence:

- 3PAppNameAnon: Mandatory. The application name anonymized corresponding to the operator managed application information. Real application name cannot be obtained by the operator from this parameter because only 3rd party application server should have the keys to do it.

| “Get3PAppInfo” configuration parameters | Type | Values | Description |
| --- | --- | --- | --- |
| 3PAppNameAnon | String | Any string value | 3rd party application name anonymized by the 3rd party application server as per application information managed by the operator. |

Table gg. Configuration Parameters – Get3PAppInfo ODSA Operation

Table hh presents an example for the Get3PAppInfo operation for an ODSA application.

GET ? terminal_id = 06170799658& token = es7w1erXjh%2FEC%2FP8BV44SBmVipg& terminal_vendor = TVENDOR& terminal_model = TMODEL& terminal_sw_version = TSWVERS& entitlement_version = ENTVERS& app = ap2015& App_token = ab2d52xaix%2FEC%2FoMNs12Sammctz& operation = Get3PAppInfo& vers = 1 HTTP/1.1 Host: entitlement.telco.net:9014 User-Agent: PRD-TS43 TVENDOR/TMODEL Primary-ODSA/TSWVERS OS-Android/8.0 Accept: text/html,application/xhtml+xml,application/xml;q=0.9,*/*;q=0.8 Accept-Language: en-US,en;q=0.5 Accept-Encoding: gzip, deflate Connection: keep-alive

Table hh. Example of an Get3PAppInfo ODSA Request

Table ii presents an example for the Get3PAppInfo response in XML format to a Primary ODSA application. This response provides the TS.43 client with the 3rd party application info managed by the operator.

<?xml version="1.0"?> <wap-provisioningdoc version="1.1"> <characteristic type="VERS" <parm name="version" value="1"/> <parm name="validity" value="172800"/> </characteristic> <characteristic type="TOKEN"> <parm name="token" value="ASH127AHHA88SF"/> </characteristic> <characteristic type="APPLICATION"> <parm name="AppID" value="ap2015"/> <parm name="3PAppNameAnon" value="ZacaPWVIH44fCZ3D"/> </characteristic> </wap-provisioningdoc>

Table ii. Example of a Get3PAppInfo Response in XML

### Discovering 3rd party application information callflow

Necessary preconditions for this use case:

- ECS and App Backend exchange information for App_token generation. App Backend delivers App token to 3rd party application installed in the device.

The workflow then follows as described in Figure ee:

The 3rd party App provides the App token to the TS.43 client of the device to request 3rd party application information.

The TS.43 client initiates the EAP-AKA authentication procedure with the ECS, using app_ID ap2015. Device and ECS perform EAP-AKA authentication as described in section 2.8.1.

Client requests 3rd party application information by using Get3PAppInfo operation providing App_token as input attribute.

The App_token is used by the ECS to obtain the application information associated to the application backend (server).

ECS sends Get3PAppInfo response, including 3pAppNameAnon.

3rd party application information managed by the operator can be used in the device.

Figure jj. Obtaining app information managed by the operator by using App token

End of the first change

Start of the second change

- A.12 Operator-based token authentication features

| Procedure | Operation | Entitlement Client |  | SP Entitlement Server |  |
| --- | --- | --- | --- | --- | --- |
| Authenticate the Subscriber Identity | Embedded EAP-AKA authentication | M | 2.8.1 | M | 2.8.1 |
|  | Fast authentication | O | 2.8.5 | O | 2.8.5 |
|  | Use a temporary token for AcquireOperatorToken operation (AcquireTemporaryToken) | M M | 6.1, 6.2 6.5.7 | M M | 6.2 6.5.7 6.6.6 |
|  | TemporaryToken Error Handling | O | 2.8.6 | O | 2.8.6 |
|  | AcquireOperatorToken | M | 14.1.1 14.1.2 | M | 14.1.1 14.1.2 |
| Consuming Operator Token | ValidateOperatorToken | M | 14.1.6 14.1.6.1 | M | 14.1.6 14.1.6.1 |
|  | GetSubscriberDeviceInfo | M | 14.1.6 14.1.6.2 | M | 14.1.6 14.1.6.2 |
| Consuming App token | Get3PAppInfo | M | 14.2 14.2.1 | M | 14.2 14.2.1 |

Table kk. Features & operations mapping for operator-based token features

End of the second change