| Change Request Form |  |
| --- | --- |
|  | NG.116 CR1055 On clarification of Maximum Up-link and Down-link throughput per device |
| Document Summary |  |
| Official Document Number, Document Title and Version Number | NG.116 Generic Network Slice Template v4.0 (Current) |
| Official Document Type | Non-binding Permanent Reference Document |
| Change Request Security Classification | Non-confidential |
| Is this a new document or a Major or Minor Change? | Major Update |
| Will this Change Request result in a Major or Minor version update? | Major Version |
| This document is for | Approval |
| Input Editor and Organisation | Alessio Casati (Nokia) |
| Additional Contributors | Kathleen Leach(T-Mobile); Ralf Keller(Ericsson), |
| Issuing Group/Project | NG |
| Approving Group/Project | NG |
| Change Request Creation Date | 29/01/2021 |
| What are the reasons for and benefits of creating this new document or Change Request? | these attributes are per UE but different service categories may be assigned different values of the rate in the subscription. |

© GSMA © 2021. The GSM Association (“Association”) makes no representation, warranty or undertaking (express or implied) with respect to and does not accept any responsibility for, and disclaims liability for the accuracy or completeness or timeliness of the information contained in this document. The information contained in this document may be subject to change without prior notice. This document has been classified according to the GSMA Document Confidentiality Policy. GSMA meetings are conducted in full compliance with the GSMA Antitrust Policy.

| Review Log (to be completed by GSMA Support Staff) |  |  |  |
| --- | --- | --- | --- |
| Workflow Step | Document Review Comments | GSMA Support Staff Name | Comments Date |
| Step 1: Change Request Creation (no comments required) |  |  |  |
| Step 2: Document Quality and/or Legal Review |  |  |  |
| Document Quality Team | INSERT COMMENTS HERE Please enter details for the Quality Review Confirm Document Quality Team feedback Record any issues, actions and key decisions | Javier Sendin | 01/02/2021 |
| Legal Review | INSERT COMMENTS HERE Please enter details for the Legal Review Confirm Legal feedback Record any issues, actions and key decisions | GSMA Support Staff Name | DD/MM/YY |
| Step 3: Formal Review |  |  |  |
| Group(s)/Project(s) Review(s) Comments and Feedback | INSERT COMMENTS HERE Please enter details for the Group(s)/Project(s) Review(s) Record any issues, actions and key decisions Confirm outcome of Formal Review | GSMA Support Staff Name | DD/MM/YY |
| Step 4: Formal Approval(s) |  |  |  |
| Group(s)/Project(s) Approval(s) Comments and Feedback | INSERT COMMENTS HERE Please enter details for the Group(s)/Project(s) Approval(s) Record any issues, actions and key decisions Confirm outcome of Formal Approval | GSMA Support Staff Name | DD/MM/YY |

FIRST CHANGE

### Downlink maximum throughput per UE

This attribute describes the maximum data rate supported by the network slice per UE in downlink..

The attribute is comprised of a list of Service Category parameters with the associated Maximum Downlink Throughput per UE value.

If no Service Category parameter is present all devices in the network slice, experience the same Maximum Downlink Throughput per UE value. If Service Category parameters are present, then the UEs will be associated with a specific Maximum Downlink Throughput per UE parameter value depending on which Service Category the UE is associated with for the network slice. In this case, different UEs within a network slice can experience a different maximum data rate.

Examples:

- 50 000 (Service Category 1)

- 400 000 (Service Category 2)

- 1 000 000 (Service Category 3)

Maximum Downlink Throughput per UE value

This parameter defines the Maximum Downlink Throughput per UE value. This may be associated with a Service Category parameter.

| Parameters |  |
| --- | --- |
| Measurement unit | kbps |
| Allowed Values | Any desired value |
| Tags | Character attribute /Performance KPI |

| Attribute Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional |  |
| Optional | X |

Maximum Downlink Throughput per UE Table

Service Category

This parameter defines a service category which may be assigned to certain groups of devices using the network slice. If present, it shall be associated with a Maximum Downlink Throughput Value parameter.

| Parameters |  |
| --- | --- |
| Measurement unit | NA |
| Allowed Values | Any text identifying the Service Category |
| Tags | Character attribute / Functional |

| Attribute Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional |  |
| Optional | X |

Downlink Maximum Rate Service Category Table

Additional information

Maximum throughput can be used to offer different service categories which have different maximum throughput values.

This attribute applies to 3GPP access type only, across GBR and Non-GBR QoS flows.

SECOND CHANGE

### Uplink maximum throughput per UE

This attribute describes the maximum data rate supported by the network slice per UE in uplink..

The attribute is comprised of a list of Service Category parameters with the associated Maximum Uplink Throughput per UE value.

If no Service Category parameter is present, all devices in the network slice experience the same Maximum Uplink Throughput per UE value. If Service Category parameters are present, then the UEs will be associated with a specific Maximum Uplink Throughput per UE parameter value depending on which Service Category the UE is associated with for the network slice. In this case, different UEs within a network slice can experience a different maximum data rate.

Examples:

- 10 000 (Service Category 1)

- 100 000 (Service Category 2)

- 200 000 (Service Category 3)

Maximum Uplink Throughput per UE value

This parameter defines the Maximum Uplink Throughput per UE value. This may be associated with a Service Category parameter.

| Parameters |  |
| --- | --- |
| Measurement unit | kbps |
| Allowed Values | Any desired value |
| Tags | Character attribute / Functional KPI |

| Attribute Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional |  |
| Optional | X |

Maximum Uplink Throughput per UE Table

Service Category

This parameter defines a service category which may be assigned to certain groups of devices using the network slice. If present, it shall be associated with a Maximum Uplink Throughput Value parameter.

| Parameters |  |
| --- | --- |
| Measurement unit | NA |
| Allowed Values | Any text identifying the Service Category |
| Tags | Character attribute / Functional |

| Attribute Presence |  |
| --- | --- |
| Mandatory |  |
| Conditional |  |
| Optional | X |

Uplink Rate Service Category Table

Additional information

Maximum throughput can be used to offer different service categories,  which have different maximum throughput values.

This attribute applies to 3GPP access type only, across GBR and Non-GBR QoS flows.

END of CHANGES