RECOMMENDATION  ITU-R  M.[IMT-2020. SPEC]

[Detailed specifications of the terrestrial radio interfaces of International
Mobile Telecommunications-2020 (IMT-2020)]

….

Annex 1

Specification of the 3GPP 5G radio interface technology - SRIT

Background

[to be filled by ITU-R WP5D]

# 1	Overview of the radio interface technology

## 1.1	Overview of the SRIT

The IMT-2020 terrestrial radio interface specifications known as 5G is based on LTE and NR Release 15 and Beyond both developed by 3GPP. In 3GPP terminology, the term E-UTRA (Evolved-UTRA) is also used to indicate the LTE radio interface, and 3GPP LTE Release 13 and beyond is marked as “LTE-Advanced Pro”.

5G is a Set of RITs (Radio Interface Technologies) consisting of E-UTRA/LTE as one component RIT and NR as the other component RIT. Both components are designed for operation in IMT spectrum.

5G fulfils all technical performance requirements in all five selected test environments : Indoor Hotspot – eMBB, Dense Urban – eMBB, Rural – eMBB, Urban Macro – URLLC and Urban Macro – mMTC. The

Also, 5G fulfils the spectrum requirement. Both component RITs, NR and E-UTRA/LTE, utilize the frequency bands below 6 GHz identified for IMT in the ITU Radio Regulations. In addition, the NR component RIT can also utilize the frequency bands above 6 GHz, i.e., above 24.25 GHz, identified for IMT in the ITU Radio Regulations, as well.

The complete set of standards for the terrestrial radio interface of IMT-2020 identified as 5G includes not only the key characteristics of IMT-2020 but also the additional capabilities of 5G both of which are continuing to be enhanced.

## 1.2	Overview of the Radio Interface Technology (RIT)

### 1.2.1	Overview of the component RIT : E-UTRA/LTE RIT

The E-UTRA/LTE RIT is the evolution of LTE Frequency-Division Duplex (FDD) and Time-Division Duplex operation (TDD). The E-UTRA/LTE RIT uses either 1) FDD operation and therefore is applicable for the operation with paired spectrum or 2) TDD operation and therefore is applicable for the operation with unpaired spectrum. Both full-duplex and half-duplex FDD are supported.

The TDD  provides flexibility in terms of uplink-downlink resource allocation by supporting multiple uplink-downlink resource-allocation configurations that can be used to match different traffic scenarios. The uplink-downlink resource-allocation configuration can be adapted to the varying instantaneous traffic and interference conditions even during operation. It is also designed to exploit the more extensive channel reciprocity inherent in case of TDD operation, e.g. for beamforming, and facilitates coexistence with TD-SCDMA as well as other TDD-based IMT-2000 technologies.

### 1.2.2	Overview of the component RIT : NR RIT

The NR RIT, also known as 5G, is the evolution of NR radio interface. The NR RIT uses either 1) Frequency-Division Duplex (FDD) operation and therefore is applicable for the operation with paired spectrum or 2) Time-Division Duplex operation (TDD) operation and therefore is applicable for operation with unpaired spectrum.

## 1.3	Overview of the system aspects of the E-UTRA/ LTE component RIT

The E-UTRA/LTE as one component RIT represents the evolution of the first releases of LTE FDD and TDD, respectively. Transmission bandwidths up to 640 MHz are supported, yielding peak data rates up to roughly 25 Gbit/s in the downlink and 13.6 Gbit/s in the uplink.

The downlink transmission scheme is based on conventional OFDM to provide a high degree of robustness against channel frequency selectivity while still allowing for low-complexity receiver implementations also at very large bandwidths.

The uplink transmission scheme is based on Discrete Fourier Transform-spread OFDM (DFTS-OFDM). The use of DFTS-OFDM transmission for the uplink is motivated by the lower Peak-to-Average Power Ratio (PAPR) of the transmitted signal compared to conventional OFDM. This allows for more efficient usage of the power amplifier at the terminal, which translates into an increased coverage and/or reduced terminal power consumption. The uplink numerology is aligned with the downlink numerology. The Narrow-band Internet of Things (NB-IOT) UL allows allocating a single-tone in addition to multi-tone DFTS-OFDM with the possibility of a lower subcarrier spacing in addition to the normal subcarrier spacing.

Channel coding is based on rate-1/3 Turbo coding (Tail Biting Convolutional Code for NB-IOT DL) and is complemented by Hybrid-ARQ with soft combining to handle decoding errors at the receiver side. Data modulation supports QPSK, 16QAM, 64QAM and 256QAM for both the downlink and the uplink. In downlink, 1024QAM is supported. For NB-IOT, additionally pi/2-BPSK and pi/4-QPSK are supported in uplink when a single-tone is allocated.

The E-UTRA/LTE RIT supports bandwidths from approximately 1.4 MHz to 640 MHz. NB-IOT supports 200 kHz bandwidth. Carrier aggregation, i.e. the simultaneous transmission of multiple component carriers in parallel to/from the same terminal/eNB, is used to support bandwidths larger than 20 MHz. Component carriers do not have to be contiguous in frequency and can even be located in different frequency bands in order to enable exploitation of fragmented spectrum allocations by means of spectrum aggregation. Licensed-Assisted Access (LAA) allows secondary component carriers to operate the DL in the unlicensed 5 GHz band. In order to fairly coexist, LAA uses a Listen-Before-Talk (LBT) medium access in the unlicensed frequency bands. Carrier Aggregation supports the functionality to aggregate TDD bands with different uplink and downlink allocations as well as the functionality to support multiple timing advancements. Carrier Aggregation also supports to aggregate FDD and TDD component carriers. Dual Connectivity allows aggregating component carriers of different eNBs that are connected via a non-ideal backhaul over the X2 interface.

Channel-dependent scheduling in both the time and frequency domains is supported for both downlink and uplink with the base-station scheduler being responsible for (dynamically) selecting the transmission resource as well as the data rate. The basic operation is dynamic scheduling, where the base-station scheduler takes a decision for each 1 ms Transmission Time Interval (TTI), but there is also a possibility for semi-persistent scheduling. Semi-persistent scheduling enables transmission resources and data rates to be semi-statically allocated to a given User Equipment (UE) for a longer time period than one TTI to reduce the control-signalling overhead. For better uplink coverage, TTI bundling allows UEs to transmit in four consecutive TTIs. NB-IOT and enhanced Machine-type Communication (eMTC) allows for widespread coverage extension by means of scheduling multiple TTIs (up to several thousands).

Multi-antenna transmission schemes are an integral part of E-UTRA/LTE RIT. Multi-antenna precoding with dynamic rank adaptation supports both spatial multiplexing (single-user MIMO) and beam-forming. Beam-forming by means of 2-dimensional antenna arrays can exploit the horizontal as well as the vertical domain. Spatial multiplexing with up to eight layers in the downlink and four layers in the uplink is supported. Multi-user MIMO, where multiple users are assigned the same time-frequency resources, is also supported. Coordinated MultiPoint (CoMP) operation is also supported, where multiple transmission points or reception points are coordinated in their transmission or reception respectively. The coordinated transmission points can belong to the same cell, to different cells of the same eNB or to different cells of different eNBs. A discovery reference signal can be used to identify transmission points or cells for CoMP and/or Carrier aggregation operation. Finally, transmit diversity based on Space-Frequency Block Coding (SFBC) or a combination of SFBC and Frequency Switched Transmit Diversity (FSTD) is supported.

Inter-cell interference coordination (ICIC), where neighbour cells exchange information aiding the scheduling in order to reduce interference, is supported for the E-UTRA/LTE RIT. ICIC can be used for homogenous deployments with non-overlapping cells of similar transmission power, as well as for heterogeneous deployments where a higher-power cell overlays one or several lower-power nodes. In order to increase the potential of cell range expansion, functionality for terminal-side interference mitigation of reference and synchronization signals as well as the broadcast channel exists. A terminal-side mitigation of inter-cell interference caused by the data channel is supported with network assistance. A network-side interference mitigation technique is supported, which is based on the ability to turn on and off secondary cells.

Relaying functionality is included in E-UTRA/LTE RIT. The relay node appears as a conventional base station (e-Node B) to terminals but is wirelessly backhauled to the remaining part of the radio-access network using the LTE Release 10 radio-interface technology.

E-UTRA/LTE RIT supports various types of machine-type communication. In order to better address the low-cost segment, a low-complexity terminal (Category 0) is supported, which has approximately 50% reduced modem complexity as compared to the least complex ‘normal’ UE (Category 1). Another 50% complexity reduction was made possible with eMTC (Category M1) and even more with NB-IOT (Category NB1). eMTC supports optionally half duplex (HD) operation and NB-IOT supports HD only. In addition, eMTC and NB-IOT extended the original LTE coverage area by ~15 dB and ~20 dB respectively. The narrow NB-IOT channel bandwidth of 200 kHz makes it possible to operate in re-farmed GSM channels or in LTE guardbands. In order to improve UE power consumption, a Power Saving State was introduced and extended Discontinuous Reception (eDRX) cycles range up to 10.24 s in connected mode and 43.69 min in idle mode. For eMTC and NB-IOT, it is possible to configure an additional uplink and an additional downlink carrier for traffic which is dedicated to a particular User Equipment, whilst common transmissions such as synchronization signals, and uplink transmissions during cell access, occur on a the same carrier for all User Equipments. In Rel-16, spectral efficiency is increased further for massive-MTC transmissions and reduced energy consumption for massive-MTC devices enabled e.g. uplink transmission using preconfigured resources in idle mode (allowing the device to skip random access procedures) and multi-transport-block scheduling in both the downlink and uplink transmission directions (reducing the control signaling overhead).

In order to enhance data offloading, E-UTRA/LTE RIT supports LTE/Wi-Fi interworking functionality via RAN-assisted and RAN-controlled WLAN interworking. Based on configurable rules or eNB command, the UE steers its data traffic to the most appropriate radio access. Additionally, from Rel-13 onwards, E-UTRA/LTE RIT supports LTE-WLAN aggregation (LWA) and LTE-WLAN Radio Level Integration with IPSec Tunnel (LWIP). LWA allows utilizing both LTE and WLAN in the unlicensed 2.4 GHz and 5 GHz bands simultaneously under the control of eNB.

From Rel-12 onwards, sidelink transmissions are defined for Proximity based Services (ProSe) Direct Discovery and ProSe Direct Communication between terminals. ProSe Direct Communication is targeting only Public Safety applications and allows terminals to communicate with each other directly without routing the data via the eNB. ProSe Direct Discovery allows discovering other terminals in close proximity. Direct communication is also supported when a terminal is out of LTE coverage.

In order to enhance mobility robustness and performance, additional LTE mobility enhancements is introduced in 3GPP Rel-16.  User data interruption is reduced during handover, which targets to 0ms. In addition,  the robustness during handover is improved by conditional handover.

### 1.3.1	Network architecture

The radio-access network of E-UTRA/LTE RIT has a flat architecture with a single type of node, the eNodeB, which is responsible for all radio-related functions in one or several cells. The eNodeB is connected to the core network by means of the S1 interface, more specifically to the serving gateway (S-GW) by means of the user-plane part, S1-u, and to the Mobility Management Entity (MME) by means of the control-plane part, S1-c. One eNodeB can interface to multiple MMEs/S-GWs for the purpose of load sharing and redundancy. MMEs/S-GWs can be (re)selected for support of separate dedicated core networks that are designed to meet the requirements of a certain group of devices/customers.

The X2 interface, connecting eNodeBs to each other, is mainly used to support active-mode mobility. This interface may also be used for multi-cell Radio Resource Management (RRM) functions such as ICIC or CoMP. The X2 interface is also used to support lossless mobility between neighbouring cells by means of packet forwarding.

Figure 1.1

Radio-access network interfaces

### 1.3.2	Layer 2 protocol architecture

Layer 2 (L2) consists of several sub-layers: Packet Data Convergence Protocol (PDCP), Radio Link Control (RLC) and Medium Access Control (MAC). The downlink and uplink protocol structures are illustrated in Fig. 1.2 and Fig. 1.3, respectively. Layer 2 provides one or more Radio Bearers to higher layers to which IP packets are mapped according to their Quality-of-Service (QoS) requirements. L2/MAC PDUs, also referred to as transport blocks, are created according to instantaneous scheduling decisions and delivered to the physical layer on one or several transport channels (one transport channel of the same type per component carrier).

Figure 1.2

Downlink L2 protocol structure

Figure 1.3

Uplink L2 protocol structure

In Dual Connectivity, a data radio bearer can be configured as Master Cell Group (MCG) bearer, Secondary Cell Group (SCG) bearer or a split bearer. The MCG bearer is served by the Master eNB (MeNB), SCG bearer by the Secondary eNB (SeNB) and the split bearer is served by both eNBs. For a split bearer, a common PDCP entity is located in the MeNB, but there are two RLC entities, one terminating in the MeNB and one in the SeNB. From Rel-15 onwards, a data packet of a data radio bearer can be duplicated increasing robustness. In addition, it is possible to split the signaling radio bearer.

#### 1.3.2.1	Packet Data Convergence Protocol (PDCP)

Except for NB-IOT, the main services and functions of the PDCP sublayer for the user plane include:

–	Header compression and decompression of IP data flows using ROHC.

–	Transfer of user data.

–	In-sequence delivery of upper layer PDUs at PDCP re-establishment procedure for RLC AM.

–	For split bearers in Dual Connectivity (only support for RLC AM): PDCP PDU routing for transmission and PDCP PDU reordering for reception.

–	Duplicate detection of lower layer SDUs at PDCP re-establishment procedure for RLC AM.

–	Retransmission of PDCP SDUs at handover and, for split bearers in DC and LWA bearers, of PDCP PDUs at PDCP data-recovery procedure, for RLC AM.

–	Ciphering and deciphering.

–	Timer-based SDU discard in uplink.

For NB-IOT UE when AS security is activated, the main services and functions of the PDCP sublayer for the user plane include:

–	Header compression and decompression: ROHC only;

–	Transfer of user data;

–	In-sequence delivery of upper layer PDUs at PDCP re-establishment procedure for RLC AM;

–	Duplicate detection of lower layer SDUs at PDCP re-establishment procedure for RLC AM;

–	Ciphering and deciphering;

–	Timer-based SDU discard in uplink.

The main services and functions of the PDCP for the control plane include:

–	Ciphering and Integrity Protection and Verification.

–	Transfer of control plane data.

For a NB-IOT UE that supports Control Plane CIoT EPS optimizations only, PDCP is bypassed. For a NB-IOT UE that supports both Control Plane CIoT EPS optimizations and User Plane CIoT EPS optimizations PDCP is not used until AS security is activated.

PDCP uses the services provided by the RLC sub-layer. There is one PDCP entity per radio bearer configured for a UE.

#### 1.3.2.2	Radio Link Control (RLC)

Radio Link Control (RLC) is responsible for:

–	Transfer of upper layer PDUs.

–	Error correction through ARQ (only for AM data transfer).

–	Concatenation, segmentation and reassembly of RLC SDUs (only for UM and AM data transfer).

–	Resegmentation of RLC data PDUs (only for AM data transfer).

–	Reordering of RLC data PDUs (only for UM and AM data transfer).

–	Duplicate detection (only for UM and AM data transfer).

–	Protocol error detection (only for AM data transfer).

–	RLC SDU discard (only for UM and AM data transfer).

–	RLC re-establishment, except for a NB-IOT UE that only uses Control Plane CIoT EPS optimizations.

Depending on the mode-of-operation, an RLC entity may provide all, a subset of, or none of the services above. The RLC can operate in three different modes:

–	Transparent mode (TM), where the RLC is completely transparent and is in essence bypassed. This configuration is used for control-plane broadcast channels such as Broadcast Control Channel (BCCH), Common Control Channel (CCCH) and Paging Control Channel (PCCH) only where the information should reach multiple users.

–	Unacknowledged mode (UM), where the RLC provides all the functionality above except error correction, is used when error-free delivery is not required, for example for Multicast Control Channel (MCCH) and Multicast Traffic Channel (MTCH) using Multimedia Broadcast over a Single Frequency Network (MBSFN) and for Voice-over-IP (VoIP).

–	For NB-IOT, RLC UM is not supported.

–	Acknowledged mode (AM), where the RLC provides all the services above, is the main mode-of-operation for TCP/IP packet data transmission on the Downlink Shared Channel (DL-SCH). Segmentation/reassembly, in-sequence delivery and retransmissions of erroneous data are all supported.

The RLC offers services to the PDCP in the form of radio bearers and uses services from the MAC layer in the form of logical channels. There is one RLC entity per radio bearer configured for a terminal.

#### 1.3.2.3	Medium access control (MAC)

The MAC layer is responsible for:

–	Mapping between logical channels and transport channels.

–	Multiplexing/demultiplexing of MAC SDUs belonging to one or different logical channels into/from transport blocks delivered to/from the physical layer on transport channels.

–	Scheduling information reporting.

–	Error correction through N-process stop-and-wait hybrid-ARQ (HARQ) with synchronous (for the uplink) and asynchronous (for the downlink) retransmissions.

–	Priority handling between logical channels of one UE.

–	Priority handling between UEs by means of dynamic scheduling.

–	Multimedia Broadcast/Multicast Service (MBMS) identification.

–	Transport format selection.

–	Padding.

The ProSe specific services and functions of the MAC sublayer include:

–	Radio resource selection;

–	Packet filtering for Prose Direct Communication.

In case of Dual Connectivity, the UE is configured with two independent MAC entities, one for MCG and one for SCG.

The MAC offers services to the RLC in the form of logical channels. A logical channel is defined by the type of information it carries and is generally classified as a control channel, used for transmission of control and configuration information necessary for operating an E-UTRA/LTE RIT system, or as a traffic channel, used for the user data. The set of logical-channel types specified for E-UTRA/LTE RIT includes:

–	Broadcast Control Channel (BCCH), used for broadcasting system control information.

–	Bandwidth Reduced Broadcast Control Channel (BR-BCCH), used for broadcasting system control information to bandwidth limited eMTC UEs.

–	Paging Control Channel (PCCH), a downlink channel used for paging when the network is not aware of the location of the UE and for system information change notifications.

–	Common Control Channel (CCCH), used for transmission of control information between UEs and network when the UE has no RRC connection.

–	Dedicated Control Channel (DCCH), used for transmission of control information to/from a mobile terminal when the UE has a RRC connection.

–	Multicast Control Channel (MCCH), used for transmission of control information required for reception of the MTCH.

–	Single-Cell Multicast Control Channel (SC-MCCH), used for transmission of control information required for reception of MBMS using single-cell point-to-multipoint (SC-PTM).

–	Sidelink Broadcast Channel (SBCH) used for broadcasting sidelink system information from one UE to other UE(s). This channel is used only by ProSe Direct Communication capable UEs.

–	Dedicated Traffic Channel (DTCH), used for transmission of user data to/from a mobile terminal. This is the logical channel type used for transmission of all uplink and non-MBSFN downlink user data. DTCH is not supported for a NB-IOT UE that only uses Control Plane CIoT EPS optimizations.

–	Multicast Traffic Channel (MTCH), used for downlink transmission of MBMS services.

–	Single-Cell Multicast Traffic Channel (SC-MTCH), used for downlink transmission of MBMS services using SC-PTM.

–	Sidelink Traffic Channel (STCH) is a point-to-multipoint channel, for transfer of user information from one UE to other UEs. This channel is used only by Prose Direct Communication capable UEs.

For a NB-IOT UE that only uses Control Plane CIoT EPS optimizations there is only one dedicated logical channel per UE.

From the physical layer, the MAC layer uses services in the form of Transport Channels. A transport channel is defined by how and with what characteristics the information is transmitted over the radio interface. Data on a transport channel is organized into transport blocks. In each Transmission Time Interval (TTI), at most one or two (in case of spatial multiplexing) transport blocks are transmitted per component carrier.

Associated with each transport block is a Transport Format (TF), specifying how the transport block is to be transmitted over the radio interface. The transport format includes information about the transport-block size, the modulation scheme, and the antenna mapping. The scheduler is responsible for (dynamically) determining the uplink as well as downlink transport format in each TTI.

The following transport-channel types are defined:

–	Broadcast Channel (BCH) has a fixed transport format, provided by the specifications. It is used for transmission of parts of the BCCH system information, more specifically the so-called Master Information Block (MIB).

–	Paging Channel (PCH) is used for transmission of paging information from the PCCH logical channel. The PCH supports discontinuous reception (DRX) to allow the mobile terminal to save battery power by waking up to receive the PCH only at predefined time instants.

–	Downlink Shared Channel (DL-SCH) is the main transport-channel type used for transmission of downlink data in E-UTRA/LTE RIT. It supports dynamic rate adaptation and channel-dependent scheduling, hybrid-ARQ with soft combining, and spatial multiplexing. It also supports DRX to reduce mobile-terminal power consumption while still providing an always-on experience.

The DL-SCH is also used for transmission of the parts of the BCCH system information not mapped to the BCH. In case of transmission to a terminal using multiple component carriers the UE receives one DL-SCH per component carrier.

–	Multicast Channel (MCH) is used to support MBMS. It is characterized by a semi-static transport format and semi-persistent scheduling. In case of multi-cell transmission using MBSFN, the scheduling and transport format configuration is coordinated among the cells involved in the MBSFN transmission.

–	Uplink Shared Channel (UL-SCH) is the uplink counterpart to the DL-SCH, i.e. it is the uplink transport channel used for transmission of uplink data.

–	Random Access Channel (RACH) is also defined as an uplink transport channel although it does not carry transport blocks. The RACH is used in the uplink to respond to the paging message or to initiate the move to the RRC_CONNECTED state according to terminal data transmission needs.

–	Sidelink broadcast channel (SL-BCH) uses a pre-defined transport format.

–	Sidelink discovery channel (SL-DCH) supports both UE autonomous resource selection and scheduled resource allocation by eNodeB; it uses a fixed size, pre-defined format periodic broadcast transmission.

–	Sidelink shared channel (SL-SCH) supports both UE autonomous resource selection and scheduled resource allocation by eNodeB; it supports HARQ combining and dynamic link adaptation by varying the transmit power, modulation and coding.

The mapping between logical channels, transport channels and physical channels (described in § 1.1.3.3) is illustrated in Fig. 1.4 for the downlink, Fig. 1.5 for the uplink, Fig. 1.6 for the sidelink, Fig. 1.6A for the NB-IOT downlink and Fig. 1.6B for the NB-IOT uplink.

Figure 1.4

Downlink channel mapping

Figure 1.5

Uplink channel mapping

Figure 1.6

Sidelink channel mapping

Figure 1.6a

NB-IOT DL channel mapping

Figure 1.6b

NB-IOT UL channel mapping

### 1.3.3	Physical layer

The physical layer is responsible for:

–	Modulation and demodulation of physical channels.

–	Error detection on the transport channel and indication to higher layers.

–	Forward Error Correction (FEC) encoding and decoding of transport channels.

–	Rate matching of the coded transport channel to physical channels.

–	Mapping of the coded transport channel onto physical channels according to Fig. 1.4 (downlink) and Fig. 1.5 (uplink).

–	Hybrid ARQ soft-combining.

–	Frequency and time synchronization.

–	Power weighting of physical channels.

–	Multi-antenna processing and beamforming.

–	Characteristic measurements and indication to higher layers.

–	RF processing.

A simplified overview of the processing for the DL-SCH is given in Fig. 1.7.

Figure 1.7

Simplified physical-layer processing for DL-SCH on one component carrier

#### 1.3.3.1	Physical channels

The following different types of physical channels are defined for the downlink:

–	Physical Downlink Shared Channel (PDSCH): Used for transmission of user and control plane data services.

–	Physical Multicast Channel (PMCH): Used for transmission of control and user-plane broadcast services during MBSFN subframes.

–	Physical Downlink Control Channel (PDCCH): Used for transmission of control information such as resource allocation, transport format and HARQ related information.

–	Enhanced Physical Downlink Control Channel (EPDCCH): Used for transmission of control information such as resource allocation, transport format and HARQ related information.

–	MTC physical downlink control channel (MPDCCH): Used for transmission of control information in bandwidth-reduced operation and/or using coverage extension mode.

–	Physical Broadcast Channel (PBCH): Used for conveying cell and/or system specific information.

–	Physical Control Format Indicator Channel (PCFICH): It indicates to the UE the control format (number of symbols comprising PDCCH, PHICH) of the current subframe.

–	Physical Hybrid ARQ Indicator Channel (PHICH): It conveys the ACK/NAK information for UL (PUSCH) transmissions received at the eNodeB.

Three different types of physical channels are defined for the uplink:

–	Physical Random Access Channel (PRACH): It conveys a preamble which is used to trigger a random-access procedure in the eNodeB.

–	Physical Uplink Shared Channel (PUSCH): It conveys both user data and upper layer control information.

–	Physical Uplink Control Channel (PUCCH): It conveys control information (scheduling requests, CQI, PMI, RI, HARQ ACK/NAK for PDSCH, etc.).

The following types of physical channels are defined for the sidelink:

–	Physical sidelink broadcast channel (PSBCH): Carries system and synchronization related information, transmitted from the UE.

–	Physical sidelink discovery channel (PSDCH): Carries ProSe Direct Discovery message from the UE.

–	Physical sidelink control channel (PSCCH): Carries control from a UE for ProSe Direct Communication.

–	Physical sidelink shared channel (PSSCH): Carries data from a UE for ProSe Direct Communication.

The following types of physical channels are defined for NB-IOT:

–	Narrowband Physical broadcast channel (NPBCH) : Carries the BCH for NB-IOT UEs. Used for conveying cell and/or system specific information using a transport block mapped to sixty four subframes within a 640 ms interval.

–	Narrowband Physical downlink shared channel (NPDSCH): Carries the DL-SCH and PCH for NB-IOT UEs.

–	Narrowband Physical downlink control channel (NPDCCH): Informs the NB-IOT UE about the resource allocation of PCH and DL-SCH. Carries the uplink scheduling grant for the NB-IOT UE. Carries the direct indication information.

–	Narrowband Physical uplink shared channel (NPUSCH): Carries the UL-SCH and Hybrid ARQ ACK/NAKs in response to downlink transmission for the NB-IOT UE.

–	Narrowband Physical random access channel (NPRACH): Carries the random access preamble for the NB-IOT UE.

#### 1.3.3.2	Time-domain structure and duplex schemes

Figure 1.8 illustrates the high-level time-domain structure for transmission, with each (radio) frame of length 10 ms consisting of ten equally sized subframes of length 1 ms. Each subframe consists of two equally sized slots of length Tslot = 0.5 ms with each slot consisting of a number of OFDM symbols including cyclic prefix.

Figure 1.8

E-UTRA/LTE RIT time-domain structure

E-UTRA/LTE RIT can operate in both FDD and TDD as illustrated in Fig. 1.9. Although the time-domain structure is, in most respects, the same for FDD and TDD there are some differences between the two duplex modes, most notably the presence of a special subframe in case of TDD. The special subframe is used to provide the necessary guard time for downlink-to-uplink switching.

Figure 1.9

Uplink/downlink time/frequency structure in case of FDD and TDD

In case of FDD operation (upper part of Fig. 1.9), there are two carrier frequencies for each component carrier, one for uplink transmission (fUL) and one for downlink transmission (fDL). During each frame, there are thus ten uplink subframes and ten downlink subframes and uplink and downlink transmission can occur simultaneously within a cell. Half-duplex operation at the UE side is supported by the scheduler ensuring non-simultaneous reception and transmission at the UE.

In case of TDD operation (lower part of Fig. 1.9), there is only a single carrier frequency per component carrier and uplink and downlink transmissions are always separated in time also on a cell basis. As seen in the figure, some subframes are allocated for uplink transmissions and some subframes for downlink transmission with the switch between downlink and uplink occurring in the special subframe. The special subframe is split into three parts: a downlink part (DwPTS), a guard period (GP) where the switch occurs, and an uplink part (UpPTS). The DwPTS is in essence treated as a normal downlink subframe, although the amount of data that it is possible to transmit is smaller due to the reduced length of the DwPTS. The UpPTS can be used for channel sounding or random access. The DwPTS, GP, and UpPTS have configurable individual lengths to support different deployment scenarios, and a total length of 1 ms.

Different asymmetries in terms of the amount of resources allocated for uplink and downlink transmission, respectively, are provided through seven different downlink/uplink configurations as shown in Fig. 1.10. In case of carrier aggregation, the downlink/uplink configuration is identical across component carriers in the same band and may be the same or different across component carriers in different bands.

Coexistence between the E-UTRA/LTE RIT and other (IMT-2000) TDD systems such as TD-SCDMA is catered for by aligning the switch points between the two systems and selecting the appropriate special subframe configuration and uplink-downlink asymmetry.

Figure 1.10

Uplink-downlink asymmetries supported by the E-UTRA/LTE RIT

Sidelink transmissions use the same frame structure as the frame structure that is defined for uplink and downlink when UEs are in network coverage. However, the sidelink transmissions are restricted to a sub-set of the uplink resources in time and frequency domain.

The physical channel structure is similar to uplink transmissions and the same basic transmission scheme as the UL transmission scheme is used. However, sidelink is limited to single cluster transmissions and it uses a 1 symbol gap at the end of each sidelink subframe.

#### 1.3.3.3	Physical layer processing

To the transport block(s) to be transmitted on a DL-SCH or UL-SCH, a CRC is attached, followed by rate-1/3 Turbo coding for error correction (Tail Biting Convolutional Code for NPDSCH). Rate matching is used not only to match the number of coded bits to the amount of resources allocated for the DL-SCH/UL-SCH transmission, but also to generate the different redundancy versions as controlled by the hybrid-ARQ protocol. In case of spatial multiplexing, the processing is duplicated for the two transport blocks. After rate matching, the coded bits are modulated (QPSK, 16QAM, 64QAM, 256QAM in DL). In case of multi-antenna transmission, the modulation symbols are mapped to multiple layers and precoded before being mapped to the different antenna ports. Alternatively, transmit diversity can be applied. Finally, the (precoded) modulation symbols are mapped to the time-frequency resources allocated for the transmission.

Downlink transmission is based on conventional OFDM with a cyclic prefix. The subcarrier spacing is f = 15 kHz and two cyclic prefix lengths are supported: normal cyclic prefix 4.7 µs and extended cyclic prefix 16.7 µs. In the frequency domain, the number of resource blocks can range from 6 to 100 per component carrier (for channel bandwidths ranging from 1.4 to 20 MHz respectively), where a resource block is 180 kHz in the frequency domain. There can be up to 32 component carriers transmitted in parallel implying an overall bandwidth up to 640 MHz. In inband mode NB-IOT allocates a single resource block. In standalone mode, NB-IOT uses a channel bandwidth of 200 kHz.

Uplink transmission is based on DFT-spread OFDM (DFTS-OFDM). DFTS-OFDM can be seen as a DFT precoder, followed by conventional OFDM with the same numerology as in the downlink. The NB-IOT UL allows allocating a single-tone with f = 3.75 kHz or f = 15 kHz subcarrier spacing. Multiple DFT precoding sizes, corresponding to transmission with different scheduled bandwidths, can be used.

The remaining downlink transport channels (PCH, BCH, MCH) are based on the same general physical-layer processing as DL-SCH, although with some restrictions in the set of features used.

#### 1.3.3.4	Multi-antenna transmission

A wide range of multi-antenna transmission schemes are supported in the downlink:

–	Single-antenna transmission using a single cell-specific reference signal.

–	Closed-loop spatial multiplexing, also known as codebook-based beam-forming or precoding, of up to four layers using cell-specific reference signals. Feedback reports from the terminal are used to assist the eNodeB in selecting a suitable precoding matrix.

–	Open-loop spatial multiplexing, also known as large-delay cyclic delay diversity, of up to four layers using cell-specific reference signals.

–	Spatial multiplexing of up to eight layers using UE-specific reference signals. The eNodeB may use feedback reports or exploit channel reciprocity to set the beam-forming weights.

–	Transmit diversity based on space-frequency block coding (SFBC) or a combination of SFBC and Frequency Switched Transmit Diversity (FSTD).

–	Multi-user MIMO where multiple terminals are assigned overlapping time-frequency resources.

–	Non-precoded CSI-RS operation is supported, which comprises schemes where different CSI-RS ports have the same wide beam width and direction and hence generally cell wide coverage.

–	Beamformed CSI-RS operation is supported, which comprises schemes where (at least at a given time/frequency) CSI-RS ports have narrow beam widths and hence not cell wide coverage, and (at least from the eNB perspective) at least some CSI-RS port-resource combinations have different beam directions.

–	Downlink Coordinated MultiPoint (DL-CoMP) operation where multiple transmission points are coordinated.

The following multi-antenna transmission schemes are supzported in the uplink:

–	Single-antenna transmission.

–	Precoding supporting rank-adaptive spatial multiplexing with one up to four layers.

–	Uplink Coordinated MultiPoint (UL-CoMP) operation where multiple reception points are coordinated.

#### 1.3.3.5	Link adaptation and power control

According to the radio channel conditions, the Modulation and Coding Scheme (MCS) can be adapted flexibly. The same modulation and coding is applied to all resource units assigned to the same transport block within a TTI. Uplink power control determines the average power over a DFTS-OFDM symbol in which the physical channel is transmitted.

#### 1.3.3.6	L1/L2 control signalling

Downlink control information (DCI) is transmitted on either a PDCCH or an EPDCCH. In bandwidth-reduced operation and/or when using coverage extension mode, a DCI is transmitted on an MPDCCH. For NB-IOT, a DCI is transmitted on NPDCCH.

The PDCCH is transmitted in the first one to three OFDM symbols of each downlink subframe in each component carrier with the number of OFDM symbols being indicated on the PCFICH. Downlink and uplink scheduling grants (consisting of UE identity, time-frequency resources and transport format) and hybrid-ARQ acknowledgements are transmitted on the PDCCH and PHICH, respectively. Each grant is transmitted on a separate PDCCH using QPSK modulation and cell-specific reference signals.

The EPDCCH/MPDCCH is transmitted in pairs of Physical Resource Blocks (PRBs) multiplexed in frequency with the PDSCH; it carries downlink and uplink scheduling grants (consisting of UE identity, time-frequency resources and transport format). The EPDCCH/MPDCCH uses QPSK modulation and demodulation reference signals, and it may use either frequency-localized or frequency-distributed transmission.

For NB-IOT, the NPDCCH is transmitted in all available OFDM symbols of pairs of PRBs, with no multiplexing with NPDSCH; it carries downlink and uplink scheduling grants (consisting of UE identity, time-frequency resources and transport format). The NPDCCH uses QPSK modulation and narrowband reference signals.

Sidelink resource allocation can be transmitted on the PDCCH/EPDCCH.

Uplink control information (UCI), consisting of channel-state information (CSI), scheduling requests and hybrid-ARQ acknowledgements, is transmitted at the band edges of the primary uplink component carrier. Alternatively, parts of the control signalling can be multiplexed with data on PUSCH. To support downlink CoMP transmission, a UE may be configured with multiple CSI processes.

#### 1.3.3.7	MBSFN operation

Multicast/Broadcast over Single Frequency Network (MBSFN) transmission, where the same signal is transmitted from multiple, time-synchronized cells, is supported by the MCH transport channel. One component carrier can support simultaneous unicast and broadcast support through time-domain multiplexing of MCH and DL-SCH transmissions.

## 1.4	Overview of the system aspects of the NR component RIT

The NR RIT as one component RIT represents the evolution of the first releases of NR which uses either uses either 1) FDD operation and therefore is applicable for operation with paired spectrum or 2) TDD operation and therefore is applicable for operation with unpaired spectrum. can be  to both FDD and TDD spectrum. Channel bandwidths up to 400 MHz are supported and carrier aggregation over 16 component carriers, yielding peak data rates up to roughly 140 Gbit/s in the downlink and 65 Gbit/s  in the uplink.

From Rel-16 owards, NR RIT may operate in Unlicensed Spectrum (referred to as NR-U) in a PCell, an SCell in Carrier Aggregation, or a PSCell in Dual Connectivity. In stand-alone mode, all cells are in unlicensed spectrum. When NR-U SCells are in unlicensed spectrum, the PCell may be in licensed spectrum. An NR-U SCell may or may not be configured with UL. The DC-type of operation can be configured either with E-UTRAN node (connected to EPC or 5GC) as MN or with NR node as described in TS 37.340 [21].  The gNB and UE may apply Listen-Before-Talk (LBT) before performing a transmission on NR-U cells. When LBT is applied, the transmitter listens to/senses the channel to determine whether the channel is free or busy and performs transmission only if the channel is sensed free.

### 1.4.1	Overall Architecture

An NG-RAN node is either:

-	A gNB, providing NR user plane and control plane protocol terminations towards the UE; or

-	An ng-eNB, providing E-UTRA user plane and control plane protocol terminations towards the UE.

The gNBs and ng-eNBs are interconnected with each other by means of the Xn interface. The gNBs and ng-eNBs are also connected by means of the NG interfaces to the 5GC, more specifically to the AMF (Access and Mobility Management Function) by means of the NG-C interface and to the UPF (User Plane Function) by means of the NG-U interface (see TS 23.501 [3]).

NOTE:	The architecture and the F1 interface for a functional split are defined in TS 38.401 [4].

The NG-RAN architecture is illustrated in Fig. 1.11 below.

Figure 1.11

Overall Architecture

### 1.4.2	Radio Protocol Architecture

#### 1.4.2.1	User Plane

The figure below shows the protocol stack for the user plane, where SDAP, PDCP, RLC and MAC sublayers (terminated in gNB on the network side) perform the functions listed in clause 6.

Figure 1.12

User Plane Protocol Stack

#### 1.4.2.2	Control Plane

The figure below shows the protocol stack for the control plane, where:

-	PDCP, RLC and MAC sublayers (terminated in gNB on the network side) perform the functions listed in clause 6;

-	RRC (terminated in gNB on the network side) performs the functions listed in clause 7;

-	NAS control protocol (terminated in AMF on the network side) performs the functions listed in TS 23.501 [3]), for instance: authentication, mobility management, security control.

Figure 1.13

Control Plane Protocol Stack

#### 1.4.2.3 Integrated Access and Backhaul Architecture

From NR Rel-16, Integrated access and backhaul (IAB) enables wireless relaying in NG-RAN.  The relaying node, referred to as IAB-node, supports access and backhauling via NR.The terminating node of NR backhauling on network side is referred to as the IAB-donor, which represents a gNB with additional functionality to support IAB. Backhauling can occur via a single or via multiple hops. The IAB architecture is shown in Fig. 1.14.

Figure 1.14

IAB architecture; a) IAB-node using SA mode with NGC; b) IAB-node using EN-DC

### 1.4.3	Multi-Radio Dual Connectivity

NG-RAN supports Multi-Radio Dual Connectivity (MR-DC) operation whereby a UE in RRC_CONNECTED is configured to utilise radio resources provided by two distinct schedulers, located in two different NG-RAN nodes connected via a non-ideal backhaul, one providing NR access and the other one providing either E-UTRA or NR access. Further details of MR-DC operation can be found in TS 37.340.

#### 1.4.3.1	MR-DC with the EPC

E-UTRAN supports MR-DC via E-UTRA-NR Dual Connectivity (EN-DC), in which a UE is connected to one eNB that acts as a MN and one en-gNB that acts as a SN. The eNB is connected to the EPC via the S1 interface and to the en-gNB via the X2 interface. The en-gNB might also be connected to the EPC via the S1-U interface and other en-gNBs via the X2-U interface.

The EN-DC architecture is illustrated in Fig. 1.15 below.

Figure 1.15

EN-DC Overall Architecture

#### 1.4.5.2	MR-DC with the 5GC

#### 1.4.5.2.1	E-UTRA-NR Dual Connectivity

NG-RAN supports NG-RAN E-UTRA-NR Dual Connectivity (NGEN-DC), in which a UE is connected to one ng-eNB that acts as a MN and one gNB that acts as a SN. The ng-eNB is connected to the 5GC and the gNB is connected to the ng-eNB via the Xn interface.

##### 1.4.5.2.2	NR-E-UTRA Dual Connectivity

NG-RAN supports NR-E-UTRA Dual Connectivity (NE-DC), in which a UE is connected to one gNB that acts as a MN and one ng-eNB that acts as a SN. The gNB is connected to 5GC and the ng-eNB is connected to the gNB via the Xn interface.

##### 1.4.5.2.3	NR-NR Dual Connectivity

NG-RAN supports NR-NR Dual Connectivity (NR-DC), in which a UE is connected to one gNB that acts as a MN and another gNB that acts as a SN. The master gNB is connected to the 5GC via the NG interface and to the secondary gNB via the Xn interface. The secondary gNB might also be connected to the 5GC via the NG-U interface. In addition, NR-DC can also be used when a UE is connected to two gNB-DUs, one serving the MCG and the other serving the SCG, connected to the same gNB-CU, acting both as a MN and as a SN.

### 1.4.4	Physical Layer

#### 1.4.4.1	Waveform, numerology and frame structure

The downlink transmission waveform is conventional OFDM using a cyclic prefix. The uplink transmission waveform is conventional OFDM using a cyclic prefix with a transform precoding function performing DFT spreading that can be disabled or enabled.

Figure 1.16

Transmitter block diagram for CP-OFDM with optional DFT-spreading

The numerology is based on exponentially scalable sub-carrier spacing f = 2µ × 15 kHz with µ={0,1,3,4} for PSS, SSS and PBCH and µ={0,1,2,3} for other channels. Normal CP is supported for all sub-carrier spacings, Extended CP is supported for µ=2. 12 consecutive sub-carriers form a Physical Resource Block (PRB). Up to 275 PRBs are supported on a carrier.

TABLE 1.1

Supported transmission numerologies

|  |  | Cyclic prefix | Supported for data | Supported for synch |
| --- | --- | --- | --- | --- |
| 0 | 15 | Normal | Yes | Yes |
| 1 | 30 | Normal | Yes | Yes |
| 2 | 60 | Normal, Extended | Yes | No |
| 3 | 120 | Normal | Yes | Yes |
| 4 | 240 | Normal | No | Yes |

The UE may be configured with one or more bandwidth parts on a given component carrier, of which only one can be active at a time, as described in clauses 7.8 and 6.10 respectively. The active bandwidth part defines the UE's operating bandwidth within the cell's operating bandwidth. For initial access, and until the UE's configuration in a cell is received, initial bandwidth part detected from system information is used.

Downlink and uplink transmissions are organized into frames with 10 ms duration, consisting of ten 1 ms subframes. Each frame is divided into two equally-sized half-frames of five subframes each. The slot duration is 14 symbols with Normal CP and 12 symbols with Extended CP, and scales in time as a function of the used sub-carrier spacing so that there is always an integer number of slots in a subframe.

Timing Advance TA is used to adjust the uplink frame timing relative to the downlink frame timing.

Figure 1.17

Uplink-downlink timing relation

Operation on both paired and unpaired spectrum is supported.

#### 1.4.4.2	Downlink

##### 1.4.4.2.1	Downlink transmission scheme

A closed loop Demodulation Reference Signal (DMRS) based spatial multiplexing is supported for Physical Downlink Shared Channel (PDSCH). Up to 8 and 12 orthogonal DL DMRS ports are supported for type 1 and type 2 DMRS respectively. Up to 8 orthogonal DL DMRS ports per UE are supported for SU-MIMO and up to 4 orthogonal DL DMRS ports per UE are supported for MU-MIMO. The number of SU-MIMO code words is one for 1-4 layer transmissions and two for 5-8 layer transmissions.

The DMRS and corresponding PDSCH are transmitted using the same precoding matrix and the UE does not need to know the precoding matrix to demodulate the transmission. The transmitter may use different precoder matrix for different parts of the transmission bandwidth, resulting in frequency selective precoding. The UE may also assume that the same precoding matrix is used across a set of Physical Resource Blocks (PRBs) denoted Precoding Resource Block Group (PRG).

Transmission durations from 2 to 14 symbols in a slot is supported.

Aggregation of multiple slots with Transport Block (TB) repetition is supported.

From Rel-16, enhancements for DL/UL MIMO are introduced, including enhancements on multi-TRP/panel transmission improving reliability and robustness with both ideal and non-ideal backhaul.

##### 1.4.4.2.2	Physical-layer processing for physical downlink shared channel

The downlink physical-layer processing of transport channels consists of the following steps:

-	Transport block CRC attachment;

-	Code block segmentation and code block CRC attachment;

-	Channel coding: LDPC coding;

-	Physical-layer hybrid-ARQ processing;

-	Rate matching;

-	Scrambling;

-	Modulation: QPSK, 16QAM, 64QAM and 256QAM;

-	Layer mapping;

-	Mapping to assigned resources and antenna ports.

The UE may assume that at least one symbol with demodulation reference signal is present on each layer in which PDSCH is transmitted to a UE, and up to 3 additional DMRS can be configured by higher layers.

Phase Tracking RS may be transmitted on additional symbols to aid receiver phase tracking.

The DL-SCH physical layer model is described in TS 38.202 [20].

##### 1.4.4.2.3	Physical downlink control channels

The Physical Downlink Control Channel (PDCCH) can be used to schedule DL transmissions on PDSCH and UL transmissions on PUSCH, where the Downlink Control Information (DCI) on PDCCH includes:

-	Downlink assignments containing at least modulation and coding format, resource allocation, and hybrid-ARQ information related to DL-SCH;

-	Uplink scheduling grants containing at least modulation and coding format, resource allocation, and hybrid-ARQ information related to UL-SCH.

In addition to scheduling, PDCCH can be used to for

-	Activation and deactivation of configured PUSCH transmission with configured grant;

-	Activation and deactivation of PDSCH semi-persistent transmission;

-	Notifying one or more UEs of the slot format;

-	Notifying one or more UEs of the PRB(s) and OFDM symbol(s) where the UE may assume no transmission is intended for the UE;

-	Transmission of TPC commands for PUCCH and PUSCH;

-	Transmission of one or more TPC commands for SRS transmissions by one or more UEs;

-	Switching a UE's active bandwidth part;

-	Initiating a random access procedure.

A UE monitors a set of PDCCH candidates in the configured monitoring occasions in one or more configured COntrol REsource SETs (CORESETs) according to the corresponding search space configurations.

A CORESET consists of a set of PRBs with a time duration of 1 to 3 OFDM symbols. The resource units Resource Element Groups (REGs) and Control Channel Elements (CCEs) are defined within a CORESET with each CCE consisting a set of REGs. Control channels are formed by aggregation of CCE. Different code rates for the control channels are realized by aggregating different number of CCE. Interleaved and non-interleaved CCE-to-REG mapping are supported in a CORESET.

Polar coding is used for PDCCH.

Each resource element group carrying PDCCH carries its own DMRS.

QPSK modulation is used for PDCCH.

##### 1.4.4.2.4	Synchronization signal and PBCH block

The Synchronization Signal and PBCH block (SSB) consists of primary and secondary synchronization signals (PSS, SSS), each occupying 1 symbol and 127 subcarriers, and PBCH spanning across 3 OFDM symbols and 240 subcarriers, but on one symbol leaving an unused part in the middle for SSS as show in Figure 5.2.4-1. The possible time locations of SSBs within a half-frame are determined by sub-carrier spacing and the periodicity of the half-frames where SSBs are transmitted is configured by the network. During a half-frame, different SSBs may be transmitted in different spatial directions (i.e. using different beams, spanning the coverage area of a cell).

Within the frequency span of a carrier, multiple SSBs can be transmitted. The PCIs of SSBs transmitted in different frequency locations do not have to be unique, i.e. different SSBs in the frequency domain can have different PCIs. However, when an SSB is associated with an RMSI, the SSB corresponds to an individual cell, which has a unique NCGI (see clause 8.2). Such an SSB is referred to as a Cell-Defining SSB (CD-SSB). A PCell is always associated to a CD-SSB located on the synchronization raster.

Figure 1.18

Time-frequency structure of SSB

Polar coding is used for PBCH.

The UE may assume a band-specific sub-carrier spacing for the SSB unless a network has configured the UE to assume a different sub-carrier spacing.

PBCH symbols carry its own frequency-multiplexed DMRS.

QPSK modulation is used for PBCH.

The PBCH physical layer model is described in TS 38.202 [20].

#### 1.4.4.2.5	Physical layer procedures

###### 1.4.4.2.5.1	Link adaptation

Link adaptation (AMC: adaptive modulation and coding) with various modulation schemes and channel coding rates is applied to the PDSCH. The same coding and modulation is applied to all groups of resource blocks belonging to the same L2 PDU scheduled to one user within one transmission duration and within a MIMO codeword.

For channel state estimation purposes, the UE may be configured to measure CSI-RS and estimate the downlink channel state based on the CSI-RS measurements. The UE feeds the estimated channel state back to the gNB to be used in link adaptation.

###### 1.4.4.2.5.2	Power Control

Downlink power control can be used.

#### 1.4.4.2.5.3	Cell search

Cell search is the procedure by which a UE acquires time and frequency synchronization with a cell and detects the Cell ID of that cell. NR cell search is based on the primary and secondary synchronization signals, and PBCH DMRS, located on the synchronization raster.

#### 1.4.4.2.5.4	HARQ

Asynchronous Incremental Redundancy Hybrid ARQ is supported. The gNB provides the UE with the HARQ-ACK feedback timing either dynamically in the DCI or semi-statically in an RRC configuration.

The UE may be configured to receive code block group based transmissions where retransmissions may be scheduled to carry a sub-set of all the code blocks of a TB.

#### 1.4.4.2.5.5	Reception of SIB1

The Master Information Block (MIB) on PBCH provides the UE with parameters (e.g. CORESET#0 configuration) for monitoring of PDCCH for scheduling PDSCH that carries the System Information Block 1 (SIB1). PBCH may also indicate that there is no associated SIB1, in which case the UE may be pointed to another frequency from where to search for an SSB that is associated with a SIB1 as well as a frequency range where the UE may assume no SSB associated with SIB1 is present. The indicated frequency range is confined within a contiguous spectrum allocation of the same operator in which SSB is detected.

#### 1.4.4.3	Uplink

##### 1.4.4.3.1	Uplink transmission scheme

Two transmission schemes are supported for PUSCH: codebook based transmission and non-codebook based transmission.

For codebook based transmission, the gNB provides the UE with a transmit precoding matrix indication in the DCI. The UE uses the indication to select the PUSCH transmit precoder from the codebook. For non-codebook based transmission, the UE determines its PUSCH precoder based on wideband SRI field from the DCI.

A closed loop DMRS based spatial multiplexing is supported for PUSCH. For a given UE, up to 4 layer transmissions are supported. The number of code words is one. When transform precoding is used, only a single MIMO layer transmission is supported.

Transmission durations from 1 to 14 symbols in a slot is supported.

Aggregation of multiple slots with TB repetition is supported.

Two types of frequency hopping are supported, intra-slot frequency hopping, and in case of slot aggregation, inter-slot frequency hopping.

PUSCH may be scheduled with DCI on PDCCH, or a semi-static configured grant may be provided over RRC, where two types of operation are supported:

-	The first PUSCH is triggered with a DCI, with subsequent PUSCH transmissions following the RRC configuration and scheduling received on the DCI, or

-	The PUSCH is triggered by data arrival to the UE's transmit buffer and the PUSCH transmissions follow the RRC configuration.

##### 1.4.4.3.2		Physical-layer processing for physical uplink shared channel

The uplink physical-layer processing of transport channels consists of the following steps:

-	Transport Block CRC attachment;

-	Code block segmentation and Code Block CRC attachment;

-	Channel coding: LDPC coding;

-	Physical-layer hybrid-ARQ processing;

-	Rate matching;

-	Scrambling;

-	Modulation: π/2 BPSK (with transform precoding only), QPSK, 16QAM, 64QAM and 256QAM;

-	Layer mapping, transform precoding (enabled/disabled by configuration), and pre-coding;

-	Mapping to assigned resources and antenna ports.

The UE transmits at least one symbol with demodulation reference signal on each layer on each frequency hop in which the PUSCH is transmitted, and up to 3 additional DMRS can be configured by higher layers.

Phase Tracking RS may be transmitted on additional symbols to aid receiver phase tracking.

The UL-SCH physical layer model is described in TS 38.202 [20].

##### 1.4.4.3.3		Physical uplink control channel

Physical uplink control channel (PUCCH) carries the Uplink Control Information (UCI) from the UE to the gNB. Five formats of PUCCH exist, depending on the duration of PUCCH and the UCI payload size.

-	Format #0: Short PUCCH of 1 or 2 symbols with small UCI payloads of up to two bits with UE multiplexing capacity of up to 6 UEs with 1-bit payload in the same PRB;

-	Format #1: Long PUCCH of 4-14 symbols with small UCI payloads of up to two bits with UE multiplexing capacity of up to 84 UEs without frequency hopping and 36 UEs with frequency hopping in the same PRB;

-	Format #2: Short PUCCH of 1 or 2 symbols with large UCI payloads of more than two bits with no UE multiplexing capability in the same PRBs;

-	Format #3: Long PUCCH of 4-14 symbols with large UCI payloads with no UE multiplexing capability in the same PRBs;

-	Format #4: Long PUCCH of 4-14 symbols with moderate UCI payloads with multiplexing capacity of up to 4 UEs in the same PRBs.

The short PUCCH format of up to two UCI bits is based on sequence selection, while the short PUCCH format of more than two UCI bits frequency multiplexes UCI and DMRS. The long PUCCH formats time-multiplex the UCI and DMRS. Frequency hopping is supported for long PUCCH formats and for short PUCCH formats of duration of 2 symbols. Long PUCCH formats can be repeated over multiple slots.

UCI multiplexing in PUSCH is supported when UCI and PUSCH transmissions coincide in time, either due to transmission of a UL-SCH transport block or due to triggering of A-CSI transmission without UL-SCH transport block:

-	UCI carrying HARQ-ACK feedback with 1 or 2 bits is multiplexed by puncturing PUSCH;

-	In all other cases UCI is multiplexed by rate matching PUSCH.

UCI consists of the following information:

-	CSI;

-	ACK/NAK;

-	Scheduling request.

QPSK and π/2 BPSK modulation can be used for long PUCCH with more than 2 bits of information, QPSK is used for short PUCCH with more than 2 bits of information and BPSK and QPSK modulation can be used for long PUCCH with up to 2 information bits.

Transform precoding is applied to long PUCCH.

Channel coding used for uplink control information is described in Table 1.2.

table 1.2

Channel coding for uplink control information

| Uplink Control Information size including CRC, if present | Channel code |
| --- | --- |
| 1 | Repetition code |
| 2 | Simplex code |
| 3-11 | Reed Muller code |
| >11 | Polar code |

##### 1.4.4.3.4		Random access

Random access preamble sequences, of two different lengths are supported. Long sequence length 839 is applied with subcarrier spacings of 1.25 and 5 kHz and short sequence length 139 is applied with subcarrier spacings of 15, 30, 60 and 120 kHz. Long sequences support unrestricted sets and restricted sets of Type A and Type B, while short sequences support unrestricted sets only.

Multiple PRACH preamble formats are defined with one or more PRACH OFDM symbols, and different cyclic prefix and guard time. The PRACH preamble configuration to use is provided to the UE in the system information.

The UE calculates the PRACH transmit power for the retransmission of the preamble based on the most recent estimate pathloss and power ramping counter.

The system information provides information for the UE to determine the association between the SSB and the RACH resources. The RSRP threshold for SSB selection for RACH resource association is configurable by network.

##### 1.4.4.3.4		Physical layer procedures

#### 1.4.4.3.4.1	Link adaptation

Four types of link adaptation are supported as follows:

-	Adaptive transmission bandwidth;

-	Adaptive transmission duration;

-	Transmission power control;

-	Adaptive modulation and channel coding rate.

For channel state estimation purposes, the UE may be configured to transmit SRS that the gNB may use to estimate the uplink channel state and use the estimate in link adaptation.

##### 1.4.4.3.4.2	Uplink Power control

The gNB determines the desired uplink transmit power and provides uplink transmit power control commands to the UE. The UE uses the provided uplink transmit power control commands to adjust its transmit power.

##### 1.4.4.3.3	Uplink timing control

The gNB determines the desired Timing Advance setting and provides that to the UE. The UE uses the provided TA to determine its uplink transmit timing relative to the UE's observed downlink receive timing.

##### 1.4.4.3.4.4	HARQ

Asynchronous Incremental Redundancy Hybrid ARQ is supported. The gNB schedules each uplink transmission and retransmission using the uplink grant on DCI.

The UE may be configured to transmit code block group based transmissions where retransmissions may be scheduled to carry a sub-set of all the code blocks of a transport block.

##### 1.4.4.4	Carrier aggregation

###### 1.4.4.4.1	Carrier aggregation

In Carrier Aggregation (CA), two or more Component Carriers (CCs) are aggregated. A UE may simultaneously receive or transmit on one or multiple CCs depending on its capabilities:

-	A UE with single timing advance capability for CA can simultaneously receive and/or transmit on multiple CCs corresponding to multiple serving cells sharing the same timing advance (multiple serving cells grouped in one TAG);

-	A UE with multiple timing advance capability for CA can simultaneously receive and/or transmit on multiple CCs corresponding to multiple serving cells with different timing advances (multiple serving cells grouped in multiple TAGs). NG-RAN ensures that each TAG contains at least one serving cell;

-	A non-CA capable UE can receive on a single CC and transmit on a single CC corresponding to one serving cell only (one serving cell in one TAG).

CA is supported for both contiguous and non-contiguous CCs. When CA is deployed frame timing and SFN are aligned across cells that can be aggregated. The maximum number of configured CCs for a UE is 16 for DL and 16 for UL.

###### 1.4.4.4.2	Supplementary Uplink

In conjunction with a UL/DL carrier pair (FDD band) or a bidirectional carrier (TDD band), a UE may be configured with additional, Supplementary Uplink (SUL). SUL differs from the aggregated uplink in that the UE may be scheduled to transmit either on the supplementary uplink or on the uplink of the carrier being supplemented, but not on both at the same time.

### 1.4.5	Layer 2

#### 1.4.5.1	Overview

The layer 2 of NR is split into the following sublayers: Medium Access Control (MAC), Radio Link Control (RLC), Packet Data Convergence Protocol (PDCP) and Service Data Adaptation Protocol (SDAP). The two figures below depict the Layer 2 architecture for downlink and uplink, where:

-	The physical layer offers to the MAC sublayer transport channels;

-	The MAC sublayer offers to the RLC sublayer logical channels;

-	The RLC sublayer offers to the PDCP sublayer RLC channels;

-	The PDCP sublayer offers to the SDAP sublayer radio bearers;

-	The SDAP sublayer offers to 5GC QoS flows;

-	Comp. refers to header compression and segm. to segmentation;

-	Control channels (BCCH, PCCH are not depicted for clarity).

NOTE:	The gNB may not be able to guarantee that a L2 buffer overflow will never occur. If such overflow occurs, the UE may discard packets in the L2 buffer.

Figure 1.19

Downlink Layer 2 Structure

Figure 1.20

Uplink Layer 2 Structure

Radio bearers are categorized into two groups: data radio bearers (DRB) for user plane data and signalling radio bearers (SRB) for control plane data.

#### 1.4.5.2	MAC Sublayer

###### 1.4.5.2.1	Services and Functions

The main services and functions of the MAC sublayer include:

-	Mapping between logical channels and transport channels;

-	Multiplexing/demultiplexing of MAC SDUs belonging to one or different logical channels into/from transport blocks (TB) delivered to/from the physical layer on transport channels;

-	Scheduling information reporting;

-	Error correction through HARQ (one HARQ entity per cell in case of CA);

-	Priority handling between UEs by means of dynamic scheduling;

-	Priority handling between logical channels of one UE by means of logical channel prioritisation;

-	Padding.

A single MAC entity can support multiple numerologies, transmission timings and cells. Mapping restrictions in logical channel prioritisation control which numerology(ies), cell(s), and transmission timing(s) a logical channel can use (see clause 16.1.2).

##### 1.4.5.2.4	HARQ

The HARQ functionality ensures delivery between peer entities at Layer 1. A single HARQ process supports one TB when the physical layer is not configured for downlink/uplink spatial multiplexing, and when the physical layer is configured for downlink/uplink spatial multiplexing, a single HARQ process supports one or multiple TBs.

#### 1.4.5.3	RLC Sublayer

##### 1.4.5.3.1	Transmission Modes

The RLC sublayer supports three transmission modes:

-	Transparent Mode (TM);

-	Unacknowledged Mode (UM);

-	Acknowledged Mode (AM).

The RLC configuration is per logical channel with no dependency on numerologies and/or transmission durations, and ARQ can operate on any of the numerologies and/or transmission durations the logical channel is configured with.

For SRB0, paging and broadcast system information, TM mode is used. For other SRBs AM mode used. For DRBs, either UM or AM mode are used.

##### 1.4.5.3.2	Services and Functions

The main services and functions of the RLC sublayer depend on the transmission mode and include:

-	Transfer of upper layer PDUs;

-	Sequence numbering independent of the one in PDCP (UM and AM);

-	Error Correction through ARQ (AM only);

-	Segmentation (AM and UM) and re-segmentation (AM only) of RLC SDUs;

-	Reassembly of SDU (AM and UM);

-	Duplicate Detection (AM only);

-	RLC SDU discard (AM and UM);

-	RLC re-establishment;

-	Protocol error detection (AM only).

##### 1.4.5.3.3	ARQ

The ARQ within the RLC sublayer has the following characteristics:

-	ARQ retransmits RLC SDUs or RLC SDU segments based on RLC status reports;

-	Polling for RLC status report is used when needed by RLC;

-	RLC receiver can also trigger RLC status report after detecting a missing RLC SDU or RLC SDU segment.

#### 1.4.5.4	PDCP Sublayer

##### 1.4.5.4.1	Services and Functions

The main services and functions of the PDCP sublayer include:

-	Transfer of data (user plane or control plane);

-	Maintenance of PDCP SNs;

-	Header compression and decompression using the ROHC protocol;

-	Ciphering and deciphering;

-	Integrity protection and integrity verification;

-	Timer based SDU discard;

-	For split bearers, routing;

-	Duplication;

-	Reordering and in-order delivery;

-	Out-of-order delivery;

-	Duplicate discarding.

Since PDCP does not allow COUNT to wrap around in DL and UL, it is up to the network to prevent it from happening (e.g. by using a release and add of the corresponding radio bearer or a full configuration).

##### 1.4.5.5	SDAP Sublayer

The main services and functions of SDAP include:

-	Mapping between a QoS flow and a data radio bearer;

-	Marking QoS flow ID (QFI) in both DL and UL packets.

A single protocol entity of SDAP is configured for each individual PDU session.

#### 1.4.5.6	L2 Data Flow

An example of the Layer 2 Data Flow is depicted on Figure 6.6-1, where a transport block is generated by MAC by concatenating two RLC PDUs from RBx and one RLC PDU from RBy. The two RLC PDUs from RBx each corresponds to one IP packet (n and n+1) while the RLC PDU from RBy is a segment of an IP packet (m).

NOTE:	H depicts the headers and subheaders.

Figure 1.21

Data Flow Example

#### 1.4.5.7	Carrier Aggregation

In case of CA, the multi-carrier nature of the physical layer is only exposed to the MAC layer for which one HARQ entity is required per serving cell as depicted on Fig. 1.22 and Fig. 1.23  below:

-	In both uplink and downlink, there is one independent hybrid-ARQ entity per serving cell and one transport block is generated per assignment/grant per serving cell in the absence of spatial multiplexing. Each transport block and its potential HARQ retransmissions are mapped to a single serving cell.

Figure 1.22

Layer 2 Structure for DL with CA configured

Figure 1.23

Layer 2 Structure for UL with CA configured

#### 1.4.5.8	Dual Connectivity

When the UE is configured with SCG, the UE is configured with two MAC entities: one MAC entity for the MCG and one MAC entity for the SCG. Further details of DC operation can be found in TS 37.340 [21].

#### 1.4.5.9	Supplementary Uplink

In case of Supplementary Uplink (SUL, see TS 38.101-1 [18]), the UE is configured with 2 ULs for one DL of the same cell, and uplink transmissions on those two ULs are controlled by the network to avoid overlapping PUSCH/PUCCH transmissions in time. Overlapping transmissions on PUSCH are avoided through scheduling while overlapping transmissions on PUCCH are avoided through configuration (PUCCH can only be configured for only one of the 2 ULs of the cell). In addition, initial access is supported in each of the uplink (see clause 9.2.6). An example of SUL is given in Annex B.

#### 1.4.5.10	Bandwidth Adaptation

With Bandwidth Adaptation (BA), the receive and transmit bandwidth of a UE need not be as large as the bandwidth of the cell and can be adjusted: the width can be ordered to change (e.g. to shrink during period of low activity to save power); the location can move in the frequency domain (e.g. to increase scheduling flexibility); and the subcarrier spacing can be ordered to change (e.g. to allow different services). A subset of the total cell bandwidth of a cell is referred to as a Bandwidth Part (BWP) and BA is achieved by configuring the UE with BWP(s) and telling the UE which of the configured BWPs is currently the active one.

Fig. 1.24  belows describes a scenario where 3 different BWPs are configured:

-	BWP1 with a width of 40 MHz and subcarrier spacing of 15 kHz;

-	BWP2 with a width of 10 MHz and subcarrier spacing of 15 kHz;

-	BWP3 with a width of 20 MHz and subcarrier spacing of 60 kHz.

Figure 1.24

BA Example

### 1.4.6	RRC

#### 1.4.6.1	Services and Functions

The main services and functions of the RRC sublayer include:

-	Broadcast of System Information related to AS and NAS;

-	Paging initiated by 5GC or NG-RAN;

-	Establishment, maintenance and release of an RRC connection between the UE and NG-RAN including:

-	Addition, modification and release of carrier aggregation;

-	Addition, modification and release of Dual Connectivity in NR or between E-UTRA and NR.

-	Security functions including key management;

-	Establishment, configuration, maintenance and release of Signalling Radio Bearers (SRBs) and Data Radio Bearers (DRBs);

-	Mobility functions including:

-	Handover and context transfer;

-	UE cell selection and reselection and control of cell selection and reselection;

-	Inter-RAT mobility.

-	QoS management functions;

-	UE measurement reporting and control of the reporting;

-	Detection of and recovery from radio link failure;

-	NAS message transfer to/from NAS from/to UE.

In order to enhance mobility robustness and performance, additional NR mobility enhancements is introduced in 3GPP Rel-16.  User data interruption is reduced during handover, which targets to 0ms. In addition,  the robustness during handover is improved by conditional handover.

#### 1.4.6.2	Protocol States

RRC supports the following states which can be characterised as follows:

-	RRC_IDLE:

-	PLMN selection;

-	Broadcast of system information;

-	Cell re-selection mobility;

-	Paging for mobile terminated data is initiated by 5GC;

-	DRX for CN paging configured by NAS.

-	RRC_INACTIVE:

-	PLMN selection;

-	Broadcast of system information;

-	Cell re-selection mobility;

-	Paging is initiated by NG-RAN (RAN paging);

-	RAN-based notification area (RNA) is managed by NG- RAN;

-	DRX for RAN paging configured by NG-RAN;

-	5GC - NG-RAN connection (both C/U-planes) is established for UE;

-	The UE AS context is stored in NG-RAN and the UE;

-	NG-RAN knows the RNA which the UE belongs to.

-	RRC_CONNECTED:

-	5GC - NG-RAN connection (both C/U-planes) is established for UE;

-	The UE AS context is stored in NG-RAN and the UE;

-	NG-RAN knows the cell which the UE belongs to;

-	Transfer of unicast data to/from the UE;

-	Network controlled mobility including measurements.

### 1.4.7	Support of verticals

#### 1.4.7.1. URLLC and Industrial IoT

The support of Ultra-Reliable and Low Latency Communications (URLLC) services is facilitated by the following features:

- Logical Channel Priority Restictions

- Packet duplication with dual connectivity or CA

- New QCI table for block error rate 10-5

- Physical layer desing as short transmission time interval

From Rel-16 onwards, URLCC and Industrial IoT use cases are further faciliated by

- NR PDCP duplication enhancements

- Prioritization/multiplexing enhancemetns

- NR TSC-related enhancements such as PDCP header compression

#### 1.4.7.2. V2X

From Rel-16, NG-RAN supports NR sidelink Communication and V2X sidelink Communication  The NG-RAN architecture supports the PC5 interface as illustrated in Fig.1.25. Sidelink transmission and reception over the PC5 interface are supported when the UE is inside NG-RAN coverage, irrespective of which RRC state the UE is in, and when the UE is outside NG-RAN coverage.

Figure 1.25

NG-RAN Architecture supporting the PC5 interface

Support of V2X services via the PC5 interface can be provided by NR sidelink Communication and/or V2X sidelink Communication. NR sidelink Communication may be used to support other services than V2X services.

NR sidelink Communication can support three types of transmission modes in the AS:

-	Unicast transmission, characterized by:

-	Support of at least one PC5-RRC connection between peer UEs;

-	Transmission and reception of control information and user traffic between peer UEs in sidelink;

-	Support of sidelink HARQ feedback;

-	Support of RLC AM;

-	Support of sidelink RLM for both peer UEs to detect RLF.

-	Groupcast transmission, characterized by:

-	Transmission and reception of user traffic among UEs belonging to a group in sidelink;

-	Support of sidelink HARQ feedback.

-	Broadcast transmission, characterized by:

-	Transmission and reception of user traffic among UEs in sidelink;