|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2022-2024 |  |  | SG12-LS8 |
| --- | --- | --- | --- | --- |
|  |  |  |  | STUDY GROUP 12 |
|  |  |  |  | Original: English |
| Question(s): |  | 2/12 |  | Geneva, 7-17 June 2022 |
| Ref.: SG12-TD148R1 |  |  |  |  |
| Source: |  | ITU-T Study Group 12 |  |  |
| Title: |  | LS on draft revised Recommendation ITU-T G.191: Software tools for speech and audio coding standardization |  |  |
| LIAISON STATEMENT |  |  |  |  |
| For action to: |  |  | - |  |
| For information to: |  |  | ITU-T SG16, ETSI TC STQ, 3GPP SA4 |  |
| Approval: |  |  | ITU-T SG12 (Geneva, 17 June 2022) |  |
| Deadline: |  |  | - |  |
| Contact: |  | Ludovic Malfait Dolby Laboratories United States |  | Tel: +44 7 586 054 457 E-mail: ludovic.malfait@dolby.com |

| Abstract: | This LS contains information about the recently consented draft revised Recommendation ITU-T G.191 “Software tools for speech and audio coding standardization” in ITU-T SG12. |
| --- | --- |

ITU-T Study Group 12 would like to inform you that draft revised Recommendation ITU-T G.191 “Software tools for speech and audio coding standardization” was consented by the recent ITU-T Study Group 12 meeting, Geneva, 7-17 June 2022.

The revision of ITU-T G.191, labelled STL2022, incorporates new tools, bug fixes and text edits. These updates were submitted on the open-source STL repository (https://github.com/openitu/STL), and are composed of several pull requests:

#### Non-inclusive terminology

Addressing issue #148 related to the request from 3GPP to remove non-inclusive terminology from the source code and documentation.

The proposed update replaces the following terminology in the code and documentation:

- Replace blacklist with blocklist

- Replace HARAKIRI with error_terminate

Link to the pull request: https://github.com/openitu/STL/pull/151/files

#### Implementation of ESDRU and ITU-R BS.1770-4 algorithms

This pull request proposes two algorithms:

- ESDRU (Energy-based Spatial Reference Unit) is one of the two spatial distortion reference unit defined in ITU-T P.811.

- ITU-R BS.1770-4 specifies an objective multichannel loudness measurement algorithm.

The pull request provides an update to the STL manual, an ANSI C implementation of the algorithm and with test vectors.

Link to the pull request: https://github.com/openitu/STL/pull/150/files

#### Compilation errors

Basic operators test framework:

test.c did not compile on MacOS due to missing #include <string.h>. The code now compiles in MacOS.

G.711 source code:

STL did not compile fully on Windows due to unistd.h being used and only available on Linux/Unix platform. The read/write function were replaced with fread/fwrite functions from the platform independent stdlib.h. With this update, code for  G.711 compiles on Linux, MacOS and Windows, and G.711 test vectors pass.

Rpeltp source code:

The use of unistd.h was necessary for the interactive mode. Interactive mode was disabled and include of unistd.h was removed.

Link to the pull request: https://github.com/openitu/STL/pull/154/files

Following approval, revised Recommendation ITU-T G.191 will be made available at https://www.itu.int/rec/T-REC-G.191 and STL2022 will be released at https://github.com/openitu/STL.

SG12 expresses its gratitude to all contributors to this release.

_______________________