|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2022-2024 |  | SG13-TD288/PLEN |  |
| --- | --- | --- | --- | --- |
|  |  |  | STUDY GROUP 13 |  |
|  |  |  | Original: English |  |
| Question(s): |  | 6/13 | Geneva, 15 – 26 July 2024 |  |
| TD |  |  |  |  |
| Source: |  | Editors |  |  |
| Title: |  | Draft new Recommendation ITU-T Y.3143 (formerly Y.IMT2020-qos-req-sh): “Quality of service assurance requirements and framework for smart healthcare supported by IMT-2020 and beyond” – for consent |  |  |
| Contact: |  | Guosheng Zhu Wuhan Rayton Network Technology China |  | Tel: +86-2788666186 E-mail:zhugs@rayton-networks.com |
| Contact: |  | Xiaoyun Qi Hubei University China |  | Tel: +86-27 88666186 Fax: +86-2788665505 E-mail: qixiaoyun@hubu.edu.cn |
| Contact: |  | Zhijun Sun Wuhan Rayton Network Technology China |  | Tel:+86-2788666186 E-mail:sunzj@rayton-networks.com |
| Contact: |  | Feihong Liu China |  | Tel: +86-2788665505 E-mail: liufeihong@bripath-medical.com |
| Contact: |  | Taesang Choi ETRI Republic of Korea |  | Tel: +82 42 860 5628 Fax: +82 52 860 6858 Email: choits@etri.re.kr |

| Abstract: | The baseline text is TD732/WP1 (Geneva, 4-15 March 2024). This TD was created during Geneva, 15 – 26 July 2024. The meeting agreed to accept proposals made by C-1034 based on the discussions during the meeting. The meeting agreed to submit it for consent at this meeting. |
| --- | --- |

Draft new Recommendation ITU-T Y.3143 (formerly Y.IMT2020-qos-req-sh)

Quality of service assurance requirements and framework for smart healthcare supported by IMT-2020 and beyond

Summary

This Recommendation specifies the quality of service (QoS) assurance requirements and framework for the smart healthcare supported by the international mobile telecommunications 2020 (IMT-2020) and beyond.

It first provides an overview of smart healthcare supported by IMT2020 and beyond. The QoS considerations for smart healthcare supported by IMT2020 and beyond are described. The specification of the QoS assurance requirements and a framework to reflect the identified QoS considerations is followed. Finally, the smart healthcare application scenarios with QoS metrics extended from the requirements are described in Appendix I.

Keywords

Framework, IMT-2020, QoS assurance, requirements, smart healthcare.

Table of Contents

1. Scope	4

2. References	4

3. Definitions	5

3.1 Terms defined elsewhere	5

3.1.  Terms defined in this Recommendation	5

4. Abbreviations and acronyms	5

5. Conventions	6

6. Overview	7

7. QoS considerations	7

8. QoS assurance requirements	9

8.1. High level requirements	9

8.2. QoS planning	10

8.3. QoS provisioning	10

8.4. QoS monitoring	11

8.5. QoS optimization	11

9. Framework	11

10. Security considerations	13

Appendix I   Smart healthcare application scenarios with QoS metrics	14

I.1	Remote health monitoring	14

I.2	Remote medical diagnosis	15

I.3	Remote robot aided surgery	15

I.4	Mobile emergency rescue	16

Bibliography	18

Draft new Recommendation ITU-T Y.3143 (formerly Y.IMT2020-qos-req-sh)

Quality of service assurance requirements and framework for smart healthcare supported by IMT-2020 and beyond

- Scope

This Recommendation specifies QoS assurance high-level and functional requirements and a framework for the smart healthcare supported by IMT-2020 and beyond, the scope of this recommendation is as follows:

- Overview;

- QoS considerations;

- Requirements;

- Framework;

- Applications scenarios.

- References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.810]			Recommendation ITU-T H.810 (2017), Interoperability design guidelines for personal connected health systems: Introduction.

[ITU-T F.780.5]			Recommendation ITU-T F.780.5 (2024), Requirements, reference framework and use cases for telemonitoring systems in rapid deployment hospitals.

[ITU-T Y.3102]			Recommendation ITU-T Y.3102 (2018), Framework of the IMT-2020 network.

[ITU-T Y.3104]			Recommendation ITU-T Y.3104 (2018), Architecture of the IMT-2020 network.

[ITU-T Y.3106]			Recommendation ITU-T Y.3106 (2019), Quality of service functional requirements for the IMT-2020 network.

[ITU-T Y.3107]			Recommendation ITU-T Y.3107 (2019), Functional architecture for QoS assurance management in the IMT-2020 network.

[ITU-T Y.3109]			Recommendation ITU-T Y.3109 (2021), Quality of service assurance-related requirements and framework for virtual reality delivery using mobile edge computing supported by IMT-2020.

[ITU-T Y.3124]			Recommendation ITU-T Y.3124 (2023), Quality of service monitoring requirements and framework for IMT-2020 and beyond.

[ITU-T Y.3130]			Recommendation ITU-T Y.3130 (2018), Requirements of IMT-2020 fixed mobile convergence.

[ITU-T Y.3158]			Recommendation ITU-T Y.3158 (2022), Local shunting for multi-access edge computing in IMT-2020 networks.

[ITU-T Y.3170]			Recommendation ITU-T Y.3170 (2018), Requirements for machine learning-based quality of service assurance for the IMT-2020 network.

[ITU-T Y.3172]			Recommendation ITU-T Y.3172 (2019), Architectural framework for machine learning in future networks including IMT-2020.

[ITU-T Y.3175]			Recommendation ITU-T Y.3175 (2020), Functional architecture of machine learning-based quality of service assurance for the IMT-2020 network.

[ITU-T Y.4000]			Recommendation ITU-T Y.4000/Y.2060 (2012), Overview of the Internet of things.

- Definitions

## 3.1 Terms defined elsewhere

3.1.1	IMT-2020[b-ITU-T Y.3100]: Systems, system components, and related aspects that support to provide far more enhanced capabilities than those described in [b-ITU-R M.1645].

NOTE 1 – [b-ITU-R M.1645] defines the framework and overall objectives of the future development of IMT-2000 and systems beyond IMT-2000 for the radio access network.

NOTE 2 – This definition is based on [b-ITU-R M.2083-0].

3.1.2		Internet of things (IoT) [ITU-T Y.4000]: A global infrastructure for the information society, enabling advanced services by interconnecting (physical and virtual) things based on existing and evolving interoperable information and communication technologies.

3.1.3	multi-access edge computing [b-ETSI GS MEC 001]: System which provides an IT service environment and cloud-computing capabilities at the edge of an access network which contains one or more type of access technology, and in close proximity to its users.

3.1.4	quality of experience (QoE) [b-ITU-T P.10]:The degree of delight or annoyance of the user of an application or service.

NOTE – Recognizing on-going research on this topic, this is a working definition which is expected to evolve for some time. (This note is not part of the definition.)

3.1.5	quality of service (QoS) [b-ITU-T P.10]: The totality of characteristics of a telecommunications service that bear on its ability to satisfy stated and implied needs of the user of the service (see [b-ITU-T E.800]).

3.2.6	quality of service assurance [ITU-T Y.3109]: Functionalities or mechanisms that enable service providers to make statements with a degree of confidence that the service meets the quality characteristics or objectives specified elsewhere.

  - Terms defined in this Recommendation

None.

- Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

AF		Application Function

AN		Access Network

AI		Artificial Intelligence

AR		Augment Reality

CEF		Capability Exposure Function

CN		Core Network

CT		Computed Tomography

DN		Data Network

E2E		End to End

ECG		Electrocardiogram

FMC		Fixed Mobile Convergence

HIS 		Healthcare Information System

ICT		Information and Communication Technology

IMT		International Mobile Telecommunications

MEC		Multi-access Edge Computing

ML		Machine Learning

NACF		Network Access Control Function

NFC		Near Field Communication

NFR		Network Function Registry

NSSF		Network Slice Selection Function

QoE		Quality of Experience

QoS		Quality of Service

QRC		Quick Response Code

PCF		Policy Control Function

PHD		Personal Health Devices

PHG		Personal Health Gateway

RP		Reference Point

SH		Smart Healthcare

VR		Virtual Reality

UE		User Equipment

UPF		User Plane Function

USM		Unified Subscription Management

WAN		Wide Area Network

WBASN		Wireless Body Area Sensor Network

- Conventions

This Recommendation uses the following conventions:

The term "is required to" indicates a requirement which must be strictly followed, and from which no deviation is permitted, if conformance to this Recommendation is to be claimed.

The keywords "is recommended" indicate a requirement which is recommended but which is not absolutely required. Thus this requirement need not be present to claim conformance.

- Overview

Today’s healthcare systems are under great pressure to deliver high-quality care in the face of aging world population and changing patient expectations – and to do so while becoming more efficient, competitive, and sustainable [b-wp-smart healthcare]. Smart healthcare (SH) is the application of new information and communication technologies (ICTs) such as mobile communications, the Internet of Things (IoT), cloud computing, big data, artificial intelligence (AI), etc., together with modern biotechnology to establish a smart healthcare information system (HIS) platform.

SH will transform the traditional healthcare system in many ways, making healthcare more efficient, more convenient, and more personalized.

- From the perspective of patients, the use of new ICT technologies can enhance patients’ experiences. They can use wearable devices to monitor their health at all times, seek medical support through virtual assistants.

- From the perspective of doctors, a variety of intelligent medical decision support systems are used to assist and improve diagnosis.  More advanced remote surgery can be achieved through tactile surgical robots [b-3GPP TS 22.104] and virtual reality(VR)/ augmented reality (AR) technologies [ITU-T Y.3109].

- From the perspective of hospitals, new ICT technologies can be used to manage personnel materials and the supply chain, using integrated management platforms to collect information and assist decision-making.

Through the use of these technologies, SH can effectively reduce the cost and risk of medical procedures, improve the utilization efficiency of medical resources, promote exchanges and cooperation in different regions, push the development of self-service medical care, and ultimately make personalized medical services ubiquitous [b-smart healthcare].

Capabilities in the IMT-2020 networks and beyond such as high coverage, high bandwidth, low latency, massive connectivity are key enablers for SH. They enable SH to overcome existing barriers with current connectivity solutions, save precious time and save more lives.

SH applications supported by IMT-2020 networks have diverse and dynamic QoS [b-ITU-T P.10] and quality of experience (QoE) [b-ITU-T P.10] requirements (bandwidth, latency, jitter, mobility, etc.) and heterogeneous connectivity (fixed, mobile, etc.). Knowledge of those requirements is important for all stakeholders, including developers, network providers, and regulatory authorities to facilitate safe and effective healthcare.

This recommendation summarizes the representative SH applications and identifies the QoS considerations for SH. Based on these QoS considerations, this Recommendation specifies the QoS assurance requirements and a framework for efficient network QoS planning, QoS provisioning, QoS monitoring [ITU-T Y.3124] and QoS optimization [ITU-T Y.3106] [ITU-T Y.3107].

- QoS considerations

The representative SH applications supported by IMT-2020 networks and beyond include:

1) Remote health monitoring;

2) Remote medical diagnosis;

3) Remote robot aided surgery; and

4) Mobile emergency rescue, etc., which are shown in Figure 1.

Figure 1  Representative smart healthcare applications supported by IMT-2020 and beyond

Remote health monitoring in hospital or at home can provide complex, continuous, and ubiquitous monitoring data through personal health devices (PHDs) [ITU-T H.810] [ITU-T F.780.5].  It allows a healthcare professional to remotely interpret data necessary for a patient's medical follow-up and, if necessary, make decisions regarding the patient's state of health. The example PHDs include: pulse oximeter, blood pressure monitor, thermometer or weighing scales capabilities classes, where the physical transport media could be  ZigBee, Bluetooth, USB or NFC.

These wearable and portable devices that can be organized in an individual’s wireless body area sensor network (WBASN), which collects and periodically sends patients’ data to SH data center through a personal health gateway (PHG) [ITU-T H.810] [ITU-T F.780.5]. It can be equipped with additional sensors for measuring temperature, humidity, and light exposure data. In some cases, cameras can monitor the patient activity, especially for those at risk of falling.  Physiological data of patients maybe continuously collected and processed to maintain a reliable picture of the patient’s status. Therefore, the high connection and traffic density given by a high number of monitored patients within the limited area of the hospital or densely populated urban area (e.g., with over 2000 inhabitants/km2) can challenge the IMT-2020 fix mobile convergence communication network.

The QoS requirements on IMT-2020 network and beyond include, for example, could stably approach 300 Mbps to 1Gbps (including image/video data), latency (250 ms), jitter (25 ms), and service availability (99.9%) [b-Giulia].

Remote medical  diagnosis will help to serve patients in rural, remote or underdeveloped areas, where specialized physicians are hardly present. It  involves a remote expert in a large central hospital who controls a diagnosis robotic system deployed in a local medical facility. In this scenario, an advanced fixed and mobile converged (FMC) communication network is needed to provide seamless remote video interaction with both the local physician and the patient, and to collect data from biosensors and other examination equipment through, e.g., body images. AI or ML powered medical imaging auxiliary diagnosis tools can be deployed to analyze medical images based on pattern recognition and to relieve medical staff with heavy burdens. Deep learning models can also be developed to extract visual features from volumetric chest computed tomography (CT) exams [b-COVID]. Real-time video and diagnosis image transmission brings more severe challenges to the processing capabilities, latency and reliability.

The QoS requirements on IMT-2020 network and beyond include, for example, high bandwidth (20-50Mbps), low delay (80-100ms), high reliability (99.999%) and low jitter (20ms)  [b-Giulia].

Remote robot aided surgery is seeing a rapid development and a spreading of use, given its many advantages over traditional surgery.  Remote robot aided surgery is particularly suitable to invasive surgical procedures that require delicate tissue manipulation and access to areas with difficult exposure. It is achieved through complex systems that translate the surgeon’s hand movements into smaller, precise movements of tiny instruments that can generally bend and rotate far more than a human hand is capable of doing inside the patient’s body. Typical surgery robotic systems can have around 40 actuators and the same number of sensors which allows to compute the data rate requires in each direction in order to execute a given movement.In order to make telesurgery remotely available, highly demanding communications requirements have been recently identified. Indeed, in order to experience a natural perception during surgery, physicians should be supported by a stable, real-time, haptic, and visual feedback coming from the surgical device and the camera pointing at the anatomical target.

This requires, for example, data rates of at least 1Gbps, deterministic latency (less than 5 ms) and jitters of (less than 2 ms), and the reliability should not be lower than (99.99999%)  [b-3GPP-TS-22.104][b-Giulia].

Mobile emergency rescue involving life signs and custody, diagnosis and examination and video consultation. Diagnosis and examination data volume depends on the category of  inspection, with remote. The surveillance information of life signs continues to upload, and the communication rate is about 200kbps. The mainstream of the car video diagnosis is 1080P, which requires 10Mbps communication speed.  In the future, the communication rate required for 4K high -definition video consultation will be further increased.

These solutions, as teleconference, require high data rates (more than 10Mbps) and low latency (less than 10ms), in order to guarantee smooth streaming at moderately high speed ambulance (e.g., up to 100 km/h) [b-Giulia].

Based on the high level QoS considerations described above, Clause 8 specifies QoS assurance requirements for SH supported by IMT-2020 and beyond (SH-IMT2020). To fulfil the requirements, a framework of QoS assurance for SH is specified in Clause 8.

- QoS assurance requirements

  - High level requirements

–	SH-IMT2020 is required to support different types of SH services  with different QoS objectives;

–	SH-IMT2020 is required to support life-critical service continuity and reliability for  SH users and organizations;

–	SH-IMT2020 is required to support fixed and/or mobile network connectivity for supporting different types of medical sensors and devices;

NOTE: example of medical devices: medical diagnosis equipment, medical monitoring equipment, medical robots, ambulances, office computers, cell phones, wearable equipment, personal health gateways etc.;

–	SH-IMT2020 is required to support real time massive data acquisition and monitoring of the patients' multiple physiological parameter data;

NOTE: examples of physiological parameter data are electrocardiogram (ECG), heart rate, blood pressure, blood sugar, blood oxygen saturation, and body temperature;

–	SH-IMT2020 is required to support high bandwidth for ultra-high-definition systems for efficient and intelligent medical diagnosis;

–	SH-IMT2020 is required to support high data rates and low latency and high mobility for mobile emergency rescue;

–	SH-IMT2020 is required to support end to end (E2E) QoS lifecycle management and assurance for SH services and applications;

–	SH-IMT2020 is required to support monitoring and analysis of SH network and service status;

–	SH-IMT2020 is required to support joint scheduling of network/computing/storage resources for SH service;

–	SH-IMT2020 is recommended to support high reliability for time sensitive remote medical  diagnosis and remote robot aided surgery;

  - QoS planning

–	SH-IMT2020 is required to support translation of a SH service level agreement (SLA) into the IMT-2020 and beyond network QoS policy rules;

–	SH-IMT2020 is required to support the estimation of network coverage, capacity, bandwidth, and delay with the consideration of SH service types, SH user equipment (UE) types and UE numbers;

–	SH-IMT2020 is required to support topology design and route planning per SH service type to ensure QoS assurance;

–	SH-IMT2020 is required to support joint planning of networking/computing/storage resources and SH service capabilities to ensure QoS assurance.

–	SH-IMT2020 is recommended to support seamless roaming design in the FMC IMT-2020 network for mobile SH users to ensure QoS assurance; [ITU-T Y.3130]

  - QoS provisioning

–	SH-IMT2020 is required to support unified and access-agnostic QoS provisioning for SH UEs from a core network (CN) perspective;

–	SH-IMT2020 is required to support E2E QoS provisioning;

–	SH-IMT2020 is recommended to support QoS mapping among SH UEs, PHGs, fixed/mobile access network (AN), transport network, CN, DN and smart healthcare information system, etc.;

–	SH-IMT2020 is required to support healthcare service specific QoS enforcement enabled by flow classification, marking, congestion avoidance, queue shaping and queue scheduling;

–	SH-IMT2020 is required to support joint scheduling of network/computing/storage resources and SH service capabilities to ensure QoS assurance;

–	SH-IMT2020 is recommended to support autonomous QoS policy management with global network view and agile softwarized network functions;

  - QoS monitoring

–	SH-IMT2020 is required to support QoS related status monitoring of SH UEs, PHGs, network/computing/storage resources and healthcare information system for SH service;

–	SH-IMT2020 is required to support SH service specific QoS data collection  of  SH UEs, PHGs, AN, CN, and healthcare information system [ITU-T Y.3124];

–	SH-IMT2020 is required to support SH service specific QoS data pre-processing of  UEs, PHGs, AN, CN ,and healthcare information system [ITU-T Y.3124];

–	SH-IMT2020 is required to support autonomic  SH QoS measurement and QoS anomaly detection;

–	SH-IMT2020 is required to report healthcare service specific QoS events upon detecting QoS anomalies that violate the SH service SLAs.

  - QoS optimization

–	SH-IMT2020 is recommended to support QoS optimization based on joint scheduling of network/computing/storage resources and SH service capabilities;

–	SH-IMT2020 is recommended to support healthcare service specific QoS optimization enabled by multi-access edge computing (MEC) ], machine learning (ML)/artificial intelligence (AI)[ITU-T Y.3158][b-ETSI GS MEC 001 [ITU-T Y.3170] [ITU-T Y.3172] [ITU-T Y.3175], etc.;

–	SH-IMT2020 is recommended to support healthcare service specific QoS anomaly prediction based on the analysis of QoS data [ITU-T Y.3170] [ITU-T Y.3172] [ITU-T Y.3175].

–	SH-IMT2020 is recommended to support traffic prediction based on the analysis of collected QoS data;

–	SH-IMT2020 is recommended to support healthcare service specific routing optimization, traffic switching, splitting and steering based on the current SH traffic status;

- Framework

A conceptual framework of QoS assurance for SH supported by IMT-2020 and beyond is shown in Figure 2.

CEF: capability exposure function	NFR: network function registry	PCF: policy control function

USM: unified subscription management	NACF: network access control function	NSSF: network slice selection function

SMF: session management function	ASF: authentication server function	AF: application function

UPF: user plane function	RP-tn: reference point between UE and NACF	RP-an: reference point between AN and NACF

RP-au: reference point between AN and UPF	RP-ud: reference point between UPF and data network

Figure 2 – A conceptual framework of QoS assurance for smart healthcare supported by IMT-2020 and beyond

The framework is built over an IMT-2020 FMC network with SH QoS assurance-related capabilities. The access network consists of IMT-20220 mobile access network and fixed access network. The QoS assurance-related capabilities for SH are distributed among UEs, access networks, core networks and smart healthcare information system.

The description of the role of each component is as follows.

–	The smart healthcare information system supports different SH services and management functions.  Smart healthcare information system support SLA management and service level QoS monitoring functions. It can be seen as an application function (AF) with SH capabilities supported by IMT-2020 and beyond.

–	The SH AF can interact with IMT-2020 policy control function (PCF) through service-based interfaces  or capability exposure function interfaces[ITU-T Y.3102] [ITU-T Y.3104].  The SH AF provides application function exposure services to NFs in core network. The exposed service experience information includes: service experience information, UE mobility information, user data congestion information, exception information, etc,. [b-3GPP-TS29.517]. It can also influence traffic routing by providing these service information to the PCF in support of its QoS policy rule generation.

–	E2E SH QoS control is supported in the IMT-2020 core network (CN). The QoS functionalities (planning, provisioning, monitoring and optimization) are distributed in both the control plane (CP) and user plane function (UPF). The machine learning based AI technologies can be used for SH QoS optimization [ITU-T Y.3170] [ITU-T Y.3172] [ITU-T Y.3175] .

–	The SH access network is an IMT-2020 FMC network to support various SH applications with different QoS requirements. SH QoS provisioning and monitoring for QoS enforcement and QoS data collection is supported in the FMC network.

–	The SH edge is located near to the SH UEs. The SH edge is responsible for interaction with the CN and smart healthcare information system for time sensitive service optimization [ITU-T Y.3109]. Thanks to the local access, computing, storage and bandwidth provided by the SH edge, a one-stop solution is available for efficient live video delivery. Furthermore, the traffic cost of the smart healthcare information system can be reduced.

–	There are many different types of SH terminals (e.g., medical diagnosis equipment, medical monitoring equipment, medical IoT devices, robots, ambulances, office computers, cell phones, wearable equipment, etc) [ITU-T H.810] [ITU-T F.780.5] with fixed or mobile access capabilities which have different QoS requirements (See Appendix I). These terminals are connected to the IMT-2020 FMC network directly or  through a PHG. The  PHG should support at least one kind of wide area network (WAN) to provide basic communication services, such as public cellular network (e.g.,  IMT-2020) [ITU-T F.780.5][ITU-T Y.4000] .

- Security considerations

This Recommendation describes the QoS assurance requirements and framework for SH supported by IMT-2020 and beyond, therefore, general network security requirements and mechanisms in IP-based networks and IMT-2020 network should be applied [b-ITU-T Y.2701] [b-ITU-T Y.3101].The security mechanisms such as authentication, authority, accounting and encryption should be adopted to ensure security of SH and IMT-2020 system.

Appendix I

Smart healthcare application scenarios with QoS metrics

(This appendix does not form an integral part of this Recommendation.)

## I.1	Remote health monitoring

Remote health monitoring in hospital can provide complex, continuous, and ubiquitous monitoring data (blood pressure, oxygenation, heart beat rate, and other vitals) for many patients. Data are integrated, in real-time, to determine the current health condition of the patient, and in critical situations, an assistance request can be automatically triggered.

Pervasive monitoring generates traffic with data rates up to 1 Mb/s/patient. In general units, physiological data of patients maybe continuously collected and processed (in the cloud) to maintain a reliable picture of the patient’s status. Therefore, the high connection and traffic density given by a high number of monitored patients within the limited area of the hospital can challenge the communication network. The overall data rate could stably approach 300 Mb/s (including image data), while moderate requirements on latency(250 ms), jitter (25 ms), and service availability (1- 10-3) are expected.

Remote health monitoring at home can shift monitoring from hospital to the individual’s home. In this scenario, cheaper, yet reliable, biosensors can be provided to the user to monitor for the vitals and other physiological activities, e.g., the glucose level. Moreover, a smart house can be equipped with additional tiny and unobtrusive sensors for measuring temperature, humidity, and light exposure data. In some cases, cameras can monitor the patient activity, especially for those at risk of falling. Biosensors can include traditional acquisition systems, such as Holter’s electrocardiogram (ECG), but also wearable and portable devices that can be organized in an individual’s wireless body area sensor network (WBASN), which collects and periodically sends patients’ data to a hospital, a clinical centre, or even an ambulance, for both real-time monitoring and data storage.

Here, real-time continuous and heterogeneous big data collection, transmission, analysis, and feedback must be performed by the IMT-2020 network and beyond  in order to generate alarms when needed. Therefore, the latency requirement, together with the massive data transmission, could challenge the network performance: 1 Gb/s traffic can be generated by monitoring a densely populated urban area (e.g., with over 2000 inhabitants/km2). Moreover, with IMT-2020, a specific priority could be accommodated for different pervasive monitoring at-home modalities, e.g., video streaming or alarms, offering different QoS  levels.

The key requirements of remote health monitoring for IMT-2020 and beyond are as illustrated in Table I.1 [b-Giulia] [b-Tactile].

Table I.1 –Remote health monitoring

| Application Scenario | Data type | Latency | Bandwidth | Reliability | Jitter | Connection density |
| --- | --- | --- | --- | --- | --- | --- |
| Remote health monitoring | Temperatures | ≤250ms | 10kbps | 99.9% | ≤25ms | Remote health monitoring in hospital:1000 devices/km2 Remote health monitoring at home: 20000 devices/km2 |
|  | Blood pressure | ≤250ms | 10kbps | 99.9% | ≤25ms |  |
|  | Heart rate< | ≤250ms | 10kbps | 99.9% | ≤25ms |  |
|  | Respiration rate | ≤250ms | 10kbps | 99.9% | ≤25ms |  |
|  | ECG | ≤250ms | 10kbps | 99.9% | ≤25ms |  |
|  | 3D camera | ≤150ms | 300M-1Gbps | 99.9% | ≤30ms |  |
|  | Audio | ≤150ms | 10kbps | 99% | ≤30ms |  |

## I.2	Remote medical diagnosis

Sparse points-of-care are intended to serve patients in rural, remote or underdeveloped areas, where specialized physicians are hardly present (thus with low traffic and connection density). At those sites, general physicians may be available for patients’ basic examination with medical equipment. The examination record can be stored and encoded as quick response code (QRC).  The examination record can be read and shared on the Internet remotely by scanning the QRC [b-remote-consult]. They could receive teleconsultation by more expert colleagues through video conferencing systems.

The typical data of remote medical consultation is real-time 4K video. In addition, advanced examinations could be conducted by the remote expert clinician with the help of the local physician. Image remote diagnosis provides patient clinical medical records and image data, including image data collected by medical imaging equipment and some video data [b-5G-Hospital].

AI or ML powered medical imaging auxiliary diagnosis tools can be deployed to analyse medical images based on pattern recognition and to relieve medical staff with heavy burdens. Deep learning models can also be developed to extract visual features from volumetric chest computed tomography (CT) exams [b-COVID]. Real-time video and diagnosis image transmission brings more severe challenges to the processing capabilities, latency and reliability.

The key requirements of intelligent remote medical consultation and image diagnosis for IMT-2020 and beyond are as illustrated in Table I.2 [b-Giulia] .

Table I.2 –Remote medical consultation and image diagnosis

| Application Scenario | Latency | Uplink rate | Downlink rate | Reliability | Jitter |
| --- | --- | --- | --- | --- | --- |
| Remote medical consultation | ≤100ms | 20Mbps | 20Mbps | 99.9999% | 20ms |
| Remote image diagnosis | ≤80ms | 50Mbps | 50Mbps | 99.9999% | 20ms |

## I.3	Remote robot aided surgery

Remote robot aided surgery is seeing a rapid development and a spreading of use, given its many advantages over traditional surgery. It enables visual guidance during surgery and reduce invasiveness thus making the patient recovery faster.

Remote robot aided surgery is particularly suitable to invasive surgical procedures that require delicate tissue manipulation and access to areas with difficult exposure. It is achieved through complex systems that translate the surgeon’s hand movements into smaller, precise movements of tiny instruments that can generally bend and rotate far more than a human hand is capable of doing inside the patient’s body. Typical surgery robotic systems can have around 40 actuators and the same number of sensors which allows to compute the data rate requires in each direction in order to execute a given movement.

In order to make telesurgery remotely available, highly demanding communications requirements have been recently identified. Indeed, in order to experience a natural perception during surgery, physicians should be supported by a stable, real-time, haptic, and visual feedback coming from the surgical device and the camera pointing at the anatomical target: this requires high data rates (at least 1 Gb/s), short latency (less than 5 ms), small jitters (less than 2 ms), and high reliability (not be lower than 1-10-7), with low traffic connection density.

Furthermore, in some recent attempts, the integration of holographic models of the target anatomical structure has been proposed. In this case, requirements become further demanding: indeed, the real-time holographic rendering needs end-to-end latency below 5 ms. At the same time, the user data rate could easily jump to 2 Gb/s.

The key requirements of intelligent distribution automation for IMT-2020 and beyond are as illustrated in Table I.3 [b-Giulia].

Table I.3 –Remote robot aided surgery

| Application Scenario | Data type | Latency | Uplink rate | Reliability | Jitter |
| --- | --- | --- | --- | --- | --- |
| Remote robot aided surgery | Real time Multimedia | ≤150ms | 1Gbps-2Gbps | 99.9% | ≤5ms |
|  | Physical vital signs | ≤5ms | 2Mbps | 99.99999% | ≤2ms |
|  | Haptic feed back | ≤5ms | 400Kbps | 99.99999% | ≤2ms |

## I.4	Mobile emergency rescue

Emergency rescue refers to first-aid medical services carried out by cooperation among emergency personnel, ambulance, emergency command centre and hospitals. At the rescue and natural disaster rescue site, medical staff need to urgently perform injuries for patients, check and transmit the inspection results to the emergency command centre and hospital. For patients with troubles, remote treatment will be guided by the hospital through the mobile terminal. The equipment included a 4K camera providing detailed real-time videos of the patient with high colour resolution to clinicians assisting the paramedical staff from the hospital.

Emergency rescue involving life signs and custody, diagnosis and examination and video consultation. Diagnosis and examination data volume depends on the category of inspection, with remote. The surveillance information of life signs continues to upload, and the communication rate is about 200kbps. The mainstream of the car video diagnosis is 1080P, which requires 10Mbps communication speed[b-ITU-T Y.3122]. In the future, the communication rate required for 4K high -definition video consultation will be further increased.

These solutions, as teleconference, require high data rates and low latency, in order to guarantee smooth streaming at moderately high speed ambulance (up to 100 km/h).

The key requirements of intelligent distribution automation for IMT-2020 and beyond are as illustrated in Table I.4 [b-Giulia].

Table I.4 –Mobile emergency rescue

| Application Scenario | Data | Latency | Bandwidth | Reliability | Jitter | Mobility |
| --- | --- | --- | --- | --- | --- | --- |
| Mobile emergency rescue | Information of life signs | ≤10ms | 200kbps | 99.9% | ≤2ms | 100km/h |
|  | Car video (1080P) | ≤10ms | 10Mbps | 99.9% | ≤2ms | 100km/h |

# Bibliography

[b-3GPP TS 22.104]	3GPP TS 22.104 V19.1.0 (2023-09).Technical Specification Group Services and System Aspects; Service requirements for cyber-physical control applications in vertical domains;Stage 1(Release 19).

[b-3GPP-TS29.517]	3GPP TS 29.517 V18.4.0 (2023-12).Technical Specification Group Core Network and Terminals;5G System; Application Function Event Exposure Service;Stage 3 (Release 18).

[b-COVID]				FG-AI4H Output, DT4HE Output 1(2020), Guidance on AI and digital technologies for COVID health emergency.

[b-ETSI GS MEC 001]   ETSI GS MEC 001 (2019), Multi-access edge computing (MEC) terminology.

[b-Giulia]			Cisotto, Giulia, Edoardo Casarin, and Stefano Tomasin. "Performance Requirements of Advanced Healthcare Services over Future Cellular Systems." 2019, https://arxiv.org/pdf/1906.01503

[b-remote-consult]	A QR code help consultation to solve the problem of patients in the seventies. http://www.app.dawuhanapp.com/p/27896092.html

[b-ITU-R M.1645]	Recommendation ITU-R M.1645 (2003), Framework and overall objectives of the future development of IMT-2000 and systems beyond IMT-2000.

[b-ITU-R M.2083-0]	Recommendation ITU-R M.2083-0 (2015), IMT vision – Framework and overall objectives of the future development of IMT for 2020 and beyond.

[b-ITU-T E.800]	Recommendation ITU-T E.800 (2008), Definitions of terms related to quality of service.

[b-ITU-T P.10]		Recommendation ITU-T P.10/G.100 (2017), Vocabulary for performance, quality of service and quality of experience.

[b-ITU-T Y.2701]	Recommendation ITU-T Y.2701 (2007), Security requirements for NGN release 1.

[b-ITU-T Y.3100]	Recommendation ITU-T Y.3100 (2017),Terms and definitions for IMT-2020 network.

[b-ITU-T Y.3122]	Recommendation ITU-T Y.3122 (2023), Quality of service assurance requirements and framework for smart grid supported by IMT-2020 and beyond.

[b-5G-Hospital]		Specification for Hospital Network Construction Based on 5G Technology (Radio Access Network Part); https://www.ntmchc.cn/ucenter/union/unionIndex.do?tab=synergy

[b-smart healthcare]	Tian S, Yang W, Le Grange J M, et al. Smart healthcare: making medical care more intelligent[J]. Global Health Journal, 2019, 3(3): 62-65.

[b-wp-smart healthcare]	Siemens white paper. Smart Healthcare Digital Transformation, https://www.siemens.com/us/en/industries/healthcare/smart-hospital-why-white-paper.html.

[b-tactile]			Zhang Q, Liu J, Zhao G. Towards 5G enabled tactile robotic telesurgery[J]. arXiv preprint arXiv:1803.03586, 2018.

__________________