|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2017-2020 |  |  | SG2-TD095R1/PLEN |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | STUDY GROUP 2 |  |
|  |  |  |  | Original: English |  |
| Question(s): |  |  | 7/2 | Virtual, 8-19 November 2021 |  |
| TD |  |  |  |  |  |
| Source: |  |  | Editor, Q.819 |  |  |
| Title: |  |  | Revised baseline text for Q.819 (ex. Q.rest): "REST-based management services" (for consent) |  |  |
| Purpose: |  |  | Discussion |  |  |
| Contact: |  | GUO Shaoyong Beijing University of Posts and Telecommunications( BUPT) China |  |  | Tel: +86 10 61198090 ext. 8710 Fax: +86 10 62283412 Email: syguo@bupt.edu.cn |
| Contact: |  | WANG Zhili Beijing University of Posts and Telecommunications( BUPT) China |  |  | Tel: +86 10 61198090 - 8726 Fax: +86 10 6228 3412 Email: zlwang@bupt.edu.cn |

| Keywords: | Distributed processing; REST, managed objects, network management interfaces, JSON, JSON Schema.Distributed processing; REST, managed objects, network management interfaces, JSON, JSON Schema. |
| --- | --- |
| Abstract: | This document defines a set of services required to support REST-based interfaces and along with Recommendation X.785 composes a framework for REST-based network management interfaces. It specifies protocol requirements, and defines some network management-specific support services, which are notification service, heartbeat service, and containment services. The YAML/JSON interface definitions for the network management-specific support services are also provided. The changes are mainly editorials (some more clearer explanations, formatting, font and font sizes, etc, and removed the solved comments.) |

AAP Summary:

Recommendation ITU-T Q.819 defines a set of services required to support REST-based interfaces and along with Recommendation ITU-T X.785 composes a framework for REST-based network management interfaces. It specifies protocol requirements, and defines some network management-specific support services, which are notification service, heartbeat service, and containment service. The JSON/YAML interface definitions for the network management-specific support services are also provided.

# Draft Recommendation ITU-T Q.819 (ex. Q.rest)

# REST-based Management Services

# 1	Scope

The network management architecture defined in [ITU-T M.3010] introduces the use of multiple management protocols. So far, the GDMO/CMIP, CORBA GIOP/IIOP, SMI/SNMP, Web Service/SOAP are possible choices at the application layer. Based on the management interface specification methodology defined in [ITU-T M.3020], more technology-based paradigms can be introduced into network management interfaces, and REST/HTTP is now an additional paradigm for network management.

This Recommendation, together with Recommendation [ITU-T X.785] sets out to define a framework for defining how interfaces supported by management systems and network elements should be modelled using REST/JSON schema. It is within the scope of this Recommendation to provide the following guidelines or instructions:

–	protocol requirements for using REST in network management;

–	how a REST-based notification service is used in network management interfaces;

–	how to monitor the availability of a REST notification forwarding mechanism;

–	how to access the containment information through a REST containment service;

–	compliance and conformance requirements.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

| [ITU-T M.3010] | Recommendation ITU-T M.3010 (2000), Principles for a telecommunications management network. |
| --- | --- |
| [ITU-T M.3020] | Recommendation ITU-T M.3020 (2017), Management interface specification methodology. |
| [ITU-T X.701] | Recommendation ITU-T X.701 (1997), Information technology – Open Systems Interconnection – Systems management overview. |
| [ITU-T X.703] | Recommendation ITU-T X.703 (1997), Information technology – Open Distributed Management Architecture. |
| [ITU-T X.785] | Recommendation ITU-T X.785 (2021), Guidelines for defining REST-based managed objects and management interfaces. |
| [RFC 3986] | IETF RFC 3986 (2005): Uniform Resource Identifier (URI): Generic Syntax. |
| [RFC 6585] | IETF RFC 6585 (2012): Additional HTTP Status Codes. |
| [RFC 7230] | IETF RFC 7230 (2014): Hypertext Transfer Protocol (HTTP/1.1): Message Syntax and Routing. |
| [RFC 7231] | IETF RFC 7231 (2014): Hypertext Transfer Protocol (HTTP/1.1): Semantics and Content. |
| [RFC 7232] | IETF RFC 7232 (2014): Hypertext Transfer Protocol (HTTP/1.1): Conditional Requests. |
| [RFC 7540] | IETF RFC 7540 (2015): Hypertext Transfer Protocol Version 2 (HTTP/2) |
| [RFC 8259] | IETF RFC 8259 (2017), The JavaScript Object Notation (JSON) Data Interchange Format (JSON). |

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	agent [ITU-T M.3020] :

Encapsulates a well-defined subset of management functionality. It interacts with managers using a management interface. From the manager's perspective, the agent behaviour is only visible via the management interface.

3.1.2	managed object class [ITU-T X.701] :

A named set of managed objects sharing the same (named) sets of attributes, notifications, management operations (packages), and which share the same conditions for presence of those packages.

3.1.3	manager [ITU-T M.3020] :

Models a user of agent(s) and it interacts directly with the agent(s) using management interfaces.

Since the manager represents an agent user, it gives a clear picture of what the agent is supposed to do. From the agent perspective, the manager behaviour is only visible via the management interface.

3.1.4	notification [ITU-T X.703] :

An interaction for which the contract between the invoking object (client) and the receiving object (server) is restricted to the ability of the server to receive the contents of information sent by the client.

## 3.2	Terms defined in this Recommendation

This Recommendation does not define any new terms.

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

BNF	Backus-Naur Form

CMIP	Common Management Information Protocol

CORBA	Common Object Request Broker Architecture

DN	Distinguished Name

GDMO	Guidelines for the Definition of Managed Objects

GIOP	General Inter-ORB Protocol

HTTP	Hyper Text Transfer Protocol

IIOP	Internet Inter-ORB Protocol

JSON	JavaScript Object Notation

MO	Managed Object

RDN	Relative Distinguished Name

REST	REpresentational State Transfer

SMI	Structure of Management Information

SNMP	Simple Network Management Protocol

SOAP	Simple Object Access Protocol

TMN	Telecommunications Management Network

URI 	Unified Resource Identifier

URL	Unified Resource Location

XML	extensible Markup Language

XSD	XML Schema Definition

YAML	YAML Ain’t a Markup Language

# 5	Conventions

A few conventions are followed in this Recommendation to make the reader aware of the purpose of the text. While most of the Recommendation is normative, paragraphs succinctly stating mandatory requirements to be met by a management system (managing and/or managed) are preceded by a boldface "R" enclosed in parentheses, followed by a short name indicating the subject of the requirement, and a number. For example:

(R) EXAMPLE-1 An example mandatory requirement.

Requirements that may be optionally implemented by a management system are preceded by an "O" instead of an "R". For example:

(O) EXAMPLE-2 An example optional requirement.

The requirement statements are used to create compliance and conformance profiles.

Examples of YAML and JSON are included in this Recommendation and normative YAML or JSON schema specifying the data types, base classes and other  modelling constructs of the framework are included in Annex A. The JSON/YAML schemas are written in a 10 point courier typeface:

| A JSON schema example | { "title": "root", "items": { "title": "array item" } } |
| --- | --- |
| A YAML schema example | SomeType: type: object properties: attr1: type: string attr2: type: string enum: - e1 - e2 |

# REST-based network management services goal and requirements

This clause describes the key goals of the services framework and the requirements that help the REST-based network management services support these goals. Clause 6.1 introduces the goals of the REST-based services framework. Clause 6.2 then provides terminology and requirements. The requirements in this clause are requirements that the framework must satisfy and which are based on telecommunications management needs. Clauses 7, 8, 9 then describe a framework that meets these needs and defines how to achieve the requirements of this clause by using REST-based services in a certain way.

## Goals

This Recommendation sets out to define a framework for defining how interfaces supported by management systems and network elements should be modelled. Some key goals of the framework are identified here:

•	application interoperability

•	common REST-based support services.

This clause elaborates on these two goals.

### 6.1.1	Application interoperability

A key goal of the network management architecture and in particular the information architecture, is to promote a standard framework for providing interoperability and information exchange between systems from a diverse set of network management system suppliers. Interoperability between systems involves many aspects of development. At its lowest layer, a common communication mechanism must be in place to support a common syntax, the establishment of connectivity and the exchange of operation requests/replies between systems. This aspect of interoperability is inherently supported by the REST/HTTP specifications.

For network management, there is the need to provide application interoperability. That is, management systems from diverse suppliers will be utilized within a single administration's TMN to support different functions necessary to support the management of its networks. To simplify integration of these various suppliers' systems, they must agree on the semantics of the information being exchanged. This is accomplished with the specification of an information model. Guidelines for the definition of REST-based information models are specified in [ITU-T X.785], but the services defined here should also support those guidelines.

### 6.1.2	Common REST-based support services

A second aspect of this framework is the definition of common usage and profiling of the distributed processing environment of choice. This aspect of the framework should indicate the reasonable expectations network management system suppliers may have for one another. Rather than redefining the interface capabilities needed to support common network management functions such as notification service with each information model, the modelling guidelines in [ITU-T X.785] rely upon a set of support services. These support services enable the information models to be simpler, and also enhance interoperability.

## Information modelling dependencies

As described in the previous clause, the explicit modelling of resources that are manageable across an interface is central to application interoperability. The guidelines for defining REST-based managed objects detailed in [ITU-T X.785] describe the rules for modelling manageable resources. They also embody several decisions that must be supported by the network management REST-based services framework. This clause summarizes those points.

### 6.2.1	Access granularity

REST-based service interface granularity refers to the relationship between the resources that are modelled on an interface and the means by which they are accessed using REST-based services. The objects that represent manageable resources are called managed objects  (MOs). [ITU-T X.785] uses a service-grain modelling approach which means each modelled resource is only accessible through either the generic MO Access Service, or a user defined REST service specific for various MO classes.

### 6.2.2	Representation of containment and naming

Containment is a logical representation of how modelled resources contain other modelled resources. Containment has traditionally been a very important relationship in network management applications because it is a convenient means of identifying the large number of resources that typically must be managed. [ITU-T X.785] guidelines require that a unique name be assigned to each managed object, based in part on the name of the object that contains it. The REST-based management services must provide the means to store these names (and hence the containment relationships they represent).

### 6.2.3	Object creation and deletion

The REST-based platforms do not provide clients with the particular means to create or delete objects on remote systems. Instead, these functionalities are provided by remote systems for clients to create or delete objects on the remote system. [ITU-T X.785] provides a common object accessing service, so object creation and deletion can be model-independent. When deleting an object that contains other objects, the containment relationship shall be kept, so that there will not be any orphan objects existing in the remote systems. The containment information should be stored in the remote system and be maintained when creation and deletion happen.

## 6.3	Notifications

The framework needs to support the ability to:

•	deliver notifications

•	subscribe for notification types

•	forward notifications to multiple destinations

•	uniquely identify the resource that emits the notification.

# 7	Framework overview and protocol requirements

## 7.1	Framework overview

This framework for REST-based management interfaces is a collection of capabilities.  The framework defines the support services to be standards on network management interfaces conforming to this framework. The REST interfaces for these services will be defined in Annex A.

To support the software objects representing manageable resources, the framework requires that they implement some common basic capabilities. Therefore, one base class (YAML/JSON-based) is defined in [ITU-T X.785] for use in modelling network management resources. Managed object classes in information models must inherit and implement a basic set of capabilities from the base class in order to operate within this framework. Finally, some rules and conventions are defined for information modellers developing models for use with this framework. These consist of modelling guidelines and YAML/JSON style idioms. All of these are depicted graphically in Figure 1.

Figure 1 Framework overview

Figure 1 shows the framework in gray. In the middle are the application-specific objects that are supported by the framework. Along the bottom is a box representing the communication protocol: HTTP 1.1 or 2.0. Above that are a number of boxes with names in them representing the services that compose this framework. Along the top of the figure are icons representing the super class ManagedObject, and the generic MO accessing methods. Each managed object supported by this framework must ultimately inherit from the super class. Also shown in Figure 1 are icons of pages with up-turned corners representing standard object modelling conventions.

The framework services, represented as boxes with square corners, are defined in this Recommendation. The super class, object modelling conventions and other conversions are defined in Recommendation [ITU-T X.785].

## 7.2	Protocol, languages and services requirements

This clause defines the versions of the languages and protocols that are required to support this framework. REST-based technology and protocol specifications are defined by IETF  and OAI. Table 1 shows which version (including subsequent versions released up until the date of this Recommendation) of the applicable specifications must be supported to comply with this framework.

Table 1 – REST-based language, protocol and versions

| Language, protocol | Version | References |
| --- | --- | --- |
| YAML Schema (OAI) | 3.0 | [b-OAI-OAS3] |
| JSON Schema Core (IETF) | Draft 2020-12 （Draft-08-patch-1, December, 2020) | [RFC 8259] |
| HTTP protocol (IETF) | 1.1 (2014) 2.0 (August, 2017) | [RFC 7230], [RFC 7231], [RFC 7232], [RFC 7540] |

# 8	REST-based notification service and notification format

## 8.1	REST-based notification service overview

REST-base notification service provides a standard  approach for notification consumers and providers to transfer notification information using a topic-based publish/subscribe pattern. The contents of notification service include standard message exchanges to be implemented by notification providers, operational requirements expected of service providers and requesters that participate in notifications, and an JSON model that describes topics.

The REST-based notification service supports the asynchronous exchange of event messages between clients using a subscribe-and-publish paradigm.

### 8.2	Mapping of operations from Recommendation ITU-T M.3702

[ITU-T M.3702] provides protocol neutral requirements and analysis for the generic notification management service, which defines the generic functions that are to be implemented by a protocol-specific management paradigm. In this framework, the generic notification management model is mapped to JSON/HTTP definition.

The following Table 2 indicates the mapping from [ITU-T M.3702] to the REST-based notification service.

Table 2 – Notification Operations Mapping from M.3702 to Q.819

| No. | [ITU-T M.3702] |  | ITU-T Q.819 |  |
| --- | --- | --- | --- | --- |
|  | Operation name |  | Operation name |  |
|  | Request/ response | Parameters | Request/ response | Parameters |
| 1 | subscribeNotification |  | HTTP POST |  |
|  | Input parameter | – managerId – notificationTypes – filteringCriteria – destination | Request | "POST" {notificationServiceURI}/"subscriptions" {HTTPVersion} request body schema: $ref: "notificationApiDataSchema.json#/definitions/SubscribeNotificationRequest" |
|  | Output parameter | – subscriptionId – status | Response | {HTTPVersion} {statusCode} {reasonPhrase} Location: { notificationServiceURI }/"subscriptions"/{subscriptionId}{response body} |
| 2 | unsubscribeNotification |  | HTTP DELETE |  |
|  | Input parameter | – managerId – subscriptionId | Request | "DELETE" {notificationServiceURI}/"subscriptions"/{subscriptionId} {HTTPVersion} |
|  | Output parameter | – status | Response | {HTTPVersion} {statusCode} {reasonPhrase} {response body} |
| 3 | suspendSubscription |  | HTTP POST |  |
|  | Input parameter | – managerId – subscriptionId | Request | "POST" { notificationServiceURI }/"subscriptions"/{subscriptionId}/"suspendSubscription" {HTTPVersion} |
|  | Output parameter | – status | Response | {HTTPVersion} {statusCode} {reasonPhrase} {response body} Response body schema: $ref:"notificationApiDataSchema.json#/definitions/NotificationInfo" |
| 4 | resumeSubscription |  | HTTP POST |  |
|  | Input parameter | – managerId – subscriptionId | Request | "POST" { notificationServiceURI }/"subscriptions"/{subscriptionId}/"resumeSubscriptions" {HTTPVersion} |
|  | Output parameter | – status | Response | {HTTPVersion} {statusCode} {reasonPhrase} {response body} |
| 5 | getNotificationTypes |  | HTTP GET |  |
|  | Input parameter | – notificationIRPId | Request | "GET" {notificationServiceURI}/NotificationTypes {HTTPVersion} |
|  | Output parameter | – status – notificationType List | Response | {HTTPVersion} {statusCode} {reasonPhrase} {response body} Response body schema: $ref:"notificationApiDataSchema.json# /definitions/GetNotificationTypesResponse" |
| 6 | querySubscription |  | HTTP GET |  |
|  | Input parameter | – subscriptionId | Request | "GET" {notificationServiceURI}/"subscriptions"/{subscriptionId}{HTTPVersion} |
|  | Output parameter | – subscribed NotificationType – subscriptionStatus – destination – filteringCriteria – status | Response | {HTTPVersion} {statusCode} {reasonPhrase} {response body} Response body schema: $ref: "notificationApiDataSchema.json# /definitions/QuerySubscriptionResponse" |
| 7 | modifySubscription |  | HTTP PATCH |  |
|  | Input parameter | – subscriptionId – filteringCriteria – destination – notificationTypes | Request | "PATCH" {notificationServiceURI}/"subscribtions"/{subscriptionId} {HTTPVersion}{request body} Request body schema: $ref:"notificationApiDataSchema.json#/definitions/ModifySubscriptionRequest" |
|  | Output parameter | – status | Response | {HTTPVersion} {statusCode} {reasonPhrase} {response body} Response body schema: $ref: "notificationApiDataSchema.json# /definitions/NotificationInfo" |
| 8 | listAllSubscriptionIds |  | HTTP GET |  |
|  | Input parameter | – managerId | Request | "GET" {notificationServiceURI}/"subscribtions"/listAllSubscriptionIds {HTTPVersion} {request body} Request body schema: $ref: "notificationApiDataSchema.json# /definitions/ListAllSubscriptionsIdsRequest" |
|  | Output parameter | – subscriptionIdSet – status | Response | {HTTPVersion} {statusCode} {reasonPhrase} {response body} Response body schema: $ref:"notificationApiDataSchema.json#/definitions/ListAllSubscriptionsIdsResponse" |
| 9 | report notification: |  | HTTP POST (from provider to consumer) |  |
|  |  |  | Request | "POST" {destinationURI} {HTTPVersion} {request body} Request body schema: $ref: "notificationApiDataSchema.json# /definitions/NotifyRequest" |
|  |  |  | Response | {HTTPVersion} {statusCode} {reasonPhrase} {response body} |

A detailed interface definition using YAML can be found in clause A.1.

### 8.3	Notification format definitions

#### 8.3.1	Notification format to be used in this Recommendation

This clause will provide a description for notification format to be used in this Recommendation.

#### 8.3.2	Common notification header definition

This clause provides the detailed common notification header described in [ITU-T M.3702].

The following table provides the definition of the parameters of the common notification header.

Table 6 – Common notification header definition

| Parameter name | Qualifiers | JSON/YAML schema type | Descriptions |
| --- | --- | --- | --- |
| objectClass | M | string | It is the class name of the MO which sends out this notification. See clause 7.3.5 of [ITU-T M.3702] |
| objectInstance | M | uri | It is the DN of the MO which sends out this notification. See clause 7.3.5 of [ITU-T M.3702] |
| notificationId | M | string | It is the Identifier of the notification for the purpose of notification correlation. See clause 7.3.5 of [ITU-T M.3702] |
| eventTime | M | string | It is the time when this event happened. See clause 7.3.5 of [ITU-T M.3702] |
| systemDN | M | type:string format:uri | It is the DN of the system which sends out this notification. See clause 7.3.5 of [ITU-T M.3702] |
| notificationType | M | NotificationType: type: string enum: - objectCreation - objectDeletion - attributeValueChange - stateChange - communicationAlarm - environmentalAlarm - equipmentAlarm - processingErrorAlarm - qualityOfServiceAlarm - integrityViolation - operationalViolation - physicalViolation - securityViolation - timeDomainViolation - relationshipChange - heartbeat | It indicates the type of notification. See clause 7.3.5 of [ITU-T M.3702]. The possible common notification types defined in this Recommendation can be found in the list of the left column. This list can be extended to include new notification types when they are defined. |

#### 8.3.3	Common notification contents definition

1)	Notification contents for the objectCreation and objectDeletion notifications.

Table 7 – Notification contents for objectCreation and objectDeletion

| Notification parameter | Qualifiers | Data type | Descriptions |
| --- | --- | --- | --- |
| correlatedNotifications | O | CorrelatedNotifications type: array items: type: string | List of correlated notifications. |
| additionalText | O | string | Text message. |
| additionalInfo | O | string | Additional information not in text format. (The information will be provided in JSON strings. Below is the same.) |
| sourceIndicator | O | SourceIndicator: type: string format: enum - resourceOperation - managementOperation - unknown | Cause of event. |
| attributeList | O | NVPairList: type: object properties: attributeList: type: array items: $ref: '#/.../NVPair' NVPair: type: object properties: name: type: string value: type: string type: type: string | Attribute values. |

2) 	Notification contents for the attributeValueChange notification.

Table 8 – Notification contents for attributeValueChange

| Notification parameter | Qualifiers | Data type | Descriptions |
| --- | --- | --- | --- |
| correlatedNotifications | O | CorrelatedNotifications | List of correlated notifications. |
| additionalText | O | string | Text message. |
| additionalInfo | O | string | Additional information not in text format. |
| sourceIndicator | O | SourceIndicator | Cause of event. |
| attributeChanges | M | NVPairList | Changed attributes. |

3)	Notification contents for the stateChange notification.

Table 9 – Notification contents for stateChange

| Notification parameter | Qualifiers | Data type | Descriptions |
| --- | --- | --- | --- |
| correlatedNotifications | O | CorrelatedNotifications | List of correlated notifications. |
| additionalText | O | string | Text message. |
| additionalInfo | O | string | Additional information not in text format. |
| sourceIndicator | O | SourceIndicator | Cause of event. |
| stateChanges | M | NVPairList | Changed states. |

4)	Notification contents for the communicationAlarm, environmentalAlarm, equipmentAlarm, processingErrorAlarm, qualityOfServiceAlarm notifications.

Table 10 – Notification contents for alarms

| Notification parameter | Qualifiers | Data type | Descriptions |
| --- | --- | --- | --- |
| correlatedNotifications | O | CorrelatedNotifications | List of correlated notifications. |
| additionalText | O | string | Text message. |
| additionalInfo | O | string | Additional information not in text format. |
| probableCause | M | string | The probable cause of the alarm. |
| specificProblems | O | SpecificProblems: type: array items: type: string | Non standarized problems. |
| perceivedSeverity | M | AlarmSeverity: type: string format: enum - indeterminate - critical - major - minor - warning - cleared | It indicates the perceived severity of the alarm. See type definition for details. |
| backedUpStatus | O | boolean | "True" if backed up. |
| backUpObject | O | type: string format: uri | It indicates the DN of the backup object if the backUpStatus is "false". |
| trendIndication | O | TrendIndication: type: string format: enum - lessSevere - noChange - moreSevere | See type for details. |
| thresholdInfo | O | ThresholdInfo: type: object properties: attributeID: type: string observedValue: type: number format: float thresholdLevel: $ref: '#/.../ThresholdLevelInd' armTime: type: string format: date-time ThresholdLevelInd: type: object properties: indication: $ref: '#/.../ThresholdIndication' low: type: number format: float high: type: number format: float ThresholdIndication: type: string format: enum - up - down | See type for details. |
| stateChangeDefinition | O | NVPairList | It indicates the state changes in this alarm. |
| monitoredAttributes | O | type: array items: type: string | See type for details. |
| proposedRepairActions | O | type: array items: type: string | It indicates the proposed actions to repair this fault. |
| alarmEffectOnService | O | boolean | True if alarm is service effecting. |
| alarmingResumed | O | boolean | True if alarming was just resumed, possibly resulting in delayed reporting. |
| suspectObjectList | O | type: array items: type: string format: uri | Objects possibly involved in failure. |

5)	Notification contents for the integrityViolation, operationalViolation, physicalViolation, securityViolation, timeDomainViolation notifications.

Table 11 – Notification contents for violations

| Notification parameter | Qualifiers | Data type | Descriptions |
| --- | --- | --- | --- |
| correlatedNotifications | O | CorrelatedNotifications | List of correlated notifications. |
| additionalText | O | string | Text message. |
| additionalInfo | O | string | Additional information not in text format. |
| securityAlarmCause | M | string | The cause of the security alarm. |
| securityAlarmSeverity | M | AlarmSeverity | Clears allowed? ITU-T X.721 appears to restrict the cleared value on this alarm, but clears should be allowed. |
| securityAlarmDetector | M | SecurityAlarmDetector: type: object properties: mechanism: $ref: '#/.../UID' obj: type: string format: uri UID: type: object properties: uri: type: string format: uri value: type: integer format: unsignedLong | See type for details. |
| serviceUser | M | string | The user of the service which is violated. |
| serviceProvider | M | string | The service provider of the service which is violated. |

6) 	Notification contents for the relationshipChange notification.

Table 12 – Notification contents for relationshipChange

| Notification parameter | Qualifiers | Data type | Descriptions |
| --- | --- | --- | --- |
| correlatedNotifications | O | CorrelatedNotifications | List of correlated notifications. |
| additionalText | O | string | Text message. |
| additionalInfo | O | string | Additional information not in text format. |
| sourceIndicator | O | SourceIndicator | Cause of event. |
| relationshipChanges | M | AttributeChangeSet: type: array items: $ref: '#/.../AttributeChange' AttributeChange: type: object properties: attribugteName: type: string attributeType: type: string oldValue: type: string newValue: type: string | Changed relationship attributes. |

The detailed data type definitions for notification contents can be found in clause A.1.

# REST-based heartbeat service

The heartbeat service is used to verify the operation of the notification forwarding mechanism in a managed system, as well as the communications network between the managed system and managing system.

It periodically sends a small notification to a managing system interested in receiving it and that notification identifies the system that emitted the heartbeat. After configuring this service a managing system can ensure the notification service is functioning. Since these notifications flow through the same software and networks as notifications from other resources, they periodically verify the operation of these resources.

The heartbeat service contains the following operations:

(1) Heartbeat operation:

The “heartbeat” operation is used by the managed system to send a brief heartbeat notification to the management system. The heartbeat notification information type is HeartbeatMessageType. The operation is completed by the notify operation of the notification service defined. The corresponding REST interface definition is shown in Table 9-1.

Table 13 interface definition of heartbeat operation

| Name | Schema |
| --- | --- |
| REQUEST | "POST" { destinationURI }/" notifications " {HTTPVersion} {RequestBody} |
| RequestBody | HeartbeatMessageType : : = { Allof: [ { NotificationBaseType }, { systemLabel: string period: string timeStamp: string }] } |
| RESPONSE | {HTTPVersion} {StatusCode} {ReasonPhrase} Location: { destinationURI }/" notifications "/{ notificationId } {ResponseBody} |
| ResponseBody | HeartbeatMessageType |

(2) getHeartbeatAttributes operation:

The getHeartbeatAttributes operation is used to get the value of the systemLabel and period attributes in the heartbeat notification. The REST interface definition for this operation contains the following:

a) The HTTP method: the operation of the query attribute should be mapped to the "GET" request method.

b) The request URI: In the heartbeat service, the subscribed heartbeat notification is placed in the heartbeats collection resource of the heartbeat service for management, and is identified by the subscriptionId. The query parameter attributes are optional. If no query condition is added, it means that both systemLabel and period attribute values are obtained. If you want to query a specified attribute value, the attribute name to be queried is placed in the attributes query parameter of the request URI.

Based on the above analysis, the REST interface definition of the getHeartbeatAttributes operation can be found in Table 9-2.

Table 9-2 interface definition of getHeartbeatAttributes operation

| Name | Schema |
| --- | --- |
| REQUEST | "GET" heartbeatServiceURI "/ heartbeats/"subscriptionId ["?attributes="("systemLabel" ¦ "period")] HTTPVersion |
| RESPONSE | {HTTPVersion} {StatusCode} {ReasonPhrase} {ResponseBody} |
| ResponseBody | HeartBeatAttributes : : = { systemLabel: string period: string } |

(3) setHeartbeatAttributes operation:

The setHeartbeatAttributes operation is used to set the value of the systemLabel and period attributes in the heartbeat notification. The REST interface definition for this operation contains the following:

- The HTTP method: this operation is an operation to modify the attribute, and supports modification of some attributes, so it should be mapped to the 'PATCH' request method.

b) The content of the request body: the setHeartbeatAttributes operation needs to give the attribute name and attribute value to be modified in the request body. The manager can modify the systemLabel and period attribute values at the same time, or modify only one of the attribute values.

Based on the above analysis, the REST interface definition of the setHeartbeatAttributes operation can be found in Table 9-3.

Table 9-2 interface definition of setHeartBeatAttribute operation

| Name | Schema |
| --- | --- |
| REQUEST | "PATCH" { heartbeatServiceURI }/" heartbeats "/{subscriptionId} {HTTPVersion} {RequestBody} |
| RequestBody | HeartbeatAttributes : : = { systemLabel: string period: string } |
| RESPONSE | {HTTPVersion} {StatusCode} {ReasonPhrase} |

The detailed JSON/YAML Schema and REST interface definitions can be found in Annex A.2.

# REST-based containment service

In the network management field a function is needed to be able to report which objects are contained by a superior object, to verify that a superior object exists before a subordinate is created, to make sure two objects with the same name are not created, etc. The containment service is defined in this framework to support this function.

The main functions to be supported by the containment service is to enable a managing system to query a managed system with the name of an managed object, and receive back the names of the objects contained by this object. In addition, a means of getting names added to and removed from the service will be defined. These are not for use by managing systems but internally by managed objects, factories, and other parts of a managed system. They are provided to promote the development of reusable components, possibly by third parties, and are defined on an interface separate from that used by managing systems.

The containment service provides three operations to retrieve containment information. A short description of the operation semantics in the containment service is described as follows:

- exists operation:

The exists operation is used to check whether a specified MO is registered in the containment service or not. The REST interface definition for this operation contains the following:

a) The HTTP method: the operation of the query attribute should be mapped to the "GET" request method.

b) The request URI: In the containment service, the “ContainmentService/exists” is used as the collection resource for this operation, and the moIsntance parameter in the path will provide the DN of the specified MO instance to be checked.

Based on the above analysis, the REST interface definition of the exists operation can be found in Table 10-1.

Table 10-1 interface definition of exists operation

| Name | Schema |
| --- | --- |
| REQUEST | "GET" {ContainmentServiceURI} "/ContainmentService/exists/" {moInstance} {HTTPVersion} |
| RESPONSE | {HTTPVersion} {StatusCode} {ReasonPhrase} {ResponseBody} |
| ResponseBody | boolean |

- getContained operation:

The “getContained” operation, which is used to  retrieve the names of all the resources contained in the specified MO. It uses DN (URI) to identify the target MO and then return the list of the URIs of the MOs that are contained under this MO instance. The REST interface definition for this operation contains the following:

a) The request URI: Since the operation cannot be mapped to the basic CRUD operation, it is represented by a Task Resource URI, and the actionName is used to indicate the operation to be performed after the resource object URI to be operated. The actionName of the operation is represented by ‘getContained’.

b) The HTTP method: this operation is a query operation, so the ‘GET’ request method is used.

c) There are two input parameters for this operation. The first one is the baseMO, which indicates the root MO instance of a subtree to be queried. The other parameter is scope, which indicates the scope of the subtree to be queried for containment: which are BasicObjectOnly, WholeSubTree, IndividualLevel, BaseToLevel (the semantic explanation of the above four choices can be found in clause 6.3 of [b-ITU-T Q.818]), when the choices are IndividualLevel or BaseToLevel, an extra optional parameter level is provided, which is a possitive integer.

d) Possible response codes and corresponding response body contents: The possible status codes of the method are 200, 400, 404, 405, and 500. When the status code is 200, the contained object instance URI list information in the scope is returned in the response body. The type is defined as MOList, which is an array type of string. Each element in the list is a string type in URI format.

Based on the above analysis, the REST interface definition of the getContainment operation can be found in Table 10-2.

Table 10-2 interface definition of getContained operation

| Name | Schema |
| --- | --- |
| REQUEST | "GET" {ContainmentServiceURI} "/ContainmentService/getContained/" {moInstance}/{scope} {HTTPVersion} |
| actionName | “getContained” |
| RESPONSE | {HTTPVersion} {StatusCode} {ReasonPhrase} {ResponseBody} |
| ResponseBody | list of string |

- getContainedByClass operation:

The “getContainedByClass” operation, which is used to  retrieve the names of all the resources contained in the specified MO, whose MO class should be the same value as the input parameter. The semantic of this operation is very similar to the operation “getContained”, the only difference is that the returned MO name list is filtered by the input parameter class. The REST interface definition for this operation contains the following:

a) The request URI: The operation is represented by a Task Resource URI, and the actionName is ‘getContainedByClass’.

b) The HTTP method: this operation is a query operation, so the ‘GET’ request method is used.

c) There are three input parameters, baseMO, scope and class. The first two has the same semantic as defined in getContained opeation. The last parameter class is a string, which indicates the MO class for filtering.

d) Possible response codes and corresponding response body contents: The possible status codes of the method are 200, 400, 404, 405, and 500. When the status code is 200, the contained object instance URI list information in the scope with the specified class is returned in the response body.

Based on the above analysis, the REST interface definition of the getContainmentByClass operation can be found in Table 10-3.

Table 10-3 interface definition of getContainedByClass operation

| Name | Schema |
| --- | --- |
| REQUEST | "GET" {ContainmentServiceURI} "/ContainmentService/getContainedByClass/" {moInstance}/{scope}/{class} {HTTPVersion} |
| actionName | “getContainedByClass” |
| RESPONSE | {HTTPVersion} {StatusCode} {ReasonPhrase} {ResponseBody} |
| ResponseBody | list of string |

The detailed JSON/YAML Schema and REST interface definitions can be found in Annex A.3.

[RFC 7231] and [RFC 6585] provide dozens of definitions for status code, which will likely be referennced in implementations of this Recommendation. The list of status codes can be found in table A.2-1 of [ITU-T X.785].

# 11	Compliance and conformance

This clause defines the criteria that must be met by other standards documents claiming compliance to this framework and the functions that must be implemented by systems claiming conformance to this Recommendation.

## 11.1	System conformance points

This clause summarizes the individual functions described earlier in this Recommendation. These conformance points are then combined in profiles that must be supported by systems claiming conformance to this Recommendation.

1)	An implementation claiming conformance to the notification service must:

•	support the notification service requirements and interface described in clause 8 and defined in the YAML/JSON schema in clause A.1;

•	support the notification format described in clause 8.3 and defined in the YAML/JSON schema in clause A.1.

2)	An implementation claiming conformance to the heartbeat service must:

•	support the heartbeat service requirements and interface described in clause 9 and defined in the YAML/JSON schema in clause A.2.

3)	An implementation claiming conformance to the containment service must:

•	support the containment service requirements  and interface described in clause 10 and defined in the YAML/JSON schema in clause A.3.

## 11.2	Basic conformance profile

A system claiming conformance to the [ITU-T Q.819] basic profile shall support:

- the version of JSON/YAML schema and HTTP protocol as specified in clause 7.2;

- the notification service (see conformance point 1)

- the heartbeat service (see conformance point 2)

- the containment service (see conformance point 3).

Annex A
YAML/JSON Scheme definition of framework support services

(This annex forms an integral part of this Recommendation.)

## A.1	REST-based notification service interface

## A.1.1	JSON-schema definitions for REST-based notification service interface

The following provides the HTTP requests and responses for REST-based notification service interface.

1) subscribeNotification

REQUEST:

"POST" {notificationServiceURI}/"subscriptions" {HTTPVersion}

response body schema:

$ref: "notificationApiDataSchema.json#/definitions/SubscribeNotificationRequest"

RESPONSE:

{HTTPVersion} {statusCode} {reasonPhrase}

Location: { notificationServiceURI }/"subscriptions"/{subscriptionId}

{response body}

| statusCode | reasonPhrase | response body schema |
| --- | --- | --- |
| 201 | Created | null or $ref:"notificationApiDataSchema.json#/definitions/NotificationInfo" |
| 400 | Bad Request | $ref:”commonApiDataSchema.json#/definitions/ErrorInformation” |
| 404 | Not Found | null |
| 409 | Conflict | null |
| 500 | Internal Server Error | null |

2) unsubscribeNotification

REQUEST:

"DELETE" {notificationServiceURI}/"subscriptions"/{subscriptionId} {HTTPVersion}

RESPONSE:

{HTTPVersion} {statusCode} {reasonPhrase}

{response body}

| statusCode | reasonPhrase | response body schema |
| --- | --- | --- |
| 200 | OK | null |
| 404 | Not Found | null |
| 405 | Method Not Allowed | null |
| 500 | Internal Server Error | null |

3) suspendNotification

REQUEST：

"POST" { notificationServiceURI }/"subscriptions"/{subscriptionId}/
"suspendSubscription" {HTTPVersion}

RESPONSE:

{HTTPVersion} {statusCode} {reasonPhrase}

{response body}

| statusCode | reasonPhrase | response body schema |
| --- | --- | --- |
| 200 | OK | null |
| 404 | Not Found | null |
| 405 | Method Not Allowed | null |
| 409 | Conflict | null (Already suspended) |
| 500 | Internal Server Error | null |

4) resumeSubscriptions

REQUEST:

"POST" { notificationServiceURI }/"subscriptions"/{subscriptionId}/ "resumeSubscriptions" {HTTPVersion}

RESPONSE:

{HTTPVersion} {statusCode} {reasonPhrase}

{response body}

| statusCode | reasonPhrase | response body schema |
| --- | --- | --- |
| 200 | OK | null |
| 404 | Not Found | null |
| 405 | Method Not Allowed | null |
| 409 | Conflict | null (Already resumed) |
| 500 | Internal Server Error | null |

5) getNotificationTypes

REQUEST:

"GET" {notificationServiceURI}/"subscriptions"/{subscriptionId}/"getTypes"

{HTTPVersion}

RESPONSE:

{HTTPVersion} {statusCode} {reasonPhrase}

{response body}

| statusCode | reasonPhrase | response body schema |
| --- | --- | --- |
| 200 | OK | $ref: "notificationApiDataSchema.json# /definitions/GetNotificationTypesResponse" |
| 404 | Not Found | null |
| 500 | Internal Server Error | null |

6) querySubscription

REQUEST：

"GET" {notificationServiceURI}/"subscriptions"/{subscriptionId} {HTTPVersion}

RESPONSE:

{HTTPVersion} {statusCode} {reasonPhrase}

{response body}

| statusCode | reasonPhrase | response body schema |
| --- | --- | --- |
| 200 | OK | $ref: "notificationApiDataSchema.json# /definitions/QuerySubscriptionResponse" |
| 404 | Not Found | null |
| 500 | Internal Server Error | null |

7) modifySubscription

REQUEST:

"PATCH" {notificationServiceURI}/"subscribtions"/{subscriptionId}

{HTTPVersion}

{request body}

Request body schema:

$ref:"notificationApiDataSchema.json#/definitions/ModifySubscriptionRequest"

RESPONSE:

{HTTPVersion} {statusCode} {reasonPhrase}

{response body}

| statusCode | reasonPhrase | response body schema |
| --- | --- | --- |
| 200 | OK | null or $ref: "notificationApiDataSchema.json# /definitions/NotificationInfo" |
| 400 | Bad Request | $ref: "commonApiDataSchema.json# /definitions/ErrorInformation" |
| 404 | Not Found | null |
| 405 | Method Not Allowed | null |
| 500 | Internal Server Error | null |

8) listAllSubscriptionIds

REQUEST：

"POST" {notificationServiceURI}/"subscribtions"/listAllSubscriptionIds {HTTPVersion}

{request body}

Request body schema:

$ref: "notificationApiDataSchema.json#/definitions/ListAllSubscriptionsIdsRequest"

RESPONSE:

{HTTPVersion} {statusCode} {reasonPhrase}

{response body}

| statusCode | reasonPhrase | response body schema |
| --- | --- | --- |
| 200 | OK | $ref:"notificationApiDataSchema.json# /definitions/ListAllSubscriptionsIdsResponse" |
| 400 | Bad Request | $ref:"commonApiDataSchema.json#/definitions/ErrorInformation" |
| 404 | Not Found | null |
| 500 | Internal Server Error | null |

9) notify

REQUEST:

"POST" {destinationURI} {HTTPVersion}

{request body}

Request body schema:

$ref: "notificationApiDataSchema.json#/definitions/NotifyRequest"

RESPONSE:

{HTTPVersion} {statusCode} {reasonPhrase}

{response body}

response body schema:

| statusCode | reasonPhrase | response body schema |
| --- | --- | --- |
| 200 | OK | null |
| 400 | Bad Request | $ref:”commonApiDataSchema.json# /definition/ErrorInformation” |
| 404 | Not Found | null |
| 405 | Method Not Allowed | null |
| 500 | Internal Server Error | null |

## A.1.2	YAML-schema definitions for REST-based notification service

The following provides the YAML-schema definitions for REST-based notification service interface.

Note: The “notify” operation in A.1.1 (9) is mapped to a callback in YAML-schema interface definitions for the subscribeNotification operation of path /Subscription.

Table A.1-1 YAML-schema definitions for REST-based notification servic

openapi: 3.0.0 info: title: ITU-T_NotificationService description: 'This is the formal definition of the ITU-T Notification Service.' termsOfService: https://www.itu.int/en/ITU-T/about/Pages/default.aspx contact: email: zlwang@bupt.edu.cn license: name: Apache 2.0 url: http://www.apache.org/licenses/LICENSE-2.0.html version: 1.0.0 externalDocs: description: See Recommendation ITU-T Q.819 url: https://www.itu.int/itu-t/recommendations/index.aspx?ser=X servers: - url: http://www.itu.int/sg2/rest/NotificationService/v1.0.0 - url: https://www.itu.int/sg2/rest/NotificationService/v1.0.0 tags: - name: NotificationService description: All APIs realted to the ITU-T Notification Service externalDocs: description: Find out more url: https://www.itu.int/itu-t/recommendations/index.aspx?ser=X paths: /Subscription: post: tags: - NotificationService summary: "subscribe notification" description: "create an subscription in the notification services with the specified parameters list" operationId: "subscribeNotification" requestBody: description: "The input parameters of the subscribeNotification operation" content: application/json: schema: $ref: '#/components/schemas/SubscribeNotificationRequest' required: true responses: 201: description: "Subscription is successfully created, and the new Subscription ID value will be returned." content: application/json: schema: $ref: '#/components/schemas/subscriptionId' 400: description: "Parameter Error occurred in the subscribeNotification operation" content: application/json: schema: $ref: '#/components/schemas/ErrorInfo' 404: description: "The URI does not exist" content: {} 409: description: "Subscription Conflict" content: {} 500: description: "Internal Server Error" content: {} callbacks: notification: '{$request.query.requestBody.destination}': #'{$request.query.requestBody.destination}?event={$request.query.requestBody.eventTypes}': post: requestBody: content: application/json: schema: $ref: '#/components/schemas/NotificationInfo' #example: #messageasdfasdf: Some event responses: '200': description: OK get: tags: - NotificationService summary: "Get Subscription Information" description: "get the list of Identifiers of all the existing Notification Subscriptions" operationId: "listAllSubscriptionIds" responses: 200: description: "The attribute value list of the specified MO is retrieved successfully" content: application/json: schema: type: array items: $ref: "#/components/schemas/subscriptionId" 404: description: "Specified Subscription Not found" content: {} 500: description: "Internal Server Error" content: {} /Subscription={subscriptionId}: parameters: - name: subscriptionId in: path description: "The unique Identifier of the notification Subscription" required: true schema: type: string get: tags: - NotificationService summary: "Query Subscription Information" description: "query the attributes information of a specific Notification Subscription" operationId: querySubscription" responses: 200: description: "The attribute value list of the specified MO is retrieved successfully" content: application/json: schema: $ref: "#/components/schemas/SubscriptionInfo" 404: description: "Specified Subscription Not found" content: {} 500: description: "Internal Server Error" content: {} delete: tags: - NotificationService summary: "Unsubscribe Notification" description: "delete a specific Notification Subscription" operationId: "unsubscribeNotification" responses: 200: description: "The specified Subscription is deleted successfullly" content: {} 404: description: "Specified Subscription does not exist" content: {} 405: description: "Method Not Allowed, the specified Subscription cannot be deleted" content: {} 500: description: "Internal Server Error" content: {} patch: tags: - NotificationService summary: "Modify Subscription" description: "modify a specific Notification Subscription" operationId: "modifySuscription" requestBody: description: "The input parameters of the modifySuscription operation" content: application/json: schema: $ref: '#/components/schemas/ModifySubscriptionRequest' required: true responses: 200: description: "The specified Subscription is modified successfullly, and the modified Suscription information is returned" content: application/json: schema: $ref: '#/components/schemas/SubscriptionInfo' 400: description: "Parameter Error occurred in the modifySuscription operation" content: application/json: schema: $ref: '#/components/schemas/ErrorInfo' 404: description: "Specified Subscription does not exist" content: {} 405: description: "Method Not Allowed, the specified Subscription cannot be modified" content: {} 500: description: "Internal Server Error" content: {} /Subscription={subscriptionId}/suspend: parameters: - name: subscriptionId in: path description: "The unique Identifier of the notification Subscription" required: true schema: type: string post: tags: - NotificationService summary: "Suspend Subscription " description: "Suspend the specific Notification Subscription" operationId: "suspendSubscription" responses: 200: description: "The specified Subscription is suspended successfully" content: {} 404: description: "Specified Subscription Not found" content: {} 405: description: "Method Not Allowed" content: {} 409: description: "Subscription already Suspended" content: {} 500: description: "Internal Server Error" content: {} /Subscription={subscriptionId}/resume: parameters: - name: subscriptionId in: path description: "The unique Identifier of the notification Subscription" required: true schema: type: string post: tags: - NotificationService summary: "Resume Subscription " description: "Resume the specific suspended Notification Subscription" operationId: "resumeSubscription" responses: 200: description: "The specified Subscription is resumed successfully" content: {} 400: description: "Subscription already Resumed" content: {} 404: description: "Specified Subscription Not found" content: {} 405: description: "Method Not Allowed" content: {} 500: description: "Internal Server Error" content: {} /NotificationTypes: get: tags: - NotificationService summary: "Get Motification Types" description: "get the supportted Notification Types." operationId: "getNotificationTypes" responses: 200: description: "The attribute value list of the specified MO is retrieved successfully" content: application/json: schema: type: array items: $ref: "#/components/schemas/NotificationType" 404: description: "Specified Subscription Not found" content: {} 500: description: "Internal Server Error" content: {} components: schemas: NVPair: type: object required: - name - value properties: name: type: string value: type: string type: type: string NVPairList: type: object properties: attributeList: type: array items: $ref: '#/components/schemas/NVPair' SubscribeNotificationRequest: type: object properties: managerId: type: string notificationTypeList: #empty list means subscribing all notification types. type: array items: $ref: '#/components/schemas/NotificationType' filteringCritteria: #optional type: string destination: #URI of the notification subscriber type: string format: uri required: - managerId - destination NotificationType: type: string enum: - objectCreation - objectDeletion - attributeValueChange - stateChange - communicationAlarm - environmentalAlarm - equipmentAlarm - processingErrorAlarm - qualityOfServiceAlarm - integrityViolation - operationalViolation - physicalViolation - securityViolation - timeDomainViolation - relationshipChange - heartbeat subscriptionId: type: string ErrorInfo: type: object properties: code: type: string enum: - invalidObjectInstance - noSuchObjectClass - noSuchAttribute - invalidAttributeValue - missingAttributeValue - notFound - cannotBeDeleted message: type: string required: - code SubscriptionInfo: type: object properties: subscriptionId: type: string managerId: #the identifier of manager of this notification subscription type: string notificationTypeList: #empty list means subscribing all notification types. type: array items: $ref: '#/components/schemas/NotificationType' filteringCriteria: #optional type: string destination: #URI of the notification subscriber type: string format: uri subscriptionStatus: #Whether the subscription is suspended or resumed type: string enum: - suspended - resumed required: - subcriptionId - managerId - destination - subscriptionState ModifySubscriptionRequest: type: object properties: notificationTypeList: #empty list means subscribing all notification types. type: array items: $ref: '#/components/schemas/NotificationType' filteringCriteria: #optional type: string destination: #URI of the notification subscriber type: string format: uri NotificationInfo: type: object properties: notificationHeader: $ref: '#/components/schemas/NotificationHeader' notificationBody: $ref: '#/components/schemas/NotificationBody' required: - notificationHeader - notificationBody NotificationHeader: type: object properties: objectClass: type: string objectInstance: type: string format: uri notificationId: type: string eventTime: type: string # to be confirmed. format: date-time systemDN: type: string notificationType: $ref: '#/components/schemas/NotificationType' required: - objectClass - objectInstance - notificationId - eventTime - systemDN - notificationType NotificationBody: type: object #One of the following will be used in a concreate notification properties: objectCreationBody: $ref: '#/components/schemas/ObjectCreationDeletionBody' objectDeletionBody: $ref: '#/components/schemas/ObjectCreationDeletionBody' atributeValueChangeBody: $ref: '#/components/schemas/AttributeValueChangeBody' stateChangeBody: $ref: '#/components/schemas/StateChangeBody' alarmNotificationBody: #for communicationsAlarm, environmentalAlarm, equipmentAlarm, processingErrorAlarm, qualityOfServiceAlarm $ref: '#/components/schemas/AlarmNotificationBody' violationsNoitificationBody: $ref: '#/components/schemas/ViolationsNoitificationBody' relationshipChangeNotificationBody: $ref: '#/components/schemas/RelationshipChangeNotificationBody' heartbeatNotificationBody: $ref: '#/components/schemas/HeartbeatNotificationBody' ObjectCreationDeletionBody: type: object properties: commonAttributes: $ref: '#/components/schemas/CMNotificationCommonAttrs' attributeList: $ref: '#/components/schemas/NVPairList' AttributeValueChangeBody: type: object properties: commonAttributes: $ref: '#/components/schemas/CMNotificationCommonAttrs' attributeChanges: $ref: '#/components/schemas/NVPairList' required: - attributeChanges StateChangeBody: type: object properties: commonAttributes: $ref: '#/components/schemas/CMNotificationCommonAttrs' stateChanges: $ref: '#/components/schemas/NVPairList' required: - stateChanges CMNotificationCommonAttrs: type: object properties: correlatedNotifications: type: array items: type: string additionalText: type: string additionalInfo: type: string sourceIndicator: $ref: '#/components/schemas/SourceIndicator' SourceIndicator: type: string format: enum - resourceOperation - managementOperation - unknown AlarmNotificationBody: type: object properties: correlatedNotifications: type: array items: type: string additionalText: type: string additionalInfo: type: string probableCause: type: string specificProblems: type: array items: type: string perceivedSeverity: $ref: '#/components/schemas/AlarmSeverity' backedUpStatus: type: boolean backUpObject: type: string format: uri trendIndication: $ref: '#/components/schemas/TrendIndication' thresholdInfo: $ref: '#/components/schemas/ThresholdInfo' stateChangeDefinition: $ref: '#/components/schemas/StateChangeDefinition' moniterdAttributes: type: array items: type: string proposedRepairActions: type: array items: type: string alarmEffectOnService: type: boolean alarmingResumed: type: boolean suspectObjectList: type: array items: type: string format: uri required: - probableCause - perceivedSeverity AlarmSeverity: type: string format: enum - cleared - warning - indetermined - minor - major - critical TrendIndication: type: string format: enum - lessSevere - noChange - moreSevere ThresholdInfo: type: object properties: attributeID: type: string observedValue: type: number format: float thresholdLevel: $ref: '#/components/schemas/ThresholdLevelInd' armTime: type: string format: date-time ThresholdLevelInd: type: object properties: indication: $ref: '#/components/schemas/ThresholdIndication' low: type: number format: float high: type: number format: float ThresholdIndication: type: string format: enum - up - down StateChangeDefinition: $ref: '#/components/schemas/NVPairList' ViolationsNoitificationBody: type: object properties: correlatedNotifications: type: array items: type: string additionalText: type: string additionalInfo: type: string securityAlarmCause: type: string securityAlarmSeverity: $ref: '#/components/schemas/AlarmSeverity' securityAlarmDetector: $ref: '#/components/schemas/SecurityAlarmDetector' serviceUser: type: string serviceProvider: type: string required: - securityAlarmCause - securityAlarmSeverity - securityAlarmDetector - serviceUser - serviceProvider SecurityAlarmDetector: type: object properties: mechanism: $ref: '#/components/schemas/UID' obj: type: string format: uri UID: type: object properties: uri: type: string format: uri value: type: integer format: unsignedLong RelationshipChangeNotificationBody: type: object properties: commonAttributes: $ref: '#/components/schemas/CMNotificationCommonAttrs' relationshipChanges: $ref: '#/components/schemas/AttributeChangeSet' required: - relationshipChanges AttributeChangeSet: type: array items: $ref: '#/components/schemas/AttributeChange' AttributeChange: type: object properties: attribugteName: type: string attributeType: type: string oldValue: type: string newValue: type: string HeartbeatNotificationBody: type: object properties: systemLabel: type: string format: uri period: type: integer format: unitsInSeconds timeStamp: type: string format: date-time

## A.2	REST-based heartbeat service interface

## A.1.1	JSON-schema definitions for REST-based heartbeat service

The following provides the HTTP requests and responses for REST-based heartbeat service interface.

(1) Heartbeat operation

Table A.2-1 interface definition of heartbeat operation

| Name | Schema |
| --- | --- |
| REQUEST | "POST" { destinationURI }/" notifications " {HTTPVersion} {RequestBody} |
| RequestBody | HeartbeatMessageType : : = { Allof: [ { NotificationBaseType }, { systemLabel: string period: string timeStamp: string }] } |
| RESPONSE | {HTTPVersion} {StatusCode} {ReasonPhrase} Location: { destinationURI }/" notifications "/{ notificationId } {ResponseBody} |
| ResponseBody | HeartbeatMessageType |

(2) getHeartbeatAttributes operation:

Table A.2-2 interface definition of getHeartbeatAttributes operation

| Name | Schema |
| --- | --- |
| REQUEST | "GET" heartbeatServiceURI "/ heartbeats/"subscriptionId ["?attributes="("systemLabel" ¦ "period")] HTTPVersion |
| RESPONSE | {HTTPVersion} {StatusCode} {ReasonPhrase} {ResponseBody} |
| ResponseBody | HeartBeatAttributes : : = { systemLabel: string period: string’ } |

(3) setHeartbeatAttributes operation:

Table A.2-3 interface definition of setHeartbeatAttribute operation

| Name | Schema |
| --- | --- |
| REQUEST | "PATCH" { heartbeatServiceURI }/" heartbeats "/{subscriptionId} {HTTPVersion} {RequestBody} |
| RequestBody | HeartbeatAttributes : : = { systemLabel: string period: string } |
| RESPONSE | {HTTPVersion} {StatusCode} {ReasonPhrase} |

## A.2.2	YAML-schema definitions for REST-based heartbeart service

The following provides the YAML-schema definitions for REST-based heartbeart service interface.

Table A.2-4 YAML-schema definitions for REST-based heartbeart service

openapi: 3.0.0 info: title: ITU-T_HeartbeatService description: 'This is the formal definition of the ITU-T Heartbeat Service.' termsOfService: https://www.itu.int/en/ITU-T/about/Pages/default.aspx contact: email: zlwang@bupt.edu.cn license: name: Apache 2.0 url: http://www.apache.org/licenses/LICENSE-2.0.html version: 1.0.0 externalDocs: description: See Recommendation ITU-T Q.819 url: https://www.itu.int/itu-t/recommendations/index.aspx?ser=X servers: - url: https://www.itu.int/sg2/rest/HeartbeatService/v1.0.0 - url: http://www.itu.int/sg2/rest/HeartbeatService/v1.0.0 tags: - name: HeartbeatService description: All APIs realted to the ITU-T Heartbeat Service externalDocs: description: Find out more url: https://www.itu.int/itu-t/recommendations/index.aspx?ser=X paths: /HeartbeatService/{subscriptionId}: parameters: - name: subscriptionId in: path description: "The unique Identifier of the notification Subscription" required: true schema: type: string get: tags: - HeartbeatService summary: "Get Heartbeat Service Information" description: "get the list of Identifiers of all the existing Notification Subscriptions" operationId: "getHeartbeatAttributes" responses: 200: description: "The attribute value list of the specified MO is retrieved successfully" content: application/json: schema: $ref: "#/components/schemas/HeartbeatInfo" 404: description: "Specified Subscription Not found" content: {} 500: description: "Internal Server Error" content: {} patch: tags: - HeartbeatService summary: "Modify Heartbeat service attributes" description: "modify the attribute of a heartbeat sevice attributes." operationId: "modifyHeartbeatService" requestBody: description: "The input parameters of the modifySuscription operation" content: application/json: schema: $ref: '#/components/schemas/ModifyHeartbeatServiceRequest' required: true responses: 200: description: "The specified Subscription is modified successfullly, and the modified Suscription information is returned" content: application/json: schema: $ref: '#/components/schemas/SubscriptionInfo' 400: description: "Parameter Error occurred in the modifyHeartbeatService operation" content: application/json: schema: $ref: '#/components/schemas/ErrorInfo' 404: description: "Specified Subscription does not exist" content: {} 405: description: "Method Not Allowed, the specified Subscription cannot be modified" content: {} 500: description: "Internal Server Error" content: {} components: schemas: NotificationType: type: string enum: - objectCreation - objectDeletion - attributeValueChange - stateChange - communicationAlarm - environmentalAlarm - equipmentAlarm - processingErrorAlarm - qualityOfServiceAlarm - integrityViolation - operationalViolation - physicalViolation - securityViolation - timeDomainViolation - relationshipChange - heartbeat ErrorInfo: type: object properties: code: type: string enum: - invalidObjectInstance - noSuchObjectClass - noSuchAttribute - invalidAttributeValue - missingAttributeValue - notFound - cannotBeDeleted message: type: string required: - code SubscriptionInfo: type: object properties: subscriptionId: type: string managerId: #the identifier of manager of this notification subscription type: string notificationTypeList: #empty list means subscribing all notification types. type: array items: $ref: '#/components/schemas/NotificationType' filteringCriteria: #optional type: string destination: #URI of the notification subscriber type: string format: uri subscriptionStatus: #Whether the subscription is suspended or resumed type: string enum: - suspended - resumed required: - subcriptionId - managerId - destination - subscriptionState HeartbeatInfo: type: object properties: systemLabel: type: string format: uri period: type: integer format: unitsInSeconds required: - systemLabel - period ModifyHeartbeatServiceRequest: type: object properties: systemLabel: type: string format: uri period: type: integer format: unitsInSeconds HeartbeatNotificationBody: type: object properties: systemLabel: type: string format: uri period: type: integer format: unitsInSeconds timeStamp: type: string format: date-time

## A.3	REST-based containment service interface

## A.3.1	JSON-schema definitions for REST-based containment service

The JSON schema definitions for the REST-base containemnt service interface include the following information, as show in Table A.3-1, A.3-2 and A.3-3:

- exists operation

Table A.3-1 interface definition of exists operation

| Name | Schema |
| --- | --- |
| REQUEST | "GET" {ContainmentServiceURI} "/ContainmentService/exists/" {moInstance} {HTTPVersion} |
| RESPONSE | {HTTPVersion} {StatusCode} {ReasonPhrase} {ResponseBody} |
| ResponseBody | boolean |

- getContained operation

Table A.3-2 interface definition of getContained operation

| Name | Schema |
| --- | --- |
| REQUEST | "GET" {ContainmentServiceURI} "/ContainmentService/getContained/" {moInstance}/{scope} {HTTPVersion} |
| actionName | “getContained” |
| RESPONSE | {HTTPVersion} {StatusCode} {ReasonPhrase} {ResponseBody} |
| ResponseBody | list of string |

- getContainedByClass operation

Table A.3-3 interface definition of getContainedByClass operation

| Name | Schema |
| --- | --- |
| REQUEST | "GET" {ContainmentServiceURI} "/ContainmentService/getContainedByClass/" {moInstance}/{scope}/{class} {HTTPVersion} |
| actionName | “getContainedByClass” |
| RESPONSE | {HTTPVersion} {StatusCode} {ReasonPhrase} {ResponseBody} |
| ResponseBody | list of string |

## A.3.2	YAML-schema definitions for REST-based containment service

The following provides the YAML-schema definitions for REST-based containment service interface.

Table A.3-2 YAML-schema definitions for REST-based containment service

openapi: 3.0.0 info: title: ITU-T_ContainmentService description: 'This is the formal definition of the ITU-T Containment Service.' termsOfService: https://www.itu.int/en/ITU-T/about/Pages/default.aspx contact: email: zlwang@bupt.edu.cn license: name: Apache 2.0 url: http://www.apache.org/licenses/LICENSE-2.0.html version: 1.0.0 externalDocs: description: See Recommendation ITU-T Q.819 url: https://www.itu.int/itu-t/recommendations/index.aspx?ser=X servers: - url: http://www.itu.int/sg2/rest/ContainmentService/v1.0.0 - url: https://www.itu.int/sg2/rest/ContainmentService/v1.0.0 tags: - name: ContainmentService description: All APIs realted to the ITU-T Containment Service externalDocs: description: Find out more url: https://www.itu.int/itu-t/recommendations/index.aspx?ser=X paths: /ContainmentService/exists/{moInstance}: parameters: - name: moInstance in: path description: "The DN(URI) of the specified MO" required: true schema: type: string format: uri get: tags: - ContainmentService summary: "check whether the specified MO instance is registered in the containment service or not." description: "returns true if the specified MO instance exists in the containment service, otherewise it returns false" operationId: "exists" responses: 200: description: "The specified MO is found in the containment services." content: {} 404: description: "The specified MO is not found in the containment services." content: {} 500: description: "Internal Server Error" content: {} /ContainmentService/getContained/{baseMO}/{scope}: parameters: - name: baseMO in: path description: "The DN of the specified base MO which is the root of a subtree" required: true schema: type: string format: uri - name: scope in: path description: "The scope of the containment information to be returned associated with the root MO" required: true schema: $ref: '#/components/schemas/ScopeType' get: tags: - ContainmentService summary: "returns the MO instance list which are contained by the specified base MO instance." description: "returns the list of MO instance that are contained bu the base MO instance. the scope parameter can be of 4 possible values: BaseObjectOnly, WholeSubTree, IndividualLevel, IndividualLevel" operationId: "getContained" responses: 200: description: "The operation is executed successfully, and the list of contained MO instances are returned" content: application/json: schema: $ref: '#/components/schemas/MOList' 404: description: "The specified MO is not found in the containment services." content: {} 500: description: "Internal Server Error" content: {} /ContainmentService/getContainedByClass/{baseMO}/{scope}/{class}: parameters: - name: baseMO in: path description: "The DN of the specified base MO which is the root of a subtree" required: true schema: type: string format: uri - name: scope in: path description: "The scope of the containment information to be returned associated with the root MO" required: true schema: $ref: '#/components/schemas/ScopeType' - name: class in: path description: "The class of the contained information to be returned associated with the root MO" required: true schema: type: string get: tags: - ContainmentService summary: "returns the MO instance list which are contained by the specified base MO instance." description: "returns the list of MO instance that are contained bu the base MO instance. the scope parameter can be of 4 possible values: BaseObjectOnly, WholeSubTree, IndividualLevel, IndividualLevel, and the returned MO list is filtered by the class specified in the input parameter" operationId: "getContainedByClass" responses: 200: description: "The operation is executed successfully, and the list of contained MO instances are returned" content: application/json: schema: $ref: '#/components/schemas/MOList' 404: description: "The specified MO is not found in the containment services." content: {} 500: description: "Internal Server Error" content: {} components: schemas: ScopeEnumType: type: string format: enum - BasicObjectOnly - WholeSubtree - IndividualLevel - BaseToLevel ScopeType: type: object properties: scopeInd: $ref: '#/components/schemas/ScopeEnumType' level: type: integer MOList: type: array items: type: string format: uri

Bibliography

| [b-OAI-OAS3] | OpenAPI Initiative, OpenAPI Specification Version 3.0.0 (2017), http://spec.openapis.org/oas/v3.0.0. |
| --- | --- |
| [b-ITU-T Q.818] | Recommendation ITU-T Q.818 (2012), Web services-based management services. |

_______________________

URI is defined in [RFC 3986]. Based on the descriptions provided in [RFC 3986]: URL refers to the subset of URIs that, in addition to identifying a resource, provide a means of locating the resource by describing its primary access mechanism (e.g., its network "location"). within this Recommendation, only URI is used.