|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2017-2020 |  | SG11-TD1599/GEN |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | STUDY GROUP 11 |  |  |
|  |  |  | Original: English |  |  |
| Question(s): |  | 2/11 |  | Virtual, 17-26 March 2021 |  |
| TD |  |  |  |  |  |
| Source: |  | Editor |  |  |  |
| Title: |  | Output – draft Technical Report ITU-T TR-USSD “Low resource requirement, quantum resistant, encryption of USSD messages for use in Financial services” (virtual, 17-26 March 2021) |  |  |  |
| Purpose: |  | Information |  |  |  |
| Contact: |  | Assaf Klinger Vaulto Technologies Ltd. Israel |  |  | Tel: +972502138133 E-mail: assaf.klinger@gmail.com |

| Keywords: | Security, Certificate authority, SSGW, Digital Signature, Signalling Security |
| --- | --- |
| Abstract: | This document is the current text of draft Technical Report ITU-T TR-USSD “Low resource requirement, quantum resistant, encryption of USSD messages for use in Financial services”, as discussed at Q2/11 virtual meeting on 17-26 March 2021. |

This document is the current output text of TR-USSD, a technical report on “Low resource requirement, quantum resistant, encryption of USSD messages for use in Financial services”, as discussed at Q2/11 virtual meeting on 17-26 March 2021.

This document is based on SG11-TD1399/GEN (the output of Q2/11 meeting on 22-31 July 2020) , this meeting's discussion and results on the following documents:

| No. | Title | Source | Discussion |
| --- | --- | --- | --- |
| SG11-C602 | Output – draft Technical Report ITU-T TR-USSD “Low resource requirement, quantum resistant, encryption of USSD messages for use in Financial services” (virtual, 17-26 March 2021) | Vaulto Technologies Ltd. | Accepted with modifications |
| SG11-TD1520/GEN | LS/i/r on Comments from ETSI TC CYBER QSC to ITU-T SG11 on TR-USSD (SG11-LS149) [from ETSI TC Cyber WG QSC] | ETSI TC Cyber WG QSC | Comments in this liaison had been reviewed and discussed in Q2/11 meeting. The suggestion had been partially adopted. |

Contents

1	Scope	4

2	References	4

3	Terms and definitions	4

3.1	Terms defined elsewhere	4

3.2	Terms defined here	4

4	Abbreviations and acronyms	5

5	Introduction	6

6	How does USSD work	6

6.1	Network Architecture for USSD Signalling	6

6.2	Signalling flow example of a MO-USSD session	6

7	Examples of exploiting USSD vulnerabilities on to commit DFS fraud	8

7.1	Account takeover	8

7.2	Social engineering of sensitive credentials using USSD	8

8	Quantum Safe Cryptography (QSC)	8

8.1	Approaches to quantum safe cryptography	9

8.2	New algorithms for post-quantum cryptography	9

8.3	Doubled key-space algorithms	10

8.4	PKI algorithms	10

8.5	Available post-quantum software packages	11

8.5.1	CEX	11

8.5.2	liboqs	11

8.5.3	libpqcrypto	11

9	The uSIM as a computation platform for post-quantum crypto	11

9.1	SIM Card – background	11

9.2	USIM software and hardware description	11

9.3	USIM file system and applet structure	12

9.3.1	ICC Card file system example	12

9.3.2	ICC card - telecom data file	12

9.3.3	ICC card GSM data file	13

9.3.4	SIM card UICC example	13

USIM resources applicability to post-quantum cryptography algorithms	13

9.4	13

10	Applicability matrix between USIM platform and post-quantum crypto	14

11	Bibliography	14

Draft new Technical Report ITU-T TR-USSD

Low resource requirement, quantum resistant, encryption of USSD messages for use in Financial services

# Scope

This technical report is a result of ITU-T TR-SS7-DFS “SS7 vulnerabilities and mitigation measures for digital financial services transactions”. TR-SS7-DFS states that clear-text USSD is the most common medium of DFS financial transactions in the developing world, where there is no deployment 3G or 4G cellular infrastructure. A fact that leads to large scale financial fraud. This TR surveys the available and upcoming encryption technologies that can mitigate this risk, can be implemented OTT over existing 2G cellular infrastructure and require low computation resources which enable it to be deployed to UICC (uSIM) modules.

# References

[1] ITU-T TR-SS7-DFS

[2] 3GPP TS 23.090: “Unstructured Supplementary Service Data (USSD)”

[3] 3GPP TS 22.030: "Man-Machine Interface (MMI) of the User Equipment (UE)"

[4] 3GPP TS 31.102: “Characteristics of the Universal Subscriber Identity Module (USIM) application”

[5] Peter W. Shor (1997): "Polynomial-Time Algorithms for Prime Factorization and Discrete Logarithms on a Quantum Computer".

[6] Grover L.K: Quantum mechanics helps in searching for a needle in a haystack, Physical Review Letters 79 (1997), 325–328

[7] Grover L.K, Terry Rudolph: How significant are the known collision and element distinctness quantum algorithms ? Quantum Information & Computation 4 (2003), 201–206. MR 2005c:81037.

[8] Grover L.K.: A fast quantum mechanical algorithm for database search, Proceedings, 28th Annual ACM Symposium on the Theory of Computing, (May 1996) p. 212

[9] PQCRYPTO ICT-645622 final report: https://pqcrypto.eu.org/deliverables/d5.2-final.pdf

[10] “liboqs”: an open source C library for quantum-safe cryptographic algorithms,  https://github.com/open-quantum-safe/liboqs

[11] “CEX”: open-source C library for quantum-safe cryptographic algorithms, https://github.com/Steppenwolfe65/CEX

[12] “libpqcrypto”: ibpqcrypto is a new cryptographic software library produced by the PQCRYPTO project, https://ianix.com/pqcrypto/pqcrypto-deployment.html

[13] ISO/IEC 7816: Identification cards — Integrated circuit cards

# Terms and definitions

## Terms defined elsewhere

None

## Terms defined here

None

# Abbreviations and acronyms

| BTS | Base Transceiver Station |
| --- | --- |
| DFS | Digital Financial Station |
| GTP | GPRS Tunnelling Protocol |
| HLR & VLR | Home / Visitor Location Register. |
| IMEI | International Mobile Equipment Identity |
| IMSI & TMSI | International Mobile Subscriber Identity |
| MAP | Mobile Application Part. |
| MO-SMS | Mobile Originated SMS. |
| MO-USSD | Mobile Originated USSD transaction. |
| MSISDN | Mobile Station International Subscriber Directory Number |
| MT-SMS | Mobile Terminated SMS. |
| MT-USSD | Mobile Terminated USSD transaction |
| OTP | One Time Password. |
| PIN | Personal Identification Number |
| SMS | Short Messaging Service |
| SS7 | Signalling System No. 7 |
| STK | Sim Tool Kit |
| UE | User Equipment |
| USSD | Unstructured Supplementary Service Data |
| MMI | Man Machine Interface |
| MS | Mobile Station |
| MSC | Mobile Switch Centre |
| NE | Network Element |
| IE | Information Element |
| NIST | National Institute of Standards and Technology |
| XMSS | eXtended Merkle Signature Scheme |
| LMS | Leighton-Micali Signatures |
| NSA | National Security Agency |
| DH | Diffie-Hellman |
| ECDH | Elliptic Curve Diffie-Hellman |
| ECDSA | Elliptic Curve Digital Signature Algorithm |
| CNSA | Commercial National Security Algorithm |
| SIDH | Super Singular Diffie-Hellman Algorithm |
| QSC | Quantum Safe Cryptography (a.k.a Post Quantum Cryptography) |
| PKI | Public Key Infrastructure |
| JCVM | Java Card Virtual Machine |
| JCRE | Java Card Runtime Enviroment |

# Introduction

The world of Digital Financial Services (DFS) is based mostly on telecom, since in most countries where DFS is popular, most of the end-users do not have reliable and accessible means to connect to the internet due to poor 3G/4G deployment. Which makes USSD dominant communication channels in which the end-user communicates with the DFS provider. Moreover, using SS7 signalling attacks fraudsters masquerade themselves as the DFS provider to steal money from mobile accounts.

This document intends to survey new, Quantum Safe Cryptography (QSC) encryption technologies that can safeguard USSD which use  using quantum computing resistant cryptography and estimate the applicability of such new technology to the DFS use-case.

# How does USSD work

Unstructured Supplementary Service Data (USSD) is a capability built into GSM, much like the Short Message Service (SMS). USSD differs from SMS since SMS uses a “store and forward” technique to deliver text messages while USSD information is sent directly from a sender’s mobile handset to an application platform handling the USSD service. The USSD service can be located either in the sender’s mobile network or in another connected network.

Another key difference from SMS is that USSD initiates a real-time “session” between the UE and the USSD application platform when the service is invoked, allowing data to be sent back and forth between the mobile user and the USSD application platform until the USSD service is completed. A USSD session can be invoked by either the UE or the USSD platform [2].

## Network Architecture for USSD Signalling

According to [2], the network architecture of USSD services is described by Exhibit 1 below:

Figure 1 – USSD network architecture

The USSD session can be initiated by any one of the NEs depicted in Exhibit 1, however the USSD initiation process is categorized either as MO-USSD, if the session is originated by the MS, or MT-USSD, if the session is initiated by any of the core NEs.

## Signalling flow example of a MO-USSD session

In the following exhibit below, an example of a “balance query and top up” USSD session signalling flow is described. For more signalling flows of MO-USSD and MT-USSD please refer to [2].

Figure 2 – Example of MO-USSD signalling flow

The IEs and data in the USSD session is exchanged in clear text, as can be seen in the packet capture exhibit below, this can be seen by inspecting the `USSD string` field in the packet:

Figure 3 – Example of USSD signalling packet

# Examples of exploiting USSD vulnerabilities on to commit DFS fraud

## Account takeover

In this example, a fraudster uses USSD to takeover an account that does not belong to him. To perform this attack, the fraudster first needs to spoof his victim’s phone number and dial the USSD code (this can be done by over the air interception). Once the fraudster initiates the USSD session with the DFS provider spoofing the victim’s phone number they can change the PIN code and add another phone number to the account. Once done, the fraudster performs another USSD session, this time with the new phone number they added and uses the new PIN to login to the account and transfer the money out.

## Social engineering of sensitive credentials using USSD

Unstructured Supplementary Service Data (USSD) is used for, online banking and other financially sensitive applications. Due to the high level of assumed trust by the users (when receiving USSD messages), the simplest attack to execute and scale an attack is using USSD to send a fraudulent message to the user spoofing the identity of the financial service provider, luring the user to divulge sensitive information such as account number and PIN code. For example, to phish these credentials, the attacker sends a phishing USSD message: Such as in Exhibit 2 below.

Figure 2 – Using USSD to socially engineer the user

Since there is no identification in the USSD message, and the user is used to having these messages from the network, trust is achieved, and the user divulges their account number and PIN. From there on, the attacker logs into the account and transfers the funds out.

# Quantum Safe Cryptography (QSC)

Quantum-safe cryptography refers to cryptographic algorithms that are thought to be secure against an attack by a quantum computer. Since today’s popular encryption algorithms, can be efficiently broken by a sufficiently strong quantum computer. The problem with currently popular algorithms is that their security relies on one of three hard mathematical problems: the integer factorization problem, the discrete logarithm problem, or the elliptic-curve discrete logarithm problem. All these problems can be easily solved by a sufficiently powerful quantum computer running Shor's algorithm.[5]. Even though current, publicly known, experimental quantum computers lack processing power to break any real cryptographic algorithm, many cryptographers are designing new algorithms to prepare for a time when quantum computing becomes a threat.

## Approaches to quantum safe cryptography

There are two categorical approaches researchers take when developing quantum resistant cryptography.

The first is developing new algorithms and trap door functions that have inherent resiliency to the computation advantages of quantum computers for asymmetric ciphers.

The second is to double the key-space of current symmetric algorithms, since Grover’s algorithm [6,7] proved that quantum computers reduce the primage resistance of popular hash functions using n bits input to 2n2 (and it has the same impact on the key search [8]), thus doubling the key size can effectively enable block and other symmetric ciphers to retain their current security level, provided other security parameters are not affected and the construction adapts to key size increment

In this chapter we will survey both approaches and try to compare the different solutions via a common criterion which is the applicability to USSD encryption.

## New algorithms for post-quantum cryptography

There are 5 families of new algorithms for post-quantum cryptography:

- Lattice-based cryptography: constructions of cryptographic primitives that involve lattices, either in the construction itself or in the security proof. Many lattice-based constructions are considered to be secure under the assumption that certain well-studied computational lattice problems cannot be solved efficiently by both classical and quantum computers.

- Multivariate cryptography: construction of asymmetric cryptographic primitives based on multivariate polynomials over a finite field F. In certain cases, those polynomials could be defined over both a ground and an extension field, in which case the polynomials have the degree of two. It is commonly admitted that Multivariate cryptography turned out to be more successful as an approach to build signature schemes primarily because multivariate schemes provide the shortest signature among post-quantum algorithms, however this family of algorithms produce large sized keys which may negate the advantage of the small signatures.

- Hash-based cryptography: constructions of cryptographic primitives based on the security of hash functions. So far, hash-based cryptography is limited to digital signatures schemes such as the Merkle signature scheme. Hash-based signature schemes combine a one-time signature scheme with a Merkle tree structure. Since a one-time signature scheme key can only sign a single message securely, it is practical to combine many such keys within a single, larger structure. A Merkle tree structure is used to this end. In this hierarchical data structure, a hash function and concatenation are used repeatedly to compute tree nodes. In 10/2020, the US National Institute of Standards and Technology (NIST) published a recommendation for stateful hash-based signature schemes [b-NIST SP 800-208] based on the eXtended Merkle Signature Scheme (XMSS) and Leighton-Micali Signatures (LMS).

- Code-based cryptography: cryptographic systems which rely on error-correcting codes, such as the McEliece and Niederreiter encryption algorithms and the related Courtois, Finiasz and Sendrier Signature scheme. The Post Quantum Cryptography Study Group sponsored by the European Commission has recommended the McEliece public key encryption system as a candidate for long term protection against attacks by quantum computers. However no candidates of this family made it past the first round of the NIST PQC program.

- Super-singular elliptic curve isogeny cryptography: cryptographic system that  relies on the properties of super-singular elliptic curves and super-singular isogeny graphs to create a Diffie-Hellman replacement with forward secrecy. This cryptographic system uses the well-studied mathematics of super-singular elliptic curves to create a Diffie-Hellman like key exchange that can serve as a straightforward quantum computing resistant replacement for the Diffie-Hellman and elliptic curve Diffie–Hellman key exchange methods.

The key issue with the new algorithms is that the primary research is not yet complete and until today no applicable implementation of these algorithms exists, which provides full coverage for application security, i.e. encryption, digital signature, and trap-door function. Several national standardization bodies conduct independent programs for post-quantum cryptography, for example NIST started such a program in 2016, the program is at it’s 3rd stage, and currently where are only 3-5 candidates left in each category (key exchange, digital signature and encryption / key establishment) draft standards are expected to emerge in 2022, more information can be found in [b-NIST-PQC] . Another program is the EU commission’s PQCRYPTO [9] program that ran from 2015 to 2018, which wasn’t able to reach any kind of standardization of new algorithms and states in its final report “It is very clear, that the road to standardization of post-quantum cryptography is a long one. Therefore, project partners interested in standardization of post-quantum cryptography need to continue their efforts beyond the formal termination of PQCRYPTO”

Some of the current active implementation project for post-quantum cryptography are “liboqs” [10] and CEX [11].

## Doubled key-space algorithms

Unlike the new algorithms described in the previous chapter, quantum-resilience for symmetric encryption can be achieved by extending the key length of traditional encryption, signing, and hashing algorithms. The following table contains the available symmetric encryption minimum recommendation for public-key security for the post-quantum era:

| Mechanism | Algorithm | Key length / block size / curve | Recommended by |
| --- | --- | --- | --- |
| Hash function | SHA256 | 256 bit or higher | NIST IR 8105, ITU-T X.1197 Amd1 |
| Confidentiality | AES | 256 bit or higher | NIST IR 8105, ITU-T X.1197 Amd1 |

## PKI algorithms

Most PKI cyphers are considered as quantum-broken, thus they need to be replaced by new algorithms. The following table contains the available PKI minimum recommendation for the post-quantum era:

| Mechanism | Algorithm | Key length / block size / curve | Recommended by |
| --- | --- | --- | --- |
| Hash function | SHA256 | 256 bit or higher | NIST IR 8105, ITU-T X.1197 Amd1 |
| Confidentiality | AES | 256 bit or higher | NIST IR 8105, ITU-T X.1197 Amd1 |
| Digital signatures | Crystals (Dilithium), Falcon, Rainbow | 512 bit or higher |  |
| Key exchange | Classic McEliece, CRYSTALS-KYBER, NTRU & SABER | 768-bit prime | NIST |

## Available post-quantum software packages

Editor’s note: TBC: compare resources required (cpu, gpu and ram)

Editor’s note: Contributions are welcomed to improve clause 8.5.

### CEX

AES-based AHX and RHX in CEX [11] provide a host of symmetric and asymmetric quantum-safe algorithms, while removing the caveats of the original AES due to its differentially weak key schedule. It is backwards-compatible with AES CPU optimizations/co-processors, therefore not requiring different hardware on the server side.

This library is being built in two stages; the symmetric cryptography, which consists of ciphers, hash functions, MACs, RNGs, TRNGs etc, preliminary work has been completed as of version v1.0. That work is still evolving however, as improvements and additions to the symmetric cryptography will continue throughout the library’s lifetime. The second half is the addition of asymmetric cryptography, with a strong focus on post-quantum security. This work is well under way, and this release contains the NTRU (NTRU Prime), RingLWE (New Hope), ModuleLWE (Kyber) and McEliece (Niederreiter) asymmetric ciphers, as well as the Dilithium, XMSS, Rainbow, and SPHINCS+ signature schemes

### liboqs

Liboqs [10] provides many NIST PQC candidates as options, more then CEX, but contains no symmetric ciphers that are designed to be quantum-resistant, in addition liboqs is not production ready, and it’s contributors specify that their library is meant to help with research and prototyping and recommended not to be used in production environments.

### libpqcrypto

libpqcrypto [12] is a new cryptographic software library produced by the PQCRYPTO project, that includes software for 77 cryptographic systems (50 signature systems and 27 encryption systems) from 19 of the 22 PQCRYPTO submissions

# The uSIM as a computation platform for post-quantum crypto

## SIM Card – background

- ISO/IEC 7816 Smart card (Universal Integrated Circuit Card - UICC)

- Same as the UICC banking cards, digital id / passport, or any other card with a “chip” - further reading: [b-Smart-Card]

- The SIM (Subscriber Identity Module) is an application of a UICC

- The UICC consists of a CPU, RAM, E2PROM and I/O circuits

- Modern UICC cards also have wireless NFC interface

- UICCs run Java Card Runtime Environment (JCRE) operating system with applications (named “applets”) running as JCVMs on the JCRE - further reading [b-Java-Card]

## USIM software and hardware description

| Application | 2G - SIM | 3G - SIM+uSIM | 4G - SIM+uSIM+iSIM |
| --- | --- | --- | --- |
| Smart Card type | ICC | UICC | UICC / eUICC |
| CPU | 8bit | 16bit | 32bit |
| Storage (E2PROM) | Up to 32 Kbyte | Up to 128 KByte | Up to 256 Kbyte |
| Interface | Electrical | Electrical | Electrical / NFC |
| # of identities | 1 | 2 | multiple |
| Burning | Physical | Physical + OTA | Physical + OTA |
| Cryptography | A5/1, A5/2 | A5/1, A5/2, A5/3, Kasumi, Milenage | A5/1, A5/2, A5/3, Kasumi, Milenage, AES128, PKI |

In order to support QSC, the (U)SIM shall provide storage for a 256-bit root key, as an enabler for 256-bit, lightweight post-quantum symmetric cryptography algorithms. The 3G/4G cards are able to support such a key.

## USIM file system and applet structure

### ICC Card file system example

### ICC card - telecom data file

### ICC card GSM data file

### SIM card UICC example

## USIM resources applicability to post-quantum cryptography algorithms

The USIM engine is the JCRE which runs JCVMs (Applets), the host (the UE) communicates with each applet using a simple API based on files. The Applets are written in Java and the API messages between the host and the applet are called APDU (Application Control Data Unit). The following figure details the interface of the host to the applet:

In order to support QSC, a (U)SIM applet needs to be written to the card, hardware acceleration for at least AES, including AES-256, and preferably also for HKDF expand using SHA256 will contribute greatly to the performance of the card but it is not mandatory as the UEs themselves are powerful enough to support it.

# Applicability matrix between USIM platform and post-quantum crypto

Editor’s note: TBC

To provide the broadest compatibility, new (U)SIMs should be deployed for legacy devices as mentioned in the above sections 9.1 and 9.2. Those will enable post-quantum secure mobile payment on such devices. The application, within the scope of this TR, is limited to this usage.

# Bibliography

[b-NIST-PQC]	https://csrc.nist.gov/projects/post-quantum-cryptography/round-3-submissions

[b-Smart-Card]	https://www.wikiwand.com/en/Smart_card

[b-Java-Card]	https://www.javacardos.com/wiki/start

[b-NIST SP 800-208] https://nvlpubs.nist.gov/nistpubs/SpecialPublications/NIST.SP.800-208.pdf

______________