|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2017-2020 |  |  | SG12-C.11 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | STUDY GROUP 12 |  |
|  |  |  |  | Original: English |  |
| Question(s): |  | 12, 15/12 | Geneva, 10-19 January 2016 |  |  |
| CONTRIBUTION |  |  |  |  |  |
| Source: |  | Gambia |  |  |  |
| Title: |  | Voice and data service key performance indicators for mobile services |  |  |  |
| Purpose: |  | Proposal |  |  |  |
| Contact: |  | Nicholas Jatta Public Utilities Regulatory Authority The Gambia |  |  | Tel: +220 4399601 Fax: +220 4399905 E-mail: nickjatta@hotmail.com |

| Keywords: | E.807, KPI |
| --- | --- |
| Abstract: | During the SG12 meeting held in June 2016, a session was held for newcomers. The meeting was informed that a questionnaire was sent following the Dakar SG12 African Group meeting requesting information on KPIs from member countries. However, the requested information was not forthcoming. It was also noted that KPI information on Recommendation ITU-T E.807 was insufficient and needs contributions. It is in this regard that this contribution is being made to provide a Guide on KPIs threshold that could be used as benchmark for developing countries. |

INTRODUCTION

Quality of service (QoS) is the collective effect of service performance which determines the degree of satisfaction of a user of the service indicating the performance of a network and of the degree to which the network conforms to the stipulated norms

In telecommunications, ‘quality of service’ relates to the ability to provide different resources for different applications and to accord different priorities to each. Consequently, the level of service offered must be stated in any contract or service Legal Agreement that is offered by a service provider to a consumer.

There are certain technical indicators, which serve as benchmarks by which the ‘quality of service delivery may be judged.  Such benchmarks are, of necessity, of a technical nature, but they are crucial in assessing the profile of the service delivered.  They are also instrumental in a certain period of time.  The consumer, should be provided with all the necessary information regarding these technical indicators so that, should the need arise, they will have the means with which to compare actual performance with guaranteed performance.

Regulatory Authorities particularly in developing countries make reference the ITU for international standards on ICT issues services. For mobile voice and data service the E 807 is a reference document that provided key performance indications that can be referenced to.  This recommendation needs revision considering that most of the relevant KPIs that the industry uses are either absent or have thresholds that do not conform to international standards. An example is the threshold for call drop rate. The industrial standard is ≤   2, whilst E 807 gives a value of ≤   3.

It is in this regard that this contribution to be used as a base line for the development of a recommendation on KPIs for the industry

TECHNICAL SPECIFICATIONS FOR THE VARIOUS TECHNOLOGIES

The purpose of this document is to describe the 2G, 3G and LTE KPI performance monitoring

| Generation | Access Technology | Modulation | Frequency bands | Latency | Throughput |
| --- | --- | --- | --- | --- | --- |
| GSM (2G) | TDMA/FDMA | GMSK (2 digits per symbol) | 900-1800 MHZ | 600 ms | 9,6 kps |
| GPRS (2.5G) | TDMA/FDMA | GMSK (2 digits per symbol) | 900-1800 MHZ | 500ms | 40 kps |
| EDGE (2.75G) | TDMA/FDMA | 8-PSK (3 digits per symbol) | 900-1800 MHZ | 400 ms | 384kps/s |
| UMTS (3G) | W-CDMA | QPSK (4 digits per symbol) | 1900 -2100MHz | 150 ms | 384 kbps à 2 Mps |
| HSPA (3.5G) | W-CDMA | (QPSK or 16QAM | 1900 -2100 MHz | 100 ms | 14,4 Mps en DL 5,8 Mps en UL |
| HSPA+_DC (3.75G) | W-CDMA | 64QAM (6 digits per symbol) | 1900 -2100 MHz | 50 ms | 42 Mps en DL 11 Mps en UL |
| LTE (4G) | OFDM (DL) SC-FDMA (UL) | 64QAM (6 digits per symbol) | 800-2600 MHZ | 10 ms | 50 Mps en UL 150 Mps en DL |
| LTE-Advanced (4G+) | OFDM (DL) SC-FDMA (UL) | 256QAM (8 digits per symbol) | 800-2600 MHZ |  |  |
| G |  |  |  |  |  |

KPI definitions and thresholds

The following analysis contains a list of the most common KPIs used in cellular networks. These KPIs should be monitored constantly. When the value of a KPI goes below the defined threshold, then detailed analysis should be performed in order to identify the reasons of this deterioration

KPI measurement a critical aspect of network performance, this value is obtained from one or more raw measurements taken from specific counters allow and facilitate the assessment of a position by the decision maker

SDCCH Availability

SDCCH Availability indicates the ratio of the number of available SDCCHs to the number of configured SDCCHs in a measurement period.

In case of SDCCH dynamic adjustment, the number of configured SDCCHs refers to the number of dynamically configured SDCCHs, that is, the actual number of SDCCHs after dynamic adjustment

SDCCH Availability = Mean Number of Available Channels (SDCCH)/Mean Number of Dynamically Configured Channels (SDCCH) x 100

SDCCH Congestion Rate

The congestion rate on SDCCH can be obtained through the traffic measurement results. It reflects the SDCCH collisions due to insufficient resources

SDCCH Channel Request

SDCCH Failed

Cause: No available

Ressource

SDCCH Blocking Rate

SDCCH Blocking means that you are not getting SDCCH resource for the call origination.

When MS connects with Network then RACH and AGCH are provided, but SDCCH is not performed at this time due to some problem.

SDCCH Channel Request

SDCCH Failed

Cause: other

reason

SDCCH Drop Rate

SDCCH Drop Rate indicates the probability of call drops that occurs when MSs occupy SDCCHs.

SDCCH Channel Request

SDCCH Success

SDCCH Failed

SDCCH_Drop_Rate = number of call drops on SDCCH/ SDCCH_Successfull_Seizures * 100

TCH Availability

TCH Availability indicates the ratio of the number of available TCHs to the number of configured TCHs in a measurement period.

In the case of dynamic conversion between TCH and PDCH, the number of configured TCHs refers to the number of dynamically configured TCHs, that is, the actual number of TCHs after dynamic conversion

TCH Availability = Mean Number of Available Channels (TCH)/Mean Number of Dynamically Configured Channels (TCH) x 100

TCH Blocking Rate

TCH Blocking means that you are not getting TCH resource for the call origination.

When MS connects with Network then RACH and AGCH are provided, by TCH is not provided at this this time due to some problem.

TCH Channel Request

TCH Failed

SDCCH Success

SDCCH Channel Request

Cause: other

reason

TCH Drop Rate

This counter is measured when a TCH is exceptionally released due to any cause during a call

TCH Drop Rate indicates the ratio of the number of call drops to the number of successful TCH seizures after the BSC successfully assigns TCHs to MSs.

TCH Channel Request

TCH Success

TCH Failed

TCH_Drop_Rate = number of call drops on TCH/ TCH_Successfull_Seizures * 100

TCH Congestion Rate

Congestion Ratio on TCH involves two KPIs: TCH Congestion Rate (All Channels Busy) and TCH Congestion Rate (Overflow).

TCH Congestion Rate (All Channels Busy) indicates the ratio of the number of failed TCH seizures due to busy TCHs to the number of TCH seizure requests.

TCH Congestion Rate (Overflow) indicates the ratio of the number of failed assignments after directed retry, queuing to the number of TCH seizure requests TCH Congestion Rate (All Channels Busy):

TCH Channel Request

TCH Failed

SDCCH Success

SDCCH Channel Request

Cause: No available

Ressource

TCH Congestion Rate (All Channels Busy) = Failed TCH Seizures due to Busy TCH/TCH Seizure Requests *100

Call Setup Success Rate

This KPI reflects the probability of successful calls initiated by the MS.

The CSSR is a key counter in evaluating the network performance

SDCCH Channel Request

SDCCH Channel Request

Channel Activation

Channel  Activation ACK

Assign  Command

Establish  Indication

TCH Channel Request

TCH Channel Request

CSSR = (1- SDCCH Congestion)*(1-SDCCH Drop)*(1- TCH Congestion)*(1-TCH Assignment Failure)

Call Success Rate

Call Complete Establishment Success Rate is abbreviated as CSR, which is the ratio of the number of established calls to the total number of initiated calls.

CSR (including handover) = CSSR x (1 – Call Drop Ratio) CSR (including handover) = CSSR x (1 – Call Drop Ratio)

Call Drop Rate

Call Drop Rate is assuming the number of incoming and outgoing handovers

CDR (cell_level) = (TN_DROP/N_CALLS) x 100

Where:

CDR (cell_level) - Call drop rate for that cell
TN_DROPS - Total number of dropped connections
N_CALLS - Number of calls terminated in a cell

Now to explain further N_CALLS:

I_CALLS - Number of initiated calls in a cell
Inc - Sum of all incoming handovers to a cell
Outg - Sum of all outgoing handover from a cell

N_CALLS= I_CALLS+ IncHO-OutgHO

HO Success Rate

The purpose of handover is to ensure the call continuity, improve the speech quality, and reduce the cross interference in the network, thus providing better services for the subscribers.

Success ratio of handover is the ratio of the total number of successful handovers to the total number of handover requests.

This counter directly affects the user experience and is one of the most significant KPIs for the telecom operator.

Inter-Cell HO Success Rate

Measurement Report

Measurement Result

SDCCH Channel Activate

SDCCH Channel Activate ACK

Handover Command

Handover FACCH new Access

HO Performed

TCH Call Continue

Inter-Cell Handover Success Rate= (Successful Incoming Inter-Cell Handovers + Successful Outgoing Inter-Cell Handovers) / (Incoming Inter-Cell Handover_Attempts+Outgoing Inter-Cell Handover_Attempts) * 100

Mobile Incoming Call

Paging PCH

Paging Command PCH

Paging Request PCH

Channel Request RACH

Channel Required

Channel Activation

Channel Activation ACK

Immediate Assignment Command

Immediate Assignment

Paging Response

AGCH

SDCCH

Paging Response

Paging Response

Connection Accepted

Mobile Originating Call

1. SDCCHChannel Request

1. SDCCH Channel Request

2. Sends subscriber ID (TMSI)

3. Authentication+IMEI check+new TMSI

4. Request call Information

5. Sends INFO

6. TCH Channel Assignment

7. Traffic Channel

Assignment

8. Setup Connection to    B Subscriber

1. SDCCH Assignment

1. SDCCH Assignment

Paging PCH

Paging Command PCH

Paging Request PCH

Channel Request RACH

Channel Required

Channel Activation

Channel Activation ACK

Immediate Assignment Command

Immediate Assignment

Paging Response

AGCH

SDCCH

Paging Response

Paging Response

Connection Accepted

3G KPIs

RRC Connection Success Rate

This KPI describes the ratio of all successful RRC establishments to RRC establishment attempts for UTRAN network, and is used to evaluate UTRAN and RNC or cell admission capacity for UE and/or system load.

RRC Connection Request

RRC Connection Setup

RRC Connection Setup Complete

RRC_Setup_Success_Rate (All) = RRC_ConnectionSuccess (All) / RRC_ConnectionAttempt (All) *100

RAB Establishment Success Rate

This KPI describes the ratio of all successful RAB establishments to RAB establishment attempts for UTRAN network and is used to evaluate service accessibility across UTRAN.

This KPI is obtained by the number of all successful RAB establishments divided by the total number of attempted RAB establishments.

CS RAB Success Rate

RRC Connection Request

RANAP Request

RANAP Response

RRC Connection Response

Radio Bearer Setup

Radio Bearer Setup Complete

CS_RAB_Setup_Success_Rate (All) = CS_RAB_SetupSuccess (All)/ CS_RAB_ SetupAttempt (All) *100

PS RAB Success Rate

RRC Connection Request

RANAP Request

RANAP Response

RRC Connection Response

Radio Bearer Setup

Radio Bearer Setup Complete

PS_RAB_Setup_Success_Rate (All) = PS_RAB_SetupSuccess (All) / PS_RAB_ SetupAttempt (All) *100

Call Setup Success Rate

This KPI is typically calculated by considering just 2 steps in Call Establishment: RRC Connection Setup and RAB Establishment.

They are assumed to be independent processes, and therefore CSSR was hence calculated as product of the Success Rates of each of these 2 phases.

CS Call Setup Success Rate

RRC Connection Request

RANAP Request

RANAP Response

RRC Connection Response

Radio Bearer Setup

Radio Bearer Setup Complete

CS_Call_Setup_Success_Rate (All) = CS_RAB_SetupSuccess (All) * CS_RRC_ SetupSuccess (All)

PS Call Setup Success Rate

RRC Connection Request

RANAP Request

RANAP Response

RRC Connection Response

Radio Bearer Setup

Radio Bearer Setup Complete

PS_Call_Setup_Success_Rate (All) = PS_RAB_SetupSuccess (All) * PS_RRC_ SetupSuccess (All)

Soft HO Success Rate

This KPI describes the ratio of number of successful radio link additions to the total number of radio link addition attempts.

This KPI is obtained by the number of successful radio link additions divided by the total number of radio link addition attempts.

RRC  Measurement Report

Radio Link Add Setup Request

Radio Link Add Setup Request

Radio Link Add Setup Response

Radio Link Add Setup Response

Active Set Update

Active Set Update Complete

Soft_Handover_Success_Rate= [(IntraCell_SHOSuccess+ InterCell_SHOSuccess) / (IntraCell_SHOAttempt+ InterCell_SHOAttempt)]* 100

Inter-RAT Ho Success Rate

This KPI describes the ratio of number of successful inter RAT handover to the total number of the attempted inter RAT handover from 3G to 2G for CS domain and PS domain.

This KPI is obtained by the number of successful inter RAT handover divided by the total number of the attempted inter RAT handover from 3G to 2G for CS domain (PS domain) respectively.

CS32 inter RAT HO Success Rate

Measurement Report

Relocation Required

Handover Request

Handover Request ACK

Relocation Command

Handover Command

Handover Access

Handover Complete

lu Release Command

lu Release Complete

IRATHO_CS_SR=IRATHO_CS_Success / IRATHO_CS_Attempt *100

PS32 inter RAT HO Success Rate

Measurement Report

Relocation Required

Handover Request

Handover Request ACK

Relocation Command

Handover Command

Handover Access

Handover Complete

lu Release Command

lu Release Complete

IRATHO_PS_SR=IRATHO_PS_Success / IRATHO_PS_Attempt *100

4G KPIs

RRC Connection Success Rate

This KPI describes the ratio of all successful RRC establishments to RRC establishment attempts for EUTRAN network.

RRC ConnectionSetup

RRC Connection Request

RRC Connection Setup Complete

RRC_Setup Success Rate (All)  = RRC_EstablishmentSuccess (All)  / RRC_EstablishmentAttempt (All)  * 100

ERAB Setup Success Rate (All)

This KPI can be used to evaluate the ERAB setup success rate of all services including the VoIP service in a cell or a cluster.

RRC Connection Reconfiguration

RRC Connection Reconfiguration

Complete

ERAB Setup Success Request

ERAB Setup Response

ERAB Setup Success Rate (All)= ERAB_Setup_Sucess (All) / ERAB_Setup_Attempt (All)* 100

Call Setup Success Rate

The KPI is calculated based on multiplying the KPI of RRC Setup Success Rate and the KPI of ERAB Setup Success Rate (All)

RRC Connection Setup Complete

ERAB Setup Request

RRC ConnectionSetup

RRC Connection Request

ERAB Setup Response

Call Setup Success Rate = ERAB_Setup_Sucess (All) *RRC_Setup Success (All)

Call Drop Rate

This KPI can be used to evaluate the call drop rate of all services in a cell or a cluster, including VoIP service. This KPI measures abnormal release at the eNodeB.

ERAB Release Indication

Call Drop Rate = ERAB_Abnormal Release (All) / ERAB_Release (All) *100

Handover Success Rate

This KPI can be used to evaluate the handover in success rate in a cell or a cluster. The HO includes both inter-eNodeB and intra-eNodeB scenarios.

Inter eNodeB HO Success Rate

Serving

Target

Measurement Report

Handover Required

Handover Request

Handover Request ACK

Handover Command

RRC Connection Reconfiguration

RRC Connection Reconfiguration Complete

Handover Notify

UE Context Release Command

Inter eNodeB HO SR=Internal HO Success/ Internal HO Attempt *100

Inter-RAT HO Success Rate

This KPI can be used to evaluate the inter-RAT handover out success rate in a cell or a cluster.

This KPI measures the performance of the inter-RAT handover from LTE to GSM, from LTE to WCDMA.

Inter-RAT HO LTE-WCDMA

Measurement Report

Handover Required

Relocation Required

Handover Request

Handover Request ACK

Relocation Command

Handover Command

Inter-RAT Handover

IRATHO_L2W_SR=IRATHO_L2W_Success/ IRATHO_L2W_Attempt *100

Inter-RAT HO LTE-GSM

Measurement Report

Handover Required

Relocation Required

Handover Request

Handover Request ACK

Relocation Command

Handover Command

Inter-RAT Handover

IRATHO_L2Geran_SR=IRATHO_L2Geran_Success / IRATHO_L2Geran_Attempt *100

Conclusion: KPI’s Standard Threshold

The evaluation of standards and the setting of targets for optimum performance are invaluable in ‘quality of service’ terms-to the consumer, the provider Regulatory Authorities alike.  The generation of such information.

- Enables the consumer to make informed choices

- Enables the provider to ensure optimum continuity of service

- Enables the regulatory authorities to define optimum quality levels in relation to such considerations as ensuring the interconnection and inter-operability of all networks and services.

- Assist the regulator in accurately assessing the state of the marketplace

- Facilitates such considerations as billing, service response time and call set up time

- Facilitates the setting of such service parameters as will ensure the overall quality of service to the consumer.

- Ensures the operation of fair competition.

- Evaluates the various levels of service to which the service providers commit themselves.

- Maintains or improves delivery of ‘quality of service’ both in the presence and the absence of competition.

The following tabulated thresholds are being proposed:

| Generation | KPI | Standard Threshold |
| --- | --- | --- |
| 2G | SDCCH Blocking SDCCH Drop TCH Blocking TCH Drop TCH Assignment HO Success Rate CSSR CDR | ≤0.5 ≤1.2 ≤0.5 ≤1.5 ≥98.5 ≥95 ≥98 ≤2 |
| 3G | RRC Drop RAB Drop RRC Availability SHO Success Rate CSSR CDR | ≤1.2 ≤1.5 ≥98.5 ≥95 ≥98 ≤2 |
| 4G | RRC Drop ERAB Drop RRC Availability HO Success Rate CSSR CDR | ≤1.2 ≤1.5 ≥98.5 ≥95 ≥98 ≤2 |

_______________________