|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2022-2024 |  | SG2-TD565/PLEN |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | STUDY GROUP 2 |  |  |
|  |  |  | Original: English |  |  |
| Question(s): |  | 6/2 |  | Geneva, 19-28 June 2024 |  |
| TD |  |  |  |  |  |
| Source: |  | Editor |  |  |  |
| Title: |  | Revised baseline text for M.fmcdns: "Framework for management of cross-domain network slices in IMT-2020 network and beyond" |  |  |  |
| Contact: |  | Xiaoming He China Telecom China |  |  | Tel: +86-13316097161 E-mail: hexm4@chinatelecom.cn |
| Contact: |  | Yanchuan Wang China Telecom China |  |  | Tel: +86 10 5850 1695 E-mail: wangych@chinatelecom.cn |

| Abstract: | This contribution proposes to modify Clause 8 of ITU-T M.fmcdns. |
| --- | --- |

# Summary

During the meeting of Q6/2 on 24 June 2024, C193 on “Framework for management of cross-domain network slices in IMT-2020 network and beyond” has been discussed. The main comments are as follows:

- In clause 8.1, “Network slice capability openness module” is suggested to be modified to “Network slice capability exposure module”.

- “E2E network slice topology management module” in Figure 8-2 is inconsistent with “E2E network slice resources management module” described in clause 8.2.

- Suggest updating the functional description of the cross-domain network slices QoS mapping management module.

- Suggest adding ITU-T Y.3160 and 3GPP TS 28.530 to References.

Based on the above comments, this TD is provided by the contributor and Rapporteur of Q/2 together for further discussion. The main modifications of the draft M.fmcdns are as follows:

- “Network slice capability openness module” is changed to “Network slice capability exposure module”, including updates to Figure 8-1.

- Update Figure 8-2 and make it keep consistent with “E2E network slice resources management module” described in clause 8.2.

- Update the functional description of the cross-domain network slices QoS mapping management module, including modification of table 8-1.

- Add ITU-T Y.3160 and 3GPP TS 28.530 to References.

Table of Contents

1.	Scope	3

2.	References	3

3.	Definitions	3

3.1.	Terms defined elsewhere	3

3.2.	Terms defined in this Recommendation	4

4.	Abbreviations and acronyms	4

5.	Conventions	5

6.	Overview	5

7.	Management framework for cross-domain network slices	6

8.	Management function for cross-domain network slices	7

8.1.	CSMF functionality	7

8.2.	NSMF functionality	9

8.3.	NSSMF functionality	14

9.	Procedures for cross-domain network slices interworking	14

Bibliography	15

Draft Recommendation ITU-T M.fmcdns

Framework for management of cross-domain network slices in IMT-2020 network and beyond

# Scope

This draft Recommendation specifies framework for management of cross-domain network slices in IMT-2020 network and beyond to implement efficiently management and coordination of cross-domain network slices, including cross-domain network slices interworking.

The scope of this Recommendation includes the following aspects:

- Management framework for cross-domain network slices.

- Management function for cross-domain network slices.

- Procedures for cross-domain network slices interworking.

# References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T M.3010]	ITU-T Recommendation M.3010 (2000), Principles for a telecommunications management network.

[ITU-T M.3400]	ITU-T Recommendation M.3400 (2000), TMN management functions.

[ITU-T Y.3100]		ITU-T Recommendation Y.3100 (2017), Terms and definitions for IMT-2020.

[ITU-T Y.3110]		ITU-T Recommendation Y.3110 (2017) , IMT-2020  network management and orchestration  requirements.

[ITU-T Y.3111]		ITU-T Recommendation Y.3111 (2017), IMT-2020  network management and  orchestration framework.

[ITU-T Y.3112]		ITU-T Recommendation Y.3112 (2019),  Framework for the support of network slicing in the IMT-2020 network.

[ITU-T Y.3160]		ITU-T Recommendation Y.3160 (2023),  Architectural framework of end-to-end service level objective guarantee for future networks including IMT-2020.

[3GPP TS 28.530]	3GPP TS 28.530 (2024), Management and orchestration; Concepts, use cases and requirements.

# Definitions

# Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	network slice [ITU-T Y.3100]: A logical network that provides specific network capabilities and network characteristics.

NOTE 1 – Network slices enable the creation of customized networks to provide flexible solutions for different market scenarios which have diverse requirements, with respect to functionalities, performance and resource allocation.

NOTE 2 – A network slice may have the ability to expose its capabilities.

NOTE 3 – The behaviour of a network slice is realized via network slice instance(s)

3.1.2	network slice instance [ITU-T Y.3100]: An instance of network slice, which is created based on network slice blueprint.

NOTE 1 – A network slice instance is composed of a set of managed run-time network functions, and physical/logical/virtual resources to run these network functions, forming a complete instantiated logical network to meet certain network characteristics required by the service instance(s).

NOTE 2 – A network slice instance may also be shared across multiple service instances provided by the network operator. A network slice instance may be composed of none, one or more sub-network slice instances which may be shared with another network slice instance.

3.1.3 network slice subnet [[b-3GPP-TS 28.530]: A representation of the management aspects of a set of managed functions and the required resources (e.g., compute, storage and networking resources) supporting network slice.

3.1.4 network slice subnet instance [b-3GPP-TS 28.530]: An instance of network slice subnet representing the management aspects of a set of managed function instances and the used resources (e.g., compute, storage and networking resources).

3.1.5 Single-Network Slice Selection Assistance Information (S-NSSAI) [b-3GPP-TS 23.501]: S-NSSAI,  which is made up of slice/service type (SST) and slice differentiator (SD), is used to uniquely identify a network slice.

# Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1 cross-domain network slice: an end-to-end network slice composed of multiple network slice subnets that belong to different administrative domains.

3.2.2 cross-domain network slices management: managing the lifecycle of cross-domain network slices by a set of network slicing management components in collaborative manner.

3.2.2 cross-domain network slices interworking: the capability of fulfilling seamless connection of cross-domain network slice subnet instances that belong to the same network slice instance.

3.2.3 interworking identifier: a kind of network slice subnet instance mapping identification used by packet encapsulation to realize cross-domain network slices interworking.

# Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

5QI		5G Qulity Identity

CN 		Core Network

CSMF		Communication Service Management Function

DN		Data Network

EMS		Element Management System

eMBB		Enhanced Mobile Broadband

IP		Internet Protocol

mIoT		massive Internet of Things

NMS		Network Management System

NSMF		Network Slice Management Function

NSSMF	Network Slice Subnet Management Function

NSI		Network Slice Instance

NSSI		Network Slice Subnet Instance

QoS 		Quality of Service

AN 		Access Network

S-NSSAI	Single-Network Slice Selection Assistance Information

TN 		Transport Network

uRLLC	Ultra-reliable and Low Latency Communication

# Conventions

In this Recommendation:

The keywords "is required to" indicate a requirement which must be strictly followed and from which no deviation is permitted, if conformance to this Recommendation is to be claimed.

The keywords "is recommended" indicate a requirement which is recommended but which is not absolutely required. Thus, this requirement need not be present to claim conformance.

The keywords "can optionally" indicate an optional requirement which is permissible, without implying any sense of being recommended. This term is not intended to imply that the vendor's implementation must provide the option, and the feature can be optionally enabled by the network operator/service provider. Rather, it means the vendor may optionally provide the feature and still claim conformance with this Recommendation.

# Overview

With the large-scale deployment of IMT-2020 networks, many emerging services such as eMBB,  uRLLC and mIoT have put forward more differentiated demands for the quality of service (QoS). Network slicing enables the creation of customized networks to provide flexible solutions for different scenarios which have diverse requirements. Network slicing has become a key feature and enabler for IMT-2020 networks. It is known that IMT-2020 networks are composed of Access Network (AN), Transport Network (TN) and Core Network (CN). Currently, each subnet is independently managed by its own NMS/EMS, lacking cooperation and coordination with each other.  End-to-end (E2E) network slicing management, including the whole life-cycle management of network slice from creation, activation, modification, deletion, deactivation to termination, is a quite complicated process. How to efficiently provision E2E network slicing for customers is telecom operators’ pain spot and problem faced currently. At the same time, IMT-2020 networks are evolving towards intelligence and automation, and the demand for automation of E2E network slice provisioning is impending.

As the cross-doamin network slice for IMT-2020 network is composed of multiple different network slice subnets, which belong to different administrative domains. In 3GPP administrative domains such as AN and CN, NSSI is uniquely identified by S-NSSAI, but it is invisible to non-3GPP domain such as TN, the mapping relation between S-NSSAI (NSSI) and interworking identifier (e.g., VLAN ID, MPLS label, UDP port) is required so that the protocol data unit (PDU)  packets from the specified AN/CN NSSI can be correctly forwarded into the corresponding TN-NSSI. On the other hand,  3GPP domains (AN/CN) use 5QI to process packet priority, but TN uses different packet QoS fields depending on different tansport protocols (e.g., MPLS, SRv6). In order to obtain the consistent QoS forwarding behaviour for cross-domain network slice, the mapping relation between 5QI and packet QoS field is also required. Hence it is essential to have the coordination mechanism for slice management systems to set up E2E tunnel on network slicing user plane. The growing demand for network slice makes E2E network slice interworking become a cumbersome task. The coordination of the three network slice subnet management systems (AN, TN and CN) is quite necessary, including but not limited to:

- How to decompose network slice service profile among the three network slice subnets.

- How to configure network connection parameters between network slice subnets.

- How to management and maintain the mapping relations among Single-Network Slice Selection Assistance Information (S-NSSAI), network slice instance (NSI), network slice subnet instance (NSSI) and network slice interworking ID.

- How to realize network slice interworking between network slice subnets.

- How to realize QoS parameters mapping between network slice subnets.

- How to realize the lifecycle management of the interworking identifiers, including interworking ID allocation, update, deletion and recycling.

The traditional provisioning mode depending on individual network slice subnet management system is inefficient and prone to errors. It is essential to establish the mechanism to coordinate with multiple subnet management systems.

# Management framework for cross-domain network slices

Management framework for cross-domain network slices is shown in Figure 7-1, which comprises of two parts: cross-domain network slices management plane and cross-domain network slices user plane.

Figure 1 Management framework for cross-domain network slices in IMT-2020 network and beyond.

Cross-domain network slices management plane is composed of the following network functional elements:

- Communication Service Management Function (CSMF).

- Network Slice Management Function (NSMF).

- Network Slice Subnet Management Function (NSSMF), including AN-NSSMF, TN-NSSMF and CN-NSSMF.

CSMF serves as network slice communication service management function, supporting customer-oriented services as well as network-oriented services. Customer-oriented services include network slice design, network slice directory management, network slice subscription, network slice capability openness, etc; while network-oriented services include network slice resources query, network slice lifecycle management service, network slice performance assurance service, etc.

NSMF is responsible for network slice management function, including network slice lifecycle management, performance management, fault management, network slice identification and resources management, etc. Among them, network slice lifecycle management is one of the most important management functions, involving network slice creation, activation, modification, deletion, de-activation, termination.

AN-NSSMF acts as network slice subnet management function of AN, including AN slice subnet lifecycle management, performance management, fault management and network resources management.

TN-NSSMF acts as network slice subnet management function of TN, including TN slice subnet lifecycle management, performance management, fault management and network resources management.

CN-NSSMF acts as network slice subnet management function of CN, including CN slice subnet lifecycle management, performance management, fault management and network resources management.

Cross-domain network slices user plane is composed of user terminal equipment, network device and network as follows:

- User equipment (UE), including various mobile terminals, IOT terminals, etc.

- Access network (AN), specifically for IMT-2020 network and beyond.

- Transport network (TN), specifically for IP bearer networks.

- Core network (CN), specifically for IMT-2020 network and beyond.

- Data network (DN), including, for example, internet access, 3th party network or customer network.

# Management function for cross-domain network slices

# CSMF functionality

CSMF serves as management functionality of network slice communication service, supporting customer-oriented services and network slice-oriented services, which are shown in Figure 8-1. Specifically, it includes the following aspects.

- Customer-oriented services

Customer-oriented services include:

- Network slice design: CSMF supports the product design of network slice, including slice profile (e.g., service characteristics, quality of service (QoS) requirements, service level object (SLO).), slice price, slice billing, etc.

- Network slice directory management: Provides the available slice product lists, including the number of the available slices, slice types and slice capabilities, etc.

- Network slice SLA input and translation:  When a user initiates a slicing service subscription or update, especially with SLA requirements based on intent-driven input, it can translate such SLA requirements into specific SLO indicators.

- Network slice subscription: Provide network slice subscription services according to the input SLA requirement parameters.

- Network slice capability exposure:

- Provide the subscribed slice instances to users

- Provide users with the real-time query of the subscribed slice instances related to the monitoring information.

- Provide users with the real-time query of the subscribed slice instances related to the running status information.

- Provide vertical industry users with the real-time query of the subscribed slice instances related to key performance indicators.

- Provide users with the query of the subscribed slice instances related to the major alarm information. By calling the real-time alarm information, alarm statistics and alarm analysis from NSMF, user can obtain the running state of the subscribed slice instance and potential problem, so as to make timely adjustment of SLO parameters for service quality guarantee.

2)  Network slice-oriented services

Network slice-oriented services include:

- Network slice resource management: When CSMF accepts the slice subscription service from user, it will call NSMF for the related slice resource allocation and pre-occupation. Also, when it accepts the slice unsubscribing service from user, it will call NSMF for the related slice resource release.

- Network slice lifecycle management includes:

- Activation and deactivation of slice instance: Support the switch operation between the activation and deactivation of the subscribed slice according to the different phases of vertical industry application scenarios.

- Slice parameters change: Support changing slice attributes or parameters according to changes in vertical industry application scenarios.

- Slice unsubscribing: Support unsubscribing of slice after the service related to the subscribed slice terminates or subscription duration expires. The process of slice unsubscribing mainly includes the initiation of unsubscribing, the deletion of the slice instance and slice resource release.

- Network slice performance guarantee: CSMF communicates with network management system (NMS)/NSMF, collecting the related performance measurement data, to assure the performance of slicing service.

Figure 8-1 CSMF functional modules

# Figure 8-1 CSMF functional modules NSMF functionality

NSMF serves as cross-domain network slices management functionality, including E2E network slice service profile decomposition, E2E network slice resources management, cross-domain network slices interworking management, cross-domain network slices QoS  mapping management, cross-domain network slices lifecycle management, cross-domain network slices performance management, cross-domain network slices fault &alarm monitoring, exposure of network slice management data, etc.  NSMF functional modules are shown in Figure 8-2.  Specifically, it includes the following aspects.

Figure 8-2 NSMF functional modules

- E2E network slice service profile decomposition

NSMF receives a network slicing service subscription from CSMF, which includes a set of slicing service attributes, and decomposes network slice service profile (SLA) into the three network slice subnet profiles (SLA) and distributes them to AN-NSSMF, TN-NSSMF and CN-NSSMF, respectively.

Network slice service profile represents the properties of the network slice related requirements that should be supported by a network slice instance, which is the SLA agreed between the user and the network slice provider (NSP). It may include service profile ID, S-NSSAI, slice type (e.g.,  eMBB,  uRLLC and mIoT), maximum number of  user equipments (UEs), coverage area, maximum number of protocol data unit (PDU) sessions, downlink/uplink(DL/UL) throughput, DL/UL latency, slice sharing level, survival time, availability, reliability, etc.

The AN slice subnet profile is decomposed from the network slice service profile, and may include AN slice profile ID, S-NSSAI, maximum number of UEs, slice subnet type, maximum number of PDU sessions, maximum bit error ratio (BER), coverage area, DL/UL throughput, DL/UL latency, terminal mobility level and slice subnet sharing level, etc.

The CN slice subnet profile is decomposed from the network slice service profile, and may include CN slice profile ID, S-NSSAI, maximum number of UEs, slice subnet type, maximum number of PDU sessions, DL/UL throughput, DL/UL latency, slice subnet sharing level, etc.

The TN slice subnet profile is decomposed from the network slice service profile, and may include TN slice profile ID, guaranteed latency, guaranteed bandwidth, jitter, maximum packet loss rate, service protection and recovery time, slice subnet sharing level (e.g., hard slice or soft slice), etc.

- E2E network slice resources  management

NSMF collects network slice subnet resources from AN/CN/TN NSSMF, including the respective network slice subnet topology information and slice subnet connection  information (e.g., device ID, physical  or logical interface, QoS parameters), and constructs the E2E network slice topology by aggregating three slice subnets topology information.

NSMF manages and maintains the existing E2E network slice topology information, and presents the slice topology in the form of visualization, so as to expose to users in a more intuitive way.

NSMF compute the reserved network resources and the available network resources according to the collected network resources and topology information.  When NSMF receives the request for allocation of the network slice instance with certain characteristics, and the request contains network slice related requirements and the information indicating whether the requested NSI could be shared with other NSI or be exclusive. NSMF executes network slice feasibility check. If the requested NSI can be shared and if an existing NSI can be used, NSMF decides to use the existing NSI, and modification of the existing NSI may be needed to satisfy the NSI related requirements. Otherwise, NSMF triggers to create a new NSI. NSMF decides on the constituent NSSIs and the topology of the NSI to be created using the information from service profile. For the constituent NSSIs, NSMF derives network slice subnet related requirements from the network slice related requirements. For the required NSSI(s), NSMF sends network slice subnet related requirements to NSSMF to request allocation of the required NSSI(s).

- Cross-domain network slices interworking management

As the E2E network slice for IMT-2020 network is composed of multiple different network slice subnets, which belong to different administrative domains. In 3GPP administrative domains such as AN and CN, NSSI is uniquely identified by S-NSSAI, but it is invisible to non-3GPP domain such as TN, the mapping relationship between S-NSSAI (NSSI) and interworking identifier (e.g., VLAN ID, MPLS label, UDP port) is required so that the protocol data unit (PDU) packets from the specified AN/CN NSSI can be correctly forwarded into the corresponding TN-NSSI. NSMF needs to manage and maintain interworking IDs between AN NSSI and TN NSSI and between TN NSSI and CN NSSI, pertaining to the same NSI.

NSMF collects slice subnet connection information between TN slice subnet and AN slice subnet, including access network base station (BS) ID and port ID, transport network access device ID and port ID, and number of available VLAN IDs, etc, and slice subnet connection information between TN slice subnet and CN slice subnet, including core network user plane function (UPF) ID and port ID, transport network border device ID and port ID, and number of available VLAN IDs, etc. When receiving a new NSI creation request, according to the slice subnet connection information obtained above, NSMF decides on allocation of interworking IDs for every created NSI. For example, for a new created NSI, VLAN 100 as interworking ID is used between AN NSSI and TN NSSI, and VLAN 200 as interworking ID is used between TN NSSI and CN NSSI.  For the allocated interworking IDs used by a new NSI to be created, NSMF sends these interworking IDs to AN/TN/CN NSSMF to request configuration of the specified NSSI.

NSMF collects AN/TN/CN NSSI ID reported from AN-NSSMF, AN-NSSMF, and AN-NSSMF respectively, establishes the mapping relationship among NSSI ID, slice subnet connection information between TN slice subnet and AN slice subnet, and interworking ID configured by TN NSSMF and AN NSSMF, as well as the mapping relationship among NSSI ID, slice subnet connection information between TN slice subnet and CN slice subnet, and interworking ID configured by TN NSSMF and CN NSSMF. For example, for a new created NSI, NSMF manages and maintains the mapping relationship among NSSI ID, slice subnet connection information, and interworking ID configured by TN NSSMF and AN NSSMF as follows:

| TN-NSSI ID | AN-NSSI ID | TN access device ID | TN access device port ID | AN BS ID | AN BS port ID | interworking ID |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | 1 | Router1 | Router P1 | gNB1 | gNB P1 | VLAN100 |

NSMF manages and maintains the mapping relationship among NSSI ID, slice subnet connection information, and interworking ID configured by TN NSSMF and CN NSSMF as follows:

| TN-NSSI ID | CN-NSSI ID | TN border device ID | TN border device port ID | CN UPF ID | CN UPF port ID | interworking ID |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | 3 | Router10 | Router P6 | UPF2 | UPF P2 | VLAN200 |

- Cross-domain network slices QoS mapping management

In 3GPP domain, AN and  CN use 5QI to handle PDU session priority, but TN make PDU packet priority scheduling based on different QoS field of packet header, depending on different tansport protocols (e.g., pure IPv4/6, SR-MPLS, SRv6). In order to obtain the consistent QoS forwarding behaviour for cross-domain network slices, the mapping relationship between 5QI and QoS field of packet header is also required. NSMF needs to manage and maintain the mapping of  network slices QoS  parameters.  A set of QoS parameters mapped into TN from 5QI in AN/CN are shown in Table 8-1.

Table 8-1 A set of  QoS parameters mapped into TN from 5QI in AN/CN

| 5QI value | Service type | DSCP field |  |  |
| --- | --- | --- | --- | --- |
| 1 | Conversational Voice | 46 |  |  |
| 2 | Conversational Video (Live Streaming) | 34 |  |  |
| 3 | Real Time Gaming, V2X messages | 34 |  |  |
| 4 | Non-Conversational Video (Buffered Streaming) | 34 |  |  |
| 5 | IMS Signalling | 48 |  |  |
| 6 | Video (Buffered Streaming) TCP-based (e.g. www, e-mail, chat, ftp) | 18 |  |  |
| 7 | Voice, Video (Live Streaming) Interactive Gaming | 18 |  |  |
| 8 | Ordinary data service | 18 |  |  |
| 9 | Ordinary data service | 18 |  |  |

For a set of QoS parameters mapped by a new NSI to be created, NSMF sends this set of QoS parameters to AN/TN/CN NSSMF to request configuration of the specified NSSI.

- Cross-domain network slices lifecycle management

Cross-domain network slices lifecycle management includes the following aspects:

- Creation of network slice:

- NSMF receives a network slicing service subscription from CSMF, and decomposes network slice SLA into the three network slice subnet SLA and distributes them to AN-NSSMF, TN-NSSMF and CN-NSSMF, respectively.

- NSMF executes network slice feasibility check. If the requested NSI can be shared and if an existing NSI can be used, NSMF decides to use the existing NSI, and modification of the existing NSI may be needed to satisfy the NSI related requirements. Otherwise, NSMF triggers to create a new NSI.

- NSMF sends slice subnet creation request to AN/TN/CN NSSMF respectively. Every slice subnet creation request message includes the corresponding slice subnet connection information and interworking ID so that AN/TN/CN NSSMF can create the respective slice subnet instance with the interworking ID configured on the corresponding device and port.

- NSMF distributes the QoS parameters mapping between NSSI(s) to AN/TN/CN NSSMF to request configuration of the specified NSSI.

- Activation/deactivation of network slice:

- NSMF receives the activation request of NSI from CSMF, and checks whether NSSIs associated with the NSI are all active, if there is an inactive NSSI, it requests the specified NSSMF to activate the corresponding NSSI.

- NSMF receives the deactivation request of NSI from CSMF, and checks whether NSSIs associated with the NSI are all inactive, if there is an active NSSI, it requests the specified NSSMF to deactivate the corresponding NSSI.

- Modification of network slice:

- NSMF receives the modification request of NSI with new set of network slice related requirements from CSMF, and decides to modify an existing NSI. NSMF identifies the NSSI(s) associated with the NSI to be modified and generates network slice subnet related requirements for the NSSI(s). NSMF sends the modification requests to the specified NSSMFs.

- Termination of network slice:

- Delete a certain NSI in deactivation state.

- When NSMF receives the notification of a certain NSI not used, it decides that this NSI does not be shared by any network slice, then deletes this NSI.

- Cross-domain network slices performance management

Cross-domain network slices performance management includes:

- NSMF collects network slice subnets performance measurement data reported from AN/TN/CN NSSMF, including slice performance KPIs, such as DL/UL throughput, DL/UL latency, maximum bit error ratio (BER), maximum packet loss rate and so on.

- NSMF aggregates network slice subnet performance measurement data from NSSI(s) and generates E2E network slice performance data.

- NSMF monitors the performance of the E2E network slice instance and checks whether the performance requirements are satisfied. Also, NSMF requests AN/TN/CN NSSMF to check whether the performance requirements can be met by the corresponding NSSI by utilizing the respective slice subnet profile. If the performance requirements of NSI are not satisfied, NSMF may modify the capacity of the network slice, or modify the network slice configuration to guarantee the performance requirements.

- NSMF supports querying network slice performance data from CSMF.

- Cross-domain network slices fault &alarm monitoring

Cross-domain network slices fault &alarm monitoring includes:

- NSMF receives the fault &alarms reported from AN/TN/CN NSSMF, and clears the related fault &alarms when they do not occur again.

- NSMF can set the performance thresholds for E2E network slice or NSSI(s). When the collected performance data exceed the thresholds, NSMF can generate the corresponding threshold alarms.

- NSMF supports querying network slice performance thresholds from CSMF.

- Exposure of network slice management data

NSMF should expose network slice or network slice instance related resources, performance data and fault &alarm data to the authenticated third parties or users, including the corresponding relationship among network slice, NSI and NSSI, etc.

# NSSMF functionality

NSSMF functionality includes the following aspects:

- Accept network slice subnet profile from NSMF and configure the corresponding service level objective (SLO) parameters for AN/TN/CN-NSSI.

- Accept network connection parameters between two NSSIs from NSMF and configure subnetwork connection.

- Implement NSSI lifecycle management, including slice subnet creation, activation, modification, deletion, deactivation and termination.

- Manage and maintain the mapping relationship between NSSI (S-NSSAI) and interworking ID.

- Configure the mapping table between NSSI (S-NSSAI) and interworking ID for the specified NSSI related network devices.

- Configure the mapping table between 5QI and QoS priority fields of user packets for the specified NSSI (S-NSSAI)  related network devices.

To be updated.

# Procedures for cross-domain network slices interworking

[Editor's Note] This clause specifies procedures for E2E network slice interworking.

TBD

# Bibliography

[b-3GPP TS 23.501] System Architecture for the 5G System; stage 2 (16.5.0)

_______________________