|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2018-2021 |  |  | SG2-TD1107R1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | STUDY GROUP 2 |  |
|  |  |  |  | Original: English |  |
| Question(s): |  | 5/2 | Virtual, 27 May - 5 June 2020 |  |  |
| TD |  |  |  |  |  |
| Source: |  | Editor, M.resm-AI |  |  |  |
| Title: |  | Revised baseline text for M.resm-AI:" Requirements for energy saving management of 5G RAN system with AI" |  |  |  |
| Purpose: |  | Proposal |  |  |  |
| Contact: |  | Tiantian Lv China Telecom China |  |  | Tel: +86(10)-50902375 Fax: +86(10)-50902230 E-mail: lvtt@chinatelecom.cn |
| Contact: |  | Yanqin Wu China Telecom China |  |  | Tel: +86(10)-50902892 Fax: +86(10)-50902230 E-mail: wuyq@chinatelecom.cn |

| Keywords: | AI, telecom operation management, 5G, energy saving |
| --- | --- |
| Abstract: | This draft recommendation provides the new baseline text for revisions to ITU-T Recommendation M.resm-AI based on the discussion during 27 May-05 June 2020. |

Summary:

This is output of draft M.resm-AI “Requirements for energy saving management of 5G RAN system with AI” based on C253R1 (27 May-05 June 2020) and Q5/2 meeting during 27 May-05 June 2020. This draft proposed some modifications as follows:

- There are some editorial and format revisions in this contribution.

- Add references in Chapter 2, and add terms defined elsewhere and this Recommendation in Chapter 3.

- Add colour of Figure 7-1 in Chapter 7 to make it clearer, and modify some descriptions in Chapter 7.3 and Chapter 7.4.

- Delete DPI system and DPI related content in this document.

- Add relationship between functional block diagram of energy saving management of 5G RAN system with AI and AITOM in Figure 7-2 of Chapter 7.

- Delete the contents in Chapter 8, because they should be included in [ITU-T M.AI-tom], and this document can refer to it.

- Add some descriptions of each use case and scenario of part “Description” in Annex A, and remove Appendix III because it is unnecessary.

Draft ITU-T Recommendation M.resm-AI

Requirements for energy saving management of 5G RAN system with AI

- Scope

This draft Recommendation describes the requirements for energy saving management of 5G (Radio Access Network) RAN system with AI. As an essential technology, AI is applied to the energy saving management of 5G RAN system across vendors and communication systems, such as 4G and 5G. The goal of the draft is to explain the requirements of using AI technology to achieve energy saving management for communication units and virtualized hardware resources of base station, via EMS and open interfaces provided by vendors, from the OSS level. In addition, this draft Recommendation includes process recommendations for sending intelligent energy saving strategies from OSS to EMS and then to wireless equipment. This Recommendation covers the following aspects:

1)	Functional requirements for energy saving management of 5G RAN system with AI;

2)	Use cases and scenarios of energy saving management of 5G RAN system with AI.

The scope does not include requirements for energy saving control directly at the equipment level, requirements for generating energy saving strategies directly at the radio base station, energy saving requirements for non-communication units such as air conditioning refrigeration and hardware material design.

- Reference

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T L.1303] 	Recommendation ITU-T L.1303 (2018), Functional requirements and framework of green data centre energy saving management system.

[ITU-T M.3010] 	Recommendation ITU-T M.3010 (2000), Principles for a telecommunications management network.

[ITU-T M.3050.1]	Recommendation ITU-T M.3050.1(2007), Enhanced Telecom Operations Map (eTOM) - The business process framework.

[ITU-T M.3300]	Recommendation ITU-T M.3300 (1998), TMN F interface requirements.

[ITU-T M.AI-tom] Framework of AI enhanced Telecom Operation and Management (AITOM).

[ITU-T Q.9] 		Recommendation ITU-T Q.9(1990), VOCABULARY OF SWITCHING AND SIGNALLING TERMS.

[ITU-T Y.3100] 	Recommendation ITU-T Y.3100 (2017), Terms and definitions for IMT-2020 network.

[ITU-T Y.3172] Recommendation ITU-T Y.3172 (2019), Architectural framework for machine learning in future networks including IMT-2020

[O-RAN-WG6.CAD-V01.00.00]	Cloud Architecture and Deployment Scenarios for O-RAN Virtualized RAN, TR.

- Definitions

## 3.1 Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	functional block (in SDL) [ITU-T Q.9]: A functional block is an object of manageable size and relevant internal relationship, containing one or more processes.

3.1.2	management [ITU-T Y.3100]: In the context of IMT-2020, the processes aiming at fulfilment, assurance, and billing of services, network functions, and resources in both physical and virtual infrastructure including compute, storage, and network resources.

3.1.3	service [ITU-T M.3050.1]: Services are developed by a Service Provider for sale within Products. The same service may be included in multiple products, packaged differently, with different pricing, etc.

3.1.4	user [ITU-T M.3010]: A person or process applying management services for the purpose of fulfilling management operations.

3.1.5	user interface [ITU-T M.3300]: Human-computer interface or human-machine interface.

## 3.2 Terms defined in this Recommendation

3.2.1	artificial intelligence-based energy saving strategy: A plan of action intended to accomplish energy saving goal based on artificial intelligence technology.

- Acronyms and abbreviation

| AAU | Active Antenna Unit |
| --- | --- |
| AI | Artificial Intelligence |
| BBU | Base Band Unite |
| BSS | Business Support System |
| CPU | Central Processing Unit |
| CQI | Channel Quality Indication |
| CU | Centralized Unit |
| DU | Distribution Unit |
| E-RAB | Evolved Radio Access Bearer |
| EMS | Element Management System |
| KPI | Key Performance Indicator |
| NMS | Network Management System |
| OPEX | Operational Expenses |
| OSS | Operation Support Systems |
| PRB | Physical RB |
| RF | Radio Frequency |
| RRC | Radio Resource Control |
| RRU | Remote Radio Unit |
| RSRP | Reference Signal Received Power |
| RSRQ | Reference Signal Receiving Quality |
| TCP | Transmission Control Protocol |
| TDD | Test-Driven Development |
| UI | User Interface |

- Conventions

None.

- Overview of energy saving management of 5G RAN system with AI

This Recommendation proposes the requirements of energy saving management of 5G RAN system with AI. First, the 5G energy saving problem in cost management, which is the most urgent problem to be solved by operators in 5G commercial applications, will be studied, including use cases and scenarios. In this way, at the OSS level, through the open interface provided by EMS and suppliers, the virtualized hardware resources of communication units and base stations are managed for energy saving, thereby reducing OPEX. This draft Recommendation mainly contains the functional requirements and use case scenarios of energy saving management for 5G RAN system with AI. The goal of the draft is to explain the requirements of using AI technology to achieve energy saving management for communication units and virtualized hardware resources of base station, via open interfaces provided by EMS and vendors, from the OSS level. In addition, this draft Recommendation includes process recommendations for sending intelligent energy saving strategies from OSS to EMS and then to wireless equipment.

- Functional requirements of energy saving management of 5G RAN system with AI

[Editor’s note] The functional block diagram of this chapter can be used as the basis for the use case and scenario flow in Annex A.

Figure 7-1 Functional block diagram of energy saving management of 5G RAN system with AI

This clause presents afunctional block diagram of energy saving management of 5G RAN system with AI is shown in the Figure 7-1, including:

- Data lake block;

- Data process block;

- AI energy saving model library block;

- AI-based energy saving strategy block;

- Commandinterfaceblock;

- User interface block.

Between AI-based energy saving strategy block, command interface block and RAN system need to be fed back status of energy saving strategy reception.

The functional block diagram of energy saving management of 5G RAN system with AI is compliant with functional framework of AITOM. Figure 7-2 shows the relationship between it and AITOM.

Figure 7-2 Relationship between functional block diagram of energy saving management of 5G RAN system with AI and AITOM

This document will introduce the functional requirements of each functional block.

## 7.1 Functional requirements of data lake block

Data lake is the basis of AI data training. Before energy saving, a large amount of data should be collected, and the collected data should be stored, managed and updated in the "Data Lake". The Data lake provides data for the" Data process block" to process data.

For the requirements of data collection and storage, please refer to the requirements for collecting and storing data given in [ITU-T L.1303].The data collection terminal of the 5G RAN system includes BSS,NMS and EMS of the external management system.The following requirements should be followed in the data lake of each terminal:

- Openness of the interface: The interfaces of external management system should be ensured to be open so that data can be collected at any time to avoid problems during the process.

- Consistent data format: Data with the same characteristics shall be stored in "data lake" with consistent format to efficiently control the energy management 5G RAN energy saving system. All collected data shall be in an identical format even though they are from different interfaces.

- A proper collection period: The periodicity of data collection should be defined according to different business requirements. For example, when predicting dynamic shutdown based on PRB and RRC, you need to periodically query the RRC of the cell in real time. You can choose minutes or hours or other periods according to business requirements.

- Accuracy and completeness of data collection: During collecting data from interfaces and storing them into the "data lake" errors in data or data losses are not allowable. Corrupted data should be re-collected or handled simply.

Each collection terminal includes but is not limited to the following data that needs to be collected:

- Performance data: including network KPI index and flow data in EMS system. Such as PRB utilization rate, RRC connection number, load flow rate, CPU utilization, RRC success rate, E-RAB success rate, CQI, delay, TCP delay and video traffic, etc.

- Customer data: including customer order data in the BSS system.

- Parameter data: including base station information data and location data in the NMS system. Such as latitude and longitude, direction angle, transmit power, beam parameters (Such as the inclination angle, the number of antennas, beam width and so on), RSRP, RSRQ, MR latitude and longitude, frequency, etc.

The data in BSS system are optional, but in EMS and NMS system they are mandatory. They need to be chosen based on specific service scenarios and requirements.

## 7.2 Functional requirements of data process block

Data processing is an important requirement in AI data management. It is necessary to be managed of data in data lake, including data cleaning, feature construction, label and evaluation system construction, data set construction, etc.

- Data cleaning: Including missing value filling, outlier processing, data normalization and standardization, etc.

- Feature extraction: Including feature construction and feature selection. In the process of feature extraction needs to consider feature normalization, standardization and feature dimensionality reduction.

- Feature data storage: Based on data cleaning and feature extraction, the processed features are stored in the database. It is necessary to pay attention to the consistency of the feature data format during the storage process.

## 7.3 Functional requirements of AI energy saving model library block

The AI energy saving model library mainly trains, storages and manages different AI energy saving models, such as the energy saving scenario recognition model, base station traffic prediction model, energy saving strategy selection model and base station cooperative energy saving technology, etc.

Business logic needs to be combined to deploy AI model to the "AI-based energy saving strategy block". In the process of model training, the fusion between different models shall be considered to improve the accuracy of model prediction and the efficiency of energy saving.

## 7.4 Functional requirements of AI-based energy saving strategy block

The AI-based energy saving strategy block predicts the type of base station through the AI model. At the same time, based on the trained AI model, the data process block is used to provide feature data to generate AI-based energy saving strategies. In the whole energy saving process, energy saving strategy generation, energy saving strategy distribution, and energy saving effects evaluation should be considered.

- Energy saving strategy generation: The energy saving strategy can be generated according to the AI energy saving model defined in Section 7.2. For example, the automatic recognition of typical scenes can be realized through the energy saving scene recognition model, which can improve the accuracy of time series prediction. Then combined the results of automatic scene recognition and base station traffic prediction model to predict the load time series of base stations with different granularity, respectively.

- Energy saving strategy delivery: Deliver energy saving strategies commands to “command interface block”.

- Energy saving effects evaluation: energy saving effect evaluation block needs to evaluate the energy saving effect through the energy saving effect evaluation index, and feeds back to the energy saving strategy generation module to realize closed-loop feedback of the energy saving system and automatic adjustment of the energy saving strategy.

### 7.4.1Energy saving strategy generation

Energy saving strategy generation is a key requirement step for modelling of AI.It is necessary to combine the business scenarios to generate the optimal energy saving strategy based on the trained AI model.

Base station load traffic forecasting is a key demand step in the generation of energy saving strategies. It is necessary to predict the base station load traffic based on the AI model trained by the AI energy saving model library. The integrity and consistency of data and characteristics, the consistency between models and the accuracy of model prediction should be paid attention to when forecasting base station load traffic.

### 7.4.2Energy saving strategies delivery

The energy saving strategies distribution block needs to deliver the generated energy saving strategy to the RAN system device through the commandinterface block.

In terms of energy saving control of base station equipment, the following five energy saving methods can be considered: carrier frequency shutdown, channel shutdown, symbol shutdown, deep sleep, etc.

### 7.4.3Energysaving effect evaluation

Energy saving effect evaluation is an important step for AI test optimization. Through the evaluation based on the prediction results, the advantages and disadvantages of energy saving strategies and potential problems shall be evaluated, which can provide real-time feedback to the energy saving strategy module.

The evaluation of energy saving effects should have the following requirements:

- The evaluation of energy saving effects should be based on the evaluation index system of scenarios and business needs, including base station performance index data and user perception data;

- There should be a feedback mechanism between the energy saving strategy selection model and the energy saving effect evaluation to realize iterative optimization;

## 7.5Functional requirements of command interface block

After generating the AI-based energy saving strategy, the system needs to send commands in the command interface block to control the base station equipment through the interface in order to execute the energy saving strategy.

The control block needs to have a unified command format and can provide status feedback to the "AI-based energy saving strategy block".

The interfaces include interface of RAN EMS and RAN virtualized device EMS. It needs to be delivered to BBU / CU / DU through the interface provided by RAN EMS, and then delivered to RRU / AAU. At the same time, it is delivered to the virtualized base station equipment through the interface provided by the RAN virtualized device EMS. During the delivery process, the relevant interface protocol should be followed and RRU / AAU needs to be controlled through BBU / CU / DU.

## 7.6Functional requirements of user interface block

Through the user interface block, the UI display of the energy saving effect and the range of system coverage are realized on the computer side and the mobile device side.

- Security consideration

# The security consideration in this document refer to [ITU-T M.AI-tom].

# Annex A

Use cases and scenarios of energy saving management of 5G RAN system with AI

(This annex forms an integral part of this Recommendation.)

## A.1 Use case template

The use cases developed in Appendix I should adopt the following unified format for consistent readability and convenient material organization.

| Title | Title of the use case |
| --- | --- |
| Description | Scenarios description of the use case, including the purpose and related process to achieve them |
| Roles | Roles involved in the use case, including any primary roles / persona that is directly responsible for this use case and any secondary roles / persona that are related to this use case (such as Customer Care Manager, Network Operations Centre Manager) |
| Figure (optional) | Figure to explain the use case, but is not mandatory |
| Pre-conditions (optional) | The necessary pre-conditions that should be achieved before starting the use case. |
| Post-conditions (optional) | The post-condition that will be carried out after the termination of current use case. |
| Derived requirements | Requirements derived from the use cases, whose detailed descriptions are presented in the dedicated clauses. |

## A.2 Typical use cases and scenarios of energy saving management of 5G RAN system with AI

### A.2.1 Energy saving for different deployment scenario

| Title | Energy saving for different deployment scenario |
| --- | --- |
| Description | Different deployment scenario (like shopping mall, school, office building) has different traffic characteristics. Based on these characteristics, specific energy saving strategy should be adopted to reduce OPEX. In the part P, when 5G energy saving is performed, generate power saving strategy should be combined service logic with different deployment scenario in the AI-based energy saving strategy block. The part P consists of the following three steps: - Use AI technology to judge the scenario of base station; - Apply corresponding AI load prediction technology according to different scenarios; - Finally, the AI-based energy saving strategy selection model can generate different shutdown strategies. |
| Roles | Communication service provider, Infrastructure vendor, customer, OAM engineer |
| Figure (optional) | The flow figure refers to [ITU-T Y.3172]. |
| Pre-conditions (optional) | Traffic characteristics need to be determined. |
| Post-conditions (optional) | Optimal performance of energy saving is achieved based on specific strategy. |
| Derived requirements | All requirements take section 7 as basis |

### A.2.2 Energy saving for different type of base station

| Title | Energy saving for different type of base station |
| --- | --- |
| Description | Macro base station is used for larger cell and micro base station is used for indoor scenario or smaller cell. For macro base station, the target is to save energy of RF while for micro base station, both energy of RF and BBU need to be saved. In the part P, when 5G energy saving is performed, generate power saving strategy should be combined service logic with different type of base station in the AI-based energy saving strategy block. |
| Roles | Communication service provider, Infrastructure vendor, customer, OAM engineer |
| Figure (optional) |  |
| Pre-conditions (optional) | Type of base station needs to be determined. |
| Post-conditions (optional) | Optimal performance of energy saving is achieved based on specific strategy. |
| Derived requirements | All requirements take section 7 as basis |

### A.2.3 Energy saving for cooperation among multiple cells

| Title | Energy saving for cooperation between multiple cells |
| --- | --- |
| Description | Each cell may have independent AI strategy for energy saving but that will cause collision and impact the performance. For the whole network, cooperation of multiples cells from energy saving strategy point of view is needed. In the part P, when 5G energy saving is performed, generate power saving strategy should be combined service logic based on cooperation between multiple cells in the AI-based energy saving strategy block. |
| Roles | Communication service provider, Infrastructure vendor, customer, OAM engineer |
| Figure (optional) |  |
| Pre-conditions (optional) | Cooperation of multiple cells is feasible |
| Post-conditions (optional) | Optimal performance of energy saving of the network is achieved based on specific strategy. |
| Derived requirements | All requirements take section 7 as basis |

### A.2.4 Energy saving for different duplex mode

| Title | Energy saving for different duplex mode |
| --- | --- |
| Description | For TDD, the energy saving strategy adopted for downlink needs to take uplink traffic into account. For FDD, the energy saving strategy needs to be considered for DL and UL, respectively. In the part P, when 5G energy saving is performed, generate power saving strategy should be combined service logic with different duplex mode in the AI-based energy saving strategy block. |
| Roles | Communication service provider, Infrastructure vendor, customer, OAM engineer |
| Figure (optional) |  |
| Pre-conditions (optional) | Duplex mode needs to determined |
| Post-conditions (optional) | Optimal performance of energy saving is achieved based on specific strategy. |
| Derived requirements | All requirements take section 7 as basis |

### A.2.5 Energy saving for status switching of base station

| Title | Energy saving for status switching of base station |
| --- | --- |
| Description | Using AI to realize optimal status switching of base station among “ON”, “Standby”, “Sleep” and “OFF”. ON: both BBU and RRU are activated. Standby: Only RRU and TCXO heater are inactive. Other components like BBU are active. Sleep: only power supply, backend connection and CPU are active. OFF: all components are inactive. In the part P, when 5G energy saving is performed, generate power saving strategy should be based on status switching od base station in the AI-based energy saving strategy block. |
| Roles | Communication service provider, Infrastructure vendor, customer, OAM engineer |
| Figure (optional) |  |
| Pre-conditions (optional) | Four statuses of energy saving are supported. |
| Post-conditions (optional) | Optimal performance of energy saving is achieved based on specific strategy. |
| Derived requirements | All requirements take section 7 as basis |

### A.2.6 Energy saving for virtualized hardware resources

| Title | Energy saving for virtualized hardware resources |
| --- | --- |
| Description | BBU/ (CU, DU) (may as well as RRU/AAU) will be virtualized on top of cloud platform, which will consume the energy. AI based strategy is used to realize optimal resource utilization of cloud resources so that energy is saved. In the part P, when formulating energy saving strategies, the energy consumption of virtualized device resources of the part SINK should be considered. |
| Roles | Communication service provider, Infrastructure vendor, customer, OAM engineer |
| Figure (optional) |  |
| Pre-conditions (optional) | Virtualized cloud platform is implemented for wireless communication units like BBU/(CU,DU) or RRU/AAU. |
| Post-conditions (optional) | Optimal performance of energy saving is achieved based on specific strategy. |
| Derived requirements | All requirements take section 7 as basis |

# Appendix I

Relation to ITU-T Recommendations L.5G_sav –Energy saving technologies and best practices for 5G RAN equipment

(This appendix does not form an integral part of this Recommendation.)

L.5G_sav focuses on equipment level, for example test methods and the best practices for 5G RAN equipment. However, M.resm-AI focuses on requirements and general scenarios of energy saving for 5G RAN system from telecom operation management point of view.

In addition, M.resm-AI focuses on OSS level. While considering energy saving for 5G RAN system, other factors, such as assurance of customer experience and cell coverage, should be considered comprehensively. M.resm-AI will not consider the impact and deployment of the energy consumption test method and the best practices for 5G RAN equipment on telecom operation management.

The output of M.resm-AI could be input to L.5G_sav, and affect the formulation of energy saving methods.

# Appendix II

The background of Requirements for energy saving management of 5G RAN system with AI

(This appendix does not form an integral part of this Recommendation.)

Compared with 4G Network, 5G network has higher frequency band and weaker coverage, so the number of base stations needed has increased greatly, and the energy consumption has also increased sharply. The power consumption of 5G single station is about 2.5-3.5 times of 4G, of which the power consumption of wireless communication unit is more than 50% of that of base station. At the same time, the complexity of network and the difficulty of operation and management make OPEX increase rapidly.

The energy saving technology of the traditional general model can’t meet the requirement of 5G large-scale deployment and high-efficiency energy saving, so it is necessary to introduce AI technology to realize intelligent monitoring and optimization of equipment and network. So, how to use AI to save energy in wireless communication unit has become a key issue to be considered in telecom operation and management. In addition, as what is discussed in O-RAN [O-RAN-WG6.CAD-V01.00.00], for wireless communication units like BBU and RRU, they may be implemented over cloud platform as virtualized network function. For these cloud platform (e.g., memory and computer resource), there would be some energy consumption as well. So, using AI to realize optimal cloud resource utilization is also beneficial for energy saving. At the same time, the application of telecom operation management needs to be driven and realized by designing a new architecture(Reference [ITU-T M.AI-tom]) and that integrates AI capabilities, which will drive telecom operation management technology to bring major changes in the telecom operation management model, make service processes been redefined and designed.

Although the research on energy saving of 5G RAN has been carried out in some SDOs, the current researches have not solved the problem of how AI is applied to energy saving management of 5GRAN system. For example, the research of TMF focuses on AI maturity assessment model, AI service management architecture and user case collection. The researches of ITU-T FG-ML5G and SG13 are mainly focused on IMT2020, most of which are still in the discussion stage. ITU-R WP 5D does not have any work at this time related to Energy Savings Management and AI. ETSI is more focused on network-level AI applications, and so far, only a few use cases have been proposed. 3GPP has no relevant standard output for operation management in AI. Although the standardization research on AI has been carried out in several major international SDO, it has not systematically explained how AI is used in telecom operation and management in the published results, as a general technical means.

______________