| # Liaison Statement |  |  |  |
| --- | --- | --- | --- |
| Title: |  | Liaison Statement answer to 3GPP SA2 on MTC Identification aspects |  |
| Date: |  | July 1st 2011 |  |
| From (source): |  | ETSI TC M2M |  |
| Contact(s): |  | Zhou, Jingyi, Jingyi.Zhou@VerizonWireless.com, David Boswarthick, M2Msupport@etsi.org |  |
| To: |  | 3GPP TSG SA WG2 |  |
| Copy to: |  | ITU-T SG2 , 3GPP TSG SA WG1, 3GPP2 TSG-S, 3GPP2 TSG-X |  |
| Response to: (if applicable) |  | M2M(11)0295 |  |
| Attachments: (if applicable) |  |  |

1. Overall description:

ETSI TC M2M would like to thank 3GPP TSG SA WG2 for the liaison on identifiers (Copy to 3GPP SA1 and ITU-T SG2) to be used on interfaces between the MTC Server and the 3GPP Core Network.

This liaison generated extensive discussion within ETSI TC M2M. We would like to provide the following input pertaining to the key aspects of the 3GPP SA2 liaison.

- The requirements ETSI TC M2M has currently identified for guiding the definition of any device identifier, on what interfaces those are believed to be applicable

ETSI M2M has only defined requirement for the ETSI M2M device identifier. The ETSI M2M device identifier is intended to be used only on interfaces defined by ETSI M2M.

- ETSI TC M2M view on to what extent device identifiers that are used internally within a specific Transport network such as 3GPP, need to be aligned with device identifiers used at the ETSI TC M2M service layer ( exposed through the mIa, dIa, and mId interfaces);

After extensive discussion ETSI TC M2M came to the conclusion that the indentifier that is used on external interfaces to 3GPP is to be mapped (by the ETSI M2M Platform) to the ETSI M2M identifier.

- Whether ETSI TC M2M believe the identifier should identify the MTC device or e.g. the MTC subscription.

Our understanding of MTC subscription is that such a subscription may group multiple MTC devices, e.g. all smart meters belonging to a given utility company. Therefore the notion of MTC subscription shall not, in our view, replace the notion of MTC device identifier. It may be used in addition to the MTC device identifier as appropriate.

ETSI M2M currently supports multiple applications on a single MTC device and understands the needs to charge on a per application basis and will investigate solutions in the future.

We like also to inform that ETSI M2M release1 specifications are expected in autumn 2011.

2. Actions to 3GPP SA WG2:

Please take note of the above information and the timing of ETSI TC M2M Release 1.

3. Date of next meetings of the originator:

12-16 Sep 2011: M2M#16     - TBD JP

24-28 Oct 2011:  M2M-16 Bis     - Sophia Antipolis FR

26-27 Oct 2011: M2M-Workshop      Sophia Antipolis Cedex FR