### 5.1.2	Detailed specification of the radio interface

The standards contained in this section are derived from the global core specifications for IMT 2000 contained at
http://ties.itu.int/u/itu-r/ede/rsg5/IMT-2000/GCS/GCSrev13. Only the specifications listed below are relevant to this Recommendation. For releases prior to Release 8, see Recommendation ITU-R M.1457-12.

The following notes apply to the sections below, where indicated:

1)   The relevant SDOs should make their reference material available from their web site.

2)   This information was supplied by the recognized external organizations and relates to their own deliverables of the transposed global core specification.

#### 5.1.2.1	25.200 series

##### 5.1.2.1.1	TS 25.201

Physical layer – General description

This specification gives a general description of the physical layer of the UTRA radio interface.

##### 5.1.2.1.2	TS 25.211

Physical channels and mapping of transport channels onto physical channels (FDD)

This specification describes the characteristics of the Layer 1 transport channels and physical channels in the FDD mode of UTRA. The main objectives of the specification are to be a part of the full description of the UTRA Layer 1, and to serve as a basis for the drafting of the actual technical specification.

##### 5.1.2.1.3	TS 25.212

Multiplexing and channel coding (FDD)

This specification describes the characteristics of the Layer 1 multiplexing and channel coding in the FDD mode of UTRA.

##### 5.1.2.1.4	TS 25.213

Spreading and modulation (FDD)

This specification describes spreading and modulation for UTRA physical layer FDD mode.

##### 5.1.2.1.5	TS 25.214

Physical layer procedures (FDD)

This specification describes and establishes the characteristics of the physical layer procedures in the FDD mode of UTRA.

##### 5.1.2.1.6	TS 25.215

Physical layer; Measurements (FDD)

This specification describes the measurements done at the UE and network in order to support operation in idle mode and connected mode for

FDD mode.

#### 5.1.2.2	25.300 series

##### 5.1.2.2.1	TS 25.301

Radio interface protocol architecture

This specification describes an overview and overall description of the UE-UTRAN radio interface protocol architecture. Details of the radio

protocols will be specified in companion documents.

##### 5.1.2.2.2	TS 25.302

Services provided by the physical layer

This specification describes a technical specification of the services provided by the physical layer of UTRA to upper layers.

##### 5.1.2.2.3	TS 25.303

Interlayer procedures in Connected Mode

This specification describes informative interlayer procedures to perform the required tasks. This specification attempts to provide a comprehensive overview of the different states and transitions within the connected mode of universal mobile telecommunications system (UMTS) terminal.

##### 5.1.2.2.4	TS 25.304

User Equipment (UE) procedures in idle mode and procedures for cell reselection in connected mode

This specification describes the overall idle mode process for the UE and the functional division between the non-access stratum and access stratum in the UE. The UE is in idle mode when the connection of the UE is closed on all layers, e.g. there is neither an MM connection nor an RRC connection. 
This specification presents also examples of inter-layer procedures related to the idle mode processes and describes idle mode functionality of a dual mode UMTS/GSM UE.

##### 5.1.2.2.5	TS 25.305

Stage 2 functional specification of User Equipment (UE) positioning in UTRAN

This document specifies the Stage 2 of the UE Positioning function of UTRAN, which provides the mechanisms to support the calculation of

the geographical position of a UE.

##### 5.1.2.2.6	TS 25.306

UE Radio Access capabilities

This document identifies the parameters of the access stratum part of the UE radio access capabilities. Furthermore, some reference configurations of these values are defined. The intention is that these configurations will be used for test specifications.

##### 5.1.2.2.7	TS 25.307

Requirements on User Equipments (UEs) supporting a release-independent frequency band

This document specifies requirements on UEs supporting a frequency band that is independent of release.

This document specifies requirements on UEs supporting a frequency band that is independent of release.

##### 5.1.2.2.8	TS 25.308

UTRA High Speed Downlink Packet Access (HSDPA); Overall description; Stage 2

This document is a technical specification of the overall support of High Speed Downlink Packet Access in UTRA.

##### 5.1.2.2.9	TS 25.319

Enhanced uplink; Overall description; Stage 2

This document is a technical specification of the overall support of FDD and TDD Enhanced Uplink in UTRA.

##### 5.1.2.2.10	TS 25.321

Medium Access Control (MAC) protocol specification

This specification describes the MAC protocol.

##### 5.1.2.2.11	TS 25.322

Radio Link Control (RLC) protocol specification

This specification describes the RLC protocol.

##### 5.1.2.2.12	TS 25.323

Packet Data Convergence Protocol (PDCP) specification

This document provides the description of the Packet Data Convergence Protocol (PDCP). PDCP provides its services to the NAS at the UE or the relay at the Radio Network Controller (RNC). PDCP uses the services provided by the Radio Link Control (RLC) sublayer.

##### 5.1.2.2.13	TS 25.324

Broadcast/Multicast Control (BMC)

This document provides the description of the Broadcast/Multicast Control Protocol (BMC). This protocol adapts broadcast and multicast

services on the radio interface.

##### 5.1.2.2.14	TS 25.327

High Speed Packet Access (HSPA); Requirements on User Equipments (UEs) supporting a release-independent frequency band and multi-carrier configuration

This document specifies requirements on UEs supporting a DB-DC-HSDPA configuration that is independent of release. TSG-RAN has agreed that the standardisation of new configurations may be independent of a release. However, in order to implement a UE that conforms to a particular release but supports a configuration that is specified in a later release, it is necessary to specify some extra requirements.

##### 5.1.2.2.15	TS 25.331

Radio Resource Control (RRC); Protocol specification

This specification describes the RRC protocol for the radio system. The scope of this specification contains also the information to be transported in a transparent container between source RNC and target RNC in connection to SRNC relocation.

##### 5.1.2.2.16	TS 25.346

Introduction of the Multimedia Broadcast/Multicast Service (MBMS) in the Radio Access Network (RAN); Stage 2

This document is a technical specification of the overall support of Multimedia Broadcast and Multicast Services in UTRA.

##### 5.1.2.2.17	TS 25.367

Mobility procedures for Home Node B (HNB); Overall description; Stage 2

This document provides a high level description of the mobility procedures applicable to Home NodeB support.

#### 5.1.2.3		25.400 series

##### 5.1.2.3.1	TS 25.401

UTRAN overall description

This specification describes the overall architecture of the UTRAN, including internal interfaces and assumptions on the radio and Iu interfaces.

##### 5.1.2.3.2	TS 25.402

Synchronisation in UTRAN Stage 2

This document constitutes the Stage 2 specification of different synchronisation mechanisms in UTRAN and on Uu.

##### 5.1.2.3.3	TS 25.410

UTRAN Iu interface: General aspects and principles

This specification describes an introduction to the 25.41x series of technical specifications that define the Iu interface for the interconnection of RNC component of the UTRAN to the CN.

##### 5.1.2.3.4	TS 25.411

UTRAN Iu interface layer 1

This specification describes the standards allowed to implement Layer 1 on the Iu interface. The specification of transmission delay requirements and O&M requirements are not in the scope of this specification.

##### 5.1.2.3.5	TS 25.412

UTRAN Iu interface signalling transport

This specification describes the standards for Signalling Transport to be used across Iu Interface.

##### 5.1.2.3.6	TS 25.413

UTRAN Iu interface Radio Access Network Application Part (RANAP) signalling

This specification describes the signalling between the CN and the UTRAN over the Iu interface.

##### 5.1.2.3.7	TS 25.414

UTRAN Iu interface data transport and transport signalling

This specification describes the standards for user data transport protocols and related signalling protocols to establish user plane transport bearers over the Iu interface.

##### 5.1.2.3.8	TS 25.415

UTRAN Iu interface user plane protocols

This specification describes the protocols being used to transport and control over the Iu interface, the Iu user data streams.

##### 5.1.2.3.9	TS 25.419

UTRAN Iu-BC interface: Service Area Broadcast Protocol (SABP)

This document specifies the Service Area Broadcast Protocol (SABP) between the Cell Broadcast Centre (CBC) and the Radio Network Controller (RNC).

##### 5.1.2.3.10	TS 25.420

UTRAN Iur interface general aspects and principles

This specification describes an introduction to the TSG RAN TS 25.42x series of technical specifications that define the Iur interface. It is a logical interface for the interconnection of two RNC components of the UTRAN.

##### 5.1.2.3.11	TS 25.421

UTRAN Iur interface layer 1

This specification describes the standards allowed to implement Layer 1 on the Iur interface. The specification of transmission delay requirements and O&M requirements are not in the scope of this specification.

##### 5.1.2.3.12	TS 25.422

UTRAN Iur interface signalling transport

This specification describes the standards for Signalling Transport to be used across Iur Interface.

##### 5.1.2.3.13	TS 25.423

UTRAN Iur interface Radio Network Subsystem Application Part (RNSAP) signalling

This specification describes the radio network layer signalling procedures between RNCs in UTRAN.

##### 5.1.2.3.14	TS 25.424

UTRAN Iur interface data transport & transport signalling for Common Transport Channel data streams

This specification describes the UTRAN RNS-RNS (Iur) interface data transport and transport signalling for common transport channel data streams.

##### 5.1.2.3.15	TS 25.425

UTRAN Iur interface user plane protocols for Common Transport Channel data streams

This specification describes the UTRAN RNS-RNS (Iur) interface user plane protocols for common transport channel data streams.

##### 5.1.2.3.16	TS 25.426

UTRAN Iur and Iub interface data transport & transport signalling for DCH data streams

This specification describes the transport bearers for the DCH data streams on UTRAN Iur and Iub interfaces. The corresponding transport network control plane is also specified. The physical layer for the transport bearers is outside the scope of this TS.

##### 5.1.2.3.17	TS 25.427

UTRAN Iub/Iur interface user plane protocol for DCH data streams

This specification describes the UTRAN Iur and Iub interfaces user plane protocols for dedicated transport channel data streams.

##### 5.1.2.3.18	TS 25.430

UTRAN Iub Interface: general aspects and principles

This specification describes the TSG RAN TS 25.43x series of UMTS technical specifications that define the Iub interface. The Iub interface is a logical interface for the interconnection of Node B and RNC components of the UTRAN.

##### 5.1.2.3.19	TS 25.431

UTRAN Iub interface Layer 1

This specification describes the standards allowed to implement Layer 1 on the Iub interface. The specification of transmission delay requirements and O&M requirements is not in the scope of this specification.

##### 5.1.2.3.20	TS 25.432

UTRAN Iub interface: signalling transport

This specification describes the signalling transport related to the Node B application part (NBAP) signalling to be used across the Iub interface. The Iub interface is a logical interface for the interconnection of Node B and RNC components of the UTRAN. The RNC signalling between these nodes is based on NBAP.

##### 5.1.2.3.21	TS 25.433

UTRAN Iub interface Node B Application Part (NBAP) signalling

This specification describes the standards for NBAP specification to be used over Iub interface.

##### 5.1.2.3.22	TS 25.434

UTRAN Iub interface data transport and transport signalling for Common Transport Channel data streams

This specification describes the UTRAN RNC-Node B (Iub) interface data transport and transport signalling for CCH data streams.

##### 5.1.2.3.23	TS 25.435

UTRAN Iub interface user plane protocols for Common Transport Channel data streams

This specification describes the UTRAN RNC-Node B (Iub) interface user plane protocols for common transport channel data streams.

##### 5.1.2.3.24	TS 25.442

UTRAN implementation-specific O&M transport

This specification describes the transport of implementation specific O&M signalling between Node B and the management platform in case that the transport is routed via the RNC.

##### 5.1.2.3.25	TS 25.446

MBMS synchronisation protocol (SYNC)

This specification describes the MBMS Synchronisation Protocol. For the release of this specification it is used on Iu towards UTRAN.

##### 5.1.2.3.26	TS 25.450

UTRAN Iupc interface general aspects and principles

This document is an introduction to the TSG RAN TS 25.45z series of UMTS Technical Specifications that define the Iupc Interface. The Iupc interface is a logical interface for the interconnection of Standalone SMLC (SAS) and Radio Network Controller (RNC) components of the Universal Terrestrial Radio Access Network (UTRAN) for the UMTS system.

##### 5.1.2.3.27	TS 25.451

UTRAN Iupc interface layer 1

This document specifies the standards allowed to implement Layer 1 on the Iupc interface.

##### 5.1.2.3.28	TS 25.452

UTRAN Iupc interface: signalling transport

This document specifies the signalling transport related to PCAP signalling to be used across the Iupc interface.

##### 5.1.2.3.29	TS 25.453

UTRAN Iupc interface Positioning Calculation Application Part (PCAP) signalling

This document specifies the Positioning Calculation Application Part (PCAP) between the Radio Network Controller (RNC) and the Stand-alone SMLC (SAS).

##### 5.1.2.3.30	TS 25.460

UTRAN Iuant interface: General aspects and principles

This document is an introduction to the TSG RAN TS 25.46x series of UMTS Technical Specifications that define the Iuant Interface. The logical Iuant interface is a Node B internal interface between the implementation specific O&M function and the Remote Electrical Tilting (RET) Antenna Control unit function of the Node B.

##### 5.1.2.3.31	TS 25.461

UTRAN Iuant interface: Layer 1

This document specifies the standards allowed to implement Layer 1 on the Iuant interface. The specification of transmission delay requirements and O&M requirements are not in the scope of this document.

##### 5.1.2.3.32	TS 25.462

UTRAN Iuant interface: Signalling transport

This document specifies the signalling transport related to RETAP signalling to be used across the Iuant interface.

##### 5.1.2.3.33	TS 25.466

UTRAN Iuant interface: Application part

This document specifies the Remote Electrical Tilting Application Part (RETAP) between the implementation specific O&M transport function and the RET Antenna Control unit function of the Node B. The document also specifies the Tower Mounted Amplifier Application Part (TMAAP) between the implementation specific O&M transport function and the TMA control function of the Node B. It defines the Iuant interface and its associated signaling procedures.

##### 5.1.2.3.34	TS 25.467

UTRAN architecture for 3G Home Node B (HNB); Stage 2

This document specifies the UTRAN architecture for 3G Home NodeB (3G HNB).

##### 5.1.2.3.35	TS 25.468

UTRAN Iuh Interface RANAP User Adaption (RUA) signalling

This document specifies the RANAP User Adaption (RUA) between the Home Node B (HNB) and the Home Node B Gateway (HNB-GW).

##### 5.1.2.3.36	TS 25.469

UTRAN Iuh interface Home Node B (HNB) Application Part (HNBAP) signalling

This document specifies the Home Node B Application Part (HNBAP) between the Home Node B (HNB) and the Home Node B Gateway (HNB_GW).

##### 5.1.2.3.37	TS 25.470

UTRAN Iuh Interface PCAP User Adaption (PUA) signalling

This document specifies the PCAP User Adaption (PUA) between the Home Node B (HNB) and the Home Node B Gateway (HNB-GW). It fulfils the HNB- HNB-GW communication requirements specified in TS 25.467 and is defined over the Iuh – reference point. It provides transport for PCAP messages.

##### 5.1.2.3.38	TS 25.471

UTRAN Iurh interface Radio Network Subsystem Application Part (RNSAP) User Adaption (RNA) signalling

This document specifies the RNSAP User Adaption (RNA) supporting Iurh-connectivity between HNBs as specified in TS 25.467 by adapting the services made available by the Iurh signalling transport layer to the needs of RNSAP. It provides transparent transport for RNSAP messages in connection-oriented and connectionless mode and an Iurh setup function.

##### 5.1.2.3.39	TS 25.484

Automatic Neighbour Relation (ANR) for UTRAN; Stage 2

This document specifies the Automatic Neighbour Relation (ANR) function for UTRAN.

#### 5.1.2.4	25.100 series

##### 5.1.2.4.1	TS 25.101

User Equipment (UE) radio transmission and reception (FDD)

This document establishes the minimum RF characteristics of the UTRA User Equipment (UE) operating in the FDD mode. The values in the TS make no allowance for measurement uncertainty in conformance testing. Test limits to be used for conformance testing are specified separately in the UE conformance test specifications TS 34.121.

##### 5.1.2.4.2	TS 25.106

UTRA repeater radio transmission and reception

This document establishes the minimum radio frequency performance of UTRA repeaters.

##### 5.1.2.4.3	TS 25.133

Requirements for support of radio resource management (FDD)

This specification describes the requirements for support of radio resource management for FDD including requirements on measurements in UTRAN and the UE as well as on node dynamic behaviour and interaction, in terms of delay and response characteristics.

##### 5.1.2.4.4	TS 25.104

Base Station (BS) radio transmission and reception (FDD)

This specification describes the base station minimum RF characteristics of the FDD mode of UTRA. The values in the TS make no allowance for measurements uncertainties in conformance testing. Test limit to be used for conformance testing are specified separately in the BS conformance test Specification TS 25.141.

##### 5.1.2.4.5	TS 25.141

Base Station (BS) conformance testing (FDD)

This specification describes the RF test methods and conformance requirements for UTRA base transceiver stations (BTS) operating in the FDD mode. These have been derived from, and are consistent with, the core UTRA specifications specified in the requirements reference subclause of each test.

The maximum acceptable measurement uncertainty is specified in the TS for each test, where appropriate.

##### 5.1.2.4.6	TS 25.113

Base station (BS) and repeater electromagnetic compatibility (EMC)

This specification describes the assessment of base stations and associated ancillary equipment in respect of EMC. This specification does not include the antenna port immunity and emissions.

##### 5.1.2.4.7	TS 25.143

UTRA repeater conformance testing

This document specifies the Radio Frequency (RF) test methods and Minimum Requirements for UTRA Repeaters. These have been derived from, and are consistent with the UTRA Repeater specifications defined in TS 25.106.

##### 5.1.2.4.8	TS 25.171

Requirements for support of Assisted Global Positioning System (A-GPS); Frequency Division Duplex (FDD)

This document establishes the minimum performance requirements for A-GPS for FDD mode of UTRA for the User Equipment (UE).

##### 5.1.2.4.9	TS 25.144

User Equipment (UE) and Mobile Station (MS) over the air performance requirements

This document establishes Over the Air antenna minimum requirements for User Equipment (UE) and Mobile Station (MS).

##### 5.1.2.4.10	TS 25.111

Location Measurement Unit (LMU) performance specification; User Equipment (UE) positioning in UTRAN

This document establishes the Location Measurement Unit (LMU) minimum RF characteristics of the FDD mode of UTRA.

#### 5.1.2.5		34.100 series

##### 5.1.2.5.1	TS 34.108

Common test environments for User Equipment (UE); Conformance testing

This document contains definitions of reference conditions and test signals, default parameters, reference Radio Bearer configurations, common requirements for test equipment and generic set-up procedures for use in UE conformance tests.

##### 5.1.2.5.2	TS 34.109

Terminal logical test interface; Special conformance testing functions

This document specifies for User Equipment (UE), in UMTS system, for FDD and TDD modes, those UE functions that are required for conformance testing purposes.

##### 5.1.2.5.3	TS 34.114

User Equipment (UE) / Mobile Station (MS) Over The Air (OTA) antenna performance; Conformance testing

This document describes the test procedure for the radiated performances measurements of the 3G/2G user equipment/mobile stations (UE/MS) in active mode in both the uplink and the downlink.

##### 5.1.2.5.4	TS 34.121-1

User Equipment (UE) conformance specification; Radio transmission and reception (FDD); Part 1: Conformance specification

This document specifies the UTRA measurement procedures for the conformance test of the user equipment (UE) that contain transmitting characteristics, receiving characteristics and performance requirements in addition to requirements for support of RRM (Radio Resource Management) in FDD mode. 
In addition it specifies conformance testing of RRM requirements for support of E-UTRA when the UE operates in UTRA mode.

NOTE: Conformance testing of RRM requirements for support of UTRA when the UE operates in E-UTRA mode are specified in TS 36.521-3.

The requirements are listed in different clauses only if the corresponding parameters deviate. More generally, tests are only applicable to those mobiles that are intended to support the appropriate functionality. To indicate the circumstances in which tests apply, this is noted in the "definition and applicability" part of the test.

For example only Release 5 and later UE declared to support HSDPA shall be tested for this functionality. In the event that for some tests different conditions apply for different releases, this is indicated within the text of the test itself.

##### 5.1.2.5.5	TS 34.121-2

User Equipment (UE) conformance specification; Radio transmission and reception (FDD); Part 2: Implementation Conformance Statement (ICS)

This document provides the Implementation Conformance Statement (ICS) proforma for 3rd Generation User Equipment (UE), in compliance with the relevant requirements, and in accordance with the relevant guidance given in ISO/IEC 9646-7 and ETS 300 406.

This document also specifies a recommended applicability statement for the test cases included in TS 34.121-1. These applicability statements are based on the features implemented in the UE.

Special conformance testing functions can be found in 3GPP TS 34.109 and the common test environments are included in 3GPP TS 34.108 and 3GPP TS 36.508.

This document is valid for UE implemented according to 3GPP releases starting from Release 99 up to the Release indicated on the cover page of this document.

##### 5.1.2.5.6	TS 34.123-1

User Equipment (UE) conformance specification; Part 1: Protocol conformance specification

This document specifies the protocol conformance testing for the 3rd Generation User Equipment (UE). This is the first part of a multi-part test specification.

##### 5.1.2.5.7	TS 34.123-2

User Equipment (UE) conformance specification; Part 2: Implementation conformance statement (ICS) proforma specification

This document provides the Implementation Conformance Statement (ICS) proforma for 3rd Generation User Equipment (UE), in compliance with the relevant requirements, and in accordance with the relevant guidance given in ISO/IEC 9646-7 and ETS 300 406. This document also specifies a recommended applicability statement for the test cases included in TS 34.123-1. These applicability statements are based on the features implemented

in the UE.

##### 5.1.2.5.8	TS 34.123-3

User Equipment (UE) conformance specification; Part 3: Abstract test suite (ATS)

This document specifies the protocol conformance testing in TTCN for the 3GPP User Equipment (UE) at the Uu interface.

This document is the 3rd part of a multi-part test specification, 3GPP TS 34.123. The following TTCN test specification and design considerations can be found in this document:

–   the overall test suite structure;

–   the testing architecture;

–   the test methods and PCO definitions;

–   the test configurations;

–   the design principles, assumptions, and used interfaces to the TTCN tester (System Simulator);

–   TTCN styles and conventions;

–   the partial PIXIT proforma;

–   the TTCN.MP and TTCN.GR forms for the mentioned protocols tests.

The Abstract Test Suites designed in the document are based on the test cases specified in prose (3GPP TS 34.123_1). This document is valid for UE implemented according to 3GPP releases starting from Release 1999 up to the Release indicated on the cover page of this document.

##### 5.1.2.5.9	TS 34.124

Electromagnetic compatibility (EMC) requirements for mobile terminals and ancillary equipment

This document establishes the essential EMC requirements for “3rd generation” digital cellular mobile terminal equipment and ancillary accessories in combination with a 3GPP user equipment (UE).

#### 5.1.2.6		36.200 series

##### 5.1.2.6.1	TS 36.201

Evolved Universal Terrestrial Radio Access (E-UTRA); LTE physical layer; General description

This specification describes an overview of the physical layer of the EUTRA radio interface.

##### 5.1.2.6.2	TS 36.211

Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation

This specification describes the physical channels for evolved UTRA.

##### 5.1.2.6.3	TS 36.212

Evolved Universal Terrestrial Radio Access (E-UTRA); Multiplexing and channel coding

This specification describes the coding, multiplexing and mapping to physical channels for E-UTRA.

##### 5.1.2.6.4	TS 36.213

Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures

This specification describes the characteristics of the physical layer procedures of E-UTRA.

##### 5.1.2.6.5	TS 36.214

Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer; Measurements

This specification describes the measurements done at the UE and network in order to support operation in idle mode and connected mode.

##### 5.1.2.6.6	TS 36.216

Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer for relaying operation

This document describes the characteristics of eNodeB – relay node transmissions.

#### 5.1.2.7		36.300 series

##### 5.1.2.7.1	TS 36.300

Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2

This specification describes an overview and overall description of the E-UTRA radio interface and E-UTRAN architecture and radio interface protocol architecture.

##### 5.1.2.7.2	TS 36.302

Evolved Universal Terrestrial Radio Access (E-UTRA); Services provided by the physical layer

This specification describes the services provided by the physical layer of E-UTRA to upper layers.

##### 5.1.2.7.3	TS 36.304

Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode

This specification describes the Access Stratum (AS) part of the Idle Mode procedures applicable to a UE and the model for the functional division between the NAS and AS in a UE.

##### 5.1.2.7.4	TS 36.306

Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio access capabilities

This specification defines the E-UTRA UE Radio Access Capability Parameters.

##### 5.1.2.7.5	TS 36.307

Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements on User Equipments (UEs) supporting a release-independent frequency band

This document specifies requirements on UEs supporting a frequency band and inter-band/intra-band CA configurations that are independent of release.

##### 5.1.2.7.6	TS 36.314

Evolved Universal Terrestrial Radio Access (E-UTRA); Layer 2 – Measurements

This document contains the description and definition of the measurements performed by E-UTRAN that are transferred over the standardised interfaces in order to support E-UTRA radio link operations, radio resource management (RRM), network operations and maintenance (OAM), and self-organising networks (SON).

##### 5.1.2.7.7	TS 36.321

Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification

This specification describes the E-UTRA MAC protocol.

##### 5.1.2.7.8	TS 36.322

Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Link Control (RLC) protocol specification

This specification describes the E-UTRA Radio Link Control (RLC) protocol.

##### 5.1.2.7.9	TS 36.323

Evolved Universal Terrestrial Radio Access (E-UTRA); Packet Data Convergence Protocol (PDCP) specification

This specification describes the Packet Data Convergence Protocol (PDCP).

##### 5.1.2.7.10	TS 36.331

Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification

This specification describes the Radio Resource Control protocol for the UE-eUTRAN radio interface.

##### 5.1.2.7.11	TS 36.305

Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Stage 2 functional specification of User Equipment (UE) positioning in 
E-UTRAN

This document specifies the stage 2 of the UE positioning function of E-UTRAN, which provides the mechanisms to support or assist the calculation of the geographical position of a UE. The purpose of this stage 2 specification is to define the E-UTRAN UE Positioning architecture, functional entities and operations to support positioning methods. This description is confined to the E-UTRAN Access Stratum. This stage 2 specification covers the E-UTRAN positioning methods, state descriptions, and message flows to support UE positioning.

##### 5.1.2.7.12	TS 36.355

Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol (LPP)

This document contains the definition of the LTE Positioning Protocol (LPP).

##### 5.1.2.7.13	TS 36.360

Evolved Universal Terrestrial Radio Access (E-UTRA); LTE-WLAN Aggregation Adaptation Protocol (LWAAP) specification

This document document specifies the E-UTRA LTE-WLAN Aggregation Adaptation Protocol (LWAAP).

##### 5.1.2.7.14	TS 36.361

Evolved Universal Terrestrial Radio Access (E-UTRA); LTE-WLAN Radio Level Integration Using Ipsec Tunnel (LWIP) encapsulation; Protocol specification

This document specifies the LWIP Encapsulation Protocol.

#### 5.1.2.8		36.400 series

##### 5.1.2.8.1	TS 36.401

Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Architecture description

This specification describes the overall architecture of the EUTRAN, including internal interfaces and assumptions on the radio, S1 and X2 interfaces.

##### 5.1.2.8.2	TS 36.410

Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 general aspects and principles

This specification provides an introduction to the 3GPP TS 36.41x series of technical specifications that define the S1 interface.

##### 5.1.2.8.3	TS 36.411

Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 layer 1

This specification describes the standards allowed to implement Layer 1 on the S1 interface.

##### 5.1.2.8.4	TS 36.412

Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 signalling transport

This specification describes the Signalling Transport to be used across S1 interface.

##### 5.1.2.8.5	TS 36.413

Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)

This specification describes the E-UTRAN radio network layer signalling protocol for the S1 interface.

##### 5.1.2.8.6	TS 36.414

Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 data transport

This specification describes the standards for user data transport protocols and related signalling protocols to establish user plane transport bearers over the S1 interface.

##### 5.1.2.8.7	TS 36.420

Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 general aspects and principles

This specification provides an introduction to the TSG RAN TS 36.42x series of UMTS Technical Specifications that define the X2 interface.

##### 5.1.2.8.8	TS 36.421

Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 layer 1

This specification describes the standards allowed to implement Layer 1 on the X2 interface.

##### 5.1.2.8.9	TS 36.422

Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 signalling transport

This specification describes the Signalling Transport to be used across X2 interface.

##### 5.1.2.8.10	TS 36.423

Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 Application Protocol (X2AP)

This specification describes the E-UTRAN radio network layer signalling protocol for the X2 interface.

##### 5.1.2.8.11	TS 36.424

Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 data transport

This specification describes the standards for user data transport protocols and related signalling protocols to establish user plane transport bearers over the X2 interface.

##### 5.1.2.8.12	TS 36.425

Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 interface user plane protocol

This document specifies the X2 user plane protocol being used over the X2 interface.

##### 5.1.2.8.13	TS 36.440

Evolved Universal Terrestrial Radio Access Network (E-UTRAN); General aspects and principles for interfaces supporting Multimedia Broadcast Multicast Service (MBMS) within E-UTRAN

This document describes the overall architecture of the interface for the provision of MBMS in the E-UTRAN. This includes also a description of the general aspects, assumptions and principles guiding the architecture and interface. The MBMS functions to be provided within that architecture are summarized. It provides an introduction to the TSG RAN TS 36.44x series of UMTS technical specifications that define the different interfaces introduced for MBMS provision in E-UTRAN.

##### 5.1.2.8.14	TS 36.441

Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Layer 1 for interfaces supporting Multimedia Broadcast Multicast Service (MBMS) within E-UTRAN

This document specifies the standards allowed to implement layer 1 on the interfaces supporting Multimedia Broadcast Multicast Service (MBMS) within E-UTRAN. In the following, “layer 1” and “physical layer” are assumed to be synonymous.

##### 5.1.2.8.15	TS 36.442

Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Signalling Transport for interfaces supporting Multimedia Broadcast Multicast Service (MBMS) within E-UTRAN

This document specifies the standards for signalling transport to be used across M2 and M3 interfaces. M2 interface is a logical interface between the eNodeB and the MCE. M3 interface is a logical interface between the MCE and the MME. This document describes how the M2-

AP signalling messages are transported over M2, and how the M3-AP signalling messages are transported over M3.

##### 5.1.2.8.16	TS 36.443

Evolved Universal Terrestrial Radio Access Network (E-UTRAN); M2 Application Protocol (M2AP)

This document specifies the E-UTRAN radio network layer signalling protocol for the M2 interface. The M2 Application Protocol (M2AP) supports the functions of M2 interface by signalling procedures defined in this document.

##### 5.1.2.8.17	TS 36.444

Evolved Universal Terrestrial Radio Access Network (E-UTRAN); M3 Application Protocol (M3AP)

This document specifies the E-UTRAN radio network layer signalling protocol for the M3 interface. The M3 Application Protocol (M3AP) supports the functions of M3 interface by signalling procedures defined in this document.

##### 5.1.2.8.18	TS 36.445

Evolved Universal Terrestrial Radio Access Network (E-UTRAN); M1 data transport

This document specifies the standards for user data transport protocols over the E-UTRAN M1 interface.

##### 5.1.2.8.19	TS 36.455

Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol A (LPPa)

This document specifies the control plane radio network layer signalling procedures between eNodeB and E-SMLC. LPPa supports the concerned functions by signalling procedures defined in this document.

##### 5.1.2.8.20	TS 36.456

SLm interface general aspects and principles

This document is an introduction to the 3GPP TS 36.45x series of technical specifications that define the SLm interface for the interconnection of the Evolved Serving Mobile Location Centre (E-SMLC) to the Location Measurement Unit (LMU) components of the Evolved Universal Terrestrial Radio Access Network (E-UTRAN).

##### 5.1.2.8.21	TS 36.457

SLm interface layer 1

This document specifies the standards allowed to implement layer 1 on the SLm interface.

##### 5.1.2.8.22	TS 36.458

SLm interface signalling transport

This document specifies the standards for signalling transport to be used across the SLm interface. The SLm interface is a logical interface between the LMU and the E-SMLC in the E-UTRAN core network. This document describes how the SLmAP signalling messages are transported over SLm.

##### 5.1.2.8.23	TS 36.459

SLm interface Application Protocol (SLmAP)

This document specifies the E-UTRAN radio network layer signalling protocol for the SLm interface. The SLm Application Protocol (SLmAP) supports the functions of the SLm interface by signalling procedures defined in this document.

##### 5.1.2.8.24	TS 36.461

Evolved Universal Terrestrial Radio Access Network (E-UTRAN) and Wireless LAN (WLAN); Xw layer 1

This document specifies the standards allowed to implement Layer 1 on the Xw interface.

The specification of transmission delay requirements and O&M requirements are not in the scope of this document.

In the following ‘Layer 1’ and ‘Physical Layer’ are assumed to be synonymous.

##### 5.1.2.8.25	TS 36.462

Evolved Universal Terrestrial Radio Access Network (E-UTRAN) and Wireless LAN (WLAN); Xw signalling transport

This document specifies the standards for Signalling Transport to be used across the Xw interface. The Xw interface is a logical interface between the eNB and the WT. This document describes how the Xw-AP signalling messages are transported over Xw.

##### 5.1.2.8.26	TS 36.463

Evolved Universal Terrestrial Radio Access Network (E-UTRAN) and Wireless Local Area Network (WLAN); Xw application protocol (XwAP)

This present document specifies the signalling procedures of the control plane between an eNB and WLAN Termination (WT). The Xw Application Protocol (XwAP) supports the functions of Xw interface by signalling procedures defined in this document.

##### 5.1.2.8.27	TS 36.464

Evolved Universal Terrestrial Radio Access Network (E-UTRAN) and Wireless Local Area Network (WLAN); Xw data transport

This document specifies the standards for user data transport protocols and related signalling protocols to establish user plane transport bearers over the Xw interface for LTE/WLAN Aggregation (LWA) and enhanced LTE WLAN Radio Level Integration with IPsec Tunnel (eLWIP).

##### 5.1.2.8.28	TS 36.465

Evolved Universal Terrestrial Radio Access Network (E-UTRAN) and Wireless Local Area Network (WLAN); Xw interface user plane protocol

This present document specifies the Xw user plane protocol being used over the Xw interface for LTE/WLAN Aggregation (LWA) and LTE WLAN Radio Level Integration with IPsec Tunnel (LWIP).

#### 5.1.2.9	36.100 series

##### 5.1.2.9.1	TS 36.101

Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception

This specification describes the User Equipment (UE) minimum RF characteristics of E-UTRA for both FDD and TDD modes.

##### 5.1.2.9.2	TS 36.104

Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception

This specification describes the Base Station minimum RF characteristics of E-UTRA in paired and unpaired bands.

##### 5.1.2.9.3	TS 36.106

Evolved Universal Terrestrial Radio Access (E-UTRA); FDD repeater radio transmission and reception

This document establishes the minimum RF characteristics of E-UTRA FDD Repeater.

##### 5.1.2.9.4	TS 36.111

Location Measurement Unit (LMU) performance specification; Network based positioning systems in Evolved Universal Terrestrial Radio Access Network (E-UTRAN)

This document establishes the Location Measurement Unit (LMU) minimum UTDOA positioning requirement for the FDD and TDD mode of E-UTRAN.

##### 5.1.2.9.5	TS 36.112

Location Measurement Unit (LMU) conformance specification; Network based positioning systems in Evolved Universal Terrestrial Radio Access Network (E-UTRAN)

Location Measurement Unit (LMU) conformance specification; Network based positioning systems in Evolved Universal Terrestrial Radio Access Network (E-UTRAN).

##### 5.1.2.9.6	TS 36.113

Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) and repeater ElectroMagnetic Compatibility (EMC)

This document covers the assessment of E-UTRA base stations, repeaters and associated ancillary equipment in respect of Electromagnetic Compatibility (EMC).

##### 5.1.2.9.7	TS 36.116

Evolved Universal Terrestrial Radio Access (E-UTRA); Relay radio transmission and reception

This document establishes the minimum RF characteristics and minimum performance requirements of E-UTRA Relay

Evolved Universal Terrestrial Radio Access (E-UTRA); Relay radio transmission and reception

This document establishes the minimum RF characteristics and minimum performance requirements of E-UTRA Relay

##### 5.1.2.9.8	TS 36.117

Evolved Universal Terrestrial Radio Access (E-UTRA); Relay conformance testing

This document specifies the Radio Frequency (RF) test methods and conformance requirements for E-UTRA Relay. These have been derived from, and are consistent with the E-UTRA Relay specifications defined in TS 36.116.

##### 5.1.2.9.9	TS 36.124

Evolved Universal Terrestrial Radio Access (E-UTRA); Electromagnetic compatibility (EMC) requirements for mobile terminals and ancillary equipment

This document establishes the essential EMC requirements for “3rd generation” digital cellular mobile terminal equipment and ancillary accessories in combination with a 3GPP E-UTRA user equipment (UE).

##### 5.1.2.9.10	TS 36.133

Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management

This specification specifies requirements for support of Radio Resource Management for the FDD and TDD modes of Evolved UTRA. These requirements include requirements on measurements in UTRAN and the UE as well as requirements on node dynamical behaviour and interaction, in terms of delay and response characteristics.

##### 5.1.2.9.11	TS 36.141

Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) conformance testing

This document specifies the Radio Frequency (RF) test methods and conformance requirements for E-UTRA Base Stations (BS).

##### 5.1.2.9.12	TS 36.143

Evolved Universal Terrestrial Radio Access (E-UTRA); FDD repeater conformance testing

This document specifies the Radio Frequency (RF) test methods and Minimum Requirements for UTRA FDD Repeaters.

##### 5.1.2.9.13	TS 36.171

Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for Support of Assisted Global Navigation Satellite System
(A-GNSS)

This document establishes the minimum performance requirements for A-GNSS (including A-GPS) for FDD or TDD mode of E-UTRA for the User Equipment (UE).

#### 5.1.2.10	36.500 series

##### 5.1.2.10.1	TS 36.508

Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); Common test environments for User Equipment (UE) conformance testing

This document contains definitions of reference conditions and test signals, default parameters, reference radio bearer configurations used in radio bearer interoperability testing, common radio bearer configurations for other test purposes, common requirements for test equipment and generic set-up procedures for use in conformance tests for the 3rd Generation E-UTRAN User Equipment (UE).

##### 5.1.2.10.2	TS 36.509

Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); Special conformance testing functions for User Equipment (UE)

This document defines for User Equipment (UE) those special functions and their activation methods that are required in User Equipment (UE) for conformance testing purposes.

##### 5.1.2.10.3	TS 36.521-1

Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) conformance specification; Radio transmission and reception;
Part 1: Conformance testing

This document specifies the measurement procedures for the conformance test of the user equipment (UE) that contain transmitting characteristics, receiving characteristics and performance requirements.

5.1.2.10.4	TS 36.521-2

Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) conformance specification; Radio transmission and reception;

Part 2: Implementation Conformance Statement (ICS)

This document provides the Implementation Conformance Statement (ICS) proforma for 3G Evolved Universal Terrestrial Radio Access (E-UTRA) User Equipment (UE), in compliance with the relevant requirements, and in accordance with the relevant guidance given in ISO/IEC 9646-1 and 
ISO/IEC 9646-7.

##### 5.1.2.10.5	TS 36.521-3

Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) conformance specification; Radio transmission and reception; 
Part 3: Radio Resource Management (RRM) conformance testing

This document specifies the measurement procedures for the conformance test of the user equipment (UE) that contain requirements for support of RRM (Radio Resource Management) as part of the 3G Long Term Evolution (3G LTE).

The requirements are listed in different clauses only if the corresponding parameters deviate. More generally, tests are only applicable to those mobiles that are intended to support the appropriate functionality. To indicate the circumstances in which tests apply, this is noted in the “test  applicability” part of the test.

For example only Release 8 and later UE declared to support LTE shall be tested for this functionality. In the event that for some tests different conditions apply for different releases, this is indicated within the text of the test itself.

##### 5.1.2.10.6	TS 36.523-1

Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); User Equipment (UE) conformance specification; Part 1: Protocol conformance specification

This document specifies the protocol conformance testing for the 3rd Generation E-UTRAN User Equipment (UE).

##### 5.1.2.10.7	TS 36.523-2

Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); User Equipment (UE) conformance specification; Part 2: Implementation Conformance Statement (ICS) proforma specification

This document provides the Implementation Conformance Statement (ICS) proforma for 3rd Generation User Equipment (UE), in compliance with the relevant EPS (E-UTRA/EPC) requirements, and in accordance with the relevant guidance given in ISO/IEC 9646-1 and ISO/IEC 9646-7.

##### 5.1.2.10.8	TS 36.523-3

Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); User Equipment (UE) conformance specification; Part 3: Test suites

This document specifies the protocol and signalling conformance testing in TTCN-3 for the 3GPP UE at the UE_E-UTRAN radio interface.

The following TTCN test specification and design considerations can be found in this document:

–   the test system architecture;

–   the overall test suite structure;

–   the test models and ASP definitions;

–   the test methods and usage of communication ports definitions;

–   the test configurations;

–   the design principles and assumptions;

–   TTCN styles and conventions;

–   the partial PIXIT proforma;

–   the test suites.

The Abstract Test Suites designed in the document are based on the test cases specified in prose (3GPP TS 36.523_1). The applicability of the individual test cases is specified in the test ICS proforma specification (3GPP TS 36.523_2). This document is valid for UE implemented according to 3GPP Rel-9 upwards.

#### 5.1.2.11	Multiple radio access technology aspects specifications

##### 5.1.2.11.1	TS 37.104

E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) radio transmission and reception

This specification establishes the minimum RF characteristics of E-UTRA, UTRA and GSM/EDGE Multi-Standard Radio (MSR) Base Station (BS). Requirements for multi-RAT and single-RAT operation of MSR BS are covered in this document. The requirements in this document for E_UTRA and UTRA single-RAT operation of MSR BS are also applicable to E-UTRA and UTRA multi-carrier capable single-RAT BS. Requirements for GSM BS that are only single-RAT capable are not covered.

##### 5.1.2.11.2	TS 37.105

Active Antenna System (AAS) Base Station (BS) transmission and reception

This document establishes the RF characteristics, the RF minimum requirements and minimum performance requirements for E-UTRA AAS Base Station (BS), the FDD mode of UTRA AAS Base Station (BS), the 1,28 Mcps TDD mode of UTRA AAS Base Station (BS) in single RAT and any MSR AAS Base Station (BS) implementation of these RATs.

This document does not establish minimum RF characteristics or minimum performance requirements for Narrow-Band Internet of Things (NB-IoT) in band, NB-IoT guard band, or standalone NB-IoT operation, for AAS BS in single RAT E-UTRA operation or in MSR operation using E-UTRA.

##### 5.1.2.11.3	TS 37.113

E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) Electromagnetic Compatibility (EMC)

This document covers the assessment of E-UTRA, UTRA and GSM/EDGE Multi-Standard Radio (MSR) Base Stations and associated ancillary equipment in respect of Electromagnetic Compatibility (EMC). This document specifies the applicable test conditions, performance assessment and performance criteria for E-UTRA, UTRA and GSM/EDGE Base Stations and associated ancillary equipment in one of the following categories:

(i) Multi-Standard Radio (MSR) Base Stations for E-UTRA, UTRA and GSM/EDGE meeting the requirements of TS 37.104, with conformance demonstrated by compliance to TS 37.141;

(ii) Base Stations for E-UTRA meeting the requirements of TS 36.104, with conformance demonstrated by compliance to TS 36.141;

(iii) Base Stations for UTRA FDD meeting the requirements of TS 25.104, with conformance demonstrated by compliance to TS 25.141;

(iv) Base Stations for UTRA TDD meeting the requirements of TS 25.105, with conformance demonstrated by compliance to TS 25.142;

(v) Base Stations for GSM/EDGE meeting the requirements of TS 45.005, with conformance demonstrated by compliance to TS 51.021.

The environment classification used in this document refers to the environment classification used in IEC 61000-6-1 and IEC 61000-6-3. The EMC requirements have been selected to ensure an adequate level of compatibility for apparatus at residential, commercial and light industrial environments.

The levels, however, do not cover extreme cases which may occur in any location but with low probability of occurrence.

##### 5.1.2.11.4	TS 37.114

Active Antenna System (AAS) Base Station (BS) Electromagnetic Compatibility (EMC)

This document covers the assessment of UTRA TDD, UTRA FDD, E-UTRA and Multi-Standard Radio (MSR) Active Antenna Systems Base Stations in respect of Electromagnetic Compatibility (EMC).

NOTE:	Whenever the UTRA AAS BS is referred in the following section, UTRA TDD and UTRA FDD shall be considered.

This document specifies the applicable test conditions, performance assessment and performance criteria for base stations and associated ancillary equipment in the following categories:

-	Active Antenna System Base Station for UTRA, E-UTRA and MSR meeting the conducted requirements of 3GPP TS 37.105, with conformance demonstrated by compliance to 3GPP TS 37.145-1,

-	Active Antenna System Base Station for E-UTRA, UTRA and MSR meeting the OTA requirements of 3GPP TS 37.105, with conformance demonstrated by compliance to 3GPP TS 37.145-2.

This document does not specify test conditions, performance assessment and performance criteria for the Narrow-Band Internet of Things (NB-IoT) in band, NB-IoT guard band, or standalone NB-IoT operation, for AAS BS in single RAT E-UTRA operation as defined in 3GPP TS 36.113, or for AAS BS in MSR operation using E-UTRA as defined in 3GPP TS 37.113.

This document does not specify test conditions, performance assessment and performance criteria for Band 46 operation as it is not supported by AAS BS.

The scope of this document is twofold:

-	Requirement, procedures and values of an AAS BS with TAB connectors for every transceiver unit at the transceiver array boundary (TAB) , subject to conducted requirements,

-	Requirements, procedures and values of an AAS Base Station without TAB connectors are not covered by the Rel-13 version of this document and are FFS.

The environment classification used in this document refers to the residential, commercial and light industrial environment classification used in IEC 61000-6-1 and IEC 61000-6-3.

The EMC requirements have been selected to ensure an adequate level of compatibility for apparatus at residential, commercial and light industrial environments. The levels, however, do not cover extreme cases which may occur in any location but with low probability of occurrence.

##### 5.1.2.11.5	TS 37.141

E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) conformance testing

This document specifies the Radio Frequency (RF) test methods and conformance requirements for E-UTRA, UTRA and GSM/EDGE Multi-

Standard Radio (MSR) Base Station (BS).

##### 5.1.2.11.6	TS 37.144

User Equipment (UE) and Mobile Station (MS) GSM, UTRA and E-UTRA over the air performance requirements

This document establishes over the air antenna minimum requirements for user equipment (UE) and mobile station (MS).

Handheld UE requirements are defined for roaming bands for the speech position (beside the head and beside the head and hand) and hand phantom browsing mode position. Laptop mounted equipment requirements are defined for roaming bands for the data transfer position (laptop ground plane phantom). Laptop embedded equipment requirements are defined for roaming bands for the data transfer position (free space).

All bands are potential roaming bands, and the requirements for roaming bands shall therefore be fulfilled for all bands supported by a UE/MS.

Requirements for operating bands are dependent on how the network has been built and are thus operator specific and cannot be specified here. Recommended performance values for operating bands (Annex B) are however included in this specification for information. It should be recognised that the ability to meet the recommended performance values depends on the number of frequency bands supported by the UE/MS.

##### 5.1.2.11.7	TS 37.145-1

Active Antenna System (AAS) Base Station (BS) conformance testing; Part 1: conducted conformance testing

This document specifies the conducted test methods and conformance requirements for single RAT E-UTRA operation, single RAT UTRA operation (FDD and TDD) and Multi-Standard Radio (MSR) UTRA and E-UTRA Active Antenna System (AAS) Base Station (BS). These have been derived from, and are consistent with the non-AAS BS specifications in 3GPP TS 25.104, 3GPP TS 25.105, 3GPP TS 36.104 or 3GPP TS 37.104. The technical specification 3GPP TS 37.145 is in 2 parts, part 3GPP TS 37.145-1  (this document) covers conducted requirements and part 3GPP TS 37.145-2 covers radiated requirements.

This document does not establish conducted test methods and conformance requirements for Band 46 operation as it is not supported by AAS BS, but Band 46 test requirements are still applicable for AAS BS for protection of and against Band 46 operation.

This document does not establish conducted test methods and conformance requirements for Narrow-Band Internet of Things (NB-IoT) in band, NB-IoT guard band, or standalone NB-IoT operation, for AAS BS in single RAT E-UTRA operation as defined in 3GPP TS 36.141, or for AAS BS in MSR operation using E-UTRA as defined in 3GPP TS 37.141

##### 5.1.2.11.8	TS 37.145-2

Active Antenna System (AAS) Base Station (BS) conformance testing; Part 2: radiated conformance testing

This document specifies radiated test methods and conformance requirements for single RAT E-UTRA operation, single RAT UTRA operation (FDD and TDD) and Multi-Standard Radio (MSR) UTRA and E-UTRA Active Antenna System (AAS) Base Station (BS). These have been derived from, and are consistent with non-AAS BS specifications in 3GPP TS 25.104, 3GPP TS 25.105, 3GPP TS 36.104 or 3GPP TS 37.104. The technical specification 3GPP TS 37.145 is in 2 parts, part 3GPP TS 37.145-1 covers conducted requirements and part 3GPP TS 37.145-2 (this document) covers radiated requirements.

This document does not establish radiated test methods and conformance requirements for Band 46 operation as it is not supported by AAS BS. Conducted Band 46 test requirements are still applicable for AAS BS for protection of and against Band 46 operation, as specified in 3GPP TS 37.145-1.

##### 5.1.2.11.9	TS 37.171

Universal Terrestrial Radio Access (UTRA) and Evolved UTRA (E-UTRA); User Equipment (UE) performance requirements for RAT-Independent positioning enhancements

This document establishes the minimum performance requirements for RAT-Independent Positioning Enhancements (e.g. MBS positioning technology) for FDD or TDD mode of UTRA and E-UTRA for the User Equipment (UE).

##### 5.1.2.11.10	TS 37.320

Universal Terrestrial Radio Access (UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRA); Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; Stage 2

This document provides an overview and overall description of the minimization of drive tests functionality. The document describes functions and procedures to support collection of UE-specific measurements for MDT using Control Plane architecture, for both UTRAN and E-UTRAN

##### 5.1.2.11.11	TS 37.544

Universal Terrestrial Radio Access (UTRA) and Evolved UTRA (E-UTRA); User Equipment (UE) Over The Air (OTA) performance; Conformance testing

This document describes the test procedure for the radiated performances measurements of the user equipment (UE).

The handheld UE measurement procedures explained in this document are defined for roaming bands for the speech mode position (beside the head and beside the head and hand) and hand phantom browsing mode position. Laptop mounted equipment (LME) measurement procedures are defined for roaming bands for the data mode transfer position (laptop ground plane phantom). Laptop embedded equipment (LEE) measurement procedures are defined for roaming bands for the data mode transfer position (free space).

The browsing mode position explained in this document applies when the UE is held in hand, but not pressed against ear (such as web browsing and navigation).

The data mode transfer position (free space) explained in this document applies when the UE is used away from the user’s head. Free space measurements are applicable to devices used in the data mode transfer position that consist of the laptop mounted equipment (LME) plug-in UEs and laptop embedded equipment (LEE) UEs.

All bands are potential roaming bands, and the requirements for roaming bands shall therefore be fulfilled for all bands supported by a UE.

Radiated performance of multiple-antenna receivers for handheld UE is defined for roaming bands in free space configuration.

Recommended performance values for operating bands (Annex I) are however included in this specification for information. It should be recognised that the ability to meet the recommended performance values depends on the number of frequency bands supported by the UE.

The radiated radio tests considered here are:

1. The measurement of the Total Radiated Power (TRP)

2. The measurement of the Total Radiated Sensitivity (TRS)

3. The measurement of Total Radiated Multi-antenna Sensitivity (TRMS)

The test procedure described in this document measures the performance of the transmitter and the receiver, including the antenna and also the effects of the user.

##### 5.1.2.11.12	TS 37.571-1

Universal Terrestrial Radio Access (UTRA) and Evolved UTRA (E-UTRA) and Evolved Packet Core (EPC); User Equipment (UE) conformance specification for UE positioning; Part 1: Conformance test specification

This document specifies the procedures for the conformance test of the measurement requirements for FDD mode of UTRA and FDD or TDD mode of 
E-UTRA for the User Equipment (UE) that supports one or more of the defined positioning methods. These positioning methods are for UTRA: Assisted Global Positioning System (A-GPS), Assisted Global Navigation Satellite Systems (A-GNSS) and for E-UTRA: Assisted Global Navigation Satellite System (A-GNSS), Observed Time Difference of Arrival (OTDOA), Enhanced Cell ID (ECID).

Tests are only applicable to those mobiles that are intended to support the appropriate functionality. To indicate the circumstances in which tests apply, this is noted in the “Test applicability " part of the test.

The Implementation Conformance Statement (ICS) pro-forma could be found in the 3rd part of this document.

##### 5.1.2.11.13	TS 37.571-2

Universal Terrestrial Radio Access (UTRA) and Evolved UTRA (E-UTRA) and Evolved Packet Core (EPC); User Equipment (UE) conformance specification for UE positioning; Part 2: Protocol conformance

This document specifies the protocol conformance testing for the 3rd Generation UTRAN and E-UTRAN User Equipment (UE) supporting UE positioning.

This is the second part of a multi-part test specification. The following information can be found in this part:

–   the overall protocol conformance test structure;

–   the protocol conformance test configurations;

–   the conformance requirement and reference to the core specifications;

–   the test purposes; and

–   a brief description of the test procedure, the specific test requirements and short message exchange table. The Implementation Conformance Statement (ICS) pro-forma could be found in the 3rd part of this document.

This document is valid for UE supporting UE positioning implemented according to 3GPP releases starting from Release 99 up to the Release indicated on the cover page of this document.

##### 5.1.2.11.14	TS 37.571-3

Universal Terrestrial Radio Access (UTRA) and Evolved UTRA (E-UTRA) and Evolved Packet Core (EPC); User Equipment (UE) conformance specification for UE positioning; Part 3: Implementation Conformance Statement (ICS)

This document provides the Implementation Conformance Statement (ICS) proforma for 3rd Generation UTRAN and E-UTRAN User Equipment (UE) supporting UE positioning, in compliance with the relevant requirements, and in accordance with the relevant guidance given in ISO/IEC 9646-1 and ISO/IEC 9646-7.

This document also specifies a recommended applicability statement for the test cases included in 3GPP TS 37.571-1 and 3GPP TS 37.571-2. These applicability statements are based on the features implemented in the UE. Special conformance testing functions can be found in 3GPP TS 34.109 for UTRA and 3GPP TS 36.509 for E-UTRA.

The common test environments are included in 3GPP TS 34.108 for UTRA and in 3GPP TS 36.508 for E-UTRA.

This document is valid for UE supporting UE positioning implemented according to 3GPP releases starting from Release 99 up to the Release indicated on the cover page of this document.

##### 5.1.2.11.15	TS 37.571-4

Universal Terrestrial Radio Access (UTRA) and Evolved UTRA (E-UTRA) and Evolved Packet Core (EPC); User Equipment (UE) conformance specification for UE positioning; Part 4: Test suites

This document specifies the protocol and signalling conformance testing in TTCN for the UE:

–   A-GPS at the UTRA Uu interface;

–   LTE positioning at the LTE-Uu interface;

–   A-GNSS at the UTRA Uu interface.

The following TTCN test specification and design considerations can be found in this document:

–   Test system architecture;

–   Test models and ASP definitions;

–   Test methods and usage of communication ports definitions;

–   Test configurations;

–   Design principles and assumptions;

–   TTCN styles and conventions;

–   Partial PIXIT proforma;

–   Test suites in TTCN-2 and TTCN-3;

–   The Test Suites designed and implemented in this document are based on the test specifications in prose in 3GPP TS 37.571_2;

–   The applicability of the individual test cases is specified in the test ICS proforma specification in 3GPP TS 37.571_3.

##### 5.1.2.11.16	TS 37.571-5

Universal Terrestrial Radio Access (UTRA) and Evolved UTRA (E-UTRA) and Evolved Packet Core (EPC); User Equipment (UE) conformance specification for UE positioning; Part 5: Test scenarios and assistance data

This document specifies the test scenarios and assistance data required for the conformance test for FDD or TDD mode of UTRA and E-UTRA for the User Equipment (UE) that supports one or more of the defined positioning methods. For UTRA these are Assisted Global Positioning System (A-GPS) and Assisted Global Navigation Satellite System (A-GNSS). For E-UTRA these are A-GNSS, Observed Time Difference of Arrival (OTDOA) and Enhanced Cell ID (ECID).

#### 5.1.2.12	Core network, Terminal and System aspects

IMT-2000 CDMA DS specification also includes the following documents which are useful and related to this Recommendation.

#### 5.1.2.12.1	Requirements

##### 5.1.2.12.1.1	TS 21.101

Technical Specifications and Technical Reports for a UTRAN-based 3GPP system

The listed Specifications are required to build a system based on UTRAN radio technology.

##### 5.1.2.12.1.2	TS 21.111

USIM and IC card requirements

This document defines the requirements of the USIM (Universal Subscriber Identity Module) and the IC card for 3G (UICC). These are derived from the service and security requirements defined in TS 22.100 and TS 22.101. The USIM is a 3G application on an IC card. It inter-operates with a 3G terminal and provides access to 3G services. This document is intended to serve as a basis for the detailed specification of the USIM and the UICC, and the interface to the 3G terminal.

##### 5.1.2. 12.1.3	TS 21.201

Technical Specifications and Technical Reports for an Evolved Packet System (EPS) based 3GPP system

This document identifies the 3GPP Technical Specifications and Technical Reports required or potentially required to build a system based on the Evolved Packet System/LTE/E UTRAN radio technology.

##### 5.1.2. 12.1.4	TS 21.202

Technical Specifications and Technical Reports relating to the Common IP Multimedia Subsystem (IMS)

This document identifies the 3GPP Technical Specifications and Technical Reports specifically relating to the Common IP Multimedia Subsystem (IMS) maintained by 3GPP. Standards organizations adopting the Common IP Multimedia Subsystem (IMS) might not need to use all listed specifications.

#### 5.1.2.12.2	Service aspects ("stage 1")

##### 5.1.2.12.2.1	TS 22.001

Principles of circuit telecommunication services supported by a Public Land Mobile Network (PLMN)

This document covers the definition of the circuit telecommunication services supported by a PLMN. The purpose of this document is to provide a method for the characterization and the description of these telecommunication services.

##### 5.1.2.12.2.2	TS 22.002

Circuit Bearer Services (BS) supported by a Public Land Mobile Network (PLMN)

This 3G specification describes a set of bearer services to be provided to 3G subscribers by a 3G network itself and in connection with other networks. This document is also be used as a reference for defining the corresponding required mobile network capabilities which are specified by means of the connection type concept.

##### 5.1.2.12.2.3	TS 22.003

Circuit Teleservices supported by a Public Land Mobile Network (PLMN)

This document describes and defines a recommended set of Circuit Teleservices to be supported by a PLMN in connection with other networks as a basis for defining the network capabilities required.

##### 5.1.2.12.2.4	TS 22.004

General on supplementary services

This specification describes a recommended set of supplementary services to the teleservices and bearer services which will be supported by a 3G network in connection with other networks as a basis for the definition of the network capabilities required.

##### 5.1.2.12.2.5	TS 22.011

Service accessibility

This specification describes the service access procedures as presented to the user. The document contains definitions and procedures are provided for international roaming, national roaming and regionally provided service. These are mandatory in relation to the technical realization of the UE.

##### 5.1.2.12.2.6	TS 22.016

International Mobile station Equipment Identities (IMEI)

This specification describes the principal purpose and use of unique equipment identities.

##### 5.1.2.12.2.7	TS 22.022

Personalization of Mobile Equipment (ME); Mobile functionality specification

This specification describes functional specifications of five features to personalize UE. These features are called: – network personalization; – network subset personalization; – service provider (SP) personalization; – corporate personalization; – UMTS subscriber identity module (USIM) personalization. This specification describes requirements for UE, which provide these personalization features.

##### 5.1.2.12.2.8	TS 22.034

High Speed Circuit Switched Data (HSCSD); Stage 1

This specification describes the Stage 1 description of HSCSD. HSCSD is a feature that allows users subscribing to the general bearer services to access user rates that can be achieved with one or more traffic channel. HSCSD also defines a flexible use of air interface resources, which makes efficient and flexible use of higher user rates feasible.

##### 5.1.2.12.2.9	TS 22.038

(U)SIM Application Toolkit (USAT); Service description; Stage 1

This specification describes the Stage 1 description of the SAT primarily from the subscribers and serving environments points of view, and does not deal with the details of the human interface itself. It includes information applicable to network operators, serving environments and terminal, switch and database manufacturers and contains the core requirements for a SAT which are sufficient to provide a complete service.

##### 5.1.2.12.2.10	TS 22.041

Operator Determined Barring (ODB)

This document specifies the network feature Operator Determined Barring (ODB), which allows the network operator or service provider to regulate, by means of an exceptional procedure, access by the subscribers to services (both Circuit and Packet Oriented), by the barring of certain categories of outgoing or incoming calls/Packet Oriented Services or of roaming.

##### 5.1.2.12.2.11	TS 22.057

Mobile Execution Environment (MExE); Service description; Stage 1

This document defines the stage one description of the Mobile Execution Environment (MExE). The scope of this 3GPP TS encompasses the MExE functionality in the UE, interaction with the MExE service environment. The MExE service environment is not necessarily restricted to the PLMN, and nodes providing MExE services (i.e. MExE servers) may also exist outside the PLMN. Aspects of the support provided by MExE servers within the MExE service environment (such as charging aspects, security level classification etc.) are covered by this specification, but not the MExE servers themselves.

##### 5.1.2.12.2.12	TS 22.060

General Packet Radio Service (GPRS); Service description; Stage 1

This specification describes the Stage 1 description of the GPRS.

##### 5.1.2.12.2.13	TS 22.067

Enhanced Multi Level Precedence and Pre-emption service (eMLPP); Stage 1

This specification describes the Stage 1 description of the enhanced multi-level precedence and pre-emption (eMLPP) service. This service has two parts: precedence and pre-emption. Precedence involves assigning a priority level to a call in combination with fast call set-up. Pre-emption involves the seizing of resources, which are in use by a call of a lower precedence, by a higher level precedence call in the absence of idle resources. Pre-emption can also involve the disconnection of an on-going call of lower precedence to accept an incoming call of higher precedence.

##### 5.1.2.12.2.14	TS 22.071

Location Services (LCS); Service description; Stage 1

LCS is a network provided enabling technology consisting of standardized service capabilities which enables the provision of location applications. 
This application may be service provider specific. The description of the numerous and varied possible location applications which are enabled by this technology are outside the scope of this specification. However, clarifying examples of how the functionality being specified may be used to provide specific LCS is included in various sections of the specification.

##### 5.1.2.12.2.15	TS 22.078

Customized Applications for Mobile network Enhanced Logic (CAMEL); Service description; Stage 1

This specification describes the Stage 1 description for CAMEL feature which provides the mechanisms to support services consistently independently of the serving network. The CAMEL features shall facilitate service control of operator specific services external from the serving network. The CAMEL feature is a network feature and not a supplementary service. It is a tool to help the network operator to provide the subscribers with the operator specific services even when roaming outside the home network.

##### 5.1.2.12.2.16	TS 22.081

Line Identification supplementary services; Stage 1

This document specifies the supplementary services belonging to the group Line Identification Supplementary Services, which consist of Calling line identification presentation (CLIP), Calling line identification restriction (CLIR), Connected line identification presentation (COLP), and Connected line identification restriction (COLR).

##### 5.1.2.12.2.17	TS 22.082

Call Forwarding (CF) Supplementary Services; Stage 1

This document specifies the supplementary services belonging to the group Call Offering Supplementary Services, which consist of Call forwarding unconditional, Call forwarding on mobile subscriber busy, Call forwarding on no reply, and Call forwarding on mobile subscriber not reachable.

##### 5.1.2.12.2.18	TS 22.083

Call Waiting (CW) and Call Holding (HOLD); Supplementary Services; Stage 1

This document specifies the supplementary services belonging to the group Call Completion Supplementary Services, which consist of Call waiting and Call hold.

##### 5.1.2.12.2.19	TS 22.084

MultiParty (MPTY) supplementary service; Stage 1

This document specifies the supplementary services belonging to the group MultiParty Supplementary Services, which consist of MultiParty service.

##### 5.1.2.12.2.20	TS 22.085

Closed User Group (CUG) supplementary services; Stage 1

This document specifies the supplementary services belonging to the group Community Of Interest Supplementary Services, which consist of Closed user group.

##### 5.1.2.12.2.21	TS 22.086

Advice of Charge (AoC) supplementary services; Stage 1

This document specifies the supplementary services belonging to the group Charging Supplementary Services, which consist of Advice of Charge (Information) and Advice of Charge (Charging).

##### 5.1.2.12.2.22	TS 22.087

User-to-User Signalling (UUS); Service description; Stage 1

This document specifies the User-to-User Signalling (UUS) supplementary service, which allows a mobile subscriber to send/receive a limited amount of information to/from another PLMN or ISDN subscriber over the signalling channel in association with a call to the other subscriber.

##### 5.1.2.12.2.23	TS 22.088

Call Barring (CB) supplementary services; Stage 1

This document specifies the supplementary services belonging to the group Call Restriction Supplementary Services, which consist of barring of outgoing calls and barring of incoming calls.

##### 5.1.2.12.2.24	TS 22.090

Unstructured Supplementary Service Data (USSD); Stage 1

There are two modes of USSD: MMI-mode and application mode. MMI-mode USSD is for the transparent transport of MMI strings entered by the user to the network and for the transparent transport of text strings from the network that are displayed by the mobile for user information. Application mode USSD is for the transparent transport of data between the network and the mobile station. Application mode USSD is intended to be used by applications in the network and their peer applications in the UE. The communication over the radio interface takes place on the signalling channels using short dialogues with peak data throughput rate capabilities of up to approximately 600 bit/s outside of a call and 1 000 bit/s during a call.

##### 5.1.2.12.2.25	TS 22.091

Explicit Call Transfer (ECT) supplementary service; Stage 1

This document specifies the stage 1 description of Explicit Call Transfer (ECT) from the service subscriber's and user's points of view, in particular, the procedures for normal operation with successful outcome, the action to be taken in exceptional circumstances, and the interaction with other supplementary services.

##### 5.1.2.12.2.26	TS 22.093

Completion of Calls to Busy Subscriber (CCBS); Service description, Stage 1

This document specifies the stage 1 description of Completion of Calls to Busy Subscriber (CCBS) from the subscriber's and user's points of view, in particular, the procedures for normal operation with successful outcome, the action to be taken in exceptional circumstances, and the interaction with other supplementary services.

##### 5.1.2.12.2.27	TS 22.094

Follow Me service description; Stage 1

This document specifies the stage 1 description for the Follow Me feature, which enables a mobile subscriber A to manipulate the Follow Me data of a party B in such a way that - under certain conditions - subsequent calls directed to party B will be forwarded to subscriber A.

##### 5.1.2.12.2.28	TS 22.096

Name identification supplementary services; Stage 1

This document specifies the supplementary services belonging to the group Name Identification supplementary services, which consist of Calling Name Presentation (CNAP).

##### 5.1.2.12.2.29	TS 22.101

Service aspects; Service principles

This specification describes the service principles of the UMTS.

##### 5.1.2.12.2.30	TS 22.105

Services and service capabilities

Pre-UMTS systems have largely standardized the complete sets of bearer services, teleservices and supplementary services which they provide. One major difference between UMTS and pre-UMTS systems is that service capabilities rather than services are standardized for UMTS, allowing service differentiation and system continuity. This document describes how and what kind of services the UMTS user has access to.

##### 5.1.2.12.2.31	TS 22.115

Service aspects; Charging and billing

This specification describes the service aspects of charging and billing of the UMTS. This standard is not intended to duplicate existing standards or standards being developed by other groups on these topics, and will reference these where appropriate. This standard will elaborate on the charging requirements described in the charging principles in TS 22.101 UMTS service principles. It will allow the generation of accurate charging information to be used in the commercial and contractual relationships between the parties concerned.

##### 5.1.2.12.2.32	TS 22.129

Service aspects; Handover requirements between UTRAN and GERAN or other radio systems

This specification describes service requirements for handover (terms are defined below) within UMTS systems and between UMTS, other IMT_2000 family members and second generation systems. Particular emphasis has been placed on the description of requirements for handover between UMTS and GSM but requirements specific to other systems are incorporated as required.

##### 5.1.2.12.2.33	TS 22.135

Multicall; Service description; Stage 1

This specification describes multicall scenarios and requirements for UMTS phase 1 release 1999. Multicall feature specifies functionality and interactions related to usage of several simultaneous bearers between a terminal and a network. Multicall features allows both circuit-switched call(s) and packet session(s) to exist simultaneously.

##### 5.1.2.12.2.34	TS 22.141

Presence Service; Stage 1

This specification defines the requirements for the support of the presence service. The presence service results in presence information of a user and information on a user's devices, services and services components being managed by the network.

##### 5.1.2.12.2.35	TS 22.146

Multimedia Broadcast/Multicast Service (MBMS); Stage 1

The document describes MBMS User Services that use the capabilities of MBMS. Application scenarios including charging, QoS aspects and related service requirements derived from them are described. These scenarios and service requirements can be used as guidance for the design of codecs and bearers.

##### 5.1.2.12.2.36	TS 22.153

Multimedia priority service

The document specifies the service requirements for Multimedia Priority Service (MPS). Its scope is to specify those requirements of MPS necessary to provide an end-to-end service and to interwork with external networks where needed. Service interactions with external networks are considered within the scope of this document although these interactions may be specified in other standards.

##### 5.1.2.12.2.37	TS 22.168

Earthquake and Tsunami Warning System (ETWS) requirements; Stage 1

The document defines the stage one description of the Earthquake and Tsunami Warning System (ETWS) Requirements. Stage one is the set of requirements seen primarily from the users and service providers points of view. It includes information applicable to network operators, service providers, terminal and network manufacturers, in case of deployment of ETWS. ETWS deployment depends on operator decision or national regulations. The TS contains the core requirements for the Earthquake and Tsunami Warning System, which are sufficient to provide a complete service. It also contains regional requirements for Earthquake and Tsunami Warning System.

##### 5.1.2.12.2.38	TS 22.173

IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services; Stage 1

The document defines the IMS Multimedia Telephony service and the minimum set of capabilities required to secure multi-vendor and multi-

operator inter-operability for Multimedia Telephony and related Supplementary Services.

##### 5.1.2.12.2.39	TS 22.179

Mission Critical Push to Talk (MCPTT) over LTE; Stage 1

This document provides the service requirements for operation of the MCPTT Service. MCPTT makes use of capabilities included in Group Communications System Enablers for LTE (GCSE_LTE) and Proximity Services (ProSe), with additional requirements specific to the MCPTT Service. The MCPTT Service can be used for public safety applications and also for general commercial applications (e.g., utility companies and railways).

##### 5.1.2.12.2.40	TS 22.182

Customized Alerting Tones (CAT) requirements; Stage 1

This document specifies the requirements and technical considerations for Customized Alerting Tone (CAT) service in both CS and PS domains, especially additional features for roaming and interoperability support.

##### 5.1.2.12.2.41	TS 22.183

Customized Ringing Signal (CRS) requirements; Stage 1

The document specifies the requirements and technical considerations for Customized Ringing Signal (CRS) service in the PS and CS domains, especially additional features for roaming and interoperability support.

##### 5.1.2.12.2.42	TS 22.185

Service requirements for V2X services

This document provides 3GPP support for V2X service requirements to be supported by LTE transport. These requirements are identified by taking into account the V2X service requirements defined in other SDOs, e.g. ETSI ITS, US SAE. The specification includes requirements of safety and non-safety aspects.

##### 5.1.2.12.2.43	TS 22.220

Service requirements for Home Node B (HNB) and Home eNode B (HeNB)

This specification defines the service requirements for the basic functionalities for the support of Home NodeB (HNB) and Home eNodeB (HeNB) – jointly referred to as H(e)NB – and the further functionalities that will enable the mobile operators to provide more advanced services as well as improving the user experience.

##### 5.1.2.12.2.44	TS 22.226

Global Text Telephony (GTT); Stage 1

This Technical Specification defines the stage one description of the Global Text Telephone Feature, GTT.

##### 5.1.2.12.2.45	TS 22.228

Service requirements for the Internet Protocol (IP) multimedia core network subsystem (IMS); Stage 1

This specification describes all IP Multimedia services offered by UMTS Systems and second generation systems.

##### 5.1.2.12.2.46	TS 22.234

Requirements on 3GPP system to Wireless Local Area Network (WLAN) interworking

The document specifies the functional requirements placed on the 3GPP system for interworking WLAN with the 3GPP system. Guidance is given for WLAN operators intending to provide the interworked WLAN capability.

##### 5.1.2.12.2.47	TS 22.246

Multimedia Broadcast/Multicast Service (MBMS) user services; Stage 1

This document describes MBMS User Services that use the capabilities of MBMS. Application scenarios including charging, QoS aspects and related service requirements derived from them are described. These scenarios and service requirements can be used as guidance for the design of codecs and bearers.

##### 5.1.2.12.2.48	TS 22.268

Public Warning System (PWS) requirements

This document covers the core requirements for the PWS that are sufficient to provide a complete service. This TS also covers subsystem additional requirements for the Earthquake and Tsunami Warning System (ETWS) and the Commercial Mobile Alert System (CMAS).

##### 5.1.2.12.2.49	TS 22.278

Service requirements for the Evolved Packet System (EPS)

This document describes the service requirements for the Evolved Packet System.

##### 5.1.2.12.2.50	TS 22.279

Combined Circuit Switched (CS) and IP Multimedia Subsystem (IMS) sessions; Stage 1

This document specifies service requirements for Combining CS and IMS services using a CS speech or CS multimedia call in association with an IMS session.

##### 5.1.2.12.2.51	TS 22.280

Mission Critical Services Common Requirements

This document provides the service requirements that are common across two or more mission critical services, that is MCPTT, MCData and MCVideo. The mission critical services make use of capabilities included in Group Communications System Enablers for LTE (GCSE_LTE) and Proximity Services (ProSe), with additional requirements specific to the MCPTT Service as specified in 3GPP TS 22.179, MCVideo Service as specified in 3GPP TS 22.281, and MCData Service as specified in 3GPP TS 22.282. The mission critical services can be used for public safety applications and also for general commercial applications (e.g., utility companies and railway operators).

##### 5.1.2.12.2.52	TS 22.281

Mission Critical Video over LTE

This document provides the service requirements for operation of the MCVideo service. MCVideo makes use of capabilities included in Group Communication System Enablers for LTE (GCSE_LTE), Proximity Services (ProSe), Isolated E-UTRAN operation for Public Safety (IOPS) and Mission Critical Services Common Requirements (MCCoRe) with additional requirements specific to the MCVideo Service. The MCVideo Service can be used for public safety applications and also for general commercial applications (e.g. utility companies and railways).

##### 5.1.2.12.2.53	TS 22.282

Mission Critical Data over LTE

This document provides the service requirements for operation of the MCData service. MCData makes use of capabilities included in Group Communication System Enablers for LTE (GCSE_LTE), Proximity Services (ProSe), Isolated E-UTRAN operation for Public Safety (IOPS) and Mission Critical Services Common Requirements (MCCoRe) with additional requirements specific to the MCData Service. The MCData Service can be used for public safety applications and also for general commercial applications (e.g., utility companies and railways).

##### 5.1.2.12.2.54	TS 22.346

Isolated Evolved Universal Terrestrial Radio Access Network (E-UTRAN) operation for public safety; Stage 1

This document specifies the service requirements for Isolated E-UTRAN Operation in support of mission critical network operation for Public Safety. In particular, requirements are specified for:

-	Initiation of Isolated E-UTRAN operation.

-	Ongoing Isolated E-UTRAN operation.

-	Termination of Isolated E-UTRAN operation.

-	Security aspects of Isolated E-UTRAN operation.

##### 5.1.2.12.2.55	TS 22.368

Service requirements for Machine-Type Communications (MTC); Stage 1

This document specifies the service requirements for Network Improvements for Machine Type Communications. In particular it will:

–   identify and specify general requirements for machine type communications;

–   identify service aspects where network improvements (compared to the current human-to-human oriented services) are needed to cater for the specific nature of machine-type communications;

–   specify machine type communication requirements for these service aspects where network improvements are needed for machine type communication.

##### 5.1.2.12.2.56	TS 22.468

Group Communication System Enablers for LTE (GCSE_LTE)

This document collects the requirements as relevant to improve the EPC and E-UTRAN for system enablers to the 3GPP system to support Group Communication over LTE for Public Safety and Critical Communication.

The US requirements as specified in NPSTC (Mission Critical Voice Requirements), the TETRA + Critical Communications Association (TCCA) and ITU inputs are taken as starting point.

Other regional requirements may also be reflected in the work. The requirements are worded in a way to easily accommodate future requirements from other regions or stakeholders.

##### 5.1.2.12.2.57	TS 22.519

Business communication requirements

This document specifies network requirements:

- to support connection and interoperation of business communication capabilities (either hosted in NGCN or NGN) to the NGN; and

- to support connection and interoperation of business communication capabilities to other business communication capabilities (either hosted in NGCN or NGN); and

- to support connection and interoperation of business communication capabilities to other business communication capabilities located in or connected to the ISDN and PSTN; and

- to support PABX functionality (hosted enterprise services) in an NGN.

NOTE 1: Network requirements to support connection of NGCN directly connected to an NGN are specified.

NOTE 2: Attachment of legacy PBX functionality to the NGN is not specified in this document. It is assumed that existing legacy service requirements apply in this case.

This document also specifies network requirements for communication between NGCN capabilities (including user equipment) to other NGCN capabilities of the same enterprise through the NGN (e.g. geographically separated).

This document does not specify NGCN services, nor does it specify network based application services provided to a user of an NGCN.

#### 5.1.2.12.3	Technical realization ("stage 2")

##### 5.1.2.12.3.1	TS 23.002

Network architecture

This specification describes the possible architectures of the mobile system.

##### 5.1.2.12.3.2	TS 23.003

Numbering, addressing and identification

This document defines the principal purpose and use of International Mobile station Equipment Identities (IMEI) within the digital cellular telecommunications system and the 3GPP system.

##### 5.1.2.12.3.3	TS 23.007

Restoration procedures

The data stored in location registers are automatically updated in normal operation; the main information stored in a location register defines the location of each mobile station and the subscriber data required to handle traffic for each mobile subscriber. The loss or corruption of these data will seriously degrade the service offered to mobile subscribers; it is therefore necessary to define procedures to limit the effects of failure of a location register, and to restore the location register data automatically. This document defines the necessary procedures.

##### 5.1.2.12.3.4	TS 23.008

Organization of subscriber data

This document provides details concerning information to be stored in home subscriber servers, visitor location registers, GPRS Support Nodes and Call Session Control Function (CSCF) concerning mobile subscriber.

##### 5.1.2.12.3.5	TS 23.009

Handover procedures

This document contains a detailed description of the handover procedures to be used in PLMNs. The purpose of the handover procedures are to ensure that the connection to the Mobile Station (MS) or User Equipment (UE) is maintained as it moves from one cell or radio network to another. The document defines the circuit switched handover functionality based on the service requirements in 3GPP TS 22.129. For the circuit switched handover functionality related to SRVCC, it is based on the service requirements in 3GPP TS 23.216.

##### 5.1.2.12.3.6	TS 23.011

Technical realization of Supplementary Services

This document describes the general aspects on how supplementary services in the 3GPP system are realised from a technical point of view. Description of technical realisation for specific supplementary services can be found in 3GPP TS 23.072. All supplementary services may require signalling on the radio path. For some supplementary services information needs to be transferred between the Home Location Register (HLR), the Visitor Location Register (VLR), the Mobile services Switching Centre (MSC) and the Serving GPRS Support Node (SGSN). Signalling procedures for such information transfer are defined in 3GPP TS 29.002.

##### 5.1.2.12.3.7	TS 23.018

Basic call handling; Technical realization

This document specifies the technical realization of the handling of calls originated by a UMTS or GSM mobile subscriber and calls directed to a UMTS or GSM mobile subscriber, up to the point where the call is established. Normal release of the call after establishment is also specified. Trunk Originated call is also modelled.

##### 5.1.2.12.3.8	TS 23.032

Universal Geographical Area Description (GAD)

This document defines an intermediate universal Geographical Area Description which subscriber applications, GSM or UMTS services can use and the network can convert into an equivalent radio coverage map. For GSM or UMTS services which involve the use of an “area”, it can be assumed that in the majority of cases the Service Requester will be forbidden access to data on the radio coverage map of a particular PLMN and that the Service Requester will not have direct access to network entities (e.g. BSC/BTS or RNC/Node B). The interpretation by the PLMN operator of the geographical area in terms of cells actually used, cells that are partly within the given area and all other technical and quality of service aspects are out of the scope of this document. This specification also provides a description of velocity that may be associated with a universal Geographical Area Description when both are applied to a common entity at a common time.

##### 5.1.2.12.3.9	TS 23.034

High Speed Circuit Switched Data (HSCSD); Stage 2

This document contains the stage 2 service description for a High Speed Circuit Switched Data (HSCSD) on GSM/GERAN in A/Gb mode and Iu mode. HSCSD utilizes the multislot mechanism, i.e. using multiple traffic channels (/bearers) for the communication. Additionally, this document specifies some HSCSD related requirements for multi system mobile stations operating in UTRAN Iu mode. In UTRAN Iu mode one bearer can provide all needed data rates, and the multislot mechanism is therefore not needed. However, for inter-system handover to GERAN, certain information has to be provided by the mobile station during the service negotiation.

##### 5.1.2.12.3.10	TS 23.038

Alphabets and language-specific information

This document defines the character sets, languages and message handling requirements for SMS, CBS and USSD and may additionally be used for Man Machine Interface (MMI) (3GPP TS 22.030).

##### 5.1.2.12.3.11	TS 23.040

Technical realization of the Short Message Service (SMS)

This document describes the Short Message Service (SMS) for GSM/UMTS networks. It defines:

–   the services and service elements;

–   the network architecture;

–   the Service Centre functionality;

–   the SMS Router functionality;

–   the MSC functionality (with regard to the SMS);

–   the SGSN functionality (with regard to the SMS);

–   the routing requirements;

–   the protocols and protocol layering; for the Mobile Originated and Mobile Terminated Short Message Service Teleservices, as specified in 3GPP TS 22.003 and 3GPP TS 22.105. The use of radio resources for the transfer of short messages between the MS and the MSC or the SGSN is described in 3GPP TS 24.011.

##### 5.1.2.12.3.12	TS 23.041

Technical realization of Cell Broadcast Service (CBS)

This document describes the Cell Broadcast short message service (CBS) for GSM and UMTS.

For GSM it defines the primitives over the Cell Broadcast Centre – Base Station System (CBC-BSS) interface and the message formats over the Base Station System – Mobile Station (BSS-MS) interface for Teleservice 23 as specified in 3GPP TS 22.003. For UMTS it defines the interface requirements for the Cell Broadcast Center – UMTS Radio Network System (RNS) interface and the radio interface requirements for UMTS Radio Access Networks to support CBS as specified in 3GPP TS 22.003. The document also describes the Public Warning System (PWS) for GSM, UMTS and E-UTRAN, see 3GPP TS 22.268.

##### 5.1.2.12.3.13	TS 23.042

Compression algorithm for text messaging services

This document introduces the concepts and mechanisms involved in the compression and decompression of a stream of data.

##### 5.1.2.12.3.14	TS 23.057

Mobile Execution Environment (MExE); Functional description; Stage 2

This document defines stage 2 and stage 3 of the Mobile Execution Environment (MExE). Stage 2 identifies the functional capabilities and information flows needed to support the service described in stage 1. MExE uses a number of technologies to realize the requirements of the stage 1 description (3GPP TS 22.057). The document describes how the service requirements are realized with the selected technologies. The specification covers areas of MExE common to all technologies.

##### 5.1.2.12.3.15	TS 23.060

General Packet Radio Service (GPRS); Service description; Stage 2

This specification describes a general overview over the GPRS architecture as well as a more detailed overview of the MS – CN protocol architecture. Details of the protocols will be specified in companion documents.

##### 5.1.2.12.3.16	TS 23.066

Support of Mobile Number Portability (MNP); Technical realization; Stage 2

This document describes several alternatives for the realization of Mobile Number Portability. It is left to operator and implementation decisions which option, or combination of options, is used, taking into account the regulatory and architectural constraints that may prevail.

##### 5.1.2.12.3.17	TS 23.078

Customised Applications for Mobile network Enhanced Logic (CAMEL) Phase 4; Stage 2

This document specifies the stage 2 description for the fourth phase (see 3GPP TS 22.078) of the Customized Applications for Mobile network Enhanced Logic (CAMEL) feature which provides the mechanisms to support services of operators which are not covered by standardized services even when roaming outside the HPLMN.

##### 5.1.2.12.3.18	TS 23.081

Line Identification supplementary services; Stage 2

This document gives the stage 2 description of the call identification supplementary services.

The group line identification supplementary services are divided into the following four supplementary services:

–   Calling line identification presentation CLIP;

–   Calling line identification restriction CLIR;

–   Connected line identification presentation COLP;

–   Connected line identification restriction COLR.

##### 5.1.2.12.3.19	TS 23.082

Call Forwarding (CF) supplementary services; Stage 2

This document gives the stage 2 description of the call forwarding supplementary services.

The group of supplementary services call offering supplementary services is divided into 4 different supplementary services:

–   Call forwarding unconditional (CFU);

–   Call forwarding on mobile subscriber busy (CFB);

–   Call forwarding on no reply (CFNRy);

–   Call forwarding on mobile subscriber not reachable (CFNRc).

##### 5.1.2.12.3.20	TS 23.083

Call Waiting (CW) and Call Hold (HOLD) supplementary services; Stage 2

This document gives the stage 2 description of the call completion supplementary services.

The group of call completion supplementary services is divided into the following two supplementary services:

–   Call waiting (CW);

–   Call hold (HOLD).

##### 5.1.2.12.3.21	TS 23.084

Multi Party (MPTY) supplementary service; Stage 2

This document gives the stage 2 description of the multi party supplementary services.

Only one multi party supplementary service has been defined, this is the Multi Party (MPTY) service.

##### 5.1.2.12.3.22	TS 23.085

Closed User Group (CUG) supplementary service; Stage 2

This document gives the stage 2 description of the closed user group supplementary service.

The community of interest supplementary service defined is:

–   closed user group (CUG).

##### 5.1.2.12.3.23	TS 23.086

Advice of Charge (AoC) supplementary services; Stage 2

This document gives the stage 2 description of the Advice of Charge (AoC) supplementary services.

The charging supplementary services currently defined are:

–   Advice of Charge (Information) (AoCI);

–   Advice of Charge (Charging) (AoCC).

##### 5.1.2.12.3.24	TS 23.087

User-to-User Signalling (UUS) supplementary service; Stage 2

This document gives the stage 2 description of the User-to-User signalling supplementary services.

The User-to-user supplementary service is divided into 3 different services:

–   Service 1 (UUS1);

–   Service 2 (UUS2);

–   Service 3 (UUS3).

##### 5.1.2.12.3.25	TS 23.088

Call Barring (CB) Supplementary Services; Stage 2

This document gives the stage 2 description of the call barring services.

The possibility for a mobile subscriber to have certain categories of calls barred originated from or terminated at his access:

Barring of outgoing calls:

–   Barring of all outgoing calls (BAOC) (Barring program 1);

–   Barring of outgoing international calls (BOIC) (Barring program 2);

–   Barring of outgoing international calls EXCEPT those directed to the home PLMN country (BOIC-exHC) (Barring program 3).

Barring of incoming calls:

–   Barring of all incoming calls (BAIC) (Barring program 1);

–   Barring of incoming calls when roaming outside the home PLMN country (BIC-Roam) (Barring program 2)

–   Anonymous Call Rejection (ACR) (Barring program 3). The call barring program "incoming calls when roaming outside the home PLMN country" is only relevant if as a general rule the called mobile subscriber pays the charges for the forwarded part of the call from his home PLMN country to any other country.

##### 5.1.2.12.3.26	TS 23.090

Unstructured Supplementary Service Data (USSD); Stage 2

This document gives the stage 2 description of Unstructured Supplementary Service Data (USSD). The unstructured supplementary service data (USSD) mechanism allows the Mobile Station (MS) user and a PLMN operator defined application to communicate in a way which is transparent to the MS and to intermediate network entities. The mechanism allows development of PLMN specific supplementary services. This document defines the requirements for handling USSD at the MS and network entities.

##### 5.1.2.12.3.27	TS 23.091

Explicit Call Transfer (ECT) supplementary service; Stage 2

This document gives the stage 2 description of the call transfer supplementary services.

Only one call transfer supplementary service has been defined, this is the Explicit Call Transfer (ECT) supplementary service, and it is described in this document.

##### 5.1.2.12.3.28	TS 23.093

Technical realization of Completion of Calls to Busy Subscriber (CCBS); Stage 2

This document gives the stage 2 description of the Completion of Calls to Busy Subscriber (CCBS) supplementary service.

##### 5.1.2.12.3.29	TS 23.094

Follow-Me (FM); Stage 2

This document specifies the stage 2 description for the Follow Me feature. The Follow Me feature enables a mobile subscriber A to manipulate the Follow Me data of a remote party B in such a way that subsequent calls directed to remote party B will be forwarded to subscriber A.

##### 5.1.2.12.3.30	TS 23.096

Name identification supplementary services; Stage 2

This document gives the stage 2 description of the Name Identification Supplementary Services.

The group of Name Identification Supplementary Services contains the following Supplementary Service: CNAP - Calling name presentation.

##### 5.1.2.12.3.31	TS 23.101

General Universal Mobile Telecommunications System (UMTS) architecture

This specification describes the basic physical and functional separation of UMTS. The content of this specification is limited to those features that are common to all UMTS networks independent of their origin. It identifies and names the reference points and functional groupings appearing at this level.

##### 5.1.2.12.3.32	TS 23.107

Quality of Service (QoS) concept and architecture

This specification describes the framework for QoS in UMTS. The document shall be used as a living document which will cover all issues related QoS in UMTS.

##### 5.1.2.12.3.33	TS 23.108

Mobile radio interface layer 3 specification, core network protocols; Stage 2

This specification describes the procedures used at the radio interface for Call Control (CC), Mobility Management (MM) and Session Management (SM). It contains examples of the structured procedures.

##### 5.1.2.12.3.34	TS 23.110

Universal Mobile Telecommunications System (UMTS) access stratum; Services and functions

This specification describes the detailed specifications of the protocols which rule the information flows, both control and user data, between the access stratum and the parts of UMTS outside the access stratum, and of the detailed specifications of the UTRAN. These detailed specifications are to be found in other technical specifications.

##### 5.1.2.12.3.35	TS 23.122

Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode

This specification provides an overview of the tasks undertaken by a Mobile Station (MS) when in idle mode, that is, switched on but not having a dedicated channel allocated, e.g. not making or receiving a call, or when in group receive mode, that is, receiving a group call or broadcast call but having a dedicated connection. It also describes the corresponding network functions.

##### 5.1.2.12.3.36	TS 23.141

Presence service; Architecture and functional description

This document describes the stage 2 description (architectural solution and functionalities) for the Presence Service, which includes the elements necessary to realize the stage 1 requirements in TS 22.141.

##### 5.1.2.12.3.37	TS 23.142

Value-added Services for SMS (VAS4SMS); Interface and signalling flow

The specification describes the Stage 2 of the VAS4SMS (Value Added Service for SMS). It includes: The logic architecture; The logic elements functionality; The signaling flows; The interaction with other features.

##### 5.1.2.12.3.38	TS 23.153

Out of band transcoder control; Stage 2

This document specifies the stage 2 description of the Out-of-Band Transcoder Control for speech services. It describes the principles and procedures to support Transcoder Free Operation, Tandem Free Operation and the interworking between TrFO and TFO. Transcoder at the edge is also part of this document.

##### 5.1.2.12.3.39	TS 23.161

Network-Based IP Flow Mobility (NBIFOM); Stage 2

The scope of this document is to specify the support of NBIFOM (Network based IP Flow Mobility) i.e. IP flow mobility based on network mobility protocols. This feature supports PDN connections that are simultaneously connected over 3GPP access (i.e. S5/S8 connection to a PDN GW) and a WLAN access (i.e. S2a or S2b connection to the same PDN GW).

##### 5.1.2.12.3.40	TS 23.167

IP Multimedia Subsystem (IMS) emergency sessions

This document defines the stage 2 service description for emergency services in the IP Multimedia Core Network Subsystem (IMS), including the elements necessary to support IP Multimedia (IM) emergency services. This document covers also the Access Network aspects that are crucial for the provisioning of IMS emergency services. Other 3GPP specifications that are related to the IMS emergency services are TS 23.228 on IMS in general, including fixed broadband access aspects, TS 23.060 describing GPRS (UTRAN), TS 23.401, TS 23.060; TS 23.402 describing EPS (UTRAN and E-UTRAN); TS 23.234 describing 3GPP/WLAN Interworking; TS 23.271 that covers location services and TS 23.216 and TS 23.237 describing Single Radio Voice Call Continuity for IMS Emergency session. TS 25.301 contains an overall description of the UMTS Terrestrial Radio Access Network TS 36.300 contains an overall description of the Evolved Universal Terrestrial Radio Access

(E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN). Other non-3GPP specifications that are related to the IMS emergency services include 3GPP2 cdma2000 HRPD IP CAN, as specified in 3GPP2 X.S0060 when the UE is connected to a PDS core network and 3GPP2 X.S0057 A when the UE is connected to an EPC core network. The emergency support in different IP-CANs is described in the Informative Annex E.

##### 5.1.2.12.3.41	TS 23.179

Functional architecture and information flows to support mission critical communication services; Stage 2

This document specifies the functional architecture, procedures and information flows needed to support the mission critical push to talk (MCPTT) service including the common services core architecture for identity management, group management, and configuration management required to support the MCPTT voice service. Support for both MCPTT group calls and MCPTT private calls operating in on-network and off-network modes of operation is specified.

The corresponding service requirements are defined in 3GPP TS 22.179.

This document is applicable primarily to MCPTT voice service using E-UTRAN access based on the EPC architecture defined in 3GPP TS 23.401. Certain MCPTT functions such as dispatch and administrative functions could also be supported via non-3GPP access networks but no additional functionality is specified to support non-3GPP access.

The MCPTT service requires preferential handling compared to normal telecommunication services e.g. in support of police or fire brigade including the handling of prioritised MCPTT calls for emergency and imminent threats.

The MCPTT service can be used for public safety applications and also for general commercial applications e.g. utility companies and railways.

In this document, MCPTT calls between MCPTT users on different MCPTT systems are considered, however, for roaming only EPC-level roaming and IMS-level roaming are considered i.e. partner MCPTT system based roaming (also known as "migration") is out of scope.

##### 5.1.2.12.3.42	TS 23.203

Policy and charging control architecture

This document specifies the overall stage 2 level functionality for Policy and Charging Control that encompasses the following high level functions for IP CANs (e.g. GPRS, I WLAN, Fixed Broadband, etc.): (i) Flow Based Charging, including charging control and online credit control; (ii) Policy control (e.g. gating control, QoS control, QoS signalling, etc.).

##### 5.1.2.12.3.43	TS 23.204

Support of Short Message Service (SMS) over generic 3GPP Internet Protocol (IP) access; Stage 2

This document specifies the new capabilities and enhancements needed to support SMS over a generic IP Connectivity Access Network (IP-

CAN) using IMS capabilities (TS 23.228).

##### 5.1.2.12.3.44	TS 23.205

Bearer-independent circuit-switched core network; Stage 2

This document defines the stage 2 description for the bearer independent CS core network. The stage 2 covers the information flow between the GMSC server, MSC server and media gateways. The document shows the CS core network termination of the Iu interface in order to cover the information flow stimulus to the core network and describe the interaction with the supplementary and value added services and capabilities. The protocol used over the Nc interface is an enhanced call control protocol supporting call bearer separation such as BICC. The protocol used over the Mc interface is H.248. SIP-I based CS core network is further specified in 3GPP TS 23.231. Local Call Local Switch (LCLS) functionality is further specified in 3GPP TS 23.284.

This document is applicable only for ATM or IP transport in the CS core network.

##### 5.1.2.12.3.45	TS 23.207

End-to-end Quality of Service (QoS) concept and architecture

This document provides the framework for end-to-end Quality of Service involving GPRS and complements TS 23.107 which describes the framework for Quality of Service within UMTS. The document describes the interaction between the TE/MT Local Bearer Service, the GPRS Bearer Service, and the External Bearer Service, and how these together provide Quality of Service for the End-to-End Service. The document also describes IP level mechanisms necessary in providing end-to-end Quality of Service involving GPRS networks, including possible interaction between the IP level and the GPRS level, as well as the application level and the IP level.

##### 5.1.2.12.3.46	TS 23.214

Architecture enhancements for control and user plane separation of EPC nodes

This document specifies the overall stage 2 level functionality for control and user plane separation of EPC's SGW, PGW and TDF. This enables a flexible placement of the separated control plane and user plane functions for supporting diverse deployment scenarios (e.g. central or distributed user plane function) without affecting the overall functionality provided by these EPC entities.

##### 5.1.2.12.3.47	TS 23.216

Single Radio Voice Call Continuity (SRVCC); Stage 2

This Technical Specification specifies the architecture enhancements for Single Radio Voice Call Continuity (SRVCC) between E-UTRAN access and 3GPP2Æs 1xCS, and between E-UTRAN access and 3GPPÆs UTRAN/GERAN accesses and between UTRAN (HSPA) access and 3GPPÆs UTRAN/GERAN accesses, for Circuit Switched (CS) calls that are anchored in the IMS.

##### 5.1.2.12.3.48	TS 23.218

IP Multimedia (IM) session handling; IM call model; Stage 2

This document specifies the IP Multimedia (IM) Call Model for handling of an IP multimedia session origination and termination for an IP Multimedia subscriber. The document includes interactions between an Application Server and IP multimedia sessions.

##### 5.1.2.12.3.49	TS 23.221

Architectural requirements

This document covers the architectural requirements for 3GPP systems based on UTRAN, E-UTRAN and Iu mode GERAN. In particular it details the high level requirements for the Circuit Switched (CS) Domain and the stage 2 procedures that span more than one domain/subsystem. The reference model to which these procedures apply can be found within TS 23.002. In addition, A mode to Iu mode handover for CS services is addressed. Detailed architectural requirements within the subsystems are contained within the remainder of the 23 series of specifications e.g. the requirements for the Packet Switched (PS) domain are contained within TS 23.060, TS 23.401 and the requirements for the Bearer Independent CS Core Network are contained in TS 23.205.

##### 5.1.2.12.3.50	TS 23.226

Global text telephony (GTT); Stage 2

This 3GPP Technical Specification defines the stage 2 description of the real time Text Conversation Feature called Global Text Telephony, GTT. GTT Stage 2 identifies the functional capabilities needed to support the service described in GTT Stage 1. This TS contains the core functions for a real time Text Conversation Feature GTT, to be used in combination with other media in conversational services. GTT offers real time conversation in text, to be used alone or in combination with other conversational media, and interworking with current and emerging text conversation features in the fixed networks and other mobile networks. GTT uses a number of functional entities to realize the requirements of the stage 1 description (TS 22.226). This TS describes how the service requirements are realized with these functional entities. As far as possible existing protocols shall be used for the realization of the Global Text Telephony Feature. This may include e.g. SIP, 3G.324, or Circuit Switched Voice service as protocol environments, and CTM, AL1 and RTP/text as transmission protocols. It also means usage of existing text presentation format ITU-T Recommendation T.140, common to all GTT text conversation environments.

##### 5.1.2.12.3.51	TS 23.228

IP Multimedia Subsystem (IMS); Stage 2

This specifications describes the architectural requirement for an IP Multimedia Components incorporated in an UMTS System as well as second generation systems for GSM inside the core network and identify relevant interfaces to the existing system and the new one in between the new components incorporated.

##### 5.1.2.12.3.52	TS 23.231

SIP-I based circuit-switched core network; Stage 2

This document defines the stage 2 description for the SIP-I based CS core network. The logical architecture for the SIP-I based CS core network is defined in 3GPP TS 23.205. Stage 2 covers the information flows between the GMSC server, MSC server and media gateways that are required to support a SIP-I based Nc interface.

The document shows the CS core network termination of the Iu and A interfaces in order to cover the information flow stimulus to the core network and describe the interaction with the supplementary and value added services and capabilities. The Nc interface profile is based on ITU-T Q.1912.5 SIP-I profile C and is specified in 3GPP TS 29.231. The Mc interface profile is based on ITU-T H.248.1 and is specified in 3GPP TS 29.232. Local Call Local Switch (LCLS) functionality is further specified in 3GPP TS 23.284. This document is applicable only for IP transport in the CS core network.

##### 5.1.2.12.3.53	TS 23.234

3GPP system to Wireless Local Area Network (WLAN) interworking; System description

This document specifies system description for interworking between 3GPP systems and Wireless Local Area Networks (WLANs), which extends 3GPP services and functionality to the WLAN access environment. It is not limited to WLAN technologies, but also valid for other IP based Access Networks that support the same capabilities towards the interworking system as WLAN does.

##### 5.1.2.12.3.54	TS 23.236

Intra-domain connection of Radio Access Network (RAN) nodes to multiple Core Network (CN) nodes

UMTS will build on the success of GSM and is likely to become even more widespread, increasing the importance of a flexible network structure to permit the different operational configurations in which these networks will be deployed. The requirements to have a RNC or BSC controlled by a single MSC server or SGSN lead to certain limitations. Allowing the BSCs and RNCs to connect to a number of MSC servers or SGSNs increases the networks performance in terms of scalability, distributing the network load amongst the serving entities, and reducing the required signalling as the user roams.

##### 5.1.2.12.3.55	TS 23.237

IP Multimedia Subsystem (IMS) Service Continuity; Stage 2

This document specifies the architectural requirements and procedures for delivery of IMS Service Continuity.

The scope of the specification includes:

–   Access Transfer related functionality:

–   PS-CS Access Transfer;

–   PS-PS Access Transfer;

–   PS-PS Access Transfer in conjunction with PS-CS Access Transfer;

–   Adding and/or removing media flows to support service;

–   MSC Server assisted mid-call feature;

–   SRVCC session transfer of IMS emergency session.

–   Inter UE Transfer related functionality:

–   Establishment and release of a Collaborative Session;

–   Addition of media flows to, modification of media flows in, and release of media flows from a Collaborative Session;

–   Transfer of media flows from one UE to another;

–   Transfer of the Collaborative Session Control with or without transfer of media flows;

–   Transfer of all media flows to a target UE without establishing a Collaborative Session;

–   Session discovery;

–   Inter-UE Transfer initiated by the target UE or by the SCC AS;

–   Replication of media flows by means of Inter-UE Transfer procedures;

–   Authorization and other aspects to support Inter-UE Transfer across multiple IMS subscriptions. The solution is restricted to service continuity using IMS procedures, i.e. mobility mechanisms on the IP-CAN level are not within the scope of this specification.

##### 5.1.2.12.3.56	TS 23.240

3GPP Generic User Profile (GUP); Architecture (Stage 2)

This document defines the stage 2 architecture description to the 3GPP Generic User Profile (GUP), which includes the elements necessary to realize the stage 1 requirements in TS 22.240. This document includes the GUP reference architecture with descriptions of functional entities, and their interfaces and procedures, as well as the high-level information model for the GUP data.

##### 5.1.2.12.3.57	TS 23.246

Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description

This document describes the stage 2 description (architectural solution and functionalities) for the MBMS Bearer Service, which includes, together with MBMS User Services defined in TS 26.346, all the elements necessary to realize the stage 1 requirements in TS 22.146 and TS 22.246. This document encompasses both GPRS and EPS.

##### 5.1.2.12.3.58	TS 23.251

Network sharing; Architecture and functional description

This document covers the details of Network Sharing for GERAN, UTRAN and E-UTRAN. It shows how several core network operators can share one radio access network and details the impacts on the network architecture. All UEs shall comply with existing requirements, among them PLMN selection and system information reception. This document also defines requirements for network-sharing supporting UEs

##### 5.1.2.12.3.59	TS 23.259

Personal Network Management (PNM); Procedures and information flows; Stage 2

This document provides the procedure details and the information flows for support of Personal Network Management including the PN UE redirection and PN access control applications enabled by Personal Network Management (PNM).

##### 5.1.2.12.3.60	TS 23.261

IP flow mobility and seamless Wireless Local Area Network (WLAN) offload; Stage 2

This document specifies the Stage 2 system description for IP flow mobility between a 3GPP and a WLAN. The technical solution is based on the working principles of DSMIPv62 and it is applicable to both the Evolved Packet System and the I-WLAN mobility architecture. The specification covers the system description of seamless WLAN offload and IP flow mobility between 3GPP and WLAN as well as the respective interactions with the PCC and ANDSF frameworks. The system description for non-seamless WLAN offload is covered in 3GPP TS 23.402. This document specifies also the detailed extensions to S2c and H1 reference points for IP flow mobility. The extensions to the PCC and to the ANDSF framework are specified respectively in 3GPP TS 23.203 and in 3GPP TS 23.402.

##### 5.1.2.12.3.61	TS 23.271

Functional stage 2 description of Location Services (LCS)

This document specifies the stage 2 of the Location Services (LCS) feature in UMTS, GSM and EPS (for E-UTRAN), which provides the mechanisms to support mobile location services for operators, subscribers and third party service providers.

##### 5.1.2.12.3.62	TS 23.272

Circuit Switched (CS) fallback in Evolved Packet System (EPS); Stage 2

This document defines the Stage 2 architecture and specification for the CS Fallback and for SMS over SGs for EPS or CS Fallback and SMS over S102. The scope of this document includes the architecture enhancements for functionality to enable fallback from E-UTRAN access to UTRAN/GERAN CS domain access and to CDMA 1x RTT CS domain access, and functionality to reuse of voice and other CS-domain services (e.g. CS UDI video/LCS/USSD) by reuse of the CS domain. The functionality specified to support SMS over SGs does not trigger any CS Fallback to UTRAN/GERAN.

The functionality specified to support SMS over S102 does not trigger any CS Fallback to CDMA 1xRTT CS domain

##### 5.1.2.12.3.63	TS 23.278

Customized Applications for Mobile network Enhanced Logic (CAMEL) Phase 4; Stage 2; IM CN Interworking

This document specifies the stage 2 description for the Customized Applications for Mobile network Enhanced Logic (CAMEL) feature which provides the mechanisms to support services for the IP Multimedia Core Network (IM CN) Subsystem.

##### 5.1.2.12.3.64	TS 23.279

Combining Circuit Switched (CS) and IP Multimedia Subsystem (IMS) services; Stage 2

This document provides architectural details to combine CS services and IMS services for using them in parallel between the same two users in a peer-to-peer context. The document provides a detailed description of how capabilities and identities are exchanged to enable the combination of CS and IMS services between the same two UEs.

##### 5.1.2.12.3.65	TS 23.280

Common functional architecture to support mission critical services; Stage 2

This document specifies the common functional architecture, procedures and information flows needed to support mission critical services including the common services core architecture.

The corresponding service requirements are defined in 3GPP TS 22.179, 3GPP TS 22.280, 3GPP TS 22.281 and 3GPP TS 22.282.

This document is applicable primarily to mission critical services using E-UTRAN access based on the EPC architecture defined in 3GPP TS 23.401. Certain MC service functions such as dispatch and administrative functions could also be supported via non-3GPP access networks but no additional functionality is specified to support non-3GPP access.

The common functional architecture to support mission critical services can be used for public safety applications and also for general commercial applications e.g. utility companies and railways.

##### 5.1.2.12.3.66	TS 23.281

Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2

This document specifies the functional model, procedures and information flows needed for the mission critical video (MCVideo) service. Support for both MCVideo group streaming and MCVideo private streaming operating in on-network and off-network modes of operation is specified.

The corresponding service requirements are defined in 3GPP TS 22.280, and 3GPP TS 22.281.

This document is applicable primarily to mission critical video service using E-UTRAN access based on the common functional architecture for mission critical services defined in 3GPP TS 23.280 and the EPC architecture defined in 3GPP TS 23.401.

The MCVideo service can be used for public safety applications and also for general commercial applications e.g. utility companies and railways.

##### 5.1.2.12.3.67	TS 23.282

Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2

This document specifies the functional architecture, procedures and information flows needed to support the Mission Critical Data (MCData) services. MCData is a suite of services which utilizes the common functional architecture defined in 3GPP TS 23.280 to support MC services over LTE including the common services core.

MCData services suite consists of the following sub-services:

-	short data service (SDS);

-	file distribution (FD.

MCData features include:

-	conversation management;

-	transmission and reception control;

-	communication release; and

-	enhanced status.

The corresponding service requirements are defined in 3GPP TS 22.282 and 3GPP TS 22.280.

This document is applicable primarily to MCData service using E-UTRAN access based on the EPC architecture defined in 3GPP TS 23.401. Certain application functions of the MCData service could also be supported via non-3GPP access networks but no additional functionality is specified to support non-3GPP access.

The MCData service can be used for public safety applications and also for general commercial applications e.g. utility companies and railways.

##### 5.1.2.12.3.68	TS 23.285

Architecture enhancements for V2X services

This document specifies architecture enhancements to facilitate vehicular communications for Vehicle-to-Everything (V2X) services, which includes Vehicle-to-Vehicle (V2V), Vehicle-to-Pedestrian (V2P), Vehicle-to-Infrastructure (V2I), and Vehicle-to-Network (V2N), according to the use cases and service requirements defined in TS 22.185.

The following vehicular communications for V2X services are specified:

-	V2X communication over PC5 reference point.

-	V2X communication over LTE-Uu reference point.

##### 5.1.2.12.3.69	TS 23.292

IP Multimedia Subsystem (IMS) centralized services; Stage 2

This document specifies the architectural requirements for delivery of consistent services to the user regardless of the attached access type (e.g. CS domain access, or IP-CAN). This is achieved by implementing the services in the IP Multimedia Subsystem (IMS).

##### 5.1.2.12.3.70	TS 23.303

Proximity-based services (ProSe); Stage 2

This document specifies the Stage 2 of the Proximity Services (ProSe) features in EPS. ProSe features consist of: ProSe discovery (direct or EPC-level) and ProSe Direct Communication (using E-UTRAN or WLAN direct).

ProSe discovery identifies that ProSe-enabled UEs are in proximity, using E-UTRAN (with or without E-UTRAN) or EPC.

ProSe Direct Communication enables establishment of communication paths between two or more ProSe-enabled UEs that are in direct communication range. The ProSe Direct Communication path could use E-UTRAN or WLAN.

For Public Safety specific usage:

- ProSe-enabled Public Safety UEs can establish the communication path directly between two or more ProSe-enabled Public Safety UEs, regardless of whether the ProSe-enabled Public Safety UE is served by E-UTRAN.

- ProSe Direct Communication is also facilitated by the use of a ProSe UE-to-Network Relay, which acts as a relay between E-UTRAN and UEs not served by E-UTRAN.

Security aspects of ProSe are defined in TS 33.303.

##### 5.1.2.12.3.71	TS 23.327

Mobility between 3GPP-Wireless Local Area Network (WLAN) interworking and 3GPP systems

This document specifies the stage 2 system description for providing mobility between 3GPP Wireless Local Area Network Interworking (I WLAN) and 3GPP Systems. It is to define a technical solution based on the working principles of DSMIPv6 with necessary enhancement of the I WLAN architecture for supporting mobility and roaming between 3GPP WLAN Interworking system and 3GPP Systems so that ongoing 3GPP PS based services can be maintained with minimal impact on the end user's perceived quality on the services at a change of the access network (between I WLAN and 3GPP Access Systems).

##### 5.1.2.12.3.72	TS 23.333

Multimedia Resource Function Controller (MRFC) – Multimedia Resource Function Processor (MRFP) Mp interface: Procedures descriptions

This specification describes the functional requirements and information flows that generate procedures between the Multimedia Resource Function Controller (MRFC) and the Multimedia Resource Function Processor (MRFP), the Mp Interface. This specification is limited to information flows relevant to the Mp Interface; in order to define these procedures and make the functional requirements clear some triggers from an external interface may be described; these may be specified within the Mr interface for example or within an AS in which the MRFC function resides. However for the overall stage 2 procedures of IMS. The protocol on the Mp interface is defined to comply with ITU-T H.248.1 Gateway Control Protocol.

##### 5.1.2.12.3.73	TS 23.334

IP Multimedia Subsystem (IMS) Application Level Gateway (IMS-ALG) – IMS Access Gateway (IMS-AGW) interface: Procedures descriptions

Annex G of 3GPP TS 23.228 gives out an IMS Application Level Gateway (IMS-ALG) and IMS Access Media Gateway (IMS-AGW) based reference model to support NAPT-PT, gate control and traffic policing between IP-CAN and IMS domain. The Iq reference point is between the P-CSCF (IMS-ALG) and the IMS-AGW. It conveys the necessary information that is needed to allocate, modify and release (IP) transport addresses. This document defines the stage 2 description for the Iq reference point. Stage 2 covers the information flow between the P-CSCF (IMS-ALG) and IMS-AGW. The protocol used over the Iq interface is the gateway control protocol according ITU-T Recommendation H.248 (which is specified for Iq by an H.248 profile according 3GPP TS 29.334).

##### 5.1.2.12.3.74	TS 23.335

User Data Convergence (UDC); Technical realization and information flows; Stage 2

This document describes the procedures and signalling flows associated to the technical realization of the 3GPP User Data Convergence (UDC). 
It furthermore indicates some requirements for the Stage 3 specifications. Special consideration is put in the following areas: – reference architecture for the UDC concept; – general description of procedures for the user data manipulation (e.g. create, delete, update, etc.); – identification of the requirements on the UDC for the applicability of the mechanisms described in this document. User data convergence is an optional concept to ensure data consistency and simplify creation of new services by providing easy access to the user data, as well as to ensure the consistency of storage and data models and to have minimum impact on traffic mechanisms, reference points and protocols of network elements.

##### 5.1.2.12.3.75	TS 23.379

Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2

This document specifies the functional architecture, procedures and information flows needed to support the mission critical push to talk (MCPTT) service. The MCPTT service utilizes the common functional architecture to support MC services over LTE including the common services core defined in 3GPP TS 23.280.Support for both MCPTT group calls and MCPTT private calls operating in on-network and off-network modes of operation is specified.

The corresponding service requirements are defined in 3GPP TS 22.179 and 3GPP TS 22.280.

This document is applicable primarily to MCPTT voice service using E-UTRAN access based on the EPC architecture defined in 3GPP TS 23.401. Certain application functions of the MCPTT service such as dispatch and administrative functions could also be supported via non-3GPP access networks but no additional functionality is specified to support non-3GPP access.

The MCPTT service requires preferential handling compared to normal telecommunication services e.g. in support of police or fire brigade including the handling of prioritised MCPTT calls for emergency and imminent threats.

The MCPTT service can be used for public safety applications and also for general commercial applications e.g. utility companies and railways.

In this document, MCPTT calls between MCPTT users on different MCPTT systems are considered.

##### 5.1.2.12.3.76	TS 23.380

IMS Restoration Procedures

This document specifies the IMS procedures required to handle a S-CSCF service interruption scenario with minimum impact to the service to the end user.

##### 5.1.2.12.3.77	TS 23.401

General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access

This Technical Specification defines the Stage 2 service description for the Evolved 3GPP Packet Switched Domain – also called the Evolved Packet System (EPS) in this document. The Evolved 3GPP Packet Switched Domain provides IP connectivity using the Evolved Universal Terrestrial Radio Access Network (E-UTRAN). The specification also covers mobility between E-UTRAN and pre-E-UTRAN 3GPP radio access technologies.

##### 5.1.2.12.3.78	TS 23.402

Architecture enhancements for non-3GPP accesses

This Technical Specification defines the Stage 2 service description for providing IP connectivity using non-3GPP accesses to the Evolved 3GPP Packet Switched domain. In addition, for E-UTRAN and non-3GPP accesses, the specification describes the Evolved 3GPP PS Domain where the protocols between its Core Network elements are IETF based.

##### 5.1.2.12.3.79	TS 23.468

Group Communication System Enablers for LTE (GCSE_LTE); Stage 2

This document provides the stage 2 description for the 3GPP system provided enablers to support group communication services using EUTRAN access. The corresponding stage 1 requirements are defined in TS 22.468 .

The group communication system is represented by an Application Server (GCS AS) using the 3GPP system provided enablers for transferring its application signalling and for delivering application data, e.g. media data, to a group of UEs either (i) over MBMS Bearer Services using the Broadcast Mode of MBMS (TS 23.246); or (ii) over EPS Bearers; or (iii) over both MBMS and EPS bearer services.

This document covers both roaming and non-roaming scenarios and also includes support for service continuity, i.e. support for continuous reception of application data when a UE changes between reception of application data via EPS and MBMS Bearer Services.

Application level interactions between the UE and the GCS Application Server are out of scope of this specification.

##### 5.1.2.12.3.80	TS 23.682

Architecture enhancements to facilitate communications with packet data networks and applications

This document specifies architecture enhancements to facilitate communications with packet data networks and applications (e.g. Machine Type Communication (MTC) applications on the (external) network/MTC servers) as per the use cases and service requirements defined in TS 22.368, TS 22.101 and related 3GPP requirements specifications. Both roaming and non-roaming scenarios are covered.

–   Device triggering by applications/servers (e.g.: MTC applications on the (external) network/MTC servers).

–   PS-Only support with and without MSISDN.

#### 5.1.2.12.4	Signalling protocols ("stage 3") - user equipment to network

##### 5.1.2.12.4.1	TS 24.002

GSM – UMTS Public Land Mobile Network (PLMN) Access Reference Configuration

This document describes the reference configuration for access to a PLMN. A user accesses a PLMN via a number of interfaces, including the MS_BS 
(in A/Gb mode and GERAN Iu mode), UE-UTRAN (in UTRAN Iu mode) and UE-E-UTRAN interface. The purpose of this document is to indicate the possible access arrangements that may be used in conjunction with the MS-BS (in A/Gb mode and GERAN Iu mode), UE-UTRAN (in UTRAN Iu mode) and UE-E – UTRAN interface.

##### 5.1.2.12.4.2	TS 24.007

Mobile radio interface signalling layer 3; General Aspects

This specification describes the principal architecture of Layer 3 and its sub-layers on the GSM Um interface, i.e. the interface between mobile station (MS) and network; for the CM sub-layer, the description is restricted to paradigmatic examples, CC, supplementary services, and short message services for non-general packet radio service (GPRS) services. It also defines the basic message format and error handling applied by the Layer 3 protocols.

##### 5.1.2.12.4.3	TS 24.008

Mobile radio interface Layer 3 specification; Core network protocols; Stage 3

This specification describes the procedures used at the radio interface for Call Control, Mobility Management and Session Management. The procedures currently described are for the CC of circuit-switched connections, SM for GPRS services, MM and radio resource management for circuit-switched and GPRS services. MBMS is also added.

##### 5.1.2.12.4.4	TS 24.010

Mobile radio interface layer 3; Supplementary services specification; General aspects

This specification describes the general aspects of the specification of supplementary services at the Layer 3 radio interface. Details will be specified in other TS.

##### 5.1.2.12.4.5	TS 24.011

Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface

This specification describes the procedures used across the mobile radio interface by the signalling Layer 3 function short message control (SMC) and short message relay (SM-RL) function for both circuit-switched GSM and GPRS.

##### 5.1.2.12.4.6	TS 24.022

Radio Link Protocol (RLP) for circuit switched bearer and teleservices

This specification describes the RLP for data transmission over the UMTS public land mobile network (PLMN). RLP covers the Layer 2 functionality of the ISO OSI reference model (IS 7498).

It is based on ideas contained in IS 3309, IS 4335 and IS 7809 (HDLC of ISO) as well as ITU-T Recommendations X.25, Q.921 and Q.922 (LAP-B and LAP-D, respectively). RLP has been tailored to the special needs of digital radio transmission. RLP provides to its users the OSI data link service (IS 8886).

##### 5.1.2.12.4.7	TS 24.080

Mobile radio interface layer 3 supplementary services specification; Formats and coding

This specification describes the coding of information necessary for support of supplementary service operation on the mobile radio interface L3. Details are specified in other TS.

##### 5.1.2.12.4.8	TS 24.081

Line Identification supplementary services; Stage 3

This document specifies the procedures used at the radio interface for normal operation, registration, erasure, activation, deactivation, invocation and interrogation of line identification supplementary services. Provision and withdrawal of supplementary services is an administrative matter between the mobile subscriber and the service provider and cause no signalling on the radio interface.

In 3GPP TS 24.010 the general aspects of the specification of supplementary services at the layer 3 radio interface are given. 3GPP TS 24.080 specifies the formats and coding for the supplementary services.

Definitions and descriptions of supplementary services are given in 3GPP TS 22.004 and 3GPP TS 22.08x and 3GPP TS 22.09x series. 3GPP TS 22.081 is related specially to line identification supplementary services:

–   Calling line identification presentation (CLIP);

–   Calling line identification restriction (CLIR);

–   Connected line identification presentation (COLP);

–   Connected line identification restriction (COLR).

##### 5.1.2.12.4.9	TS 24.082

Call Forwarding (CF) supplementary services; Stage 3

This document specifies the procedures used at the radio interface (reference point Um as defined in 3GPP TS 24.002) for normal operation, registration, erasure, activation, deactivation, interrogation, and network invocation of call offering supplementary services.

In 3GPP TS 24.010, the general aspects of the specification of supplementary services at the layer 3 radio interface are given.

3GPP TS 24.080 specifies the formats and coding for the supplementary services.

Definitions and descriptions of supplementary services are given in 3GPP TS 22.004 and 3GPP TS 22.08x and 3GPP TS 22.09x series. 3GPP TS 22.082 is related specially to call offering supplementary services.

Technical realization of supplementary services is described in 3GPP TS 23.011 and 3GPP TS 23.08x and 3GPP TS 23.09x series. 3GPP TS 23.082 is related specially to call offering supplementary services.

The procedures for Call Control, Mobility Management and Radio Resource management at the layer 3 radio interface are defined in 3GPP TS 24.007 and 3GPP TS 24.008. The following supplementary services belong to the call offering supplementary services and are described in this document:

–   Call forwarding unconditional (CFU);

–   Call forwarding on mobile subscriber busy (CFB);

–   Call forwarding on no reply (CFNRy);

–   Call forwarding on mobile subscriber not reachable (CFNRc).

##### 5.1.2.12.4.10	TS 24.083

Call Waiting (CW) and Call Hold (HOLD) supplementary services; Stage 3

This document specifies the procedures used at the radio interface (reference point Um as defined in 3GPP TS 24.002) for normal operation, registration, erasure, activation, deactivation, invocation and interrogation of call completion supplementary services.

In 3GPP TS 24.010 the general aspects of the specification of supplementary services at the layer 3 radio interface are given.

3GPP TS 24.080 specifies the formats and coding for the supplementary services.

Definitions and descriptions of supplementary services are given in 3GPP TS 22.004 and 3GPP TS 22.08x and 3GPP TS 22.09x series. 3GPP TS 22.083 is related specially to call completion supplementary services.

Technical realization of supplementary services is described in 3GPP TS 23.011 and GSM 03.8x and GSM 03.9x series.

3GPP TS 23.083 is related specially to call completion supplementary services.

The procedures for Call Control, Mobility Management and Radio Resource management at the layer 3 radio interface are defined in 3GPP TS 24.007 and 3GPP TS 24.008.

The following supplementary services belong to the call completion supplementary services and are described in this document:

–   Call waiting (CW);

–   Call hold (HOLD).

##### 5.1.2.12.4.11	TS 24.084

Multi Party (MPTY) supplementary service; Stage 3

This document specifies the procedures used at the radio interface (reference point Um as defined in 3GPP TS 24.002) for normal operation and invocation of Multi Party supplementary services.

In 3GPP TS 24.010 the general aspects of the specification of supplementary services at the layer 3 radio interface are given.

3GPP TS 24.080 specifies the formats and coding for the supplementary services.

Definitions and descriptions of supplementary services are given in 3GPP TS 22.004 and the 3GPP TS 22.08x and 3GPP TS 22.09x series.

3GPP TS 22.084 is related specially to Multi Party supplementary services.

Technical realization of supplementary services is described in 3GPP TS 23.011 and the 3GPP TS 23.08x and 3GPP TS 23.09x series.

3GPP TS 23.084 is related specially to Multi Party supplementary services.

The procedures for Call Control, Mobility Management and Radio Resource management at the layer 3 radio interface are defined in 3GPP TS 24.007 and 3GPP TS 24.008.

The following supplementary service belongs to the Multi Party supplementary services and is described in this document:

–   Multi Party service (MPTY).

##### 5.1.2.12.4.12	TS 24.085

Closed User Group (CUG) supplementary service; Stage 3

This document specifies the procedures used at the radio interface (reference point Um as defined in 3GPP TS 24.002) for normal operation, registration, erasure, activation, deactivation, invocation and interrogation of community of interest supplementary services.

In 3GPP TS 24.010, the general aspects of the specification of supplementary services at the layer 3 radio interface are given.

3GPP TS 24.080 specifies the formats coding for the supplementary services.

Definitions and descriptions of supplementary services are given in 3GPP TS 22.004 and 3GPP TS 22.08x and 3GPP TS 22.09x-series. 3GPP TS 22.085 is related to the community of interest supplementary services.

Technical realization of supplementary services is described in technical specifications 3GPP TS 23.011 and 3GPP TS 23.08x and 3GPP TS 23.09x-series. 3GPP TS 23.085 is related to the community of interest supplementary services.

The procedures for Call Control, Mobility Management and Radio Resource management at the layer 3 radio interface are defined in 3GPP TS 24.007 and 3GPP TS 24.008.

Signalling interworking for supplementary services between 3GPP TS 29.002 and 3GPP TS 24.008 and between 3GPP TS 29.002 and 3GPP TS 24.080 is defined in 3GPP TS 29.011.

The following supplementary services belong to the community of interest supplementary services and are described in this TS:

–   Closed User Group (CUG).

##### 5.1.2.12.4.13	TS 24.086

Advice of Charge (AoC) supplementary services; Stage 3

This document specifies the procedures used at the radio interface (reference point Um as defined in 3GPP TS 24.002) for normal operation, registration, erasure, activation, deactivation, invocation and interrogation of charging supplementary services.

In 3GPP TS 24.010 the general aspects of the specification of supplementary services at the layer 3 radio interface are given.

3GPP TS 24.080 specifies the formats and coding for the supplementary services.

Definitions and descriptions of supplementary services are given in 3GPP TS 22.004 and 3GPP TS 22.08x and 3GPP TS 22.09x series. 3GPP TS 22.024 and 02.86 are related to the charging supplementary services.

Technical realization of supplementary services is described in 3GPP TS 23.011 and 3GPP TS 23.08x and 3GPP TS 23.09x series. 3GPP TS 23.086 is related to the charging supplementary services.

The procedures for Call Control, Mobility Management and Radio Resource management at the layer 3 radio interface are defined in 3GPP TS 24.007 and 3GPP TS 24.008.

The following supplementary services belong to the charging supplementary services and are described in this document:

–   Advice of Charge (Information) (AoCI);

–   Advice of Charge (Charging) (AoCC).

##### 5.1.2.12.4.14	TS 24.087

User-to-User Signalling (UUS) Supplementary Service; Stage 3

This document gives the stage 3 description of the three User-to-User signalling supplementary services.

##### 5.1.2.12.4.15	TS 24.088

Call Barring (CB) supplementary service; Stage 3

This document specifies the procedures used at the radio interface (reference point Um as defined in 3GPP TS 24.002) for normal operation, registration, erasure, activation, deactivation, invocation and interrogation of call barring supplementary services.

In 3GPP TS 24.010 the general aspects of the specification of supplementary services at the layer 3 radio interface are given.

3GPP TS 24.080 specifies the formats and coding for the supplementary services.

Definitions and descriptions of supplementary services are given in 3GPP TS 22.004, 22.08x and 22.09x series.

Technical realization of supplementary services is described in 3GPP TS 23.011, 23.08x and 23.09x series.

The procedures for Call Control, Mobility Management and Radio Resource management at the layer 3 radio interface are defined in 3GPP TS 24.007 and 3GPP TS 24.008.

The following supplementary services belong to the call restriction supplementary services and are described in this specification:

–   Barring of outgoing calls;

–   Barring of incoming calls

##### 5.1.2.12.4.16	TS 24.090

Unstructured Supplementary Service Data (USSD); Stage 3

This document gives the stage 3 description of the Unstructured Supplementary Service Data (USSD) operations. The group of unstructured supplementary service data operations is divided into two different classes:

–   Network initiated unstructured supplementary service data operations;

–   Mobile initiated unstructured supplementary service data operations.

##### 5.1.2.12.4.17	TS 24.091

Explicit Call Transfer (ECT) supplementary service; Stage 3

This document gives the stage 3 description of the call transfer supplementary services. It specifies the procedures used at the radio interface (reference point Um as defined in 3GPP TS 24.002) for normal operation, registration, erasure, activation, deactivation, invocation and interrogation of call transfer supplementary services.

In 3GPP TS 24.010 the general aspects of the specification of supplementary services at the layer 3 radio interface are given.

3GPP TS 24.080 specifies the formats and coding for the supplementary services.

Definitions and descriptions of supplementary services are given in 3GPP TS 22.004, 3GPP TS 22.08x and 3GPP TS 22.09x series. 3GPP TS 22.091 is related specifically to call transfer supplementary services.

The technical realization of supplementary services is described in 3GPP TS 23.011, 3GPP TS 23.08x and 3GPP TS 23.09x series. 3GPP TS 23.091 is related specifically to call transfer supplementary services.

The procedures for Call Control, Mobility Management and Radio Resource management at the layer 3 radio interface are defined in 3GPP TS 24.007 and 3GPP TS 24.008.

The following supplementary services belong to the call transfer supplementary services and are described in this document:

–   Explicit Call Transfer (ECT).

##### 5.1.2.12.4.18	TS 24.093

Call Completion of Calls to Busy Subscriber (CCBS); Stage 3

This document gives the stage 3 description of the Completion of Calls to Busy Subscriber (CCBS) supplementary service. It specifies the procedures used at the radio interface (reference point Um as defined in 3GPP TS 24.002) for normal operation, activation, deactivation, invocation and interrogation of the completion of calls to busy subscriber supplementary services.

In 3GPP TS 24.010 the general aspects of the specification of supplementary services at the layer 3 radio interface are given.

3GPP TS 24.080 specifies the formats and coding for the supplementary services.

Definitions and descriptions of supplementary services are given in 3GPP TS 22.004, 3GPP TS 22.08x and 3GPP TS 22.09x-series. Technical specification 3GPP TS 22.093 is related specifically to the Completion of Calls to Busy Subscriber supplementary service.

The technical realization of supplementary services is described in technical specifications 3GPP TS 23.011, 3GPP TS 23.08x and 3GPP TS 23.09x_series. 3GPP TS 23.093 is related specifically to Completion of Calls to Busy Subscriber supplementary service.

The procedures for Call Control, Mobility Management and Radio Resource management at the layer 3 radio interface are defined in 3GPP TS 24.007 and 3GPP TS 24.008.

The following supplementary services belong to the call completion supplementary services and are described in this document:

–   Completion of Calls to Busy Subscriber (CCBS).

##### 5.1.2.12.4.19	TS 24.096

Name Identification supplementary services; Stage 3

This document specifies the procedures used at the radio interface for normal operation, registration, erasure, activation, deactivation, invocation and interrogation of name identification supplementary services. In 3GPP TS 24.010 the general aspects of the specification of supplementary services at the layer 3 radio interface are given. 3GPP TS 24.080 specifies the formats and coding for the supplementary services.

Definitions and descriptions of supplementary services are given in 3GPP TS 22.004 and 3GPP TS 22.08x and 3GPP TS 22.09x series. 3GPP TS 22.096 is related specially to name identification supplementary services.

Technical realization of supplementary services is described in 3GPP TS 23.011 and 3GPP TS 23.08x and 3GPP TS 23.09x series. Technical specification 3GPP TS 23.096 is related specially to name identification supplementary services.

The procedures for Call Control, Mobility Management and Radio Resource management at the layer 3 radio interface are defined in 3GPP TS 24.007 and 3GPP TS 24.008.

The following supplementary services belong to the name identification supplementary services and are described in this TS:

–   Calling name presentation (CNAP).

##### 5.1.2.12.4.20	TS 24.103

Telepresence using the IP Multimedia (IM) Core Network (CN) Subsystem (IMS); Stage 3

This document provides the protocol details for telepresence using the IP Multimedia (IM) Core Network (CN) subsystem based on the Session Initiation Protocol (SIP), the Session Description Protocol (SDP), the Binary Floor Control Protocol (BFCP) and the ControLling mUltiple streams for tElepresence (CLUE) controlling multiple streams for telepresence based on service requirements.

This document addresses the areas of describing and negotiating IM session with multiple media streams based on the IM CN subsystem, including point to point calls as specified in 3GPP TS 24.229 and multiparty conferences as specified in 3GPP TS 24.147, to facilitate the support of telepresence.

The functionalities for conference policy control and the signalling between a MRFC and a MRFP are not specified in this document.

Where possible, this document specifies the requirements for this protocol by reference to specifications produced by the IETF within the scope of SIP, SDP, CLUE and BFCP, either directly, or as modified by 3GPP TS 24.229.

This document is applicable to Application Servers (ASs), Multimedia Resource Function Controllers (MRFCs), Multimedia Resource Function Processors (MRFP) and User Equipment (UE) providing IM session supporting telepresence capabilities.

##### 5.1.2.12.4.21	TS 24.105

Application specific Congestion control for Data Communication (ACDC) Management Object (MO)

This document defines Management Object (MO) that is used to configure the UE with parameters related to Application specific Congestion control for Data Communication (ACDC) functionality.

The MO is compatible with the OMA Device Management (DM) protocol specifications, version 1.2 and upwards, and is defined using the OMA DM Device Description Framework (DDF) as described in the Enabler Release Definition OMA-ERELD-DM-V1_2.

The MO consists of relevant parameters for provisioning of ACDC at a UE.

##### 5.1.2.12.4.22	TS 24.109

Bootstrapping interface (Ub) and network application function interface (Ua); Protocol details

This document defines stage 3 for the HTTP Digest AKA based implementation of Ub interface (UE-BSF), the Disposable-Ks model based implementation of Upa interface (NAF-UE) and the HTTP Digest and the PSK TLS based implementation of bootstrapped security association usage over Ua interface (UE-NAF) in Generic Authentication Architecture (GAA) as specified in 3GPP TS 33.220. The purpose of the Ub interface is to create a security association between UE and BSF for further usage in GAA applications. The purpose of the Upa interface is to provide a push mechanism to create a bootstrapped security association between the UE and NAF for secure communication of pushed messages. The purpose of the Ua interface is to use the so created bootstrapped security association between UE and NAF for secure communication.

This document also defines stage 3 for the Authentication Proxy usage as specified in 3GPP TS 33.222.

This document also defines stage 3 for the subscriber certificate enrolment as specified in 3GPP TS 33.221 which is one realization of the Ua interface. The subscriber certificate enrolment uses the HTTP Digest based implementation of bootstrapped security association usage to enrol a subscriber certificate and the delivery of a CA certificate.

##### 5.1.2.12.4.23	TS 24.116

Stage 3 aspects of system architecture enhancements for TV services

This document defines the stage 3 aspects of the architecture enhancements for broadcast TV service (see 3GPP TS 23.246).

This document specifies:

-	the requirements on the device in Receive only mode;

-	the procedure for selecting a network to receive broadcast TV service; and

-	the usage of the information configured in the TV service configuration.

The clause 4 applies to a device in Receive only mode only. The remaining clauses are applicable to:

-	a 3GPP UE wishing to receive broadcast TV service; and

-	a device in Receive only mode.

##### 5.1.2.12.4.24	TS 24.117

TV service configuration Management Object (MO)

This document defines a Management Object (MO) that can be used to configure the parameters related to TV service provided via a PLMN.

The MO is compatible with the OMA Device Management (DM) protocol specifications, version 1.2 and upwards, and is defined using the OMA DM Device Description Framework (DDF) as described in the Enabler Release Definition OMA-ERELD-DM-V1_2.

The MO consists of nodes and leaves conveying configuration parameters used for selecting and receiving TV service from a PLMN, according to the procedures described in 3GPP TS 23.246 and 3GPP TS 24.116.

##### 5.1.2.12.4.25	TS 24.139

3GPP system - fixed broadband access network interworking; Stage 3

This document specifies the UE – EPC procedures for accessing 3GPP Evolved Packet Core (EPC) via a Fixed Broadband Access network. 
The specification covers the Support of QoS, Tunnel management procedures including the support of NAT traversal, and non-seamless offloading impacts. The document is applicable to the UE and the network (i.e. the 3GPP EPC).

##### 5.1.2.12.4.26	TS 24.141

Presence service using the IP Multimedia (IM) Core Network (CN) subsystem; Stage 3

This specification provides the protocol details for the presence service within the IP Multimedia (IM) Core Network (CN) subsystem based on the Session Initiation Protocol (SIP) and SIP Events as defined in 3GPP TS 24.229.

##### 5.1.2.12.4.27	TS 24.147

Conferencing using the IP Multimedia (IM) Core Network (CN) subsystem; Stage 3

This specification provides the protocol details for conferencing within the IP Multimedia Core Network subsystem (IMS) based on the Session Initiation Protocol (SIP), SIP Events, the Session Description Protocol (SDP) and the Binary Floor Control Protocol (BFCP).

##### 5.1.2.12.4.28	TS 24.161

Network-Based IP Flow Mobility (NBIFOM); Stage 3

This document specifies the procedures of network-based IP flow mobility (NBIFOM). NBIFOM is based on network mobility protocols (i.e. GTP or PMIP) which is performed between 3GPP access and WLAN access network as specified in 3GPP TS 23.161.

NOTE:	IP flow mobility based on PMIP is not supported in this release of the specification.

##### 5.1.2.12.4.29	TS 24.166

3GPP IP Multimedia Subsystem (IMS) conferencing Management Object (MO)

This document defines the IMS conferencing management object. The management object is compatible with OMA Device Management protocol specifications, version 1.2 and upwards, and is defined using the OMA DM Device Description Framework as described in the Enabler Release Definition OMA-ERELD_DM-V1_2. The IMS conferencing management object consists of relevant parameters that can be managed for IMS conferencing capabilities.

##### 5.1.2.12.4.30	TS 24.167

3GPP IMS Management Object (MO); Stage 3

This document defines a mobile device 3GPP IMS Management Object. The management object is compatible with OMA Device Management protocol specifications, version 1.2 and upwards, and is defined using the OMA DM Device Description Framework as described in the Enabler Release Definition OMA-ERELD _DM-V1_2. The 3GPP IMS Management Object consists of relevant parameters that can be managed for the IM CN Subsystem. 
This includes the basic framework defined in 3GPP TS 23.228 and 3GPP TS 24.229, and early IMS as defined in 3GPP TS 23.221. This also includes relevant parameters that can be managed for the application of SMS over IP networks defined in 3GPP TS 24.341.

The IMS Management Object defines a repository of data into the ME including parameters that are provisioned from the ISIM application (i.e. 3GPP TS 31.103) or, after derivation, from the USIM application (i.e. 3GPP TS 31.102).

##### 5.1.2.12.4.31	TS 24.171

Control Plane Location Services (LCS) procedures in the Evolved Packet System (EPS)

This document specifies the operations and information coding for the Non-access Stratum (NAS) layer protocol for supporting the Location Services (LCS) in the Evolved Universal Terrestrial Radio Access Network (E-UTRAN). This document is applicable to the User Equipment (UE) and to the Mobility Management Entity (MME) in the Evolved Packet System (EPS). This document is developed in accordance to the general principles stated in 3GPP TS 23.271

##### 5.1.2.12.4.32	TS 24.173

IMS Multimedia telephony communication service and supplementary services; Stage 3

This specification provides the protocol details for multimedia telephony communication service and associated supplementary services in the IP Multimedia (IM) Core Network (CN) subsystem based on the requirements from 3GPP TS 22.173. Multimedia telephony and supplementary services allow users to establish communications between them and enrich that by enabling supplementary services.

##### 5.1.2.12.4.33	TS 24.182

IP Multimedia Subsystem (IMS) Customized Alerting Tones (CAT); Protocol specification

This document provides the protocol details for the Customized Alerting Tones (CAT) service in the IP Multimedia (IM) Core Network (CN) subsystem based on the requirements from 3GPP TS 22.182. The CAT service is an operator specific service by which an operator enables the subscriber to customize the media which is played to the calling party during alerting of the called party.

##### 5.1.2.12.4.34	TS 24.183

IP Multimedia Subsystem (IMS) Customized Ringing Signal (CRS); Protocol specification

The specification provides the protocol details for the Customized Ringing Signal (CRS) service in the IP Multimedia (IM) Core Network (CN) subsystem based on the requirements from 3GPP TS 22.183. The CRS service is an operator specific service by which an operator enables the subscriber to customize the media which is played to the called party as an incoming communication indication during establishment of a communication. This document is applicable to User Equipment (UE) and Application Servers (AS) which are intended to support the CRS service.

##### 5.1.2.12.4.35	TS 24.216

Communication Continuity Management Object (MO)

This document defines the Communication Continuity Management Object. The management object is compatible with OMA Device Management protocol specifications, version 1.2 and upwards, and is defined using the OMA DM Device Description Framework as described in the Enabler Release Definition OMA-ERELD _DM-V1_2. The Communication Continuity Management Object consists of relevant parameters that can be managed for Communication Continuity capabilities.

##### 5.1.2.12.4.36	TS 24.229

IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3

This specification document defines a call control protocol for use in the IP Multimedia (IM) Core Network (CN) subsystem based on the Session Initiation Protocol (SIP), and the associated Session Description Protocol (SDP).

##### 5.1.2.12.4.37	TS 24.234

3GPP system to Wireless Local Area Network (WLAN) interworking; WLAN User Equipment (WLAN UE) to network protocols; Stage 3

The document specifies the network selection, including authentication and access authorization using authentication, authorization and accounting (AAA) procedures used for the interworking of the 3GPP system and WLANs. In addition to these it also specifies the tunnel management procedures used for establishing an end-to-end tunnel from the WLAN UE to the 3GPP network via the Wu reference point. The document is applicable to the WLAN user equipment (UE) and the network. In this technical specification the network includes the WLAN and 3GPP network. Furthermore, the document specifies procedures within I-WLAN necessary in order for IMS emergency calls to be supported when I-WLAN is used as the underlying access network. These involve both network selection as well as tunnel management procedures.

##### 5.1.2.12.4.38	TS 24.235

3GPP System to Wireless Local Area Network (WLAN) interworking Management Object (MO)

This document defines the 3GPP System to Wireless Local Area Network interworking Management Object (MO) for I-WLAN PLMN selection as specified in 3GPP TS 24.234. The 3GPP System to Wireless Local Area Network interworking Management Object consists of relevant parameters that can be managed for WLAN UE as specified in 3GPP TS 24.234.

##### 5.1.2.12.4.39	TS 24.237

IP Multimedia (IM) Core Network (CN) subsystem IP Multimedia Subsystem (IMS) service continuity; Stage 3

This specification provides the protocol details for voice call continuity between the IP Multimedia (IM) Core Network (CN) subsystem based on the Session Initiation Protocol (SIP) and the Session Description Protocol (SDP) and the protocols of the 3GPP Circuit-Switched (CS) domain (CAP, MAP, ISUP, BICC and the NAS call control protocol for the CS access). Voice call continuity allows users to move between the CS domain and the IP Connectivity Access Network (e.g. WLAN interworking) with home IM CN subsystem functionality.

##### 5.1.2.12.4.40	TS 24.238

Session Initiation Protocol (SIP) based user configuration; Stage 3

This document provides a Session Initiation Protocol (SIP) based protocol framework that serves as a means of user configuration of supplementary services in the IP Multimedia (IM) Core Network (CN) subsystem. The protocol framework relies upon the contents of the Request-URI in a SIP INVITE request to enable basic configuration of services without requiring use of the Ut interface.

##### 5.1.2.12.4.41	TS 24.239

Flexible Alerting (FA) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification

This document provides the protocol details for the Flexible Alerting supplementary service in the IP Multimedia (IM) Core Network (CN) subsystem. Flexible Alerting (FA) causes a call to a pilot identity to branch the call into several legs to alert several termination addresses (group members) simultaneously. The first leg to be answered is connected to the calling party. The other call legs are abandoned. The document is applicable to User Equipment (UE) and Application Servers (AS) which are intended to support the FA supplementary service.

##### 5.1.2.12.4.42	TS 24.244

Wireless LAN control plane protocol for trusted WLAN access to EPC; Stage 3

This document specifies the procedures of the Wireless LAN control plane protocol (WLCP) for trusted WLAN access to EPC which is used between User Equipment (UE) and Trusted WLAN Access Gateway (TWAG) for multi-connection mode as specified in 3GPP TS 23.402.

This document also defines the message format, information elements coding, error handling and system parameters applied by the WLCP protocol.

##### 5.1.2.12.4.43	TS 24.247

Messaging service using the IP Multimedia (IM) Core Network (CN) subsystem; Stage 3

This specification provides the protocol details for the messaging service within the IP Multimedia CN Subsystem (IMS) based on the Session Initiation Protocol (SIP), the Session Description Protocol (SDP) and, the Message Session Relay Protocol (MSRP).

##### 5.1.2.12.4.44	TS 24.250

Protocol for Reliable Data Service between UE and SCEF; Stage 3

This document specifies a Reliable Data Service (RDS) protocol between the UE and Service Capability Exposure Function (SCEF).

This document defines the frame structure, format of fields and procedures for operation of the RDS protocol. RDS is mainly intended to be used for acknowledged data transfer, but it also supports unacknowledged data transfer.

This document is applicable to the UE and to the SCEF in the Evolved Packet System (EPS).

##### 5.1.2.12.4.45	TS 24.259

Personal Network Management (PNM); Stage 3

Personal Network Management (PNM) is a home network-based application and provides the home network-based management of Personal Network (PN) consisting of multiple devices belonging to a single user, as described in 3GPP TS 22.259 and 3GPP TS 23.259. This document provides the protocol details for enabling Personal Network management (PNM) services in the IP Multimedia (IM) Core Network (CN) subsystem based on the protocols of XML. Configuration Access Protocol (XCAP), Session Initiation Protocol (SIP) and the Session Description Protocol (SDP). The document provides the protocol details for enabling Personal Network management (PNM) services in Circuit Switched (CS) domain based on the protocols of CAP, MAP, ISUP, USSD and BICC.

##### 5.1.2.12.4.46	TS 24.275

Management Object (MO) for Basic Communication Part (BCP) of IMS Multimedia Telephony (MMTEL) communication service

This document defines the management object (MO) for Basic Communication Part (BCP) of IMS Multimedia Telephony (MMTEL) communication service.

The MO for BCP of MMTEL communication service is compatible with OMA device management protocol specifications, version 1.2 and upwards, and is defined using the OMA DM device description framework as described in the enabler release definition OMA-ERELD_DM-V1_2.

The MO for BCP of MMTEL communication service consists of relevant configuration parameter that can be managed for a UE supporting the timer RequestTimeout specified in 3GPP TS 24.173.

##### 5.1.2.12.4.47	TS 24.279

Combining Circuit Switched (CS) and IP Multimedia Subsystem (IMS) services; Stage 3

This specification provides the technical realization for the combination of Circuit Switched calls and IM sessions when using them simultaneously between the same two users. It also describes the use of CS and IM services in combination, using the existing procedures that have been defined for CS and IMS.

It includes the necessary function as adding an IM session to an ongoing CS call, adding a CS call to an ongoing IM session, supplementary services as they relate to CSICS and supporting capability exchange.

##### 5.1.2.12.4.48	TS 24.281

Mission Critical Video (MCVideo) signalling control; Protocol specification

This document specifies the signalling control needed to support Mission Critical Video (MCVideo) service.

The MCVideo service and its associated media plane control protocols can be used for public safety applications and also for general commercial applications (e.g., utility companies and railways).

This document is applicable to User Equipment (UE) supporting MCVideo client, and MCVideo server.

##### 5.1.2.12.4.49	TS 24.282

Mission Critical Data (MCData) signalling control; Protocol specification

This document specifies the signalling control protocols needed to support Mission Critical Data (MCData) communications as specified by 3GPP TS 23.282. This document specifies both on-network and off-network protocols.

This document utilises the common functional architecture to support mission critical services as specified in 3GPP TS 23.280, in support of MCData communications.

The MCData service can be used for public safety applications and also for general commercial applications e.g. utility companies and railways.

This document is applicable to User Equipment (UE) supporting the MCData client functionality, and to application servers supporting the MCData server functionality.

##### 5.1.2.12.4.50	TS 24.285

Allowed Closed Subscriber Group (CSG) list; Management Object (MO)

This document specifies an Allowed Closed Subscriber Group (CSG) List Management Object (MO). The Allowed CSG List MO is compatible with the OMA Device Management (DM) protocol specifications, version 1.2 and upwards, and is defined using the OMA DM Device Description Framework (DDF) as described in the Enabler Release Definition OMA-ERELD-DM-V1_2. The Allowed CSG List MO consists of relevant parameters for that can be used by the UE to select the appropriate CSG cell based on its subscription. The Allowed CSG List MO defines the relevant parameters related to the Allowed CSG List and to the Operator CSG List. The usage of the Allowed CSG List for in the idle mode process and the mobility management procedure is defined in 3GPP TS 23.122, 3GPP TS 24.008 and 3GPP TS 24.301.

##### 5.1.2.12.4.51	TS 24.286

IP Multimedia (IM) Core Network (CN) subsystem Centralized Services (ICS); Management Object (MO)

This document defines the IMS Centralised Services Management Object (MO). The management object is compatible with OMA Device Management protocol specifications, version 1.2 and upwards, and is defined using the OMA DM Device Description Framework as described in the Enabler Release Definition OMA-ERELD _DM-V1_2. The IMS Centralised Services Management Object consists of relevant parameters that can be managed for ICS.

##### 5.1.2.12.4.52	TS 24.292

IP Multimedia (IM) Core Network (CN) subsystem Centralized Services (ICS); Stage 3

IP Multimedia (IM) Core Network (CN) subsystem centralized services (ICS) allow for the delivery of consistent IMS services to the user regardless of the attached access type (e.g. CS domain access or IP-CAN). This document provides the protocol details for the realization of ICS based on the Session Initiation protocol (SIP), the Session Description Protocol (SDP), the I1 protocol, and the protocols of the 3GPP Circuit-Switched (CS) domain (e.g. CAP, MAP, ISUP, BICC and the NAS call control protocol for the CS access).

##### 5.1.2.12.4.53	TS 24.294

IP Multimedia Subsystem (IMS) Centralized Services (ICS) protocol via I1 interface

This document describes the I1 interface between IMS Centralized Services (ICS) UE and Service Centralization and Continuity (SCC) Application Server (AS). This specification defines a new application layer protocol over I1 interface, specifies the interaction between the ICS UE and the SCC AS including session control procedures and supplementary services control procedures. The protocol is intended to be independent of the transport protocol used so it can be applied to a number of technologies that need different transport protocols. The overall ICS architecture is specified in 3GPP TS 23.292

##### 5.1.2.12.4.54	TS 24.301

Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3

This document specifies the procedures used by the protocols for mobility management and session management between User Equipment (UE) and Mobility Management Entity (MME) in the Evolved Packet System (EPS). These protocols belong to the non-access stratum (NAS).

##### 5.1.2.12.4.55	TS 24.302

Access to the 3GPP Evolved Packet Core (EPC) via non-3GPP access networks; Stage 3

This document specifies the discovery and network selection procedures for access to 3GPP Evolved Packet Core (EPC) via non-3GPP access networks and includes Authentication and Access Authorization using Authentication, Authorization and Accounting (AAA) procedures used for the interworking of the 3GPP EPC and the non-3GPP access networks. The document also specifies the Tunnel management procedures used for establishing an end-to-end tunnel from the UE to the ePDG to the point of obtaining IP connectivity and includes the selection of the IP mobility mode. The non-3GPP access networks considered in this document are cdma2000® HRPD and Worldwide Interoperability for Microwave Access (WiMAX), and any access technologies covered in 3GPP TS 23.402. These non-3GPP access networks can be trusted or untrusted access networks.

##### 5.1.2.12.4.56	TS 24.303

Mobility management based on Dual-Stack Mobile IPv6; Stage 3

This document specifies the signalling procedures for accessing the 3GPP Evolved Packet Core network and handling the mobility between 3GPP and non-3GPP accesses via the S2c reference point defined in 3GPP TS 23.402. The document is applicable to the User Equipment (UE) and the network node implementing the Home Agent functionality. In addition the document specifies the procedures used for the DSMIPv6 Home Agent discovery, for bootstrapping the DSMIPv6 security association between the UE and the Home Agent and for managing the DSMIPv6 tunnel. The specification of these procedures is compliant to IETF RFCs. DSMIPv6 procedures can be used independently of the underlying access technology.

##### 5.1.2.12.4.57	TS 24.304

Mobility management based on Mobile IPv4; User Equipment (UE) – foreign agent interface; Stage 3

This document describes the stage 3 aspects of mobility management for User Equipment (UE) using IETF Mobile IPv4 foreign agent mode to access the Evolved Packet Core Network (EPC) through trusted non-3GPP access networks and for mobility management of UE between the 3GPP access network and trusted non-3GPP access networks. In particular, the document describes the UE – Mobile IPv4 Foreign Agent (FA) interface stage 3 aspects, where the FA functionality is located within the access network in the non-3GPP access domain. The document is applicable to the User Equipment (UE) and the network node implementing the FA functionality.

##### 5.1.2.12.4.58	TS 24.305

Selective Disabling of 3GPP User Equipment Capabilities (SDoUE) Management Object (MO)

This document specifies a mobile device Selective Disabling of 3GPP UE Capabilities (SDoUE) Management Object (MO) and the rules and corresponding behaviour of the UE with regard to the selective disabling of 3GPP UE capabilities, for example, when services or functions are disabled/enabled. 
The SDoUE MO is compatible with the OMA Device Management (DM) protocol specifications, version 1.2 and upwards, and is defined using the OMA DM Device Description Framework (DDF) as described in the Enabler Release Definition OMA-ERELD-DM-V1_2. The SDoUE MO consists of the relevant parameters that can be managed for selective disabling of 3GPP UE capabilities. The SDoUE MO defines a repository of data into the ME. 
The service requirements for selective disabling of 3GPP UE capabilities are defined in 3GPP TS 22.011.

##### 5.1.2.12.4.59	TS 24.312

Access Network Discovery and Selection Function (ANDSF) Management Object (MO)

This document defines management objects that can be used by the Access Network Discovery and Selection Function (ANDSF) and the UE. The Management Object (MO) is compatible with the OMA Device Management (DM) protocol specifications, version 1.2 and upwards, and is defined using the OMA DM Device Description Framework (DDF) as described in the Enabler Release Definition OMA-ERELD-DM-V1_2. The MO consists of relevant parameters for intersystem mobility policy- and access network discovery information that can be managed by the ANDSF. The service requirements and the functional requirements for the access network discovery and selection are described in 3GPP TS 22.278 and in 3GPP TS 23.402 respectively.

##### 5.1.2.12.4.60	TS 24.315

IP Multimedia Subsystem (IMS) Operator Determined Barring (ODB); Stage 3: protocol specification

This document specifies the stage three, Protocol Description of the network feature Operator Determined Barring (ODB) of IMS Multimedia Telephony Services. It is based on the requirements described in 3GPP TS 22.041. It provides the protocol details in the IP Multimedia (IM) Core Network (CN) subsystem based on the Session Initiation Protocol (SIP) and the Session Description Protocol (SDP). In addition, it provides protocol details for XCAP for barring of supplementary services configuration.

##### 5.1.2.12.4.61	TS 24.322

Tunnelling of IP Multimedia Subsystem (IMS) services over restrictive access networks; Stage 3

This document specifies procedures and protocol elements for tunnelling of IMS traffic over restrictive access networks, specifically procedures and protocol elements for establishing, maintaining, and sending traffic via a firewall traversal tunnel between the UE and an enhanced firewall traversal function (EFTF) in the network. This document is applicable to UE and EFTF.

This document applies only to the case when the IMS traffic is not routed via EPC of a PLMN. Procedures for tunnelling of IMS traffic that is routed via EPC are specified in 3GPP TS 24.302 annex F.

##### 5.1.2.12.4.62	TS 24.327

Mobility between 3GPP Wireless Local Area Network (WLAN) interworking (I-WLAN) and 3GPP systems; General Packet Radio System (GPRS) and 3GPP I-WLAN aspects; Stage 3

This document specifies the signalling procedures for handling the mobility of a UE between 3GPP Wireless Local Area Network Interworking (I-WLAN) and GPRS systems and is applicable to the User Equipment (UE) and the network nodes supporting mobility between 3GPP I-WLAN and GPRS systems. In addition, it specifies the procedures used between the UE and the network nodes for the attach and the detach cases. It also specifies how the UE performs handover when moving from 3GPP I-WLAN to GPRS systems and vice-versa.

##### 5.1.2.12.4.63	TS 24.333

Proximity-services (ProSe) Management Objects (MO)

This document defines Management Objects (MO) that are used to configure the ProSe-enabled UE.

The MOs are compatible with the OMA Device Management (DM) protocol specifications, version 1.2 and upwards, and are defined using the OMA DM Device Description Framework (DDF) as described in the Enabler Release Definition OMA-ERELD-DM-V1_2.

The MOs consist of relevant parameters for provisioning and authorisation of ProSe at the ProSe-enabled UE.

The protocol aspects for ProSe are described in 3GPP TS 24.334.

In this release of the specification, ProSe direct discovery is supported only when the UE is served by E-UTRAN.

##### 5.1.2.12.4.64	TS 24.334

Proximity-services (ProSe) User Equipment (UE) to ProSe function protocol aspects; Stage 3

This document specifies the protocols for Proximity-based Services (ProSe) between:

- the ProSe-enabled UE and the ProSe Function (over the PC3 interface); and

- two ProSe-enabled UEs (over the PC5 interface).

This document defines the associated procedures for ProSe service authorisation, ProSe direct discovery, EPC-level ProSe discovery and ProSe direct communication.

This document also defines the message format, message contents, error handling and system parameters applied by the protocols for ProSe.

This document is applicable to:

- the ProSe-enabled UE; and

- the ProSe Function.

##### 5.1.2.12.4.65	TS 24.337

IP Multimedia (IM) Core Network (CN) subsystem IP Multimedia Subsystem (IMS) inter-UE transfer; Stage 3

IP Multimedia (IM) Core Network (CN) subsystem inter-UE transfer (IUT) provides the capability of continuing ongoing communication sessions with multiple media across different user equipments (UEs) under the control of the same or different subscribers, and as part of Service Continuity (SC). This document provides the protocol details for enabling IMS inter-UE transfer based on the Session Initiation protocol (SIP) and the Session Description Protocol (SDP).

##### 5.1.2.12.4.66	TS 24.341

Support of SMS over IP networks; Stage 3

This specification provides the protocol details for SMS over IP functionality within the IP Multimedia (IM) Core Network (CN) subsystem based on the Session Initiation Protocol (SIP). The architecture for the SMS over IP functionality is specified in 3GPP TS 23.204.

##### 5.1.2.12.4.67	TS 24.368

Non-Access Stratum (NAS) configuration Management Object (MO)

This document defines a Management Object (MO) that can be used to configure the UE with parameters related to Non-Access Stratum (NAS) functionality. The MO is compatible with the OMA Device Management (DM) protocol specifications, version 1.2 and upwards, and is defined using the OMA DM Device Description Framework (DDF) as described in the Enabler Release Definition OMA-ERELD-DM-V1_2. The MO consists of relevant parameters for NAS related configuration of a UE.

##### 5.1.2.12.4.68	TS 24.371

Web Real-Time Communications (WebRTC) access to the IP Multimedia (IM) Core Network (CN) subsystem (IMS); Stage 3; Protocol specification

This document provides the details for allowing Web Real-Time Communication (WebRTC) IMS Clients (WIC) to access the IP Multimedia (IM) Core Network (CN) subsystem.

This document is applicable to WebRTC IMS client (WIC), eP-CSCF, eIMS-AGW, WebRTC Web Server Function (WWSF) and WebRTC Authorization Function (WAF).

##### 5.1.2.12.4.69	TS 24.379

Mission Critical Push To Talk (MCPTT) call control; Protocol specification

This document specifies the session control protocols needed to support Mission Critical Push To Talk (MCPTT). This document specifies both on-network and off-network protocols.

Mission critical communication services are services that require preferential handling compared to normal telecommunication services, e.g. in support of police or fire brigade.

The MCPTT service can be used for public safety applications and also for general commercial applications (e.g., utility companies and railways).

This document is applicable to User Equipment (UE) supporting the MCPTT client functionality, and to application servers supporting the MCPTT server functionality.

##### 5.1.2.12.4.70	TS 24.380

Mission Critical Push To Talk (MCPTT) media plane control; Protocol specification

This document specifies the media plane control protocols and interactions with the media needed to support Mission Critical Push To Talk (MCPTT).

This document specifies protocol for using pre-established session to setup calls, floor control and managing MBMS subchannels over MBMS bearers on-network and off-network protocols for floor control.

Mission critical communication services are services that require preferential handling compared to normal telecommunication services, e.g. in support of police or fire brigade. Floor control provides a mechanism for managing the right to transmit at a point in time during an MCPTT call.

The MCPTT service and its associated media plane control protocols can be used for public safety applications and also for general commercial applications (e.g., utility companies and railways).

This document is applicable to User Equipment (UE) supporting the floor participant functionality, setting up calls using pre-established SIP sessions and using MBMS bearers for group communication and to floor control servers supporting these functions in the MCPTT system.

##### 5.1.2.12.4.71	TS 24.381

Mission Critical Push To Talk (MCPTT) group management; Protocol specification

This document specifies the group management protocols needed to support Mission Critical Push To Talk (MCPTT). Group management applies only when the UE operates on the network.

Mission critical communication services are services that require preferential handling compared to normal telecommunication services, e.g. in support of police or fire brigade.

The MCPTT service can be used for public safety applications and also for general commercial applications (e.g., utility companies and railways).

This document is applicable to User Equipment (UE) supporting the group management client (GMC) functionality, to application server supporting the group management server (GMS) functionality, and to application server supporting the MCPTT server functionality.

##### 5.1.2.12.4.72	TS 24.382

Mission Critical Push To Talk (MCPTT) identity management; Protocol specification

This document specifies the identity management and authentication protocols needed to support Mission Critical Push To Talk (MCPTT). Identity management applies only to on-network operation.

Mission critical communication services are services that require preferential handling compared to normal telecommunication services, e.g. in support of police or fire brigade.

The MCPTT service can be used for public safety applications and also for general commercial applications (e.g., utility companies and railways).

This document is applicable to User Equipment (UE) supporting the identity management client functionality, and to application servers supporting the identity management server functionality.

##### 5.1.2.12.4.73	TS 24.383

Mission Critical Push To Talk (MCPTT) Management Object (MO)

This document defines a number of Mission Critical Push To Talk (MCPTT) Management Objects (MO) that are configured for the MCPTT UE for the operation of the MCPTT Service. The management objects are compatible with OMA Device Management protocol specifications, version 1.2 and upwards, and is defined using the OMA DM Device Description Framework as described in the Enabler Release Definition OMA OMA-ERELD_DM-V1_2.

Mission critical communication services are services that require preferential handling compared to normal telecommunication services, e.g. in support of police or fire brigade.

The MCPTT service can be used for public safety applications and also for general commercial applications (e.g., utility companies and railways).

This document is applicable to an MCPTT UE supporting on-line, off-line or both on-line and off-line configuration of the configuration management client.

This document is applicable to an MCPTT UE supporting off-line configuration of the group management client.

The parameters defined by the management objects in this document are configured in the MCPTT UE using on-line configuration over the CSC-4 reference point and using off-line configuration over the CSC-11 and CSC-12 reference points. Other specifications define how these parameters are used in the operation of MCPTT, and whether they are applicable to on-network operation or off-network operation, or both:

-	3GPP TS 24.379

-	3GPP TS 24.380

-	3GPP TS 24.381

-	3GPP TS 24.382

-	3GPP TS 24.384

The common network operation means both on-network operation and off-network operation in this document.

The following management objects are defined in this document:

-	MCPTT UE configuration MO

-	MCPTT user profile MO

-	MCPTT group configuration MO

-	MCPTT service configuration MO

-	MCPTT UE initial configuration MO

The MOs listed above define repositories of data in the ME.

##### 5.1.2.12.4.74	TS 24.384

Mission Critical Push To Talk (MCPTT) configuration management; Protocol specification

This document specifies the configuration management documents and protocols needed to support Mission Critical Push To Talk (MCPTT) online configuration over the CSC-4 and CSC-5 reference points and the procedures to support Mission Critical Push To Talk (MCPTT) offline configuration over the CSC-11and CSC-12 reference points. Configuration management documents defined in this document includes:

MCPTT UE initial configuration document;

MCPTT UE configuration document;

MCPTT user profile configuration document; and

MCPTT service configuration document.

Mission critical communication services are services that require preferential handling compared to normal telecommunication services, e.g. in support of police or fire brigade.

The MCPTT service can be used for public safety applications and also for general commercial applications (e.g., utility companies and railways).

This document is applicable to an MCPTT UE supporting the configuration management client functionality, to application server supporting the configuration management server functionality, and to application server supporting the MCPTT server functionality.

##### 5.1.2.12.4.75	TS 24.385

V2X services Management Object (MO)

This document defines Management Object (MO) that is used to configure the UE for V2X services.

The MO is compatible with the OMA Device Management (DM) protocol specifications, version 1.2 and upwards, and is defined using the OMA DM Device Description Framework (DDF) as described in the Enabler Release Definition OMA-ERELD-DM-V1_2.

The MO consists of relevant parameters for provisioning and authorization of V2X communication at the UE.

The protocol aspects for V2X are described in 3GPP TS 24.386.

##### 5.1.2.12.4.76	TS 24.386

User Equipment (UE) to V2X control function; protocol aspects; Stage 3

This document specifies the protocols:

-	for V2X authorization between the UE and the V2X control function (over the V3 interface);

-	for V2X communication among the UEs (over the PC5 interface); and

-	for V2X communication between the UE and the V2X application server (over the LTE-Uu interface).

This document defines the associated procedures for V2X authorization and V2X communication.

This document also defines the message format, message contents, error handling and system parameters applied by the protocols for V2X.

This document is applicable to:

-	the UE;

-	the V2X control function; and

-	the V2X application server.

##### 5.1.2.12.4.77	TS 24.390

Unstructured Supplementary Service Data (USSD) using IP Multimedia (IM) Core Network (CN) subsystem IMS; Stage 3

This document describes the procedures for using Unstructured Supplementary Service Data (USSD) operations for mobile initiated MMI mode over IP Multimedia Core Network Subsystem (IMS). MMI mode is for the transparent transport of MMI strings entered by the user to the Application Servers (AS) and for the transparent transport of text strings back to the User Equipment (UE) to be displayed for user information.

Support of USSD service is optional and only applicable for an operator's Public Land Mobile Network (PLMN). This document is applicable to UE and AS which are intended to support USSD operations over IP Multimedia Core Network Subsystem (IMS) in mobile initiated MMI mode.

##### 5.1.2.12.4.78	TS 24.391

Unstructured Supplementary Service Data (USSD) using IP Multimedia (IM) Core Network (CN) subsystem (IMS) Management Object (MO)

This document defines the unstructured supplementary service data (USSD) using IP Multimedia (IM) Core Network (CN) subsystem (IMS) management object (MO). The MO is compatible with OMA device management protocol specifications, version 1.2 and upwards, and is defined using the OMA DM device description framework as described in the enabler release definition OMA-ERELD_DM-V1_2.

The USSD using IMS (USSI) MO consists of relevant configuration parameters that can be managed for the UE's USSI capabilities specified in 3GPP TS 24.390.

##### 5.1.2.12.4.79	TS 24.417

Management Object (MO) for Originating Identification Presentation (OIP) and Originating Identification Restriction (OIR) using IP Multimedia (IM) Core Network (CN) subsystem

This document defines a Management Object (MO) for Originating Identification Presentation (OIP) and Originating Identification Restriction (OIR). The management object is compatible with OMA Device Management protocol specifications, version 1.2 and upwards, and is defined using the OMA DM Device Description Framework as described in the Enabler Release Definition OMA-ERELD _DM-V1_2.

The OIP/OIR MO consists of relevant parameters that can be managed for the UE configuration related to the Originating Identification Presentation (OIP) and Originating Identification Restriction (OIR) service defined in 3GPP TS 24.607.

##### 5.1.2.12.4.80	TS 24.424

Management Object (MO) for Extensible Markup Language (XML) Configuration Access Protocol (XCAP) over the Ut interface for Manipulating Supplementary Services (SS)

This document defines the management object (MO) for extensible markup language (XML) configuration access protocol (XCAP) over the Ut interface for manipulating supplementary services (SS).

The MO for XCAP over Ut interface for manipulating SS is compatible with OMA device management protocol specifications, version 1.2 and upwards, and is defined using the OMA DM device description framework as described in the enabler release definition OMA-ERELD_DM-V1_2.

The MO for XCAP over Ut interface for manipulating SS consists of relevant configuration parameters that can be managed for a UE supporting the UE role specified in 3GPP TS 24.623.

##### 5.1.2.12.4.81	TS 24.481

Mission Critical Services (MCS) group management; Protocol specification

This document specifies the group management protocols needed to support Mission Critical Services (MCSs).

Group management applies only when the UE operates on the network.

MCSs are services that require preferential handling compared to normal telecommunication services, e.g. in support of police or fire brigade.

MCSs can be used for public safety applications and also for general commercial applications (e.g., utility companies and railways).

This document is applicable to User Equipment (UE) supporting the group management client (GMC) functionality, to application server supporting the group management server (GMS) functionality, and to application server supporting the Mission Critical Push To Talk (MCPTT) server functionality.

##### 5.1.2.12.4.82	TS 24.482

Mission Critical Services (MCS) identity management; Protocol specification

This document specifies the identity management and authentication protocols needed to support Mission Critical Services (MCSs). Identity management applies only to on-network operation.

MCSs are services that require preferential handling compared to normal telecommunication services, e.g. in support of police or fire brigade.

MCSs can be used for public safety applications and also for general commercial applications (e.g., utility companies and railways).

This document is applicable to User Equipment (UE) supporting the identity management client functionality, and to application servers supporting the identity management server functionality.

##### 5.1.2.12.4.83	TS 24.483

Mission Critical Services (MCS) Management Object (MO)

This document defines a number of Mission Critical Services (MCSs) Management Objects (MO) that are configured for the UE for the operation of MCSs. The management objects are compatible with OMA Device Management protocol specifications, version 1.2 and upwards, and is defined using the OMA DM Device Description Framework as described in the Enabler Release Definition OMA OMA-ERELD_DM-V1_2.

MCSs are services that require preferential handling compared to normal telecommunication services, e.g. in support of police or fire brigade.

MCSs can be used for public safety applications and also for general commercial applications (e.g., utility companies and railways).

This document is applicable to an UE supporting on-line, off-line or both on-line and off-line configuration of the configuration management client.

This document is applicable to an UE supporting off-line configuration of the group management client.

The parameters defined by the management objects in this document are configured in the UE using on-line configuration over the CSC-4 reference point and using off-line configuration over the CSC-11 and CSC-12 reference points. Other specifications define how these parameters are used in the operation of MCSs, and whether they are applicable to on-network operation or off-network operation, or both:

-	3GPP TS 24.379;

-	3GPP TS 24.380;

-	3GPP TS 24.481;

-	3GPP TS 24.482;

-	3GPP TS 24.484;

-	3GPP TS 24.282;

-	3GPP TS 24.582;

-	3GPP TS 24.281; and

-	3GPP TS 24.581.

The common network operation means both on-network operation and off-network operation in this document.

The following management objects are defined in this document:

-	MCPTT UE configuration MO;

-	MCPTT user profile MO;

-	MCS group configuration MO;

-	MCPTT service configuration MO;

-	MCS UE initial configuration MO;

-	MCData UE configuration MO;

-	MCData user profile MO;

-	MCData service configuration MO;

-	MCVideo UE configuration MO;

-	MCVideo user profile MO; and

-	MCVideo service configuration MO.

The MOs listed above define repositories of data in the ME.

##### 5.1.2.12.4.84	TS 24.484

Mission Critical Services (MCS) configuration management; Protocol specification

This document specifies the configuration management documents and protocols needed to support Mission critical service online configuration over the CSC-4 and CSC-5 reference points and the procedures to support Mission critical service offline configuration over the CSC-11and CSC-12 reference points.

Mission critical services include:

-	Mission Critical Push To Talk (MCPTT);

-	Mission Critical Video (MCVideo); and

-	Mission Critical Data (MCData).

Configuration management documents defined in this document includes:

-	MC UE initial configuration document;

-	MCPTT UE configuration document;

-	MCPTT user profile configuration document;

-	MCPTT service configuration document;

-	MCVideo UE configuration document;

-	MCVideo user profile configuration document;

-	MCVideo service configuration document;

-	MCData UE configuration document;

-	MCData user profile configuration document; and

-	MCData service configuration document.

Mission critical services are services that require preferential handling compared to normal telecommunication services, e.g. in support of police or fire brigade.

The Mission critical services can be used for public safety applications and also for general commercial applications (e.g., utility companies and railways).

This document is applicable to an MC UE supporting the configuration management client functionality, to application servers supporting the configuration management server functionality, and to application servers supporting the:

-	MCPTT server functionality;

-	MCVideo server functionality; or

-	MCData server functionality.

##### 5.1.2.12.4.85	TS 24.523

Core and enterprise Next Generation Network (NGN) interaction scenarios; Architecture and functional description

This document provides the possible scenarios for:

- the interconnection of an Next Generation Corporate Network (NGCN) with a Next Generation Network (NGN); and

- the support of NGCN capabilities within an NGN, either towards a User Equipment (UE) or to an NGCN.

Unless otherwise specified by reference to other documents, all requirements relating to architecture and functional requirements are contained within this document.

##### 5.1.2.12.4.86	TS 24.524

Hosted enterprise services; Architecture, functional description and signaling

This document describes the architecture and functionality required to support enterprise and corporate services as IMS applications hosted in the NGN operator's network on behalf of an enterprise (Hosted Enterprise Services).

This document also specifies the protocol requirements for the UE to attach to the NGN (in particular the IMS) and also any protocol requirements related to application servers provided in support of hosted enterprise services.

##### 5.1.2.12.4.87	TS 24.525

Business trunking; Architecture and functional description

This document provides architecture and functional requirements for business trunking for the Next Generation Network (NGN).

This document also specifies the protocol requirements for the Next Generation Corporate Networks (NGCNs) to attach to the NGN (in particular the IM CN subsystem) and also any protocol requirements relation to application servers provided in support of business trunking.

Business trunking is a set of NGN capabilities that may be applied to communications between NGCNs using the NGN as a transit.

##### 5.1.2.12.4.88	TS 24.581

Mission Critical Video (MCVideo) media plane control; Protocol specification

This document specifies the media plane control and transmission control protocols and interactions with the media needed to support Mission Critical Video (MCVideo) service.

The MCVideo service and its associated media plane control protocols can be used for public safety applications and also for general commercial applications (e.g., utility companies and railways).

This document is applicable to User Equipment (UE) supporting MCVideo client, and MCVideo server supporting media distribution function and transmission control server.

##### 5.1.2.12.4.89	TS 24.582

Mission Critical Data (MCData) media plane control; Protocol specification

This document specifies the media plane control protocols and interactions with the media needed to support Mission Critical Data (MCData) services, which are Short Data Service (SDS) and File Distribution (FD).

Mission critical communication services are services that require preferential handling compared to normal telecommunication services, e.g. in support of police or fire brigade.

The MCData service and its associated media plane control protocols can be used for public safety applications and also for general commercial applications (e.g., utility companies and railways).

This document is applicable to User Equipment (UE) supporting MCData client functionality and to servers in the MCData system.

##### 5.1.2.12.4.90	TS 24.604

Communication Diversion (CDIV) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification

This document specifies the Stage 3, Protocol Description of the Communications Diversion (CDIV) supplementary services, based on stage one and two of the ISDN Communication diversion supplementary services. It provides the protocol details in the IP Multimedia (IM) Core Network (CN) subsystem based on the Session Initiation Protocol (SIP) and the Session Description Protocol (SDP).

##### 5.1.2.12.4.91	TS 24.605

Conference (CONF) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification

This document specifies the stage three Protocol Description of the Conference (CONF) service based on stage one and two of the ISDN CONF supplementary service. It provides the protocol details in the IP Multimedia (IM) Core Network (CN) subsystem based on the Session Initiation Protocol (SIP) and the Session Description Protocol (SDP).

##### 5.1.2.12.4.92	TS 24.606

Message Waiting Indication (MWI) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification

This document specifies the stage three Protocol Description of the Message Waiting Indication (MWI) service, based on stage one and two of the ISDN MWI supplementary services. It provides the protocol details in the IP Multimedia (IM) Core Network (CN) subsystem based on the Session Initiation Protocol (SIP) and the Session Description Protocol (SDP).

##### 5.1.2.12.4.93	TS 24.607

Originating Identification Presentation (OIP) and Originating Identification Restriction (OIR) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification

This document specifies the stage three (protocol description) of the Originating Identification Presentation (OIP) supplementary service and the Originating Identification Restriction (OIR) supplementary services, based on stage one and two of the ISDN CLIP and CLIR supplementary service.

It provides the protocol details in the IP Multimedia (IM) Core Network (CN) subsystem based on the Session Initiation Protocol (SIP) and the Session Description Protocol (SDP).

##### 5.1.2.12.4.94	TS 24.608

Terminating Identification Presentation (TIP) and Terminating Identification Restriction (TIR) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification

This document specifies the stage three protocol description of the Terminating Identification Presentation (TIP) and Terminating Identification Restriction (TIR) services, based on stage one and two of the ISDN COLP and COLR supplementary services. It provides the protocol details in the IP Multimedia (IM) Core Network (CN) subsystem based on the Session Initiation Protocol (SIP) and the Session Description Protocol (SDP).

##### 5.1.2.12.4.95	TS 24.610

Communication HOLD (HOLD) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification

This document specifies the stage three Protocol Description of the Communication Hold (HOLD) services, based on stages one and two of the ISDN Hold (HOLD) supplementary services. It provides the protocol details in the IP Multimedia (IM) Core Network (CN) subsystem based on the Session Initiation Protocol (SIP) and the Session Description Protocol (SDP).

##### 5.1.2.12.4.96	TS 24.611

Anonymous Communication Rejection (ACR) and Communication Barring (CB) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification

This document specifies the stage three, Protocol Description of the Anonymous Communication Rejection (ACR) and Communication Barring (CB) supplementary service, based on stage one and two of the ISDN supplementary service Anonymous Call Rejection (ACR), Incoming Communication Barring (ICB) and Outgoing Communication Barring (OCB). It provides the protocol details in the IP Multimedia (IM) Core Network (CN) subsystem based on the Session Initiation Protocol (SIP) and the Session Description Protocol (SDP).

##### 5.1.2.12.4.97	TS 24.615

Communication Waiting (CW) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol Specification

This document specifies the stage 3, Protocol Description of the Communication Waiting (CW) service, based on Stage 1 and Stage 2 of the ISDN call waiting supplementary services. It provides the protocol details in the IP Multimedia (IM) Core Network (CN) subsystem based on the Session Initiation Protocol (SIP) and the Session Description Protocol (SDP).

##### 5.1.2.12.4.98	TS 24.616

Malicious Communication Identification (MCID) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification

This document specifies the stage three Protocol Description of the Malicious Call Communication Identification (MCID) service based on the stage one and two of ISDN Malicious Call Identification supplementary service. It provides the protocol details in the IP Multimedia (IM) Core Network (CN) subsystem based on the Session Initiation Protocol (SIP) and the Session Description Protocol (SDP). The MCID service will store session related information independent of the service requested.

##### 5.1.2.12.4.99	TS 24.623

Extensible Markup Language (XML) Configuration Access Protocol (XCAP) over the Ut interface for Manipulating Supplementary Services

This document defines a protocol used for manipulating data related to supplementary services. The protocol is based on the eXtensible Markup Language (XML) Configuration Access Protocol (XCAP) RFC 4825. A new XCAP application usage is defined for the purpose of manipulating the supplementary services data. The common XCAP related aspects that are applicable to supplementary services are specified in this document. The protocol allows authorized users to manipulate service related data either when they are connected to IMS or when they are connected to non IMS networks (e.g. the public Internet).

##### 5.1.2.12.4.100	TS 24.628

Common Basic Communication procedures using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification

This document describes the stage three protocol for basic communication procedures common to several services in the IP Multimedia (IM) Core Network (CN) subsystem when at least one Application Server (AS) is included in the communication. The common procedures are based on stage three specifications for supplementary services.

##### 5.1.2.12.4.101	TS 24.629

Explicit Communication Transfer (ECT) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification

This document specifies the stage three (protocol description) of the Explicit Communication transfer (ECT) supplementary service, based on stage one and two of the ISDN ECT supplementary service. It provides the protocol details in the IP Multimedia (IM) Core Network (CN) subsystem based on the Session Initiation Protocol (SIP) and the Session Description Protocol (SDP).

##### 5.1.2.12.4.102	TS 24.642

Completion of Communications to Busy Subscriber (CCBS) and Completion of Communications by No Reply (CCNR) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification

This document specifies the stage three Protocol Description of the Completion of Communications to Busy Subscriber (CCBS) service and the Completion of Communication on no Reply (CCNR) service, based on stage one and two of the ISDN supplementary services. It provides the protocol details in the IP Multimedia (IM) Core Network (CN) subsystem based on the Session Initiation Protocol (SIP) and the Session Description Protocol (SDP).

##### 5.1.2.12.4.103	TS 24.647

Advice Of Charge (AOC) using IP Multimedia (IM) Core Network (CN) subsystem

This document specifies the stage three Protocol Description of the Advice Of Charge (AOC) service, based on Stage 1 and 2 of the ISDN Supplementary Service Advice Of Charge for all calls (permanent mode). It provides the protocol details in the IP Multimedia (IM) Core Network (CN) subsystem based on the Session Initiation Protocol (SIP) and the Session Description Protocol (SDP).

##### 5.1.2.12.4.104	TS 24.654

Protocol specification Closed User Group (CUG) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol Specification

This document specifies the stage three Protocol Description of the Closed User Group (CUG) service, based on stage one and two of the ISDN Communication diversion supplementary services. It provides the protocol details in the IP Multimedia (IM) Core Network (CN) subsystem based on the Session Initiation Protocol (SIP) and the Session Description Protocol (SDP).

##### 5.1.2.12.4.105	TR 24.969

Business trunking; Next Generation Corporate Network (NGCN) - Next Generation Network (NGN) interfaces implementation guide

The purpose of this document is to give an implementation guide to the relevant Common IMS specifications and functions used in the interconnection of a Next Generation Corporate Network site (NGCN site) to the NGN.

This document addresses control plane signalling (usage of SIP and SDP protocols, required SIP headers) as well as other interconnecting aspects like security, numbering/naming/addressing and user plane issues such as transport protocol, media and codecs actually covered in a widespread set of 3GPP and ETSI TISPAN specifications, as seen from the perspective of an NGCN site.

Advice-of-charge aspects are addressed as far as SIP signalling is concerned.

This document is based on 3GPP TS 24.229.

#### 5.1.2.12.5	CODECs

##### 5.1.2.12.5.1	TS 26.071

Mandatory speech CODEC speech processing functions; AMR speech Codec; General description

This specification describes an introduction to the set of the adaptive multi-rate (AMR) specifications.

##### 5.1.2.12.5.2	TS 26.090

Mandatory Speech Codec speech processing functions; Adaptive Multi-Rate (AMR) speech codec; Transcoding functions

This specification describes a detailed description of the AMR speech codec transcoding functions.

##### 5.1.2.12.5.3	TS 26.091

Mandatory Speech Codec speech processing functions; Adaptive Multi-Rate (AMR) speech codec; Error concealment of lost frames

This specification describes example procedures for the error concealment, also called frame substitution or muting procedure, of lost speech or silence indicator frames.

##### 5.1.2.12.5.4	TS 26.092

Mandatory speech codec speech processing functions; Adaptive Multi-Rate (AMR) speech codec; Comfort noise aspects

This specification describes the detailed requirements for the correct operation of the background acoustic noise evaluation, noise parameter encoding/decoding and comfort noise generation for the AMR speech codec during source controlled rate (SCR) operation.

##### 5.1.2.12.5.5	TS 26.093

Mandatory speech codec speech processing functions; Adaptive Multi-Rate (AMR) speech codec; Source controlled rate operation

This specification describes the operation of the AMR speech codec during SCR operation.

##### 5.1.2.12.5.6	TS 26.094

Mandatory speech codec speech processing functions; Adaptive Multi-Rate (AMR) speech codec; Voice Activity Detector (VAD)

This specification describes two alternatives for the VAD to be used during SCR operation in conjunction with the AMR codec.

##### 5.1.2.12.5.7	TS 26.110

Codec for circuit switched multimedia telephony service; General description

This specification describes an introduction to the set of specifications for the support of circuit-switched 3G-324M multimedia telephony service.

##### 5.1.2.12.5.8	TS 26.111

Codec for circuit switched multimedia telephony service; Modifications to H.324

This specification describes the modifications applicable to the ITU-T Recommendation H.324, Annex C for the support of circuit-switched 3G 324M multimedia telephony service.

##### 5.1.2.12.5.9	TS 26.116

Television (TV) over 3GPP services; Video profiles

This document specifies requirements and guidelines on video source formats (frame rate, resolution, aspect ratio, colorimetry, bit depth…) and encoding parameters (codec format, random access point period, SEI messages…) for different types of TV services, including linear TV, catch-up TV or on-demand services. A limited set of Operation Points (e.g. SDTV, HDTV…) are defined to provide confidence to content providers/broadcasters on the quality of experience offered by 3GPP services when used for TV-like distribution. Operation Points define format and encoding restrictions, but may also be viewed as compatibility points for UEs.

In particular, the Operation Points defined in this document may serve as the primary tested configurations for TV centric video distribution. The initial set of Operation Points are defined based on the analysis and findings in the technical report TR 26.949.

In addition, in the context of DASH operations, not only the main distribution format are defined, but also a subset of spatial and temporal resolutions. In order to minimize testing for seamless switching experience, suitable lower resolutions of distribution formats are defined. Furthermore, to compensate congestion situations, a minimum service quality is defined in order to provide service continuity.

##### 5.1.2.12.5.10	TS 26.179

Mission Critical Push To Talk (MCPTT); Codecs and media handling

This document specifies the codecs and media handling for MCPTT. The corresponding service requirements are defined in 3GPP TS 22.179. The corresponding functional architecture, procedures and information flows are defined in 3GPP TS 23.179.

##### 5.1.2.12.5.11	TS 26.223

Telepresence using the IP Multimedia Subsystem (IMS); Media handling and interaction

This document specifies a client for the IMS-based telepresence service supporting conversational speech, video and text transported over RTP. Telepresence is defined as a conference with interactive audio-visual communications experience between remote locations, where the users enjoy a strong sense of realism and presence between all participants (i.e. as if they are in same location) by optimizing a variety of attributes such as audio and video quality, eye contact, body language, spatial audio, coordinated environments and natural image size. A telepresence system is defined as a set of functions, devices and network elements which are able to capture, deliver, manage and render multiple high quality interactive audio and video signals in a telepresence conference. An appropriate number of devices (e.g. cameras, screens, loudspeakers, microphones, codecs) and environmental characteristics are used to establish telepresence.

The media handling capabilities of a telepresence client (TP UE) are specified in this document. A TP UE supports Multimedia Telephony Service for IMS (MTSI) UE media handling capabilities, but it also supports more advanced media handling capabilities. The media handling aspects of a TP UE within the scope of this document include media codecs, media configuration and session control, data transport, audio/video parameters, and interworking with MTSI.

##### 5.1.2.12.5.12	TS 26.281

Mission Critical Video (MCVideo); Codecs and media handling

This document specifies the codecs and media handling for MCVideo. The requirements for MCVideo are specified by SA1 in two technical specifications:

a. The requirements specific to Mission Critical Video have been specified in 3GPP TS 22.281.

b. The requirements common to multiple mission critical services have been specified in 3GPP TS 22.280.

The architectural work for MCVideo is specified by SA6 in two technical specifications:

a.	architectural and feature work specific to MCVideo have been specified in 3GPP TS 23.281.

b.	architectural aspects common to multiple mission critical services have been specified in 3GPP TS 23.280.

##### 5.1.2.12.5.13	TS 26.307

Presentation layer for 3GPP services

This document specifies an HTML5 profile as a common presentation layer for 3GPP services.

##### 5.1.2.12.5.14	TS 26.346

Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs

This document defines a set of media codecs, formats and transport/application protocols to enable the deployment of MBMS user services either over the MBMS bearer service or other UMTS Bearer Services within the 3GPP system. In this version of the specification, only MBMS download and streaming delivery methods are specified. This document does not preclude the use of other delivery methods. This document includes information applicable to network operators, service providers and manufacturers.

##### 5.1.2.12.5.15	TS 26.347

Multimedia Broadcast/Multicast Service (MBMS); Application Programming Interface and URL

This document provides application methods and interfaces between an MBMS-aware application and the UE MBMS Client to access 3GPP MBMS User services. The purpose of the document is the definition of enablers in order to simplify the usage of MBMS in web-centric as well as app-based service environments.

This document defines several APIs to access MBMS User Services and a URL to access resources available as part of an MBMS User Service. The MBMS User Services are defined in TS 26.346 and are not part of this document.

##### 5.1.2.12.5.16	TS 26.441

Codec for Enhanced Voice Services (EVS); General overview

This document is an introduction to the audio processing parts of the Enhanced Voice Services coder. A general overview of the audio processing functions is given, with reference to the documents where each function is specified in detail.

##### 5.1.2.12.5.17	TS 26.442

Codec for Enhanced Voice Services (EVS); ANSI C code (fixed-point)

This document contains an electronic copy of the ANSI‑C code for the Enhanced Voice Services (EVS) Codec. The ANSI‑C code is necessary for a bit exact implementation of the EVS Codec (3GPP TS 26.445), Voice Activity Detection (VAD) (3GPP TS 26.451), Comfort Noise Generation (CNG) (3GPP TS 26.449), Discontinuous Transmission (DTX) (3GPP TS 26.450), Packet Loss Concealment (PLC) of Lost Packets (3GPP TS 26.447), Jitter Buffer Management (JBM) (3GPP TS 26.448), and AMR-WB Interoperable Function (3GPP TS 26.446).

Requirements for any implementation of the EVS codec to be standard compliant are specified in 3GPP TS 26.444 (Test sequences).

##### 5.1.2.12.5.18	TS 26.443

Codec for Enhanced Voice Services (EVS); ANSI C code (floating-point)

This document contains an electronic copy of the ANSI C floating-point code for the Enhanced Voice Services (EVS) Codec. This ANSI C code is the unique alternative reference specification besides the ANSI-C fixed-point code for the EVS Codec (3GPP TS 26.442) for a standard compliant implementation of the EVS Codec (3GPP TS 26.445), Voice Activity Detection (VAD) (3GPP TS 26.451), Comfort Noise Generation (CNG) (3GPP TS 26.449), Discontinuous Transmission (DTX) (3GPP TS 26.450), Packet Loss Concealment (PLC) of Lost Packets (3GPP TS 26.447), Jitter Buffer Management (JBM) (3GPP TS 26.448), and AMR-WB Interoperable Function (3GPP TS 26.446).

Requirements for any implementation of the EVS codec to be standard compliant are specified in 3GPP TS 26.444 (Test sequences).

##### 5.1.2.12.5.19	TS 26.444

Codec for Enhanced Voice Services (EVS); Test sequences

This document specifies the digital test sequences for the Enhanced Voice Services (EVS) Codec. These sequences test for a bit-exact implementation of the EVS Codec (3GPP TS 26.445), Voice Activity Detection (VAD) (3GPP TS.26.451), Comfort Noise Generation (3GPP TS 26.449), Discontinuous Transmission (DTX) (3GPP TS 26.450), Error Concealment of Lost Packets (3GPP TS 26.447), Jitter Buffer Management (JBM) (3GPP TS 26.448), and AMR-WB Interoperable Function (3GPP TS 26.446). In addition, this document specifies conformance testing (FFS).

##### 5.1.2.12.5.20	TS 26.445

Codec for Enhanced Voice Services (EVS); Detailed algorithmic description

This document is a detailed description of the signal processing algorithms of the Enhanced Voice Services coder.

##### 5.1.2.12.5.21	TS 26.446

Codec for Enhanced Voice Services (EVS); Adaptive Multi-Rate - Wideband (AMR-WB) backward compatible functions

This document specifies the AMR-WB backward compatible functions of the EVS codec.

This document is a high level overview of the functionality with reference to the Codec Detailed Algorithmic Description where the functionality is specified in detail.

##### 5.1.2.12.5.22	TS 26.447

Codec for Enhanced Voice Services (EVS); Error concealment of lost packets

This document defines a frame loss concealment procedure, also termed frame substitution and muting procedure, which is executed by the Enhanced Voice Services (EVS) decoder when one or more frames (speech or audio or SID frames) are unavailable for decoding due to e.g. packet loss, corruption of a packet or late arrival of a packet.

##### 5.1.2.12.5.23	TS 26.448

Codec for Enhanced Voice Services (EVS); Jitter buffer management

This document defines the Jitter Buffer Management solution for the Codec for Enhanced Voice Services (EVS).

##### 5.1.2.12.5.24	TS 26.449

Codec for Enhanced Voice Services (EVS); Comfort Noise Generation (CNG) aspects

This document gives an overview for the requirements of the background acoustic noise evaluation, noise parameter encoding/decoding and comfort noise generation for the Enhanced Voice Services (EVS) speech codec during Discontinuous Transmission (DTX) operation.

##### 5.1.2.12.5.25	TS 26.450

Codec for Enhanced Voice Services (EVS); Discontinuous Transmission (DTX)

This document specifies the system level aspects of the Discontinuous Transmission (DTX) function of the EVS codec.

##### 5.1.2.12.5.26	TS 26.451

Codec for Enhanced Voice Services (EVS); Voice Activity Detection (VAD)

This document specifies the Voice Activity Detector (VAD) used in the Discontinuous Transmission (DTX) of the EVS Codec. Although the main application of the VAD algorithm is the detection of speech or voice signals, the algorithm is more accurately described as a Signal Activity Detection (SAD) algorithm.

This document is a high level overview of the functionality with reference to the Codec Detailed Algorithmic Description where the functionality is specified in detail.

##### 5.1.2.12.5.27	TS 26.453

Codec for Enhanced Voice Services (EVS); Speech codec frame structure

The Enhanced Voice Services (EVS) Codec is specified in the series of Technical Specifications 3GPP TS 26.441 to TS 26.451 and it is characterized in TR 26.952.

This document describes the "generic frame format" for the EVS Codec for the application in 3G Circuit-Switched Networks. This format is based on the RTP framing, as specified in TS 26.445. This generic frame format will be used as a common reference point, when interfacing speech frames between different elements of the 3G system. Appropriate mappings to and from this generic frame format will be used within and between each system element.

##### 5.1.2.12.5.28	TS 26.454

Codec for Enhanced Voice Services (EVS); Interface to Iu, Uu, Nb and Mb

The Enhanced Voice Services (EVS) Codec is specified in Technical Specifications 3GPP TS 26.441 to TS 26.451. The generic frame format for 3G CS networks is specified in TS 26.453. The allowed EVS Configurations of the UMTS_EVS Codec Type are specified in TS 26.103.

This document specifies the mapping of the EVS generic frame format (3GPP TS 26.453) to the Iu Interface (3GPP TS 25.415) and the Uu Interface, including handling of the EVS-CMR by the UE and the Iu-terminating MGW.

This document specifies also the mapping to the Nb Interface in a BICC-based circuit switched core network. (3GPP TS 29.415) and the Nb-Interface in a SIP-I-based circuit switched core network (3GPP TS 23.231).

This document specifies also the interworking of different EVS Configurations on the terminations of the MGW and the interworking between EVS and AMR-WB on the terminations of the MGW.

##### 5.1.2.12.5.29	TR 26.906

Evaluation of High Efficiency Video Coding (HEVC) for 3GPP services

This document reports the evaluation of the High Efficiency Video Coding (HEVC) codec in 3GPP services. It provides an overview of the codec and a comparison to H.264 (AVC) codec. The support of H.264 (AVC) is mandated for 3GP-DASH (TS 26.247), PSS (TS 26.234), MBMS (TS 26.346), 3GPP file format (TS 26.244), MTSI (TS 26.114) and MMS (TS 26.140) in Release 11. This document reports on the performance of H.265 (HEVC) when used in 3GPP services for video coding in comparison to H.264 (AVC) and the performance of H.265 (HEVC) when used in 3GPP services for image coding in comparison to JPEG. Performance is evaluated in typical 3GPP service environments taking into account bandwidth and coding efficiency, user experience and complexity. Based on the performance results, recommendations are provided for the proper inclusion of H.265 (HEVC) in 3GPP services.

##### 5.1.2.12.5.30	TR 26.907

HTML5 for a new presentation layer in 3GPP services

This document is a study of existing scene description solutions for 3GPP services and an evaluation of HTML5 as an alternative and unified solution for providing a presentation layer for 3GPP services.

##### 5.1.2.12.5.31	TR 26.911

Codec(s) for Circuit-Switched (CS) multimedia telephony service; Terminal implementor’s guide

This report describes non-mandatory Recommendations for the use of the different codec implementation options for the circuit-switched 3G-

324M multimedia telephony service based on ITU-T Recommendation H.324, Annex C. These Recommendations address issues specific to the third generation operating environment, including guaranteeing sufficient error resilience and inter-working between terminals.

##### 5.1.2.12.5.32	TR 26.938

Packet-switched Streaming Service (PSS); Improved support for dynamic adaptive streaming over HTTP in 3GPP

This document covers

- deployment guidelines for DASH in 3GPP networks and architectures,

- use cases for the improved support of DASH in 3GPP networks and architectures as well as requirements to support those use cases,

- recommendations for potentially necessary normative specification work in 3GPP,

- recommendations for the documentation of potentially informative guide lining work.

##### 5.1.2.12.5.33	TR 26.952

Codec for Enhanced Voice Services (EVS); Performance characterization

This document provides information on the Enhanced Voice Services (EVS) codec Selection, Verification and Characterization Phases which were run using the fixed-point code (3GPP TS 26.442). Experimental test results from the subjective quality testing are reported to illustrate the behaviour of the EVS codec. Additional information is provided on implementation complexity of the EVS codec and objective test results. Also the verification results for the floating-point version of the EVS codec (3GPP TS 26.443) are presented.

#### 5.1.2.12.6	Data

##### 5.1.2.12.6.1	TS 27.005

Use of Data Terminal Equipment – Data Circuit terminating Equipment (DTE – DCE) interface for Short Message Service (SMS) and Cell Broadcast Service (CBS)

This document defines three interface protocols for control of SMS functions within a GSM/UMTS mobile telephone from a remote terminal via an asynchronous interface. The “block mode” protocol includes error protection and is suitable for use where the link may not be completely reliable. It is of particular use where control of remote devices is required. Efficient transfer of binary encoded user data is possible. The “text mode”, based on AT commands, is suitable for unintelligent terminals or terminal emulators, and for application software built on command structures like those defined in V.25ter. The character based “PDU mode” is suitable for software drivers based on AT command structures which do not understand the content of the message blocks and can only pass them between the MT and higher level software resident in the TE.

In all three modes, the terminal is considered to be in control for SMS/CBS transactions. The document considers the mobile termination to be a single entity. Other 3GPP/GSM Technical Specifications describe the split of functionality between the mobile equipment and (U)SIM.

##### 5.1.2.12.6.2	TS 27.007

AT command set for User Equipment (UE)

This document specifies a profile of AT commands and recommends that this profile be used for controlling Mobile Termination (MT) functions and GSM/UMTS network services from a Terminal Equipment (TE) through Terminal Adaptor (TA). The command prefix +C is reserved for Digital Cellular in ITU-T Recommendation V.250. The document has also the syntax details used to construct these extended GSM/UMTS commands. Commands from ITU-T Recommendation V.250 and existing digital cellular standards (TIA IS 99 and TIA IS 135) are used whenever applicable. Some of the new commands are defined such way that they can be easily applied to MT of networks other than GSM/UMTS. ITU-T Recommendation T.31 and ITU-T Recommendation T.32 fax AT commands may be used for GSM/UMTS fax transmission from TE. GSM/UMTS Short Message Service AT commands are defined in 3GPP TS 27.005. AT commands for GPRS and EPC are defined in this specification. The document assumes an abstract architecture comprising a TE (e.g. a computer) and a MT interfaced by a TA.

The commands described in this document may be observed on the link between the TE and the TA. However, most of the commands retrieve information about the MT, not about the TA.

##### 5.1.2.12.6.3	TS 27.010

Terminal Equipment to User Equipment (TE-UE) multiplexer protocol

This document defines a multiplexing protocol between a UE and a TE. The multiplexing protocol can be used to send any data, for instance voice, SMS, USSD, fax etc. The document describes the protocol, but not the commands or data transported with it.

#### 5.1.2.12.7	Signalling protocols ("stage 3") -(RSS-CN) and OAM&P and Charging

##### 5.1.2.12.7.1	TS 28.301

Telecommunication management; Licensed Shared Access (LSA) Controller (LC) Integration Reference Point (IRP); Requirements

This document specifies the requirements of the Licensed Shared Access (LSA) Controller (LC) Integration Reference Point (IRP). This IRP allows the NM to interact with the LC. This interaction with the LC enables the NM to configure NEs utilizing LSA spectrum resources.

The following two deployment scenarios are supported:

-	one scenario in which the LC communicates LSA spectrum resource availability information to the NM, and

-	one scenario in which the LC determines and communicates to the NM constraints on parameters of cells liable to use LSA spectrum resources.

##### 5.1.2.12.7.2	TS 28.302

Telecommunication management; Licensed Shared Access (LSA) Controller (LC) Integration Reference Point (IRP); Information Service (IS)

This document specifies the LSA controller (LC) Integration Reference Point (IRP) management operations and notifications that can be communicated between an IRPAgent and one or several IRPManagers.

This document specifies the semantics and behaviour of operations, notifications and their parameters visible across the reference point in a protocol and technology neutral way. It does not define their syntax and encoding.

##### 5.1.2.12.7.3	TS 28.303

Telecommunication management; Licensed Shared Access (LSA) Controller (LC) Integration Reference Point (IRP); Solution Set (SS) definitions

The purpose of this document is to define the mapping of the LSA Controller (LC) Integration Reference Point (IRP) (see 3GPP TS 28.302) to the protocol specific details necessary for implementation of this IRP in a CORBA/IDL environment and in a SOAP/WSDL environment.

This Solution Set specification is related to 3GPP TS 28.302 V14.0.X.

##### 5.1.2.12.7.4	TS 28.390

Telecommunication management; Fixed Mobile Convergence (FMC); Interface Integration Reference Point (IRP) Solution Profiles (SPs)

The objective of this specification is to define a set of Solution Profiles (SPs) for Interface IRPs. Each SP identifies the necessary and sufficient subset of the 3GPP IRP defined solutions (solutions) required for the network management of a specific operating environment or domain, including that defined for the converged management environments.

A specific operating environment or domain is characterised by a set of Requirement statements. In other words, the solutions identified in a specific SP would satisfy the Requirements listed in that same SP.

An SP, in its simplest form, is nothing else than a list of solutions (e.g. 3GPP IRP defined operations). The purpose is to place a combination of solutions under a common (SP) name so that:

- Operators and vendors need not decide arbitrarily which solutions are to be used to satisfy the Requirements;

- There is one document where readers can have a clear view of solutions, specified in various IRP specifications, required to satisfy the Requirements.

This version of the TS contains a number of SPs defined in clause 4 and onwards, one clause for each SP.

##### 5.1.2.12.7.5	TS 28.402

Telecommunication management; Performance Management (PM); Performance measurements for Evolved Packet Core (EPC) and non-3GPP access interworking system

This document describes the measurements for EPC and non-3GPP access network interworking.

TS 32.401 describes Performance Management concepts and requirements.

This document is valid for all measurement types provided by an implementation of EPC and non-3GPP access Interworking System. Only measurement types that are specific to EPC and non-3GPP access interworking are defined within this documents. Vendor specific measurement types used in EPC and non-3GPP access Interworking System are not covered. Instead, these could be applied according to manufacturer's documentation.

Measurements related to "external" technologies (such as ATM or IP) as described by "external" standards bodies (e.g. ITU-T or IETF) are only be referenced within this specification, wherever there is a need identified for the existence of such a reference.

The definition of the standard measurements is intended to result in comparability of measurement data produced in a multi-vendor network, for those measurement types that can be standardised across all vendors' implementations.

The structure of this document is as follows:

- Header 1: Network Element (e.g. measurements related to ePDG);

- Header 2: Measurement function (e.g. Tunnel establishment);

- Header 3: Measurements.

##### 5.1.2.12.7.6	TS 28.403

Telecommunication management; Performance Management (PM); Performance measurements for Wireless Local Area Network (WLAN)

This document describes the measurements for WLAN.

TS 32.401 describes Performance Management concepts and requirements.

This document is valid for all measurement types provided by an implementation of a WLAN.

Only measurement types that are specific to WLAN are defined within this document. Vendor specific measurement types used in WLAN are not covered. Instead, these could be applied according to manufacturer's documentation.

Measurements related to "external" technologies (such as WLAN or IP) as described by "external" standards bodies (e.g. IEEE or IETF) are only referenced within this document, wherever there is a need identified for the existence of such a reference.

The definition of the standard measurements is intended to result in comparability of measurement data produced in a multi-vendor network, for those measurement types that can be standardised across all vendors' implementations.

##### 5.1.2.12.7.7	TS 28.500

Telecommunication management; Management concept, architecture and requirements for mobile networks that include virtualized network functions

This document specifies the management concepts, the management requirements and use cases from operators’ perspective for mobile networks that include virtualized network functions which can be part of EPC or IMS, and provides the management architecture for these mobile networks.

##### 5.1.2.12.7.8	TS 28.510

Telecommunication management; Configuration Management (CM) for mobile networks that include virtualized network functions; Requirements

This document (together with the relevant requirements described in 3GPP TR 21.905, 3GPP TS 32.102, 3GPP TS 32.600 and 3GPP TS 28.500) specifies the requirements applicable to Configuration Management (CM) of virtualized network functions which can be part of EPC or IMS.

##### 5.1.2.12.7.9	TS 28.511

Telecommunication management; Configuration Management (CM) for mobile networks that include virtualized network functions; Procedures

This document specifies the Configuration Management (CM) procedures for mobile networks that include virtualized network functions which can be part of EPC or IMS.

##### 5.1.2.12.7.10	TS 28.512

Telecommunication management; Configuration Management (CM) for mobile networks that include virtualized network functions; Stage 2

This document is Configuration Management stage 2 specification for mobile networks that include virtualized network functions which can be part of EPC or IMS.

##### 5.1.2.12.7.11	TS 28.513

Telecommunication management; Configuration Management (CM) for mobile networks that include virtualized network functions; Stage 3

This document is Configuration Management stage 3 specification for mobile networks that include virtualized network functions which can be part of EPC or IMS.

##### 5.1.2.12.7.12	TS 28.515

Telecommunication management; Fault Management (FM) for mobile networks that include virtualized network functions; Requirements

This document (together with the relevant requirements described in TS 28.500, TS 32.101, TS 32.102 and TS 32.111-1) specifies the requirements applicable to Fault Management (FM) of mobile networks that include virtualized network functions which can be part of EPC or IMS.

##### 5.1.2.12.7.13	TS 28.516

Telecommunication management; Fault Management (FM) for mobile networks that include virtualized network functions; Procedures

This document specifies the Fault Management procedures for mobile networks that include virtualized network functions which can be part of EPC or IMS.

##### 5.1.2.12.7.14	TS 28.517

Telecommunication management; Fault Management (FM) for mobile networks that include virtualized network functions; Stage 2

This document is Fault Management stage 2 specification for mobile networks that include virtualized network functions which can be part of EPC or IMS.

##### 5.1.2.12.7.15	TS 28.518

Telecommunication management; Fault Management (FM) for mobile networks that include virtualized network functions; Stage 3

This document is Fault Management stage 3 specification for mobile networks that include virtualized network functions which can be part of EPC or IMS.

##### 5.1.2.12.7.16	TS 28.520

Telecommunication management; Performance Management (PM) for mobile networks that include virtualized network functions; Requirements

This document (together with the relevant requirements described in TS 32.401, TS 32.411, TS 28.500, TS 32.101 and TS 32.102) specifies the requirements applicable to Performance Management (PM) of mobile networks that include virtualized network functions which can be part of EPC or IMS.

##### 5.1.2.12.7.17	TS 28.521

Telecommunication management; Performance Management (PM) for mobile networks that include virtualized network functions; Procedures

This document specifies the Performance Management procedures for mobile networks that include virtualized network functions which can be part of EPC or IMS.

##### 5.1.2.12.7.18	TS 28.522

Telecommunication management; Performance Management (PM) for mobile networks that include virtualized network functions; Stage 2

This document is the Performance Management stage 2 specification for mobile networks that include virtualized network functions, which can be part of EPC or IMS.

##### 5.1.2.12.7.19	TS 28.523

Telecommunication management; Performance Management (PM) for mobile networks that include virtualized network functions; Stage 3

This document is the Performance Management stage 3 specification for mobile networks that include virtualized network functions, which can be part of EPC or IMS.

##### 5.1.2.12.7.20	TS 28.525

Telecommunication management; Life Cycle Management (LCM) for mobile networks that include virtualized network functions; Requirements

This document (together with the relevant requirements described in TS 28.500, TS 32.101 and TS 32.102) specifies the requirements applicable to Lifecycle Management (LCM) for mobile networks that include virtualized network functions which can be part of EPC or IMS.

##### 5.1.2.12.7.21	TS 28.526

Telecommunication management; Life Cycle Management (LCM) for mobile networks that include virtualized network functions; Procedures

This document specifies the Life Cycle Management (LCM) procedures for mobile networks that include virtualized network functions, which can be part of EPC or IMS.

##### 5.1.2.12.7.22	TS 28.527

Telecommunication management; Life Cycle Management (LCM) for mobile networks that include virtualized network functions; Stage 2

This document is the Life Cycle Management (LCM) stage 2 specification for mobile networks that include virtualized network functions, which can be part of EPC or IMS.

##### 5.1.2.12.7.23	TS 28.528

Telecommunication management; Life Cycle Management (LCM) for mobile networks that include virtualized network functions; Stage 3

This document is the Life Cycle Management (LCM) stage 3 specification for mobile networks that include virtualized network functions, which can be part of EPC or IMS.

##### 5.1.2.12.7.24	TS 28.611

Telecommunication management; Evolved Packet Core (EPC) and non-3GPP access interworking system Network Resource Model (NRM) Integration Reference Point (IRP); Requirements

This document describes the Network Resource Model (NRM) Integration Reference Point (IRP) requirements for the EPC and WLAN interworking system according to the structure defined in 3GPP TS 23.402 (e.g. ePDG, 3GPP AAA, etc).

##### 5.1.2.12.7.25	TS 28.612

Telecommunication management; Evolved Packet Core (EPC) and non-3GPP access interworking system Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)

This document is an Integration Reference Point (IRP) named "Evolved Packet Core (EPC) and non-3GPP access interworking system Network resource Model (NRM) IRP; Information Service (IS)", through which an 'IRPAgent' (typically an Element Manager or Network Element) can communicate configuration management information to one or several 'IRPManagers' (typically Network Managers) concerning interworking network resources.

This document specifies the semantics and behaviour of Information Object Class (IOC) attributes and relations visible across the reference point in a protocol and technology neutral way.  It does not define their syntax and encoding.

It reuses relevant parts of the generic NRM in 3GPP TS 28.622, either by direct reuse or sub-classing, and in addition to that defines specific IOCs in EPC and non-3GPP access interworking systems.

In order to access the information defined by this NRM, an interface IRP is needed, such as the Basic CM IRP IS (3GPP TS 32.602) or the Bulk CM IRP IS (3GPP TS 32.612). However, which interface IRP is applicable is outside the scope of this document.

##### 5.1.2.12.7.26	TS 28.616

Telecommunication management; Evolved Packet Core (EPC) and non-3GPP access interworking system Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions

This document is part of an Integration Reference Point (IRP) named Evolved Packet Core (EPC) and non-3GPP access Interworking System Network Resource Model (NRM) IRP, through which an IRPAgent can communicate configuration management information to one or several IRPManagers concerning EPC and non-3GPP access interworking system resources. The EPC and non-3GPP access Interworking System NRM IRP comprises a set of specifications defining requirements, a protocol neutral information service and one or more solution sets.

This document specifies the solution sets for the EPC and non-3GPP access interworking system NRM IRP.

##### 5.1.2.12.7.27	TS 28.667

Telecommunication management; Evolved Packet Core (EPC) and non-3GPP access interworking system Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions

This document specifies the requirements of the Radio Planning Tool Access (RPTA) Integration Reference Point (IRP). This IRP allows to read site and antenna data from RPTs.

##### 5.1.2.12.7.28	TS 28.668

Telecommunication management; Radio Planning Tool Access (RPTA) Integration Reference Point (IRP); Information Service (IS)

This document specifies the Radio Planning Tool Access (RPTA) management operations as well as support object classes, attributes and relations that can be communicated between the Service Provider in the RPT and one or several Service Consumers in the NM.

This document specifies the semantics and behaviour of management operations, support object classes, attributes and relations visible across the reference point in a protocol and technology neutral way. It does not define their syntax and encoding.

This IRP allows the NM to read planned site and antenna data from the RPT.

##### 5.1.2.12.7.29	TS 28.669

Telecommunication management; Radio Planning Tool Access (RPTA) Integration Reference Point (IRP); Solution Set (SS) definitions

This document specifies the Solution Set definitions (SS) of the Radio Planning Tool Access (RPTA) Integration Reference Point (IRP). This IRP allows the NM to read planned site and antenna data from the RPT.

##### 5.1.2.12.7.30	TS 28.680

Telecommunication management; Wireless Local Area Network (WLAN) management; Concepts and requirements

This document describes the concepts and requirements of WLAN management that focus on WLAN performance monitoring and alarm notifications.

##### 5.1.2.12.7.31	TS 28.681

Telecommunication management; Wireless Local Area Network (WLAN) Network Resource Model (NRM) Integration Reference Point (IRP); Requirements

This document describes the NRM IRP requirements of WLAN management.

##### 5.1.2.12.7.32	TS 28.682

Telecommunication management; Wireless Local Area Network (WLAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)

This document is part of an Integration Reference Point (IRP) named Wireless Local Area Networks (WLAN) Management Network Resource Model (NRM) IRP, through which an IRPAgent can communicate management information to one or several IRPManagers concerning WLAN management. The WLAN management NRM IRP comprises a set of specifications defining Requirements, a protocol neutral Information Service and one or more Solution Set(s).

This document specifies the protocol neutral WLAN management NRM IRP: Information Service (IS).

This document also contains stage 2 descriptions for those functionalities for the WLAN Management.

##### 5.1.2.12.7.33	TS 28.683

Telecommunication management; Wireless Local Area Network (WLAN) Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions

This document is part of an Integration Reference Point (IRP) named Wireless Local Area Network (WLAN) Management Network Resource Model (NRM) IRP, through which an IRPAgent can communicate management information to one or several IRPManagers concerning WLAN management. The WLAN management NRM IRP comprises a set of specifications defining Requirements, a protocol neutral Information Service and one or more Solution Set(s).

This document specifies the Solution Sets for the WLAN NRM IRP.

#### 5.1.2.12.8	Signalling protocols ("stage 3") - intra-fixed-network

##### 5.1.2.12.8.1	TS 29.002

Mobile Application Part (MAP) specification

It is necessary to transfer between entities of a Public Land Mobile Network (PLMN) information specific to the PLMN in order to deal with the specific behaviour of roaming Mobile Stations (MS)s. The Signalling System No. 7 specified by CCITT is used to transfer this information.

##### 5.1.2.12.8.2	TS 29.016

General Packet Radio Service (GPRS); Serving GPRS Support Node (SGSN) – Visitors Location Register (VLR); Gs interface network service specification

This document specifies the subset of MTP and SCCP which is used for the reliable transport of BSSAP+ messages in the Gs interface. The document references 3GPP TS 29.202 which specifies alternative transport layers that can be applied instead of the MTP. The document also specifies the SCCP addressing capabilities to be provided in the Gs interface.

##### 5.1.2.12.8.3	TS 29.018

General Packet Radio Service (GPRS); Serving GPRS Support Node (SGSN) – Visitors Location Register (VLR); Gs interface layer 3 specification

This document specifies procedures used on the Serving GPRS Support Node (SGSN) to Visitors Location Register (VLR) interface for interoperability between circuit switched services and packet data services. The document specifies the layer 3 messages and procedures on the Gs interface to allow coordination between databases and to relay certain messages related to GSM circuit switched services over the GPRS subsystem.

##### 5.1.2.12.8.4	TS 29.060

General Packet Radio Service (GPRS); GPRS Tunnelling Protocol (GTP) across the Gn and Gp interface

This document defines the second version of GTP used on: – the Gn and Gp interfaces of the General Packet Radio Service (GPRS); – the Iu, Gn and Gp interfaces of the UMTS system.

##### 5.1.2.12.8.5	TS 29.061

Interworking between the Public Land Mobile Network (PLMN) supporting packet based services and Packet Data Networks (PDN)

This document defines the requirements for Packet Domain interworking between a: a) PLMN and PDN; b) PLMN and PLMN. This document is valid for a PLMN in A/Gb mode as well as for a PLMN in Iu mode. If text applies only for one of these systems it is explicitly mentioned by using the terms “A/Gb mode” and “Iu mode”. The A interface does not play any role in the scope of this document although the term “A/Gb mode” is used.

##### 5.1.2.12.8.6	TS 29.078

Customised Applications for Mobile network Enhanced Logic (CAMEL) Phase 4; CAMEL Application Part (CAP) specification

This document specifies the CAMEL Application Part (CAP) supporting the fourth phase of the network feature Customized Applications for Mobile network Enhanced Logic. CAP is based on a sub-set of the ETSI Core INAP CS-2 as specified by ETSI EN 301 140-1. Descriptions and definitions provided by ETSI EN 301 140-1 are directly referenced by this standard in the case no additions or clarifications are needed for the use in the CAP.

##### 5.1.2.12.8.7	TS 29.079

Optimal media routeing within the IP Multimedia Subsystem (IMS); Stage 3

This document defines optional Optimal Media Routeing (OMR) procedures that can be applied by entities in the IP Multimedia Subsystem (IMS) that control media resources and are capable of manipulating the Session Description Protocol (SDP) as defined by IETF RFC 4566. The OMR procedures relate to the handling of OMR-specific SDP attributes that are documented in TS 24.229. The OMR procedures use SDP offer/answer related procedures in IETF RFC 3264 and in 3GPP TS 24.229 in a backward-compatible manner.

##### 5.1.2.12.8.8	TS 29.116

Representational state transfer over xMB reference point between content provider and BM-SC

This document describes the REST-based protocol for the xMB reference point between the Content Provider and the BM-SC. The xMB reference point and related stage 2 protocol procedures are defined in 3GPP TS 23.246 and in 3GPP TS 26.346.

##### 5.1.2.12.8.9	TS 29.118

Mobility Management Entity (MME) – Visitor Location Register (VLR) SGs interface specification

CS Fallback in the Evolved Packet System (EPS) enables the provisioning of CS-domain services (e.g. voice call, Location Services (LCS) or supplementary services) by reuse of CS infrastructure when the UE is served by E-UTRAN. Additionally, SMS delivery via the CS core network is realized without CS fallback. This document specifies the procedures and the SGs Application Part (SGsAP) messages used on the SGs interface between the Mobility Management Entity (MME) in the EPS and the Visitor Location Register (VLR), to allow location management coordination and to relay certain messages related to GSM circuit switched services over the EPS system. It also specifies the use of Stream Control Transmission Protocol (SCTP) for the transport of SGsAP messages.

##### 5.1.2.12.8.10	TS 29.128

Mobility Management Entity (MME) and Serving GPRS Support Node (SGSN) interfaces for interworking with packet data networks and applications

This document describes the Diameter-based interfaces between the SCEF/IWK-SCEF and other network entities such as MME/SGSN for the Architecture enhancements to facilitate communications with packet data networks and applications.

In particular, this document specifies the T6a interface between the MME and the SCEF, the T6ai interface between the MME and the IWK-SCEF, the T6b interface between the SGSN and the SCEF, the T6bi interface between the SGSN and the SCEF and the T7 interface between the SCEF and the IWK-SCEF. The procedures over those interfaces are defined in 3GPP TS 23.682.

##### 5.1.2.12.8.11	TS 29.153

Service capability exposure functionality over Ns reference point

This document defines the protocol for Ns reference point between the Service Capability Exposure Function (SCEF) and RAN Congestion Awareness Function (RCAF).

The Ns reference point and related stage 2 procedures are defined in 3GPP TS 23.682.

##### 5.1.2.12.8.12	TS 29.154

Service capability exposure functionality over Nt reference point

This document defines the protocol for Nt reference point. The functional requirements and the stage 2 specifications of the Nt reference point are contained in 3GPP TS 23.203. The Nt reference point lies between Service Capability Exposure Function (SCEF) and Policy and Charging Rules Function (PCRF).

##### 5.1.2.12.8.13	TS 29.155

Traffic steering control; Representational State Transfer (REST) over St reference point

This document describes the Representational State Transfer (REST) protocol-based St reference point, which is used to provision the traffic steering control information to the TSSF from the PCRF.

##### 5.1.2.12.8.14	TS 29.161

Interworking between the Public Land Mobile Network (PLMN) supporting packet based services with Wireless Local Area Network (WLAN) access and Packet data Networks (PDN)

This document defines the requirements for Packet Domain interworking between a PLMN with WLAN access and PDN; and the requirements for Packet Domain interworking between PLMN with WLAN access and PLMN. This document also defines the usage of Radius at the Pp Reference Point between the Packet Data Gateway and the Presence Network Agent.

##### 5.1.2.12.8.15	TS 29.162

Interworking between the IM CN subsystem and IP networks

The IM CN subsystem interworks with the external IP networks through the Mb reference point. This document details the interworking between the IM CN subsystem and external IP networks for IM service support. It addresses the issues of control plane interworking and, user plane interworking for specific interworking use cases. The document describes the IMS-Ix interface requirements in the form of Use Cases which require H.248 protocol procedures and also details the additional Information Elements required to perform the specific procedures. The IP version Interworking, between IP version 4 (IETF RFC 791) and IP version 6 (IETF RFC 2460) detailed in terms of the processes and protocol mappings required in order to support both mobile originated and terminated calls.

##### 5.1.2.12.8.16	TS 29.163

Interworking between the IP Multimedia (IM) Core Network (CN) subsystem and Circuit Switched (CS) networks

This document specifies the principles of interworking between the 3GPP IM CN subsystem and BICC/ISUP based legacy CS networks, in order to support IM basic voice, data and multimedia calls. This document addresses the areas of control and user plane interworking between the IM CN subsystem and CS networks through the network functions, which include the MGCF and IM-MGW. For the specification of control plane interworking, areas such as the interworking between SIP and BICC or ISUP are detailed in terms of the processes and protocol mappings required for the support of both IM originated and terminated voice and multimedia calls. Other areas addressed encompass the transport protocol and signalling issues for negotiation and mapping of bearer capabilities and QoS information.

##### 5.1.2.12.8.17	TS 29.164

Interworking between the 3GPP CS domain with BICC or ISUP as signalling protocol and external SIP-I networks

This document defines interworking procedures between a 3GPP CS domain (see 3GPP TS 23.205) which applies either BICC or ISUP as signalling protocol, and external networks that use SIP-I (see ITU-T Q.1912.8.5, Profile C) as signalling protocol. The document also describes the related interworking architecture. The control plane interworking is performed by an interworking unit at the interconnection between the 3GPP CS domain and an external SIP-I network. The user plane interworking is performed by an MGW. The document defines stage 2 procedures for the control of the MGW.

##### 5.1.2.12.8.18	TS 29.165

Inter-IMS Network to Network Interface (NNI)

The objective of this document addresses the Inter-IMS Network to Network Interface (II-NNI) consisting of Ici and Izi reference points between IMS networks in order to support end-to-end service interoperability. The document addresses the issues related to control plane signalling (3GPP usage of SIP and SDP protocols, required SIP header fields) as well as other interconnecting aspects like security, numbering/naming/addressing and user plane issues as transport protocol, media and codecs actually covered in a widespread set of 3GPP specifications. A profiling of the Inter-IMS Network to Network Interface (II-NNI) is also provided. Charging aspects are addressed as far as SIP signalling is concerned.

##### 5.1.2.12.8.19	TS 29.168

Cell Broadcast Centre interfaces with the Evolved Packet Core; Stage 3

This document specifies the procedures and the SBc Application Part (SBc-AP) messages used on the SBc-AP interface between the Mobility Management Entity (MME) and the Cell Broadcast Centre (CBC). This document supports the following functions.- Warning Message Transmission function in the EPS.

##### 5.1.2.12.8.20	TS 29.171

Location Services (LCS); LCS Application Protocol (LCS-AP) between the Mobile Management Entity (MME) and Evolved Serving Mobile Location Centre (E-SMLC); SLs interface

This document specifies the procedures and information coding for LCS Application Protocol (LCS-AP) that is needed to support the location services in E-UTRAN. The LCS-AP message set is applicable to the SLs interface between the E-SMLC and the MME. LCS-AP is developed in accordance to the general principles stated in 3GPP TS 23.271.

##### 5.1.2.12.8.21	TS 29.172

Location Services (LCS); Evolved Packet Core (EPC) LCS Protocol (ELP) between the Gateway Mobile Location Centre (GMLC) and the Mobile Management Entity (MME); SLg interface

This document specifies the procedures and information coding for the EPC LCS Protocol (ELP) that is needed to support the location services in E-UTRAN. The ELP message set is applicable to the SLg interface between the MME and the GMLC. ELP is developed in accordance to the general principles stated in 3GPP TS 23.271

##### 5.1.2.12.8.22	TS 29.173

Location Services (LCS); Diameter-based SLh interface for Control Plane LCS

This document describes the Diameter-based SLh interface between the GMLC and the HSS defined for the Control Plane LCS in EPC.

##### 5.1.2.12.8.23	TS 29.201

Representational State Transfer (REST) reference point between Application Function (AF) and Protocol Converter (PC)

This document describes the Representational State Transfer (REST) reference point, which is used to exchange application level session information between the Protocol Converter (PC) and the Application Function (AF). REST shall be used as an architectural style as appropriate.

##### 5.1.2.12.8.24	TS 29.202

Signalling System No. 7 (SS7) signalling transport in core network; Stage 3

This document defines the possible protocol architectures for transport of SS7 signalling protocols in Core Network.

##### 5.1.2.12.8.25	TS 29.204

Signalling System No. 7 (SS7) security gateway; Architecture, functional description and protocol details

This document provides the functional description of the SS7 Security Gateway. The document covers also network architecture, routeing considerations, and protocol details.

##### 5.1.2.12.8.26	TS 29.205

Application of Q.1900 series to bearer independent Circuit Switched (CS) core network architecture; Stage 3

This document describes the protocols to be used when ITU-T Q.1902 “Bearer Independent Call Control” is used as call control protocol in a 3GPP Bearer Independent CS core network 3GPP TS 23.205 The Q.1902 operates between (G)MSC servers. The BICC architecture as described in ITU-T Q.1902 consists of a number of protocols. The following types of protocols are described: call control protocol, bearer control protocols and a resource control protocol for this architecture. The architecture complies with the requirements imposed by 3GPP TS 23.205 and TS 23.153.

##### 5.1.2.12.8.27	TS 29.212

Policy and Charging Control (PCC) over Gx/Sd reference point

This document provides the stage 3 specification of the Gx and Gxx reference points. The functional requirements and the stage 2 specifications of the Gx and Gxx reference point are contained in 3GPP TS 23.203. The Gx reference point lies between the Policy and Charging Rule Function and the Policy and Charging Enforcement Function. The Gxx reference point lies between the Policy and Charging Rule Function and the Bearer Binding and Event Reporting Function. Whenever possible the document specifies the requirements for the protocol by reference to specifications produced by the IETF within the scope of Diameter. Where this is not possible, extensions to Diameter are defined within the document itself.

5.1.2.12.8.28	TS 29.213

Policy and charging control signalling flows and Quality of Service (QoS) parameter mapping

This specification adds detailed flows of Policy and Charging Control (PCC) over the Rx, Gx, Gxx and S9 reference points and their relationship with the bearer level signalling flows over the Gn/Gp, S4, S5/S8, S2a and S2c interfaces. The call flows depicted represent usual cases, but not all situations are covered. The document also describes the binding and the mapping of QoS parameters among SDP, UMTS QoS parameters, and QoS authorization parameters, as well as PCRF addressing using DRA.

##### 5.1.2.12.8.29	TS 29.214

Policy and charging control over Rx reference point

This document provides the stage 3 specification of the Rx reference point. The functional requirements and the stage 2 specifications of the Rx reference point are contained in 3GPP TS 23.203. The Rx reference point lies between the Application Function and the Policy and Charging Rule Function. Whenever possible this document specifies the requirements for the protocol by reference to specifications produced by the IETF within the scope of Diameter. Where this is not possible, extensions to Diameter are defined within the document itself.

##### 5.1.2.12.8.30	TS 29.215

Policy and Charging Control (PCC) over S9 reference point; Stage 3

This document provides the Stage 3 specification of the S9 reference point. The functional requirements of Stage 2 specification for the S9 reference point are contained in 3GPP TS 23.203. The S9 reference point lies between the PCRF in the home PLMN (also known as H-PCRF) and the PCRF in the visited PLMN (also known as V-PCRF). Whenever it is possible this document specifies the requirements for the protocols by reference to specifications produced by the IETF within the scope of Diameter. Where this is not possible extensions to Diameter are defined within this document.

##### 5.1.2.12.8.31	TS 29.217

Policy and Charging Control (PCC); Congestion reporting over Np reference point

This document provides the stage 3 specification of the Np reference point. The functional requirements and the stage 2 specifications of the Np reference point are contained in 3GPP TS 23.203. The Np reference point lies between the RAN Congestion Awareness Function (RCAF) and the Policy and Charging Rules Function (PCRF) for the non-roaming case, between the RCAF and the H-PCRF for the home-routed scenario and between the RCAF and the V-PCRF for the visited access scenario.

NOTE:	If not specified explicitly, the PCRF also means H-PCRF for the home-routed scenario or V-PCRF in the visited access scenario in the specification.

##### 5.1.2.12.8.32	TS 29.219

Policy and charging control: Spending limit reporting over Sy reference point

This document provides the protocol specification related to the Sy reference point. The Sy reference point lies between the Policy and Charging Rule Function (PCRF) and the Online Charging System (OCS). It enables transfer of policy counter status information relating to subscriber spending from OCS to PCRF.

##### 5.1.2.12.8.33	TS 29.228

IP Multimedia (IM) Subsystem Cx and Dx Interfaces; Signalling flows and message contents

This 3GPP Technical Specification (TS) specifies the interactions between the HSS (Home Subscriber Server) and the CSCF (Call Session Control Functions), referred to as the Cx interface, and the interactions between the CSCF and the SLF (Server Locator Function), referred to as the Dx interface.

##### 5.1.2.12.8.34	TS 29.229

Cx and Dx interfaces based on the Diameter protocol; Protocol details

This specification defines a transport protocol for use in the IP multimedia (IM) Core Network (CN) subsystem based on Diameter.

##### 5.1.2.12.8.35	TS 29.230

Diameter applications; 3GPP specific codes and identifiers

This document lists the 3GPP specific Diameter protocol codes, including the AVP codes and Experimental result codes. This document lists also the application identifiers assigned to 3GPP specific Diameter applications by IANA and the Diameter command code range which is assigned to 3GPP by IANA.

##### 5.1.2.12.8.36	TS 29.231

Application of SIP-I Protocols to Circuit Switched (CS) core network architecture; Stage 3

This specification describes the protocols to be used when SIP-I is optionally used as call control protocol in a 3GPP CS core network on Nc interface. The SIP-I protocol operates between (G)MSC servers. The SIP-I architecture consists of a number of protocols. The following types of protocols are described: call control protocol, resource control protocols and user plane protocol for this architecture.

##### 5.1.2.12.8.37	TS 29.232

Media Gateway Controller (MGC) – Media Gateway (MGW) interface; Stage 3

This document describes the protocol to be used on the Media Gateway Controller (MGC) – Media Gateway (MGW) interface. The Media Gateway Controllers covered in this specification are the MSC server and the GMSC server. The basis for this interface profile is the H.248.1 protocol. The usage of this protocol is described in 3GPP TS 23.205 and 3GPP TS 29.205 for BICC circuit switched core network, in 3GPP TS 23.231 and 3GPP TS 29.231 for SIP-I circuit switched core network.

This profile includes the support for the enhanced MSC server to provide access to IMS Centralised services as described in 3GPP TS 23.292 and thus the procedures related to this interface are described in 3GPP TS 29.292. As a result the interworking with SIP and the associated MGW control procedures described in 3GPP TS 29.163 for the interworking between BICC/ISUP circuit switched core network and IP Multimedia core network are incorporated. This specification describes the changes to H.248 which are needed to handle 3GPP specific traffic cases. This is done by using the H.248 standard extension mechanism. In addition certain aspects of the base protocol H.248 are not needed for this interface and thus excluded by this profile.

##### 5.1.2.12.8.38	TS 29.234

3GPP system to Wireless Local Area Network (WLAN) interworking; Stage 3

This document defines the stage-3 protocol description for several reference points in the WLAN-3GPP Interworking System.

The document is applicable to:

–   the Dw reference point between the 3GPP AAA Server and an SLF;

–   the Wa reference point between the WLAN AN and the 3GPP AAA Proxy;

–   the Wd reference point between the 3GPP AAA Proxy and 3GPP AAA Server;

–   the Wx reference point between the 3GPP AAA Server and the HSS;

–   the Wm reference point between the 3GPP AAA Server and the PDG;

–   the Wg reference point between the 3GPP AAA Server/Proxy and the WAG;

–   the Pr reference point between the 3GPP AAA Server and the PNA.

##### 5.1.2.12.8.39	TS 29.235

Interworking between SIP-I based circuit-switched core network and other networks

This specification defines the interworking between SIP-I based circuit-switched core network with out-of-band transcoder control related procedures and: – an external SIP-I based signalling network – an ISUP based network such as an ISUP based 3GPP CS Domain or an PSTN – an BICC based network such as an BICC based 3GPP CS Domain – an Internet Multimedia Subsystem.

##### 5.1.2.12.8.40	TS 29.238

Interconnection Border Control Functions (IBCF) – Transition Gateway (TrGW) interface, Ix interface; Stage 3

This document describes the protocol to be used on the Interconnection Border Control Function (IBCF) – Transition Gateway (TrGW) interface and the CS-IBCF – CS-TrGW interface. The basis for this protocol is the H.248 protocol. The Profile provides MG control function for IMS and CS Border Control. The IMS architecture is described in 3GPP TS 23.228. The underlying reference model and stage 2 information is described in 3GPP TS 23.228 and in 3GPP TS 29.162. The CS architecture is described in 3GPP TS 29.235.

This specification describes the application of H.248 Ix profile for both Ix and CS-Ix interfaces. Required extensions use the H.248 standard extension mechanism. In addition certain aspects of the base protocol H.248 are not needed for this interface and thus excluded by this profile.

##### 5.1.2.12.8.41	TS 29.244

Interface between the Control plane Plane and the User Plane of EPC Nodes

This document specifies the Packet Forwarding Control Protocol (PFCP) used on the interface between the control plane and the user plane function in a split SGW, PGW and TDF architecture in EPC.

The architecture reference model and stage 2 information are specified in 3GPP TS 23.214.

PFCP shall be used over the Sxa, Sxb, Sxc and the combined Sxa/Sxb reference points.

PFCP shall also be used over the Sxa' and Sxb' reference points specified in 3GPP TS 33.107. In the rest of this specification, no difference is made between Sxa and Sxa', or between Sxb and Sxb'. The Sxa' and Sxb' reference points reuse the protocol specified for the Sxa and Sxb reference points, but comply in addition with the security requirements specified in clause 8 of 3GPP 33.107.

##### 5.1.2.12.8.42	TS 29.250

Nu reference point between SCEF and PFDF for sponsored data connectivity

This document provides the stage 3 specification of the Nu reference point. The functional requirements and the stage 2 specifications of the Nu reference point are specified in 3GPP TS 23.682. The Nu reference point lies between the Packet Flow Description Function (PFDF) and the Service Capability Exposure Function (SCEF).

##### 5.1.2.12.8.43	TS 29.251

Gw and Gwn reference point for sponsored data connectivity

This document provides the stage 3 specification of the Gw and Gwn reference points. The functional requirements and the stage 2 specifications of the Gw and Gwn reference points are specified in 3GPP TS 23.203. The Gw reference point lies between the Packet Flow Description Function (PFDF) and the Policy and Charging Enforcement Function (PCEF). The Gwn reference point lies between the Packet Flow Description Function (PFDF) and the Traffic Detection Function (TDF).

##### 5.1.2.12.8.44	TS 29.272

Evolved Packet System (EPS); Mobility Management Entity (MME) and Serving GPRS Support Node (SGSN) related interfaces based on Diameter protocol

This document describes the Mobility Management Entity (MME) and Serving GPRS Support Node (SGSN) related diameter-based interfaces towards the Home Subscriber Server (HSS), and the MME and the SGSN related diameter-based interface towards the Equipment Identity Register (EIR). This specification defines the Diameter application for the MME-HSS, S6a reference point, and for the SGSN-HSS, S6d reference point. The interactions between the HSS and the MME/SGSN are specified, including the signalling flows. This specification defines the Diameter application for the MME-EIR, S13 reference point, and for the SGSN-EIR, S13’ reference point. The interactions between the MME/SGSN and the EIR are specified, including the signalling flows.

##### 5.1.2.12.8.45	TS 29.273

Evolved Packet System (EPS); 3GPP EPS AAA interfaces

This document defines the stage-3 protocol description for several reference points for the non-3GPP access in EPS.

##### 5.1.2.12.8.46	TS 29.274

3GPP Evolved Packet System (EPS); Evolved General Packet Radio Service (GPRS) Tunnelling Protocol for Control plane (GTPv2-C); Stage 3

This document specifies the stage 3 of the control plane of the GPRS Tunnelling Protocol, Version 2 for Evolved Packet System interfaces (GTPv2_C).

In this document, unless otherwise specified the S5 interface refers always to “GTP-based S5” and S8 interface refers always to “GTP_based S8” interface.

##### 5.1.2.12.8.47	TS 29.275

Proxy Mobile IPv6 (PMIPv6) based Mobility and Tunnelling protocols; Stage 3

This document specifies the Stage 3 of the PMIPv6 Based Mobility and Tunnelling Protocols used over the PMIP-based S2a, S2b, S5, and S8 reference points defined in 3GPP TS 23.402, and are thus applicable to the Serving GW, PDN Gateway, ePDG, and Trusted Non-3GPP Access. Protocols specifications are compliant with relevant IETF RFCs. In this specification PMIP refers to PMIPv6 as defined in IETF RFC 5213.

##### 5.1.2.12.8.48	TS 29.276

3GPP Evolved Packet System (EPS); Optimized handover procedures and protocols between E-UTRAN access and cdma2000 HRPD Access;
Stage 3

This document specifies the Stage 3 of the Evolved Packet System S101 interface between the MME and the HRPD Access Network. The S101 interface supports procedures for Pre-Registration, Session Maintenance and Active handoffs between E-UTRAN and HRPD networks.

##### 5.1.2.12.8.49	TS 29.278

Customized Applications for Mobile network Enhanced Logic (CAMEL) Phase 4; CAMEL Application Part (CAP) specification for IP Multimedia Subsystems (IMS)

This document specifies the CAMEL Application Part (CAP) supporting the fourth phase of the network feature Customized Applications for Mobile network Enhanced Logic for IP Multimedia CN subsystems. CAP is based on a sub-set of the ETSI Core INAP CS-2 as specified by ETSI EN 301 140?1. Descriptions and definitions provided by ETSI EN 301 140?1 are directly referenced by this standard in case no additions or clarifications are needed for the use in the CAP.

##### 5.1.2.12.8.50	TS 29.280

Evolved Packet System (EPS); 3GPP Sv interface (MME to MSC, and SGSN to MSC) for SRVCC

This document describes the Sv interface between the Mobility Management Entity (MME) or Serving GPRS Support Node (SGSN) and 3GPP MSC server enhanced for SRVCC. Sv interface is used to support Inter-RAT handover from VoIP/IMS over EPS to CS domain over 3GPP UTRAN/GERAN access or from UTRAN (HSPA) to 3GPP UTRAN/GERAN access.

##### 5.1.2.12.8.51	TS 29.281

General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U)

This document defines the user plane of GTP used on: – the Gn and Gp interfaces of the General Packet Radio Service (GPRS); – the Iu, Gn and Gp interfaces of the UMTS system; – the S1-U, X2, S4, S5, S8 and S12 interfaces of the Evolved Packet System (EPS).

##### 5.1.2.12.8.52	TS 29.282

Mobile IPv6 vendor specific option format and usage within 3GPP

This document specifies the format and usage of the Mobile IPv6 Vendor Specific Option within the Third Generation Partnership Project.

##### 5.1.2.12.8.53	TS 29.283

Diameter data management applications

The common functional architecture for support of Mission Critical (MC) services is specified in 3GPP TS 23.280.

The functional architecture for support of Mission Critical Push To Talk (MCPTT) services is specified in 3GPP TS 23.379.

The functional architecture for support of Mission Critical Video (MCVideo) services is specified in 3GPP TS 23.281.

The functional architecture for support of Mission Critical Data (MCData) services is specified in 3GPP TS 23.282.

This 3GPP Technical Specification (TS) specifies:

1.	The interactions between the MC Service User Database and the MC Service Server:

-	This interface between the MCPTT User Database and the MCPTT Server is referred to as the MCPTT-2 reference point, as specified in 3GPP TS 23.379.

-	This interface between the MCVideo User Database and the MCVideo Server is referred to as the MCVideo-2 reference point, as specified in 3GPP TS 23.281.

-	This interface between the MCData User Database and the MCData Server is referred to as the MCData-2 reference point, as specified in 3GPP TS 23.282.

-	This interface between the MCData User Database and the MCData Server is referred to as the MCVideo-2 reference point, as specified in 3GPP TS 23.282.

2.	The interactions between the MC Service User Database and the Configuration Management Server. This interface is referred to as the CSC-13 reference point.

##### 5.1.2.12.8.54	TS 29.292

Interworking between the IP Multimedia (IM) Core Network (CN) subsystem (IMS) and MSC Server for IMS Centralized Services (ICS)

IMS Centralized Services (ICS) enable the delivery of IM CN subsystem based multimedia telephony and supplementary services as defined in 3GPP TS 24.173 to users regardless of the attached access network type; e.g. CS domain access or IP-CAN.

This document specifies the principles of interworking between the IM CN subsystem and CS domain in order to enable ICS for UEs using CS domain access. The document addresses the area of registration procedures interworking between the CS domain and IM CN subsystem, and the areas of control and user plane interworking between the IM CN subsystem and CS domain through an MSC Server enhanced for ICS and CS-MGW respectively. This includes the signalling procedures between the MSC Server and CS-MGW. For the specification of control plane interworking, the document defines the protocol interworking between the 3GPP profile of SIP as described in 3GPP TS 24.229 and NAS signalling as described in 3GPP TS 24.008 required for the support of IM CN subsystem based multimedia telephony and supplementary services. The document addresses the area of supplementary service configuration interworking between the CS domain and IM CN subsystem. It is applicable to the MSC Server and CS-MGW.

##### 5.1.2.12.8.55	TS 29.303

Domain Name System Procedures; Stage 3

This document describes Domain Name System (DNS) Procedures for the Evolved Packet System. This document covers the Evolved Packet Core gateway node selection using DNS (e.g. SGW and PGW nodes) excluding all User Equipment (UE) initiated DNS-based discovery and selection procedures.

##### 5.1.2.12.8.56	TS 29.305

InterWorking Function (IWF) between MAP based and Diameter based interfaces

This document is to specify the InterWorking Functions (IWF):

–   between MAP-based Gr, Gf interfaces and Diameter-based S6a, S6d, S13, S13a interfaces;

–   between the S6a interface with SMS subscription data on the MME side and the S6a interface without SMS subscription data plus the MAP-based D interface for SMS subscription on the HSS side;

–   between the S6a interface with SMS subscription data on the MME side and the S6a interface without SMS subscription data plus the MAP-based D interface for SMS subscription on the HSS side;

–   between MAP-based C for SMS interface and Diameter-based S6c interface;

–   between MAP-based E for SMS interface and Diameter-based SGd interface.

##### 5.1.2.12.8.57	TS 29.311

Service level interworking for Messaging Services

This document specifies the protocol details of service level interworking between Instant Message (OMA-TS-SIMPLE_IM) or Chat Session 
(OMA-TS-CPM_Conv_Fnct) using the 3GPP IP Multimedia CN subsystem and the Short Message Service over both legacy CS/PS network as specified in the 3GPP TS 23.040 and a generic IP Connectivity Access Network (IP-CAN) as specified in the 3GPP TS 24.341. These include:

–   Procedures to implement service level interworking between IM and SM;

–   Procedures to implement service level interworking between CPM and SM;

–   Enhancement of the IP-SM-GW as an Application Server to support service selection, authorization and mapping between IM or CPM and SM protocols;

–   Interaction between service level interworking and transport layer interworking.

##### 5.1.2.12.8.58	TS 29.328

IP Multimedia (IM) Subsystem Sh interface; Signalling flows and message contents

This document specifies:

–   The interactions between the HSS (Home Subscriber Server) and the SIP AS (Application Server) and between the HSS and the OSA SCS (Service Capability Server). This interface is referred to as the Sh reference point.

–   The interactions between the SIP AS and the SLF (Subscription Locator Function) and between the OSA SCS and the SLF. This interface is referred to as the Dh reference point.

The IP Multimedia (IM) Core Network Subsystem stage 2 is specified in 3GPP TS 23.228 and the signalling flows for the IP multimedia call control based on SIP and SDP are specified in 3GPP TS 24.228.

The IP Multimedia (IM) Session Handling with the IP Multimedia (IM) call model is specified in 3GPP TS 23.218. This document addresses the signalling flows and message contents for the protocol at the Sh and Dh interface and also addresses how the functionality of Ph interface is accomplished.

The Presence Service Stage 2 description (architecture and functional solution) is specified in 3GPP TS 23.141.

##### 5.1.2.12.8.59	TS 29.329

Sh interface based on the Diameter protocol; Protocol details

This document defines a transport protocol for use in the IP multimedia (IM) Core Network (CN) subsystem based on Diameter and is applicable to:

–   The Sh interface between an AS and the HSS.

–   The Sh interface between an SCS and the HSS. Whenever it is possible this document specifies the requirements for this protocol by reference to specifications produced by the IETF within the scope of Diameter. Where this is not possible, extensions to Diameter are defined within this document.

##### 5.1.2.12.8.60	TS 29.332

Media Gateway Control Function (MGCF) – IM Media Gateway; Mn interface

This document describes the protocol to be used on the Media Gateway Control Function (MGCF) – IM Media Gateway (IM-MGW) interface. This interface provides the Media Gateway Control for interworking between the IP Multimedia Subsystem (IMS) and CS domain (ISUP, BICC and SIP-I). The basis for this protocol is the H.248 protocol. The IMS architecture is described in 3GPP TS 23.228.

The interaction of the MGCF-IM MGW interface signalling procedures in relation to the SIP, and BICC/ISUP signalling at the MGCF are described in 3GPP TS 29.163.

The interaction of the MGCF-IM MGW interface signalling procedures in relation to the IMS SIP and SIP-I on Nc at the MGCF are described in 3GPP TS 29.235. This specification describes the application of H.248 on the Mn interface. Required extensions use the H.248 standard extension mechanism. In addition certain aspects of the base protocol H.248 are not needed for this interface and thus excluded by this profile.

In addition this profile provides support for PSTN/ISDN Emulation as required by ETSI TISPAN. The main body of the specification provides an introduction to the use of the profile for the Mn interface and introduces any specific functionality (e.g. new packages) associated with the Mn.

##### 5.1.2.12.8.61	TS 29.333

Multimedia Resource Function Controller (MRFC) – Multimedia Resource Function Processor (MRFP) Mp interface; Stage 3

This document describes the protocol to be used on the Multimedia Resource Function Controller (MRFC) – Multimedia Resource Function Processor (MRFP) interface (Mp interface). The IMS architecture is described in 3GPP TS 23.228, the functional requirements are described in 3G TS 23.333. This specification defines a profile of the Gateway Control Protocol (H.248.1) for controlling Multimedia Resource Function Processor supporting in-band user interaction, conferencing and transcoding for multimedia-services.

##### 5.1.2.12.8.62	TS 29.334

IMS Application Level Gateway (IMS-ALG) – IMS Access Gateway (IMS-AGW); Iq Interface; Stage 3

This document describes the protocol to be used on the IMS Application Level Gateway (ALG) – IMS Access Gateway (IMS-AGW) interface. The basis for this protocol is the H.248 protocol as specified in ITU-T. The IMS architecture is described in 3GPP TS 23.228. The underlying reference model and stage 2 information is described in 3GPP TS 23.228 and in 3GPP TS 23.334. This specification describes the application of H.248 on the Iq interface. Required extensions use the H.248 standard extension mechanism. In addition certain aspects of the base protocol H.248 are not needed for this interface and thus excluded by this profile.

##### 5.1.2.12.8.63	TS 29.335

User Data Convergence (UDC); User data repository access protocol over the Ud interface; Stage 3

This document describes the Stage 3 user data repository access protocol over Ud interface.

##### 5.1.2.12.8.64	TS 29.336

Home Subscriber Server (HSS) diameter interfaces for interworking with packet data networks and applications

This document describes the Diameter-based interfaces between the HSS and other network elements involved in the architecture for interworking with packet data networks and applications, such as Machine-Type Communications (MTC). In particular, this document specifies the S6m interface between the Home Subscriber Server (HSS) and the MTC Interworking Function (MTC-IWF) and the S6n interface between the HSS and the MTC-AAA. 
The procedures over those interfaces are defined in 3GPP TS 23.682.

##### 5.1.2.12.8.65	TS 29.337

Diameter-based T4 Interface for communications with packet data networks and applications

This document describes the Diameter based interface between the Machine Type Communications-InterWorking Function (MTC-IWF) and the Short Message Service-Service Centre (SMS-SC) for communications with packet data networks and applications. This specification defines the Diameter application for the T4 reference point between the MTC-IWF and the SMS-SC. The interactions between the MTC-IWF and the SMS-SC are specified. 
The stage 2 description for communications with packet data networks and applications (architecture and functionality) is specified in the 3GPP TS 23.682.

##### 5.1.2.12.8.66	TS 29.338

Diameter based protocols to support Short Message Service (SMS) capable Mobile Management Entities (MMEs)

This document defines the Diameter-based interfaces specific to SMS when they are used in conjunction with the "SMS in MME" architecture specified in 3GPP TS 23.272. It comprises:

- the Diameter application for the S6c interface between the HSS and the SMS-GMSC or the SMS Router and between the SMS-GMSC and the SMS Router;

- the Diameter application for the SGd interface between the MME and the SMS-IWMSC or the SMS-GMSC or the SMS Router and between the SMS-GMSC and the SMS Router.

##### 5.1.2.12.8.67	TS 29.343

Proximity-services (ProSe) function to ProSe application server aspects (PC2); Stage 3

This document provides the stage 3 specification of the PC2 reference point. The functional requirements and the stage 2 procedures of the PC2 reference point are contained in 3GPP TS 23.303. The PC2 reference point lies between the ProSe Function and ProSe Application Server.

##### 5.1.2.12.8.68	TS 29.344

Proximity-services (ProSe) function to Home Subscriber Server (HSS) aspects; Stage 3

This document describes the Diameter-based PC4a interface between the Proximity-based Services (ProSe) Function and the Home Subscriber Server (HSS) defined for ProSe.

This specification defines the Diameter application for PC4a reference point between the ProSe Function and the HSS.The interactions between the ProSe Function and the HSS are specified.

##### 5.1.2.12.8.69	TS 29.345

Inter-Proximity-services (Prose) function signalling aspects; Stage 3

This document  describes the Diameter-based interfaces between the Proximity Services (ProSe) Function in the HPLMN and the ProSe Function in a local PLMN (PC6 interface) or between the Proximity Services (ProSe) Function in the HPLMN and the ProSe Function in a visited PLMN (PC7 interface).

This specification defines the Diameter application for PC6/PC7 reference points between the ProSe Functions. The interactions between the ProSe Functions are specified.

The stage 2 description for Proximity Services (ProSe) features in EPS is specified in 3GPP TS 23.303.

##### 5.1.2.12.8.70	TS 29.364

IP Multimedia Subsystem (IMS) Application Server (AS) service data descriptions for AS interoperability

This specification standardizes the structure and the coding of the service data that are transported over the Sh interface between an Application Server supporting Multimedia Telephony supplementary services as defined in 3GPPP TS 22.173 and the HSS. Two optional formats are specified. One is based on a binary coding of the service data and supports the subset of MMTEL services corresponding to PSTN/ISDN and CS supplementary services. The other uses an XML format and supports the full set of MMTEL Services.

##### 5.1.2.12.8.71	TS 29.368

Tsp interface protocol between the MTC Interworking Function (MTC-IWF) and Service Capability Server (SCS)

This document provides the protocol specification related to the Tsp reference point which is part of the architecture for Machine Type Communication.  The Tsp reference point lies between the Service Capability Server (SCS) and the Machine Type Communication Inter Working Function (MTC-IWF).

##### 5.1.2.12.8.72	TS 29.388

V2X Control Function to Home Subscriber Server (HSS) aspects (V4); Stage 3

This document describes the Diameter-based V4 interface between the V2X Control Function and the Home Subscriber Server (HSS) defined for V2X services.

This specification defines the Diameter application for V4 reference point between the V2X Control Function and the HSS. The interactions between the V2X Control Function and the HSS are specified.

The stage 2 description for V2X service features in EPS is specified in 3GPP TS 23.285.

##### 5.1.2.12.8.73	TS 29.389

Inter-V2X Control Function Signalling aspects (V6); Stage 3

This document describes the Diameter-based interfaces between the V2X Control Function in the HPLMN and the V2X Control Function in the VPLMN (V6 interface).

This specification defines the Diameter application for V6 reference points between the V2X Control Functions. The interactions between the V2X Control Functions are specified.

The stage 2 description for V2X service features in EPS is specified in 3GPP TS 23.285.

##### 5.1.2.12.8.74	TS 29.405

Nq and Nq Application Protocol (Nq-AP); Stage 3

This document specifies the procedures and the Nq and Nq' Application Protocol (Nq-AP) messages used on the Nq/Nq' interfaces between the RAN Congestion Awareness Function (RCAF) and the Mobility Management Entity (MME) or the Serving GPRS Support Node (SGSN). The related stage 2 requirements are specified in 3GPP TS 23.401 and 3GPP TS 23.060.

##### 5.1.2.12.8.75	TS 29.468

Group Communication System Enablers for LTE (GCSE_LTE); MB2 reference point; Stage 3

This document defines the protocol for the MB2 reference point between the Group Communication Service Application Server (GCS AS) and the Broadcast-Multicast Service Centre (BM-SC).

The MB2 reference point and related stage 2 procedures are defined in TS 23.468 as part of the Group Communication System Enablers for LTE. The stage 1 requirements for Group Communication System Enablers for LTE are specified in TS 22.468.

##### 5.1.2.12.8.76	TS 29.658

SIP Transfer of IP Multimedia Service Tariff Information; Protocol specification

This document specifies the protocol for the real-time transfer of Tariff Information between a Charge Determination Point (CDP) and a Charge Generation Point (CGP) by means of the Session Initiation Protocol (SIP). It identifies the protocol procedures and switching functions needed to support the transfer of tariff information related to IP multimedia services.

#### 5.1.2.12.9	Subscriber Identity Module (SIM / USIM), IC Cards. Test specs

##### 5.1.2.12.9.1	TS 31.101

UICC-terminal interface; Physical and logical characteristics

This document specifies the interface between the UICC and the Terminal for 3G telecom network operation. It specifies:

–   the requirements for the physical characteristics of the UICC;

–   the electrical interface between the UICC and the Terminal;

–   the initial communication establishment and the transport protocols;

–   the model which serves as a basis for the logical structure of the UICC;

–   the communication commands and the procedures;

–   the application independent files and protocols.

##### 5.1.2.12.9.2	TS 31.102

Characteristics of the Universal Subscriber Identity Module (USIM) application

This document defines the USIM application for 3G telecom network operation. It specifies:

–   specific command parameters;

–   file structures;

–   contents of EFs (Elementary Files);

–   security functions;

–   application protocol to be used on the interface between UICC (USIM) and ME. This is to ensure interoperability between a USIM and an ME independently of the respective manufacturer, card issuer or operator

##### 5.1.2.12.9.3	TS 31.103

Characteristics of the IP Multimedia Services Identity Module (ISIM) application

This document defines the ISIM application for access to IMS services. This document specifies:

–   specific command parameters;

–   file structures;

–   contents of EFs (Elementary Files);

–   security functions;

–   application protocol to be used on the interface between UICC (ISIM) and Terminal. This is to ensure interoperability between an ISIM and Terminal independently of the respective manufacturer, card issuer or operator.

This document does not define any aspects related to the administrative management phase of the ISIM. Any internal technical realization of either the ISIM or the Terminal is only specified where these are reflected over the interface. This document does not specify any of the security algorithms that may be used.

##### 5.1.2.12.9.4	TS 31.104

Characteristics of the Hosting Party Subscription Identity Module (HPSIM) application

This document defines the Hosting Party Subscription Identity Module (HPSIM) application. This application resides on the UICC, an IC card specified in 3GPP TS 31.101.

3GPP TS 31.104 applies to a H(e)NB supporting the HPSIM for H(e)NB Hosting Party authentication and specifies:

–   identification of the Hosting Party;

–   security mechanism, e.g. authentication based on EAP-AKA method.

##### 5.1.2.12.9.5	TS 31.111

Universal Subscriber Identity Module (USIM) Application Toolkit (USAT)

This document defines the ISIM application for access to IMS services. It specifies:

–   specific command parameters;

–   file structures;

–   contents of EFs (Elementary Files);

–   security functions;

–   application protocol to be used on the interface between UICC (ISIM) and Terminal. This is to ensure interoperability between an ISIM and Terminal independently of the respective manufacturer, card issuer or operator. This document defines the interface between the UICC and the Mobile Equipment (ME), and mandatory ME procedures, specifically for “USIM Application Toolkit”.

The document mainly refers to ETSI TS 102 223, which describes the generic aspects of application toolkits within the UICC. USAT is a set of commands and procedures for use during the network operation phase of 3G/LTE, in addition to those defined in TS 31.101.

Specifying the interface is to ensure interoperability between a UICC and an ME independently of the respective manufacturers and operators.

This document defines for 3G/LTE technology:

–   the commands;

–   the application protocol;

–   the mandatory requirements on the UICC and ME for each procedure.

##### 5.1.2.12.9.6	TS 31.115

Secured packet structure for (Universal) Subscriber Identity Module (U)SIM Toolkit applications

This document specifies the structure of the Secured Packets in implementations using Short Message Service Point to Point (SMS-PP), Short Message Service Cell Broadcast (SMS-CB), Unstructured Supplementary Service Data (USSD) and Hyper Text Transfer Protocol (HTTP) based on ETSI TS 102 225. The structure of the Secured Packets complies with the one defined in ETSI TS 102 225. This document only contains additional requirements or explicit limitations for SIM/USIM applications. It is applicable to the exchange of secured packets between an entity in a 3G or GSM PLMN and an entity in the (U)SIM. Secured Packets contain application messages to which certain mechanisms according to ETSI TS 102 224 have been applied. Application messages are commands or data exchanged between an application resident in or behind the PLMN and on the (U)SIM. The Sending/Receiving Entity in the 3G or GSM PLMN and the UICC are responsible for applying the security mechanisms to the application messages and thus turning them into Secured Packets.

##### 5.1.2.12.9.7	TS 31.116

Remote APDU Structure for (U)SIM Toolkit applications

This document defines the remote management of files and applets on the SIM/USIM/ISIM.

It describes the APDU format for remote management. Furthermore the document specifies:

–   a set of commands coded according to this APDU structure and used in the remote file management on the SIM/USIM specified in TS 51.011, 
TS 31.101, TS 31.102, TS 31.103;

–   a set of commands coded according to this APDU structure and used in the remote applet management on the SIM/USIM. This is based 
on ETSI TS 102 226.

The remote APDU structure for SIM/USIM/ISIM applications complies with the one defined in ETSI TS 102 226. This document only contains additional requirements or explicit limitations for SIM/USIM/ISIM applications.

##### 5.1.2.12.9.8	TS 31.130

(U)SIM Application Programming Interface (API); (U)SIM API for Java Card

This document defines the (U)SIM Application Programming Interface extending the UICC API for Java Card, ETSI TS 102 241. This API allows the development of a (U)SAT application running together with a (U)SIM application and using GSM/3G network features.

##### 5.1.2.12.9.9	TS 31.133

IP Multimedia Services Identity Module (ISIM) Application Programming Interface (API); ISIM API for Java Card

This document defines the ISIM Application Programming Interface extending the UICC API for Java Card, ETSI TS 102 241. This API allows the development of an application running together with a ISIM application.

##### 5.1.2.12.9.10	TS 31.220

Characteristics of the Contact Manager for 3GPP UICC applications

This document defines the Contact Manager for 3GPP UICC applications based on OMA DS. It specifies the external interface between the Contact Manager Server in the UICC and the Contact Manager External Client in the ME.

##### 5.1.2.12.9.11	TS 31.221

Contact Manager Application Programming Interface (API); Contact Manager API for Java Card

This document defines the internal interface characteristics of the Contact Manager for 3GPP UICC applications.

The internal interface between the Contact Manager Server application on the UICC and the Contact Manager Client application on the UICC enables Java Card platform based applets to invoke and register to the Contact Manager Server services. In particular, the Contact Manager Java Card based API provides methods to:

–   Read/Update/Create/Delete contact(s) in the Contact Manager Server;

–   Manage group of contacts in the Contact Manager Server;

–   Search for a contact in the Contact Manager Server storage;

–   manage the contacts structure;

–   Register/Un-register the application to pre-defined events (e.g. application to be notified when contacts are modified in the Contact Manager Server).

#### 5.1.2.12.10	OAM&P and Charging

##### 5.1.2.12.10.1	TS 32.101

Telecommunication management; Principles and high level requirements

This document establishes and defines the management principles and high-level requirements for the management of PLMNs. In particular, this document identifies the requirements for:

–   the upper level of a management system;

–   the reference model, showing the elements the management system interacts with;

–   the network operator processes needed to run, operate and maintain a network;

–   the functional architecture of the management system;

–   the principles to be applied to management interfaces.

The requirements identified in this document are directed to the further development of management specifications as well as the development of management products. This document can be seen as guidance for the development of all other technical specifications addressing the management of PLMNs.

##### 5.1.2.12.10.2	TS 32.102

Telecommunication management; Architecture

This document identifies and standardizes the most important and strategic contexts in the physical architecture for the management of PLMNs. It serves as a framework to help define a telecom management physical architecture for a planned PLMN and to adopt standards and provide products that are easy to integrate. The requirements identified in this document are applicable to all further development of 3GPP Telecom Management specifications as well as the development of PLMN Management products. This document can be seen as guidance for the development of all other technical specifications addressing the management of PLMNs, except TS 32.101.

##### 5.1.2.12.10.3	TS 32.103

Telecommunication management; Integration Reference Point (IRP) overview and usage guide

This document gives an overview about 3GPP's management interface capabilities and related functionality. It provides high-level information about IRP Framework, available IRPs as well as their relationship with each other. This document is intended to be a guide into the world of 3GPP management specifications, enabling also non-subject matter experts to gain an understanding about 3GPP's management solutions. It also provides IRP packaging suggestions, intended to guide service providers as well as equipment and solutions providers to identify and choose suitable standardized management interface capabilities.

##### 5.1.2.12.10.4	TS 32.130

Telecommunication management; Network sharing; Concepts and requirements

This document describes concepts and high-level requirements for the Operations, Administration, Maintenance and Provisioning (OAM&P) of network sharing.

Network sharing scenarios considered in this document are Multiple Operator Core Network (MOCN) and Gateway Core Network (GWCN) for GERAN, UTRAN and E-UTRAN, as defined in TS 23.251).

##### 5.1.2.12.10.5	TS 32.150

Telecommunication management; Integration Reference Point (IRP) Concept and definitions

This document provides the overall concept for all Integration Reference Point (IRP) specifications. Relevant IRP overview and high-level definitions are already provided in 3GPP TS 32.101 and TS 32.102. IRP specifications are intended to be applicable to any management interface developed in 3GPP SA5.

##### 5.1.2.12.10.6	TS 32.253

Telecommunication management; Charging management; Control Plane (CP) data transfer domain charging

This document is part of a series of documents that specify charging functionality and charging management in GSM/UMTS/LTE networks. The GSM/UMTS/LTE core network charging architecture and principles are specified in TS 32.240, which provides an umbrella for other charging management TSs that specify:

-	the content of the CDRs per domain / subsystem / service (offline charging);

-	the content of real-time charging messages per domain / subsystem / service (online charging);

-	the functionality of online and offline charging for those domains / subsystems / services;

-	the interfaces that are used in the charging framework to transfer the charging information (i.e. CDRs or charging events).

The complete document structure for these TSs is defined in TS 32.240.

This document specifies the offline and online charging description for Control Plane (CP) data transfer domain, based on the functional stage 2 description in TS 23.682 for Non-IP Data Delivery (NIDD) using SCEF procedures. This charging description includes the offline and online charging architecture and scenarios specific to Control Plane (CP) data transfer domain, as well as the mapping of the common 3GPP charging architecture specified in TS 32.240 onto the Control Plane (CP) data transfer domain. It further specifies the structure and content of the CDRs for offline charging. This document is related to other 3GPP charging TSs as follows:

-	The common 3GPP charging architecture is specified in TS 32.240.

-	The parameters, abstract syntax and encoding rules for the CDRs are specified in TS 32.298.

-	A transaction based mechanism for the transfer of CDRs within the network is specified in TS 32.295.

-	The file based mechanism used to transfer the CDRs from the network to the operator's billing domain (e.g. the billing system or a mediation device) is specified in TS 32.297.

-	The 3GPP Diameter application that is used for Control Plane (CP) data transfer domain offline and online charging is specified in TS 32.299.

Control Plane (CP) data transfer is also provided by Short Message Service (SMS), for which charging functionalities are not specified in this document. SMS charging functionality is defined for Circuit Switched domain in TS 32.250, for Packet Switched domain in TS 32.251 and for SMS Nodes in TS 32.274.

##### 5.1.2.12.10.7	TS 32.276

Telecommunication management; Charging management; Voice Call Service (VCS) charging

This document is part of a series of documents that specify charging functionality and charging management in GSM/UMTS networks. The GSM/UMTS core network charging architecture and principles are specified in TS 32.240, which provides an umbrella for other charging management TSs.

This document specifies the online charging description for Voice Call Service (VCS) charging from a Proxy Function, based on the functional description of the Voice Call Service charging in the EU Roaming Regulations III specifications.

The concept of voice call is interpreted as any CS call, whatever the teleservice used (speech, 3.1 kHz audio, Fax, or CS data) except CS video telephony calls (BS 37, 64 kbit/s unrestricted digital info mode).

Voice over LTE is not included in this definition. This charging specification includes the online charging architecture and scenarios specific to VCS charging from a Proxy Function, as well as the mapping of the common 3GPP charging architecture specified in TS 32.240 onto VCS. It further specifies the charging events for online charging.

##### 5.1.2.12.10.8	TS 32.277

Telecommunication management; Charging management; Proximity-based Services (ProSe) charging

This document is part of a series of documents that specify charging functionality and charging management in GSM/UMTS networks. The GSM/UMTS core network charging architecture and principles are specified in TS 32.240, which provides an umbrella for other charging management TSs

This document specifies the offline and online charging description for the Proximity-based Services (ProSe), based on the stage 2 description of ProSe in TS 23.303. This charging description includes the offline and online charging architecture and scenarios specific to the ProSe, as well as the mapping of the common 3GPP charging architecture specified in TS 32.240 onto the ProSe. It further specifies the structure and content of the CDRs for offline charging, and the charging events for online charging.

##### 5.1.2.12.10.9	TS 32.278

Telecommunication management; Charging management; Monitoring event charging

This document is part of a series of documents that specify charging functionality and charging management in GSM/UMTS/LTE networks. The GSM/UMTS/LTE core network charging architecture and principles are specified in TS 32.240, which provides an umbrella for other charging management TSs that specify:

-	the content of the CDRs per domain / subsystem / service (offline charging);

-	the content of real-time charging messages per domain / subsystem / service (online charging);

-	the functionality of online and offline charging for those domains / subsystems / services;

-	the interfaces that are used in the charging framework to transfer the charging information (i.e. CDRs or charging events).

The complete document structure for these TSs is defined in TS 32.240.

This document specifies the Offline Charging description for Monitoring Events, based on the functional stage 2 description in TS 23.682. This charging description includes the offline charging architecture and scenarios specific to Monitoring Events, as well as the mapping of the common 3GPP charging architecture specified in TS 32.240 onto the 3GPP Architecture for Service Capability Exposure. It further specifies the structure and content of the CDRs for offline charging. This document is related to other 3GPP charging TSs as follows:

-	The common 3GPP charging architecture is specified in TS 32.240.

-	The parameters, abstract syntax and encoding rules for the CDRs are specified in TS 32.298.

-	A transaction based mechanism for the transfer of CDRs within the network is specified in TS 32.295.

-	The file based mechanism used to transfer the CDRs from the network to the operator's billing domain (e.g. the billing system or a mediation device) is specified in TS 32.297.

-	The 3GPP Diameter application that is used for Monitoring Event offline charging is specified in TS 32.299.

The online charging functionality for Monitoring Events is outside the scope of 3GPP standardisation.

All references, abbreviations, definitions, descriptions, principles and requirements, used in this document, that are common across 3GPP TSs, are defined in the 3GPP Vocabulary, TR 21.905. Those that are common across charging management in GSM/UMTS/LTE domains, services or subsystems are provided in the umbrella document TS 32.240 and are copied into clause 3 of this document for ease of reading. Finally, those items that are specific to this document are defined exclusively in this document.

##### 5.1.2.12.10.10	TS 32.281

Telecommunication management; Charging management; Announcement service

During any phase of a voice or video call, the Online Charging System (OCS) may need to deliver billing/charging in-session notifications to the end user via announcements as part of the rating, balance management and billing process. For example, the OCS may need to inform the user about usage states, threshold crossings, offer statuses, reason for call rejection, alerts about low balances, etc.

In Circuit Switched (CS) domain, online charging of a voice session is performed using Customised Applications for Mobile network Enhanced Logic (CAMEL) mechanisms. In order to deliver in-session notifications or to interact with the end user, OCS can use CAMEL messages such as: Play Announcement, Prompt and Collect, Connect to Resource, Establish Temporary Connection. Such notifications can be delivered pre-, mid- or post-call. However, in IMS and MMTel charging defined in TS 32.260 and TS 32.275 as well as in Diameter Credit-Control Application (DCCA) defined in RFC 4006, announcement capabilities, available at the OCS level, are limited to redirection of a session to a given SIP URI at completion of the session. When redirected the session may be connected to a resource that delivers an announcement.

The Announcement service in this specification provides announcement capabilities for use in a Diameter based online charging session. The OCS is able to specify that a given announcement be played to a call party in order to deliver charging/billing related notifications.

##### 5.1.2.12.10.11	TS 32.293

Telecommunication management; Charging management; Proxy function

This document is part of a series of documents that specify charging functionality and charging management in GSM/UMTS networks. The GSM/UMTS core network charging architecture and principles are specified in TS 32.240, which provides an umbrella for other charging management TSs.

This document specifies the Proxy Function in Domestic Service Provider (DSP).

This document contains the architecture and functions of the Proxy Function logical components and thereby derives the functionality of the Proxy Function interfaces. This charging description only defines the interworking between CAMEL and Diameter. The Ro interface between the Proxy Function and Online Charging System (OCS) is out of scope of this document.

##### 5.1.2.12.10.12	TS 32.508

Telecommunication management; Procedure flows for multi-vendor plug-and-play eNode B connection to the network

This document describes the procedure flows between network entities involved in the  multi-vendor plug-and-play eNB connection to network.

These procedures are based on requirements and use cases specified in 3GPP TS 32.501.

The format of the data exchanged in these procedures is defined in 3GPP TS 32.509.

##### 5.1.2.12.10.13	TS 32.509

Telecommunication management; Data formats for multi-vendor plug and play eNode B connection to the network

This document describes the data formats used between network entities involved in the multi-vendor plug and play eNB connection to network.

These data formats are based on requirements and use cases specified in 3GPP TS 32.501.

The procedure flows where these data exchanged are defined in 3GPP TS 32.508.

#### 5.1.2.12.11	Security aspects

##### 5.1.2.12.11.1	TS 33.102

3G security; Security architecture

Provides a specification of all security mechanisms and protocols, except algorithms.

##### 5.1.2.12.11.2	TS 33.105

3G Security; Cryptographic algorithm requirements

Defines requirements for standard cipher and integrity algorithm.

##### 5.1.2.12.11.3	TS 33.106

3G security; Lawful interception requirements

Defines all requirements for network based lawful interception.

##### 5.1.2.12.11.4	TS 33.107

3G security; Lawful interception architecture and functions

This document describes the architecture and functional requirements within a Third Generation Mobile Communication System (3GMS) and the Evolved Packet System (EPS). The specification shows the service requirements from a Law Enforcement point of view only. The aim of this document is to define a 3GMS and EPS interception system that supports a number of regional interception regulations, but these regulations are not repeated here as they vary. Regional interception requirements shall be met in using specific (regional) mediation functions allowing only required information to be transported.

##### 5.1.2.12.11.5	TS 33.108

3G security; Handover interface for Lawful Interception (LI)

This specification addresses the handover interfaces for Lawful Interception (LI) of Packet-Data Services, Circuit Switched Services, Multimedia Services within the UMTS network and Evolved Packet System (EPS). The handover interface in this context includes the delivery of Intercept Related Information (HI2) and Content of Communication (HI3) to the Law Enforcement Monitoring Facility.

##### 5.1.2.12.11.6	TS 33.110

Key establishment between a Universal Integrated Circuit Card (UICC) and a terminal

This document describes the security features and mechanisms to provision a shared key between a UICC and a terminal that may host the UICC or be connected to the device hosting the UICC via a local interface. Candidate applications to use this key establishment mechanism include but are not restricted to secure channel between a UICC and a terminal. The scope of this specification includes an architecture overview and the detailed procedure how to establish the shared key between the UICC and the terminal.

##### 5.1.2.12.11.7	TS 33.116

Security Assurance Specification (SCAS) for the MME network product class

This document contains objectives, requirements and test cases that are specific to the MME network product class. It refers to the Catalogue of General Security Assurance Requirements and formulates specific adaptions of the requirements and test cases given there, as well as specifying requirements and test cases unique to the MME network product class.

##### 5.1.2.12.11.8	TS 33.117

Catalogue of general security assurance requirements

This document contains objectives, requirements and test cases that are deemed applicable, possibly after adaptation, to several network product classes.

It has been recognized that several network product classes will share very similar if not identical security requirements for some aspects. Therefore, these are collected in this "catalogue" document applicable to many network product classes. In addition to this catalogue, requirements specific to different network product classes will be captured in separate documents.

##### 5.1.2.12.11.9	TS 33.141

Presence service; Security

This document is the Stage 2 specification for the security requirements, security architecture, security features and security mechanisms for the Presence Service, which includes the elements necessary to realize the requirements in TS 22.141 and TS 23.141. As far as SIP-based procedures are concerned, this specification refers to TS 33.203. The main content of this specification is the security for the Ut reference point, which is HTTP–based, as applied in presence services. The document includes information applicable to network operators, service providers and manufacturers.

##### 5.1.2.12.11.10	TS 33.179

Security of Mission Critical Push To Talk (MCPTT) over LTE

This document specifies the security architecture, procedures and information flows needed to protect the mission critical push to talk (MCPTT) service. The architecture includes mechanisms for authentication, protection of MCPTT signalling and protection of MCPTT media. Security for both MCPTT group calls and MCPTT private calls operating in on-network and off-network modes of operation is specified.

The functional architecture for MCPTT is defined in 3GPP TS 23.179, the corresponding service requirements are defined in 3GPP TS 22.179.

The MCPTT service can be used for public safety applications and also for general commercial applications e.g. utility companies and railways. As the security model is based on the public safety environment, some security features may not be applicable to MCPTT for commercial purposes.

##### 5.1.2.12.11.11	TS 33.180

Security of the mission critical service

This document specifies the security architecture, procedures and information flows needed to protect the mission critical service (MCX). The architecture includes mechanisms to protect the Common Functional Architecture and security mechanisms for mission critical applications. This includes Push-To-Talk (MCPTT), Video (MCVideo) and Data (MCData). Additionally, security mechanisms relating to on-network use, off-network use, roaming, migration, interconnection, interworking and multiple security domains are described.

This specification complements the Common Functional Architecture defined in TS 23.280, the functional architecture for MCPTT defined in 3GPP TS 23.379, the functional architecture for MCVideo defined in 3GPP TS 23.281 and the functional architecture for MCData defined in 3GPP TS 23.282.

The MC service can be used for public safety applications and also for general commercial applications e.g. utility companies and railways. As the security model is based on the public safety environment, some security features may not be applicable to MCPTT for commercial purposes.

##### 5.1.2.12.11.12	TS 33.185

Security aspect for LTE support of Vehicle-to-Everything (V2X) services

This document specifies the security aspects of V2X features in LTE, including security architecture, security requirements on the network entities that are used to support V2X services, as well as the procedures and solutions which are provided to meet those requirements.

##### 5.1.2.12.11.13	TS 33.187

Security aspects of Machine-Type Communications (MTC) and other mobile data applications communications enhancements

This document specifies the security architecture enhancements (i.e., enhancements to the security features and the security mechanisms) to facilitate Machine-Type and other mobile data applications Communications enhancements (MTCe) as per the use cases and service requirements defined in 3GPP TS 22.368 and the architecture enhancements and procedures defined in 3GPP TS 23.682.

##### 5.1.2.12.11.14	TS 33.203

3G security; Access security for IP-based services

The scope for this technical specification is to specify the security features and mechanisms for secure access to the IM subsystem (IMS) for the 3G mobile telecommunication system. Since the scope also encompasses the use of these security features and mechanisms for secure access to IMS in the context of fixed broadband networks and 3GPP2 networks, Annex L and Annex S specify how the material in the main body and other normative Annexes of this document apply to the fixed broadband networks and 3GPP2 networks respectively. The IMS supports IP Multimedia applications such as video, audio and multimedia conferences. SIP, Session Initiation Protocol, was chosen as the signalling protocol for creating and terminating Multimedia sessions, cf. RFC 3261. This specification only deals with how the SIP signalling is protected between the subscriber and the IMS, how the subscriber is authenticated and how the subscriber authenticates the IMS.

##### 5.1.2.12.11.15	TS 33.204

3G Security; Network Domain Security (NDS); Transaction Capabilities Application Part (TCAP) user security

This technical specification covers the security mechanisms and procedures necessary to protect all TCAP user messages which are sent between different security domains. The complete set of enhancements and extensions to facilitate security protection for the TCAP protocol is termed TCAPsec and it covers transport security in the TCAP protocol itself and the security management procedures. This technical specification contains the stage 2 specification for security protection of the TCAP protocol. The actual implementation (stage 3) specification can be found in TS 29.204.

##### 5.1.2.12.11.16	TS 33.210

3G security; Network Domain Security (NDS); IP network layer security

This document defines the security architecture for network domain IP based control planes, which shall be applied to NDS/IP-networks (i.e. 3GPP and fixed broadband networks). The scope of network domain control plane security is to cover the control signalling on selected interfaces between network elements of NDS/IP networks.

##### 5.1.2.12.11.17	TS 33.220

Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)

This document describes the security features and a mechanism to bootstrap authentication and key agreement for application security from the 3GPP AKA mechanism. Candidate applications to use this bootstrapping mechanism include but are not restricted to subscriber certificate distribution TS 33.221. Subscriber certificates support services whose provision mobile operator assists, as well as services that mobile operator provides. The scope of this specification includes a generic AKA bootstrapping function, an architecture overview and the detailed procedure how to bootstrap the credential. Clause 4 of this specification describes a mechanism, called GBA_ME, to bootstrap authentication and key agreement, which does not require any changes to the UICC. Clause 5 of this specification describes a mechanism, called GBA_U, to bootstrap authentication and key agreement, which does require changes to the UICC, but provides enhanced security by storing certain derived keys on the UICC.

##### 5.1.2.12.11.18	TS 33.221

Generic Authentication Architecture (GAA); Support for subscriber certificates

This document describes subscriber certificate distribution by means of generic bootstrapping architecture (GBA) TS 33.220. Subscriber certificates support services whose provision the mobile operator assists, as well as services that are offered by the mobile operator. The scope of this specification presents signalling procedures for support of issuing certificates to subscribers and the standard format of certificates and digital signatures. It is not intended to duplicate existing standards being developed by other groups on these topics, and will reference these where appropriate.

##### 5.1.2.12.11.19	TS 33.222

Generic Authentication Architecture (GAA); Access to network application functions using Hypertext Transfer Protocol over Transport Layer Security (HTTPS)

This document specifies secure access methods to Network Application Functions (NAF) using HTTP over TLS in the Generic Authentication Architecture (GAA), and provides Stage 2 security requirements, principles and procedures for the access. This document describes both direct access to an Application Server (AS) and access to an Application Server through an Authentication Proxy (AP).

##### 5.1.2.12.11.20	TS 33.223

Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA) push layer

This document specifies a Push Function as a functional add-on for the Generic Authentication Architecture (GAA).

##### 5.1.2.12.11.21	TS 33.224

Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA) push layer

This document specifies a generic push layer that makes use of the GBA Push Function as specified in TS 33.223. The GPL specification includes a message format, cipher suites and processing model. GPL assumes that keys and other SA parameters have been preinstalled in the Push-NAF and UE in the form of a NAF SA. GPL is a protection protocol that can be applied in a unidirectional fashion. The rationale for GPL is that having each application specify its own security mechanisms would for obvious reasons lead to duplication of work, specifications and implementations.

Using a generic secure push layer avoids these problems. A generic secure push layer may also relieve the applications using the service of having to be aware of inner working of the security layer.

##### 5.1.2.12.11.22	TS 33.234

3G security; Wireless Local Area Network (WLAN) interworking security

This document specifies the security architecture; trust model and security requirements for the interworking of the 3GPP System and WLAN Access Networks. This specification is not limited to WLAN technologies. It is also valid for other IP based Access Networks that support the same security capabilities towards the interworking system as WLAN does. Specifications of the appropriate mechanisms for user and network authentication, key management, service authorization, confidentiality and integrity protection of user and signalling data are also provided.

##### 5.1.2.12.11.23	TS 33.246

3G Security; Security of Multimedia Broadcast/Multicast Service (MBMS)

The Technical Specification covers the security procedures of the Multimedia Broadcast/Multicast Service (MBMS) for 3GPP systems (UTRAN, GERAN and E-UTRAN). MBMS is a 3GPP system network bearer service over which many different applications could be carried. The actual method of protection may vary depending on the type of MBMS application.

##### 5.1.2.12.11.24	TS 33.250

Security assurance specification for the PGW network product class

This document contains requirements and test cases that are specific to the PGW network product class. It refers to the Catalogue of General Security Assurance Requirements and formulates specific adaptions of the requirements and test cases given there, as well as specifying requirements and test cases unique to the PGW network product class.

##### 5.1.2.12.11.25	TS 33.259

Key establishment between a UICC hosting device and a remote device

This document describes the security features and mechanisms to provision a shared key between a UICC Hosting Device and a Remote Device connected via a local interface. The shared secret is then intended to be used to secure the interface between the Remote Device and the UICC hosting device. Candidate applications to use this key establishment mechanism include but are not restricted to Personal Network Management (see TS 22.259). The scope of this specification includes an architecture overview and the detailed procedure how to establish the shared key between the UICC Hosting Device and the Remote Device. This is different from the Technical Specification TS 33.110 that describes an architecture overview and the detailed procedure how to establish the shared key between the UICC itself and the terminal hosting the UICC. The use cases utilizing the mechanisms described in this specification are seen to be different to the use cases where “Key establishment between a UICC and a terminal”, IETF RFC 4279 is utilized. The solution described in this document is built on the existing infrastructure defined in “GBA”, TS 33.220.

##### 5.1.2.12.11.26	TS 33.303

Proximity-based Services (ProSe); Security aspects

This document specifies the security aspects of the Proximity Services (ProSe) features in EPS.

Based on the common security procedures (clause 5) for

- interfaces between network entities (using NDS),

- configuration of ProSe-enabled UEs, and

- data transfer between the ProSe Function and a ProSe enabled UE (PC3 interface)

security for the following  ProSe features is covered:

- Open ProSe Direct Discovery in network coverage (clause 6.1);

- One-to-many ProSe direct communication for ProSe-enabled Public Safety UEs (clause 6.2);

- EPC-level Discovery of ProSe-enabled UEs (clause 6.3);

- EPC support for WLAN Direct Discovery and Communication (clause 6.4).

##### 5.1.2.12.11.27	TS 33.310

Network Domain Security (NDS); Authentication Framework (AF)

The scope of this Technical Specification is limited to authentication of network elements, which are using NDS/IP or TLS. In the case of NDS/IP this specification includes both the authentication of Security Gateways (SEG) at the corresponding Za-interfaces and the authentication between NEs and between NEs and SEGs at the Zb-interface. Authentication of end entities (i.e. NEs and SEGs) in the intra-operator domain is considered an internal issue for operators. This is quite much in line with 3GPP TS 33.210 which states that only Za is mandatory and that the security domain operator can decide if the Zb-interface is deployed or not, as the Zb-interface is optional for implementation. Validity of certificates may be restricted to the operator’s domain in case of Zb interface or in case of Za-interface between two security domains of the same operator. In the case of TLS this Specification concentrates on authentication of TLS entities across inter-operator links. For example, TLS is specified for inter-operator communications between IMS and non-IMS networks TS 33.203 and on the Zn’ interface in GBA TS 33.220. Authentication of TLS entities across intra-operator links is considered an internal issue for operators. However, NDS/AF can easily be adapted to the intra-operator use case since it is just a simplification of the inter-operator case when all TLS NEs and the PKI infrastructure belong to the same operator. Validity of certificates may be restricted to the operator’s domain. An Annex contains information on the manual handling of TLS certificates in case automatic enrolment and revocation according to NDS/AF for TLS is not implemented.

##### 5.1.2.12.11.28	TS 33.320

Security of Home Node B (HNB)/Home evolved Node B (HeNB)

This document specifies the security architecture for the H(e)NB subsystem. This includes security requirements on Home Node Bs, Home eNode Bs, and other H(e)NB-associated network nodes (e.g. SeGW and H(e)MS), as well as the procedures and features which are provided to meet those requirements.

##### 5.1.2.12.11.29	TS 33.328

IP Multimedia Subsystem (IMS) media plane security

This document presents IMS media plane security for RTP based media which is designed to meet the following three main objectives:

–   to provide security for media usable across all access networks;

–   to provide an end-to-end (e2e) media security solution to satisfy major user categories;

–   to provide end-to-end (e2e) media security for important user groups like enterprises, National Security and Public Safety (NSPS) organizations and different government authorities who may have weaker trust in the inherent IMS security and/or may desire to provide their own key management service.

The media plane security in this release of the TS is based on the well-established protocol SRTP. Key management solutions for SRTP are defined in this specification.

##### 5.1.2.12.11.30	TS 33.401

3GPP System Architecture Evolution (SAE); Security architecture

This document specifies the security architecture, i.e. the security features and the security mechanisms for the Evolved Packet System and the Evolved Packet Core, and the security procedures performed within the evolved Packet System (EPS) including the Evolved Packet Core EPC) and the Evolved UTRAN (E-UTRAN).

##### 5.1.2.12.11.31	TR 33.905

Recommendations for Trusted Open Platforms

This technical report investigates relevant trust standards and technologies, both existing as well as the ones that are work-in-progress. It develops the recommendations for trusted open platforms for delivery of new applications and services to open platforms.

##### 5.1.2.12.11.32	TR 33.919

3G Security; Generic Authentication Architecture (GAA); System description

This 3GPP Technical Report gives an overview of the different mechanisms that mobile applications can rely upon for authentication between server and client (i.e. the UE). Additionally it provides guidelines related to the use of GAA and to the choice of authentication mechanism in a given situation and for a given application. To this end the TR puts the different 3GPP GAA related specifications into perspective. It clarifies the logic for having the technical specifications and technical reports, sketches their content and explains the inter-relation between these 3GPP TSs and TRs and their relation with this TR. The heart of GAA consists out of the Generic Bootstrapping Architecture (GBA): The GBA core specifications consist out of TS 33.220, TS 24.109 and TS 29.109. GBA in turn is then used by many other TSs and TRs to enable specific usages e.g. HTTPS, subscriber certificates.

##### 5.1.2.12.11.33	TR 33.924

Identity management and 3GPP security interworking; Identity management and Generic Authentication Architecture (GAA) interworking

The objective is to extend identity management as outlined in TS 33.220, TS 33.222, TS 29.109 and TR 33.980 with the latest developments on identity management outside of the 3GPP sphere. This will allow a better integration and usage of identity management for services in 3GPP and seamless integration with existing services that are not standardized in 3GPP. This report outlines the interworking of GBA and OpenID.

##### 5.1.2.12.11.34	TR 33.937

Study of mechanisms for Protection against Unsolicited Communication for IMS (PUCI)

The scope of this report is to highlight alternative solutions that could be used to protect mobile subscribers from receiving unsolicited communication over IMS and to analyze these solutions in respect of their requirements and impacts on standardized interfaces. This activity took into account the study done in ETSI TISPAN TR 187 009 on “Feasibility study of prevention of unsolicited communications in the NGN”.

##### 5.1.2.12.11.35	TR 33.980

Liberty Alliance and 3GPP security interworking; Interworking of Liberty Alliance Identity Federation Framework (ID-FF), Identity Web Services Framework (ID-WSF) and Generic Authentication Architecture (GAA)

This document provides guidelines on the interworking of the Generic Authentication Architecture (GAA) and the Liberty Alliance architecture. The document studies the details of possible interworking methods between the Security Assertion Markup Language v2.0, SAML v2.0 (or alternatively the Liberty Alliance Identity Federation Framework, ID-FF), the Identity Web Services Framework (ID-WSF), the Security Assertion Markup Language (SAML) and a component of GAA called the Generic Bootstrapping Architecture (GBA). This document only applies if Liberty Alliance and GBA or SAML v2.0 and GBA are used in combination.

#### 5.1.2.12.12	Security algorithms

##### 5.1.2.12.12.1	TS 35.201

3G Security; Specification of the 3GPP confidentiality and integrity algorithms; Document 1: f8 and f9 specification

This specification gives a detailed specification of the 3GPP confidentiality algorithm f8, and the 3GPP integrity algorithm f9.

##### 5.1.2.12.12.2	TS 35.202

3G Security; Specification of the 3GPP confidentiality and integrity algorithms; Document 2: Kasumi specification

This specification gives a detailed specification of the 3GPP Algorithm KASUMI. KASUMI is a block cipher that forms the heart of the 3GPP confidentiality algorithm f8, and the 3GPP integrity algorithm f9.

##### 5.1.2.12.12.3	TS 35.203

3G Security; Specification of the 3GPP confidentiality and integrity algorithms; Document 3: Implementors’ test data

This specification gives detailed test data for implementors of the algorithm set. It provides visibility of the internal state of the algorithm to aid in the realization of the algorithms.

##### 5.1.2.12.12.4	TS 35.204

3G Security; Specification of the 3GPP confidentiality and integrity algorithms; Document 4: Design conformance test data

This specification gives black-box test data for the algorithm set. The test data has been selected to give a high degree of confidence that the implementation is correct. However, no claim is made that conformance with this test data guarantees a correct implementation.

##### 5.1.2.12.12.5	TS 35.205

3G Security; Specification of the MILENAGE algorithm set: An example algorithm set for the 3GPP authentication and key generation functions f1, f1*, f2, f3, f4, f5 and f5*; Document 1: General

This report is a description of the work undertaken by an ETSI SAGE Task Force on the design of the Milenage Algorithm Set: an example set of 3GPP Authentication and Key Generation Functions.

The 3GPP Authentication and Key Generation Functions are not standardized. An example set of these algorithms has been produced on request from 3GPP with the intent that it shall be offered to the UMTS operators, to utilize instead of developing their own. An ETSI SAGE Task Force has carried out this work.

The requirement specification from 3GPP SA3 stated that operator personalization of the example set must be possible and that the basic kernel must be possible to replace.

The example set is based on the block cipher Rijndael, which at the time was one of the AES candidates and the specification describes how the 7 algorithms used in 3GPP authentication and key generation are scheduled around this basic kernel. The specification and associated test data for the example algorithm set is documented in three documents:

–   A formal specification of both the modes and the example kernel;

–   A detailed test data document, covering modes and the example kernel;

–   A “black box” test data document. A detailed summary of the evaluation is provided in a public evaluation report, ETSI SAGE 3GPP AF TF: “Report on the design and evaluation of 3GPP Authentication and Key Generation Functions”. This report gives an overview of the overall work by the task force.

##### 5.1.2.12.12.6	TS 35.206

3G Security; Specification of the MILENAGE algorithm set: An example algorithm set for the 3GPP authentication and key generation functions f1, f1*, f2, f3, f4, f5 and f5*; Document 2: Algorithm specification

This document has been prepared by the 3GPP Task Force, and contains an example set of algorithms which may be used as the authentication and key generation functions f1, f1*, f2, f3, f4, f5 and f5*. (It is not mandatory that the particular algorithms specified in this document are used – all seven functions are operator-specifiable rather than being fully standardized.)

##### 5.1.2.12.12.7	TS 35.207

3G Security; Specification of the MILENAGE algorithm set: An example algorithm set for the 3GPP authentication and key generation functions f1, f1*, f2, f3, f4, f5 and f5*; Document 3: Implementors’ test data

This document has been prepared by the 3GPP Task Force, and contains an example set of algorithms which may be used as the authentication and key generation functions f1, f1*, f2, f3, f4, f5 and f5*. (It is not mandatory that the particular algorithms specified in this document are used – all seven functions are operator-specifiable rather than being fully standardized.)

##### 5.1.2.12.12.8	TS 35.208

3G Security; Specification of the MILENAGE algorithm set: An example algorithm set for the 3GPP authentication and key generation functions f1, f1*, f2, f3, f4, f5 and f5*; Document 4: Design conformance test data

This document has been prepared by the 3GPP Task Force, and contains an example set of algorithms which may be used as the authentication and key generation functions f1, f1*, f2, f3, f4, f5 and f5*. (It is not mandatory that the particular algorithms specified in this document are used – all seven functions are operator-specifiable rather than being fully standardized.)

##### 5.1.2.12.12.9	TS 35.215

Specification of the 3GPP Confidentiality and Integrity Algorithms UEA2 & UIA2; Document 1: UEA2 and UIA2 specifications

This document specifies the 3GPP confidentiality and integrity algorithms known as UEA2 and UIA2.

##### 5.1.2.12.12.10	TS 35.216

Specification of the 3GPP Confidentiality and Integrity Algorithms UEA2 & UIA2; Document 2: SNOW 3G specification

This document specifies the 3GPP confidentiality and integrity algorithms known as UEA2 and UIA2.

##### 5.1.2.12.12.11	TS 35.217

Specification of the 3GPP Confidentiality and Integrity Algorithms UEA2 & UIA2; Document 3: Implementors’ test data

This document specifies the 3GPP confidentiality and integrity algorithms known as UEA2 and UIA2.

##### 5.1.2.12.12.12	TS 35.218

Specification of the 3GPP Confidentiality and Integrity Algorithms UEA2 & UIA2; Document 4: Design conformance test data

This document specifies the 3GPP confidentiality and integrity algorithms known as UEA2 and UIA2.

##### 5.1.2.12.12.13	TS 35.231

Specification of the TUAK algorithm set: A second example algorithm set for the 3GPP authentication and key generation functions f1, f1*, f2, f3, f4, f5 and f5*; Document 1: Algorithm specification

This document and the other Technical Specifications in the series, TS 35.232 and 35.233 contain an example set of algorithms which could be used as the authentication and key generation functions f1, f1*, f2, f3, f4, f5 and f5* for 3GPP systems. All seven functions are operator-specifiable rather than being fully standardised and other algorithms could be envisaged.

##### 5.1.2.12.12.14	TS 35.232

Specification of the TUAK algorithm set: A second example algorithm set for the 3GPP authentication and key generation functions f1, f1*, f2, f3, f4, f5 and f5*; Document 2: Implementers’ test data

This document and the other Technical Specifications in the series, TS 35.231 and TS 35.233, contain an example set of algorithms which could be used as the authentication and key generation functions f1, f1*, f2, f3, f4, f5 and f5* for 3GPP systems. In particular, this document defines the test data:

- for the Keccak permutation used within Tuak,

- for the authentication algorithms f1 and f1*,

- for the algorithms f2, f3, f4, f5 and f5*.

##### 5.1.2.12.12.15	TS 35.233

Specification of the TUAK algorithm set: A second example algorithm set for the 3GPP authentication and key generation functions f1, f1*, f2, f3, f4, f5 and f5*; Document 3: Design conformance test data

This document and the other Technical Specifications in the series, TS 35.231 and TS 35.232, contain an example set of algorithms which could be used as the authentication and key generation functions f1, f1*, f2, f3, f4, f5 and f5* for 3GPP systems. This document provides sets of input/output test data for ‘black box’ testing of physical realizations of all algorithms, and in particular:

- Test data for the Keccak permutation used within Tuak.

- Test data for the MILENAGE authentication and key generation algorithms f1, f1*, f2, f3, f4, f5 and f5*.

##### 5.1.2.12.12.16	TR 35.909

3G Security; Specification of the MILENAGE algorithm set: an example algorithm set for the 3GPP authentication and key generation functions f1, f1*, f2, f3, f4, f5 and f5*; Document 5: Summary and results of design and evaluation

This report contains a detailed summary of the work performed during the design and evaluation of the 3GPP Authentication Functions denoted as the MILENAGE algorithm set. It contains all results and findings from this work and should be read as a supplement to the specifications of the algorithms in ETSI/SAGE Specification of the MILENAGE Algorithm Set: an Example Algorithm Set for the 3GPP Authentication and Key generation Functions, f1, f1*, f2, f3, f4, f5 and f5*; Document 1: Algorithm Specification. and the general project report on the Design and Evaluation of the 3GPP Authentication and Key generation Functions.

##### 5.1.2.12.12.17	TR 35.919

Specification of the 3GPP Confidentiality and Integrity Algorithms UEA2 & UIA2; Document 5: Design and evaluation report

This document specifies the 3GPP confidentiality and integrity algorithms known as UEA2 and UIA2.

##### 5.1.2.12.12.18	TR 35.934

Specification of the TUAK algorithm set: A second example algorithm set for the 3GPP authentication and key generation functions f1, f1*, f2, f3, f4, f5 and f5*; Document 4: Report on the design and evaluation

This document describes the design rationale, and presents evaluation results, on the Tuak algorithm set – a second example set of algorithms which may be used as the authentication and key generation functions f1, f1*, f2, f3, f4, f5 and f5*, e.g. as an alternative to MILENAGE.

##### 5.1.2.12.12.19	TR 35.935

Specification of the TUAK algorithm set: A second example algorithm set for the 3GPP authentication and key generation functions f1, f1*, f2, f3, f4, f5 and f5*; Document 5: Performance evaluation

This document provides a reference to an independent performance evaluation report of the Tuak algorithm set carried out by the ISG Smart Card Centre of the Royal Holloway University of London.

##### 5.1.2.12.12.20	TR 35.936

Specification of the TUAK algorithm set: A second example algorithm set for the 3GPP authentication and key generation functions f1, f1*, f2 ,f3, f4, f5 and f5*; Document 6: Security assessment

This document provides a reference to an independent security assessment of the Tuak algorithm set carried out by the University of Waterloo, Canada.

#### 5.1.2.13	Vocabulary

##### 5.1.2.13.1	TR 21.905

Vocabulary for 3GPP Specifications

Document 21.905 is a collection of terms, definitions and abbreviations related to the baseline documents defining the objectives and systems framework. This document provides a tool for further work on the technical documentation and facilitates their understanding.

____________________