| # Liaison Statement |  |
| --- | --- |
| Title: | LS to 3GPP RAN WG4 on Release 15 of the IMT harmonised standard |
| Date: | 2020-09-11 |
| From (source): | MSG TFES (Mr. Dominique Everaere, dominique.everaere@ericsson.com) |
| Contact(s): | Mr. Guillermo Vietti (Guillermo.Vietti@etsi.org;MSGTFESsupport@etsi.org) |
| To: | 3GPP RAN WG4 (Mr. Steven Chen, steven.x.chen@apple.com ) |
| Copy to: | TC ERM (Mr. Holger Butscheidt, holger.butscheidt@bnetza.de) |
|  | Marcello Pagnozzi (Marcello.Pagnozzi@etsi.org); ERMSupport@etsi.org) |
|  | 3GPP TSG RAN (Mr. Balazs Bertenyi, balazs.bertenyi@nokia.com, 3GPPLiaison@etsi.org) |
| Response to: (if applicable) | - |

1. Overall description:

ETSI MSG TFES is in the process of completing Release 15 of the European Harmonised Standard for IMT Cellular networks (EN 301 908), corresponding to 3GPP Release 15. In this process, requirements are reviewed, and some requirements may be adjusted to align with the latest European regulation and spectrum use. MSG TFES would like to make RAN WG4 aware of the following two updates made to EN 301 908:

- Band n257: Support for operation in Band n257 is introduced in the European harmonised standard, in order to facilitate roaming of Band n257 UEs in Europe. Operation in Europe will be limited to the frequency range 26.5 – 27.5 GHz.

- Additional limit for Band 1 operation: One of the Base Station transmitter requirements in EN 301 908 for operation in Band 1 (2110 – 2170 MHz downlink) is an additional limit being part of the unwanted emissions, providing a “slope” in the region 5-10 MHz outside the operating band. It has in previous releases been documented as follows in the different EN 301 908 parts:

  - Part 3 (UTRA BS):	4.2.4.2.3 Co-existence with services in adjacent frequency bands

  - Part 14 (LTE BS):	4.2.2.2.7 Limits for protection of adjacent band services

  - Part 18 (MSR BS): 	4.2.2.2.5 Limits for co-existence with services in adjacent frequency bands

ETSI MSG TFES has concluded (TFES(20)065014) that the additional limit referred to above is not part of the ECC or EC regulation for operation in the bands 1920-1980 MHz and 2110-2170 MHz. This view has been confirmed through an LS exchange with ECC PT1 (TFES(20)000011, ECC PT1 conclusion here). The limit is therefore removed from the European Harmonized Standards for UTRA BS, LTE BS and MSR BS.

2. Actions:

ETSI MSG TFES invites TSG RAN WG4 to take the above information into account when including European requirements in the 3GPP specifications.

3. Date of next meetings of the originator:

30 November 2020- 2 December 2020			TFES#67	   on-line meeting