# 3GPP TSG SA WG3 (Security) Meeting #94 ad-hoc	S3-190687

# Kista (Sweden), 11 March – 15 March 2019

# Source:	Huawei, HiSilicon

# Title:	Conclusion for KI #2

# Document for:	Approval

Agenda Item:	5.13

1	Decision/action requested

This contribution proposes a pCR for the conclusion for KI #2 of TR 33.825.

2	References

[1]	3GPP TR 23.725 Study on enhancement of Ultra-Reliable Low-Latency Communcation (URLLC) v0.4.0

3	Rationale

The contribution proposes to add the conclusion for KI #2.

For KI #2, the potential security requirements as follows are proposed:

- Confidentiality protection over the air shall be supported for UP data via redundant multiple transmission paths.

- Integrity protection over the air shall be supported for UP data via redundant multiple transmission paths.

The solution 5 is related to the confidential protection and integrity protection keys, meanwhile, solution 3 and solution 1 propose to how to cipher and integrity protection the UP data for URLLC services by using these security keys. In this case, solution 5, solution 1 and solution 3 shall be considered together to fulfil the security requirement of KI #2.

4	Detailed proposal

It is proposed to approve the following changes for inclusion in TR 33.825.

***	BEGIN CHANGES	***

7	Conclusions

Editor’s Note: This clause contains the agreed conclusions.

7.2	Key Issue #2: Support of security for high reliability by redundant data transmission in user plane

For Key Issue #2, it is recommended that:

- Solution 5 is used as the basis for normative work to address how to generate the confidentiality protection and Integrity protection keys for UP data via redundant multiple transmission paths.

- Solution 1 and solution 3 are used as the basis for normative work to address how to make sure the multiple PDU sessions for redundant data transmission have the same UP security policy. The UP security policy dictates when the confidentialiy protection and/or Integrity protection is activated.

***	END OF CHANGES	***