# 3GPP TSG SA WG3 (Security) Meeting #94 ad-hoc	S3-190924

# 11 – 15 March 2019, Kista (Sweden)	revision of S3-190702

# Source:	Huawei, Hisilicon

# Title:	Key issue on Key freshness in AKMA

# Document for:	Approval

Agenda Item:	5.5

1	Decision/action requested

It is requested to approve the proposal into TS 33.835.

2	References

[1]	3GPP TR 33.835 Study on authentication and key management for applications based on 3GPP credential in 5G

3	Rationale

A weakness of AKMA as a key agreement scheme is that there is no guarantee of freshness of the application key KAF without forcing a re-run of AKA. That is if an Application Function that requests a key from the Anchor Function, that key might have already been used. In general key freshness is a desirable property of any method used to establish keys and should be included in AKMA.

4	Detailed proposal

*************** Start of Change 1 ****************

5.X	Key Issue #X: Application Key freshness of AKMA

5.X.1 Issue details

AKMA as a key agreement scheme should guarantee of freshness of the application key KAF. That is if an AKMA AF that requests a key from the Anchor Function, that key might have already been used. In general key freshness is a desirable property of any method used to establish keys and should be included in AKMA.

5.X.2 Security Threats

If a KAF is used without freshness, then a weakness between UE and AKMA AF may allow an attacker to pretend to be a particular AKMA AF and obtain KAF. The attacker can masquerade as the UE towards the real AKMA AF.

### 5.X.3	Potential security requirements

It shall be possible to ensure the freshness of keys used between the UE and the AKMA application function.

*************** End of Change 1 ****************