3GPP TSG-CT Meeting #98-e	CP-223021

# E-Meeting, 12th – 14th December 2022

(revision of CP-yyxxxx)

# Source:	CT4

# Title:	New WID on 5GS support of NR RedCap UE with long eDRX for RRC_INACTIVE State

# Document for:	Agreement

# Agenda Item:	18.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	5GS support of NR RedCap UE with long eDRX for RRC_INACTIVE State

## Acronym: NR_REDCAP_Ph2

## Unique identifier: 	980001

Potential target Release: Rel-18

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  | X |  | X |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

|  | Feature |
| --- | --- |
| X | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| NR_REDCAP_Ph2 | SA2 | 970013 | 5GS support of NR RedCap UE with long eDRX for RRC_INACTIVE State |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 970080 | NR_redcap_enh | RAN Work Item on RedCap enhancements |

## 3	Justification

This work item aims at specifying stage 3 system enhancements required for 5G system to support data and signalling handling enhancement when NR RedCap UE applies long eDRX (i.e. >10.24s) value in 5GMM-CONNECTED mode with RRC inactive indication.

## 4	Objective

The objective is to specify enhancements to 5GS as per stage 2 requirements for introducing support of long eDRX (i.e., >10.24s) for UEs in 5GMM-CONNECTED mode with RRC inactive indication.

The following impacts on 3GPP CT working groups are identified.

CT1

-	Removal of the limitation on the applicability of eDRX values >10.24s for UEs in 5GMM-CONNECTED mode with RRC inactive indication.

CT4

-	Enhancements for MT data/signalling handling during UE unreachable time due to long eDRX for UE in CM-CONNECTED with RRC_INACTIVE state which includes:

-	Impact on Nsmf_PDUSession service, e.g.  using SmContext Update request to inform the SMF to stop user plane DL data transmissions and apply CN based MT handling;

-	Impact on Namf_MT service, e.g. to document relevant stage 2 procedures.

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| N/A | N/A | N/A | N/A | N/A | N/A |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 24.008 | Updates to Extended DRX parameters | CT#102 (Dec. 2023) | CT1 |
| 29.502 | Impact on Nsmf_PDUSession service, e.g. to use SmContext Update request message to inform the SMF to stop user plane DL data transmissions. | CT#102 (Dec. 2023) | CT4 |
| 29.518 | Possible impact on Namf_MT service, e.g. to document relevant stage 2 procedure. | CT#102 (Dec. 2023) | CT4 |

## 6	Work item Rapporteur(s)

Frank Yong Yang, Ericsson, frank.yong.yang@ericsson.com

## 7	Work item leadership

CT4

## 8	Aspects that involve other WGs

None

## 9	Supporting Individual Members

Supporting IM name

Ericsson

Apple

Nokia

Nokia Shanghai Bell

Huawei

Cisco