3GPP TSG CT#99 	CP-230025

Rotterdam, Netherlands, 20th - 21st March 2023	(revision of xx-yyxxxx)

# Source:	TSG CT WG4

# Title:	Enhancements on Service-based support for SMS in 5GC

# Document for:	Approval

# Agenda Item:	18.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Enhancements on Service-based support for SMS in 5GC

Acronym:	eSMS_SBI

Unique identifier:	990003

Potential target Release:	Rel-18

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  |  |
| Don't know |  |  |  |  | X |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
| X | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A |  |  |  |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 890001 | Service-based support for SMS in 5GC | Rel-17 WID of Service-based support for SMS in 5GC |

# 3	Justification

In the Rel-17, 5G SMS_SBI (i.e. Service-based SMS in 5GC) framework, procedure and interfaces are defined. Service-based SMS framework and procedures are depicted in TS 23.540, and new NF services are depicted in TS 29.577 / TS 29.578 / TS 29.579. At the end of Rel-17 research, some technical problems were proposed and planned to be solved in Rel-18, mainly includes:

- Incompatibility issues between SBI-based SMS and legacy SMS, e.g. during SM MT delivery, when SMS-GMSC makes use of MAP or Diameter to retrieve RoutingInfoForSM from legacy UDM/HSS (i.e. UDM/HSS do not support SBI-based SMS), the response message is still lack of SBI-support information of SMSF/IP-SM-GW/SMS Router, which may cause incompatibility problems.

- Protocol selection for SMS-GMSC towards UDM/HSS during MT SM delivery. In Rel-17, SBI-based method is adopted by default, and no protocol selection is considered for SMS-GMSC RoutingInfoForSM towards UDM/HSS. It may cause problems when SMS-GMSC use SBI to get RoutingInfoForSM from legacy UDM/HSS.

- Some stage 2 and stage 3 misalignment of SMS TSs, e.g. Table 5.3.2-1 of TS 23.540 defines Application errors for MO SM delivery response, HTTP status code of it only includes 403 and 504, but the stage 3 TS 29.579 includes 307/308/400/403/504.

The e-mail discussion in Rel-17 SMS_SBI has exposed these important issues. It is proposed to set up a new WID in Rel-18 to solve the legacy problems.

# 4	Objective

The main objective of this work item proposal is to solve legacy problems of SMS_SBI (Rel-17), i.e. incompatibility problems and protocol selection problems, and make SBI-based SMS NFs and legacy SMS NFs work in harmony. This translates to the following:

-	improve the mechanism of protocol selection of SMS, especially for MT SM Routing Info Retrieval, it is proposed to consider different cases and promote compatibility between SBI-based NFs and non-SBI-based NFs.

-	Alignment of stage 2 and stage 3 SMS TSs.

# 5	Expected Output and Time scale

It is proposed to proceed in two steps:

-	First, normative stage 2 work to specify the function/procedure enhancement to capture the requirements described in Objective.

-	Then, normative stage 3 work to fulfill the interface/protocol/service enhancement related to the work in the first step.

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| N/A. |  |  |  |  |  |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.540 | Enhancement on NF compatibility (SBI NFs and legacy NFs) and protocol selection; stage 2 and stage 3 alignment of SMS. | CT#101 (Sep. 2023) | This TS covers Stage 2 |
| 29.338 | Potential enhancement on Diameter based protocols for SMS NFs compatibility and protocol selection. | CT#102 (Dec. 2023) | This TS covers Stage 3 |
| 29.329 | Potential enhancement on Sh interface (based on Diameter protocol) for SMS NFs compatibility and protocol selection. | CT#102 (Dec. 2023) | This TS covers Stage 3 |
| 29.230 | Potential enhancement on Diameter applications 3GPP specific codes and identifiers for SMS NFs compatibility and protocol selection. | CT#102 (Dec. 2023) | This TS covers Stage 3 |

# 6	Work item Rapporteur(s)

Liu, Liu, China Telecom, liuliu66@chinatelecom.cn

# 7	Work item leadership

CT4

# 8	Aspects that involve other WGs

N/A.

# 9	Supporting Individual Members

Supporting IM name

China Telecom

CATT

ZTE

vivo

Nokia

Nokia Shanghai Bell