3GPP TSG RAN Meeting#84	R-191588

Newport Beach, USA, June 3-6, 2019	(revision of RP-190175)

# Source:	KDDI Corporation

# Title:	Revised WID: LTE/NR spectrum sharing in Band 41/n90

# Document for:	Approval

Agenda Item:	9.5.13

3GPP™ Work Item Description

For guidance, see 3GPP Working Procedures, article 39; and 3GPP TR 21.900.
Comprehensive instructions can be found at http://www.3gpp.org/Work-Items

Title:	LTE/NR spectrum sharing in Band 41/n90

Acronym: NR_n41_LTE_41_coex

Unique identifier:

NOTE:	For new WIs/SIs leave the Unique identifier empty but you may make a proposal for an Acronym.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part | x |
| --- | --- |
| This WID includes a Performance part | x |

or:

| This WID includes a Testing part |  |  |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access |  |
|  | Core Network |  |
|  | Services |  |

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X |  |  |
| No | X |  |  | X |  |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

|  | Feature |
| --- | --- |
| X | Building Block |
|  | Work Task |
|  | Study Item |

NOTE:	Normally, Core/Perf./Testing parts in RAN WIDs are Building Blocks. Only if they are under an SA or CT umbrella, we define them as work tasks. If you are in doubt, please contact MCC.

2.2	Parent and child Work Items

| Parent and child Work Items |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 820080 | LTE/NR spectrum sharing in Band 41/n41 | Parent WID |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one WID. Therefore the table above should just include the feature WI Unique ID and title and Nature of relationship is "parent WID".

2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

NOTE:	Classical examples: List a preceding SI or a preceding WI (e.g. if you further enhance a topic). Also related or dependent WIs in other TSGs should be indicated.

3	Justification

LTE Band 41 is a widely deployed band operated in United States, China, Japan and etc. It is also a NR candidate band which has been specified in Rel-15. To consider the re-farming of band 41, dynamic spectrum sharing in both DL and UL of LTE and NR is a feasible solution especially for networks with heavy LTE traffic load.

Spectrum allocation of band 41 in Japan is shown in Figure 3-1. Although operator B has 40MHz bandwidth (2605 – 2645 MHz) operated in LTE, it is difficult to re-farm the whole LTE component carrier (e.g. 2625-2645MHz) to NR because of heavy traffic load of current LTE service. Spectrum sharing of RB level is necessary for the re-farming plan of operator B.

Operator B

Operator A

Figure 3-1: Spectrum allocation in Japan around 2.5GHz

For RB level spectrum sharing, frequency alignment between LTE and NR is necessary to avoid inter-subcarrier interference. As mentioned in RP-180155, the following two features are needed for band 41 spectrum sharing.

- 100kHz channel raster support

As shown in the following table, 100 kHz channel raster is not supported for n41 in Rel-15.

Referred from TS38.101-1 Table 5.4.2.3-1: Applicable NR-ARFCN per operating band

| NR Operating Band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| ~ omitted ~ |  |  |  |
| n41 | 15 | 499200 – <3> – 537999 | 499200 – <3> – 537999 |
|  | 30 | 499200 – <6> – 537996 | 499200 – <6> – 537996 |
| ~ omitted ~ |  |  |  |

As shown in Figure 3-2, SCS based NR raster cannot align with LTE channel raster. 100 kHz raster of NR band n41 is necessary for spectrum sharing.

Figure 3-2: LTE/NR frequency alignment for band 41

- Sync raster specification correction

As shown in NOTE 1 of the following table, n41 sync raster has a step size of 1 since it has SCS spaced channel raster. If 100kHz channel raster is introduced for n41, sync raster specification (i.e. Range of GSCN and step size in Table 5.4.3.3-1 of TS38.101-1) need to be corrected correspondingly.

Referred from TS38.101-1 Table 5.4.3.1-1: GSCN parameters for the global frequency raster

| Frequency range | SS Block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 0 – 3000 MHz | N * 1200kHz + M * 50 kHz, N=1:2499, M ϵ {1,3,5} (Note 1) | 3N + (M-3)/2 | 2 – 7498 |
| 3000-24250 MHz | 3000 MHz + N * 1.44 MHz N = 0:14756 | 7499 + N | 7499 – 22255 |
| NOTE 1: The default value for operating bands with SCS spaced channel raster is M=3. |  |  |  |

Referred from TS38.101-1 Table 5.4.3.3-1: Applicable SS raster entries per operating band

| NR Operating Band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| ~ omitted ~ |  |  |  |
| n41 | 15kHz | Case A | 6246 – <3> – 6717 |
|  | 30 kHz | Case C | 6252 – <3> – 6714 |
| ~ omitted ~ |  |  |  |

- UL 7.5kHz frequency shift

This feature is for UL sub-carrier alignment of UL spectrum sharing which is described in detail in R4-1801570.

Given above justifications, spectrum sharing for n41 would be necessary and the following requirements should be supported for n41 in Rel-16.

- 100kHz channel raster support

- Sync raster specification correction

- UL 7.5kHz frequency shift

4	Objective

4.1	Objective of SI or Core part WI or Testing part WI

The work item aims to specify spectrum sharing requirements in frequency range of 2496 – 2690 MHz.  In order to minimize impacts on existing requirements, 2 possible solutions are identified.  RAN-WG4 discusses which option to be adopted.

Solution#1: Create new NR band with requirements to achieve spectrum sharing (LTE and NR) in 2496 – 2690 MHz.

Solution#2: Addition of new requirements addition into n41 as below;

- Introduce 100kHz channel raster support [RAN4]

- Sync raster specification correction [RAN4]

- Introduce UL 7.5kHz frequency shift [RAN4]

* above three requirements are at maximum for RAN4.  If unnecessary, some of requirements may not be added into n41.

RAN-WG4 discussed and agreed that Solution#1 is to be adopted.  It was also agreed that new NR band number of “n90” is allocated for spectrum sharing requirements in frequency range of 2496 – 2690 MHz. All n41 requirements including band combinations (i.e. CA, DC, SUL) are applied to this new NR band n90. UL 7.5kHz shift and 100kHz channel raster are mandatorily supported in this new band.

This work item aims to specify this new NR band “n90”.

4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed leave the field empty otherwise enter a number in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each of them in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Proposed Spec no. or series | Type (see note 1) | Rapporteur(s) (see note 2) | For info at TSG# | For approval at TSG# | Remarks |

Note 1:	Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
"Internal TR" is intended for 3GPP internal use only whereas "External TR" may be transposed by OPs.

NOTE:	If this is a RAN WID including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.101-1 | NR; User Equipment (UE) radio transmission and reception | RAN#84 | Core Part |
| 38.104 | NR; Base Station (BS) radio transmission and reception | RAN#84 | Core Part |
| 38.101-3 | NR; User Equipment (UE) radio transmission and reception: Range 1 and Range 2 Interworking operation with other radios | RAN#84 | Core Part |
| 38.307 | NR; Requirements on User Equipments (UEs) supporting a release-independent frequency band | RAN#84 | Performance Part |
| 38.331 | NR; Radio Resource Control (RRC); Protocol specification | RAN#84 | Core Part |

NOTE:	If this is a RAN WID including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

6	Work item Rapporteur(s)

Xiao Shao, KDDI Corporation, ko-shou@kddi.com

7	Work item leadership

RAN WG4

8	Aspects that involve other WGs

{Free text. If applicable, specify the other WG(s) and their task, e.g.: "SA1 for the Service Aspects", "SA5 for the Charging Aspects", "SA3 for the Security aspects", etc. If not applicable, indicate "None" or "None identified yet".}

NOTE:	For RAN WIDs: Section 8 applies only toWGs outside of TSG RAN because RAN WG aspects have to be covered in section 4.

9	Supporting Individual Members

{At least 4 supporting Individual Members are needed. There is an expectation that these companies will provide resources to progress the work. Note that having 4 supporting companies is a necessary but not sufficient condition: the usual TSG approval process by consensus is needed for the WID approval.}

Supporting IM name

KDDI

Anristu

Sumitomo

Samsung

Kyocera

Mitsubishi Electric

Sprint