3GPP TSG RAN Meeting #97-e	 RP-222656

Electronic Meeting, Sept 12-16, 2022	                                                    (previous version RP-213540)

# Source:	Qualcomm

# Title:	Revised WID:  Extending current NR operation to 71 GHz

# Document for:	Approval

Agenda Item:	9.5.1.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: Extending current NR operation to 71GHz

## Acronym: NR_ext_to_71GHz

## Unique identifier: 860041

NOTE:	For new WIs/SIs leave the Unique identifier empty and make a proposal for an Acronym.

For a revised WI/SI: Take Unique identifier and acronym as shown in 3GPP workplan.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part | X |

or:

| This WID includes a Testing part |  |  |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access |  |
|  | Core Network |  |
|  | Services |  |

Potential target Release: Rel-17

Note that this field above indicates the proposed Release at the time of submission of the WID to TSG approval. It can later be changed without a need to revise the WID. The updated target Release is indicated in the Work Plan. In case of contradiction with the target dates of clause 5, clause 5 determines the target release.

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X |  |  |
| No | X |  |  | X |  |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

NOTE:	Normally, Core/Perf./Testing parts in RAN WIDs are Building Blocks. Only if they are under an SA or CT umbrella, they are defined as work tasks. If you are in doubt, please contact MCC.

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| FS_NR_beyond_52GHz | RAN | 800090 | Study on NR beyond 52.6 GHz |
| FS_NR_52_to_71GHz | RAN1 | 860037 | Study on supporting NR from 52.6 GHz to 71GHz |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one 	WID. Therefore the table above should just include the feature WI data (In case the feature covers Core and 	Perf. part, please list under Working Group the leading WG of the Core part).

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

NOTE:	Also related or dependent WIs/SIs in other TSGs should be indicated.

Dependency on non-3GPP (draft) specification:

## 3	Justification

NR Rel-15 defined two frequency ranges for operation:

- FR1 spanning from 410MHz to 7.125GHz

- FR2 spanning from 24.25GHz to 52.6GHz

RAN carried out a Rel-16 study on NR beyond 52.6GHz (FS_NR_beyond_52GHz) with corresponding TR in 38.807. From this study, it became apparent the global availability of bands in the 52.6GHz to 71GHz range, most notably in the form of the original 60GHz band (57-66GHz) and extended 60GHz band (57-71GHz). Moreover, WRC19 recently identified the 66-71GHz frequency range for IMT operation in certain regions.

The proximity of this frequency range (57-71GHz) to FR2 and the imminent commercial opportunities for high data rate communications makes it compelling for 3GPP to address NR operation in this frequency regime.

In order to minimize the specification burden and maximize the leverage of FR2 based implementations, 3GPP has decided to extend FR2 operation up to 71GHz with the adoption of one or more new numerologies (i.e., larger subcarrier spacings). That or those new numerologies will be identified by the study on waveform for NR>52.6GHz in the first half of 2020.  NR-U defined procedures for operation in unlicensed spectrum will also be leveraged towards operation in the unlicensed 60GHz band.

RAN1 carried out a Rel-17 study on supporting NR from 52.6 GHz to 71GHz. Subcarrier spacing of 120kHz with NCP was recommended to be supported. Additional subcarrier spacings 480kHz, and 960kHz in addition to 120kHz are introduced for this frequency range. The spec impact for each subcarrier spacing choice has been identified. Additional areas for further physical layer enhancements have been identified as well. For channel access, both LBT mode and no-LBT mode are recommended to be supported to cover a wide range of use cases and regulatory requirements. For LBT mode, the channel access mechanism defined in EN 302 567 has been identified as the baseline and enhancements have been identified for further discussion.

## 4	Objective

### 4.1	Objective of SI or Core part WI or Testing part WI

According to the outcome of the study item on Supporting NR above 52.6GHz and leveraging FR2 design to the extent possible, this WI extends NR operation up to 71GHz considering, both, licensed and unlicensed operation, with the following objectives:

- Physical layer aspects including [RAN1]:

  - In addition to 120kHz SCS, specify new SCS, 480kHz and 960kHz, and define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals, only NCP supported.

Note: Except for timing line related aspects, a common design framework shall be adopted for 480kHz to 960kHz

  - Timeline related aspects adapted to 480kHz and 960kHz, e.g., BWP and beam switching timing, HARQ timing, UE processing, preparation and computation timelines for PDSCH, PUSCH/SRS and CSI, respectively.

  - Support of up to 64 SSB beams for licensed and unlicensed operation in this frequency range.

  - Supports 120kHz SCS for SSB and 120kHz SCS for initial access related signals/channels in an initial BWP.

    - Study and specify, if needed, additional SCS (480kHz, 960kHz) for SSB for cases other than initial access.

    - Note: coverage enhancement for SSB is not pursued.

  - In addition to 120kHz, support 480 kHz SSB for initial access with support of CORESET#0/Type0-PDCCH configuration in the MIB with following constraints:

    - Limited sync raster entry numbers

      - It is assumed that RAN4 supports a channelization design which results in the total number of synchronization raster entries considering both licensed and unlicensed operation in a 52.6 – 71 GHz band no larger than 665 (Note: the total number of synchronization raster entries in FR2 for band n259 + n257 is 599). If the assumption cannot be satisfied, it’s up to RAN4 to decide its applicability to bands in 52.6 – 71 GHz.

    - only 480kHz CORESET#0/Type0-PDCCH SCS supported for 480 kHz SSB SCS.

    - Prioritize support SSB-CORESET#0 multiplexing pattern 1. Other patterns discussed on a best effort basis.

    - 960 kHz numerology for the SSB is not supported by the UE for initial access in Rel-17.

    - Note: Strive to minimize specification impact by reusing tables for CORESET#0 and type0-PDCCH CSS set configuration defined for FR2 in Rel-15, as much as possible

    - Note: 480 kHz is an optional SSB numerology for initial access for the UE. A UE supporting a band in 52.6-71 GHz must at least support 120 kHz SCS (for initial access and after initial access)

    - Note: Dependency or lack thereof for a UE supporting 480kHz and/or 960kHz numerology for data and control to also support 480kHz SSB numerology for initial access is to be tackled as part of UE capability discussion.

  - Support ANR and PCI confusion detection for 120, 480 and 960kHz SCS based SSB, support CORESET#0/Type0-PDCCH configuration in MIB of 120, 480 and 960kHz SSB

    - FFS: additional method(s) to enable support to obtain neighbour cell SIB1 contents related to CGI reporting

    - Only 1 CORESET#0/Type0-PDCCH SCS supported for each SSB SCS, i.e., (120, 120), (480, 480) and (960, 960).

    - Prioritize support SSB-CORESET#0 multiplexing pattern 1. Other patterns discussed on a best effort basis.

    - Note: Strive to minimize specification impact by reusing tables for CORESET#0 and type0-PDCCH CSS set configuration defined for FR2 in Rel-15, as much as possible

    - Note: From UE perspective, ANR detection for 480/960kHz SCS based SSB is not supported if the UE does not support 480/960 SCS for SSB.

    - Note: for ANR, when reading the MIB, the cell containing the SSB is known to the UE, as defined in 38.133 specification.

  - Specify timing associated with beam-based operation to new SCS (i.e., 480kHz and/or 960kHz), study, and specify if needed, potential enhancement for shared spectrum operation

    - Rel-15/16 and any Rel-17 beam management enhancements can be considered for 52.6-71 GHz. Whether particular features should be excluded for 52.6-71 GHz can be further discussed

    - Note: As per usual procedure, duplication of work between work items in Rel-17 should be avoided

  - Support enhancement for PUCCH format 0/1/4 to increase the number of RBs under PSD limitation in shared spectrum operation.

  - Support enhancements for multi-PDSCH/PUSCH scheduling and HARQ support with a single DCI

Note: coverage enhancement for multi-PDSCH/PUSCH scheduling is not pursued

  - Support enhancement to PDCCH monitoring, including blind detection/CCE budget, and multi-slot span monitoring, potential limitation to UE PDCCH configuration and capability related to PDCCH monitoring.

  - Specify support for PRACH sequence lengths (i.e. L=139, L=571 and L=1151) and study, if needed, specify support for RO configuration for non-consecutive RACH occasions (RO) in time domain for operation in shared spectrum

  - Evaluate, and if needed, specify the PTRS enhancement for 120kHz SCS, 480kHz SCS and/or 960kHz SCS, as well as DMRS enhancement for 480kHz SCS and/or 960kHz SCS.

- Physical layer procedure(s) including [RAN1]:

  - Channel access mechanism assuming beam-based operation in order to comply with the regulatory requirements applicable to unlicensed spectrum for frequencies between 52.6GHz and 71GHz.

    - Specify both LBT and No-LBT related procedures, and for No-LBT case no additional sensing mechanism is specified.

    - Study, and if needed specify, omni-directional LBT, directional LBT and receiver assistance in channel access

    - Study, and if needed specify, energy detection threshold enhancement

- Radio interface protocol architecture and procedures [RAN2]:

  - For operation in this frequency range: Introduce higher layer support of enhancements listed above that are agreed to be specified.

  - Note: RAN2 is to prioritize protocol support of RAN1 design and not on optimizations on items not discussed in RAN1.

- Core specifications for UE, gNB and RRM requirements [RAN4]:

  - Specify new band(s) for the frequency range from 52.6GHz-71GHz. The band(s) definition should include UL/DL operation and excludes ITS spectrum in this frequency range.

  - Specify gNB and UE RF core requirements for the band(s) in the above frequency range, including a limited set of example band combinations (see Note 1).

    - For the case of FR2-2 DC or CA with an anchor in FR1 the following three example band combinations shall be considered:

      - n79 + Nx

      - n77 + Nx

      - n41 + Nx

    - where Nx is the 57-71 GHz band for unlicensed operation and the [66-71] GHz for licensed operation.

    - RAN4 to further discuss the need for single or multiple bands relevant for FR2-2 licensed/unlicensed operation.

  - Specify RRM/RLM/BM core requirements.

Note 1: The WI can be completed provided requirements for at least one band combination involving a new NR-U band is specified as long as it is in line with country-specific regulatory directives.

Note 2: UEs supporting a band in the range of 52.6GHz-71GHz are not required to support 480kHz SCS and 960kHz SCS.

Note 3: The maximum FFT size required to operate the system in 52.6GHz-71GHz frequency is 4096, and the maximum of RBs per carrier is 275 RBs.

Note 4: the system is designed to support both single-carrier and multi-carrier operation.

Note 5: FR2 is extended to cover 24.25GHz to 71GHz with FR2-1 for 24.25-52.6GHz and FR2-2 for 52.6-71GHz.

  - The related UE capabilities and their applicability to the frequency range 52.6 to 71 GHz will have to be analyzed on a case-by-case basis

  - The application of any of the UE feature introduced for 52.6-71 GHz to existing FR1/FR2 should be discussed case by case.

  - TSG RAN specifications shall make it very clear (to readers) that frequency bands in the 52.6-71GHz range are only Release-independent from Rel-17 onwards, to ensure that there is clear industry understanding about which FR2 features are applicable for operation in 52.6-71GHz range.

NOTE 5a: Whenever the FR2 is referred, both FR2-1 and FR2-2 frequency sub-ranges shall be considered in this release, unless otherwise stated.

NOTE 5b: The designations FR2-1 and FR2-2 should only be used when needed.

Similar to regular NR and NR-U operations below 52.6GHz, NR/NR-U operation in the 52.6GHz to 71GHz can be in stand-alone or aggregated via CA or DC with an anchor carrier.

### 4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

Specify the following requirements [RAN4]

- Base station demodulation performance requirements

- UE demodulation performance requirements

- Radio Resource Management performance requirements, including RRM/RLM test cases

- Base station conformance testing

  - Study and define NR 52.6-71 GHz BS OTA methods

### 4.3	RAN time budget request (not applicable to RAN5 Wis/Sis)

NOTE:	For all new RAN related Wis/Sis which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed, then leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID’s zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

Additional comments to the time budget request in the attached Excel table:

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |

NOTE:	If this is a RAN WI including Core and Perf. Part, then all new Core part specs have to be listed first and then all new Perf. Part specs. Indicate “Core part” or “Perf. Part” under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.211 | Extending NR operation up to 71GHz | RAN#94 | Core part |
| 38.212 | Extending NR operation up to 71GHz | RAN#94 | Core part |
| 38.213 | Extending NR operation up to 71GHz | RAN#94 | Core part |
| 38.214 | Extending NR operation up to 71GHz | RAN#94 | Core part |
| 38.215 | Extending NR operation up to 71GHz | RAN#94 | Core part |
| 37.213 | Extending NR operation up to 71GHz | RAN#94 | Core part |
| 38.300 | Extending NR operation up to 71GHz | RAN#95 | Core part |
| 38.321 | Extending NR operation up to 71GHz | RAN#95 | Core part |
| 38.331 | Extending NR operation up to 71GHz | RAN#95 | Core part |
| 38.101-1 | Extending NR operation up to 71GHz (core) | RAN#95 | Core part |
| 38.101-2 | Extending NR operation up to 71GHz (core) | RAN#95 | Core part |
| 38.101-3 | Extending NR operation up to 71GHz (core) | RAN#95 | Core part |
| 38.104 | Extending NR operation up to 71GHz (core) | RAN#95 | Core part |
| 38.124 | Extending NR operation up to 71GHz (core) | RAN#95 | Core part |
| 38.133 | Extending NR operation up to 71GHz (core) | RAN#95 | Core part |
| 38.174 | Extending NR operation up to 71GHz (core) | RAN#95 | Core part |
| 38.423 | Extending NR operation up to 71GHz (core) | RAN#97 | Core part |
| 38.473 | Extending NR operation up to 71GHz (core) | RAN#97 | Core part |
| 38.101-4 | Extending NR operation up to 71GHz (performance) | RAN#98 | Perf. Part |
| 38.104 | Extending NR operation up to 71GHz (performance) | RAN#98 | Perf. Part |
| 38.113 | Extending NR operation up to 71GHz (performance) | RAN#98 | Perf. Part |
| 38.133 | Extending NR operation up to 71GHz (performance) | RAN#98 | Perf. part |
| 38.141-2 | Extending NR operation up to 71GHz (performance) | RAN#98 | Perf. Part |
| 37.941 | Extending NR operation up to 71GHz (performance) | RAN#98 | Perf. Part |

NOTE:	If this is a RAN WI including Core and Perf. Part, then all new Core part specs have to be listed first and then all new Perf. Part specs. Indicate “Core part” or “Perf. Part” under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. Part), then it has to be listed twice with appropriate approval dates.

## 6	Work item Rapporteur(s)

Jing Sun, Qualcomm, jingsun@qti.qualcomm.com

Aida Vera Lopez, Intel, aida.l.vera.lopez@intel.com

## 7	Work item leadership

RAN1

RAN2, RAN4 (secondary)

## 8	Aspects that involve other WGs

NOTE:	For RAN Wis: Section 8 applies only to WGs outside of TSG RAN because RAN WG aspects have to be covered in section 4.

## 9	Supporting Individual Members

Supporting IM name

Qualcomm

Intel

Nokia

Nokia Shanghai Bell

Ericsson

vivo

Samsung

Deutsche Telekom

LG Electronics

Charter Communications

Motorola Mobility

Lenovo

OPPO

NEC

Huawei

HiSilicon

TCL Communications

ZTE

CATT

CMCC

Sanechips

NTT DOCOMO

AT&T