# 3GPP TSG RAN Meeting #99	RP-230289

Rotterdam, Netherlands, Mar 20th - 23rd, 2023	(revision of RP-223065)

# Source:	CMCC, CATT

# Title:	Revised WID on UE Conformance - Enhancement of Network Slicing Phase 2

# Document for:	Approval

Agenda Item:	13.3.19

# 3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: 	UE Conformance - Enhancement of Network Slicing Phase 2

Acronym: eNS_Ph2-UEConTest

Unique identifier: 950065

| This WID includes a Testing part |  | X |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access |  |
|  | Core Network | X |
|  | Services |  |

# Potential target Release: Rel-17.

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  |  |  |
| No | X | X | X | X | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a

|  | Feature |
| --- | --- |
| X | Building Block |
|  | Work Task |
|  | Study Item |

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| eNS_Ph2 | SA2 | 900011 | Stage 2 for Enhancement of Network Slicing Phase 2 |
| eNS_Ph2 | CT1 | 910041 | CT1 aspects of eNS_Ph2 |

2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

# 3	Justification

The Rel-17 work item on Enhancement of Network Slicing Phase 2 in SA and CT is for the purpose of Network Slice Access Control (NSAC), including the following UE requirements:

- Restriction on the maximum number of UEs per network slice with a proper cause and a backoff timer.

- Restriction on the maximum number of PDU sessions per network slice with a proper cause and a backoff timer.

- Support of subscription-based mechanism ensuring that a UE is only allowed to be registered with compatible network slices.

The completion level of the 3GPP Rel-17 work item on Enhancement of Network Slicing Phase 2 has achieved 80% at CT#94-e (Dec-2021). There is a need to introduce an associated RAN5 work item to enable UE conformance testing for Enhancement of Network Slicing Phase 2.

# 4	Objective

4.1	Objective of SI or Core part WI or Testing part WI

The objective of the proposed Work Item is to define UE conformance requirements corresponding to the WID on Rel-17 enhancement of Network Slicing Phase2, including protocol test cases and associated test environment, test applicability, etc.

# 5	Expected Output and Time scale

| New specifications |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |

| Impacted existing TS/TR |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 38.508-1 | Definition of common environment for Rel-17 enhancement of Network Slicing Phase2 | TSG RAN#100 (June-23) |  |
| TS 38.508-2 | Introduction of common implementation conformance statements for Rel-17 enhancement of Network Slicing Phase2 | TSG RAN#100 (June-23) |  |
| TS 38.523-1 | Introduction of the SIG test cases for Rel-17 enhancement of Network Slicing Phase2 | TSG RAN#100 (June-23) |  |
| TS 38.523-2 | Introduction of test applicability for SIG test cases impacted by Rel-17 enhancement of Network Slicing Phase2 | TSG RAN#100 (June-23) |  |
| TS 38.523-3 | Introduction of test model Rel-17 enhancement of Network Slicing Phase2 | TSG RAN#100 (June-23) | Note: Progress of TTCN development is tracked in MCC TF160 reports to RAN5/RAN. |

6	Work item Rapporteur(s)

Danni Song (China Mobile)

songdan@chinamobile.com

Xiaozhong Chen (CATT)

chenxiaozhong@catt.cn

# 7	Work item leadership

RAN5

# 8	Aspects that involve other WGs

None

# 9	Supporting Individual Members

Supporting IM name

AT&T

China Mobile

China Telecom

China Unicom

CATT

DISH Network

Ericsson

Huawei

Hisilicon

Lenovo

Motorola Mobility

Orange

Qualcomm

R&S

T-Mobile USA

Verizon

ZTE