3GPP TSG RAN Meeting #100	RP-231308

Taipei, Taiwan, June 12-14, 2023	(revision of RP-221548)

# Source:	MediaTek Inc.

# Title:	Revised WID on UE Conformance – NB-IoT/eMTC support for Non-Terrestrial Networks (NTN) including EPS aspects

# Document for:	Approval

Agenda Item:	13.3.25

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	UE Conformance – NB-IoT/eMTC support for Non-Terrestrial Networks (NTN) including EPS aspects

## Acronym: LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest

## Unique identifier:

| This WID includes a Testing part |  | X |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access | X |
|  | Core Network | X |
|  | Services |  |

Potential target Release: Rel-17

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  |  |  |
| No | X |  | X | X |  |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a

|  | Feature |
| --- | --- |
| X | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| LTE_NBIOT_eMTC_NTN | RAN1 | 920069 | NB-IoT/eMTC support for Non-Terrestrial Networks (NTN) |
| LTE_NBIOT_eMTC_NTN-Core | RAN1 | 920169 | Core part: NB-IoT/eMTC support for Non-Terrestrial Networks (NTN) |
| IoT_SAT_ARCH_EPS | CT1 | 940001 | CT1 aspects of NB-IoT/eMTC Non-Terrestrial Networks in EPS |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

## 3	Justification

IoT operation is critical in remote areas with low/no cellular connectivity for many different industries, including e.g.:

-	Transportation (maritime, road, rail, air) & logistics

-	Solar, oil & gas harvesting

-	Utilities

-	Farming

-	Environment monitoring

-	Mining etc.

The capabilities of NB-IoT and eMTC are a good fit to the above but will require satellite connectivity to provide coverage beyond terrestrial deployments, where IoT connectivity is required. There is an urgent need for a standardized solution allowing global IoT operation anywhere on Earth, in view of other solutions already available.

RAN#92-e approved a Rel-17 Work Item LTE_NBIOT_eMTC_NTN (latest WID in RP-220938) to address the above demand for IoT. The work item is carried out to enable NB-IoT and eMTC to support Non-Terrestrial Networks (NTN) under the following assumptions:

- Transparent payload based GEO and NGSO network scenarios addressing at least 3GPP power class 3 UE with GNSS capability in both Earth fixed &/or moving cell configurations.

The Work Item LTE_NBIOT_eMTC_NTN included objectives to enhance RAN1, RAN2, and RAN3 specifications in Rel-17 to enable NB_IoT and eMTC to operate in an NTN.

And SA#93-e approved a Rel-17 Work Item IoT_SAT_ARCH_EPS in SP-211306 to define minimum essential functionality to introduce support for NB-IoT and eMTC Non-Terrestrial Networks in EPS using 5GSAT_ARCH solutions as baseline and adjusting them to EPS and NB-IoT/eMTC characteristics as necessary, in alignment with the RAN Work Item.

The completion level of the 3GPP Rel-17 work item on NB-IoT/eMTC support for Non-Terrestrial Networks (NTN) has achieved 99% at RP#95-e (Mar-2022), and the Target Completion Date of the WI is June 2022. The completion level of the 3GPP Rel-17 work item on CT1 aspects of NB-IoT/eMTC Non-Terrestrial Networks in EPS is 75% at CT1#135-e (Apr-2022). To fulfil the strong demand of IoT NTN, there is a need to introduce an associated RAN5 work item to enable UE conformance testing for NB-IoT/eMTC support for Non-Terrestrial Networks (NTN) including EPS aspects.

## 4	Objective

### 4.1	Objective of SI or Core part WI or Testing part WI

The objective of this work item is to enable UE conformance testing for Rel-17 NB-IoT/eMTC support for Non-Terrestrial Networks (NTN) including EPS aspects WI corresponding to the WID on LTE_NBIOT_eMTC_NTN with Unique identifier 920069 and the WID on IoT_SAT_ARCH_EPS with Unique identifier 940001.

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 36.508 | Definition of common test environment for Rel-17 NB-IoT/eMTC support for Non-Terrestrial Networks (NTN) including EPS aspects test cases | TSG RAN#101 (Sep-23) |  |
| TS 36.523-1 | Introduction of the SIG test cases for Rel-17 NB-IoT/eMTC support for Non-Terrestrial Networks (NTN) including EPS aspects | TSG RAN#101 (Sep-23) |  |
| TS 36.523-2 | Introduction of ICS and test applicability for SIG test cases impacted by Rel-17 NB-IoT/eMTC support for Non-Terrestrial Networks (NTN) including EPS aspects | TSG RAN#101 (Sep-23) |  |
| TS 36.523-3 | Introduction of test model for Rel-17 NB-IoT/eMTC support for Non-Terrestrial Networks (NTN) including EPS aspects | TSG RAN#101 (Sep-23) | Progress of TTCN development of the new protocol test cases is tracked in MCC TF160 reports to RAN5/RAN. |

## 6	Work item Rapporteur(s)

Daiwei Zhou (MediaTek)

daiwei.zhou@mediatek.com

## 7	Work item leadership

RAN5

## 8	Aspects that involve other WGs

None

## 9	Supporting Individual Members

Supporting IM name

MediaTek

ZTE

SRTC

Tejet

Ericsson

Verizon