3GPP TSG RAN meeting #103	RP-240451

Maastricht, Netherlands, March 18 – 21	(Revision of RP-233259)

Agenda Item:	10.3.2

Source:	MediaTek Inc.

Title: 	Revised WID: FDD band (L+S band) for IoT NTN operation

Document for:	Discussion

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Introduction of a new FDD band (L+S band) for IoT NTN operation

Acronym: IOT_NTN_LSBand

Unique identifier:

NOTE:	For new WIs/SIs leave the Unique identifier empty and make a proposal for an Acronym.

For a revised WI/SI: Take Unique identifier and acronym as shown in 3GPP workplan.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part | X |

or:

| This WID includes a Testing part |  |  |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access |  |
|  | Core Network |  |
|  | Services |  |

Target Release:	Rel-18

NOTE: In case of contradiction with the target dates of clause 5, clause 5 determines the target release.

# 1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X |  |  |
| No | X |  |  | X | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a …

|  | Feature |
| --- | --- |
| X | Building Block |
|  | Work Task |
|  | Study Item |

NOTE:	Normally, Core/Perf./Testing parts in RAN WIDs are Building Blocks. Only if they are under an SA or CT umbrella, they are defined as work tasks. If you are in doubt, please contact MCC.

### 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one 	WID. Therefore the table above should include the feature WI data (In case the feature covers Core and Perf. 	part, please list under Working Group the leading WG of the Core part).

### 2.3	Other related Work Items and dependencies

| Other related Work/Study Items (if any) |  |  |  |
| --- | --- | --- | --- |
| Acronym | Unique ID | Title | Nature of relationship |
|  |  |  | {optional free text} |

NOTE:	Also related or dependent WIs/SIs in other TSGs shall be indicated here.

# 3	Justification

To address various IoT demands and enable IoT operation in an NTN, the ongoing Rel-18 WI on IoT NTN finally adds the missing RAN4 piece by specifying RAN4 core and performance requirements in addition to the related RAN1/2/3 work carried out in Rel-17, and these Rel-18 requirements will be applicable to the Rel-17 specifications for NB-IoT and eMTC operation over NTN in a release dependence manner [1]. The WI specifies two new FDD bands NB-IoT/eMTC NTN operation by leveraging the studies and requirements of NR NTN bands n255 and n256:

  - S-band (1980-2010 MHz in UL, and 2170-2200 MHz in DL)

  - L band (1626.5 MHz – 1660.5 MHz in UL, and 1525 MHz – 1559 MHz in DL)

Recently some demands for new IoT NTN bands are emerging, for example, a new band demand with 1610 – 1626.5 MHz in UL at L-band, and 2483.5 – 2500 MHz in DL at S-band.

# 4	Objective

### 4.1	Objective of SI or Core part WI or Testing part WI

The objective of this Work Item is to introduce a new FDD band for NB-IoT/eMTC operation over NTN with the objective:

- Specify the following new FDD frequency bands for NB-IoT/eMTC NTN operation:

  - 1610 – 1626.5 MHz in UL, and 2483.5 – 2500 MHz in DL

- Support UE power class 3 (+23dBm) and power class 5 (+20dBm)

This band will be Release-independent from Rel-17 for the UE.

NOTE: leverage the studies and requirements from NR NTN bands.

### 4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

The objective of the performance part is to:

-	Define SAN conformance requirements.

NOTE: reuse decision from NR NTN discussion on n254, where possible.

### 4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed, then leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |
| TR | 36.764 | IoT NTN operating bands | #101 | #102 | Core part; Huanren Fu, MediaTek, huanren.fu@mediatek.com |

{Note 1: Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
"Internal TR" is intended for 3GPP internal use only whereas "External TR" may be transposed by OPs.}

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 36.102 | Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception for satellite access | RAN#102 | Core |
| 36.108 | Evolved Universal Terrestrial Radio Access (E-UTRA); Satellite Access Node radio transmission and reception | RAN#102 | Core |
| 36.133 | Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management | RAN#102 | Core |
| 36.307 | Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements on User Equipments (UEs) supporting a release-independent frequency band | RAN#102 | Core |
| 36.181 | Evolved Universal Terrestrial Radio Access (E-UTRA); Satellite Access Node conformance testing | RAN#104 | Performance |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

# 6	Work item Rapporteur(s)

Huanren Fu, MediaTek Inc. huanren.fu@mediatek.com

NOTE:	The first listed Rapporteur has the overall responsibility for this WI (incl all secondary tasks).

# 7	Work item leadership

RAN WG4

# 8	Aspects that involve other WGs

{Specify all the other WG(s) to be involved and, if specific, their task. E.g.: "SA2, SA3, SA5. CT6 for storage, and potentially SA4". If not applicable, indicate "None" or "None identified yet".}

NOTE:	For RAN WIs: Section 8 applies only toWGs outside of TSG RAN because all RAN WG aspects have to be covered in section 4.

# 9	Supporting Individual Members

{At least 4 supporting Individual Members are needed. There is an expectation that these companies will provide resources to progress the work. Note that having 4 supporting companies is a necessary but not sufficient condition: the usual TSG approval process by consensus is needed for the WID approval.}

Supporting IM name

MediaTek Inc.

Qualcomm

ZTE

Kepler Communications

Apple

Globalstar

Skyworks Solutions Inc.

Huawei

HiSilicon

III