TSG SA Meeting #SP-94E	SP-211651

14 - 20 December 2021, Electronic meeting

TSG SA Meeting #SP-94E	SP-211616

14 - 20 December 2021, Electronic meeting

TSG SA Meeting #SP-94E	SP-211331r04

14 - 20 December 2021, Electronic meeting

TSG SA Rel-18 Prioritization Workshop	SP-211195r01

9-10 December 2021, Electronic meeting

TSG SA Rel-18 Prioritization Workshop	SP-211149

9-10 December 2021, Electronic meeting

SA WG2 Meeting #S2-147E	S2-2108164

18 - 22 October, 2021, Electronic meeting	(revision of S2-2107451)

Source:		Thales, Xiaomi, Novamint, TNO, ABS

Title:		New SID: 5GC enhancement for satellite access Phase 2

Document for:	Approval

Agenda Item:	9.1.3

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Study on satellite access Phase 2

Acronym:	FS_5GSAT_Ph2

Unique identifier:	940074

Potential target Release:	Rel-18

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No |  |  |  |  |  |
| Don't know | X |  |  |  | X |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A |  |  |  |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 770002 | Study on using Satellite Access in 5G | Use cases and requirements for satellite access in 5G |
| 800048 | Stage 1 of 5GSAT | Service requirements of satellite access in 5G |
| 860010 | Guidelines for Extra-territorial 5G Systems | New regulatory requirements |
| 800026 | Study on architecture aspects for using satellite access in 5G | Unresolved key issue leftover from R17 |
| 860005 | (Stage 2 of) Integration of satellite components in the 5G architecture | Baseline of 5G architecture to support satellite access |
| 890022 | Study on vehicle-mounted relays | Service requirements related to satellite access |

Dependency on non-3GPP (draft) specification:

# 3	Justification

5GSAT_ARCH has been developed in R17 to address service requirements of satellite access in 5G; however, some aspects have not been considered nor fully realized. One example is NGSO regenerative-based satellite access. Key Issue #6 has been specified in TR 23.737; however, due to the lack of support of this feature by RAN, there was no solution developed in R17.

Note1: After prioritization process for R18 studies, study of NGSO regenerative-based satellite access impacts on 5GC architecture are to be considered for further releases.

Besides, there are also features proposed in RAN WG having 5GC impact, i.e. Discontinuous coverage. Dynamic support of discontinuous coverage is required for initial NGSO constellation deployment but as well to support evolution of the constellations such as loss of satellites, different releases supported in a given constellation. UE may have access to satellite service coverage only at specific time and places due to sparse constellation. UE location may not be timely aware of by the network to enable efficient paging, due to which mobility management mechanism needs to be enhanced. Moreover, UE may not always have to stay awake for the sake of power efficiency, especially for MIoT UE. Hence, the prediction, mechanisms on awareness & notification of UE wake-up time and data storage & forwarding for UEs temporarily out of coverage may be needed. There is also an important aspect of discontinuous coverage related to dynamic GEO systems where beams/cells are intermittent due to use of dynamic beam configuration. This issue also applies to broadcast/multicast case.

Note2: After prioritization process for R18 studies, architectural enhancements to support discontinuous coverage for broadcast/multicast is de-scoped from this SID and might be considered again during R18 in case of RAN request.

In summary, several enhancements to 5GC have to be considered for satellite access in Rel18, in collaboration with RAN TSGs

# 4	Objective

The study item aims at investigating on further 5GC/EPC enhancements to support satellite access based on the R17 5GSAT_ARCH achievements with the following 5GC/EPC areas for study:

- WT#1: Discontinuous coverage

WT#1.1: Architectural enhancements to support discontinuous coverage for mobility enhancement (e.g. paging enhancement)

WT#1.2: Architectural enhancements considering prediction, awareness & notification of UE wake-up time, power saving optimizations.

## TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
| WT#1 | 4 | 1 |  | WT#1 is self-contained. |
| WT#1.1 | 2 | 0.5 | Yes |  |
| WT#1.2 | 2 | 0.5 | Yes |  |

Total TU estimates for the study phase:     4

Total TU estimates for the normative phase:    1

Total TU estimates 4+1=     5

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 23.xyz | Study on Integration of satellite components in the 5G architecture Phase II | SA#96 (June. 2022) | SA#97 (Sept. 2022) | Fine, Jean-Yves , Thales, jean-yves.fine@thalesgroup.com Yuxin Mao Xiaomi, maoyuxin1@xiaomi.com |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

# 6	Work item Rapporteur(s)

Rapporteur: Fine, Jean-Yves, Thales, jean-yves.fine@thalesgroup.com.

Co-rapporteur: Yuxin Mao , Xiaomi, maoyuxin1@xiaomi.com

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

SA3 for the Security aspects, SA5 for the Charging aspects, RAN for the RAN related issues.

# 9	Supporting Individual Members

Supporting IM name

Thales

Xiaomi

OPPO

Tencent

ESA

Avanti

Gatehouse Satcom

Sateliot

Intelsat

Ligado

TNO

Novamint

Gilat Satelite networks Ltd

Inmarsat

Hughes Network Systems

Hispasat

SES

CATT

Eutelsat

Leonardo

Toyota

ABS

Mediatek