TSG SA Meeting #SP-99	SP-230150

21 - 24 March 2023, Rotterdam, Netherlands

# Source:	SA WG3

# Title:	New WID on Automated certificate management in SBA

Document for:  Approval

3GPP TSG-SA3 Meeting #110 	S3-231563

Athens, Greece, 20 - 24 February 2023

# Source:	Nokia, Nokia Shanghai Bell

# Title:	New WID on Automated certificate management in SBA

# Document for:	Approval

# Agenda Item:	6

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Automated certificate management in SBA

Acronym: ACM_SBA

Unique identifier:	990037

Potential target Release:	Rel-18

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  |  |
| Don't know |  |  |  |  | X |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| FS_ACM_SBA | SA3 | 920022 | Study on Standardising Automated Certificate Management in SBA |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

# 3	Justification

The purpose of TR 33.876 “Study on Standardising Automated Certificate Management in SBA” was to define a standardized model and set of associated procedures for automated certificate management in 5G SBA.

The study has analysed the following aspects:

- Suitable single protocol family to address the procedures of enrolment and renewal of certificates

- The security aspects related to NF certificate enrolment and update procedures

- Validation of the trust chain of certificate authority hierarchy

- Revocation procedures

- The relation between the certificate and NF management lifecycles

- Security considerations on having multiple certificates associated with a NF

- Protection of the NF instance identifiers

- Procedures of certificate management in Network Slicing use cases

In relation with to the conclusive statements per key issue in clause 7 of the study, i.e., whether and how to move forward with normative work, this WID will adopt the decisions and implement the conclusions accordingly mainly into TS 33.310, and if applicable, into TS 33.501.

# 4	Objective

The objective of this work item is the normative work according to the conclusions from the study TR 33.876 described in clause 7:

- KI#1: Adopt and specify the CMP protocol for the procedures of certificate enrolment and certificate renewal in 5G SBA.

- KI#2: It is proposed to pursue an approach where the OAM system of the NF provides the initial trust to be used during NF operator certificate enrolment procedure. The initial trust can be implemented by an initial (OAM) certificate (although other implementation options are also valid, e.g., signature of certain NF parameters), which can be verified by the operator CA/RA, since the trust between the OAM system and the operator CA/RA has been pre-established.

- KI#5: For certificate revocation, it is agreed to pursue CRL in the normative phase. OCSP and OCSP stapling are possible additional certificate revocation schemas for SBA. The support of those mechanisms will be decided during normative phase.

- KI#7: Investigate the possibility to adopt the extensions introduced in IETF draft-reddy-lamps-jose-eku-00, which is work in progress in IETF at the time of writing.

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 33.310 | Updates in a new chapter (10), and informative annex may be required to collect SBA Certificate management aspects. | SA#101 |  |

# 6	Work item Rapporteur(s)

Peinado, German, Nokia, german.peinado@nokia.com

# 7	Work item leadership

SA WG3

# 8	Aspects that involve other WGs

To be defined according to further conclusions. If any, CT4 for stage 3 work.

# 9	Supporting Individual Members

Supporting IM name

Nokia

Nokia Shanghai Bell

Huawei

HiSilicon

Ericsson

AT&T

Intel

Cablelabs

NTT DOCOMO INC.

China Telecom

Deutsche Telekom

Charter

Mavenir

China Mobile