TSG SA Meeting #104	SP-240994

18-21 June 2024, Shanghai, China	(Revision of SP-240636)

# Source:	SA WG2

# Title:	New WID on Multi-Access (ATSSS_Ph4)

# Document for:	Approval

Agenda Item:	6.2.2

SA WG2 Meeting #163	S2-2407370

Jeju, Korea, May 27 – May 31, 2024

# Source:	Apple (Primary Rapporteur)

# Title:	New WID on Multi-Access (ATSSS_Ph4)

# Document for:	Approval

Agenda Item:	30.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: 	Multi-Access (ATSSS_Ph4)

Acronym:	MASSS

Unique identifier:	1040034

Potential target Release:	Rel-19

# Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  | X |  | X |
| Don’t know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| FS_MASSS | SA2 | 1020070 | Study on Multi-Access (DualSteer and ATSSS_Ph4) |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 940070 | Access Traffic Steering, Switch and Splitting support in the 5G system architecture; Phase 3 | Rel-18 Work Item |

# 3	Justification

1)	In Rel-18, a high-layer steering functionality “MPQUIC steering functionality using UDP proxying over HTTP” was defined that enables steering, switching, and splitting UDP traffic based on IETF protocols. It would be beneficial to specify how to enable the MPQUIC steering functionality to also steer, switch, and split non-UDP traffic (IP & Ethernet traffic).

# 4	Objective

The objective of this work item is to specify the architecture enhancements, functionalities and procedures to support MASSS features based on conclusions of TR 23.700-54 (clause 8).

Specifically, the detailed objectives include:

- New functionalities and procedures to support MPQUIC steering functionalities in order to steer, switch, and split non-UDP traffic.

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.501 | Description of new MPQUIC steering functionalities to support steering, switching and splitting of IP and Ethernet traffic. | TSG#106 (Dec. 2024) |  |
| TS 23.502 | Procedures to support steering, switching and splitting of IP and Ethernet traffic using MPQUIC steering functionality. | TSG#106 (Dec. 2024) |  |
| TS 23.503 | Policy control to support steering, switching and splitting of IP and Ethernet traffic using MPQUIC steering functionality. | TSG#106 (Dec. 2024) |  |

# 6	Work item Rapporteur(s)

Primary Rapporteur: Krisztian Kiss (Apple), krisztian@apple.com

Secondary Rapporteur: Zhuoyi Chen (China Telecom), chenzy34@chinatelecom.cn

(in charge of producing reports to the WG plenary on progress)

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

N/A

# 9	Supporting Individual Members

Supporting IM name

Apple

BT

CableLabs

CATT

Charter Communications, Inc

China Telecom

Comcast

Ericsson

ETRI

InterDigital

Lenovo

LG Electronics

NEC

Nokia

SHARP

ZTE