3GPP TSG-RAN Meeting #88-e	RP-200655

Online, 29 June – 03 July 2020	(revision of RP-200100)

# Source:	Ericsson

# Title:	Revised WID on Rel-16 LTE intra-band Carrier Aggregation for x CC DL/y CC UL including contiguous and non-contiguous spectrum (x>=y)

# Document for:	Approval

Agenda Item:	10.4.1

3GPP™ Work Item Description

For guidance, see 3GPP Working Procedures, article 39; and 3GPP TR 21.900.
Information about Work Items can be found at http://www.3gpp.org/Work-Items

# Title: Revised WID on Rel-16 LTE intra-band Carrier Aggregation for x CC DL/y CC UL including contiguous and non-contiguous spectrum (x>=y)

## Acronym: LTE_CA_R16_intra

## Unique identifier: 	800060

NOTE:	For new WIs/SIs leave the Unique identifier empty or you can make a proposal for an Acronym.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part | X |

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  |  |  |
| No | X |  | X | X | X |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a … {Tick one box. "Feature / Building Block / Work Task" form a hierarchical structure. E.g. no Building Block can be proposed without a corresponding parent Feature. The full structure of all existing Work Items is shown in the 3GPP Work Plan in ftp://ftp.3gpp.org/Information/WORK_PLAN }

|  | Feature |
| --- | --- |
| X | Building Block |
|  | Work Task |
|  | Study Item |

NOTE:	Normally, Core/Perf./Testing parts in RAN WIDs are Building Blocks. Only if they are under an SA or CT umbrella, we define them as work tasks. If you are in doubt, please contact MCC.

### 2.2	Parent and child Work Items

{For a Feature: list here the children Building Blocks (optional) and Work Tasks (optional)}

{For a Building Block: list here the parent Feature (mandatory) and children Work Tasks (optional)}

{For a Work Task: list here the parent Building Block (mandatory)}

| Parent and child Work Items |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 800060 | Rel16 LTE Intra-band CA for xDL/yUL including contiguous and non-contiguous spectrum, (x>=y) | Parent WID |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one WID. Therefore the table above should just include the feature WI Unique ID and title and Nature of relationship is "parent WID".

### 2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one but are not part of the hierarchical structure, such as preceding SI or a preceding WI (e.g. if you further enhance a topic).}

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 800160 | Core part: Rel16 LTE Intra-band CA for xDL/yUL including contiguous and non-contiguous spectrum, (x>=y) | Child WID |
| 800260 | Perf. part: Rel16 LTE Intra-band CA for xDL/yUL including contiguous and non-contiguous spectrum, (x>=y) | Child WID |

NOTE:	Also related or dependent WIs in other TSGs should be indicated.

## 3	Justification

CA intra-band configurations are fundamental part of all higher order combinations and all new intra-band configurations will be defined under this WI. New configurations still emerge from exiting bands and whenever new band is specified, it will create a potential for several new intra-band configurations.

The preconditions

To propose DL CA for LTE Intra-band CA for x CC DL/1 CC UL including contiguous and non-contiguous spectrum in rel-16, the constituent LTE band requirements shall be completed and specified in advance.

To propose UL CA UL CA of x CC DL/y CC UL in rel-16 are as follows.

- Constituent x CC DL/ (y-1) CC UL shall be completed and specified in advance.

Example 1: If the following configuration is proposed,

| CA configuration | Uplink CA configuration |
| --- | --- |
| CA_1C | None |

- LTE Band 1 requirements shall be completed and specified in advance.

Example 2: If the following configuration is proposed,

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| CA_1E | CA_1D |

- LTE CA of DL_CA_1E_UL_1C shall be specified in advance.

## 4	Objective

### 4.1	Objective of SI or Core part WI or Testing part WI

- Specify the configuration specific RF requirements for all listed LTE intra-band CA combinations for x CC DL/y CC UL including contiguous and non-contiguous spectrum

  - Applicable frequencies

  - Applicable bandwidths and bandwidth sets

- Analyse combinations that have self-desensitization due to following reasons:

  - TX frequency being in close proximity of one of the receive bands

  - Any other identified reasons

- For the combination where self-desensitization exists, specify at least needed

  - Reference sensitivity excerptions

  - UL RB restrictions for REFSENS test

- Add conformance testing in RAN5 specifications (to follow at a later stage)

of all REL-16 CA configurations that fall into the category defined by the WI title. An overview table of these CA configurations is provided here:

CA configurations for Intra-band
*Unless otherwise stated, the number of UL CC is one for all band in the configuration.

Table 1-1: Individual combination names, proponents and supporting companies for Intra band contiguous CA configurations

| Num CC | CA configuration | REL-indep. from | contact name, company | contact email | other supporting companies (min. 3) | status (new, ongoing, completed, stopped) | supported next level fallback modes (in DL and UL) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | CA_3DL_41D_3UL_41D_BCS0 | REL-12 | Shao Zhe, CMCC | shaozhe@chinamobile.com | Huawei, CATT, ZTE, Nubia | Completed | CA_3DL_41D_2UL_41C_BCS0-Completed |
| 4 | CA_4DL_41E_3UL_41D_BCS0 | REL-12 | George Cummings, Sprint | george.cummings@sprint.com | Nokia, Ericsson, Motorola Mobility, Qualcomm | Completed | CA_4DL_41E_2UL_41C_BCS0-Completed CA_3DL_41D_3UL_41D_BCS0-Ongoing |
| 5 | CA_5DL_41F_3UL_41D_BCS0 | REL-12 | George Cummings, Sprint | george.cummings@sprint.com | Nokia, Ericsson, Motorola Mobility, Qualcomm | Completed | CA_5DL_41F_2UL_41C_BCS0-Completed CA_4DL_41E_3UL_41D_BCS0-New |
| 2 | CA_2DL_48C_2UL_48C_BCS0 | REL-10 | Frank Azcuy Charters Communications | c-Frank.Azcuy@charter.com | Cable Labs, Comcast, Qorvo | Completed | CA_48C_1UL_48A BCS0-completed |
| 3 | CA_3DL_48D_2UL_48C_BCS0 | REL-10 | Nelson Ueng, T-Mobile USA | nelson.ueng@T-Mobile.com | Ericsson, Nokia, Deutsche Telekom, Samsung | Completed | CA_3DL_48D_1UL_48A_BCS0-completed CA_2DL_48C_2UL_48C_BCS0-new |
| 4 | CA_4DL_48E_2UL_48C_BCS0 | REL-10 | Nelson Ueng, T-Mobile USA | nelson.ueng@T-Mobile.com | Ericsson, Nokia, Deutsche Telekom, Samsung | Completed | CA_4DL_48E_1UL_48A_BCS0- Completed CA_3DL_48D_2UL_48C_BCS0-New |

Table 12-2: Individual combination names, proponents and supporting companies for Intra band non-contiguous CA configurations

| Num CC | CA configuration | REL-indep. From | contact name, company | contact email | other supporting companies (min. 3) | status (new, ongoing, completed, stopped) | supported next level fallback modes (in DL and UL) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | CA_3DL_48A-48C_2UL_48C_BCS0 | REL-10 | Frank Azcuy Charters Communications | c-Frank.Azcuy@charter.com | Cable Labs, Comcast, Qorvo | Completed | CA_2DL_48C_2UL_48C-new CA_2DL_48A-48C_1UL_48A-completed |
| 4 | CA_4DL_48C-48C_2UL_48C_BCS0 | REL-10 | Frank Azcuy Charters Communications | c-Frank.Azcuy@charter.com | Cable Labs, Comcast, Qorvo | Completed | CA_3DL_48A-48C_2UL_48C-BCS0-new CA_4DL_48C-48C_1UL_48A-completed |
| 2 | 42A-42A_BCS1 | REL-10 | Kenichi Kihara, SoftBank | kenichi.kihara@g.softbank.co.jp | Nokia, Ericsson, ZTE, Huawei, HiSilicon | Completed | None |
| 3 | 42A-42C_BCS1 | REL-10 | Kenichi Kihara, SoftBank | kenichi.kihara@g.softbank.co.jp | Nokia, Ericsson, ZTE, Huawei, HiSilicon | Completed | 1B_2CC_DL_42A-42A_BCS1-new |
| 4 | 42C-42C_BCS1 | REL-10 | Kenichi Kihara, SoftBank | kenichi.kihara@g.softbank.co.jp | Nokia, Ericsson, ZTE, Huawei, HiSilicon | Completed | 1B_3CC_DL_42A-42C_BCS1-new |
| 3 | 42A-42C_UL_42C_BCS1 | REL-10 | Kenichi Kihara, SoftBank | kenichi.kihara@g.softbank.co.jp | Nokia, Ericsson, ZTE, Huawei, HiSilicon | Completed | 1B_3CC_DL_42A-42C_BCS1-new 1B_2CC_DL_42C_UL_42C_BCS1-completed |
| 4 | 42C-42C_UL_42C_BCS1 | REL-10 | Kenichi Kihara, SoftBank | kenichi.kihara@g.softbank.co.jp | Nokia, Ericsson, ZTE, Huawei, HiSilicon | Completed | 1B_4CC_DL_42C-42C_BCS1-new 1B_3CC_DL_42A-42C_UL_42C-BCS1-new |

Table 1-3: Individual combination names, proponents and supporting companies for Intra band non-contiguous CA configurations

| Combi nation | # CC for DL | CA configuration | Uplink Configuration | REL-indep. from | contact name, company | contact email | other supporting companies (min. 3) | status (new, ongoing, completed, stopped) | supported next level fallback modes (in DL and UL) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Table 1-4: Individual combination names, proponents and supporting companies for Intra band non-contiguous CA configurations

| CA configuration | Uplink Configuration | BCS | contact name, company | contact email | other supporting companies (min. 3) | status (new, ongoing, completed, stopped) | supported next level fallback modes (in DL and UL) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Note: Only one CA configuration can be added per row in the first column while multiple UL configurations can be listed in the second column separated by a newline. |  |  |  |  |  |  |  |

Bandwidth combination set for Intra band

Table 2-1 Bandwidth combinations for Intra band contiguous CA configurations

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Maximum aggregated bandwidth | Bandwidth combination set |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_41D | CA_41D | 41 | 10 | 20 | 15 |  |  |  | 60 | 0 |
|  |  |  | 10 | 15, 20 | 20 |  |  |  |  |  |
|  |  |  | 15 | 20 | 10, 15 |  |  |  |  |  |
|  |  |  | 15 | 10, 15, 20 | 20 |  |  |  |  |  |
|  |  |  | 20 | 15, 20 | 10 |  |  |  |  |  |
|  |  |  | 20 | 10, 15, 20 | 15, 20 |  |  |  |  |  |
| CA_41E | CA_41D | 41 | 15, 20 | 15, 20 | 15, 20 | 20 |  |  | 80 | 0 |
| CA_41F | CA_41D | 41 | 10, 15, 20 | 15, 20 | 20 | 20 | 20 |  | 100 | 0 |
| CA_48C | CA_48C | 48 | 5, 10, 15, 20 | 20 |  |  |  |  | 40 | 0 |
|  |  |  | 20 | 5, 10, 15 |  |  |  |  |  |  |
| CA_48D | CA_48C | 48 | 5, 10, 15, 20 | 20 | 20 |  |  |  | 60 | 0 |
|  |  |  | 20 | 20 | 5, 10, 15 |  |  |  |  |  |
| CA_48E | CA_48C | 48 | 5, 10, 15, 20 | 20 | 20 | 20 |  |  | 80 | 0 |
|  |  |  | 20 | 20 | 20 | 5, 10, 15 |  |  |  |  |
| CA_48B | CA_48B | 48 | 10 | 10 |  |  |  |  | 20 | 9 |

Table 2-2 Bandwidth combinations for Intra band non-contiguous CA configurations

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations (NOTE 4) | E-UTRA Bands | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Maximum aggregated bandwidth | Bandwidth combination set |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_48A-48C | CA_48A-48A CA_48C | 48 | 5, 10, 15, 20 | See CA_48C Bandwidth combination set 0 in 36.101 Table 5.6A.1-1 |  |  |  |  | 60 | 0 |
|  |  |  | See CA_48C Bandwidth combination set 0 in 36.101 Table 5.6A.1-1 |  | 5, 10, 15, 20 |  |  |  |  |  |
| CA_48C-48C | CA_48C | 48 | See CA_48C Bandwidth combination set 0 in 36.101 Table 5.6A.1-1 |  | See CA_48C Bandwidth combination set 0 in 36.101 Table 5.6A.1-1 |  |  |  | 80 | 0 |
| CA_48A-48A-48A | CA_48A-48A | 48 | See CA_48A-48A-48A Bandwidth combination set 0 in the Table 5.6A.1-4 |  |  |  |  |  | 60 | 0 |
| CA_48A-48A | CA_48A-48A | 48 | See CA_48A-48A Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  | 40 | 0 |
| CA_42A-42A | - | 42 | 10, 15, 20 | 10, 15, 20 |  |  |  |  | 40 | 1 |
| CA_42A-42C | - | 42 | 10, 15, 20 | See CA_42C_BCS1 in Table 5.6A.1-1 |  |  |  |  | 60 | 1 |
| CA_42C-42C | - | 42 | See CA_42C_BCS1 in Table 5.6A.1-1 |  | See CA_42C_BCS1 in Table 5.6A.1-1 |  |  |  | 80 | 1 |
| CA_42A-42C | CA_42C | 42 | 10, 15, 20 | See CA_42C_BCS1 in Table 5.6A.1-1 |  |  |  |  | 60 | 1 |
| CA_42C-42C | CA_42C | 42 | See CA_42C_BCS1 in Table 5.6A.1-1 |  | See CA_42C_BCS1 in Table 5.6A.1-1 |  |  |  | 80 | 1 |
| CA_66A-66B | CA_66B | 66 | See CA_66A-66B Bandwidth combination set 0 in 36.101 Table 5.6A.1-3 |  |  |  |  |  | 40 | 0 |

### 4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

This Perf. Part WI has to standardize the Perf. Part requirements:

- New LTE intra-band CA for combinations to be introduced in 36.141.

- Required changes to be added to release independence TS 36.307.

of all REL-16 CA combinations that fall into the category defined by the WI title. See overview table in 4.1 above.

### 4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | Series | Title | For info at TSG# | For approval at TSG# | Remarks |
| Internal TR | TR 36.716-01-01 | LTE Advanced Intra-band Carrier Aggregation |  | TSG#88 | Core part |

{Note 1: Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
"Internal TR" is intended for 3GPP internal use only whereas "External TR" may be transposed by OPs.}

NOTE:	If this is a RAN WID including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 36.101 | UE radio transmission and reception | TSG#88 | Core part |
| 36.104 | BS radio transmission and reception | TSG#88 | Core part |
| 36.141 | BS conformance testing | TSG#88 | Perf. part |
| 36.307 | Release independence | TSG#88 | Perf. part |

NOTE:	If this is a RAN WID including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

## 6	Work item Rapporteur(s)

Per Lindell, Ericsson, per.lindell@ericsson.com

## 7	Work item leadership

R4

## 8	Aspects that involve other WGs

None

NOTE:	For RAN WIDs: Section 8 applies only toWGs outside of TSG RAN because RAN WG aspects have to be covered in section 4.

## 9	Supporting Individual Members

Supporting IM name

NTT DOCOMO, INC.

Qualcomm

Nokia

Nokia Shanghai Bell

Huawei

Ericsson

LGE

Charters Communications

Cable Labs

Comcast

Qorvo

Verizon