3GPP TSG-RAN WG4 Meeting # 91-e	RP-210346

Electronic Meeting, Mar. 22-Mar. 26, 2021

# Source:	ZTE Corporation

# Title:	Revised WID on Rel-17 Dual Connectivity (DC) of x bands (x=1,2,3) LTE inter-band CA (xDL/1UL) and 3 bands NR inter-band CA (3DL/1UL)

# Document for:	Information

Agenda Item:	9.8.19.7

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Rel-17 Dual Connectivity (DC) of x bands (x=1,2,3) LTE inter-band CA (xDL/1UL) and 3 bands NR inter-band CA (3DL/1UL)

Acronym:  DC_R17_xBLTE_3BNR_yDL2UL

Unique identifier: 	881004

NOTE:	For new WIs/SIs leave the Unique identifier empty or you can make a proposal for an Acronym.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part | X |

Potential target Release: Rel-17

Note that this field above indicates the proposed Release at the time of submission of the WID to TSG approval. It can later be changed without a need to revise the WID. The updated target Release is indicated in the Work Plan. NOTE: In case of contradiction with the target dates of clause 5, clause 5 determines the target release.

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  |  |  |
| No | X |  | X | X | X |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Feature |
| --- | --- |
| X | Building Block |
|  | Work Task |
|  | Study Item |

NOTE:	Normally, Core/Perf./Testing parts in RAN WIDs are Building Blocks. Only if they are under an SA or CT umbrella, we define them as work tasks. If you are in doubt, please contact MCC.

2.2	Parent and child Work Items

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| DC_R17_xBLTE_3BNR_yDL2UL | RAN4 | 881004 | Rel-17 Dual Connectivity (DC) of x bands (x=1,2,3) LTE inter-band CA (xDL/1UL) and 3 bands NR inter-band CA (3DL/1UL) |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one WID. Therefore the table above should just include the feature WI Unique ID and title and Nature of relationship is "parent WID".

2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one but are not part of the hierarchical structure, such as preceding SI or a preceding WI (e.g. if you further enhance a topic).}

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 881104 | Core part: Rel-17 Dual Connectivity (DC) of x bands (x=1,2,3) LTE inter-band CA (xDL/1UL) and 3 bands NR inter-band CA (3DL/1UL) | Child WID |
| 881204 | Perf. part: Rel-17 Dual Connectivity (DC) of x bands (x=1,2,3) LTE inter-band CA (xDL/1UL) and 3 bands NR inter-band CA (3DL/1UL) | Child WID |

NOTE:	Also related or dependent WIs in other TSGs should be indicated.

3	Justification

All new DC configurations including 3 bands NR inter band CA for both EN-DC and NE-DC will be defined under this WI. New configurations still emerge from exiting bands and whenever new band is specified, it will create a potential for several new DC of LTE CA for up to 3 different bands DL with 1 band UL + NR CA for 3 different bands DL with 1 band UL.

The DC configurations including 3 bands NR CA will be introduced in a release independent manner based on TS38.307, which will be updated depending on newly introduced DC configurations including NR CA.

The preconditions to propose DC of LTE CA for up to 3 different bands DL with 1 band UL + NR CA for 3 different bands DL with 1 band UL in rel-17 are as follows.

- Constituent NR inter band CA including intra band CA for 3 different bands DL with 1 band UL shall be completed and specified in advance.

- Constituent LTE inter band CA including intra band CA for up to 3 different bands DL with 1 band UL shall be completed and specified in advance.

- Each of the paired DC configurations of 3 different LTE band including intra band CA + 2 NR band used in a certain DC of LTE CA for up to 3 different bands DL with 1 band UL + NR CA for 3 bands DL with 1 band UL shall be completed and specified in advance.

Example 1: If the following configuration is proposed,

| DC Downlink configuration | DC Uplink configuration |
| --- | --- |
| DC_1A-2A_n3A-n4A-n5A | DC_1A_n3A, DC_1A_n4A, DC_1A_n5A, DC_2A_n3A, DC_2A_n4A, DC_2A_n5A |

- NR CA of 3 different bands DL with 1 band UL of CA_n3A-n4A-n5A shall be completed and specified in advance.

- LTE CA of 2 different bands DL with 1 band UL of CA_1A-2A shall be completed and specified in advance

- DC of DC_1A-2A_n3A-n4A, DC_1A-2A_n3A-n5A and DC_1A-2A_n4A-n5A shall be completed and specified in advance.

Example 2: If the following configuration is proposed,

| DC Downlink configuration | DC Uplink configuration |
| --- | --- |
| DC_1C-2A_n3C-n4A-n5A | DC_1C_n3C |

- NR CA of 3 different bands DL with 1 band UL of CA_n3C-n4A-n5A shall be completed and specified in advance.

- LTE CA of 2 different bands DL with 1 band UL of CA_1C-2A shall be completed and specified in advance

- DC of DC_1C-2A_n3C-n4A, DC_1C-2A_n3C-n5A and DC_1C-2A_n3A-n4A-n5A shall be completed and specified in advance.

4	Objective

4.1	Objective of SI or Core part WI or Testing part WI

- Specify the DC configurations listed in the tables below,  including at least

  - Applicable frequencies if necessary

  - Applicable bandwidths and bandwidth sets if necessary

- Specify the band-combination specific RF requirements for these DC configurations, i.e.

  - Analyse combinations that have self-desensitization due to following reasons:

    - TX Harmonic and/or intermodulation overlap of receive band

    - TX signal overlap of receiver harmonic frequency

    - TX frequency being in close proximity of one of the receive bands

    - Any other identified reasons such that insufficient cross band isolation, harmonic mixing

  - For the combination where self-desensitization exists, specify at least needed

    - ∆TIB, c and ∆RIB, c

    - Reference sensitivity exceptions including MSD test cases

of all REL-17 DC including EN-DC and NE-DC of LTE CA for up to 3 different bands DL with 1 band UL + NR CA for 3 different bands DL with 1 band UL that fall into the category is defined by the WI title.

An overview table of these EN-DC and NE-DC configurations are provided in the EXCEL file zipped together with WORD file WID.

Unless stated otherwise, the release independent for EN-DC and NE-DC configurations are from Rel-15.

4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

This Perf. Part WI has to standardize the Perf. Part requirements:

- Required changes to be added to release independence TS 38.307.

of all REL-17 DC combinations including EN-DC and NE-DC that fall into the category is defined by the WI title.

4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | Series | Title | For info at TSG# | For approval at TSG# | Remarks |
| Internal TR | 37.717-11-31 | DC of x bands (x=1,2,3) LTE inter-band CA (xDL/1UL) and 3 bands NR inter-band CA (3DL/1UL) | TSG#94 | TSG#95 | Wubin, Zhou, ZTE, Zhou.wubin@zte.com.cn |

NOTE:	If this is a RAN WID including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.101-3 | Add new DC band combinations and related RF core requirements | TSG#95 | Core part |
| 38.307 | Release independent manner will be applied to all new DC band combinations according to each DC band combination | TSG#95 | Perf. part |

NOTE:	If this is a RAN WID including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

6	Work item Rapporteur(s)

Wubin Zhou, ZTE,  zhou.wubin@zte.com.cn

7	Work item leadership

R4

8	Aspects that involve other WGs

None

NOTE:	For RAN WIDs: Section 8 applies only toWGs outside of TSG RAN because RAN WG aspects have to be covered in section 4.

9	Supporting Individual Members

Supporting IM name

CMCC

CATT

ZTE

Sanechips

T-Mobile

Xiaomi

CHTTL

SoftBank