Sophia Antipolis							                  GSMOB-03

Joint CEPT SE7, ETSI ERM TG25, ETSI MSG, EP RT,
 3GPP TSG RAN4, 3GPP TSG GERAN1 meeting on GSM on board aircrafts

September 9, 2005

- Agenda Item:	4

Source: 	QUALCOMM Europe S.A.R.L

Title: 	On interference aspects of an airborne GERAN system

Document for:	Discussion/information

- 1	Introduction

1.1	Background

QUALCOMM is an original member of the RTCA Special Committee 202 (SC-202) which was formed in May 2003 in the USA at the request of the Federal Aviation Administration (FAA) to investigate the potential for interference to avionics from passenger carried transmitting devices such as mobile phones.

Participation in SC-202 and interaction with other industry groups has led QUALCOMM to engage in a program of development, analysis, and testing to assess the potential for mobile phone interference with aircraft systems and terrestrial mobile phone networks.

Anticipating that in-cabin systems would operate in the same spectrum bands allocated to terrestrial service providers, one of the earliest tasks QUALCOMM pursued was to evaluate the interference effects airborne wireless networks may have on terrestrial wireless networks.

This paper presents a summary of the work performed in this area and is not intended to be definitive.

The need to do more analysis and test in order to fully assess the impact of the airborne picocell network to the ground networks is fully acknowledged. This analysis should be performed jointly with the terrestrial service providers (SP’s) as they are the owners of the spectrum and therefore best positioned to assess interference based on their ground network design and capacity/coverage planning.

1.2	Organization of the work

It is only through the actual system design of an aircraft cabin mobile phone network that a valid interference assessment can be made. When designing the cabin network using standard mobile phone technology such as GSM or WCDMA/CDMA, a number of design parameters need to be considered as follows:

- Propagation and fading margin within the aircraft cabin

  - What is the path loss exponent governing transmission between a picocell antenna and a passenger mobile?

  - Reflections within the cabin randomly arriving in phase and out-of-phase result in up and down fades

- Interference from terrestrial systems

  - Signals from multiple phones and base stations on the ground penetrate the aircraft cabin with varying amplitudes, depending on the altitude and position of the aircraft relative to the ground networks

- Aircraft penetration loss

    - The fuselage provides a level of shielding to prevent or limit cabin picocell signals coupling to the aircraft antennas (nav/comms systems) and the ground networks

- Multiple Equipment factor

  - Cumulative effects of having simultaneous users of mobile phones in the aircraft cabin

- Multiple aircraft factor

  - The system design needs to satisfy conditions for terrestrial interference when there are multiple aircraft “in view” of terrestrial networks

QUALCOMM has performed a number of airborne and ground based tests using private and commercial aircraft to establish values for the design parameters defined above. These parameters were then used in RF link budget analysis in order to establish the interference threat to terrestrial networks.

The interference scenarios addressed in this paper are based on a GSM airborne network which also includes a Network Control Unit (NCU) whose function is to prevent non- authorized devices from connecting to the terrestrial networks.

RF Link budgets were developed for the case whereby a GSM airborne picocell is over-flying terrestrial GSM and WCDMA commercial networks, namely:

- Forward Link Analysis

  - Airborne System to Ground

    - GSM Picocell + NCU (w/Leaky Coax) to GSM Mobile on Ground

    - GSM Picocell + NCU (w/Leaky Coax) to WCDMA Mobile on Ground

  - NCU to Ground

    - WCDMA NCU (w/Leaky Coax) to GSM Mobile on Ground

    - WCDMA NCU (w/Leaky Coax) to WCDMA Mobile on Ground

- Reverse Link Analysis

  - On board GSM Mobile to GSM BTS on Ground

  - On board GSM Mobile to WCDMA BTS on Ground

RF Link Budgets are presented in Section 3.

1.3	Other general aspects

One of the key parameters which define the terrestrial interference levels is the aircraft shielding and this continues to be a heavily debated topic that requires further research and definition. In our test program we determined this value to be less than 2 dB and so for the purposes of the analysis we used 0dB shielding.

Using this value in the RF link budget analysis indicates that airborne GSM networks will present interference to the terrestrial networks with a correspondent desensitization of the ground based receivers.

It is QUALCOMM’s interpretation that it will be up to the existing terrestrial service providers to determine which level of interference they are willing to accept in return for the revenue generating opportunities an airborne mobile phone service would bring.

- 2	Airborne Network System Design

### 2.1	Airborne Network High Level Requirements

- Provide voice, SMS and data services to passenger devices using standardized mobile phone technology

  - Proposed in-cabin mobile phone and data module networks will allow passengers to use their own devices in flight

  - Such a design necessitates operation in spectrum licensed to and used by terrestrial mobile networks and phones

- The system must not produce harmful interference to aircraft systems

  - Limit intentional and out-of-band emissions below susceptibility threshold for aircraft systems

- The system must not produce harmful interference to terrestrial networks

  - Limit intentional transmissions below the susceptibility threshold for terrestrial mobile phone networks

  - Any elevation in the ambient noise level results in degraded voice quality, data performance, and system capacity

### System Design Constraints

### An airborne mobile phone network is subject to the following three fundamental design constraints for both the forward and reverse links:

- Total transmitted power (picocell or combined phone EIRP) inside the cabin must not cause significant degradation to terrestrial mobile and base station receivers

- Transmitted powers inside the cabin must be sufficient to overcome interference from terrestrial networks and phones

- In-Band and Out-of-band emissions from in-cabin picocell and mobile phones must not interfere with avionics equipment

The first constraint above represents an upper bound at which an in-cabin picocell and mobile phone devices can transmit.

The second constraint above represents a lower bound at which an in-cabin picocell and mobile phone devices may transmit.

With respect to passenger handsets, the third constraint above is in the U.S. the subject of ongoing work by RTCA Special Committee 202, in which QUALCOMM is an active participant. The guidelines published by SC-202 will permit airlines and aircraft manufacturers to evaluate objectively whether an installed in-flight system meets this requirement. Furthermore, the picocell system and air-to-ground backhaul equipment is subject to FAA certification, since it is installed permanently onboard the aircraft.

### 2.2.1	Forward Link Design Constraints

For the airborne picocell generated interference (forward link) there is a threshold for susceptibility that needs to be defined for the terrestrial mobiles.

The terrestrial network susceptibility threshold is being debated in the standards bodies and with the terrestrial carriers and to date there is no consensus on what this threshold is for a given technology.

Therefore, for the purpose of analysis, we have selected the following arbitrary definition for susceptibility to terrestrial mobiles:

- Total Picocell EIRP must not de-sense terrestrial mobiles by >1dB

  - Cumulative power from all aircraft in vicinity of mobile

  - Limit of -121.9 dBm (per 200 kHz) for GSM

    - Assuming receiver noise floor = Thermal + RX Noise Figure

Further, as the aircraft cabin is illuminated with interference signals coming from the ground networks it is necessary to set the airborne picocell traffic channel power to a sufficiently high level to overcome interference from terrestrial forward links (GSM, WCDMA + other noise in the band) and close the link with the cabin located mobiles.

In order to satisfy the two constraints defined above there are only a limited number of relevant parameters that can be adjusted in an airborne network design that is based on existing mobile phone technology and installed in legacy commercial aircraft.

- Max Total Picocell EIRP

- Path loss between aircraft cabin located Picocell and mobiles (antenna design and placement)

- Path loss to the ground networks (altitude at which system can operate plus additional cabin shielding e.g. window treatments)

### Reverse Link Design Constraints

### For the airborne mobile phone interference (reverse link) there are similar constraints.

The susceptibility threshold for the terrestrial base station was also selected as an arbitrary value for the purpose of the analysis and defined as:

- Combined TX power of all mobiles on all aircraft must not de-sense terrestrial base stations by >1dB

    - Limit of -121.9 dBm (per 200 kHz) for GSM

      - Assuming receiver noise floor = Thermal + RX Noise Figure

As the aircraft cabin is illuminated with interference signals coming from the ground mobile phones the airborne mobile phone EIRP must be sufficient to overcome interference from terrestrial mobiles (GSM, WCDMA + other noise in the band)

In order to satisfy the two constraints defined above there is a limited number of relevant parameters that can be adjusted in an airborne network design that is based on existing mobile phone technology and installed in legacy commercial aircraft.

- Path loss between aircraft cabin located Picocell and mobiles (antenna design and placement)

- Path loss to the ground networks (altitude at which system can operate plus additional cabin shielding e.g. window treatments)

- Use of BTS messaging to limit power level of access burst and traffic channel to 0 dBm minimum TX power

### Network Control Unit (NCU) / Noise Floor Lifter

The system design outlined by a number of companies in various occasions includes a proposal for a device that controls non-authorized mobile phone devices on board an aircraft configured with a picocell network. The device is being referred to as a “noise floor lifter” or “network control unit (NCU)”.

This device effectively introduces noise into the receive bands of the on-board mobiles thereby preventing mobiles on board from accessing terrestrial networks. This increases the probability of either acquiring the onboard system or remaining in an idle state if the mobiles on board are not authorized for on board use.

QUALCOMM has included the interference effects of this unit in the RF link budget analysis.

- 3	RF Link Budgets

### 3.1	Link Budget Parameters

- With reference to the first and second constraints defined in Section 2, we have constructed link budgets for a hypothetical in-cabin network.

- Relevant system parameters for the link budget were established either through on-aircraft test programs conducted by QUALCOMM or are assumed:

- Network operates in 1800MHz GSM Band

- Aircraft Altitude = 3000 m AGL

- Multiple Equipment Factor (MEF) = 0 dB (7 simultaneous GSM calls)

- Interference from ground: -88.9 dBm/200kHz from BTSs; -97.9 dBm/200kHz from phones

  - Based on 90th percentile of QC in-flight measurements taken on multiple flights

- “Multiple Aircraft Factor” = 0 dB

  - No accounting for multiple aircraft interfering with ground

- In-cabin fade margin = 20 dB

  - Derived from on-aircraft testing and post-processing for 1.25MHz bandwidth

  - Fades for 200 kHz bandwidth expected to be greater.

  - Required to account for multipath fades

  - On-aircraft measurements show similar fade depths for patch antennas and leaky coax

- Aircraft Penetration Loss = 0 dB (aircraft provides zero RF shielding based on actual measurements taken on commercial aircraft)

- All antenna gains average out to isotropic

### 3.2	Interference Scenarios

The interference scenarios addressed in this paper are based on GSM airborne service and includes the device referred to by many different names such Network Control Unit (NCU), Noise Floor Lifter, Electromagnetic Screening Device (EMSD), etc (see Section 2.3).

The function of this device is to prevent airborne mobile phones from acquiring the terrestrial networks.

RF Link budgets were developed for the case whereby a GSM airborne picocell is over-flying terrestrial GSM and WCDMA commercial networks.

In these cases the NCU device:

- Is blocking signals from terrestrial GSM base stations on all frequencies other than the picocell channel

- Is also blocking WCDMA signals from terrestrial networks operating in various bands.

The interference link budgets presented in this paper are as follows:

- Forward Link Analysis -- GSM Picocell + NCU (w/Leaky Coax) to GSM Mobile on Ground

- Forward Link Analysis -- GSM Picocell + NCU (w/Leaky Coax) to WCDMA Mobile on Ground

- Forward Link Analysis -- WCDMA NCU (w/Leaky Coax) to GSM Mobile on Ground

- Forward Link Analysis -- WCDMA NCU (w/Leaky Coax) to WCDMA Mobile on Ground

- Reverse Link Analysis – On board GSM Mobile to GSM BTS on Ground

- Reverse Link Analysis -- On board GSM Mobile to WCDMA BTS on Ground

### Forward Link Analysis

### 3.3.1	Forward link Analysis for GSM Picocell + GSM Terrestrial Signal NCU

Tables 3.1 and 3.2 present the forward link analysis for interference from the onboard system to GSM and WCDMA terrestrial networks, respectively.

The analysis for each technology is presented in three parts as defined in the following paragraphs.

### Forward Link Budget for Interference to Ground

This part of the analysis presents the forward link budget for a hypothetical in-cabin network system design with the constraint of 1 dB desense to terrestrial phones (which is an input to the link budget).

The final product in this analysis (and output of the link budget) is the path loss margin/deficit within the cabin.

### 3.3.1.2	Forward Link Budget for Minimum Link to In-Cabin Mobile

This part of the analysis presents the forward link budget for a system design with the constraint of providing 9 dB GSM SNR throughout the cabin (which is an input to the link budget).

The final product (and output of the link budget) is the resulting desense to a terrestrial phone.

3.3.1.3	Forward Link Budget for In-Cabin NCU

- This part of the analysis presents the forward link budget for a system design that includes an NCU to reduce SNR from terrestrial GSM BTSs to 5 dB. This reduction is an input to the link budget.    The NCU signal is assumed to be carried in the same leaky coax as the picocell signals.  It is also assumed to have the picocell frequency notched out.

- The final product is the resulting desense to a terrestrial phone.

    - Forward Link Analysis for WCDMA Terrestrial Signal NCU

Tables 3.3 and 3.4 present the forward link analysis for interference to GSM and WCDMA terrestrial networks.

This interference is due to the power being transmitted by the airborne NCU device attempting to prevent any dual mode (GSM/WCDMA) devices on board the aircraft from acquiring the terrestrial 2.1 GHz WCDMA networks.

In this case, the NCU values are based on preventing acquisition of the UMTS CPICH (Common Pilot Channel).  This channel is typically transmitted at 5-10% of the total BTS power.  A power fraction of -10 dB is assumed for this analysis.  Acquisition of the CPICH requires an Ec/No in the range of -16 to -20 dB. For this analysis, we assume -20 dB Ec/No is required to block acquisition of the CPICH.

- Given those NCU values, the final product is the resulting desense to a terrestrial network, for both the GSM and WCDMA cases.

Table 3.1 Forward Link Budget for Hypothetical In-Cabin Network - GSM to GSM

- Table 3.2 Forward Link Budget for Hypothetical In-Cabin Network - GSM to WCDMA

- Table 3.3 Forward Link Budget for Hypothetical In-Cabin Network – WCDMA NCU to WCDMA Mobile

- Table 3.4 Forward Link Budget for Hypothetical In-Cabin Network– WCDMA NCU to GSM Mobile

### 3.4 	Reverse Link Analysis

### Reverse Link Budget for Airborne GSM Mobile to Terrestrial GSM BTS

Table 3.5 presents the reverse link budget for a hypothetical in-cabin system design with the constraint of 1 dB desense to terrestrial base stations.

This link budget highlights the conflict between the mobile phone transmit power required to protect the terrestrial GSM network and the phone transmit power required to provide connectivity within the aircraft cabin. This is showed in the first two sets of calculations.

Further, the third set of calculations shows how applying the minimum achievable transmit power for a GSM mobile phone of 0dBm results in a GSM BTS receiver desense of 10.5 dB.

- Table 3.5 Reverse Link Budget – Airborne GSM Mobile to Terrestrial GSM BTS

### Reverse Link Budget for Airborne GSM Mobile to Terrestrial WCDMA Node B

Table 3.6 presents the reverse link budget for a hypothetical in-cabin system design with the constraint of 1 dB desense to terrestrial base stations.

The final product in this analysis is highlighting the conflict between the mobile phone transmit power required to protect the terrestrial GSM network and the phone transmit power required to provide connectivity within the aircraft cabin.

Further, the third set of calculations shows how applying the minimum achievable transmit power for a GSM mobile phone of 0dBm results in a WCDMA Node B receiver desense of  1.5 dB.

- Table 3.6 Reverse Link Budget – Airborne GSM Mobile to Terrestrial WCDMA Node B

- 4	Observations

### Observations on Forward Link Analysis

### The following observations were made for the Forward Link Analysis:

- For a narrowband interferer (GSM picocell), susceptibility of ground receivers decreases with higher victim receiver bandwidth. So a WCDMA phone on the ground is less susceptible than a GSM phone.  Wider bandwidth means higher noise floor, which means higher interference power required to desense by 1dB.  In other words, for the same interference source, interference PSD decreases with increasing bandwidth.

- For a wideband interferer (NCU), relative susceptibility of ground receivers is independent of receiver bandwidth.  The wider-band interferer has an equal effect on interference PSD, as long as the receiver bandwidth is < or = to the interferer.

- For a GSM airborne network with NCU intended to prevent acquisition of GSM terrestrial networks, the interference generated by the NCU is less than the GSM picocell forward link. The NCU can operate 5 dB below the ambient ground interference, where the picocell has to operate 9 dB above it.  Consequently, there is always a 14 dB difference between the required picocell power and the required NCU power

- For GSM airborne picocell interference to terrestrial GSM mobiles, approximately 8 dB of aircraft shielding is required to comply with the 1 dB desense susceptibility threshold assumed for the terrestrial victim receiver in this analysis.

- For a GSM airborne picocell to terrestrial WCDMA mobile the analysis indicates compatibility with terrestrial network 1dB desense threshold with approximately 5 dB of margin.  This does not take into account the multiple aircraft factor and so this margin may not be realizable in a commercial service with multiple aircraft.

- If the GSM airborne system offers multiple carriers then the multiple equipment factor (MEF) must be considered especially for terrestrial WCDMA phone interference assessment.

4.2	Observations on Reverse Link Analysis

As with the forward link, desense decreases with the victim bandwidth.

That said, a single GSM phone with no penetration loss will elevate a WCDMA Node B noise floor by 1.5 dB, and 10.5 dB for a GSM BTS.

For both forward link and reverse link analysis we have assumed that the distance to the terrestrial victim is equal to the aircraft altitude and this may not be realistic when you consider the leakage pattern associated with the aircraft fuselage. Further work is required in this area but initial measurements suggests that the victim receivers are not directly below the aircraft but at some distance away where the side of the aircraft fuselage appears at a low elevation angel relative to the victim receiver.

### Other Considerations

This paper has not explicitly taken into account the implementation of advanced receivers in the mobile station, which may impact the analysis of the effect of the NCU on the terminals.

Essentially, the presence of an advanced receiver among the terminals on board of the aircraft means that those mobiles will potentially become more tolerant to the NCU. This will create the risk that the NCU will have to operate at higher levels in order to prevent those terminals from accessing the terrestrial network.

At the time of writing one class of such receivers has been specified, and another class has been proposed and is currently undergoing evaluation by 3GPP GERAN WG1.

The former class consists of single antenna interference cancellation (SAIC) terminals. This indicates terminals offering enhanced performance in a number of interference limited scenarios (mostly where interference consists of GMSK-modulated waveforms). The details of the NCU waveform are not completely known, thus it is difficult to conclude whether SAIC terminals will be more tolerant to the NCU or not. If the NCU waveform was simply white-noise we could conclude that SAIC terminals have no effect.

The latter class consists of receive diversity terminals. This indicates terminals offering enhanced performance in both sensitivity (noise)-limited scenarios and in interference-limited scenarios (irrespective of the interfering waveform). Since such terminals will provide better receiver capabilities in all cases, they will also become more tolerant to the NCU. Thus, in the link budgets of Table 2.1 and 2.2, the "jamming SNR" will likely have to be corrected from 5dB to a lower figure. This in turn means that the NCU will have to operate at a higher output power thus causing a higher receiver desense to the terrestrial phones.

In other words, the presence on board of receive diversity phones (or, more in general, of advanced receivers) will only worsen the effect of the NCU on terrestrial phones.

## Appendix A - Leaky Coax Transmission Properties

## Leaky coaxial cable is used frequently as a wireless antenna system for indoor or vehicular tunnel applications. In situations such as these where the distance between mobile users and the antenna is relatively constrained, leaky coax has the advantage of providing a more uniform distribution of coverage than fixed point antennas. This is accomplished by covering relatively large distances by conducted rather than radiated transmission. The principal disadvantages of leaky coax compared to point source antennas are cost and ease of installation. Compared to traditional antennas, leaky coax provides no advantage in terms of multipath fading mitigation.

## Principal technical specifications for leaky coaxial cable are the coupling loss and the longitudinal attenuation. The coupling loss describes the loss of RF power between the input to the feeder line and a point at a radial reference distance from the cable. The longitudinal attenuation describes the spatial rate of RF loss along the length of the cable.

## For the purpose of in-cabin system design utilizing leaky coax, the RF propagation characteristics of the transmission system must be considered over two disparate distance scales: 1) transmission within the aircraft cabin, where D (distance from the cable) << L (length of the cable); and 2) interference transmission to the ground, where D >> L. The radiation characteristics must be treated differently for these two cases.

## Short-Range Propagation

## Morgan [1] shows that for the case of D<<L (where L can be assumed to be infinite from the receiver’s point of view), the total received power by a half wave dipole is

where Π is the power radiated per unit length of the radiating cable.

Assume a manufacturer-specified coupling loss C = Pin/P(Do), where Pin is the input power into the coax, and P(Do) is the received power at the coupling loss reference distance Do. Substituting yields a constant expression for Π:

It follows that for D<<L, the total loss Ctotal from cable input to measurement point, obeys a simple linear relationship:

Total loss (close-range propagation)

Long-Range Propagation

For the air-to-ground interference path, where D>>L, the coax system can be treated as a point source with EIRP = Π·L. Assuming free space propagation, it follows that power received from an isotropic antenna on the ground is:

Substituting for Π as before, we find that total loss Ctotal, from cable input to measurement point can be expressed as:

Total loss (long-range propagation)

In the included link budgets, these two derived expressions for Ctotal are used for the in-cabin and air-to-ground links respectively.

Reference

1. S.P. Morgan, “Prediction of Indoor Wireless Coverage by Leaky Coaxial Cable Using Ray Tracing,” IEEE Trans Veh. Tech., vol. 48(6), pp. 2005-2014, Nov. 1999.

The fade margin for a link budget is typically determined according to an acceptable outage probability, based on the statistics of the propagation environment and the transmission characteristics.  For example, it is common to compute the fade margin as the dB power loss that is exceeded 1% of the time.  Among other factors, the fade margin depends on the delay spread of the propagation environment and the bandwidth of the transmitted signal.  Smaller delay spreads result in the channel fading being less frequency selective.  A larger transmission bandwidth results in frequency diversity (and a lower fade margin) because it is less likely that the received power integrated over the transmission bandwidth will fade down by a given amount.  The fade margins for GSM may be larger than cdma2000 and WCDMA because the GSM transmission bandwidth is smaller than the bandwidths of cdma2000 and WCDMA.