3GPP TSG-CT Meeting #105	CP-242024

Melbourne, Australia; 9th – 10th September 2024	(revision of CP-24abcd)

# Source:	CT4

# Title:	New WID on CT Aspects of Indirect Network Sharing (TEI19_NetShare)

# Document for:	Approval

# Agenda Item:	19.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	CT Aspects of Indirect Network Sharing

Acronym:	TEI19_NetShare

Unique identifier:	 1050004

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
| X | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| TEI19_NetShare | SA2 | 1030011 | Indirect Network Sharing |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

Dependency on non-3GPP (draft) specification: None

# 3	Justification

SA#103 has agreed New 3GPP Rel-19 WID on Indirect Network Sharing (TEI19_NetShare, UI: 1030011), see SP -240488.

In SA WG2, the new network sharing mechanism called as “Indirect Network Sharing” has been studied and specified for operators who intend to use the network sharing mechanism to complement the existing network, taking operators’ business considerations into account.

SA#104 has approved related CRs on TS 23.501 and TS 23.502 as stage 2 normative works. And the TEI19 WI has been completed after SA#104.

The outcome of stage 2 WI is stable to initiate the stage 3 normative implementations.

# 4	Objective

The objective of this CT WID is to implement the outcome of the Stage 2 TEI19_NetShare WID. Below is an outline of implications on CT specifications.

CT1 Impacts:

- Potential clarification on the provision of the list of equivalent PLMNs in the case of Indirect Network Sharing.

- Potential clarification on the network handling and the use of existing cause value(s) for the case that the UE which is not belonging to participating operator tries to access the shared network in the case of Indirect Network Sharing.

- Potentially update the network slicing handling at the network during registration procedure, UE configuration update procedure and PDU session establishment procedure in the case of Indirect Network Sharing.

CT4 Impacts:

- NSSF interface:

  - Update the description of the Nnssf_NSSelection Service to support the network slicing handling in the case of Indirect Network Sharing.

- NRF interface:

  - Update the description of the Nnrf_NFManagement Service and the Nnrf_NFDiscovery Service to support the H-SMF discovery and selection in the case of Indirect Network Sharing.

  - Support the Indirect Network Sharing case in N27 interface.

- SMF interface:

  - Support the Indirect Network Sharing case in the Nsmf_PDUSession Service.

  - Support the Indirect Network Sharing case in N16 interface.

CT4 Potential Impacts:

- AUSF interface:

  - Clarify the serving network name used in the Nausf_UEAuthentication Service in the case of Indirect Network Sharing.

- UDM interface：

  - Clarify the serving network name used in the Nudm_UEAuthentication Service in the case of Indirect Network Sharing.

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 24.501 | Potential clarification on the network handling on the provision of the list of equivalent PLMNs, the use of existing cause value(s) and the network slicing handling in the case of Indirect Network Sharing. | TSG#109 (Sept 2025) | CT1 |
| TS 29.531 | Update the description of the Nnssf_NSSelection Service to support the network slicing handling in the case of Indirect Network Sharing. | TSG#109 (Sept 2025) | CT4 |
| TS 29.510 | Update the description of the Nnrf_NFManagement Service and the Nnrf_NFDiscovery Service to support the H-SMF discovery and selection in the case of Indirect Network Sharing. Support the Indirect Network Sharing case in N27 interface. | TSG#109 (Sept 2025) | CT4 |
| TS 29.502 | Support the Indirect Network Sharing case in the Nsmf_PDUSession Service. Support the Indirect Network Sharing case in N16 interface. | TSG#109 (Sept 2025) | CT4 |
| TS 29.509 | Clarify the serving network name used in the Nausf_UEAuthentication Service in the case of Indirect Network Sharing. | TSG#109 (Sept 2025) | CT4 |
| TS 29.503 | Clarify the serving network name used in the Nudm_UEAuthentication Service in the case of Indirect Network Sharing. | TSG#109 (Sept 2025) | CT4 |

# 6	Work item Rapporteur(s)

Xing Tianqi, China Unicom, xingtq@chinaunicom.cn

# 7	Work item leadership

CT4

# 8	Aspects that involve other WGs

None

# 9	Supporting Individual Members

Supporting IM name

China Unicom

Huawei

ZTE

China Telecom

Tencent

CATT

vivo

China Mobile

Ericsson

Samsung

Nokia

Oracle

OPPO

Lenovo