3GPP TSG-CT3 Meeting #136	C3-244402

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024				(revision of C3-244339)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.558 | CR | 0211 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Out of service area ACR management event |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Vodafone, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph3 |  |  |  |  |  | Date: |  |  | 2024-08-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per the agreed CR#S6-242639 the requirement is to allow an EES to use EAS service area to monitor UE location for ACR and indicate that the NEF monitoring event API for UE mobility can be used for ACR. Hence an "out of service area" ACR management event is added so that an EAS or EES can instigate ACR if a UE moved into or out of a service area. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR prposes to introduce new ACR management subscription event for out of service area and related new event report notification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage-2 requirement on moved into or out of service area event reporting is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.5.1, 8.6.5.2.3, 8.6.5.2.6, 8.6.5.3.3, 8.6.5.3.6, 8.6.5.3.9(new), 8.6.7, A.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR provides backward compatible feature updates to the open API Eees_ACRManagementEvent API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First Change * * * *

#### 8.6.5.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 7.2 apply to this API

Table 8.6.5.1-1 specifies the data types defined specifically for the Eees_ACRManagementEvent API service.

Table 8.6.5.1-1: Eees_ACRManagementEvent API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| AcrMgntEventFailureCode | 8.6.5.3.6 | Represents the reason for ACR Management subscription failure for an event. |  |
| AcrMgntEventFilter | 8.6.5.3.4 | Represents the ACR Management Event filter. |  |
| AcrMgntEventsSubscription | 8.6.5.2.2 |  |  |
| AcrMgntEventSubsc | 8.6.5.2.3 |  |  |
| AcrMgntEventsSubscriptionPatch | 8.6.5.2.4 |  |  |
| AcrMgntEventsNotification | 8.65.2.5 |  |  |
| AcrMgntEventReport | 8.6.5.2.6 |  |  |
| ACRParameters | 8.6.5.2.13 | Represents ACR parameters. | EdgeApp_2 |
| ActStatus | 8.6.5.3.5 | Represents ACT status, i.e. ACT start or stop. |  |
| AvailabilityNotif | 8.6.5.2.11 | Represents the availability information of user plane path management events monitoring via the 3GPP 5GC network. |  |
| AvailabilityStatus | 8.6.5.3.7 | Represents the availability status. |  |
| EasAckInformation | 8.6.5.2.15 | Represents the acknowledgement information from EAS in response to ACR notifications. | EdgeApp_2 |
| EasInBundleInfo | 8.6.5.2.16 | Represents EAS in a bundle information. | EdgeApp_2 |
| FailureAcrMgntEventInfo | 8.6.5.2.7 |  |  |
| AcrMgntEvent | 8.6.5.3.3 |  |  |
| IndtUeIdentification | 8.6.5.2.10 | Contains individual UE identification information. |  |
| ResultCode | 8.6.5.3.8 | Contains the EAS acknowledgement response message. | EdgeApp_2 |
| SelectedACRScenarios | 8.6.5.2.12 | Represents the selected ACR scenario(s) applicable for a given combination of AC and UE. | EdgeApp_2 |
| TargetUeIdentification | 8.6.5.2.8 | Contains target UE(s) identification information. |  |
| TrafficFilterInfo | 8.6.5.2.14 | Represents the traffic filter information. | EdgeApp_2 |
| InOutArea | 8.6.5.3.9 | Contains UE information on whether the UE moved into or out of a service area | EdgeApp_3 |
| UpPathChangeInfo | 8.6.5.2.9 | Contains user plane path change information. |  |

Table 8.6.5.1-2 specifies data types re-used by the Eees_ACRManagementEvent API service.

Table 8.6.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ACRScenario | Clause 9.1.5.3.3 | Represent the ACR scenarios. | EdgeApp_2 |
| EasCharacteristics | 3GPP TS 24.558 [14] | Represents the EAS characteristics. |  |
| DateTime | 3GPP TS 29.122 [6] |  |  |
| Dnai | 3GPP TS 29.571 [8] | Identifies a DNAI. |  |
| DnaiChangeType | 3GPP TS 29.571 [8] |  |  |
| DnaiChangeTypeRm | 3GPP TS 29.571 [8] |  |  |
| DomainNameProtocol | 3GPP TS 29.122 [6] | Represents the additional protocol and protocol field for domain names to be matched. | EdgeApp_2 |
| Endpoint | Clause 8.1.5.2.5 | Represents the endpoint information. |  |
| ExternalGroupId | 3GPP TS 29.571 [8] | Represents an external group of UEs. |  |
| ExternalId | 3GPP TS 29.122 [6] | Represents an external identifier of a UE. |  |
| Gpsi | 3GPP TS 29.571 [8] |  |  |
| GroupId | 3GPP TS 29.571 [8] | Represents an internal group of UEs. |  |
| IpAddr | 3GPP TS 29.571 [8] |  |  |
| Ipv4Addr | 3GPP TS 29.122 [6] | Identifying the IPv4 address of the UE. |  |
| Ipv6Prefix | 3GPP TS 29.571 [8] | Identifies an IPv6 Prefix. |  |
| ReportingInformation | 3GPP TS 29.523 [13] |  |  |
| RouteToLocation | 3GPP TS 29.571 [8] | Describes the traffic routes to the locations of the application. |  |
| SupportedFeatures | 3GPP TS 29.571 [8] | Used to negotiate the applicability of optional features. |  |
| ServiceArea | Clause 9.1.5.2.5 | Represents the topological and geographic areas. | EdgeApp_2 |
| TestNotification | 3GPP TS 29.122 [6] | This type represents a notification that can be sent to test whether a chosen notification mechanism works |  |
| Uri | 3GPP TS 29.122 [6] |  |  |
| WebsockNotifConfig | 3GPP TS 29.122 [6] | This type represents configuration for the delivery of notifications over Websockets. |  |

# * * * * 2nd Change * * * *

##### 8.6.5.2.3	Type: AcrMgntEventSubsc

Table 8.6.5.2.3-1: Definition of type AcrMgntEventSubsc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| event | AcrMgntEvent | M | 1 | Indicates the subscribed ACR management event. |  |
| eventFilter | AcrMgntEventFilter | O | 0..1 | Represents the event filter for the subscribed ACR management event. This attribute may be provided only if the "event" attribute is set to "ACR_MONITORING". |  |
| evtReq | ReportingInformation | O | 0..1 | Indicates the event reporting requirements (e.g. reporting periodicity, immediate reporting indication, etc). (NOTE 1, NOTE 2) |  |
| tgtUeId | TargetUeIdentification | C | 0..1 | Contains the UE identification information. This attribute shall be provided only if the "event" attribute is set to either "UP_PATH_CHG", "OUT_OF_SERVICE_AREA" (if the "EdgeApp_3" feature is supported), "ACR_MONITORING" or "ACR_FACILITATION". |  |
| dnaiChgType | DnaiChangeType | O | 0..1 | Identifies a type of notification regarding UP path management event. This attribute may be provided only if the "event" attribute is set to "UP_PATH_CHG". |  |
| easAckInd | boolean | O | 0..1 | Identifies whether EAS acknowledgement of UP path change event notifications is to be expected. Set to "true" if the EAS acknowledgement is expected. Set to "false" if the EAS acknowledgement is not expected. Default value is "false". This attribute may be provided only if the "event" attribute is set to "UP_PATH_CHG". |  |
| easChars | array(EasCharacteristics) | O | 1..N | Represents a list of EAS characteristics. This attribute may be provided only if the "event" attribute is set to either "ACR_MONITORING" or "ACR_FACILITATION". |  |
| trafFilterInfo | TrafficFilterInfo | O | 0..1 | Represents the traffic filter information. This attribute may be present only if the "event" attribute is set to "UP_PATH_CHG", "ACR_MONITORING" and/or "ACR_FACILITATION". | EdgeApp_2 |
| servContPlanInd | boolean | O | 0..1 | Contains the service continuity planning indication (i.e., whether or not the EES shall monitor whether the UE(s) enter the predicted location). - When set to "true", it indicates that service continuity planning is required. - When set to "false", it indicates that service continuity planning is not required. - The default value when this attribute is omitted is false. This attribute may be present only when the "event" attribute is set to "ACR_MONITORING" or "ACR_FACILITATION". | EdgeApp_2 |
| easAckSvcCont | boolean | O | 0..1 | Indicates Whether the EAS will provide an acknowledgement as a response to the notification of ACR management notification related to service continuity planning. - When set to "true", it indicates that the EAS acknowledgement is expected. - When set to "false", it indicates the EAS acknowledgement is not expected. - The default value is "false", when this attribute is omitted. This attribute may be present only if the "event" attribute is set to "ACR_MONITORING" or "ACR_FACILITATION". | EdgeApp_2 |
| NOTE 1: The "evtReq" takes precedence over the "evtReq" attribute of the AcrMgntEventsSubscription data structure when both are present. NOTE 2: This attribute shall not be present when the "event" attribute is set to "ACT_START_STOP" or "ACR_SELECTION". |  |  |  |  |  |

# * * * * 3rd Change * * * *

##### 8.6.5.2.6	Type: AcrMgntEventReport

Table 8.6.5.2.6-1: Definition of type AcrMgntEventReport

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| event | AcrMgntEvent | M | 1 | Indicates the subscribed ACR management event. |  |
| timeStamp | DateTime | O | 0..1 | Represents the time stamp of the detected event. |  |
| easEndPoint | EndPoint | C | 0..1 | Represents the endpoint information of the EAS. This attribute shall be provided either: - when the "event" attribute is set to either "ACR_MONITORING" or "ACR_FACILITATION". In such case, it shall contain the endpoint information of the T-EAS; or - when the "event" attribute is set to "ACT_START_STOP". In such case, it shall contain the endpoint information of the EAS to/from which the ACT needs to be started/stopped. |  |
| actStatus | ActStatus | C | 0..1 | Represents the reported ACT status, i.e. ACT start or ACT stop. This attribute shall be provided only when the "event" attribute is set to "ACT_START_STOP". |  |
| acId | string | C | 0..1 | Represents the identifier of the concerned AC. This attribute shall be present only when the "event" attribute is set to "ACT_START_STOP", or when the "EdgeApp_2" feature is supported, "ACR_SELECTION". |  |
| ueid | TargetUeIdentification | C | 0..1 | Represents the identifier of the UE. This attribute shall be present only when the "event" attribute is set to "ACT_START_STOP"" or "ACR_SELECTION". (NOTE) | EdgeApp_2 |
| selACRScen | array(SelectedACRScenarios) | C | 1..N | Represents the selected ACR scenario(s) for a given combination of AC and UE. This attribute shall be provided only when the "event" attribute is set to "ACR_SELECTION". | EdgeApp_2 |
| easInBdlInfoList | array(EasInBundleInfo) | C | 1..N | Represents the list of EAS in a bundle related information. This attribute shall be provided only when the "event" attribute is set to "ACR_SELECTION". | EdgeApp_2 |
| acrParams | ACRParameters | O | 0..1 | Represents the ACR Parameters. This attribute may be present only when the "actStatus" attribute is present and set to "ACT_START". | EdgeApp_2 |
| upPathChgInfo | UpPathChangeInfo | C | 0..1 | Represents the UP Path change information. This attribute shall be provided when the "event" attribute is set to "UP_PATH_CHG". |  |
| servContPlanInd | Boolean | O | 0..1 | Contains the service continuity planning indication (i.e., whether or not the EES shall monitor whether the UE(s) enter the predicted location or not). - When set to "true", it indicates that service continuity planning shall be performed. - When set to "false", it indicates that service continuity planning shall not be performed. - The default value when this attribute is omitted is false. This attribute may be present only when the "event" attribute is set to "ACT_START_STOP". | EdgeApp_2 |
| inOutOfServArea | InOutArea | C | 0..1 | Contains the information on whether the UE moved into or out of the service area. This attribute shall be present only when the "event" attribute is set to "OUT_OF_SERVICE_AREA". | EdgeApp_3 |
| NOTE: Only the GPSI, within the "gpsi" attribute, or the Edge UE ID, within the "edgeUeId" attribute, are applicable for this attribute as it shall convey the identifier of a single UE. |  |  |  |  |  |

# * * * * 4th Change * * * *

##### 8.6.5.3.3	Enumeration: AcrMgntEvent

Table 8.6.5.3.3-1: Enumeration AcrMgntEvent

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| UP_PATH_CHG | Indicates that the ACR Management Event is user plane path change. |  |
| ACR_MONITORING | Indicates that the ACR Management Event is ACR monitoring. |  |
| ACR_FACILITATION | Indicates that the ACR Management Event is ACR facilitation. |  |
| ACT_START_STOP | Indicates that the ACR Management Event is ACT start/stop. |  |
| ACR_SELECTION | Indicates that the ACR Management Event is ACR selection. | EdgeApp_2 |
| OUT_OF_SERVICE_AREA | Indicates that the ACR Management Event is out of service area. | EdgeApp_3 |

# * * * * 5th Change * * * *

##### 8.6.5.3.6	Enumeration: AcrMgntEventFailureCode

Table 8.6.5.3.6-1: Enumeration AcrMgntEventFailureCode

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| 3GPP_UP_PATH_CHANGE_MON_NOT_AVAILABLE | Indicates that the ACR Management Event Subscription failed because user plane path management event notifications from the 3GPP network is not available. This value is only applicable for the "UP_PATH_CHG", "ACR_MONITORING" and "ACR_FACILITATION" events. |  |
| 3GPP_SERVICE_AREA_MON_NOT_AVAILABLE | Indicates that the ACR Management Event Subscription failed because the area of interest location monitoring from the 3GPP network is not available. This value is only applicable for the "OUT_OF_SERVICE_AREA" event. |  |
| OTHER_REASONS | Indicates that the ACR Management Event Subscription failed for other reasons. This value is applicable for all events. |  |

# * * * * 6th Change * * * *

##### 8.6.5.3.9	Enumeration: InOutArea

Table 8.6.5.3.9-1: Enumeration InOutArea

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| MOVED_INTO_AREA | Indicates that the UE moved into the service area. |  |
| MOVED_OUT_OF_AREA | Indicates that the UE moved out of the service area. |  |

Editor's Note:	It is FFS whether any other value should be provided within this data type and whether also apply for periodic or immediate reporting.

# * * * * 7th Change * * * *

### 8.6.7	Feature negotiation

General feature negotiation procedures are defined in clause 7.8. Table 8.6.7-1 lists the supported features for Eees_ACRManagementEvent API.

Table 8.6.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | Notification_test_event | Testing of notification connection is supported according to clause 7.6. |
| 2 | Notification_websocket | The delivery of notifications over Websocket is supported according to clause 7.6. This feature requires that the Notification_test_event feature is also supported. |
| 3 | EdgeApp_2 | This feature indicates the support of the enhancements to the Edge Applications. Within this feature the following enhacements are covered: - Support of the "ACR Selection" event reporting. - Support of additional ACT Start/Stop event related information. - Support of the service continuity planning support indication. - Support of the EAS acknowledgement for service continuity planning indication. - Support of EAS bundle information reporting. - Support of the Edge UE ID as a possible UE ID. |
| 4 | EdgeApp_3 | This feature indicates the support of the enhancements to the Edge Applications. Within this feature the following enhancements are covered: - Support of the "OUT_OF_SERVICE_AREA" event reporting. |

# * * * * 8th Change * * * *

# A.7	Eees_ACRManagementEvent API

openapi: 3.0.0

info:

title: EES ACR Management Event_API

description: |

API for EES ACR Management Event.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: 1.1.0

externalDocs:

description: >

3GPP TS 29.558 V18.6.0 Enabling Edge Applications;

Application Programming Interface (API) specification; Stage 3

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.558/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/eees-acrmgntevent/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 7.5 of 3GPP TS 29.558.

paths:

/subscriptions:

post:

summary: Creates a new Individual ACR Management Events Subscription

operationId: CreateACRMngEventSubscr

tags:

- ACR Management Events Subscriptions (Collection)

description: Create an Individual ACR Management Event Subscription resource.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AcrMgntEventsSubscription'

callbacks:

ACRManagementEventsNotification:

'{$request.body#/notificationDestination}':

post:

requestBody:  # contents of the callback message

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AcrMgntEventsNotification'

responses:

'200':

description: OK (The notification is received successfully)

content:

application/json:

schema:

$ref: '#/components/schemas/EasAckInformation'

'204':

description: No Content (successful notification)

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

UPPathChangeAvailabilityNotif:

'{$request.body#/notificationDestination}/report-availability':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AvailabilityNotif'

responses:

'204':

description: No Content. The notification is successful received.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

responses:

'201':

description: Created (Successful creation)

content:

application/json:

schema:

$ref: '#/components/schemas/AcrMgntEventsSubscription'

headers:

Location:

description: 'Contains the URI of the newly created resource'

required: true

schema:

type: string

'204':

description: >

Successful case. The resource has been successfully created and no

additional content is to be sent in the response message.

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

get:

summary: Read all ACR Management Events Subscriptions

operationId: GetACRMngEventSubscrs

tags:

- ACR Management Events Subscriptions (Collection)

description: Retrieve all the ACR Management Events Subscriptions information.

parameters:

- name: supp-feat

in: query

description: Features supported by the EAS.

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: OK (Successful get all of the active subscriptions)

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/AcrMgntEventsSubscription'

minItems: 1

description: All the active ACR management events subscriptions

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

get:

summary: Read an Individual ACR Management Events Subscription

operationId: GetIndACRMngEventSubscr

tags:

- Individual ACR Management Events Subscription (Document)

description: Retrieve an Individual ACR Management Events Subscription.

parameters:

- name: subscriptionId

in: path

description: Subscription Id.

required: true

schema:

type: string

- name: supp-feat

in: query

description: Features supported by the EAS.

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: OK (Successful get the active subscription).

content:

application/json:

schema:

$ref: '#/components/schemas/AcrMgntEventsSubscription'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Update an Individual ACR Management Events Subscription

operationId: UpdateIndACRMngEventSubscr

tags:

- Individual ACR Management Events Subscription (Document)

description: Fully replace an existing Individual ACR Management Events Subscription.

parameters:

- name: subscriptionId

in: path

description: Subscription Id.

required: true

schema:

type: string

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AcrMgntEventsSubscription'

responses:

'200':

description: OK (Successful get the active subscription).

content:

application/json:

schema:

$ref: '#/components/schemas/AcrMgntEventsSubscription'

'204':

description: No Content

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Modify an Individual ACR Management Events Subscription

operationId: ModifyIndACRMngEventSubscr

tags:

- Individual ACR Management Events Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: Subscription Id.

required: true

schema:

type: string

requestBody:

description: Partial update an existing Individual ACR Management Events Subscription.

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/AcrMgntEventsSubscriptionPatch'

responses:

'200':

description: >

The Individual ACR Management Events Subscription is successfully modified

and the updated subscription information is returned in the response.

content:

application/json:

schema:

$ref: '#/components/schemas/AcrMgntEventsSubscription'

'204':

description: No Content.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Delete an Individual ACR Management Events Subscription

operationId: DeleteIndACRMngEventSubscr

tags:

- Individual ACR Management Events Subscription (Document)

description: Delete an existing Individual ACR Management Events Subscription.

parameters:

- name: subscriptionId

in: path

description: Subscription Id.

required: true

schema:

type: string

responses:

'204':

description: The individual subscription is deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

# Components

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

AcrMgntEventsSubscription:

type: object

description: Represents an Individual ACR Management Events Subscription.

properties:

self:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

easId:

type: string

description: Identifier of an EAS.

eventSubscs:

type: array

items:

$ref: '#/components/schemas/AcrMgntEventSubsc'

minItems: 1

description: The subscribed ACR management events.

evtReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

notificationDestination:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

eventReports:

type: array

items:

$ref: '#/components/schemas/AcrMgntEventReport'

minItems: 1

description: The ACR management event report(s).

availabilityInfo:

$ref: '#/components/schemas/AvailabilityNotif'

failEventReports:

type: array

items:

$ref: '#/components/schemas/FailureAcrMgntEventInfo'

minItems: 1

description: Failure event reports.

requestTestNotification:

type: boolean

description: >

Set to true by the EAS to request the EES to send a test notification.

Set to false or omitted otherwise.

websockNotifConfig:

$ref: 'TS29122_CommonData.yaml#/components/schemas/WebsockNotifConfig'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- easId

- eventSubscs

- notificationDestination

AcrMgntEventSubsc:

type: object

description: Represents an ACR Management Event Subscription.

properties:

event:

$ref: '#/components/schemas/AcrMgntEvent'

eventFilter:

$ref: '#/components/schemas/AcrMgntEventFilter'

evtReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

tgtUeId:

$ref: '#/components/schemas/TargetUeIdentification'

dnaiChgType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DnaiChangeType'

easAckInd:

type: boolean

description: >

Identifies whether EAS acknowledgement of UP path change event notifications is to be

expected. Set to "true" if the EAS acknowledgement is expected. Set to "false" if

the EAS acknowledgement is not expected. Default value is "false" if ommited.

This attribute may be provided only if the "event" attribute is set to "UP_PATH_CHG".

easChars:

type: array

items:

$ref: 'TS24558_Eees_EASDiscovery.yaml#/components/schemas/EasCharacteristics'

minItems: 1

description: A list of EAS characteristics.

trafFilterInfo:

$ref: '#/components/schemas/TrafficFilterInfo'

servContPlanInd:

type: boolean

description: >

Represents the service continuity planning indication (i.e., whether or not the EES

shall monitor whether the UE(s) enter the predicted location).

When set to true, it indicates that service continuity planning is required.

When set to false, it indicates that Service continuity planning is not required.

The default value when this attribute is omitted is false.

easAckSvcCont:

type: boolean

description: >

Indicates that the EAS will provide an acknowledgement as a response to the notification

of ACR management notification related to service continuity planning. Set to "true" if

the EAS acknowledgement is expected. Default value is "false". This attribute may be

provided only if the "event" attribute is set to "ACR_MONITORING" and/or

"ACR_FACILITATION".

required:

- event

AcrMgntEventsSubscriptionPatch:

type: object

description: >

Represents a modification request of Individual ACR Management Events Subscription.

properties:

eventSubscs:

type: array

items:

$ref: '#/components/schemas/AcrMgntEventSubsc'

minItems: 1

description: The subscribed ACR management events.

evtReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

notificationDestination:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

AcrMgntEventsNotification:

type: object

description: Represents the ACR management events notification.

properties:

subpId:

type: string

description: >

String identifying the Individual ACR Management Events Subscription

for which the notification is delivered.

eventReports:

type: array

items:

$ref: '#/components/schemas/AcrMgntEventReport'

minItems: 1

description: A list of ACR management event reports.

required:

- subpId

- eventReports

AcrMgntEventReport:

type: object

description: Represents an ACR management event report.

properties:

event:

$ref: '#/components/schemas/AcrMgntEvent'

timeStamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

upPathChgInfo:

$ref: '#/components/schemas/UpPathChangeInfo'

easEndPoint:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

actStatus:

$ref: '#/components/schemas/ActStatus'

acrParams:

$ref: '#/components/schemas/ACRParameters'

acId:

type: string

ueId:

$ref: '#/components/schemas/TargetUeIdentification'

selACRScen:

type: array

items:

$ref: '#/components/schemas/SelectedACRScenarios'

minItems: 1

easInBdlInfoList:

type: array

items:

$ref: '#/components/schemas/EasInBundleInfo'

minItems: 1

description: Represents the list of EAS in a bundle related information.

servContPlanInd:

type: boolean

description: >

Represents the service continuity planning indication (i.e., whether or not the EES will

monitor whether the UE(s) enter the predicted location).

When set to true, it indicates that service continuity planning will be performed.

When set to false, it indicates that Service continuity planning will not be performed.

The default value when this attribute is omitted is false.

inOutOfServArea:

$ref: '#/components/schemas/InOutOfArea'

required:

- event

ACRParameters:

type: object

description: Represents the ACR parameters.

properties:

predictExpTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

FailureAcrMgntEventInfo:

type: object

description: Represents a failure ACR management event.

properties:

event:

$ref: '#/components/schemas/AcrMgntEvent'

failureCode:

$ref: '#/components/schemas/AcrMgntEventFailureCode'

required:

- event

- failureCode

TargetUeIdentification:

description: Identifies the target UE information.

type: object

properties:

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

edgeUeId:

type: string

intGrpId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

extGrpId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ExternalGroupId'

ueIpAddr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/IpAddr'

oneOf:

- required: [gpsi]

- required: [edgeUeId]

- required: [intGrpId]

- required: [extGrpId]

- required: [ueIpAddr]

AvailabilityNotif:

type: object

description: >

Represents the availability information of user plane path management events monitoring

via the 3GPP 5GC network.

properties:

availabilityStatus:

$ref: '#/components/schemas/AvailabilityStatus'

required:

- availabilityStatus

UpPathChangeInfo:

description: Represents user plane path change information.

type: object

properties:

ueId:

$ref: '#/components/schemas/IndUeIdentification'

dnaiChgType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DnaiChangeType'

sourceTrafficRoute:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RouteToLocation'

targetTrafficRoute:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RouteToLocation'

sourceDnai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

targetDnai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

srcUeIpv4Addr:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Ipv4Addr'

srcUeIpv6Prefix:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

tgtUeIpv4Addr:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Ipv4Addr'

tgtUeIpv6Prefix:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

required:

- ueId

- dnaiChgType

IndUeIdentification:

description: Represents identification information of a UE.

type: object

properties:

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

externalId:

$ref: 'TS29122_CommonData.yaml#/components/schemas/ExternalId'

ueIpAddr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/IpAddr'

oneOf:

- required: [gpsi]

- required: [externalId]

- required: [ueIpAddr]

TrafficFilterInfo:

description: Represents the Traffic Filter Information.

type: object

properties:

ipFlows:

type: array

items:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/FlowDescription'

minItems: 1

description: Contains the flow description for the Uplink and/or Downlink IP flows.

uris:

type: array

items:

type: string

minItems: 1

description: Indicates URI(s) matching criteria.

domainNames:

type: array

items:

type: string

minItems: 1

description: Indicates Domain Name matching criteria.

dnProtocol:

$ref: 'TS29122_PfdManagement.yaml#/components/schemas/DomainNameProtocol'

anyOf:

- required: [ipFlows]

- required: [uris]

- required: [domainNames]

SelectedACRScenarios:

type: object

description: >

Represents the selected ACR scenario(s) applicable for a given combination of AC and UE.

properties:

acrList:

type: array

items:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ACRScenario'

minItems: 0

acId:

type: string

ueId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

required:

- acrList

- acId

- ueId

EasAckInformation:

type: object

description: Represents the EAS acknowledgement information.

properties:

resCode:

$ref: '#/components/schemas/ResultCode'

required:

- resCode

EasInBundleInfo:

type: object

description: >

Represents EAS in a bundle information.

properties:

easId:

type: string

description: Contains the identifier of the EAS that is within an EAS bundle.

dnais:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

minItems: 1

svcArea:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ServiceArea'

required:

- easId

#

# ENUMERATIONS DATA TYPES

#

AcrMgntEvent:

anyOf:

- type: string

enum:

- UP_PATH_CHG

- ACR_MONITORING

- ACR_FACILITATION

- ACT_START_STOP

- ACR_SELECTION

- OUT_OF_SERVICE_AREA

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the ACR Management event.

Possible values are:

- UP_PATH_CHG: Indicates that ACR Management Event is the User plane path change event.

- ACR_MONITORING: Indicates that ACR Management Event is the ACR monitoring event.

- ACR_FACILITATION: Indicates that ACR Management Event is the ACR facilitation event.

- ACT_START_STOP: Indicates that ACR Management Event is the ACT start/stop event.

- ACR_SELECTION: Indicates that ACR Management Event is the ACR selection event.

- OUT_OF_SERVICE_AREA: Indicates that ACR Management Event is the out of service area event.

AcrMgntEventFilter:

anyOf:

- type: string

enum:

- INTRA_EDN_MOBILITY

- INTER_EDN_MOBILITY

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the filtering criteria for the ACR Management event.

Possible values are:

- INTRA_EDN_MOBILITY: Indicates that the ACR Management Event filter is intra-EDN mobility.

- INTER_EDN_MOBILITY: Indicates that the ACR Management Event filter is inter-EDN mobility.

ActStatus:

anyOf:

- type: string

enum:

- ACT_START

- ACT_STOP

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the ACT status.

Possible values are:

- ACT_START: Indicates ACT start.

- ACT_STOP: Indicates ACT stop.

AcrMgntEventFailureCode:

anyOf:

- type: string

enum:

- 3GPP_UP_PATH_CHANGE_MON_NOT_AVAILABLE

- 3GPP_SERVICE_AREA_MON_NOT_AVAILABLE

- OTHER_REASONS

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the failure reason for the ACR Management event.

Possible values are:

- 3GPP_UP_PATH_CHANGE_MON_NOT_AVAILABLE: Indicates that the ACR Management Event

Subscription failed because user plane path management event notifications from the 3GPP

network is NOT available. This value is only applicable for the "UP_PATH_CHG",

"ACR_MONITORING" and "ACR_FACILITATION" events.

- 3GPP_SERVICE_AREA_MON_NOT_AVAILABLE: Indicates that ACR Management Event Subscription

failed because area of interest location monitoring from the 3GPP network is not

available. This value is only applicable for the "OUT_OF_SERVICE_AREA" event.

- OTHER_REASONS: Indicates that the ACR Management Event Subscription failed for other

reasons. This value is applicable for all events.

AvailabilityStatus:

anyOf:

- type: string

enum:

- AVAILABLE

- NOT_AVAILABLE

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the availability status.

Possible values are:

- AVAILABLE: Indicates availability.

- NOT_AVAILABLE: Indicates unavailability.

ResultCode:

anyOf:

- type: string

enum:

- ACCEPTED

- REJECTED

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the result code of ACT acceptance by EAS.

Possible values are:

- ACCEPTED: Indicates acceptance of the ACT.

- REJECTED: Indicates rejection of the ACT.

InOutArea:

anyOf:

- type: string

enum:

- MOVED_INTO_AREA

- MOVED_OUT_OF_AREA

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the presence status.

Possible values are:

- MOVED_INTO_AREA: Indicates that the UE moved into the service area.

- MOVED_OUT_OF_AREA: Indicates that the UE moved out of the service area.

# * * * * End of Changes * * * *