3GPP TSG-CT3 Meeting #136	C3-244403

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024			(revision of C3-244341)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.558 | CR | 0213 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Service continuity for common EAS serving UE(s) of the same application group |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Samsung, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph3 |  |  |  |  |  | Date: |  |  | 2024-08-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per the agreed CR S6-242642 and S6-240792 in Rel-19 of TS 23.558, the requirement is to provide Application Group ID information if it is connected to common EAS for specific application group for the purpose of Service continuity for common EAS which serves multiple UEs who are part of the same application group. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes to solve the above issue with the below implementation: - the new attribute appGrpId is included in IndividualSessionContext data structure. - The new attribute eecCntxs is included in the EECContextPush data structure. - New feature EdgeApp_3 is introduced. - Update Eees_EECContextRelocation API definition with the above change in the data model. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not alligned with the stage-2 requirement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.7.5.2.3, 8.7.5.2.4, 8.7.7, A.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces a backwards compatible correction into the OpenAPI file of the Eees_EECContextRelocation API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First Change * * * *

##### 8.7.5.2.3	Type: IndividualSessionContext

Table 8.7.5.2.3-1: Definition of type IndividualSessionContext

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| easId | string | M | 1 | Contains the application identifier (e.g., URI, FQDN) of the Application Server (e.g., EAS, CAS) providing the application services. |  |
| endPt | EndPoint | M | 1 | Contains the end point information of the Application Server provided within the "easId" attribute. |  |
| acId | string | O | 0..1 | Contains the identifier of the AC for which the service session information is provided. |  |
| acrList | array(ACRScenario) | O | 1..N | Contains the list of the selected ACR scenario(s). | EdgeApp_2 |
| eecId | string | O | 0..1 | Contains the identifier of the EEC. | enNB1 |
| appGrpId | string | O | 0..1 | Contains the application group identifier. | EdgeApp_3 |

# * * * * 2nd Change * * * *

##### 8.7.5.2.4	Type: EECContextPush

Table 8.7.5.2.4-1: Definition of type EECContextPush

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eesId | string | M | 1 | Contains the identifier of the requesting entity (e.g., S-EES, CES) pushing the EEC context. |  |
| eecCntx | EECContext | M | 1 | Contains the EEC Context to be relocated. |  |
| tgtEas | EndPoint | O | 0..1 | Contains the endpoint information of the selected Application Server (e.g., T-EAS, CAS). | EdgeApp_2 |
| acrScenariosSelReq | boolean | O | 0..1 | Contains the ACR scenarios selection request. - When set to "true", this attribute indicates to the EES to select the ACR scenarios list. - When set to "false", this attribute indicates to the EES not to select the ACR scenarios list. - The default value when this attribute is omitted is "false". | EdgeApp_2 |
| eecCntxs | array(EECContext) | C | 1..N | Contains the list of the additional EEC Context(s) to be relocated. | EdgeApp_3 |

# * * * * 3rd Change * * * *

### 8.7.7	Feature negotiation

General feature negotiation procedures are defined in clause 7.8. Table 8.7.7-1 lists the supported features for Eees_EECContextRelocation API.

Table 8.7.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | EdgeApp_2 | This feature indicates the support of the phase 2 of the definition of EDGE applications support. Within this feature, the following enhancements are covered: - Support the "ACR Selection" event subscription and reporting. |
| 2 | enNB1 | This feature indicates the support of the enhancements of this application layer API. Within this feature, the following enhancements are covered: - Support the the EEC ID within the IndividualSessionContext data type. |
| 3 | EdgeApp_3 | This feature indicates the support of the phase 3 of the definition of EDGE applications support. Within this feature, the following enhancements are covered: - Support the application group identifier within the IndividualSessionContext data type. - Support more than one element of the EEC Context(s) to be relocated within EECContextPush data type. |

# * * * * 4th Change * * * *

# A.8	Eees_EECContextRelocation API

openapi: 3.0.0

info:

title: EES EEC Context Relocation API

version: 1.1.0

description: |

API for EEC Context Relocation.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.558 V18.6.0 Enabling Edge Applications;

Application Programming Interface (API) specification; Stage 3

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.558/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/eees-eeccontextreloc/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 7.5 of 3GPP TS 29.558.

paths:

/eec-contexts:

post:

summary: Push EEC Context information.

operationId: PushEecContexts

tags:

- EEC contexts (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/EECContextPush'

responses:

'200':

description: >

OK. The EEC context has been successfully pushed to the EES and related

information is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/EECContextPushRes'

'204':

description: No Content. The EEC context has been successfully pushed to the EES.

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

get:

summary: Pull EEC Context information.

operationId: PullEecContexts

tags:

- EEC contexts (Collection)

parameters:

- name: ees-id

in: query

description: Unique identifier of the requesting entity.

required: true

schema:

type: string

- name: eec-cntx-id

in: query

description: Unique identifier of the EEC context.

required: true

schema:

type: string

- name: sess-cntxs

in: query

description: List of service session context information being requested.

required: false

schema:

$ref: '#/components/schemas/SessionContexts'

responses:

'200':

description: >

OK. The EEC context information matching the query parameters in the request

shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/EECContext'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

SessionContexts:

type: object

description: Represents the list of service session contexts information.

properties:

sessCntxs:

type: array

items:

$ref: '#/components/schemas/IndividualSessionContext'

minItems: 1

description: List of service session contexts information.

required:

- sessCntxs

IndividualSessionContext:

type: object

description: Represents a single service session context information.

properties:

easId:

type: string

description: Identifier of the Application Server providing the application services.

endPt:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

acId:

type: string

description: Identifier of the AC for which the service session information is provided.

acrList:

type: array

items:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ACRScenario'

minItems: 1

description: The list of selected ACR scenarios.

eecId:

type: string

appGrpId:

type: string

required:

- easId

- endPt

EECContextPush:

type: object

description: Represents the EEC context push request data.

properties:

eesId:

type: string

description: Identifier of the entity that is pushing the EEC context information.

eecCntx:

$ref: '#/components/schemas/EECContext'

tgtEas:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

acrScenariosSelReq:

type: boolean

description: >

Set to true if the EEC indicates to select the ACR scenarisos.

Set to false if it is not required to select the ACR scenarios.

The default value when omitted is false.

eecCntxs:

type: array

items:

$ref: '#/components/schemas/EECContext'

minItems: 1

required:

- eesId

- eecCntx

EECContextPushRes:

type: object

description: Represents the EEC context push response data.

properties:

implReg:

$ref: '#/components/schemas/ImplicitRegDetails'

selAcrScenariosList:

type: array

items:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ACRScenario'

minItems: 1

ImplicitRegDetails:

type: object

description: Represents the EEC implicit registration details.

properties:

regId:

type: string

description: Identifier of the EEC registration.

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

required:

- regId

EECContext:

type: object

description: Represents the EEC context information.

properties:

eecId:

type: string

description: Unique idenitfier of the EEC.

cntxId:

type: string

description: Unique idenitfier assigned to the EEC context.

ueId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

e1Subs:

type: array

items:

type: string

minItems: 1

description: List of subscription IDs for the capability expsoure for the EEC ID.

ueLoc:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

acProfs:

type: array

items:

$ref: 'TS24558_Eees_EECRegistration.yaml#/components/schemas/ACProfile'

minItems: 1

description: List AC profiles.

sessCntxs:

$ref: '#/components/schemas/SessionContexts'

eecSrvContSupp:

$ref: '#/components/schemas/EECSrvContinuitySupport'

ueMobSuppInd:

type: boolean

description: >

Set to true to indicate that UE Mobility support is required.

Set to false to indicate that UE mobility support is not required.

The default value when omitted is false.

required:

- eecId

- cntxId

EECSrvContinuitySupport:

type: object

description: Represents the EEC service continuity support details.

properties:

srvContSupp:

type: boolean

description: >

Set to true if EEC supports service continuity.

Set to false if EEC does not supports service continuity.

The default value when omitted is false.

acrScenarios:

type: array

items:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ACRScenario'

minItems: 1

description: The list of ACR scenarios supported by EEC.

required:

- srvContSupp

# * * * * End of Changes * * * *