3GPP TSG-CT WG1 Meeting #150	C1-244533

# Maastricht, Netherlands, 19-23 August 2024	(was C1-244050)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 6334 | rev | 1 | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of IEI for Protocol description IE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2024-08-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The Protocol description IE is a type 6 information element for which, according to TS 24.007, the IEI shall be encoded as follows: "The receiver shall assume the following rule for IEs with unknown IEI: … Bit 8 of the IEI octet set to "0" and bits 7 to 5 set to "1" indicates a TLV-E formatted type 6 IE, i.e. the following two octets are length octets. Bit 8 of the IEI octet set to "0" and bit 7 to 5 set to any other bit combination indicates a TLV formatted type 4 IE, i.e. the following octet is a length octet". The IEI values 36 and 38 which have been assigned for the Protocol description IE are violating this rule. Backward compatibility anaylsis: This CR is not backward compatible since a UE implemented based on privous versions would expect a different IEI and would even assume a Type4 IE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The IEIs for the Protocol description IE in the PDU SESSION ESTABLISHMENT ACCEPT and PDU SESSION MODIFICATION COMMAND message are corrected to a value which is in line with the requirements in TS 24.007. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A legacy UE receiving the Protocol description IE will treat the IE as a Type 4 IE. This can result in serious message decoding problems, e.g., in the detection of an "invalid information error" if the comprehension required mechanism is triggered. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.2.1, 8.3.9.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.3.2.1	Message definition

The PDU SESSION ESTABLISHMENT ACCEPT message is sent by the SMF to the UE in response to PDU SESSION ESTABLISHMENT REQUEST message and indicates successful establishment of a PDU session. See table 8.3.2.1.1.

Message type:	PDU SESSION ESTABLISHMENT ACCEPT

Significance:	dual

Direction:	network to UE

Table 8.3.2.1.1: PDU SESSION ESTABLISHMENT ACCEPT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | PDU session ID | PDU session identity 9.4 | M | V | 1 |
|  | PTI | Procedure transaction identity 9.6 | M | V | 1 |
|  | PDU SESSION ESTABLISHMENT ACCEPT message identity | Message type 9.7 | M | V | 1 |
|  | Selected PDU session type | PDU session type 9.11.4.11 | M | V | 1/2 |
|  | Selected SSC mode | SSC mode 9.11.4.16 | M | V | 1/2 |
|  | Authorized QoS rules | QoS rules 9.11.4.13 | M | LV-E | 6-65538 |
|  | Session AMBR | Session-AMBR 9.11.4.14 | M | LV | 7 |
| 59 | 5GSM cause | 5GSM cause 9.11.4.2 | O | TV | 2 |
| 29 | PDU address | PDU address 9.11.4.10 | O | TLV | 7-31 |
| 56 | RQ timer value | GPRS timer 9.11.2.3 | O | TV | 2 |
| 22 | S-NSSAI | S-NSSAI 9.11.2.8 | O | TLV | 3-10 |
| 8- | Always-on PDU session indication | Always-on PDU session indication 9.11.4.3 | O | TV | 1 |
| 75 | Mapped EPS bearer contexts | Mapped EPS bearer contexts 9.11.4.8 | O | TLV-E | 7-65538 |
| 78 | EAP message | EAP message 9.11.2.2 | O | TLV-E | 7-1503 |
| 79 | Authorized QoS flow descriptions | QoS flow descriptions 9.11.4.12 | O | TLV-E | 6-65538 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.11.4.6 | O | TLV-E | 4-65538 |
| 25 | DNN | DNN 9.11.2.1B | O | TLV | 3-102 |
| 17 | 5GSM network feature support | 5GSM network feature support 9.11.4.18 | O | TLV | 3-15 |
| 18 | Serving PLMN rate control | Serving PLMN rate control 9.11.4.20 | O | TLV | 4 |
| 77 | ATSSS container | ATSSS container 9.11.4.22 | O | TLV-E | 3-65538 |
| C- | Control plane only indication | Control plane only indication 9.11.4.23 | O | TV | 1 |
| 66 | IP header compression configuration | IP header compression configuration 9.11.4.24 | O | TLV | 5-257 |
| 1F | Ethernet header compression configuration | Ethernet header compression configuration 9.11.4.28 | O | TLV | 3 |
| 72 | Service-level-AA container | Service-level-AA container 9.11.2.10 | O | TLV-E | 4-65538 |
| 71 | Received MBS container | Received MBS container 9.11.4.31 | O | TLV-E | 9-65538 |
| 70 | N3QAI | N3QAI 9.11.4.36 | O | TLV-E | 9-n |
| 73 | Protocol description | Protocol description 9.11.4.39 | O | TLV-E | 6-n |

* * * Next Change * * * *

#### 8.3.9.1	Message definition

The PDU SESSION MODIFICATION COMMAND message is sent by the SMF to the UE to indicate a modification of a PDU session. See table 8.3.9.1.1.

Message type:	PDU SESSION MODIFICATION COMMAND

Significance:	dual

Direction:	network to UE

Table 8.3.9.1.1: PDU SESSION MODIFICATION COMMAND message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | PDU session ID | PDU session identity 9.4 | M | V | 1 |
|  | PTI | Procedure transaction identity 9.6 | M | V | 1 |
|  | PDU SESSION MODIFICATION COMMAND message identity | Message type 9.7 | M | V | 1 |
| 59 | 5GSM cause | 5GSM cause 9.11.4.2 | O | TV | 2 |
| 2A | Session AMBR | Session-AMBR 9.11.4.14 | O | TLV | 8 |
| 56 | RQ timer value | GPRS timer 9.11.2.3 | O | TV | 2 |
| 8- | Always-on PDU session indication | Always-on PDU session indication 9.11.4.3 | O | TV | 1 |
| 7A | Authorized QoS rules | QoS rules 9.11.4.13 | O | TLV-E | 7-65538 |
| 75 | Mapped EPS bearer contexts | Mapped EPS bearer contexts 9.11.4.8 | O | TLV-E | 7-65538 |
| 79 | Authorized QoS flow descriptions | QoS flow descriptions 9.11.4.12 | O | TLV-E | 6-65538 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.11.4.6 | O | TLV-E | 4-65538 |
| 77 | ATSSS container | ATSSS container 9.11.4.22 | O | TLV-E | 3-65538 |
| 66 | IP header compression configuration | IP header compression configuration 9.11.4.24 | O | TLV | 5-257 |
| 74 | Port management information container | Port management information container 9.11.4.27 | O | TLV-E | 4-65538 |
| 1E | Serving PLMN rate control | Serving PLMN rate control 9.11.4.20 | O | TLV | 4 |
| 1F | Ethernet header compression configuration | Ethernet header compression configuration 9.11.4.28 | O | TLV | 3 |
| 71 | Received MBS container | Received MBS container 9.11.4.31 | O | TLV-E | 9-65538 |
| 72 | Service-level-AA container | Service-level-AA container 9.11.2.10 | O | TLV-E | 4-65538 |
| 5A | Alternative S-NSSAI | S-NSSAI 9.11.2.8 | O | TLV | 3-10 |
| 70 | N3QAI | N3QAI 9.11.4.36 | O | TLV-E | 9-n |
| 73 | Protocol description | Protocol description 9.11.4.39 | O | TLV-E | 6-n |

NOTE:	It is possible for networks compliant with version 15.2.1 or earlier versions of this specification to send the Mapped EPS bearer contexts IE with IEI of value "7F" for this message.

* * * End of Changes * * * *