3GPP TSG-CT Meeting #105	CP-242241

# Melbourne, Australia; 9th – 10th September 2024

3GPP TSG-CT WG1 Meeting #150	C1-245057

# Maastricht, Netherlands, 19-23 August 2024

# Title:	Reporting back on CEN's requirements for eCall over IMS

# Response to:	LS (C1-244033/SP-240973) on "Reply LS on alignment of eCall over IMS with CEN" from SA

# Release:	Rel-19

Source:	CT1

To:	SA

Cc:	CT, SA1, SA2

Contact Person:

Name:	ChenHo CHIN

Tel. Number:	+32 477 413371

E-mail Address:	chin.chenho@oppo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	C1-245026, C1-245025, C1-244731

1. Overall Description:

CT1 thanks SA for the LS (SP-240973/C1-244033) tasking CT1 to analyse the gaps in our TSes to meet CEN's requirements in CEN's case #1 to #9 and communicate back to SA (cc to SA1 and SA2) its findings. CT1 understands that CT1 leaves up to SA the "final decision on how to proceed with normative work".

Herewith are CT1's findings:

Finding i):	CT1 considers that all 9 CEN cases can be aligned, but also considers that CEN case #1 on test call is of utmost urgency. Because of this urgency, CT1 has endorsed  a CR in C1-245026, (see attached). 
However even with the attached CR in C1-245026, there are some outstanding issues to aligning to CEN case #1, see following findings.

Question 1	Is aligning CEN requirement #1 first before aligning other CEN requirements agreeable to SA?

Finding ii)	CT1 considers that all 9 CEN cases can be aligned. However, for now CT1 have also progressed CEN case #2, #3, #4 and #5 and have endorsed CRs in C1-245025 (for CEN case#2 and #5) and in C1-244731 (for CEN case #3 and #4).

Question 2:	Can SA inform CT1 whether CT1 should further progress CRs for CEN cases #6 to case #9?

Question 3:	For the attached endorsed CRs, CT1 has agreed a WID dedicated to the CEN requirements. 
Is it agreeable to SA to have a dedicated WI for the normative work to align CEN requirements?

Finding iii):	On CEN Case #1: test call is a normal IMS call based on CEN requirements. CT1 considered that using dedicated URNs can work. However, as the test call is executed as a normal IMS call, then in a home routed scenario, the home P-CSCF has to be profiled to recognise that the normal call is actually to be treated as a test emergency call else, for example, the MSD transfer would not be possible or allowed.

Question 4	SA and SA WGs to decide if this shortfall is acceptable?

Finding iv)	CT1 note that CEN requirements if aligned to our specifications in a non-discriminatory way will change existing behaviours and requirements as those CEN requirements will be mandated to all parties. Thus, CT1 feels that existing behaviours are to be kept but make the changes for CEN mandatory to where and when CEN requirements apply. i.e. CT1 will perform changes for alignment to CEN use cases for IVS only and where and when CEN requirements are applicable.

Question 5	Does SA agree that existing behaviours are to be kept and that CEN requirements are mandated only where applicable to CEN?

Finding v)	On PSAP callback – part of CEN Case#1, #2, #3, CT1 finds that 
a)	for test eCall being conducted as a normal IMS call, there is no need for the UE to support PSAP callback.
b)	there need to be a wait time that the UE waits for a PSAP callback and that this time can be variable and be provisioned by the PLMN/Operator.
The attached CRs reflects this.

Question 6	Is this way-forward agreeable to SA?

Finding vi)	On CEN case #5, CEN wishes to make the support of in-band modem optional for PSAP. In the current architecture, in-band modem support in PSAP is needed for requesting and providing MSD in the cases:-

1)	an IMS emergency call of the automatically or manually initiated eCall type of emergency service, which was established in PS and later on handed over to CS;

2)	PSAP callback, which was established in PS and later on handed over to CS;

3)	PSAP callback, which was accepted by the UE in CS; or

4)	IMS non-emergency call of the test eCall type, which was established in PS and later on handed over to CS.

A way forward is to make the PSAP requirements of support of in-band modem conditional to support requesting and providing MSD in the cases 1), 2), 3), 4) above.
i.e Full optionality is not possible but optionality except requesting and providing MSD in cases 1), 2) 3) and 4) above can be a way forward. This way is reflected in the attached CR in C1-245025.

Question 7	Is this way-forward agreeable to SA?

Finding vii)	For all 9 CEN use cases, CT1 considers that the TS (potentially) impacted will be:

Required cases as per CEN requirements/LS:

For Case-1, alignment will impact CT1 TS 24.229. See also case-5.

For Case-2, alignment will impact SA1 TS 22.101, SA2 TS 23.167, possibly TS 23.292 and CT1 TS 24.229. See also case-5.

For Case-3, alignment with CEN impacts SA2 TS 23.167, and CT1 TS 24.229,

For Case-4, alignment with CEN impacts CT1 TS 24.229.

For Case-5, TBD as consideration of above findings need to be taken first. If way forward in finding vi) is agreeable, CT1 TS 24.229 is impacted.

Optional cases as per CEN requirements/LS:

For Case-6, Information provided by CEN seems contradicting, but if alignment is needed, there will be impact in TS 23.167, and TS 24.229.

For Case-7, alignment with CEN, impacts SA2 TS 23.167 and CT1 TS 24.229.

For Case-8, alignment with CEN impacts SA1 TS 22.101, SA2 TS 23.167 and CT1 TS 24.229.

For Case-9, alignment with CEN impacts SA1 TS 22.101, SA2 TS 23.167 and CT1 TS 24.229.

2. Actions:

To SA group.

ACTION: 	CT1 kindly request SA to take note of CT1's findings and provide answers to above questions and provide guidance on proceeding with further normative work.

3. Date of Next CT1 Meetings:

CT1#151                           14th October- 18th  October 2024..............Hefei, China

CT1#152                           18th November- 22nd  October 2024..............Orlando, US