3GPP TSG-CT WG1 Meeting #152	C1-246438

Orlando, US, 18– 22 November 2024

Source:	Nokia

Title:	Discussion paper on procedure to handle unavailable alternative slices

Agenda item:	5GProtoc19, eNS_Ph3

Document for:	DISCUSSION

# 1	Introduction

This discussion paper further describes the issues previously discussed in CT1#150 in C1-244391 regarding the missing behavior for network slicing feature when handling an alternative slice, i.e., the slice that is used as replacement for a replaced slice, that is unavailable, e.g., due to unexpected congested in the network. This paper describes the problem to be solved and provides a solution which is included in our companion CRs (C1-245538 and C1-245544).

# 2	Discussion

For network slicing up to Rel-18 specification, the feature of network slice replacement has not considered the issue that arises when an alternative slice, i.e., a slice which is used for replacing another slice, becomes unavailable. This behavior needs to be included in the specification in order to avoid having the network slice replacement feature not working as intended.

Observation 1:  Currently in the specification there is no text for both the network and the UE that describes the 		procedure to follow when the alternative slice – for which the UE has been updated to operate with 		during the replacement of the original slice – becomes not available.

Based on the combination of the feature of network slice replacement and the potential (un-)availability of the alternative slice, we can depict the following 3 scenarios (S1, S2, S3), where Trep = replacement period/congestion time of the replaced slice, Talt = time availability of the alternative slice:

- Scenario 1: Replacement period of the replaced slice and the alternative slice have the same time duration: Trep = Talt

- Scenario 2: Replacement period of the replaced slice has a shorter time duration than the alternative slice duration: Trep < Talt and,

- Scenario 3: Replacement period of the replaced slice has a longer time duration than the alternative slice duration: Trep > Talt.

The time availability of the alternative slice can be shorter than the time needed for the slice to be used as replacement, i.e., Trep > Talt, due to the alternative slice being congested itself or based on some other time constraint.

- In this case, the availability of the alternative slice for the UE will stop before the expected availability of the replacement slice leaving the UE without an available slice within the network slice procedure thus requiring a certain action.

- It is not always possible to know by the network before using the alternative slice as replacement slice about the specific time availability Talt, especially if the alternative slice becomes congested itself during the network slice replacement operation. Therefore, it is hard to estimate the needed time for the replacement (Trep) in particular, if the replacement is triggered due to congestion and not simply due to maintenance.

The only scenario that the specification currently covers is the case when the replaced slice (i.e., original slice) is the one not available, e.g., due to time availability.

Observation 2: 	There are cases and scenarios that the standard is not covering such as when the duration of the alternative slice is longer than that of the replacement slice. In this case, the UE will not be able to access neither of the two slices after the duration of the alternative slice expires. This can create issues at the implementation and deployment of the two features.

The second aspect to consider is whether there are other 3GPP working groups or network functions impacted when solving the issue. In our view, from a network perspective only the AMF and UE are impacted when introducing this solution, i.e., only CT1 group needs to do some work, while the rest of groups are not required to do any normative changes. Moreover, no new requirements are needed from stage 2 perspective since the changes proposed in our solution tackle the missing behavior of the already implemented feature, i.e., network slice replacement and slice availability.

Observation 3: 	The changes proposed in this paper only require CT1 group to perform some specification work and do not mandate new requirements from stage 2 for this feature.

In the following section, we highlight a potential solution to solve the issue highlighted in the previous section.

# 3	Proposed Solutions

3.1	     Alternative 1

The first alternative we are proposing is a backwards compatible solution where we define the currently missing procedure/behavior both for the network and the UE when the alternative slice is not available, and it is currently in use. In order to include this behavior, we propose to create a new IE in order to allow to include more than one replaced slice per replaced slice in the IE. This does not mean that several alternative slices are mapped at the same time for a given replaced slice but that the UE when the further replacement operation is triggered has information about more than one alternative slice.

Observation 4: 	This solution is backwards compatible and allows for session continuity, i.e., the PDU session is not      	stopped and restarted, while moving from one slice to the other.

A new UE behavior is to be specified to ignore the previous Alternative NSSAI IE (in Rel-18) if the Extended Alternative NSSAI IE (new IE for Rel-19) is received and trigger SSC mode 3 upon receiving the new Extended Alternative NSSAI (i.e. setup the new PDU session with alternative S-NSSAI_2 before releasing the PDU session with alternative S-NSSAI_1).   Additionally, and extension an extension of the PDU Session Modification Command to enable seamless PDU session modification between several alternative slices mapped to the same replaced slice. This will enable seamless switch of alternative S-NSSAI similar to the legacy case of the network slice replacement.

Observation 4: 	This will enable seamless switch of alternative S-NSSAI similar to the legacy case of the network 	slice replacement.

Proposal 1: Include the solution described in C1-246440 to solve the identified issue of the missing UE and network   behavior for the case when the alternative slice becomes unavailable while being in used for network slice replacement.

3.2	Alternative 2

The second alternative involves that once the first alternative slice that has been used to replace the original slice of the UE is not available anymore, e.g., Talt is exceeded because the alternative slice was a temporary slice or due to congestion, the alternative NSSAI IE is updated removing the previous alternative S-NSSAI with Talt and adding new network slice replacement for the replaced S-NSSAI. In the current specification the following text will be added:

If the alternative slice is not available and the AMF determines that the S-NSSAI which has been replaced is not yet available, the AMF provides the updated alternative NSSAI excluding the non-available alternative S-NSSAI to the UE during the UE configuration update procedure or during the registration procedure.

This addition has the advantage of “cleaning/flashing” the Alternative NSSAI, (i.e., removing the non-available Alternative S-NSSAI), which currently is only updated when the replaced S-NSSAI becomes available again.  Additionally, in the current specification in the Alternative NSSAI, the Replaced S-NSSAI and the Alternative S-NSSAI, i.e., each entry, is not deleted/updated. Therefore, it is possible that we run out of entries since the maximum number is eight. We need to trigger the update of Alternative NSSAI IE by using UE configuration update procedure or registration procedure.

Proposal 2:  Include the solution described in C1-246441 where the alternative NSSAI IE is updated removing the previous alternative S-NSSAI due to unavailability and adding new network slice replacement for the replaced S-NSSAI.

# 4	Conclusion

In this section, we highlight the observations and proposal(s) from this discussion paper:

Observation 1:  Currently in the specification there is no text for both the network and the UE that describes the 		procedure to follow when the alternative slice – for which the UE has been updated to operate with 		during the replacement of the original slice – becomes not available.

Observation 2: 	There are cases and scenarios that the standard is not covering such as when the duration of the alternative slice is longer than that of the replacement slice. In this case, the UE will not be able to access neither of the two slices after the duration of the alternative slice expires. This can create issues at the implementation and deployment of the two features.

Observation 3: 	The changes proposed in this paper only require CT1 group to perform some specification work and do not mandate new requirements from stage 2 for this feature.

Observation 4: 	This will enable seamless switch of alternative S-NSSAI similar to the legacy case of the network 	slice replacement.

Proposal 1: Include the solution described in C1-246440 to solve the identified issue of the missing UE and network   behavior for the case when the alternative slice becomes unavailable while being in used for network slice replacement.

Proposal 2: Include the solution describes in C1-246441 where the alternative NSSAI IE is updated removing the previous alternative S-NSSAI due to unavailability and adding new network slice replacement for the replaced S-NSSAI.