# 3GPP TSG-CT WG4 Meeting #126	C4-245069

# Orlando, US, 18 – 22 November, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.244 | CR | 0884 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Description for handling of allowed VLAN tags and use of VLAN Handling Information in SMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_VLANSUB |  |  |  |  |  | Date: |  |  | 2024-11-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Stage 2 has agreed to introduce VLAN tag handling information and list of allowed VLAN tags in SM subscription data in UDM. - VLAN handling information (e.g. the C-TAG to be inserted or removed, S-TAG to be inserted or removed); this shall apply only for PDU Session of Ethernet PDU type and is further described in clause 5.6.10.2. - The SMF may receive a list of allowed VLAN tags from DN-AAA (for a maximum of 16 VLAN tags) or as part of subscription data from UDM (for a maximum of 16 VLAN tags) or may be locally configured with allowed VLAN tags values. The corresponding behaviour for the SMF upon receiving allowed VLAN tags and VLAN handling information should be described. |  |  |  |  |  |  |  |  |
| d |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds the description for handling of allowed VLAN tags and use of VLAN Handling Information in SMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with stage 2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.13.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501 CR 5693 TS 23.502 CR 5048 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Proposed changes:

# *** 1st Change ***

### 5.13.6	Ethernet PDU session anchor relocation

The UPF (PSA) of an Ethernet PDU session may be relocated as specified in clause 4.3.5.8 of 3GPP TS 23.502 [29].

A UPF that supports the Ethernet PDU session relocation procedure shall set the ETHAR bit in the UP Function Features (see clause 8.2.25).

If the UPF indicated support of Ethernet PDU session anchor relocation, the SMF may request the UPF to update its list of MAC addresses and possibly associated VLAN tag(s) associated to the PDU session, during an Ethernet PDU session relocation procedure, by sending a PFCP Session Modification Request with Ethernet context information including the MAC addresses (and possibly associated VLAN tag(s)) received from the old Ethernet PDU session anchor.

Upon receipt of such information, the UPF shall consider these MAC addresses (and possibly associated VLAN tag(s)) as associated to the PDU session and the UPF may assist in the update of Ethernet forwarding tables of Ethernet switches in the DN as specified in clause 4.3.5.8 of 3GPP TS 23.502 [29].

### 5.13.x	Handling of allowed VLAN tags and VLAN Handling Information.

The SMF may be provisioned with a list of allowed VLAN tags (for a maximum of 16 VLAN tags) from the UDM or the DN-AAA or may be locally configured for a PDU Session. If allowed VLAN tags are available for a PDU Session, based on the local configuration, the SMF decides whether the allowed VLAN tags correspond to a C-TAG or S-TAG and provision them to the UPF in the C-TAG and/or S-TAG IEs accordingly in the Ethernet Packet Filter IE as shows in table 7.5.2.2-3, so packets not matching the C-TAG and/or S-TAG IE(s) in the Ethernet Packet Filter shall be dropped at the UPF.

The SMF shall also determine instructions on VLAN handling, i.e. using the Outer Header Creation IE to insert a C-TAG and/or S-TAG for the UL packets, and the Outer Header Removal to remove the C-TAG and/or S-TAG for the DL packets correspondingly, based on local configuration, or VLAN handling information in subscription data received from UDM, or VLAN handling information received from the DN-AAA as specified in clause 5.6.10.2 of 3GPP TS 23.501 [28].

The allowed VLAN tags and VLAN handling information received from the DN-AAA takes precedence over the allowed VLAN tags and VLAN handling information in subscription data received from the UDM. Similarly, the allowed VLAN tags and the VLAN handling information from the UDM and the DN-AAA takes precedence over the local configuration.

*** End of Changes ***