3GPP TSG-CT WG4 Meeting #126	C4-245168

Orlando, U.S; 18th – 22nd November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.518 | CR | 1145 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NRPPa Periodic Indication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_DLPMR |  |  |  |  |  | Date: |  |  | 2024-10-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SA2#164 has agreed S2-2410962 which specified the solution to use NRPP Periodic Reporting mechanism for 5G-MT-LR with deferred periodic reporting procedure. As agreed in the CR, when the LMF has requested the NG-RAN for periodic reporting via NRPPa, the LMF shall provide NRPPa Periodic Indication in determine location response to the (H-)GMLC via AMF via Namf_Location_EventNotify operation: ### 6.3.x Deferred 5GC-MT-LR Procedure for Periodic Location Events based on NRPPa Periodic Measurement ### 6.3.x.1 Initiation and Reporting of Periodic Events based on NRPPa Periodic Measurement … 3. Deferred 5GC-MT-LR steps 18-21 are executed as described in clause 6.3.1 with the following changes - The NRPPa periodic indication is added to step 18-20 to indicate Periodic Location Estimates are generated based on NRPPa Periodic Measurement. (H)GMLC stores NRPPa periodic indication. After receiving the NRPPa periodic indication, the (H-)GMLC will include it in subsequent Cancel Deferred Location procedure to AMF (via V-GMLC if required). The AMF when received this indication shall skip the cancel location transaction to the UE: ### 6.3.x.2 Cancellation of Reporting of Periodic Events by an AF, an NF or External LCS Client or GMLC Cancellation of Reporting of Periodic Events by an AF, an NF or External LCS Client or GMLC based on NRPPa Periodic Measurement executed as decribed in 6.3.3 with the following changes: - Steps from 6 to 9 are not executed based on NRPPa periodic indication provided by VGMLC in step 4. (ie.: UE’s related procedures are not executed) This CR add the NRPPa periodic indication in AMF Location API. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1/ Add new IE in NotifiedPosInfo and CancelPosInfo data types to carry the NRPPa periodic indication. 2/ Update service descriptions for the usage of the new indications. 3/ Update the OpenAPI accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NRPPa Periodic Indication cannot be provided by LMF to (H-)GMLC and cannot be used in subsequent cancel location procedure, stage 2 requirement cannot be fulfilled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.2.3.1, 5.5.2.5.1, 6.4.6.2.4, 6.4.6.2.7, A.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.273 CR 0589 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible new features on OpenAPI files of: - TS29518_Namf_Location.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 5.5.2.3.1	General

The EventNotify service operation is used in the following procedure:

-	5GC-NI-LR Procedure (see 3GPP TS 23.273 [42], clause 6.10.1)

-	Location Continuity for Handover of an Emergency session from NG-RAN (see 3GPP TS 23.273 [42], clause 6.10.3)

-	Completion of a deferred location for the UE available event or activation of deferred location for periodic location, area event triggered location or motion event triggered location (see 3GPP TS 23.273 [42], clause 6.3.1)

The EventNotify service operation notifies the NF Service Consumer (i.e. GMLC) about the UE location related event information related to regulatory services e.g. the initiation, handover or termination of an emergency session. The notification is delivered to:

-	the callback URI registered in the NRF, if the GMLC registered to the NRF with notification endpoints for location notifications (see clauses 6.1.6.2.4 and 6.1.6.3.4 of 3GPP TS 29.510 [29]); or

-	GMLC URI locally provisioned in the AMF.

The EventNotify service operation notifies the NF Service Consumer (i.e. GMLC) about the UE location related event information related to deferred location request, i.e. the activation of deferred location request or the delivery of event reports. The notification is delivered to:

-	the callback URI received from the GMLC during an earlier ProvidePositioningInfo service operation;

NOTE:	During a handover procedure, both the source AMF and the target AMF can invoke the EventNotify service operation, based on the local configuration.

The operation is invoked by issuing a POST request to the callback URI of the NF Service Consumer (See clause 6.4.5.2.2). See also figure 5.5.2.3.1-1.

Figure 5.5.2.3.1-1: UE Location Notification

1.	The AMF shall send a POST request to the callback URI provided by the NF service consumer determined as described above. The request body shall include the type of location related event and UE Identification (SUPI or PEI), and may include the GPSI, Geodetic Location, Local Location, Civic Location, MSC server identity, the Position methods used or a serving LMF identification for activation of periodic or triggered location. During deferred periodic location procedure, the nrppaPeriodicInd with the value true shall also be included if the LMF has indicated that the Periodic Location Estimates are generated based on NRPPa Periodic Measurement.

2a.	On success, "204 No content" shall be returned by the NF Service Consumer.

2b.	On failure or redirection, the appropriate HTTP status code (e.g. "403 Forbidden") indicating the error shall be returned and appropriate additional error information should be returned.

* * * Next Changes * * * *

##### 5.5.2.5.1	General

This service operation is used in the following procedure:

-	Cancellation of Reporting of Location Events by an AF or External LCS Client (see 3GPP TS 23.273 [42], clause 6.3.3)

The CancelLocation service operation shall be invoked by the NF Service Consumer (e.g. GMLC) to cancel reporting periodic or events triggered location.

The NF Service Consumer shall invoke the service operation by sending a POST request to the URI of the "cancel-pos-info" custom operation on the "Individual UE Context" resource (See clause 6.4.3.2.4.4). See also figure 5.5.2.5.1-1.

Figure 5.5.2.5.1-1: Cancellation of reporting periodic or events triggered location of the UE

1.	The NF Service Consumer shall send a POST request to the resource URI of "cancel-pos-info" custom operation of the "Individual UE context" resource of the AMF. The content of the POST request shall contain a "CancelLocInfo" data structure indicating the desired cancellation of reporting periodic or events triggered location of the UE.

2a.	On success, AMF responds with "204 No Content". If the nrppaPeriodicInd IE with the value true is received. The AMF shall skip the cancel location procedures towards the UE.

2b.	On failure or redirection, one of the HTTP status code listed in Table 6.4.3.2.4.4-2 shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application errors.

* * * Next Changes * * * *

##### 6.4.6.2.4	Type: NotifiedPosInfo

Table 6.4.6.2.4-1: Definition of type NotifiedPosInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| locationEvent | LocationEvent | M | 1 | This IE shall contain the type of event that caused the location procedure to be initiated. |  |
| supi | Supi | C | 0..1 | This IE shall contain the SUPI if available (see NOTE 1). |  |
| gpsi | Gpsi | C | 0..1 | This IE shall contain the GPSI if available (see NOTE 1). |  |
| pei | Pei | C | 0..1 | This IE shall contain the PEI if available (see NOTE 1). |  |
| locationEstimate | GeographicArea | O | 0..1 | If present, this IE shall contain an estimate of the location of the UE in universal coordinates and the accuracy of the estimate. |  |
| localLocationEstimate | LocalArea | O | 0..1 | When present, this IE shall indicate a local area in renference system. |  |
| ageOfLocationEstimate | AgeOfLocationEstimate | O | 0..1 | If present, this IE shall contain an indication of how long ago the location estimate was obtained. |  |
| timestampOfLocationEstimate | DateTime | O | 0..1 | When present, this IE shall indicate the estimated UTC time when the location estimate corresponded to the UE location (i.e. when the location estimate and the actual UE location was the same). |  |
| velocityEstimate | VelocityEstimate | O | 0..1 | If present, this IE shall contain an estimate of the velocity of the target UE, composed by horizontal speed, vertical speed, and their respective uncertainty. |  |
| positioningDataList | array(PositioningMethodAndUsage) | O | 0..9 | If present, this IE shall indicate the usage of each non-GANSS positioning method that was attempted to determine the location estimate, either successfully or unsuccessfully. |  |
| gnssPositioningDataList | array(GnssPositioningMethodAndUsage) | O | 0..9 | If present, this IE shall indicate the usage of each GANSS positioning method that was attempted to determine the location estimate, either successfully or unsuccessfully. |  |
| ecgi | Ecgi | O | 0..1 | If present, this IE shall contain the current EUTRAN cell location of the target UE as delivered by the 5G-AN. |  |
| ncgi | Ncgi | O | 0..1 | If present, this IE shall contain the current NR cell location of the target UE, or the NR cell location of the relay UE if remoteUeInd IE is present with the value true, as delivered by the 5G-AN. |  |
| remoteUeInd | boolean | C | 0..1 | This IE shall be present if received from LMF. Presence of this IE with the value false shall be prohibited. |  |
| servingNode | NfInstanceId | O | 0..1 | If present, this IE shall contain the address of the serving node. For intra-5GS handover of an IMS Emergency Call, this IE shall contain the address of the target side serving node. For mobility of a UE with periodic or triggered location, this IE shall contain the address of the new serving node, if available. |  |
| targetMmeName | DiameterIdentity | C | 0..1 | This IE shall be present for handover of IMS emergency call to EPS, i.e. the target node is an MME. When present, this IE shall indicate the Diameter host name of the target MME. |  |
| targetMmeRealm | DiameterIdentity | C | 0..1 | This IE shall be present for handover of IMS emergency call to EPS, i.e. the target node is an MME. When present, this IE shall indicate the Diameter realm of the target MME. |  |
| utranSrvccInd | boolean | C | 0..1 | This IE shall be present with value "true" for 5G-SRVCC to 3GPP UTRAN of IMS emergency call, i.e. target node is an MSC. When present, this IE shall be set for the following value: true: IMS emergency call handover to UTRAN false: No IMS emergency call handover to UTRAN |  |
| civicAddress | CivicAddress | O | 0..1 | If present, this IE contains a location estimate for the target UE expressed as a Civic address. |  |
| barometricPressure | BarometricPressure | O | 0..1 | If present, this IE contains the barometric pressure measurement as reported by the target UE. |  |
| altitude | Altitude | O | 0..1 | If present, this IE indicates the altitude of the positioning estimate. |  |
| hgmlcCallBackURI | Uri | C | 0..1 | This IE contains the callback URI of the H-GMLC This IE shall be included for a locationEvent related to deferred location when the consumer NF is not the H-GMLC. |  |
| ldrReference | LdrReference | C | 0..1 | This IE contains an LDR Reference. This IE shall be included for a locationEvent related to deferred location. |  |
| servingLMFIdentification | LMFIdentification | C | 0..1 | This IE contains the identification of a serving LMF and shall be included for a locationEvent related to deferred location with periodic or triggered location if a serving LMF is used. |  |
| terminationCause | TerminationCause | C | 0..1 | This IE indicates a reason for termination and shall be included for a locationEvent related to deferred location if deferred location has been terminated. |  |
| achievedQos | MinorLocationQoS | O | 0..1 | When present, this IE shall contain the achieved Location QoS Accuracy of the estimated location. This IE shall be present if received from LMF. | MUTIQOS |
| mscServerId | E164Number | O | 0..1 | This IE may be sent from AMF to GMLC, during a 5G-SRVCC handover from NG-RAN to UTRAN procedure. When present, it shall contain the international E.164 number of the MSC Server selected by the MME_SRVCC. |  |
| haGnssMetrics | HighAccuracyGnssMetrics | C | 0..1 | This IE should be included when received from LMF. When present, this IE shall indicate the high accuracy GNSS metrics for the location estimate. |  |
| indoorOutdoorInd | IndoorOutdoorInd | O | 0..1 | When present, this IE shall indicate whether the UE is indoor or outdoor. |  |
| relatedApplicationlayerId | ApplicationlayerId | O | 0..1 | Identifies the application layer ID of the related UE for ranging and sidelink positioning, such as located UE, reference UE, etc. | Ranging_SL |
| distanceDirection | RangeDirection | O | 0..1 | When present, this IE identifies a distance and direction from a point A to a point B, comprising a distance from point A to point B, an azimuth direction from point A to point B and an elevation direction from point A to point B. | Ranging_SL |
| 2dRelativeLocation | 2DRelativeLocation | O | 0..1 | When present, this IE identifies a relative 2D location with uncertainty ellipse, characterised by a point described in 2D local co-ordinates with origin corresponding to another known point, distances r1 and r2 and an angle of orientation A. | Ranging_SL |
| 3dRelativeLocation | 3DRelativeLocation | O | 0..1 | When present, this IE identifies a relative 3D location with uncertainty ellipsoid, characterised by a point described in 3D local co-ordinates with origin corresponding to another known point, distances r1 (the "semi-major uncertainty"), r2 (the "semi-minor uncertainty") and r3 (the "vertical uncertainty") and an angle of orientation A (the "angle of the major axis"). | Ranging_SL |
| relativeVelocity | VelocityEstimate | O | 0..1 | When present, this IE identifies UE velocity relative to the UE identified with relatedApplicationlayerId. | Ranging_SL |
| integrityResult | IntegrityResult | C | 0..1 | This IE should be present when the integrity requirements are present in the request. When present, this IE shall indicate the integrity result. |  |
| nrppaPeriodicInd | boolean | C | 0..1 | This IE shall be present with the value true to indicate that the Periodic Location Estimates are generated based on NRPPa Periodic Measurement. Presence of this IE with the value false shall be prohibited. |  |
| NOTE 1: At least one of these IEs shall be present in the message. |  |  |  |  |  |

* * * Next Changes * * * *

##### 6.4.6.2.7	Type: CancelPosInfo

Table 6.4.6.2.7-1: Definition of type CancelPosInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supi | Supi | M | 1 | SUPI |
| hgmlcCallBackURI | Uri | M | 1 | Callback URI of the H-GMLC |
| ldrReference | LdrReference | M | 1 | LDR Reference |
| servingLMFIdentification | LMFIdentification | C | 0..1 | Serving LMF identification. This IE shall be included if available. |
| supportedFeatures | SupportedFeatures | C | 0..1 | This IE shall be present if at least one feature defined in clause 6.4.8 is supported. |
| nrppaPeriodicInd | boolean | C | 0..1 | This IE shall be present with the value true if the LMF has indicated that the Periodic Location Estimates are generated based on NRPPa Periodic Measurement. Presence of this IE with the value false shall be prohibited. |

* * * Next Changes * * * *

# A.5	Namf_Location

*************** Text Skipped for Clarity ********************

NotifiedPosInfo:

description: Data within EventNotify notification

type: object

properties:

locationEvent:

$ref: '#/components/schemas/LocationEvent'

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

pei:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

locationEstimate:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/GeographicArea'

localLocationEstimate:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/LocalArea'

ageOfLocationEstimate:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/AgeOfLocationEstimate'

timestampOfLocationEstimate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

velocityEstimate:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/VelocityEstimate'

positioningDataList:

type: array

items:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/PositioningMethodAndUsage'

minItems: 0

maxItems: 9

gnssPositioningDataList:

type: array

items:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/GnssPositioningMethodAndUsage'

minItems: 0

maxItems: 9

ecgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ecgi'

ncgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ncgi'

remoteUeInd:

type: boolean

enum:

- true

servingNode:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

targetMmeName:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DiameterIdentity'

targetMmeRealm:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DiameterIdentity'

utranSrvccInd:

type: boolean

civicAddress:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/CivicAddress'

barometricPressure:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/BarometricPressure'

altitude:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/Altitude'

hgmlcCallBackURI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

ldrReference:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/LdrReference'

servingLMFIdentification:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/LMFIdentification'

terminationCause:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/TerminationCause'

achievedQos:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/MinorLocationQoS'

mscServerId:

$ref: 'TS29503_Nudm_UECM.yaml#/components/schemas/E164Number'

haGnssMetrics:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/HighAccuracyGnssMetrics'

indoorOutdoorInd:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/IndoorOutdoorInd'

losNlosMeasureInd:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/LosNlosMeasureInd'

relatedApplicationlayerId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationlayerId'

distanceDirection:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/RangeDirection'

2dRelativeLocation:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/2DRelativeLocation'

3dRelativeLocation:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/3DRelativeLocation'

relativeVelocity:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/VelocityEstimate'

integrityResult:

$ref: 'TS29515_Ngmlc_Location.yaml#/components/schemas/IntegrityResult'

nrppaPeriodicInd:

type: boolean

enum:

- true

required:

- locationEvent

*************** Text Skipped for Clarity ********************

CancelPosInfo:

description: Data within a Cancel Location Request

type: object

properties:

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

hgmlcCallBackURI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

ldrReference:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/LdrReference'

servingLMFIdentification:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/LMFIdentification'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

nrppaPeriodicInd:

type: boolean

enum:

- true

required:

- supi

- hgmlcCallBackURI

- ldrReference

*************** Text Skipped for Clarity ********************

* * * End of Changes * * * *