3GPP TSG-CT WG1 Meeting #152	C1-246134

# Orlando, US, 18-22 November 2024

3GPP TSG CT WG3 Meeting #138	C3-246037

# Orlando, US, 18th – 22nd November, 2024

3GPP TSG CT WG4 Meeting #126	C4-245199

# Orlando, US, 18th – 22nd November, 2024

Source:	Huawei, HiSilicon

Title:	Discussion on the stage 2 status of AmbientIoT

Agenda item:	19.2.2

Document for:	Discussion and Information

1. Introduction

SA1 started the stage 1 study work on R19 FS_AmbientIoT in March 2022 and completed the stage 1 normative work in December 2023. Hereafter, SA approved the stage 2 SID on R19 FS_AmbientIoT in December 2023 and then SA2 has started the stage 2 study work on R19 FS_AmbientIoT since January 2024. Up to October 2024, the status on SA2 study work is 85% and the target completion date is December 2024. It is expected that a stage 2 work item for the normative work following the conclusions of TR 23.700-13 is approved in December 2024.

This paper attempts to provide a technical overview on the latest work status of SA2 study work for FS_AmbientIoT, to estimate the expected scope of a CT WI corresponding to the stage 2 WI.

Some detailed analysis of the study still relevant for the technical background was provided in C1-244394, and not repeated in this paper.

2. Discussion

2.1. Status of FS_AmbientIoT

The completion rate of TR 23.700-13 is 85% after SA2#165 (October), and with the target completion in SA#106 which leaves one meeting, SA2#166, for the remaining 15% of the study. The most recent TR updates include the addition of Interim conclusions for all three key issues. There are still a number of Editor’s Notes to resolve, but these generally capture details of the solutions and the general principles of the solutions to progress for normative specification will likely not change significantly. An evaluation of the Interim conclusions gives a reasonable estimation of the expected CT WG impact to consider as input to a CT WID.

In the following clauses the TR 23.700-13 Interim conclusions are summarized per Key issue.

2.1.1. Interim conclusions for Key issue #1 (KI#1)

"Key Issue #1: Architecture support of Ambient IoT Devices" covers the network architecture support for the A-IoT devices (which is a new device type) and the A-IoT services (which is a new service type). Both Topology 1 and Topology 2 are covered by the Interim conclusions, with two alternatives per topology. Common for all alternatives are:

- A new NF is introduced, AIoTF;

- The AIoTF communicates with the AIoT Device using a new protocol;

- A Permanent A-IoT Device ID is stored in the AIoT Device; and

- The A-IoT Device is not aware of the connectivity topology.

For topology 1 the two alternatives are shown in Figures 2.1.1-1, for direct connectivity between BE reader and AIoTF, and Figure 2.1.1-2, for BS connectivity to AIoTF via the AMF.

Figure 2.1.1-1:	Example Protocol Stack between AIoTF and AIoT Device for Topology 1 (Direct Path)

Figure 2.1.1-2: Example Protocol Between AIoTF and AIoT Device for Topology 1 (indirect Path via AMF)

For topology 2 the first alternative is shown in Figure 2.1.1-3, for user plane connectivity between UE reader and AIoTF.

Figure 2.1.1-3:	Protocol stack for the user-plane architecture for Topology 2

For topology 2 the second alternative, for RRC based connectivity between UE reader and AIoTF, it is still open whether the NG-RAN connects directly to the AIoTF or via the AMF. A decision between option A and option B will impact CT WGs, but taking the other alternatives into account it is possible that additional CT WG impact for topology 2, alternative 2 is limited independently of the option selected.

Figure 2.1.1-4:	Protocol Stack for the RRC option (Option A)

Figure 2.1.1 -5:	Protocol Stack for the RRC option (Option B)

2.1.2. Interim conclusions for Key issue #2 (KI#2)

"Key Issue #2: Identification, Subscription, Registration and Connection management" covers the required existing functions enhancements on both the UE and the network sides for A-IoT devices and services. In Interim conclusions, Identifier and Identification Management conclusions are captured:

- AIoT Device is configured with a permanent AIoT Device Identifier;

- AIoT Device Identifier structure; and

- A-IoT device state definition.

Conclusions for Subscription, Registration and Connection management remains to be concluded and should be expected after SA2#166.

2.2.3. Interim conclusions for issue #3 (KI#3)

"Key Issue #3: Support of Ambient IoT Services " covers support of information transfer for Ambient IoT services and related system functionality, including the information transfer for an Ambient IoT device and for a group of Ambient IoT Devices and additionally which Ambient IoT services are exposed to AF. In Interim conclusions a set of Ambient IoT services agreed to be supported by the 5GC is captured.

Conclusions for information transfer for Ambient IoT services and related system functionality, and NEF exposure of Ambient IoT service towards the AF remain to be concluded and should be expected after SA2#166.

2.3. Analysis on potential stage 3 work in CT

Based on 23.700-13 content including the Interim conclusions, potential required stage 3 work in CT WGs can be estimated for CT work planning and as input to a CT WID to be agreed after approval of a SA2 WID.

.

Table 1. Initial analysis on potential stage 3 work in CT

| Key Issues | Summary of SA2 solutions | Potential stage 3 work | Impacted CT WG |
| --- | --- | --- | --- |
| Key Issue #1: Architecture support of Ambient IoT Devices | - Support for different architecture solution alternatives; - Addition of AIoTF. | - A new 5GC NF with new interfaces to other existing NFs, hence stage 3 function definition and protocol definition(s) are required. - Enhancement the existing AMF functions, hence stage 3 function enhancement and protocol enhancement are required. - Enhancement to the the PCC framework (i.e., SM/AM/UE policy management) and the related UDR Policy/Application/Exposure Data management framework to support Ambient IoT services related policy and charging control. - Enhancements to the Network Analytics framework to support Ambient IoT services | CT1 CT4 CT3 |
| Key Issue #2: Identification, Subscription, Registration and Connection management | - Function enhancement #1: A-IoT device ID | - Stage 2 work on identification (TS 23.003) should be updated to cover the A-IoT device ID format definition. - Stage 3 work on allocation, transport and use of A-IoT device ID. | CT1 CT4 CT3 (potential) |
|  | - Function enhancement #2: A-IoT device subscription | Stage 3 work on A-IoT device subscription data definition, transport and use in case of the A-IoT device subscription data is stored in mobile network. Stage 3 work on network layer security mechanism based on A-IoT device subscription data stored in mobile network. | CT1 CT4 |
|  | - Function enhancement #3: A-IoT device state definition | Stage 3 work on A-IoT device new state machine definition. Stage 3 work on registration management, connection management, reachability management based on the defined A-IoT device state machine. | CT1 CT4 |
| Key Issue #3: Support of Ambient IoT Services | - A-IoT data transfer between the A-IoT device and the network/application server | Stage 3 work on protocol definition between A-IoT device and the network. Stage 3 work on protocol definition between new A-IoT NF and other existing NFs. | CT1 CT4 CT3 |
|  | - IoT services definition - A-IoT services exposure to the AF | Stage 3 work on IoT services definition and its usage. Stage 3 work on A-IoT services exposure definition and its usage, including: - The specification of a new NEF Northbound API to expose Ambient IoT services to Ambient IoT AFs. - The specification of a new AIoTF API to expose Ambient IoT services to Ambient IoT AFs via the NEF. - Enhancements of existing NEF Northbound APIs (e.g., Nnef_ParameterProvision, Nnef_EventExposure, etc.) to expose Ambient IoT services to Ambient IoT AFs. | CT1 CT3 CT4 (potential) |

3 Conclusion

This paper has provided an overview on the latest work status on SA2 study work on R19 FS_AmbientIoT to estimate the CT WG impact and content of a CT WID for the expected work.

It is clear that significant impact can be expected for CT1, CT3 and CT4, even though completion of the SA2 SI and start of SA2 WI will provide additional detail that increases or decreases the needed work for individual CT WGs. It should also be noted that there is a dependency to work in RAN WGs and some work could be done in CT or RAN, e.g. protocol related impact. Under current status, it is proposed to assume CT1 as main WG for the CT WID due to the impact from introducing a new device type and the new AIoTF and the connectivity options related to this.

A draft CT WID for discussion and preparation of upcoming work is available in C1-246133/C3-246038/C4-245200.