3GPP TSG RAN Meeting #105	RP-241748

Melbourne, Australia, September 9-12, 2024

Agenda item:	9.3.2.1

Source:	Xiaomi

Title:	Discussion on Rel-19 NR mobility enhancements

Document for:	Discussion and Decision

# 1 	Introduction

In RAN#104, Revised Work Item: NR mobility enhancements Phase 4 [1] was approved. According to the objectives of the mobility WID, the collaboration on the event triggered L1 measurement reporting between the Rel-19 MIMO and the Rel-19 mobility enhancement and whether to support conditional LTM need to be clarified.

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1] - Specify support of conditional LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Prioritise intra-CU LTM - Checkpoint to review objective at RAN#105. RAN WG work to not start before this checkpoint - Specify RRM requirements related to the above objectives as necessary [RAN4]

In this contribution, we provide our understandings and analysis on the Rel-19 WID: NR mobility enhancements Phase 4.

# 2	Discussions

2.1 Event triggered L1 measurement reporting

According to the description of Objective 2, we need to review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work. According to the study progress in the Rel-19 MIMO WI, the L1 event triggered measurement report is based on the reporting framework by using UCI. According to the study progress in the Rel-19 mobility enhancement WI, the LTM event triggered measurement report is based on the reporting framework by using MAC CE (i.e. MAC layer handles the event evaluation and measurement report triggering) and LTM events which are different from the L1 measurement events agreed in the Rel-18 MIMO WI.

Observation 1: The measurement events agreed in the Rel-19 MIMO WI and the Rel-19 mobility enhancement WI are different.

Observation 2: The reporting framework using UCI in the Rel-19 MIMO WI is different from the reporting framework using MAC CE in the Rel-19 mobility enhancement WI.

Since the purpose of the LTM event triggered measurement report is for mobility, and the purpose of the MIMO L1 event triggered measurement report is for beam management, we think that the Rel-19 MIMO WI and the Rel-19 mobility enhancement WI can progress independently on the event triggered L1 measurement reporting.

Proposal 1: To clarify that the Rel-19 MIMO WI and the Rel-19 mobility enhancement WI progress independently on the event triggered L1 measurement reporting.

2.2 Conditional LTM

For the Objective 3: Specify support of conditional LTM, we need to review objective at RAN#105. And it shall be decided at RAN#105 whether to start the RAN WG work for the conditional LTM in Rel-19.

According to the recent study progress in the Rel-19 mobility enhancement WI, Objective 1 and 2 are going smoothly. For the Objective 1: inter-CU LTM, Rel-18 intra-CU LTM is the baseline and the extra work for inter-CU LTM is relatively small, especially for RAN2. And RAN2 has started the work on two secondary priority cases of DC in RAN2#127 meeting. Regarding the security issue of inter-CU LTM, SA3 has approved a new WID [2] to find the solutions of inter-CU key change for different cases (including both DC case and non-DC case) of inter-CU LTM. RAN only needs to wait for the inputs from SA3.

For the Objective 2, Rel-18 L1 measurement can be used as a reference and RAN1 and RAN2 have good progress on measurement event design and measurement report procedures. Hence, RAN has enough TUs to devote to the extra work of conditional LTM.

Observation 3: Objective 1 and 2 are going smoothly, and RAN has enough TUs to devote to conditional LTM.

- For Objective 1: inter-CU LTM

  - Rel-18 intra-CU LTM can be the baseline and the extra work is small, especially for RAN2.

  - The security solution for inter-CU LTM, is being handled by SA3 and RAN only needs to wait for the inputs from SA3.

- For Objective 2: measurements related enhancements for purpose of supporting LTM

  - Rel-18 L1 measurement can be used as a reference.

  - RAN1 and RAN2 have good progress on measurement event design and measurement report procedures.

The current WID of the Rel-19 mobility enhancement has indicated to prioritise intra-CU case for the conditional LTM. To reduce the workload in both RAN2 and RAN3, the inter-CU conditional LTM can be excluded, to avoid the extra work on the security issues and Xn coordination between the source gNB and candidate gNBs.

Observation 4: If inter-CU conditional LTM is excluded, the workload in both RAN2 and RAN3 can be reduced.

As such, we consider that intra-CU conditional LTM can be included in Rel-19 without extra TU allocation.

Proposal 2: To support intra-CU conditional LTM in Rel-19 without extra TU allocation.

The text proposals can be found the Annex to capture the above Proposal 1 and Proposal 2.

Proposal 3: To agree on the text proposals capturing Proposal 1 and Proposal 2 in the Annex.

# 3	Conclusion

According to the analysis given above, we have the following observations and proposals:

Observation 1: The measurement events agreed in the Rel-19 MIMO WI and the Rel-19 mobility enhancement WI are different.

Observation 2: The reporting framework using UCI in the Rel-19 MIMO WI is different from the reporting framework using MAC CE in the Rel-19 mobility enhancement WI.

Observation 3: Objective 1 and 2 are going smoothly, and RAN has enough TUs to devote to conditional LTM.

- For Objective 1: inter-CU LTM

  - Rel-18 intra-CU LTM can be the baseline and the extra work is small, especially for RAN2.

  - The security solution for inter-CU LTM, is being handled by SA3 and RAN only needs to wait for the inputs from SA3.

- For Objective 2: measurements related enhancements for purpose of supporting LTM

  - Rel-18 L1 measurement can be used as a reference.

  - RAN1 and RAN2 have good progress on measurement event design and measurement report procedures.

Observation 4: If inter-CU conditional LTM is excluded, the workload in both RAN2 and RAN3 can be reduced.

Proposal 1: To clarify that the Rel-19 MIMO WI and the Rel-19 mobility enhancement WI progress independently on the event triggered L1 measurement reporting.

Proposal 2: To support intra-CU conditional LTM in Rel-19 without extra TU allocation.

Proposal 3: To agree on the text proposals capturing Proposal 1 and Proposal 2 in the Annex.

# 4 	References

[1] RP-240299, Apple Inc, China Telecom, “Revised Work Item: NR mobility enhancements Phase 4”

[2] S3-242401, Samsung, OPPO, “New WID on security aspects of NR mobility enhancement”

# 5	Annex

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1]

- Specify support of intra-CU conditional LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM